IMPALA Digest 226

Topics covered in this issue include:

1) AS&M  and Extended Warranty Contracts

by Godley@aol.com

2) Horn, Mats, and Naperville

by Mikado95@aol.com

3) Arlington Meet, Super Chevy  4-20-21,96

by DON69Z28@aol.com

4) legal gears

by "J.R. LEVITZ" <4everGreen@msn.com>

----------------------------------------------------------------------

Date: Fri, 5 Jan 1996 12:51:22 PST

From: Godley@aol.com

To: impala@east.xsis.xerox.com

Subject: AS&M  and Extended Warranty Contracts

Message-ID: <960105155122_84378362@emout05.mail.aol.com>

A few posts back someone brought up the fact that they had talked to quite a

few people that had headers installed and were rewarded by melted parts

installed close to the header(s) itself.  I asked my "connection" here in

Charlotte about this and he said as long as the headers are coated, you don't

have to worry about this happening.  So, IF YOU ORDER HEADERS MAKE SURE THEY

ARE QUALITY PARTS AND WELL COATED!

ALSO, does anyone have a connection where I can get a good deal on a Extended

Warranty Contract???  I have alway heard there is a large profit on Extended

Warranties.

Thanks,

MARK

------------------------------

Date: Fri, 5 Jan 1996 16:08:34 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: Horn, Mats, and Naperville

Message-ID: <960105190832_32969232@emout04.mail.aol.com>

We just had our Impala in to get the horn fixed today.  They're ordering us a

new horn systen and steering wheel cover.

Car Mats--I just bought the WeatherTech mats sold in the back of most auto

magazines.  For those of you in/around Chicago, their distributor is in

Downers Grove.

The work terrific and look great--at least in my Red/Black 95 Monte Carlo.

Is there a member on this list who owns a '95 Impala and a '95 Monte Carlo,

and lives in Naperville, Illinois off of River Road?  E-mail me if it's you!!

Randy in Naperville

------------------------------

Date: Fri, 5 Jan 1996 22:31:42 PST

From: DON69Z28@aol.com

To: impala@east.xsis.xerox.com

Subject: Arlington Meet, Super Chevy  4-20-21,96

Message-ID: <960106013142_107979945@mail04.mail.aol.com>

I called   Super Chevy this week and was told that  April 20-21, 1996

were the dates for the  this years Super Chevy show at the Texas

Motorplex .  I will check with the  Arlington  GM folks next week to see

if a tour is possible. If someone has already contacted the plant  please let

me know over the weekend .  I have had some  positive response on the

concept of the Arlington meet , so at the risk seaming  forward I  will

continue on .     At the least a small group could attend the Super Chevy

show as a  regional  chapter.

Don  Lambert

817-557-1529

------------------------------

Date: Sat, 6 Jan 1996 07:51:38 PST

From: "J.R. LEVITZ" <4everGreen@msn.com>

To: "Impala List" <impala@east.xsis.xerox.com>

Subject: legal gears

Message-ID: <UPMAIL01.199601061551340327@msn.com>

Rush Writes:

Does this particular component actually increase emissions?  I figure if they

test your car for emissions while it is standing still, it shouldn't matter

what gear you have installed

Rush , Believe it. Gears are part of the emmissions "package". This is

why Calloway does no change gears on their Impala kit. They could, but the EPA

FTP(Federal Test Proceedure) that would be required is very very expensive.

This is the same test that the manufacturer must do to "certify" a drive

train.

Regards John

------------------------------

End of IMPALA Digest 226

************************

IMPALA Digest 227

Topics covered in this issue include:

1) Re: 3.5 inch hose.

by Rock528@aol.com

2) BFG 17"ers in the SNOW

by lennie@earthlink.net (Lennie Dolphin)

----------------------------------------------------------------------

Date: Sat, 6 Jan 1996 15:26:57 PST

From: Rock528@aol.com

To: BGordon401@aol.com, impala@east.xsis.xerox.com

Subject: Re: 3.5 inch hose.

Message-ID: <960106182657_108462797@emout05.mail.aol.com>

In a message dated 96-01-05 21:49:30 EST, you write:

>F.Y.I. The tube in question is made by GATES, their part #24250 "Green

>Stripe" brand 3 1/2" I.D. (89mm) rubber radiator hose.  It's sold by the

>foot, but all you really need is about 3-4".  Hope this helps.

Bruce,  did you just use the hose to connect the chrome pipe to the mass air

meter or did you replace the air resonator with a long section of hose?

Having run the 3.5" chrome pipe (cut/angled/welded) for some time, I've

noticed it holds so much heat you can hardly touch it.  This defeats the cold

air induction theory.  I would like to replace the pipe with something non

metal and flexible in the same size.  Troy

------------------------------

Date: Sat, 6 Jan 1996 23:54:11 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: impala@east.xsis.xerox.com

Cc: 76165.1654@compuserve.com

Subject: BFG 17"ers in the SNOW

Message-ID: <199601070754.XAA13478@iceland.it.earthlink.net>

In digest 215 Chris wrote:

>>Traction in the snow is horrid with the ZR-17s<<

Kurt said in the 217 digest:

>I have to disagree. I live in CT and this is the second winter on the BFGs. I

have never gotten stuck, and I think the traction is excellent.

Here comes the BIG test on how well the these tires are. As I write, looks

to be a least a foot of snow outside and still coming down. So far, with 25

lbs all the way around, the tool box in the trunk, a full tank of gas, and

great posi-traction, haven't been stuck yet. Actually Kurt, I've been

letting the idle get the wheels rolling and using the ABS to stop with no

problems. The '95 SS dark cherry has only 6200m on it, can't wait to see how

the car does next winter with many miles on the tires. Safe driving to you

in "Snow Country".

... Lennie

------------------------------

End of IMPALA Digest 227

************************

IMPALA Digest 228

Topics covered in this issue include:

1) Re: 3 1/2" hose

by BGordon401@aol.com

2) O2 Sensor Info

by Steve Das <sdas@sat.net>

3) Re: cold weather impalas

by Rock528@aol.com

4) Killer upgrades!

by Scott Mueller <73145.1566@compuserve.com>

5) Cold weather, snos, etc.

by RCKOHLI@aol.com

6) Re: Too many spaces

by Kurt Kemmling <76165.1654@compuserve.com>

7) Re: IMPALA digest 226

by Steve Das <sdas@sat.net>

8) .

by CoolChevy@aol.com

9) impala sighting

by Mark Boggs <70004.173@compuserve.com>

10) Emissions Testing

by "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

11) Winter Tires

by Mfetto@aol.com

12) Exhaust system parts

by Franklin Poole <franklin@intersurf.com>

----------------------------------------------------------------------

Date: Sun, 7 Jan 1996 11:59:27 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: 3 1/2" hose

Message-ID: <960107145858_85406589@mail02.mail.aol.com>

Troy, you wrote,

>Bruce,  did you just use the hose to connect the chrome pipe to the mass air

meter or did you replace the air resonator with a long section of hose?

The former is exactly what I did with the hose.  Since there is a rather

sharp angle for the  parts to adapt to when installing the chrome piece, I

used a 3-4" section of the Gates hose at the M.A.F..  However, your thought

about the heat is almost the same as mine.  The pipe does retain the too much

heat, especially in the summer--when the chances of finding that stray SHO or

Mustang in the mood for fun seems to be greater.  To adapt the hose in place

of the pipe can be done, from what I can tell.  The hose fits very nicely

into the stock rubber coupling that's designed for the plastic resonator, but

I haven't tried to fit the hose into the elbow.  The hose might fit into the

elbow as nicely as into the M.A.F. coupling.  If you find out, drop me a

line.

For the time being, I'm going to keep my setup as is.  With the temps.

topping out at 25*, I'm not overly concerned about over-heated air.

Bruce Gordon / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Sun, 7 Jan 1996 14:31:09 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: O2 Sensor Info

Message-ID: <199601072231.QAA02239@raven.sat.net>

Hey guys,

Came across this link on one of the newsgroups.  Really interesting stuff on

the 02 sensor and how it works as well as how to test it.  Give it a look, I

think you'll like it.

http://ram.chem.tulane.edu:8080/f-body/trivia/o2sensor.html

Steve

------------------------------

Date: Sun, 7 Jan 1996 14:39:53 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: cold weather impalas

Message-ID: <960107173952_85496668@mail04.mail.aol.com>

> So far, with 25

>lbs all the way around, the tool box in the trunk, a full tank of gas, and

>great posi-traction, haven't been stuck yet

I give you guys credit as your shorts must be snugger than mine..  I don't

take my impala out of the garage if it looks like rain.

Troy

Daytona Beach

------------------------------

Date: Sun, 7 Jan 1996 16:00:03 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Killer upgrades!

Message-ID: <960108000002_73145.1566_FHD70-3@CompuServe.COM>

Happy New Year to all on the Impala list! I am still working on the roller

rocker mod, they actually work great and make a very noticible power

improvement, however new valvesprings should be installed or valve float

will occur with the stock springs at about 4,800 rpm. The problem has been

finding a compatible upgraded valvespring to fit the iron heads with out

having to machine them. The spring seat machining is very simple, and can

even be done by a simple drill arbor tool, however this requires removing

the valves, which requires removing the heads themselves. If I ever have to

take these heads off, then I would certainly recommend re-installing

aluminum heads and discarding the iron ones. The aluminum heads also have

many more springs to choose from, including the killer factory LT4

valvesprings.

In any case I think I have found the perfect bolt-on spring that will work

with the stock iron heads, and I am installing the new springs right now. I

have learned more about valvetrains including rockers, pushrods, springs,

retainers and locks in the last few weeks than I thought possible! As the

saying goes "Whatever does not kill me, makes me stronger" <g>.

How about this, I have been doing some research, and I have determined that

it would be possible to install the Overhead console (with compass and

thermometer) from the S10 Blazer in the Impala! This would look totally

factory. Another killer mod would be to install the steering wheel with

radio controls from the Monte Carlo LS in the Impala. As you may know, the

'95+ radios have the connector built-in for the steering wheel controls! The

only problem with that one is that the Monte wheel is available in black

only.

Imagine an Impala with the Camaro instrument cluster, Monte Carlo steering

wheel with radio controls, and the killer Blazer Overhead console with

compass and temperature readouts! Then throw in the seat heaters from the

Roadmaster for good measure. All of this is feasable and can be done, some

of it fairly easily! Scott.

------------------------------

Date: Sun, 7 Jan 1996 16:55:56 PST

From: RCKOHLI@aol.com

To: impala@east.xsis.xerox.com

Subject: Cold weather, snos, etc.

Message-ID: <960107195555_109373897@emout04.mail.aol.com>

Just got back from upstate N.Y. on 1/7/96 at 6PM, to my home in CT.(which

sits right on the boarder of Westchester County, N.Y.).  Anyway, N.Y. State

Gov. Pataki (hurray for mild conservatism and lower taxes, with the death

penalty to boot... Bo-oo-ooo to Liberalism, Mario, Bubba Billy Boy & Hilly

Girl---can you believe these two assholes are running our country!? I have

not yet met anybody who admits voting for them. Glad we're are not all brain

dead in the Northeast!!) declared a state of emergency for this area (NY) at

4PM.

The temp this weekend (NY) dropped to 34 degrees *below zero*, and my SS with

26,700 miles groaned twice and popped to life on one crank...BTW,it was 22

degrees below zero at 10AM with blue sky and bright sun on 1/6!   This is my

2nd winter running four 245R17 snow tires (there wasn't 6" of snow in all of

last winter in CT, the mildest winter in memory...so anything ran good last

year!), and my 34th year of winter driving, with sno's and with regular tires

and chains, with and without posi-traction, etc.!  We have had approx 1-1/2

feet of snow so far (not counting this storm).  Take my word and experience

to the bank....do not travel your SS in snow country without four snow tires,

and added weight in the trunk (unless of course you are the only one

traveling on a flat and previously plowed road), I don't care what you have

read on this List previously regarding snow tires. I had a mighty struggle

tonight *with sno tires*!!  BTW, watch out for sno/ice build up on the stock

5 spoke wheels, which causes the front end to shimmy and shake quite a

bit...pull over, clean the stuff off, and its back to normal.

Roy C. Kohli (RCKOHLI@aol.com)

Ridgefield, CT. 06877

'94 Impala SS-BIBB, Black Is Beautiful Baby, rather modified, Powered by

Lafite Rothschild, See ya' in the fast lane....RC.

------------------------------

Date: Sun, 7 Jan 1996 18:08:48 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Too many spaces

Message-ID: <960108020847_76165.1654_GHM87-4@CompuServe.COM>

To: Don Lambert

Please check whatever software you are using to send messages to the list.

They are filled with hundreds, maybe thousands of bytes of spaces.

Thank you

------------------------------

Date: Sun, 7 Jan 1996 18:17:20 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 226

Message-ID: <199601080217.UAA03838@raven.sat.net>

>  I called   Super Chevy this week and was told that  April 20-21, 1996

>were the dates for the  this years Super Chevy show at the Texas

>Motorplex .  I will check with the  Arlington  GM folks next week to see

>if a tour is possible. If someone has already contacted the plant  please let

>

>me know over the weekend .  I have had some  positive response on the

>concept of the Arlington meet , so at the risk seaming  forward I  will

>continue on .     At the least a small group could attend the Super Chevy

>show as a  regional  chapter.

>From our "for what it's worth department" I'm told that the GIANT Pate Swap

Meet, which is just south of Fort Worth, is the 24th thru the 28th of April

this year.  Those of you that are interested in older cars might want to put

this on your calender as well.  This is only about an hour's drive from the

Motorplex.

SD

------------------------------

Date: Sun, 7 Jan 1996 23:03:11 PST

From: CoolChevy@aol.com

To: impala@east.xsis.xerox.com

Subject: .

Message-ID: <960108020310_109684004@emout06.mail.aol.com>

Hello to all my Impala friends,

i'm closely following all digests and it is a great honor of being a member

of ORCA's fan club. It's like having a family in the background in case there

are troubled waters around.

Especially, thx to my cyberspace friend Scott Mueller ho helped me, an

European working for an American company who is living in Singapore/Asia to

get along with my Impala stored at my Californian companies headquarters.

Anyway, i have 95 dark cherry red SS with an ATI procharger with intercooler

and an upgrade to a 4L80e truck transmission for dependability purposes. I'm

not a friend of Paxton's or Vortech's because i had only problems with them.

I'm pretty fed up with all those gimmicks they sell to make their setup's

work (like additional injectors, ignition boxes, etc..) and my experience is

that those Mickey mouse items left me stranded always when i needed it the

last. ATI is using a simple effective intercooler to fight detonation and

there are no add. items to mount. I used a setup for a LT1 Camaro, but there

are working on a Impala SS setup which should be released soon right now.

Who has any news on the SLP exhaust system and headers ? I have AS&M headers

right now but i would consider a change if the SLP setup is better.

I read about the 3.42 gears and vette converter and i'm getting hooked to

that idea.

Could anyone give me the exact partnumbers for all necessay parts plus an

estimation of how much it would cost to have those items installed at a GM

dealer ?

I plan to do this changes this March before i ship the car to Europe due to

my latest relocation (i'm kinda gypsy, i think) but i have to get this job

done by a GM dealer, i'm not so sure about quality repair shop in the

Cupertino/ San Jose area who could tackle this job in confidence. Any owners

around from this area who could recommend someone ?

I'm very interested in the Arlington plant tour and i would fly in to visit

it. Please update me if you know more.

So long

Heinz Schenk

------------------------------

Date: Mon, 8 Jan 1996 05:32:39 PST

From: Mark Boggs <70004.173@compuserve.com>

To: Impala Mailing list <impala@east.xsis.xerox.com>

Subject: impala sighting

Message-ID: <960108133238_70004.173_FHT65-1@CompuServe.COM>

I was traveling North towards Columbus, OH on I-71, in my wife's car, when I

passed an Impala SS who was going from Northbound I-71 to Eastbound I-270.  It

was about 7:20 AM, this morning, Monday, January 8th.  So, were there any other

list members in my area this morning?

-Mark

------------------------------

Date: Mon, 8 Jan 1996 01:03:01 PST

From: "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

To: impala <impala@east.xsis.xerox.com>

Subject: Emissions Testing

Message-ID: <9601081518.AA4386@CServe-Hub4.inhouse.compuserve.com>

John:

>>Does this particular component actually increase emissions?  I figure if they

test your car for emissions while it is standing still, it shouldn't matter

what gear you have installed<<

In Illinois (and a number of other states), they are in the process of changing

the way emission testing is done.  When they first started a few years back, it

was done at stations where you pulled your car up, got some probe stuck into

your tailpipe(s), and rev'ed your car up till they got the right reading at a

given rpm.  If you timed it right at the testing stations, the whole procedure

(required once a year after a car was 3 years old) took 5 min.  You didn't even

have to get out of the car.

However, I guess they figured that the lines were not long enough at these

stations for a state-sponsered agency.  Now, they are switching to stations at

which you need to pull up your car, get out of it, and let the attendant put it

onto a special device that will allow the tires to spin as they perform the

emission test.  The process is supposed to take 2-3 times longer than the

current process.  The only good thing to come out of this is that I did not

have to test either of my cars this year, since I received a notice in the mail

telling me that they are in the process of switching the stations to the new

testing equipment.

I don't know if switching the gears on an Impala will really impact this,

though, since I understand that the car burns very cleanly, and even with a

gear switch should still remain well within their tolerances for hydrocarbons,

etc.  Heck, I've got an '80 Chev. Citation that has always passed the test with

flying colors (it still burns almost as cleanly as my '91 Cutlass).

Jeff

------------------------------

Date: Mon, 8 Jan 1996 09:49:38 PST

From: Mfetto@aol.com

To: impala@east.xsis.xerox.com

Subject: Winter Tires

Message-ID: <960108124936_85947596@mail06.mail.aol.com>

Living in Buffalo, I decided to take the precaution and install snows on my

'SS before the winter.  Installed 15" rims with 225/70 M&S's...the car goes

great...Drove it during the late '95 blizzard here with absolutely no

problems. It is worth the performance degradation for a couple of months for

safety and the preservation of the 17" wheels and tires.

BTW...the standard 15" Caprice wheel covers would not fit on to the S10 rims

that had to be installed to clear the calipers.  So I added some chrome "baby

moon" wheel covers. Needless to say, the Black SS with these tires looks

pretty bad !!

Later, Mike F.

------------------------------

Date: Mon, 8 Jan 1996 11:02:39 PST

From: Franklin Poole <franklin@intersurf.com>

To: gwb@philabs.research.philips.com

Cc: Impala List Server <impala@east.xsis.xerox.com>

Subject: Exhaust system parts

Message-ID: <30F16A4F.4187@intersurf.com>

Gert,

A trip to the local dealer got me the following information.  You have asked prices for the stock dual exhaust system for

your 4.3l LS Caprice.

You are apparently right that a 4.3 and 5.7 exhaust system are the same.

The exhaust systems are apparently the same for 94-96, but what year is your car?

Here are the prices and part numbers with basic descriptions.  I'm no expert so any input from Scott M. or others is

appreciated.

10250709	LH Muffler	84.00

10245293	RH Muffler	109.20 (muffler and pipe are one piece)

10210485	RH Resonator	101.60

10204007	LH Resonator	101.60

10249851	Shield LHR	37.20	(left hand rear)

10222996	Shield RHR	37.20

10223012	Shield Muffler	5.04

10204050	Shield Muffler	5.04

?		Hanger Bracket	9.60	(dealer forgot to write part#)

?		Pipe Bracket	14.04

======

504.52 - 30% = $354 +$10 handling + shipping $?

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

End of IMPALA Digest 228

************************

IMPALA Digest 229

Topics covered in this issue include:

1) Re: Winter Tires

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

2) Interior Mods

by "Marten, Gary" <garym@msgate.hls.com>

3) Door Panel Lights

by Scott Mueller <73145.1566@compuserve.com>

4) Re: Exhaust system parts

by Gert Bruning <gwb@philabs.research.philips.com>

5) Shifter

by Mikado95@aol.com

6) TRex has a LOUD pedal

by Franklin Poole <franklin@intersurf.com>

7) Arlington Meet, GM wants us

by DON69Z28@aol.com

8) Rear-end gasket no#

by REMANKING@aol.com

9) Factory Fog Lights!

by Scott Mueller <73145.1566@compuserve.com>

10) Re: NO Snow Tires Needed....

by Kurt Kemmling <76165.1654@compuserve.com>

----------------------------------------------------------------------

Date: Mon, 8 Jan 1996 11:53:25 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Subject: Re: Winter Tires

Message-ID: <9601082000.AA22990@karloff.fstrf.org>

At 11:40 08/1/96 PST, you wrote:

>------------------------------

>

>Date: Mon, 8 Jan 1996 09:49:38 PST

>From: Mfetto@aol.com

>To: impala@east.xsis.xerox.com

>Subject: Winter Tires

>Message-ID: <960108124936_85947596@mail06.mail.aol.com>

>

>Living in Buffalo, I decided to take the precaution and install snows on my

>'SS before the winter.  Installed 15" rims with 225/70 M&S's...the car goes

>great...Drove it during the late '95 blizzard here with absolutely no

>problems. It is worth the performance degradation for a couple of months for

>safety and the preservation of the 17" wheels and tires.

>

..edit...

Speaking of winter tires... My '96 should be coming in soon and I'm

considering winter tires/rim if the traction is bad (Mark F.: howdy

from fellow WNY'er! I'm from Niagara Falls...).  I would imagine

that the police must have a good setup on the 9C1's and want to know

if anyone knows what their local police use, or what they use if

they're an officer.  Any ideas on brand/model/size?

>Later, Mike F.

>

- mike

Happy to narrowly miss the most recent Nor'easter! :)

------------------------------

Date: Mon, 8 Jan 1996 12:06:00 PST

From: "Marten, Gary" <garym@msgate.hls.com>

To: impala <impala@east.xsis.xerox.com>

Subject: Interior Mods

Message-ID: <30F17A13@msgate.hls.com>

>How about this, I have been doing some research, and I have determined that

>it would be possible to install the Overhead console (with compass and

>thermometer) from the S10 Blazer in the Impala! This would look totally

>factory. Another killer mod would be to install the steering wheel with

>radio controls from the Monte Carlo LS in the Impala. As you may know, the

>'95+ radios have the connector built-in for the steering wheel controls!

The

>only problem with that one is that the Monte wheel is available in black

>only.

the other problem with swapping wheels is the airbag...

>Imagine an Impala with the Camaro instrument cluster, Monte Carlo steering

>wheel with radio controls, and the killer Blazer Overhead console with

>compass and temperature readouts! Then throw in the seat heaters from the

>Roadmaster for good measure. All of this is feasable and can be done, some

>of it fairly easily! Scott.

I'd be happy with a black '96 SS with the interior out of an Olds Aurora

(complete with it's light tan 8-way power seats, dual auto temp, and trip

computer)!

-Gary

------------------------------

Date: Mon, 8 Jan 1996 14:25:26 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Door Panel Lights

Message-ID: <960108222526_73145.1566_FHD63-3@CompuServe.COM>

Here is another couple of mods for everybody. From '91 through '93 the

Caprice had an optional power window/seat control flood lamp in the driver's

side door panel. This was a small triangular shaped light in the driver's

door panel only, mounted in the upper LH corner of the door release handle

bezel. This light projected light on the power window/seat controls, so they

could be more easily operated in the dark. Unfortunately this floodlight was

deleted in all '94 and up cars, including the Impala, even though the little

triangular hole remains in the door handle bezel.

It is intersting to note that this little light is still shown in the

current service manual "Component Location Views" on page 8A-201-24. You can

see the light in that illustration as the "Master Power Window Light". On

the '93 and earlier cars, you will also see the wiring for this light in the

Interior Light Dimming schematic on page 8A-117-1. The '94 and up manuals do

not show this circuit. Basically, the light is connected to the Instrument

Panel Dimmer circuit and ground.

I tried, but found that you cannot purchase the light separately, you can

only get it as a part of the entire door harness (over $200). The factory

used a tiny #37 bulb, which is a tiny wedge bulb. To cover the hole from the

inside, I fabricated a small triangular lens out of a piece of translucent

plastic from the top of a can of clear spray paint, and glued it to the

inside of the bezel. Instead of using the very dim #37 bulb, I used an

identically sized GM # 10031003 bulb, which is a #74 bulb with a blue

silicone cap. The #74 bulb is brighter than a #37 bulb, but the extra

brighness is muted by the deep blue cap.

I could not find a socket for these bulbs, so I soldered a tail of wire

directly to each of the bulb leads. While I was doing that, I made up two

bulbs just in case the first one ever burns out. I should mention that these

tiny bulbs have a very long life, and will likely never burn out at all,

thus making the search for a socket seem pointless. Also, I found that a

socket would greatly complicate the bulb mounting. All I did to mount the

bulb was to use some small pieces of duct tape to hold the tiny bulb in

place over the lens.

To wire the bulb up, I ran a single from the dimmer module under the dash,

through the door conduit, and over to the tiny bulb. Ground is provided by

tapping into the power mirror control ground wire, which is mounted in the

same bezel. Although the factory never mounted one of these bulbs on the

passenger door side, I will add the floodlight there as well.

People have commented on how you can't see the window or seat controls in

the dark, and this solves the problem. Too bad they had to remove this great

feature. Chevy and most Buick interior controls are usually not backlit, and

use tiny floodlamps to light the controls. Other cars like most Pontiacs,

use more sophisticated backlit switches that light from the inside. The new

Camaros use these tiny light projectors in the door panels to illuminate

their controls, while the Pontiac Firebird uses more sophisticated backlit

switches, which give them more of an aircraft cockpit look. Buick has the

tiny floodlights in both the driver and passenger side doors, although Chevy

only used the driver door light until '93. It was probably removed as a cost

cutting measure.

By the way, I found a loose rivet holding the window regulator bracketry to

the door, which I had to repair by installing a new rivet. Although I did

not hear any rattle, clearly it could have caused one in the future. While I

had the door panel off, I also installed several sheets of dynamat sound

deadening material to the inside of the door sheet metal. This stuff is a

thick asphalt sheeting that comes in various sized sheets. It is often used

with high end stereo installations, and really helps to quiet the car. As I

get each door panel and even the carpeting out of the car for various

reasons, I will be installing this stuff as I go. You can get it most

inexpensively from JC Whitney.

I should also mention that JC Whitney stocks what I think is the best door

panel retainer removal tool on the market. They actually sell several types,

but the best one is a tool made by Cal-Van (marked as Cal-Van #34) that has

a U-shaped handle, and a long forked plastic wedge/ramp. Unlike the metal

pry type tools also sold for removing these clips, the plastic one will not

scratch the paint under the door panel, which can lead to rust in the

future.

BTW, if you ever remove a door panel in these cars, be aware that they are

held on with a plastic retainer clip GM # 10161510 (15 cents each). Even if

you are careful, removing these clips almost always damages them, and they

are priced cheaply enough that attempting to re-use them should not be an

issue. There are 6 clips in each front door, and I don't know how many in

the rear (probably less). I would buy some extra ones for future use. At 15

cents each, 40 of them cost me only $6 total.

A tip from John Kelley; with the door panels off, you can also easily

replace the cheap factory stereo speakers. Crutchfield sells a high quality

replacement Infinity 4x6 plate speaker that will bolt-in directly as a

replacement for the factory unit in the door. They also have connector

adapters that allow these speakers to directly plug-in to the factory

harness with no wiring modifications of any kind. John tells me that they

also have killer 6x9 Infinity speakers for the rear deck as well, which are

also a bolt-in replacement available with factory connectors.

If you are installing an upgraded stereo system, and want to minimize

electrical noise, the factory has the perfect solution. All 9C1 police

package cars get a special Radio Bonding Strap Package, which consists of 8

1/4" flat braided copper ground straps in the following locations:

-Rear Bumper to Frame

-Hood to Body-from wiper motor to hood at centerline

-Trunk Lid to Body-LH hinge box

-Frame to Body Rear compartment LH side

-Exhaust Pope to Frame-LH and RH behind resonator

-Engine to Front of Dash

-Engine to Frame-RH front side

These additional grounds help to supress any electrical noise that might

interfere with Police radio equipment, however they will also do the same

for high end stereo equipment as well.

Another factory option related to the door I am installing is SEO (Police)

6N1 - Handle, Driver Door Unlock. This is a special door handle/lock rod

that unlocks the front door when you pull the inside door handle. Normally

in the Impala, if the door is locked and you pull the inside handle, the

door will not open. With SEO 6N1, pulling the door handle simultaneously

unlocks the driver's door and allows it to open. Note that this is available

for the driver's door only, and I would not want it on any other door anyway

(especially with kids). If you have the door panel off, all you have to do

is install the 6N1 lock rod, which only costs a few dollars. I don't have

the part number handy, but your dealer can easily look this one up.

I don't think I will ever be done adding factory options to this car! <G>

Scott.

------------------------------

Date: Mon, 8 Jan 1996 15:48:07 PST

From: Gert Bruning <gwb@philabs.research.philips.com>

To: franklin@intersurf.com, gwb@philabs.research.philips.com

Cc: impala@east.xsis.xerox.com

Subject: Re: Exhaust system parts

Message-ID: <199601082348.SAA18180@briar.philabs.research.philips.com>

Thanks for the prices. I think I would like to go for this.

Please send again the address for the check so I can mail it

to you.

I am surprised that one of the mufflers is less than the

other. Each one should ave the connecting pipe ( connecting

to the cat. converters ) welded on.

Please recall that I need all pipes/muffler/resonators starting

from t\both catalytic converters back to he tailpipe. The

heatshields are only needed for the driver's side muffler

and resonantor. Hangers are needed for the thedrivers side

as well. Thanks again.

Gert

------------------------------

Date: Mon, 8 Jan 1996 20:39:54 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: Shifter

Message-ID: <960108233952_110628289@mail06.mail.aol.com>

Although this pertains to my 1995 Monte Carlo Z34, I thought this list would

be helpful.

Anyone know where I can get a leather (or leather-looking) cover for my

shifter handle?

The factory one is plastic and feels/looks cheap.  By the way, I personally

like the feel of the steering wheel in the Impala better than the Monte

Carlo.   I also saw an article in Automobile magazine last month about a

rear-drive 1995 Monte Carlo show car putting out 467 horses!!!

Randy

------------------------------

Date: Mon, 8 Jan 1996 21:36:20 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: TRex has a LOUD pedal

Message-ID: <30F1FED4.1732@intersurf.com>

Yee Haa, hide the kids, keep the wife inside and call the cops! Madmaxx

is loose in your neighborhood mitout de mufflers!  This mod is

definitely not for the faint of heart!!! Don't let Scott Harris fool

you, if you tool around like your grandmother everything's ok. BUUUUUUT,

stick your foot into the throttle body like a madman and it'll remind

you of STS-53 (or was it 54?).  For the uninitiated that's a shuttle

launch.  From the VIP viewing area you see it first, then you hear the

liquid fuel engines ROOOAAAAR, theeennnnnn,  BLAAAAAMMMM, the SRB's go

off and yep, you've gone and stuck your foot down on it and every body

with his hearing aid on is looking your way and saying, "damn, you know

I thought they quit making those Impalas in the 60's!"

Seriously, I went to the local muffler shop, took the open mouth gapes

and had them replace the 50lbs of mufflers with 2" pipes and added a

crossover. IF you can keep your foot out of it it's not too bad. But,

stick it to it and no doubt, IT'S LOUD!  There's also a very annoying

drone at 60 and 70mph.  Actually about any speed where you're hovering

around 1900-2100 under load. At 80mph it's no problem.  Also a great

attention getter when you are passing someone on a two lane road.

While I'm jabberin' I've got to relate this actual conversation from

last week:

Me:	It's an Impala SS '96.  Got an LT1 'vette motor. <g>

Him:	No way, really?? (me opening hood)

Him:	Wow, says it right on the engine things, huh?!

Me:	That's what it says...<GGGG>

Me:	I heard you just bought a '95 BMW 740iL? (60K+ I think)

Him:	Sure did! Love it, it's a wonderful car!

Me:	Where is it?

Him:	In the shop.......

--

Franklin Poole, Jr.

Praecedo, Inc.

================================================

It's not how fast you go, it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Mon, 8 Jan 1996 21:38:18 PST

From: DON69Z28@aol.com

To: impala@east.xsis.xerox.com

Subject: Arlington Meet, GM wants us

Message-ID: <960109003818_110682421@mail06.mail.aol.com>

GOOD  NEWS , I talked to the GM plant today and a plant tour is possible,

unfortunately they will  not conduct one on the weekend .  I requested the

date of

19 April 1996 at 12:00 PM or 1:00 PM or as late as possible on Friday.  GM

requested a conformation letter so I sent one off today, Troy you have a copy

coming

in the regular mail .  As of today Troy was officially designated as

President of  the

94-96 SS Impala Club  and I was acting  in behalf of and under the full

authority of

our national club and the Arlington Chapter of the Certified  Chey Nuts of

America.

I guessed at a possible size of 25 people  and told them that we would

confirm once

we had a confirmation of the date.   If I hear any thing this week I will let

you

know, if you don't hear from me by Monday of next  week I will be in France

for the

balance of the week .

Check your schedules  April will be here faster than a SS can do 0 to 60, I

will start

working on a tentative schedule of events , hotels ect.

Don

PS  Erich, no problem I got the ball.....I think?

------------------------------

Date: Mon, 8 Jan 1996 21:45:03 PST

From: REMANKING@aol.com

To: impala@east.xsis.xerox.com

Subject: Rear-end gasket no#

Message-ID: <960109004502_86406469@mail06.mail.aol.com>

Can someone confirm for me that the correct Fel-pro part number for the

Impala rear end cover gasket is 55028-1.  The book shows two possible

gaskets.

Thanks in advance.

------------------------------

Date: Tue, 9 Jan 1996 02:50:04 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Factory Fog Lights!

Message-ID: <960109105004_73145.1566_FHD36-1@CompuServe.COM>

As many of you know, the direct predecessor of the Impala SS was the Caprice

LTZ. The LTZ debuted in '91 along with the re-designed Caprice. It was

basically a civillian version of the 9C1 police package, just like the

Impala SS is today. The LTZ used the police (thicker) frame, HD suspension,

HD brakes, HD steering, etc, and even had the 9C1 instrument cluster with

gauges.

If you ever get to look at the '91-'93 9C1/LTZ instrument cluster, you will

be very upset indeed! This cluster put the Impala SS cluster to shame, in

fact it is even better than the '96 Impala gauge package! The 9C1/LTZ

cluster had a digital speedo, analog tach, and full analog gauges, with

numeric calibrations, including oil pressure, temperature, voltmeter, and

fuel.

Unlike the 9C1, the LTZ was only available with the 305 (5.0L) engine in

'91-'92, while the 9C1 could get the 5.7L (350) engine. In '93, the LTZ

finally got the same engine as the 9C1, however you could only get a 3.23

axle while the 9C1 came standard with a 3.42 gear. In addition to the 5.7L

engine in '93, that year the LTZ as well as all other Caprices got the new

open rear wheel quarter panels as well as the new taillight lenses which are

still used on the Impala today. You would be interested to know that the

prototype Impala SS was based on the '93 Caprice LTZ, and in '91 the LTZ was

voted Motor Trend's Car of the Year!

In any case, the reason I am mentioning all of this is that the '91 LTZ (and

apparently only that year) had a very interesting late release option called

WM6 - Fog Lights! These are bumper mounted (we think) factory fog lights,

and this option is exceedingly rare. There were actually far fewer '91

Caprice LTZs made than even the rarest '94 Impala SS models, and even fewer

of these were equipped with the WM6 fog lights. Barry (parts manager) at

Bemis Chevy (815-895-5145) chas looked up all the parts including the

lights, brackets, wiring harness and even the dash switch, if anybody wants

to purchase these and install them. They are somewhat expensive ($350 or

so), but they are a high quality halogen fog light and of course they should

fit properly because they are factory.

What we don't know is exactly where they mount (on top or bottom of the

bumper?), or even how they look as neither of us have ever seen them on any

LTZ, and the parts books do not have any illustrations of them. I have the

Chevy '91 Caprice and even the separate '91 LTZ brochures, but neither of

these even acknowledge the option. It also does not show up in the '91 or

'92 service manuals. If anybody is willing to take a slight risk (not

knowing exactly how they look or where they mount) and order these lights,

give Barry a call! Just another rare factory option that can be added to the

Impala. <g> Scott.

------------------------------

Date: Tue, 9 Jan 1996 06:00:56 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: NO Snow Tires Needed....

Message-ID: <960109140055_76165.1654_GHM76-1@CompuServe.COM>

In digest 228, Roy Kohli wrote:

>>>  We have had approx 1-1/2

feet of snow so far (not counting this storm).  Take my word and experience

to the bank....do not travel your SS in snow country without four snow tires,

and added weight in the trunk (unless of course you are the only one

traveling on a flat and previously plowed road), I don't care what you have

read on this List previously regarding snow tires. I had a mighty struggle

tonight *with sno tires*!!  BTW, watch out for sno/ice build up on the stock

5 spoke wheels, which causes the front end to shimmy and shake quite a

bit...pull over, clean the stuff off, and its back to normal.<<<

Roy,

I just got back from a 120 mile round trip from central CT to Norwalk and back

on my 23000+ mile original equipment BFG 17 inchers. I had absolutely NO

problems driving. I was able to travel the Merrit Parkway, which you probably

know is still pretty heavily covered in snow without even a minor hint of

getting stuck. My Impala SS handles GREAT with these tires in the snow and ice.

I have to give credit to the engineers at BFG for designing the best M+S rated,

Z speed rated performance tire in the world. I do not know why you say you had

trouble driving "with sno tires" in your Impala SS. I have to take offense to

you telling the readers of this list to disregard my and other members messages

concerning our experiences with these excellent tires in the snow. This list is

here for the benefit of all of its members. The info contained within, should

not be attacked by other members just because you had a different experience.

You won't see me telling the members of this list not to listen to or to

disregard your post, and I do not think it is proper for you to do so either.

The members of this list are intelligent enough to be able to digest the

information they read on this list on their own.

I can drive with my original tires perfectly well in the snow, along with other

members of this list, and you can't. I am glad you made everyone aware of this

fact, and thank you for sharing your experience with the rest of us.

Drive safely,

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

End of IMPALA Digest 229

************************

IMPALA Digest 230

Topics covered in this issue include:

1) Reflector tape

by Clyde Twine <clyde.twine@hok.com>

2) Wow!

by Erich Arndt <EARNDT@mednet.swmed.edu>

3) Seat Heaters from Roadmaster

by "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

4) Stainless steel hose clamps

by Franklin Poole <franklin@intersurf.com>

5) Snow tires

by "J.R. LEVITZ" <4everGreen@msn.com>

6) LTZ cluster

by Scott Mueller <73145.1566@compuserve.com>

7) Seat Heaters

by Scott Mueller <73145.1566@compuserve.com>

8) Interesting Interception

by David Hardy <dhardy@crl.com>

9) RE: TRex has a LOUD pedal

by "Harris, Scott" <sharris@xyplex.com>

10) Newsletter

by SyncJam@aol.com

11) RE: Door panel lights

by "Rodger Cliff (216) 977-0036" <CLIFF#m#_RODGER@lims-a1.lerc.nasa.gov>

12) Dual exhaust for Caprice/Impala

by Franklin Poole <franklin@intersurf.com>

----------------------------------------------------------------------

Date: Tue, 9 Jan 1996 06:46:02 PST

From: Clyde Twine <clyde.twine@hok.com>

To: impala@east.xsis.xerox.com

Subject: Reflector tape

Message-ID: <9601091446.AA02244@portal.hok.com>

I did the reflector tape/bow tie thing per a fellow list member's direction

and it looks pretty cool. This could be a version of the "blue dot" :) I

dont know how it would look on a green or cherry though.

I think it would be kind of neat to have some way of distinguishing club

member cars from others and Id choose a red bow tie over some sort of

sticker or license plate holder.

ct

------------------------------

Date: Tue, 9 Jan 1996 06:58:39 PST

From: Erich Arndt <EARNDT@mednet.swmed.edu>

To: DON69Z28@aol.com, impala@east.xsis.xerox.com

Subject: Wow!

Message-ID: <s0f22e94.032@MEDNET.SWMED.EDU>

Don,

Actually, I'd say you have the situation by the throat!  I really appreciate

you jumping into the thick of it -- thanks.  (I spent the last week and a half

without success just trying to find a name of someone to talk to; I'm sure

glad you're on our side!)  I'll give you a call later today (Tuesday).

Is there anything I can do to assist on the miscellaneous details assuming

this is what the group decides to do?

Troy,

How'd the meetings with your attorney friend go about the legal stuff?

Any more word from the Late Great Chevy folks?

Group,

Looks like Don came up with the answer we were waiting for.  Now

what?  Here's a brief summary of what's up so far (Don, correct me if I'm

wrong...):

- Super Chevy is at the Texas Motorplex on the weekend of 4/20-4/21

- Arlington Plant tour on Friday 4/19 (Pending)

- Plenty of drive-in and show stuff happening

- Six Flags Over Texas opens on the weekends starting 3/2

- Wet and Wild may be open by that time

- Plenty of indoor malls, outlet malls, and sightseeing to keep the

significant others busy

All of this stuff is within a stone's throw of the plant.  The Motorplex is 45

min - 1 hour from Arlington.

Thanks,

Erich

------------------------------

Date: Tue, 9 Jan 1996 01:26:37 PST

From: "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

To: impala <impala@east.xsis.xerox.com>

Cc: "73145.1566" <73145.1566%cishub%cserve@notes2.compuserve.com>

Subject: Seat Heaters from Roadmaster

Message-ID: <9601091714.AA9327@CServe-Hub4.inhouse.compuserve.com>

Scott:

>> ...Then throw in the seat heaters from the Roadmaster for good measure. All

of this is feasable and can be done, some of it fairly easily! Scott. <<

This is a mod that I would be very interested in!  As you have said before, I

wish they had put the Bonny SSEi (or Aurora or Z28) interior into the Impala;

while I love the SS (don't take this the wrong way), its interior is, lets say,

a little bland IMO.  Then again, its a miracle that GM went through with

building this car, so I should stop complaining.

I will never understand why more GM features are not common amongst the

different vehicles, even as options at the buyer's chosing/cost.  This includes

radio controls in steering wheel (us tall <I'm 6'4"> people love that one),

traction control, 8-way power seats with deep contour, dual climate controls,

head-up display, CD + tape in one unit (Roadmaster offers this), fold-down rear

seats for access into trunk (I will never put a ski rack on the SS!), heated

leather seats, etc. and etc.

Anyway, I imagine having leather seats without a heater in the Chicagoland (and

other cold climates) makes for some pretty chilly starts in the morning.  I

have no familiarity with how these system work -- do they require some type of

special leather, or are they just a "plug-in" add-on to the existing seats?  If

an easy-to-install option, I will probably install at any cost.  Scott --

thanks again for all your posts on all the different types of mods -- very

interesting.

Also, I called ACS (an auto broker in Cincy that Scott M. recommended,  and

based on my experience so far, I will second the notion) Friday, and they said

that the Dark Cherry '96 I ordered the first week of December is scheduled to

be built this week.  If anyone has questions on this service, drop me an

e-mail.  If you hate dealing with auto salespeople, this is the way to buy a

car.

While I originally expected (and wanted) to take delivery some time after March

1 (road salt sucks!), I am looking foward to getting it as soon as possible

after reading 'bout every one else having fun with theirs.  Besides, we here in

Chicago are having a pretty mild winter, relatively speaking vs. the rest of

the U.S.!  Jeff

------------------------------

Date: Tue, 9 Jan 1996 10:15:05 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Stainless steel hose clamps

Message-ID: <30F2B0A9.3AB4@intersurf.com>

Sometimes I'm brain dead for what seems like weeks at at time.  I

replaced my coolant hoses with the green silicone ones and could not

find stainless steel clamps anywhere!  Well, duh, any good marine shop

will carry some, BUT!, the best and cheapest, easiest place I've found

is..... Home Depot!  Most of the clamps are under $1 and they are of

high quality because they are the marine style full stainless clamps.

They can be found in the plumbing area of your local store.  My local

boat store wants as much as $5 per clamp. The only difference I can see

is the bubble pack the boat store uses:-)

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Tue, 9 Jan 1996 13:28:46 PST

From: "J.R. LEVITZ" <4everGreen@msn.com>

To: "Impala List" <impala@east.xsis.xerox.com>

Subject: Snow tires

Message-ID: <UPMAIL01.199601092128300186@msn.com>

The wind chill for Phoenix Arizona today is forcasted for +78 degrees !  John

------------------------------

Date: Tue, 9 Jan 1996 10:50:56 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: LTZ cluster

Message-ID: <960109185056_73145.1566_FHD65-2@CompuServe.COM>

>How likely would it be to drop in the LTZ cluster in the dash of a '94 up?

I'm assuming the geometry of the two clusters are fairly different, other

wise you would have done it already :)

Yup, the '91-'93 and '94-'96 dashboards are COMPLETELY different. It would

be far easier to install the Camaro cluster in the new dash than to install

the LTZ cluster.

>About the door handle light, you could probably find one at the junkyard

and rip it out.  If it was small enough (plastic bezel, bulb, etc) you could

probably just pocket it and nobody would know that you took it.

That's definitely not my style! I do try to make an annual trip to the local

junkyards for little parts like screws, nuts and bolts, wire harness pieces

with various connectors, minor trim items, brackets, etc. I usually fill a

large box with this type of "junk", and then when I leave, I show them the

box full of junk and they normally charge me something like $20. I will try

to pick up a couple of these lights (among other things) the next time out,

but that won't be until the spring, and I have the door panel off now.

Patience is not one of my virtues. <g> Scott.

------------------------------

Date: Tue, 9 Jan 1996 10:51:12 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Seat Heaters

Message-ID: <960109185111_73145.1566_FHD65-3@CompuServe.COM>

Jeff,

>Anyway, I imagine having leather seats without a heater in the Chicagoland

(and other cold climates) makes for some pretty chilly starts in the

morning.  I have no familiarity with how these system work -- do they

require some type of special leather, or are they just a "plug-in" add-on to

the existing seats?  If an easy-to-install option, I will probably install

at any cost.

Well, what's your definition of "plug-in"? <g> Seriously, the heater pads

are available for both the lower and upper cushions. They simply slip

inbetween the leather seat cover and the cushion, and are about $60 each.

You would need two for each seat (upper and lower cushion). Then there is a

$120 module for each seat, and a switch $27. That would allow you to run

each seat heater separately, if you wanted to run them together, you could

probably use only one module (I am not sure about this).

I thought an easy hidden install would have the heaters switched on and off

with the rear window defogger/heated mirrors. You could do it that way, and

there would be no visible changes inside the car, or you could use the Buick

or Cadillac switches mounted on the seats to control each one independently.

I think the hardest part would be removing and re-installing the seat

covers, but you could have a trim shop do this for you. I'd sure like to

hear if somebody ever has the heaters installed! Scott.

>Also, I called ACS (an auto broker in Cincy that Scott M. recommended, and

based on my experience so far, I will second the notion) Friday, and they

said that the Dark Cherry '96 I ordered the first week of December is

scheduled to be built this week.  If anyone has questions on this service,

drop me an e-mail.  If you hate dealing with auto salespeople, this is the

way to buy a car.

I agree! I helped my mother purchase a '95 Trans Sport through ACS, and the

experience was wonderful. In the next few weeks I will be ordering a '96

Jeep Grand Cherokee Orvis edition through them as well, with every available

option! It will have the 5.2L Magnum V8 (of course), and will include the Up

Country suspension. That suspension package is not available on GC Limited

models as of '96 (only the lower end Laredo and now the Orvis), and includes

stiffer, 1" taller springs, full underbody skid plates, tow hooks, high

pressure gas shocks, revised jounce bumpers, and 16" all purpose tires.

I will be modifying the suspension further with larger swaybars that I am

having custom fabricated by a major bar manufacturer, since nobody makes

larger bars for the GC. They will dramatically improve the handling over the

stock ones, and combined with the stiffer off-road suspension, this thing

should handle pretty well. I am going to see how good the upgraded off-road

shocks are, and if I am not satisfied, I will install Bilsteins.

I have even discovered that GCs are built for German Export, which means

that some killer "T84" type headlights are available as well. I already have

a full length Borla exhaust in stock and ready to install, complete with 304

stainless headers that I also had Jet Hot coated. For even greater

performance, I am also going to change the PCM, install a 180 degree 'stat

(they use 195 stock), K&N filter, modified airbox, and will install Crane

1.6 roller rockers under the new Mopar Performance parts "Magnum" cast

aluminum valve covers.

The 5.2L Magnum engine is rated at 220 HP stock, and I figure with these

mods I should be somewhere around 275 HP! The V8 engine includes a killer

3.73 axle ratio (front and rear), which is ideal for performance. The Jeep

actually weighs less than the Impala, so imagine a high 14-second Jeep GC

and you get the picture! It should be able to take a stock Impala (sorry),

especially in the rain. <g> There, I mentioned the Impala, so this message

is not entirely off subject! <BG> If anybody is interested in Jeep GC

performance mods, send me some mail. Scott.

------------------------------

Date: Tue, 9 Jan 1996 13:44:58 PST

From: David Hardy <dhardy@crl.com>

To: Impala SS LIST <impala@east.xsis.xerox.com>

Subject: Interesting Interception

Message-ID: <Pine.SUN.3.91.960109132514.17645C-100000@crl14.crl.com>

As you may know, the new police 9C1 Caprice has the same motor and basic

powertrain of the Impala...and, the performance of these cars is pretty

similar.  I also reported that the CHP has purchased more 95 9C1 Caprices

than anyone.  Here's a bit from a pursuit I monitored that occurred near

Santa Rosa, CA a few weeks ago.  Note that the majority of the CHP

current fleet are Crown Victorias, and the 9C1 are slowly being phased in:

DISPATCH:	110 FAILURE TO YIELD

110:		FAILURE TO YIELD, SOUTH 101 AT EAST WASHINGTON, RED FIVE

PIONT OH MUSTANG, 2 ABOARD, NUMBER ONE LANE AT 120.

DISPATCH:	10-4 UNITS IN POSISTION?

112:		112 IS ENROUTE, ABOUT 3/4 MILES BEHIND

111:		111 WILL SET UP, PETALUMA BLVD.

DISPATCH:	COPY 112 ENROUTE, 111 SETTING UP.

[4 MINUTES OF LOCATIONS GRADUALLY APPROACHING PETALUMA BLVD. AT OVER 100 MPH]

110:		110 APPROACHING PETALUMA...IS THERE A CHEVY OUT HERE...I'M

LOSING SIGHT OF THIS VEHICLE.

DISPATCH:	110 REQUESTING A CHEVY [SLIGHT GIGGLING IN BACKGROUND]

111:		111 HAS A CHEVY, I'LL SET UP AT PETALUMA.

110:		111 FROM 110, THIS THING [FORD] ISN'T CUTTING IT, I'LL

LET YOU KNOW WHEN TO COME ON.

111:		10-4

110:		111 GO, NOW...APPROACHING PETALUMA BLVD. NUMBER 1 130 PLUS

111:		111, IN PURSUIT, CAL LICENSE XXXXXXX SOUTH BOUND 101

SOUTH OF PETALUMA.

DISPATCH:	COPY 111 IS PRIMARY.

[TWO MINUTES OF CHANGING LANES AND CONTINUING SOUTH AT 130+]

111:		111, LOOKS LIKE VEHICLE IS YIELDING.

DISPATCH:	10-4 10-20

110:		WITH 111 SOUTH AT 44.

DISPATCH:	10-4 UNITS, VEHICLE IS YIELDING SOUTH 101 AT MARKER 44

112:		CODE 4 TWO IN CUSTODY.

Hehe...two minutes of that big bad Chevy on the rear bumper of that 5.0

ended it!  Gotta love it.

-Dave

dhardy@crl.com  %%%%%%%%%%%%%%%%%%%%%%%%%%%%%

"The thief is sorry he is to be hanged,

not that he is a thief" (Thomas Fuller, 1732)

------------------------------

Date: Tue, 9 Jan 1996 04:58:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: "'franklin@intersurf.com'" <franklin@intersurf.com>,

Subject: RE: TRex has a LOUD pedal

Message-ID: <30F2F371@corvette.xyplex.com>

I would bet that the H pipe is not installed properly.  My car was as

loud at full throttle as you describe without the H installed.  It

sounded like a car with glasspacks, basically that's what it is.  Once

the H was put in the car became MUCH quieter.  Is the crossover installed

just after the transmission where the pipes are very close together?

That's where mine is.  You want the H as close to the end of the cats as

possible.  I hate to ask this but... did you watch them cut the holes in

the pipes before they installed the H?  From your description it wounds

like they forgot to do that.

I never tried to "fool" anyone.  I stated it was not for everyone and did

say it was louder than stock, it has to be, you removed the mufflers!  My

only complaint is the part throttle drone from 1,600-1,800 you describe

around 60 MPH.  Once the engine reaches 2,000 rpm it sounds sweet.  Who

drives at 60 MPH anyway?  75-80 is very nice, thank you.

Putting it in perspective, my car is quieter at all rpms than my 69 350

Camaro with Turbo mufflers and is quieter than almost any car with

Flowmasters.  You won't find many chances in life where you get ~20 HP

this cheaply or easily.

Scott Harris

sharris@xyplex.com

=-=-=-=-=-=

>>Yee Haa, hide the kids, keep the wife inside and call the cops! Madmaxx

is loose in your neighborhood mitout de mufflers!  This mod is

definitely not for the faint of heart!!! Don't let Scott Harris fool

you, if you tool around like your grandmother everything's ok. BUUUUUUT,

stick your foot into the throttle body like a madman and it'll remind

you of STS-53 (or was it 54?).  For the uninitiated that's a shuttle

launch.  From the VIP viewing area you see it first, then you hear the

liquid fuel engines ROOOAAAAR, theeennnnnn,  BLAAAAAMMMM, the SRB's go

off and yep, you've gone and stuck your foot down on it and every body

with his hearing aid on is looking your way and saying, "damn, you know

I thought they quit making those Impalas in the 60's!"

>>Seriously, I went to the local muffler shop, took the open mouth gapes

and had them replace the 50lbs of mufflers with 2" pipes and added a

crossover. IF you can keep your foot out of it it's not too bad. But,

stick it to it and no doubt, IT'S LOUD!  There's also a very annoying

drone at 60 and 70mph.  Actually about any speed where you're hovering

around 1900-2100 under load. At 80mph it's no problem.  Also a great

attention getter when you are passing someone on a two lane road.

------------------------------

Date: Tue, 9 Jan 1996 18:14:20 PST

From: SyncJam@aol.com

To: impala@east.xsis.xerox.com

Subject: Newsletter

Message-ID: <960109211416_36839177@mail04.mail.aol.com>

I'd like to subscribe to the Impala "snail mail" newsletter.  Does anyone

have the address?  (Is it still $10?)  Thanks a lot.

Bill

------------------------------

Date: Tue, 9 Jan 1996 18:43:00 PST

From: "Rodger Cliff (216) 977-0036" <CLIFF#m#_RODGER@lims-a1.lerc.nasa.gov>

To: "impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: RE: Door panel lights

Message-ID: <01HZTLL8MMYC8WVYYV@lmsmgr.lerc.nasa.gov>

On Mon, 8 Jan 1996 14:25:26 PST, Scott Mueller <73145.1566@compuserve.com>

wrote:

>Here is another couple of mods for everybody. From '91 through '93 the

>Caprice had an optional power window/seat control flood lamp in the driver's

>side door panel. This was a small triangular shaped light in the driver's

>door panel only, mounted in the upper LH corner of the door release handle

>bezel. This light projected light on the power window/seat controls, so they

>could be more easily operated in the dark. Unfortunately this floodlight was

>deleted in all '94 and up cars, including the Impala, even though the little

>triangular hole remains in the door handle bezel.

I have this in my '91 9C1 and it isn't very bright.  I had owned the car for a

few months before I noticed this feature.  The brighter bulb you mentioned

would be a good idea.

>I should also mention that JC Whitney stocks what I think is the best door

>panel retainer removal tool on the market. They actually sell several types,

>but the best one is a tool made by Cal-Van (marked as Cal-Van #34) that has

>a U-shaped handle, and a long forked plastic wedge/ramp. Unlike the metal

>pry type tools also sold for removing these clips, the plastic one will not

>scratch the paint under the door panel, which can lead to rust in the

>future.

Thanks for the info.  I would have to go into the doors soon.

>Another factory option related to the door I am installing is SEO (Police)

>6N1 - Handle, Driver Door Unlock. This is a special door handle/lock rod

>that unlocks the front door when you pull the inside door handle. Normally

>in the Impala, if the door is locked and you pull the inside handle, the

>door will not open. With SEO 6N1, pulling the door handle simultaneously

>unlocks the driver's door and allows it to open. Note that this is available

>for the driver's door only, and I would not want it on any other door anyway

>(especially with kids). If you have the door panel off, all you have to do

>is install the 6N1 lock rod, which only costs a few dollars. I don't have

>the part number handy, but your dealer can easily look this one up.>

I also have this on my '91 9C1.  I guarantee you that you will alienate all of

your regular passengers in short order.  It's so easy to just pull the handle

and get out of the car and then wonder what's taking everybody else so long to

get out.  My girfriend and daughters have gotten used to immediatley using the

power lock button to unlock the other doors.

Rodger Cliff      rodger.m.cliff@lerc.nasa.gov  (e-mail access restored until

the next government furlough)

------------------------------

------------------------------

Date: Tue, 9 Jan 1996 19:29:20 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Cc: Scott Mueller <73145.1566@compuserve.com>,

Subject: Dual exhaust for Caprice/Impala

Message-ID: <30F33290.73AE@intersurf.com>

Hello all you motorheads with bottomless pockets!  If you have pulled

your stock dual exhaust (and not cut it to ribbons in the process) a

list member would like a complete factory dual setup for his Caprice LS.

Scott Mueller and I have come to the agreement that maybe we can help

him get one somebody pulled off for free or cheap?!?!  Email me, or Gert

Bruning (gwb@philabs.research.philips.com) or the list if you've got one

or will have one to sell or give.

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

End of IMPALA Digest 230

************************

IMPALA Digest 231

Topics covered in this issue include:

1) Re: '93 9c1/LTZ Instrument Cluster

by BGordon401@aol.com

2) Re: Jeep GC VS. Impala

by David Cooley <cooldave@cris.com>

3) 229/No snows...

by RCKOHLI@aol.com

4) Hose clamps

by Scott Mueller <73145.1566@compuserve.com>

5) LTZ cluster

by Scott Mueller <73145.1566@compuserve.com>

6) Re:digest

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

7) Corvette Fuel Injector Covers

by thaxton@kscdm6.ksc.nasa.gov

8) Brake caliper bolt size

by John Harris <harrjo@symphony.aud.alcatel.com>

9) Re: IMPALA digest 230

by Clyde Twine <clyde.twine@hok.com>

10) Exhaust questions

by foxhound@txdirect.net

11) hypertech

by foxhound@txdirect.net

12) digital tachs

by Clyde Twine <clyde.twine@hok.com>

13) Re: Stainless (SS) Clamps

by Rock528@aol.com

----------------------------------------------------------------------

Date: Tue, 9 Jan 1996 19:39:22 PST

From: BGordon401@aol.com

To: 73145.1566@compuserve.com

Cc: impala@east.xsis.xerox.com

Subject: Re: '93 9c1/LTZ Instrument Cluster

Message-ID: <960109223846_111644933@emout06.mail.aol.com>

Scott, you wrote:

>If you ever get to look at the '91-'93 9C1/LTZ instrument cluster, you will

be very upset indeed! This cluster put the Impala SS cluster to shame, in

fact it is even better than the '96 Impala gauge package! The 9C1/LTZ

cluster had a digital speedo, analog tach, and full analog gauges, with

numeric calibrations, including oil pressure, temperature, voltmeter, and

fuel.

You know, you might be on to some thing here!!  Scott, Is there a way to

adapt this instrument cluster into our car??  If other clusters can be

retro-fitted into the Impala, why not this one??

Bruce Gordon / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Tue, 9 Jan 1996 21:15:44 PST

From: David Cooley <cooldave@cris.com>

To: impala@east.xsis.xerox.com

Subject: Re: Jeep GC VS. Impala

Message-ID: <199601100415.XAA17947@franklin-fddi.cris.com>

At 07:29 PM 1/9/96 PST, you wrote:

>The 5.2L Magnum engine is rated at 220 HP stock, and I figure with these

>mods I should be somewhere around 275 HP! The V8 engine includes a killer

>3.73 axle ratio (front and rear), which is ideal for performance. The Jeep

>actually weighs less than the Impala, so imagine a high 14-second Jeep GC

>and you get the picture! It should be able to take a stock Impala (sorry),

>especially in the rain. <g> There, I mentioned the Impala, so this message

>is not entirely off subject! <BG> If anybody is interested in Jeep GC

>performance mods, send me some mail. Scott.

>

Scott,

Remember, HP sells cars, Torque wins races.  The 5.2 in the GC only has

270-275lb/ft... The impala has 330 that come to a peak about 1500 rpm before

the 5.2's.

Later,

Dave

===================================================================

David Cooley  AKA N5XMT                      Cooldave@cris.com

Powered by WINDOWS 95!           http://www.cris.com/~Cooldave/

===================================================================

------------------------------

Date: Tue, 9 Jan 1996 16:49:00 PST

From: RCKOHLI@aol.com

To: impala@east.xsis.xerox.com

Subject: 229/No snows...

Message-ID: <960109194855_111452668@emout05.mail.aol.com>

Hey Kurt;

Get a life, my man! I didn't know I had to draw pictures in my posts, but to

answer your questions, I had trouble driving "with snow tires" because I was

traveling on one lane winding and hilly roads (routes 29, 22, & 6 in NY-and

routes 116 & 35 in CT., where the roads are plowed intermitently if at all),

where the speed limits vacillate from 25 to 55 MPH with numerous traffic

lights, steep uphills, downhills and sharp curves...you know (or do you?),

when they built roads around the local terrain...following people who don't

know how to drive in heavy snow (like me, Captain Kurt?), following BoneHeads

who think they don't need snow tires in the Blizard of '96 (like you Captain

Kurt?)-especially up and down the Berkshires, and took me 4HRS to drive 150

miles...are you getting the picture yet?

This was not the Merrit Parkway, or any other highway or interstate, where

only the brain dead can't pace themselves, and stay out of trouble in snow.

Regarding the best rated tires in the world...in the total overall scheme of

things, who gives a shit?

Sorry that you take offense, none was intended!  I don't recall naming you,

nor do I even know who the hell you are...I just remembered some posts saying

that snow tires are not necessary, and something about

controlling posi traction.  You sound a tad thinned skinned Kurt...better

not cut yourself shaving, as you might bleed to death!   Nor do I recall

"attacking" you or anybody on the List, but if I were to attack you Kurt, it

might look something like this:  The stock BFG tires suck in real snow (non

highway/thruway driving), and you Kurt, can shove the "best tire in the

world" up your ass, rims and all!   Now I really don't mean this Kurt...its

just so you know what an attack will look

like, if one ever does come your way.

So Kurt, sit back, relax, and light up a Lucky.  Count your blessings and put

the minor stuff aside, because they are usually the things that trip us all

up.  I meant no harm, disrespect, etc., so lighten up. RC.

------------------------------

Date: Tue, 9 Jan 1996 20:48:58 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Hose clamps

Message-ID: <960110044857_73145.1566_FHD41-1@CompuServe.COM>

Franklin,

>I replaced my coolant hoses with the green silicone ones and could not find

stainless steel clamps anywhere!  Well, duh, any good marine shop will carry

some, BUT!, the best and cheapest, easiest place I've found is..... Home

Depot!  Most of the clamps are under $1 and they are of high quality because

they are the marine style full stainless clamps.

The problem with most worm gear hose clamps, even full stainless ones, is

that the hose is exposed to the worm gear screw drive slots. When you

tighten the clamp, the hose is extruded through these slots and is cut or

damaged. I would not want to subject my $300 silicone hoses to this abuse,

and neither does GM. That is why factory 9C1 cars include special "extended"

clamps on the radiator hoses, where the inner band wraps fully around the

inside diameter, thus sheilding the hose from any of the worm gear drive

slots. Unfortunately even GM only uses these special clamps for the larger

hoses, so I found even better all-stainless fully lined clamps in each of

the sizes required by the radiator hoses, heater hoses, and the small bypass

hoses.

I was able to pick up extremely high quality 316 stainless clamps (touted as

even acid resistant) that are fully lined, at the local Boat US/Marine

center. These use a worm gear drive with a raised gear portion on the band,

and no slots at all to cut the hose. I used these clamps on all of the

silicone hoses, not just the radiator hoses as on the 9C1 cars.

P.S. I hope somebody responds with a take-off exhaust for your friend!

Scott.

------------------------------

Date: Tue, 9 Jan 1996 20:49:09 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: LTZ cluster

Message-ID: <960110044908_73145.1566_FHD41-2@CompuServe.COM>

Bruce,

>You know, you might be on to some thing here!!  Scott, Is there a way to

adapt this instrument cluster into our car??  If other clusters can be

retro-fitted into the Impala, why not this one??

I wish, but unfortunately I was only reporting this as interesting

information, as those older clusters simply will not physically fit. I find

it fascinating that the '91-'93 Caprice 9C1/LTZ, Buick Roadmaster and

Roadmaster Estate Wagon, and even the Oldsmobile Custom Cruiser station

wagon all had killer instrument clusters with tachometers and full gauges on

most of them! Then they decide to do a car like the Impala and it gets a

wuss instrument cluster. Of course that is because the re-designed dash was

already done by the time the Impala was approved, and there was not the time

or funding for a custom cluster until '96.

In so many ways, the Impala reminds me of my '87 Buick Grand National, that

car had no real instrumentation either. Buick remedied that very nicely in

the limited GNX version with full analog Stewart Warner instrumentation, but

they only made 547 of those.

Obviously Chevy did a custom cluster for the '96 Impala, but after seeing it

I thought "why did they bother"! I was amazed how poorly the '96 Impala

cluster turned out, especially since the '91-'93 B-car gauge clusters were

much better (especially the 9C1/LTZ and the Roadmaster), plus I saw a

factory experimental prototype (big block) Impala with the killer Camaro

cluster, and it looked so good. Why would they design a new custom cluster

with no gauges except temp. and a tach, when they could have just borrowed

from the parts bin and used the existing Camaro cluster with only a few

mods? Of course it must be a cost issue.

The reason the older clusters will not work is that they were designed for

the '91-'93 dashboards which had a completely different shape than the

current one. The '93 to present Camaro cluster is almost identical in shape

to the Caprice/Impala cluster, which is why it would be so easy to install

in the Impala. Scott.

------------------------------

Date: Wed, 10 Jan 1996 05:02:29 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: Rock528@aol.com

Cc: Impala SS <impala@east.xsis.xerox.com>

Subject: Re:digest

Message-ID: <Chameleon.960110080445.hollenbe@mustang.east.xsis.xerox.com>

On Sun, 7 Jan 1996 14:39:58 PST  Rock528@aol.com wrote:

>What forces the digest to send?  I notice some have 2 posts and others have

>multiple?

>

>post the answer to the group also.  Troy

There are two situations that cause a digest to be sent to the list:

1) 500 lines of message text (or more) are received.

2) Less than 500 lines of message text have been received and 24 hours has passed

since the last digest was sent.

So, on busy days you can actually get more than one digest.  On slow days

you'll probably get only one.

==================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street, Lower Lobby, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

==================================================

------------------------------

Date: Wed, 10 Jan 1996 05:27:36 PST

From: thaxton@kscdm6.ksc.nasa.gov

To: impala@east.xsis.xerox.com

Subject: Corvette Fuel Injector Covers

Message-ID: <9601101327.AA08753@kscdm6.ksc.nasa.gov>

Just a question for those of you who have changed the triangular resonator to

the Corvette fuel injector rail covers.  Which year Corvette are they from?  My

Chevy dealer parts department does not have pictures/sketches of the different

ones.  The ones for the '94-'96 Corvette cost half as much as the earlier ones

so I assume there is a big difference.  Which ones work better/look better?

Eric

Merritt Island, FL

------------------------------

Date: Wed, 10 Jan 1996 06:43:34 PST

From: John Harris <harrjo@symphony.aud.alcatel.com>

To: impala@east.xsis.xerox.com

Subject: Brake caliper bolt size

Message-ID: <96Jan10.064448pst."14497(14)"@alpha.xerox.com>

Hi,

Does anyone know the size of the allen-head bolts for the front brake

calipers for the '95?

I've already worn them out at 15,000 mi :-)

thanks

john

--

V        John K. Harris                (jharris@aud.alcatel.com)

+-----------+  Chief Architect                    (tel) (214) 996-7088

|  ALCATEL  |  Network Mgmt. Product Development  (fax) (214) 996-7362

+-----------+  Alcatel Network Systems

M.S. 408-200 Richardson, TX  75081-2206

WWW: http://www.airmail.net/~jkharris

Organization doesn't really accomplish anything.  Plans don't

accomplish anything, either.  Theories of management don't much

matter.  Endeavors succeed or fail because of the people involved.

Only by attracting the best people will you accomplilsh great deeds.

- Admiral Hyman G. Rickover

------------------------------

Date: Wed, 10 Jan 1996 07:26:00 PST

From: Clyde Twine <clyde.twine@hok.com>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 230

Message-ID: <9601101526.AA05560@portal.hok.com>

You want the H as close to the end of the cats as

>possible.  I hate to ask this but... did you watch them cut the holes in

>the pipes before they installed the H?  From your description it wounds

>like they forgot to do that.

You won't find many chances in life where you get ~20 HP

>this cheaply or easily.

>

I would seriously like to do this mod, especially with the prospect of ~20HP :)

One question though.  Will I still be able to pass state inspection?  I dont

think so.

ct

------------------------------

Date: Wed, 10 Jan 1996 09:36:41 PST

From: foxhound@txdirect.net

To: impala@east.xsis.xerox.com

Subject: Exhaust questions

Message-ID: <2.2.32.19960110173641.00674da0@txdirect.net>

I've seen a lot of exhausts info floating around lately. Correct me if Im

wrong but the stock exhaust diameter is 2", some have suggested using 2

1/4", 2 1/2", and some have even said 3". What are the tradeoffs with HP and

torque with using the different size pipes? Why is one better/worse than the

other? (Bigger pipe, less backpressure/more HP, less torque?) Also, Im

interested in knowing more about the mod where you remove the mufflers and

replace them with a pipe, and cut an H-pipe in after the cats. For someone

on a limited budget who wants desperately to gain hp, this sounds like a

dream come true when faced with a

$700-1,200 bill for a Borla or some other exhaust.

1. How annoying is the buzz at 60mph? I had a Honda Civic with headers and

an aftermarket exhaust that would drone at 65 but that wasn't too bad. Of

course that was also only a 1.5 liter 4cyl, not the monster lurking under

the hood of the Impala.

2. Does removing the mufflers affect the emissions in any way?

3. Are there any special details I need to be aware of when having someone

cut an H-pipe into my stock exhaust? (except of course make sure they open

up holes in the other pipes)

4. For those who have done it, did you use stainless for the H-pipe or did

you/they just add a steel piece?

5. Is this mod reversible? do the 2" pieces that replace the mufflers get

welded to the stock exhaust or are they clamped?  In other words, If I do

this and don't like it, can I just remove the replacement pipes (not the

H-pipe) and put a set of aftermarket muffs or event the stock muffs back on?

6. Any advise on how much I should expect to pay to have this done?

7. Do any of you guys on the list live in or near the San Antonio Tx area?

Thanks.

Jeff Gatcha

foxhound@txdirect.net

95 Green/Grey Impala SS,  currently bone stock  : (

------------------------------

Date: Wed, 10 Jan 1996 09:48:02 PST

From: foxhound@txdirect.net

To: impala@east.xsis.xerox.com

Subject: hypertech

Message-ID: <2.2.32.19960110174802.0068f6d4@txdirect.net>

Has anyone found out how the Hypertech D1 program differs from the C1?  I

seem to recall that someone was having a custom program made, what did you

have them change? (feel free to be frighteningly specific) And, IF I did get

a hypertech programmer, would it be best to get it last after the

airbox/exhaust mods etc are done or doesn't it matter?  Do they program it

differently depending on what you have already done?  I'd hate to buy it

now, then add an exhaust and maybe the roller rockers etc later and not be

able to take full advantage of it because my changes weren't figured into

the equation when I bought the programmer.

Thanks again for any advice/help.

Jeff Gatcha

foxhound@txdirect.net

------------------------------

Date: Wed, 10 Jan 1996 13:26:36 PST

From: Clyde Twine <clyde.twine@hok.com>

To: impala@east.xsis.xerox.com

Subject: digital tachs

Message-ID: <9601102126.AA00651@portal.hok.com>

Scott, Im about to order the digital tach and temp gauge and I wanted to

confirm that these are the ones you spoke of. The JCW catalouge doesnt

mention the brand name. The "space-saving mini" tach is 2 1/16" (PN

38NP3668X) and the cost is $54.95. The temp guage is $35.95.  Does this

sound about right?

ct

------------------------------

Date: Wed, 10 Jan 1996 15:23:36 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Stainless (SS) Clamps

Message-ID: <960110182335_37760524@emout05.mail.aol.com>

In a message dated 96-01-10 00:05:12 EST, you write:

>Most of the clamps are under $1 and they are of

>high quality because they are the marine style full stainless clamps.

>They can be found in the plumbing area of your local store.  My local

>boat store wants as much as $5 per clamp.

I believe the factory SS clamps are lined with SS therefore eliminating the

little rubber tits that squeeze through the holes in the clamp strap.  I have

the PN's for the small clamps but not the radiator hose clamps....Scott?

Troy

------------------------------

End of IMPALA Digest 231

************************

IMPALA Digest 232

Topics covered in this issue include:

1) Programming the Impala Chip

by sirjames@rmii.com (James W. Thompson)

2) RE: RE: Jeep GC VS. Impala

by "Harris, Scott" <sharris@xyplex.com>

3) RE: exhaust inspection

by "Harris, Scott" <sharris@xyplex.com>

4) RE: hypertech

by "Harris, Scott" <sharris@xyplex.com>

5) RE: exhaust

by "Harris, Scott" <sharris@xyplex.com>

6) auto door lock/unlock feature

by CoolChevy@aol.com

7) Re:digital tachs

by lennie@earthlink.net (Lennie Dolphin)

8) RE: IMPALA digest 231

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

9) RE: Profanity Reminder

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

10) .

by JOKER66@aol.com

11) Stock dual exhaust of 94-96 SS

by Gert Bruning <gwb@philabs.research.philips.com>

12) Brand New 9C1's for sale!

by Steve Das <sdas@sat.net>

13) Re: IMPALA digest 231

by Gary Rotter <gr5@elsegundoca.attgis.com>

14) Cyberdyne gauges

by Scott Mueller <73145.1566@compuserve.com>

----------------------------------------------------------------------

Date: Wed, 10 Jan 1996 20:06:00 PST

From: sirjames@rmii.com (James W. Thompson)

To: impala@east.xsis.xerox.com

Subject: Programming the Impala Chip

Message-ID: <m0taEHs-000JPrC@rmii.com>

>

>

>

>

>

>

>

>

>

Scott?

Jim

sirjames@rmii.com

------------------------------

Date: Wed, 10 Jan 1996 11:14:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: RE: Jeep GC VS. Impala

Message-ID: <30F49D3D@corvette.xyplex.com>

>>The 5.2L Magnum engine is rated at 220 HP stock, and I figure with

these

>>mods I should be somewhere around 275 HP! The V8 engine includes a

killer

>>3.73 axle ratio (front and rear), which is ideal for performance. The

Jeep

>>actually weighs less than the Impala, so imagine a high 14-second Jeep

GC

>>and you get the picture! It should be able to take a stock Impala

(sorry),

>>

>Remember, HP sells cars, Torque wins races.  The 5.2 in the GC only has

>270-275lb/ft... The impala has 330 that come to a peak about 1500 rpm

before

>the 5.2's.

A real man's Jeep would have the 5.9 Magnum with 230 HP and 330 lb./ft.

Or...even better the V-10 with 300 HP and 450 lb./ft.  These numbers are

stock and are really available in Dodge pickups  Imagine what you could

do with this as a starting point!

OK, recess is over, now back to Impalas.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Wed, 10 Jan 1996 11:14:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: exhaust inspection

Message-ID: <30F49D3F@corvette.xyplex.com>

>I would seriously like to do this mod, especially with the prospect

>of ~20HP :)  One question though.  Will I still be able to pass

>state inspection?  I dont think so.

Here in Massachusetts we have now the same emissions requirements as

Califormia.  It passes.  Why wouldn't it.  The cats are still there.  The

only emissions the mufflers affect is noise and I would never call my car

noise pollution!

Scott Harris

sharris@xyplex.com

------------------------------

Date: Wed, 10 Jan 1996 11:14:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: hypertech

Message-ID: <30F49D42@corvette.xyplex.com>

I believe the only difference between HyperTech's C & D revs is the D

supports both B & F bodies in a single part.  The rev C part is B body

only.

HyperTech charges $150 for any customizing (even putting the shift points

back down to stock from their unreasonably high rpm) and it does not

matter if the change is done in the initial purchase.

There should not be any PCM changes required with intake or exhaust mods

as the MAF will "see" more airflow and adjust accordingly.  Roller

rockers should not make enough difference to require changes either.

BUT, if you change the cam to something more radical and the engine runs

with less vacuum and needs a slightly higher idle speed you'd have to

shell out for the customizing.  Also changing the rear end gearing would

require PCM changes.  So, you can add PCM changes either before, after or

during your planned mods.

Now if you get a Z instead any customizing at the initial purchase is

free and you don't have to special order shift points because Z's are

reasonably close to stock.  If you need a change in the future it's $100

compared to HyperTech's $150.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Wed, 10 Jan 1996 11:14:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: exhaust

Message-ID: <30F49D44@corvette.xyplex.com>

The stock exhaust is 2-1/4" into the mufflers.  Only the tailpipes are

2".

The trade-off of a large tube is you start losing low rpm torque.

Although small tubing creates back pressure, it also maintains gas

velocity once created.  An overly large tube requires each pulse to

create it's own way out hitting non-moving stagnant gas it is has to

accelerate on it's own.  A correctly sized tube will keep the exhaust

flowing actually helping each pulse draw the next one out.  Think of it

as a Nascar race were a train of cars all drafting each other can run

faster than a single car on it's own.  So a low restriction correctly

sized system is what you want.  3" tubing is a little large for a 350

limited to 6000 rpm.  If you had a 502 big block or were turning 7+ grand

in a screaming small block, then you'd want the 3" tubes.

Replacing the mufflers with straight (S-shaped actually) tubing follows

on the above principals.  The only drawback is increased noise.  But as

you point out, if this ~$100 mod turns out to be too loud you can always

pull the pipes and install a set of Ultra Flows, CVX's Flowmasters, etc.

If I had the money to buy a Borla, I'd seriously consider the Callaway

for it's unique tail pipe location and sound.

Q1 - The "60 MPH buzz" is actually more of a drone at 65 MPH.  You would

have to experience it to know if you can live with it.  Personally, I

just try not to drive 65.

Q2 - As previously stated, removing the mufflers and/or installing the H

pipe should not affect emissions.

Q3 - The worrysome factor when installing an H is "what happens if I need

to drop the tranny?"  I see 3 choices: install a flange in the H

(increasing the initial cost, complexity and adding exhaust leak

possibilities), remove the exhaust section from the cats to the now

removable muffler replacement pipes as a unit or just cut the H if you

ever have to remove it and either reweld it or install flanges at that

time.

Q4 - I used mild steel MIG welded because I didn't know if it would work

and couldn't fond a shop that works with stainless locally.  If I was

doing it again I'd do stainless.

Q5 - It is a "reversible" mod in that you can remove the muffler pipe and

install a muffler if it's too loud.  Installing the pipes

Q6 - $100-150 is a realistic range for removing the mufflers, custom

bending and installing the tubes and H pipe.  If they use slip fit tubes,

make sure the joints insert from the front.  You always want the male

portion facing rear to prevent the exhaust from hitting the end of a

tube.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Wed, 10 Jan 1996 22:31:21 PST

From: CoolChevy@aol.com

To: impala@east.xsis.xerox.com

Subject: auto door lock/unlock feature

Message-ID: <960111013121_112946945@emout05.mail.aol.com>

I 'm reading a lot of messages about this replacement driver side door handle

from the 9C1 police package.

I installed an UNGO (by Clarion) alarm unit in my car (who in the hell hasn't

installed a alarm unit in his Impala anyway ?) which has a nice feature built

in.

It automatically locks all doors + trunk 10 seconds after you switched on the

ignition (or started the engine) and immediately re-opens the door locks

after shutting off the engine.

This feature is defeated if you did not close all doors within the 10 seconds

or when you turned on the ignition way before you closed the doors which is

an odd situation anyway.

In addition to that, it is very motion/shock sensitive (adjustable) and arms

automatically with a loud chirp and flashes the turn-signals once. Upon

deactivation it chirps twice and flashes once, switches on the interior

lights.

If somebody hit the car once, it will warn him with an annoying chirp just

once but won't go off, if someone shakes the car (rim antheft or tow away

protection) or hits it twice it will go off immediately. About 5 seconds

after it went off it will turn on the car's horn too. I almost forgot to

mention the starter disableing function upon activation.

All this comes for around 300 bucks plus 100 bucks installation costs at a

shop.

Further, i upgraded the factory speakers (better dumped them) with 6x9 three

way Infinitys back (bolt in) and 4x6 Pioneers front. In additon i have 10"

amplified Bazooka basstube mounted in the trunk, right behind the rear set.

For a factory setup, it sounds quite neat and is rich in bass. I'm going to

upgrade this setup with additional amplifiers which means i have to toss the

factroy CD player and install a Alpine CD changer combination. Main reason

is, the factory radio can not be used in Europe due to a different decoder

system. If anyone is interested in the factory unit, it will be available in

about two months so drop me an email.

Does anyone know the part numbers and costs for the 3.42 gears ?

Heinz Schenk

------------------------------

Date: Thu, 11 Jan 1996 01:00:27 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: impala@east.xsis.xerox.com

Cc: Clyde Twine <clyde.twine@hok.com>

Subject: Re:digital tachs

Message-ID: <199601110900.BAA28832@iceland.it.earthlink.net>

Clyde you said:

>Scott, Im about to order the digital tach and temp gauge and I wanted to

confirm that these are the ones you spoke of. The JCW catalouge doesnt

mention the brand name. The "space-saving mini" tach is 2 1/16" (PN

38NP3668X) and the cost is $54.95. The temp guage is $35.95.  Does this

sound about right?

Sounds right. Scott M. suggested I order the same ones-should be in soon.

They sure look like the Cyberdyne gauges. Don't forget to order the sending

unit for the temp guage!! I'm gonna install a brass T connector so I can use

the sender

as well as the OEM factory unit-Scott M. suggestion also.

... Lennie

------------------------------

Date: Thu, 11 Jan 1996 03:21:53 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: impala@east.xsis.xerox.com, RCKOHLI@aol.com

Subject: RE: IMPALA digest 231

Message-ID: <Chameleon.960111062631.hollenbe@mustang.east.xsis.xerox.com>

On Wed, 10 Jan 96 22:29:49 EST  impala@east.xsis.xerox.com wrote:

>Date: Tue, 9 Jan 1996 16:49:00 PST

>From: RCKOHLI@aol.com

>To: impala@east.xsis.xerox.com

>Subject: 229/No snows...

>Message-ID: <960109194855_111452668@emout05.mail.aol.com>

>

>Regarding the best rated tires in the world...in the total overall scheme of

>things, who gives a shit?

>

>Sorry that you take offense, none was intended!  I don't recall naming you,

>nor do I even know who the hell you are...I just remembered some posts saying

>that snow tires are not necessary, and something about

>

> controlling posi traction.  You sound a tad thinned skinned Kurt...better

>not cut yourself shaving, as you might bleed to death!   Nor do I recall

>"attacking" you or anybody on the List, but if I were to attack you Kurt, it

>might look something like this:  The stock BFG tires suck in real snow (non

>highway/thruway driving), and you Kurt, can shove the "best tire in the

>world" up your ass, rims and all!   Now I really don't mean this Kurt...its

>just so you know what an attack will look

>like, if one ever does come your way.

I've said this before, and I'll say it again -- profanity of ANY form is NOT

allowed in messages sent to this list.  I will also NOT tolerate anyone

flaming anyone else in this forum -- if there's a gripe, take it offline.

This is not a personal attack, just a reminder.  Since you sent your message

to the list I will send another reminder to the list as well.

==================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street, Lower Lobby, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

==================================================

------------------------------

Date: Thu, 11 Jan 1996 03:27:15 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: impala@east.xsis.xerox.com

Subject: RE: Profanity Reminder

Message-ID: <Chameleon.960111063235.hollenbe@mustang.east.xsis.xerox.com>

In digest 231 one of our members used some choice words to rebut the comments

of another list member.  I won't go into names and/or actual words used, but

I want to remind everyone that profanity of ANY form is NOT to be used in

messages sent to this list, even if the words are considered more common

in today's vernacular.

I'm sorry if this seems hard-nosed, but I want this forum to be one of open

exchange, where people can express opinions without being jumped on by

someone else.  If you have a problem with someone else's opinion, feel free

to politely express your views.  However, I will not tolerate personal

attacks and flames sent using the list.  First offense gets you a reminder,

anything beyond that gets you removed from the list.  If you feel that

you have to attack someone, don't do it on the list.  Take it offline.

I will update both the new subscriber's welcome message and the monthly

how-to message to reflect this policy.

==================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street, Lower Lobby, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

==================================================

------------------------------

Date: Thu, 11 Jan 1996 05:45:29 PST

From: JOKER66@aol.com

To: impala@east.xsis.xerox.com

Subject: .

Message-ID: <960111084527_38336013@emout05.mail.aol.com>

Hello all!

I've been reading the mail for quite some time and find the info very

interesting and helpful.

I have a question about oil.

Why does everyone seem to use Mobil 1 synth as apposed to some other brand.

In my '95 SS I used Castrol synth.

Now my'66 SS and '96 SS Impalas are approaching an oil change and I am

interested if I should use Mobil 1 because it is so much better than anything

else or are just about all synth oils equal.

Your opinions and ideas are appreciated.

Dave

White convertable '66 SS/Dark Cherry '96 SS

------------------------------

Date: Thu, 11 Jan 1996 05:53:43 PST

From: Gert Bruning <gwb@philabs.research.philips.com>

To: impala@east.xsis.xerox.com

Cc: gwb@briar.philabs.research.philips.com

Subject: Stock dual exhaust of 94-96 SS

Message-ID: <199601111353.IAA12861@briar.philabs.research.philips.com>

I recently inquired about putting the stock dual exhaust of the

SS on our normal Caprice LS (with the 4.3L V8) as I am planning

a mild performance upgrade program for this engine. Thanks for

suggestionsa to go to Borla or try to get an old system from an SS

which other members removed with a Borla etc. here is my thinking:

1. From my own experience with the Buick Grand National I know that

ALL aftermarket exhaust systems are louder, often with a bad drone

just where the car's rpm range is used most often. I do NOT want to

make the exhaust system louder. Therefore it seems that going from a

stock single to a stock dual might be the ticket, and at the same time

increase flow - I have already opened up the intake a bit and it was

already the same as for the SS in stock form.

2. I inquired about the cost of a "factory" dual exhaust system since

that has a good chance of being simple enough that I can install it ...

Since GM used welds I did not think that an older SS system could be

removed "clean" so I didn't inquire about getting one from the group -

Remember, we have the 4.3L V8, not the 5.7L, so I think the SS exhaust

system should suffice for future performance upgrades requiring more

flow.

The only question that I have is about the diameter of the pipes:

Are the SS pipes 2" in diameter? Thanks,

Gert   gwb@philabs.research.philips.com

------------------------------

Date: Thu, 11 Jan 1996 05:52:56 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: Brand New 9C1's for sale!

Message-ID: <199601111353.HAA10482@raven.sat.net>

OK, first let me say I have nothing to gain by this so please, don't

anybody think that. The dealer I got my Impala from has 7 (I believe) brand

new, fully police ready 9C1's for sale.  They will sell them to the public.

They are wired for the light bar and have the computer wiring in them as

well.  I haven't seen them this is just what the salesman told me.  They are

painted black and white just like a Texas Highway Patrol car and according

to the dealer, it is legal for a private citizen to own one in these colors

although I would repaint at least part of it.  Supposedly, they are FULL 9C1

cars.  He indicated that these things require the payment of the gas guzzler

tax and they are looking into that to see if there is a way out of that.  He

asked me not to quote a price as it might be different if the car is going

out of state but the price he told me was significantly LESS than an Impala.

I know that won't surprise most of you but with the equipment the 9C1 comes

with, I suspected the price would be a little closer.  Mike is one of the

"good guys" so if you are interested call;

Mike Henderson@

Bill Wiggins Chevrolet

West Columbia, Texas

(409)-345-5101

------------------------------

Date: Thu, 11 Jan 1996 08:51:56 PST

From: Gary Rotter <gr5@elsegundoca.attgis.com>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 231

Message-ID: <9601111651.AA18733@mailhost.ElSegundoCA.ATTGIS.COM>

>Date: Tue, 9 Jan 1996 20:49:09 PST

>From: Scott Mueller <73145.1566@compuserve.com>

>To: <impala@east.xsis.xerox.com>

>Subject: LTZ cluster

>Message-ID: <960110044908_73145.1566_FHD41-2@CompuServe.COM>

[Text deleted to save space]

>obviously Chevy did a custom cluster for the '96 Impala, but after seeing it

>I thought "why did they bother"! I was amazed how poorly the '96 Impala

>cluster turned out, especially since the '91-'93 B-car gauge clusters were

>much better (especially the 9C1/LTZ and the Roadmaster), plus I saw a

>factory experimental prototype (big block) Impala with the killer Camaro

>cluster, and it looked so good. Why would they design a new custom cluster

>with no gauges except temp. and a tach, when they could have just borrowed

>from the parts bin and used the existing Camaro cluster with only a few

>mods? Of course it must be a cost issue.

Scott,

Take a look at the 1996 Impala SS brochure.  The list of standard features

says that the Impala SS comes with temp, oil pressure, and voltage gagues.

Looks like they forgot to delete a line when to copied the features list

for the 95s. OOPS!

------------------------------

Date: Thu, 11 Jan 1996 09:54:06 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Cyberdyne gauges

Message-ID: <960111175406_73145.1566_FHD28-4@CompuServe.COM>

Clyde,

>Im about to order the digital tach and temp gauge and I wanted to confirm

that these are the ones you spoke of. The JCW catalouge doesnt mention the

brand name. The "space-saving mini" tach is 2 1/16" (PN 38NP3668X) and the

cost is $54.95. The temp guage is $35.95.  Does this sound about right?

Those are the right ones! Be sure you get the sender for the temp. guage, as

it is not included with the gauge. Scott.

------------------------------

End of IMPALA Digest 232

************************

IMPALA Digest 233

Topics covered in this issue include:

1) Exhaust Mods, Valve/Injector Covers, Stainless clamps

by Franklin Poole <franklin@intersurf.com>

2) Corvette valve covers

by Scott Mueller <73145.1566@compuserve.com>

3) Jeep specs

by Scott Mueller <73145.1566@compuserve.com>

4) Hypertech info

by Scott Mueller <73145.1566@compuserve.com>

5) Re: Immature Flame

by Kurt Kemmling <76165.1654@compuserve.com>

6) SS Colors

by Mikado95@aol.com

7) Mobil 1 Usage

by cohan02@cai.com (Andrew Cohen)

8) Re: IMPALA digest 232

by Ray Berilla <writes4u@ganet.net>

9) RE: auto door lock/unlock feature

by "Harris, Scott" <sharris@xyplex.com>

10) .

by Mbmarx@aol.com

11) Mods

by Scott Mueller <73145.1566@compuserve.com>

----------------------------------------------------------------------

Date: Thu, 11 Jan 1996 09:59:29 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Exhaust Mods, Valve/Injector Covers, Stainless clamps

Message-ID: <30F55001.1C44@intersurf.com>

Clyde Twine, Jeff Gatcha:

On my exhaust system I had the mufflers replaced with 2" aluminized

pipe.  This means the shop stretched and welded at the 2.25" pipe coming

from the cats, and clamped onto the 2" pipe going to the resonators.

The H pipe is aluminized also.  Stainless is unobtainium around here and

no muffler shop locally can weld it anyway.  I did the muffler removal

with similar thoughts as Scott Harris, thinking I might add mufflers

later.  Like Mr. Harris I have decided, for the time being anyway, to

keep the straight pipes. It is fairly loud but the local gendarmes don't

care as long as I don't go blasting past them at full throttle. At part

throttle, and cruising without at heavy load it's no problem.  I'll also

tend to concur with Scott H. that it is probably not any louder than

some of the older musclecar iron running around.  As far as tech

inspection,here in Baton Rouge, LA it won't be a problem.  If your

municipality has some sort of noise ordinance then maybe you have an

issue.  As you know the standard disclaimers ALWAYS apply: "What we did

is what we did and our responsibility, and what you do is your business

and YOUR responsibility".<G>

Eric Thaxton

Don't know what year Mr. Mueller got valve covers from but part numbers

will tell you when you order them.

You need for injector covers:

1	10208794	Cover Assembly	10.00

1	10208795	"		"

4	10108672	Stud F/IN	1.35ea

1	10214035	Hose- P/B	7.00

Price increase in JAN '96 may affect retail.

Also need 1/4" to 5/8" fitting for the intake to fit the new power brake

booster hose which is smaller than stock.  Will also need to get the

1/4" size brake booster valve to go onto the brake booster.  Both parts

are readily available at any good parts house for about $4 total. You

will need to install the 'vette plastic valve covers for this to look

really good. If not, Scott Mueller says  you need the RH injector cover

from a '92 vette which has a notch in it to clear the crankcase vent

hose.

For the Corvette valve covers you need:

1	10108605	Cover vlv

1	10108606	"

1	10108694	CAP ASM-O (oil cap)

8	10108674	Stud-VLV

8	10108675	BOLT/screw

1	10128361	Hose-c/ca (not needed for '96 impala)

Prices have recently changed but retail is around $250 or so.

My '96 SS did not need to have the right hand side crankcase vent hose

changed. I was able to simply cut it to length and fit it with the

original vent tube to the new valve covers.  Also note that the valve

covers are now shipping with the gaskets and grommets pre-installed.

Stainless clamps:

As soon as I sent that post I knew I was still in a brain fog! Gotta get

those LTZ foglights real soon now<g>.  Anyway I checked on those

stainless clamps at Home Depot and they stock two kinds locally.  I'm

gonna see both tonite and see if they have the "good" kind.  I'm also

going to go to a local saltwater marine shop (I'm in the Gulf South

after all!) and see what really good clamps they may have.  If anybody

else wants them maybe I can buy a lot for a good price...

Various parts orders:

Remember folks, I work for a living and can ORDER parts for you but

cannot take the time to do parts research.  If you want to pay me to do

research I'll do it for $65 per hour with a two hour minimum.

I am planning to make arrangements for April 19th in Texas!

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Thu, 11 Jan 1996 09:53:55 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Corvette valve covers

Message-ID: <960111175354_73145.1566_FHD28-2@CompuServe.COM>

Eric,

>a question for those of you who have changed the triangular resonator to

the Corvette fuel injector rail covers.  Which year Corvette are they from?

My Chevy dealer parts department does not have pictures/sketches of the

different ones. The ones for the '94-'96 Corvette cost half as much as the

earlier ones so I assume there is a big difference. Which ones work

better/look better?

The LH side injector cover should be ordered from the 94+ Corvette

application no matter what. That year was when the LT1 when to sequential

fuel injection and the pressure regulator and fuel inlet area changed. The

earlier injector cover will not clear the later pressure regulator.

On the RH side, it depends. If you are using the stock stamped steel

valvecovers, then you should use the RH injector cover from the '92-'93

Corvette application. This is because the stock valvecovers have the

crankcase vent tube on the top of the cover, as did the Magnesium LT1

valvecovers used on the '91-'92 Corvette. The RH injector cover for those

years has a clearance notch for the crankcase vent hose to pass through.

If however, you are also changing to the killer Corvette plastic valvecovers

(used on '94+ Corvette LT1/LT4 applications), then the RH cover has the vent

tube connection hidden on the side of the cover, and not right on top. In

that case, you should use the RH injector cover from the '94+ Corvette

application, since it does not have the clearance notch for the vent hose,

and would look much better.

An additional note is that if you order the covers from the '96 LT4

application, the embossed "Corvette" lettering is filled in with red paint.

This looks good if you use the covers as is, however in my case I have the

factory S-10 "bowtie-SS" red emblems on my injector covers below the

embossed "Corvette" lettering, which I think looks better when using the LT1

injector covers which leave the Corvette lettering in black.

BTW, although they seem expensive, the Corvette valve covers are also an

excellent addition to the engine. In addition to simply looking much better,

the Corvette plastic valvecovers offer several functional and performance

benefits over the stock stamped steel covers:

-They include special oil drippers for the rocker arms, which provides much

needed oiling to the stock stamped steel rockers. The additional oiling to

the rocker ball and tip prevents galling and reduces wear, in addition to

providing increased cooling of rocker and valvespring. The cheap stamped

steel covers lack these drippers.

-The very thick plastic construction (you will be surprised at how thick and

heavy they are) is designed to greatly reduce valvetrain noise. This

minimizes any valvetrain ticking noises you might otherwise hear with the

stamped steel covers.

-They never rust or corrode like the steel ones. All you ever have to do to

maintain them is apply a little protectant (armor-all, etc.) and they look

great!

Scott.

------------------------------

Date: Thu, 11 Jan 1996 09:53:43 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Jeep specs

Message-ID: <960111175343_73145.1566_FHD28-1@CompuServe.COM>

Dave,

>Remember, HP sells cars, Torque wins races.  The 5.2 in the GC only has

270-275lb/ft... The impala has 330 that come to a peak about 1500 rpm before

the 5.2's.

For '95 the cam was re-designed and since then the 5.2L Magnum engine in the

Jeep GC has been rated at 300 lb-ft @ 3200 rpm. Of course with the mods I

mentioned, that number will be even higher! Also worth noting is that the GC

V8 includes a 3.73 axle ratio, which multiplies torque far more than the

3.08 stock Impala gear, and which also means that a 0-60 contest would

likely go to the Jeep. <g> Scott.

------------------------------

Date: Thu, 11 Jan 1996 09:54:03 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Hypertech info

Message-ID: <960111175402_73145.1566_FHD28-3@CompuServe.COM>

Jeff,

>Has anyone found out how the Hypertech D1 program differs from the C1?  I

seem to recall that someone was having a custom program made, what did you

have them change? (feel free to be frighteningly specific)

All they said is that the D1 included their latest program revisions. They

did not say exactly what had changed between the earlier C1 version. If you

or anybody else finds out more specifically what was changed, please post!

The custom program they sent me has the D1 basic program, except I had them

put the stock shift points (4800 rpm) back, instead of the higher shift

points (5300 rpm) they normally use.

>And, IF I did get a hypertech programmer, would it be best to get it last

after the airbox/exhaust mods etc are done or doesn't it matter? Do they

program it differently depending on what you have already done?

Unless you have very serious mods like ported heads, they would not do

anything different. In other words, there is no customization necessary for

airbox or exhaust mods, or even the roller rockers. Scott.

------------------------------

Date: Thu, 11 Jan 1996 10:25:28 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Immature Flame

Message-ID: <960111182527_76165.1654_GHM68-4@CompuServe.COM>

In Impala Digest 231 RCKOHLI@aol.com wrote a completely

childish, immature and unprovoked flame directed at me. I will not re-post it

here as this is not the purpose of this list.

I thought this was a moderated list, the purpose of which is to inform

other owners of Impala SS's and to share our experiences.

The posting of obscenities and direct personal attacks should not be allowed.

I am asking the members of this list to vote for RCKOHLI@aol.com to be banned

from posting and subscribing to this list.

I want to see him taken off of this list as he cannot control himself, and is

not contributing positively to this list.

Thank you,

Kurt Kemmling

------------------------------

Date: Thu, 11 Jan 1996 11:22:57 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: SS Colors

Message-ID: <960111142256_113369196@emout04.mail.aol.com>

I recently stopped into Horizon Chevrolet in Naperville, IL.   Like most

dealerships, they have a large brochure rack, with each model displayed.

This is not the rack that has the brochures the consumers can take, but the

large flip-chart type that stays with the dealer.  Three color choices for

the Impala: Black, Dark Cherry, and GRANITE.

No green, either.  I didn't ask the dealer about it, because I consider ther

staff one of the worst I've ever encountered--I go there for there terrific

service department.

Randy

------------------------------

Date: Thu, 11 Jan 1996 10:43:50 PST

From: cohan02@cai.com (Andrew Cohen)

To: impala@east.xsis.xerox.com

Subject: Mobil 1 Usage

Message-ID: <9601111843.AA19886@cai.com>

A question arose regarding Mobil 1 as opposed to some other synthetic.  It

has been my experience that Mobil 1 works so well in every application I

have used it in that I never consider any other synthetic.  That is not to

say that the others are inferior, it simply means that I've had excellent

results.  Here are my current applications:

Ford 302, Chevy 454, Chevy LT-350, 5HP snowblower, 4HP lawnmower, and as

of this Saturday (weather permitting), Chevy (Imapala) 350.

The 302 has gone 130K miles without any internal engine problem.  The 454

is still too new to tell.  The LT-350 (truck 4bolt steel crank in vette)

has 50K miles on it, but the 350 L48 I took out (which also used Mobil 1),

had 170K miles before the bearings wore, reducing oil pressure to a minimum,

although no knock.  Breakdown of that engine sold me on Mobil 1 because it

was clean as can be.  Absolutely no varnish/buildup whatsoever.  The blower

and mower use it simply because they're 4 stroke and I know they'll last

longer without a problem.  Blower is 7 years old, and mower is 4.

I think the bottom line is that I've found the cost of Mobil 1 to be easier

to tolerate than breaking down a motor.  Sooner or later, motors wear out,

so why not make it later?  Using this stuff and frequent changes seem to be

one of the best things you can do to increase longevity.  It appears to work

well so I never consider beta-testing any other brand.

------------------------------

Date: Thu, 11 Jan 1996 14:16:04 PST

From: Ray Berilla <writes4u@ganet.net>

To: Impala list <impala@east.xsis.xerox.com>

Subject: Re: IMPALA digest 232

Message-ID: <199601112216.RAA07361@s1.GANet.NET>

-- [ From: Ray Berilla * EMC.Ver #2.5.02 ] --

> ------------------------------

>

> Date: Thu, 11 Jan 1996 05:45:29 PST

> From: JOKER66@aol.com

> To: impala@east.xsis.xerox.com

> Subject: .

> Message-ID: <960111084527_38336013@emout05.mail.aol.com>

>

> Hello all!

>

> I've been reading the mail for quite some time and find the info very

> interesting and helpful.

>

> I have a question about oil.

>

> Why does everyone seem to use Mobil 1 synth as apposed to some other brand

. In

> my '95 SS I used Castrol synth.

>

> Now my'66 SS and '96 SS Impalas are approaching an oil change and I am

> interested if I should use Mobil 1 because it is so much better than

anything

> else or are just about all synth oils equal.

>

> Your opinions and ideas are appreciated.

>

> Dave

> White convertable '66 SS/Dark Cherry '96 SS

Dave,

One of the dealer perks I got when I purchased my '94 Impala was free oil

changes for as long as I owned the car. I appreciated this but suspected

they would use some cheap bulk oil to save money. I was prepared to pay an

additional cost to use Mobil 1 but to my surprise they said that it wouldn't

be necessary because they already used Mobil 1 on Impalas. According to them

the factory recommended oil for Impalas is Mobil 1. The only other car they

use it in is Corvettes, also per factory recommendation.

Ray

A couple of other pieces of information list members might be interested in

. Earlier this week when we got the big snowstorm I unfortunately discovered

the limits of my winter tires. I had just barely made it out of my driveway

into the unplowed street when I got hung up in about a half foot of snow.

After about 10 minutes of spinning, rocking manuevering and the help of

neighbors I got back on firmer ground but I noticed that there was a large

spot of pink fluid on the snow where I had been. I knew it had to be

transmission fluid so I feared the worst. After checking the fluid level

which seemed to be fine I tentatively set out figuring I'd find out soon

enough if there was any damage. However the car shifted normally and

everything seemed to be fine. After running my errands I again checked the

fluid level and it was OK. I was relieved but still set up an appointment

with the dealer to get things checked out. They couldn't get to it right

away so I drove it cautiously for a couple of days until I was able to get

it in today.

After dye testing the transmission the dealer determined that the

transmission cooler lines were leaking and had to be replaced. Nobody in

town had them in stock so they had to be special ordered. He said I could go

ahead and drive the car until the came in (about 3-5 business days) as long

as I kept an eye on the fluid level.

I'm sure the rocking and spinnig put an unusual strain on the transmission

but it might be something other list members might want to keep an eye out

for in case it's another one of those factory defects.

One more thing...Today I got a postcard in the mail from a company called

Mid-Continental Auto Sales, Inc. located out of Shawnee, KS. The following

is the text of the message:

Allow the most prominent exporting company in your area to buy your car!

Cheverolet Caprices 1985-95

We prefer high mileage cars...

Call us before you deal with anyone else.

We will beat any other legitimate offers.

We will pick up your car.

CASH TRANSACTIONS!!!

We are looking for 4-door or station wagon Caprices,

Classic, Brougham or LS that are V-8s.

Call your state agent - Sameer

1-800-204-8009

Mid -Continental Auto Sales, Inc.

11338 W. Shawnee Mission Parkway

Suite 203

Shawnee, KS 66203

Phone: (913) 962-6604 Fax (913) 962-6607

7 days a week

24 hours a day

Price subject to Auto Inspection by Agent.

Anybody else get this card? Saudi agents perhaps?

Ray

------------------------------

Date: Thu, 11 Jan 1996 05:22:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: auto door lock/unlock feature

Message-ID: <30F59C4D@corvette.xyplex.com>

>It automatically locks all doors + trunk 10 seconds after you switched

on the

>ignition (or started the engine) and immediately re-opens the door locks

>after shutting off the engine.

Be VERY careful about anything that automatically locks doors.  I have

seen more than a few systems lock the rightful owner out of their own

car, some even while it was running.  One guy was running the Cannonball

from Boston to San Diego in a 928.  He stopped to get gas in the middle

of the night somewhere in Oklahoma.  He left the car running while he

filled up and found himself locked out.  He ended up cutting the 1/4

window rubber out to unlock the door then he duct taped the window back

in to complete the race.

>About 5 seconds after it went off it will turn on the car's horn too.

Thieves use this "feature" to test the alarm, check to see if anyone will

come if it goes off and to kill the battery so they can steal it later

quietly.

> I almost forgot to mention the starter disableing function upon

activation.

Here's another one where the owner gets into more problems than the

thief.  I'm not trying to be pessimistic, just trying to open people's

eyes before they get stranded.  There are many more owners stopped by

poorly constructed or installed alarms than thieves.

Scott Harris

------------------------------

Date: Thu, 11 Jan 1996 15:29:50 PST

From: Mbmarx@aol.com

To: impala@east.xsis.xerox.com

Subject: .

Message-ID: <960111182950_113618114@mail02.mail.aol.com>

In a message dated 96-01-11 13:16:42 EST, you write:

>I believe the only difference between HyperTech's C & D revs is the D

>supports both B & F bodies in a single part.  The rev C part is B body

>only.

Scott M.- you were working on having some of the Hypertech's

programming changed, what happened? I would seriously consider any

software change and possibly help with the cost. I'm sure others on the

list would be interested also.

Mike Marx

------------------------------

Date: Thu, 11 Jan 1996 16:19:31 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Mods

Message-ID: <960112001931_73145.1566_FHD11-2@CompuServe.COM>

Rush,

>I sat around last night looking for the hole or at least some type of

"dummy" coverup plate for the hole in the bezel and couldn't find it. I

wonder if they restyled this starting '95.

No, the hole is exceedingly small, and is not easy to spot. Look on the

upper left corner of the black plastic recessed area where the chrome door

opening handle is. You will notice a very small triangular shaped hole right

in the inside corner area. If you did not know this was supposed to be

there, you might mistake it for a casting problem with the plastic, or even

assume that the bezel was cracked or damaged! In the '91-'93 cars, there is

a light behind this hole that illuminates the door panel controls.

>After almost going blind looking through the microfiche, it was hard to

tell which parts were designated as 6N1 from the diagram, and it looked like

there were many parts required to do this mod.  (Didn't check to see if part

numbers were duplicated for different options though, as they frequently do

in the book.)

No, I think it is one rod only. I will know in a day or so when I install

mine.

>Also, I noticed in the parts catalog (I had Nov '95 edition), that the

floor console listed for 94-95 was different from the one in my '95.  Mine

has the rounded front edge at the front, exactly the same as '96, instead of

the rectangular one they listed for the '95.

NEVER take the parts catalog illustrations literally! They are often not in

any way an accurate representation of how the part actually looks. The

overall shape of the console is the same for '94-'96, despite the misleading

illustrations.

>Also tried to figure out what else would be required to convert to a '96

console.  Do you know if the shifter connects directly to the transmission

through the floorboard? If not, how feasible would it be to use the existing

column linkage for the floor shifter?

The connection to the transmission is a cable with the floor shifter, or a

linkage with the column shifter. If you were to change, you would have to

disconnect and remove the column linkage, and add the cable and necessary

brackets for the floor shifter.

>Finally, while looking at the diagram for the transmission oil cooler, they

mention that there are different inlet/outlet hoses depending on whether you

have a standard or heavy duty cooling system.  Any idea on which one we

have? (it didn't specify V08, just heavy duty, or something along those

lines)

Both the V03 and V08 cooling packages use the additional front mounted trans

cooler, while the standard V02 cooling package uses only the in-tank cooler.

That is where the lines differ. Scott.

------------------------------

End of IMPALA Digest 233

************************

IMPALA Digest 234

Topics covered in this issue include:

1) Used Caprice Bargains

by Scott Mueller <73145.1566@compuserve.com>

2) Re:LTZ Gauges

by BGordon401@aol.com

3) Steering wheel off center

by sirjames@rmii.com (James W. Thompson)

4) Jeeps

by mbailey@rinet.hunton.com (Bailey, Martin)

5) Re: Immature Flame

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

6) Impala Stock Exhaust Pipes

by thaxton@kscdm6.ksc.nasa.gov

7) Impala Production

by zarkman@ix.netcom.com

8) 1.6 Rocker mods complete yet????

by Franklin Poole <franklin@intersurf.com>

9) Roller rockers

by Scott Mueller <73145.1566@compuserve.com>

----------------------------------------------------------------------

Date: Thu, 11 Jan 1996 18:54:22 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Used Caprice Bargains

Message-ID: <960112025421_73145.1566_FHD67-6@CompuServe.COM>

If you are interested in the 4th Generation Caprice/Impala, but either

can't afford a new one, or if you are interested in a low cost Impala

clone, this information is for you! There are many bargains available on

the used Caprice market, especially in the 9C1 (Police package) Caprices

and the very rare Caprice LTZ (direct predecessor of the Impala SS). These

are available for much less than a new Impala, and yet offer a great deal

of the Impala performance package and image for a fraction of the cost.

First a breakdown on the recent Caprice body styles. Since its debut as a

high end Impala in '65, the Caprice has been available in 4 distinctly

different body styles which have ridden on 3 different chassis platforms.

The 3rd and 4th generation vehicles have the same chassis and look

virtually identical from underneath!

Generation  Model Years  Wheelbase

----------------------------------

1st       '65 - '70     119.0"

2nd       '71 - '76     121.5"

3rd       '77 - '90     115.9"

4th       '91 - '96     115.9"

The 4th generation Caprice debuted in '91 and was originally available in

three different trim levels (Sedan (base), Caprice Classic, and Caprice

LTZ). All Caprice models (except 9C1 Police) came with a 5.0L Throttle

Body Injection (TBI) engine (LO3) that was rated at 170 HP.

The Caprice 9C1 (Police package) was not generally available to the

public, but could be ordered by a savvy dealer. The 9C1 package included a

high output (205 HP) 5.7L TBI engine (LO5) with special calibration, HD

trans, thicker (stronger) frame, HD suspension, big HD 12" front disc and

HD 11" rear drum brakes, and a specific gauge package with an analog

tachometer, a very special GU5 3.42 rear axle gear for maximum

acceleration, etc.

The somewhat rare LTZ was a "civillian" version of the 9C1 police package,

and it included many features of the 9C1 package such as the heavier duty

frame, brakes, suspension, and special instrument cluster but

unfortunately NOT the LO5 5.7L engine or the GU5 3.42 rear axle. All

civillian Caprices including the LTZ had to suffer with the wimpy LO3 170

HP version of the 5.0L TBI engine. Even without the bigger engine, the LTZ

version of the Caprice was named the Motor Trend "Car of the Year" for

'91. Civillian Caprice models (including the LTZ) could get a decent GU3

3.08 rear axle ratio if they ordered the V92 trailer towing option. This

would be a very desirable option on the '93 LTZ.

The '92 model year was essentially a repeat of '91, and there were no

changes in models or engines.

The '93 model year brought some changes. The most noticible was the

formerly skirted rear quarter panel was opened up on all Caprice models

(except wagons). Secondly the LTZ model finally got the LO5 5.7L TBI

engine, but only in 180 HP form. The LTZ also did NOT get the GU5 3.42

ratio axle as with the 9C1 package, but only a GU3 3.08 instead (and that

is if you also got the optional V92 trailer towing package). The standard

axle ratio was a 2.93The 9C1 (police) package continued with the same

setup as the previous year.

There were big changes for '94. The model lineup was shuffled. The base

Caprice Sedan model was eliminated, and the Caprice Classic was now

considered the base model. The new upscale model was called the Caprice

Classic LS. Replacing the LTZ was the Impala SS. The Impala name had not

appeared since '85, and the SS designation had not appeared on an Impala

since 1969.

The biggest changes for '94 were in engines. Gone were the LO3 and LO5

5.0L and 5.7L TBI engines, and in their place were totally new Sequential

Port Injected (SFI) 4.3L (L99) and 5.7L (LT1) engines. The LT1 engine was

virtually the same as used on the Corvette and F-cars, except it had cast

iron heads and a milder camshaft. The LT1 was rated at 260 HP and was

available in any version of the Caprice from the Classic to the 9C1 police

car. The "mini-LT1" or L99 engine was standard equipment in all Caprice

vehicles except the Impala SS (and Wagon). It was virtually identical to

the LT1 except for displacement and of course power.

The LT1 engine also got you a GW9 2.93 rear axle in the standard Caprice

models, and a much better GU3 3.08 ratio in the 9C1 and Impala SS models.

An entirely new dashboard came out in '94 for all Caprice models. It

featured new instrumentation with analog gauges but a digital speedometer.

Also present for the first time were dual air bags. Unfortunately there

was no tachometer available in any version, and other than a certified

speedometer, the 9C1 police package instrumentation was now the same as

the standard Caprice.

The '94 9C1 police package now included 4-wheel disc brakes and a special

GU3 3.08 rear axle. As the LTZ before it, the Impala SS included most of

the features of the 9C1 Police package, including the thicker frame, HD

suspension, a new 4-wheel disc brake system, and a special GU3 3.08 rear

axle. Also standard on the Impala were massive 17x8.5" wheels and specific

body ornamentation including a unique rear "C" pillar angle treatment. The

Impala also included bucket seats with a floor console, although the

shifter for the automatic remained on the steering column. Since the 9C1

instrumentation was now essentially the same as the standard Caprice,

there was no tachometer available for the Impala SS as there was for its

predecessor the LTZ.

For '95 all Caprices received the unique rear pillar first used on the '94

Impala SS, although in the '94 Impala the unique angle was created with a

plastic filler piece, while now it was formed into the sheet metal. '95

also saw the appearance of a new larger breakaway outside rearview mirror

on all Caprice models. These new mirrors were mounted on the door pillar

area instead of on the door skin as the previous mirrors.

For '96 the Impala now gets a floor shifter in the console, and a new

instrument cluster with a factory tachometer. All '96 models are fully

OBDII compliant, while the '94-'95 cars had virtually all of the OBDII

systems as well except the crankshaft position sensor (to detect misfires)

and the extra O2 sensors (to monitor catalyst function).

Here is a summary of engine choices and performance for the current 4th

generation Caprice:

Model

Years    Engine    Max HP      Max Torque  Application

--------------------------------------------------------------

'91-'92  LO3 5.0L  170 @ 4000  255 @ 2400  Sedan, Classic, LTZ

'91-'92  LO5 5.7L  205 @ 4400  300 @ 2800  9C1 (Police) only

'93  LO3 5.0L  170 @ 4000  255 @ 2400  Sedan, Classic

'93  LO5 5.7L  180 @ 4200  300 @ 2800  LTZ only

'93  LO5 5.7L  205 @ 4400  300 @ 2800  9C1 (Police) only

'94-'96  L99 4.3L  200 @ 5000* 240 @ 2400  Classic, LS

'94-'96  LT1 5.7L  260 @ 5000* 330 @ 2400  Classic, LS, Impala SS

Note: some factory documentation lists the maxiumum HP figures at 4800 rpm

instead of 5000 rpm. I do not know which is more correct, or if this was

dependent on model year or other features.

Rear Axle Ratios:

LT1 civillian Caprices are only available with the GW9 2.93 rear axle

ratio. The Impala and Caprice 9C1 are standard equipped with a better GU3

3.08 rear axle. The '91-'93 9C1 package cars had an available GU5 3.42

rear axle which substantially improved acceleration over the GU3 3.08

ratio available with the trailer towing package in the '93 and earlier

civillian cars. The GU3 3.08 ratio was an option on civillian cars and was

a part of the trailer towing option (V92). Be sure that if you get a

civillian '93 or earlier model that it has the V92 option, including the

GU3 3.08 axle.

Performance bargains on the used Caprice market:

Here would be my list of choices for used Caprices in descending order of

desiriability:

1.) Any '94+ Caprice with LT1 engine.

2.) '93 LTZ with LO5 engine and GU3 3.08 axle.

3.) '91-'93 Caprice 9C1 police car with LO5 engine and GU5 3.42 axle.

3.) '91-'92 LTZ with the LO3 engine and GU3 3.08 axle.

The best performance bargain would be any used '94-'96 Caprice with the

LT1 engine. The civillian cars are required to also have the B4U sport

suspension, which is close to the 9C1 or Impala level suspension.

Civillian cars also get a GW9 2.93 rear axle, which is not great, but

still better than most. The 9C1 versions are even better, they get the

same GU3 3.08 rear axle with rear disc brakes as the Impala SS, and an

even better 7B3 Special Performance suspension. They also have the thicker

(stronger) frame as used on the Impala, and include many other features

not found on the Impala such as a better engine oil cooler, power steering

fluid cooler, special brake cooling air deflectors, special electrical

wiring, and many other minor features not found on any of the civillian

models.

Next best and the real sleeper bargain of the bunch would be any of the

older LO5 cars, of which the most desirable would be the '93 LTZ. This

would also qualify as fairly rare, since the LTZ models themselves are

even more rare than the Impala SS, and you could only get the 5.7L LO5

engine in the LTZ in '93. The LTZ was the predecessor of the Impala SS and

has a killer dashboard with real gauges and a factory tachometer. In fact

the dashboard in the LTZ or '93 or earlier 9C1 police models is actually

better than the '94-'95 and even the '96 Impala SS!

After that I would recommend a 9C1 (police) car as an excellent choice.

The '91-'93 9C1 cars were available with a special 205 HP version of the

LO5 engine along with a killer GU5 3.42 rear axle. These would qualify as

the fastest Caprices next to the '94 and up LT1 models. Used 9C1 cars are

available from several sources. Besides going to local municipal auctions,

there are several brokers who deal in used Police cars. They are Chicago

Motors (312-235-6500) and Veto enterprises (800-523-4733). They have

hundreds of cars available, and only handle clean, well maintained

examples.

I would stay away from any of the '93 or earlier Caprice Sedan or Classic

models since they really don't have anything special to offer and would

only have a wimpy 170 HP engine with a mushy base suspension. Remeber

these cars weigh over 4,000 lbs, and 170 HP is a little short. You would

also have to spend a lot of money on stiffer springs, HD shocks, bigger

swaybars, a firm feel high ratio steering box, HD trans, higher numerical

rear axle ratio, etc. to match the 9C1 or LTZ cars. Scott.

------------------------------

Date: Thu, 11 Jan 1996 18:59:43 PST

From: BGordon401@aol.com

To: 73145.1566@compuserve.com

Cc: impala@east.xsis.xerox.com

Subject: Re:LTZ Gauges

Message-ID: <960111215556_113842309@emout05.mail.aol.com>

Scott, you wrote:

>I wish, but unfortunately I was only reporting this as interesting

information, as those older clusters simply will not physically fit.

Scott, thank you for your input, but it is unfortunate that we cannot

retro-fit the LTZ guages into our car.  From what I can remember,  that

cluster would be great for the Impala SS.  Unfortunately, I will probably

have to resort to a digital tach., when I really would like to have a analog.

There's something about seeing a needle moving i/o a bunch of numbers

jumbled together as you're "travelling" past a Ford on the highway.

Thanks again, man, for the information.

Bruce Gordon / BGORDON401@aol.com

1995Cherry Metallic

------------------------------

Date: Thu, 11 Jan 1996 19:18:00 PST

From: sirjames@rmii.com (James W. Thompson)

To: impala@east.xsis.xerox.com

Subject: Steering wheel off center

Message-ID: <m0taa0r-000JRrC@rmii.com>

Does anyone else notice that the steering column/wheel seems to be off to

the left?  If I sit centered in my seat, the center of the steering column

does not line up with me.  ie.: the buttons down the middle of my shirt do

not line up with the center line of the steering column....  Comments?

Black '96 SS

Jim

sirjames@rmii.com

------------------------------

Date: Fri, 12 Jan 1996 04:14:36 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (Impala1)

Subject: Jeeps

Message-ID: <1996Jan12.070500.1049.70213@rinet.hunton.com>

Scott Mueller wrote:

For '95 the cam was re-designed and since then the 5.2L Magnum engine in the

Jeep GC has been rated at 300 lb-ft @ 3200 rpm. Of course with the mods I

mentioned, that number will be even higher! Also worth noting is that the GC

V8 includes a 3.73 axle ratio, which multiplies torque far more than the

3.08 stock Impala gear, and which also means that a 0-60 contest would

likely go to the Jeep. <g> Scott.

Now wait a minute.  Even though the Jeep has a higher axle ratio, does that

mean that the torque necessarily is multiplied more?  The Jeep has much

heavier wheels and *much* heavier tires.   Also the circumference of the

tires, depending on the option, is much larger than the impala, which

counteracts the effect of the 3.73 axle ration.  Because all of this weight

is rolling weight, moreover, the rotational inertia of the Jeep is much

higher than the Impala (with its 17" wheels and low-profile tires).  The

ultimate multiplier effect may not be much different.

Also, even if the Jeep could hang with a stock Impala through 1-60, it would

fade away soon after.  The Jeep isn't even in the same ball-park

aerodynamically and once the vehicle reaches 40-50 mph, that becomes a very

important factor in acceleration.  Isn't that why the Impla can do 140-145

mph with 260 hp while the Jeep, even with 275 will be lucky to break 110 or

115?

------------------------------

Date: Fri, 12 Jan 1996 04:35:36 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: Kurt Kemmling <76165.1654@compuserve.com>

Cc: Impala SS <impala@east.xsis.xerox.com>

Subject: Re: Immature Flame

Message-ID: <Chameleon.960112074616.hollenbe@mustang.east.xsis.xerox.com>

On Thu, 11 Jan 96 19:24:04 EST  impala@east.xsis.xerox.com wrote:

>Date: Thu, 11 Jan 1996 10:25:28 PST

>From: Kurt Kemmling <76165.1654@compuserve.com>

>To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

>Subject: Re: Immature Flame

>

>In Impala Digest 231 RCKOHLI@aol.com wrote a completely

>childish, immature and unprovoked flame directed at me. I will not re-post it

>here as this is not the purpose of this list.

Thanks -- I didn't mean to repost it myself, missed that "cc" to the list.

>I thought this was a moderated list, the purpose of which is to inform

>other owners of Impala SS's and to share our experiences.

No, the list is NOT moderated.  That means that some nasty grams can get

through.  When I see them, I respond.

>The posting of obscenities and direct personal attacks should not be allowed.

>I am asking the members of this list to vote for RCKOHLI@aol.com to be banned

>from posting and subscribing to this list.

>I want to see him taken off of this list as he cannot control himself, and is

>not contributing positively to this list.

Personal attacks are not allowed here, but I don't leave it up to the list

membership to decide who goes and who stays, and everyone gets a second

chance.  However, RC has asked that I remove him from *my* list (maybe a jab

at me (emphasis is his) since we all know that we can always remove ourselves,

right?), so I will.

I don't need this garbage, folks.  If you make a honest mistake and mildy

flame or otherwise offend someone, I'll just remind you of our policy.

I'm no dictator, but I do have my own personal and corporate standards

to uphold, and they are not negotiable.

Let's end this discussion here.  Please direct anything further to me ofline.

==================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street, Lower Lobby, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

==================================================

------------------------------

Date: Fri, 12 Jan 1996 07:24:06 PST

From: thaxton@kscdm6.ksc.nasa.gov

To: impala@east.xsis.xerox.com

Subject: Impala Stock Exhaust Pipes

Message-ID: <9601121524.AA00323@kscdm6.ksc.nasa.gov>

I was under my '96 Impala last night looking at the exhaust pipes for the best

place to put the crossover pipe.  I noticed that the driver's side pipe is

flattened into a "D" shape (flat side on top) as it leaves the cat. converter.

I don't see any reason for the flattened pipe.  There is plenty of clearance

above the pipe.  Is this a screw-up on my car or is everyone's like this?

Eric

Merritt Island, FL

------------------------------

Date: Fri, 12 Jan 1996 07:51:56 PST

From: zarkman@ix.netcom.com

To: impala@east.xsis.xerox.com

Subject: Impala Production

Message-ID: <199601121551.HAA16215@ix6.ix.netcom.com>

Hello All!  This is my first post to the group, although I've been

subscribing now for about a month.  Sorry for not introducing myself

earlier.  Anyhow, the reason I'm writing is yesterday I was at the North

American Auto Show here in Detroit, and had a quick but interesting

conversation with an individual from Arlington.  As I was walking through

the Chevy section, I noticed a small crowd lingering around one specific

vehicle.  Guess which one had more people lined up to sit in it than any of

the others; a Black 96 Impala.  After swimming through the crowd, I noticed

I was standing next to a Chevy representative.  In passing I asked when the

official cut off point for production was.  He told me that they are making

cars at the Arlington plant all the way to December of this year.  He also

commented on sad he was to see the Impala go. :(

Oh while I'm here, yesterday after walking back out to my car, there was

about 2-3 inches of snow accumulated on it.  On the way home I swear I felt

some dripping onto my left foot.  Cowl the front cowl problem mentioned in

the Thechnical Updates be prevolent on the driver's side as well?  It's kind

of scary to think I have a leak dripping through all the electronics.

Anyhow, thanks for the time.  Later,

Greg

'94 Impala (Stock) - Black of Course!!!

------------------------------

Date: Fri, 12 Jan 1996 08:38:29 PST

From: Franklin Poole <franklin@intersurf.com>

To: Scott Mueller <73145.1566@compuserve.com>

Cc: Impala List Server <impala@east.xsis.xerox.com>

Subject: 1.6 Rocker mods complete yet????

Message-ID: <30F68E85.6762@intersurf.com>

Scott:

So. Has the GM Wizard of the North completed the 1.6 roller rocker mods?

I haven't taken the time to call Summit or anybody yet but would like to

do this maybe in February. I'm letting you be the the guinea pig on this

one.

Listmembers:

Finally got a response back from Helm Publications through my local

dealer.  The facts are:  You should be able to talk your dealer out of a

$10 discount on the service manual but probably not any more.  I can't

get enough of a discount to make it worthwhile buying/shipping them.

So, call your local dealer, see what he will charge you, and compare

that (with tax!) to what Helm will charge you with shipping and make

your decision.

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Fri, 12 Jan 1996 09:30:32 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Roller rockers

Message-ID: <960112173031_73145.1566_FHD48-2@CompuServe.COM>

Bruce,

>Do you remember the posting I had two, or so, weeks ago concerning the use

of 1.6 roller rockers?? My mechanic has not had the time to do research

concerning the rub of the pushrod on the guide hole due to the added angle

of the 1.6 rocker. Have you come to any conclusions about this potential

problem??  The reason I ask, is b/c I have SEVERE reservations on any mod

that will, even remotely,have an adverse effect on the motor's normal wear

level. Even the thought of the pushrods hitting the guide holes b/c of this

mod is enough to prevent me from going ahead with it.

I would most certainly not do anything to my engine that degraded

reliability or presented abnormal wear problems. If you know how these heads

are designed, then you know that they have a pushrod guide hole that is

actually designed to rub on the pushrod slightly even with the stock

rockers.

In pre-'87 smallblocks, the rockers are held in position on the valve stem

tip by the pushrod, and the pushrod is actually held in place by guide holes

in the heads. As the rocker tries to slip off of the valve stem tip, the

pushrod is pushed into the side of the guide hole, and contact between the

pushrod and the iron head actually keeps the pushrod and rocker in place. In

essence, the pushrod "knocks around" in the guide hole, rubbing on the sides

to keep it and the rocker it is pushing on, centered over the valve stem

tip.

In other words, the pushrods are designed to rub on the guide holes and in

fact are constructed out of specially hardened steel so as not to wear

against the soft cast iron heads. In aluminum head engines, special hardened

steel guideplates also have to be bolted in since the soft aluminum head

material can not be used as a guide for the pushrod. So whether you had an

iron or aluminum head engine, there hardened pushrods would rub against

either the iron head guide holes, or against hardened steel guide plates

bolted in the aluminum heads, thus keeping the rocker centered over the

valve.

In '87 when GM went to roller lifters in the smallblock, they also

incorporated a new type of stamped steel rocker that is known as "self

guiding", meaning it centers itself over the valve tip by a notch stamped

into the end of the rocker. This prevents the rocker from sliding off of the

valve tip. Even with this design, the iron head engines still have guide

holes for the pushrods, which serve to hold them in place during assembly

and provide additional guidance during engine operation or in case of valve

float, where the rocker may actually disengage from the valve tip.

Since these heads still provide a guide hole function, there is still some

rubbing of the pushrod in the hole, but this is again not a problem as the

pushrods are constructed out of specially hardened steel and do not wear.

With the '87+ aluminum heads, they use non-hardened looser tolerance

guideplates that also still rub on the pushrods, but as with the iron heads,

the actual rubbing contact would be much less than in the pre-'87 design

since the self guiding rockers provide most of the pushrod positioning and

guidance.

In '96 with the debut of the LT4, both it and the aluminum head LT1 were

slightly re-designed in this area primarily to save money. The pushrod

guideplates have now been completely eliminated, and without guideplates to

rub on at all, a change was made to cheaper soft steel (non-hardened)

pushrods. To hold the pushrod during assembly, the head gasket was

re-designed so as to extend into the valley area and have pushrod guide

holes in the steel gasket stamping. This design is cheaper since it

eliminates the guideplates completely, plus cheaper non-hardened pushrods

can be used in all aluminum head engines.

This brings up a note, if you check the parts catalogs, you will see that a

new pushrod is specified for the LT4 and all '96 aluminum head LT1

applications. ALL previous roller cam smallblock engines from '87-'95

including the both iron and aluminum head LT1s have used the same hardened

steel pushrod. Being curious as to why a new pushrod was specififed, I

ordered one of the new '96 LT4/LT1 (aluminum head only) pushrods, as well as

a new pushrod as found in all previous roller cam engines, and found that

the '96 version is soft steel! You can determine whether a pushrod is soft

or hardened steel by scraping it with the edge of a sharp stainless steel

knife. If the knife catches and digs in to the metal, then it is soft steel,

while if the knive edge glides along the pushrod and cannot dig in, it is

hardened steel. The moral is DO NOT use soft steel (non-hardened) pushrods

in any GM smallblock, except the '96 aluminum head LT1/LT4 engines. You can

use the '95 and earlier hardened steel pushrods in place of the soft steel

'96s, but not the other way around. ANY rubbing on guide plates or guide

holes with soft steel pushrods will wear them rapidly.

Note that the iron head LT1s still have the pushrod guide holes, even in the

'96, and as such do not use any guideplates, but do indeed REQUIRE hardened

pushrods. In doing mockups between the stock 1.5 rockers and the 1.6 roller

rockers, I could not determine that the pushrod would rub any more

appreciably against the guide holes than it already does. If you take your

stock rockers off, and check the pushrods, you will notice this rubbing has

left a ring mark around most of the pushrods.

BTW, the more pressing problem with the roller rocker installation is

selecting the correct replacement valvesprings, and getting these installed

properly. I believe I have finally found the best solution in this area, and

will be testing it soon. Mike Sunstrom has been extremely helpful with this

testing, and in fact has just completed a 2,000 mile trip with the roller

rockers in place. We both can report noticible power increases with this

mod!

If somebody is anxious to get on with this modification, I can post more

details on the best choices for springs, retainers, and locks. Some of the

issues I had to research in this area are the physical size of the

valvespring, the pressure the spring will develop at valve closed (seat

pressure) and valve open (maximum lift), the clearance between maximum lift

and coil bind, the type of steel the springs are constructed out of, whether

they can be run without a damper, checking and setting the spring installed

height and the effect of installed height on open and closed pressure,

clearance between the keys (locks) and the guided rockers, etc. I don't want

to scare anybody away, but I can tell you that I have learned more about

valvetrain engineering in that last few weeks than I thought possible! <g>

Scott.

------------------------------

End of IMPALA Digest 234

************************

IMPALA Digest 235

Topics covered in this issue include:

1) PCM programming

by Scott Mueller <73145.1566@compuserve.com>

2) Steering Wheel is off to the left

by Franklin Poole <franklin@intersurf.com>

3) Home plate removal

by Jeff Gatcha <foxhound@txdirect.net>

4) Re:Posting Wrong 3 1/2" hose part # for Intake Mod

by BGordon401@aol.com

5) Jeep - ers!

by Scott Mueller <73145.1566@compuserve.com>

6) valvesprings

by Scott Mueller <73145.1566@compuserve.com>

7) Camaro cluster

by Scott Mueller <73145.1566@compuserve.com>

8) Caprice/Impala info

by Scott Mueller <73145.1566@compuserve.com>

----------------------------------------------------------------------

Date: Fri, 12 Jan 1996 09:30:08 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: PCM programming

Message-ID: <960112173008_73145.1566_FHD48-1@CompuServe.COM>

Mike,

>Scott M.- you were working on having some of the Hypertech's programming

changed, what happened? I would seriously consider any software change and

possibly help with the cost. I'm sure others on the list would be interested

also.

I have a custom chip from Hypertech with the latest D1 revision code, plus

the transmission shift points returned to their stock settings (4800 rpm)

from the normal Hypertech raised settings (5300 rpm). Unfortunately unlike

their normal Power Programmer software, anytime you have them make a custom

program for you, it is tied directly to your specific VIN number and cannot

be used on other vehicles. I am working with Superchips to develop

alternatives to the Hypertech, plus there are the various companies offering

Z-industries reprogrammed PCMs also. Scott.

Randy,

>Three color choices for the Impala: Black, Dark Cherry, and GRANITE.

Originally, Granite Metallic was suppose to replace Dark Green Grey Metallic

for the '96 Model year. Unfortunately Chevy had problems matching the

Granite color in the plastic moulded pieces (in the front and rear bumpers),

so they discontinued it before it was even available, and restored the Green

Grey in it's place. No Granite Impalas were ever produced. Scott.

------------------------------

Date: Fri, 12 Jan 1996 13:18:20 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Cc: sirjames@rmii.com

Subject: Steering Wheel is off to the left

Message-ID: <30F6D01C.857@intersurf.com>

Yo list and Jim Thompson!

I noticed myself some time ago, (and noticed that others had

commented) on the off-center steering wheel on the Impala.

Possibly it's a "feature" from GM!<g>  On my '89 Trans Am I

noticed the same thing once when the thing came back from a

theft recovery rebuild...  On the TA all I had to do was 'get

small', pull off a trim piece and take a 9/16" or something to

the steering column and move that sucker where I wanted it.

On the Impala it's a different story!  I moved that monster

driver's seat back into the next parish, took off the trim panel

and.... there's another steel plate under there!  Didn't look

like I should be messing with it without the Service Manual that

Scott M's been chiding me to get (it's ordered)<g> so I didn't.

Looked under that plate with a flashlight, some contortions and

a mirror and it doesn't even look like the mounting bracket is

slotted like on the TA.  I also noted that there are enough SIR

(airbag) warnings under there that I decided I'd chill until I

remembered to ask the list.

So, I'm asking!<g> Can we adjust that there steering wheel thing

to the right a bit (without risking an ear splitting explosion

from the SIR), or are we stuck leaning 4.5234 degrees to the

left while holding the (air) bag?!?<GG>

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Fri, 12 Jan 1996 18:01:56 PST

From: Jeff Gatcha <foxhound@txdirect.net>

To: "'Impala'" <impala@east.xsis.xerox.com>

Subject: Home plate removal

Message-ID: <01BAE129.8B9645E0@dynm01-056.txdirect.net>

Well, Ive finally gotten around to removing home Plate.  However I'm =

finding it nearly impossible to find a 3" knockout made of aluminum (or =

any metal for that matter).  I DID find one made of plastic  (thin PVC =

type plastic by the looks of it)=20

that is about 1/4 of an inch deep (maybe a hair more)  The hose clamp =

seems to hold it in place pretty well so I installed it.  Has anyone who =

has done this mod used plastic or has everyone used some type of metal?  =

I am a bit concerned that the heat from the engine might heat up the =

plastic cap enough to distort/warp it thereby letting dirt sneak in.  =

I've been to 6 hardware stores and 3 of the huge home center stores =

(home depot etc).  No one has a metal one.=20

Someone mentioned that the part number for the 95 1LE intake elbow wasnt =

valid anymore, I would like to just get that and install it. Has anyone =

found out any more info regarding that?=20

All opinions welcome.=20

Jeff Gatcha  	95 Green-Grey

------------------------------

Date: Fri, 12 Jan 1996 17:56:33 PST

From: BGordon401@aol.com

To: foxhound@txdirect.net

Cc: impala@east.xsis.xerox.com

Subject: Re:Posting Wrong 3 1/2" hose part # for Intake Mod

Message-ID: <960112205631_114867224@emout06.mail.aol.com>

Jeff, you wrote:

>Well you wrote this in digest 225 regarding the hose needed to hook up to

the MAF sensor etc:

>>F.Y.I. The tube in question is made by GATES, their part #24250 "Green

>>Stripe" brand 3 1/2" I.D. (89mm) rubber radiator hose.  It's sold by the

>>foot, but all you really need is about 3-4".  Hope this helps.

>Is the 3 1/2" the OUTside diameter or the INside diameter.  Ive called NAPA,

and several heavy truck stores (to include white/volvo/GMC, Peterbuilt etc)

and

no one has 3 1/2" diam hose. One place said that #24250 was 2 1/2" INside

diameter hose. they wanted about $20 a foot.

>So is that part number right?

>Was the 3 1/2" a mistake, did you mean 2 1/2?

Well, you got me wondering about the hose that I bought, so I pulled it out

again to double check the numbers.  I'm sorry, but I quoted the wrong number.

The RIGHT Gates hose for the intake mod is 24256.  It is Clearly marked

"GATES Green Strip 3 1/2 " I.D. (89 mm)".  Give those truck dealers another

call with the Gates 24256 number.  The "I.D.", by the way, stands for Inside

Diameter.

This hose was bought FOR LESS THAN $10.00/FOOT!!!  If you get charged more

than this, you're getting "hosed" with the price.  If you can't find a truck

parts dealer to give you $10.00/ft, drop me a line.

Bruce Gordon / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Fri, 12 Jan 1996 23:06:57 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Jeep - ers!

Message-ID: <960113070656_73145.1566_FHD36-1@CompuServe.COM>

>Even though the Jeep has a higher axle ratio, does that mean that the

torque necessarily is multiplied more?

Yes, that is what a gear does, is multiply torque. In fact you simply

multiply the torque by the gear ratio to get the output torque. A gear

changes torque for rpm, and they are directly related.

>The Jeep has much heavier wheels and *much* heavier tires.

The Jeep 16" alloy wheels actually may weigh less than the monster 17"

Impala wheels, and likewise the tires. In any case, the differences here are

inconsequential.

>Also the circumference of the tires, depending on the option, is much

larger than the impala, which counteracts the effect of the 3.73 axle

ratio.

Yes, the Jeep tires are larger. Specifically, they are 28.40 inches in

diameter compared to the 27.04 inch diameter Impala tires. This is a

difference of only 5%. For comparison, the rear axle gear ratios differ by

17.4%.

There is one thing you have not thought of, and which would have been a much

better argument in your favor (just for future reference <g>), and that is

the transmission! The Impala has a very low 3.06 first gear, while the Jeep

has a 2.45. Since a 0-60 contest would also involve 2nd gear, those ratios

also go in favor of the Impala at 1.63 compared to 1.45 for the Jeep.

Rear wheel torque calculations in first gear for stock vehicles:

Tire

Engine      1st    Rear   Torque  Size    Relative

Torque      Gear   Axle   Output  Factor  Torque Output

--------------------------------------------------------

330 lb-ft x 3.06 x 3.08 = 3,110   +5%  =  3,266 lb-ft

300 lb-ft x 2.45 x 3.73 = 2,742   +0%  =  2,742 lb-ft

As you can see, the torque output at the rear wheels is actually greater in

the Impala due to the killer low first gear. In second gear, things are more

even:

Tire

Engine      2nd    Rear   Torque  Size    Relative

Torque      Gear   Axle   Output  Factor  Torque Output

-------------------------------------------------------

330 lb-ft x 1.63 x 3.08 = 1,657   +5%  =  1,740 lb-ft

300 lb-ft x 1.45 x 3.73 = 1,623   +0%  =  1,623 lb-ft

As you can see, the big factor in all of this is the transmission gear as

well as the rear axle gear ratio. The slightly shorter Impala tires do add a

minor boost to the relative torque to the ground as well, but the

transmission and rear axle gears are the major factor here.

>Also, even if the Jeep could hang with a stock Impala through 1-60, it

would fade away soon after.

This is true!

>The Jeep isn't even in the same ball-park aerodynamically and once the

vehicle reaches 40-50 mph, that becomes a very important factor in

acceleration.

Right, the Jeep is most definitely not as slippery as the Impala. On a race

course, I'd have to go with the Impala, but on the street in the rain, I'll

take the Jeep. <g>

I can tell you are taking some of my comments way too seriously!! <G> I

don't really think that I'll be out wasting Impalas with my (wife's) Jeep,

especially not any like mine that have been breathed on! I do think that the

Jeep mods I mentioned will have it staying with or possibly out accelerating

a stock Impala up to 60! <g> I can tell you for sure I will be running both

my Impala and wife's future Jeep at Union Grove to test my mods. This is all

just a big science experiment for me. The quarter mile dyno does not lie!

<g> Scott.

------------------------------

Date: Fri, 12 Jan 1996 23:07:12 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: valvesprings

Message-ID: <960113070712_73145.1566_FHD36-3@CompuServe.COM>

Scott,

>Have you seen the "conical" valve springs?  I think Comp Cams makes them.

They are 1.4xx" at the top and neck down to 1.2xx" at the bottom to fit in

the seats.

I have a set currently in my posession, and they are being returned as they

do not clear the roller rockers. In fact I should show you my spring

collection, I have no less than 7 different sets, 2 from Crane, 1 from

Competition cams, one from K-Motion, and three from GM (not including the

original ones). Out of all of these I have personally checked, and many

others whose specs I checked on paper and rejected, only the K-Motions are

acceptable unless you are willing to remove the heads for spring seat or

valve guide machining.

>I was wondering if you weighed the >95 and 96> pushrods.  From what I read

the LT4 has lightweight pushrods.

They are the same weight, just not hardened!

>Maybe they are a different wall thickness and/or a lighter alloy. This

should help with the valve float problems.

They are not a lighter or better alloy, they are actually the same, just not

hardened. The pushrods were essentially not redesigned for the LT4, other

than that they decided that the hardening process was no longer necessary

since they now use the head gasket as an assembly guide and have no pushrod

guide plates for the pushrod to wear against. The pushrod change for '96

was solely for cost reduction. Scott.

------------------------------

Date: Fri, 12 Jan 1996 23:07:22 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Camaro cluster

Message-ID: <960113070721_73145.1566_FHD36-4@CompuServe.COM>

Rush,

>Clearly, it seems one would be better off trading for a '96 than to piece

the console together.

This is true, but that is not stopping me from persuing this further, that

is part of the disease! I have done some research, and now I think I can

easily install the console, even if I retain the existing instrument

cluster. When I move the console shifter, the gear indicator in the dash

will move also. In otherwords, I can have both the mechanical linkage to the

trans through the column as well as the cable linkage from the console both

working at the same time. All I will have to change in the colunm is the

collar that has the colunm shifter attached. I can easily swap in a collar

that has no attachment point for the shifter. When I move the floor shift,

the collar will rotate, which will also move the gear indicator in the

instrument cluster!

>If you were to attempt to place the Camaro cluster in, wouldn't the

indicator still appear in the instrumentation, but not have any letters to

show what gear you were on? Or I'm sure somebody could rig an indicator

seperately on the steering column.

I will actually be prototyping the Camaro cluster install very soon! Mike

Sundstrom has graciously consented to bringing over his father's new Camaro

for us to take apart. <g> We will be removing the cluster and doing a

prototype fitting in my Impala. This will give us an immediate pass/fail

indication of weather this is truely feasable or not. I have to do this to

prove if it can be done or not, so I can stop daydreaming about it! <g>

There are a few small issues that need to be addressed, but electrically the

install seems to present no insurmountable problems, I just want to see if

there will literally be any rough edges showing when physically mounting the

cluster. I can get a brand new Camaro cluster for about $500, which I don't

think is too bad. If I can make it look factory installed, then I will have

both the Camaro cluster and the '96 floor shifter in my car! If not, then I

can either retain the existing '94 cluster (I actually like it with my added

digital tach and digital temp. gauge) or install the '96 cluster. If I went

with the '96 cluster, then I would add a digital oil pressure gauge and

voltmeter to the cluster bezel where I currently have a tach and temp gauge.

Note that the Camaro cluster does not have a gear indicator at all, so that

will have to be removed from the column. That is no problem as long as you

also have the floor shifter, since the gear indicator is on the floor

shifter itself as well.

After installing the Floor shifter and Camaro cluster, then it will be on to

the overhead console with digital compass and indoor/outdoor temperature

indicators! Then I will really be done adding options, I mean really, for

sure... well, maybe not, I forgot about the seat heaters and power lumbar

and recline! <g> Scott.

------------------------------

Date: Sat, 13 Jan 1996 00:32:46 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Caprice/Impala info

Message-ID: <960113083246_73145.1566_FHD48-5@CompuServe.COM>

Lennie,

>1.) If the '93 9C1 had the GU5 3.42 rear, why did GM change to a GU3 3.08

rear in '94?

Probably because the higher powered LT1 could still accelerate appreciably

even with the taller 3.08 gears. They were after economy more than

performance.

>2.) Would it be a good idea to install the GU5 3.42 rear in my '95 SS?

You bet! I have done this to my Impala, as have several others on the list

as well.

>a.) Can the dealer handle the switch like an original factory installion?

If they are competent they can. Personally, I did the gear swap myself.

>b.) Would the price be about $500-600?

The gears cost me $273, and I decided to get all new bearings which added

about $100 more to the cost. If you re-use your existing bearings (which is

probably what most non-anal retentive people would do), then the total parts

cost should be right around $300 including the ABS gear reluctor (only $7),

fluid, gasket, etc. As for labor, that is up to whoever does it but I would

expect no more than $300, and probably it would be less.

>c.) Can they reprogram the PCM correctly?

Yes, the dealer can do this. They have to contact a GM Delco service center,

who can create the proper PCM program with the gear ratio change. Then the

dealer downloads this program into their Tech 1 Scan tool, and from there

downloads it into your car. Another alternative is to install a custom

aftermarket performance program such as those from Hypertech, Superchips,

Z-industries, etc. Scott.

------------------------------

End of IMPALA Digest 235

************************

IMPALA Digest 236

Topics covered in this issue include:

1) Re: No Mufflers

by Kurt Kemmling <76165.1654@compuserve.com>

2) 9C1 Driver door linkage

by Steve Das <sdas@sat.net>

3) Shifter Lock '96

by John Spina <caspers@miso.wwa.com>

4) Re: 1/4 mile Dyno.....

by Kurt Kemmling <76165.1654@compuserve.com>

5) 3.42 gears & PCM

by jsalmon@mindspring.com (John Salmon)

6) Re: IMPALA digest  - administrivia

by Don Hurter <don@sirius.com>

7) Valve adjustments

by Scott Mueller <73145.1566@compuserve.com>

8) Expertise Necessary to do some Mods.

by BGordon401@aol.com

9) Re: 3.5 inch hose

by rcheek@one.net (Robert Cheek)

10) Headlight Alignment`

by rcheek@one.net (Robert Cheek)

11) BearTracker Installation (again)

by rcheek@one.net (Robert Cheek)

----------------------------------------------------------------------

Date: Sat, 13 Jan 1996 12:38:18 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: No Mufflers

Message-ID: <960113203817_76165.1654_GHM47-3@CompuServe.COM>

In Impala Digest 232 Scott Harris wrote:

>>>Here in Massachusetts we have now the same emissions requirements as

Califormia.  It passes.  Why wouldn't it.  The cats are still there.  The

only emissions the mufflers affect is noise and I would never call my car

noise pollution!<<<

Scott,

I live in CT , and am seriously considering doing the same thing to my exhaust

system.

I know I will pass emissions even though CT announced today, that they have the

toughest

emmisions laws in the Northeast, having just passed a bunch of new ones.

The only thing I am concerned with, is that I do mostly highway driving, and

have been

reading about an annoying drone or note resonating thru the car at highway

speeds.

Is this your experience also, or does it have to do with where the H pipe is

located

in the exhaust system? Since we are almost neighbors,<G> perhaps we can arrange

a meeting 1/2 way between us, so that I may hear and view your custom exhaust

setup.

I do not plan on doing this until after the snowmelt, so time is not an issue.

TIA for you help,

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Sat, 13 Jan 1996 14:03:25 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: 9C1 Driver door linkage

Message-ID: <199601132203.QAA29804@raven.sat.net>

Went out today and played around with one of the new 9C1's the local

dealer has for sale.  Interestingly, the drivers door acts just like the

door on my '95 Impala.  It will not unlock with the door handle.  I'm not

sure if this is a specially ordered option but it apparently is not standard

on the '96 9C1 cars.  Incidentally, these cars have two spotlights and ALL

the extra wiring in them.

Steve Das......sdas@sat.net

------------------------------

Date: Sat, 13 Jan 1996 14:07:15 PST

From: John Spina <caspers@miso.wwa.com>

To: "'impala@east.xsis.xerox.com'" <impala@east.xsis.xerox.com>

Subject: Shifter Lock '96

Message-ID: <01BAE1D1.550DEB60@vh2-051.wwa.com>

Does anyone know how the shifter lock works? You can only shift out of park

if the brake is applied, leading me to assume there is an interlock

solenoid in the floor shifter mechanism.

Also, does anyone know how the driver's side power window switch

works......does that switch have some sort of mechanical latch that keeps

it locked in the down position while the window is travelling?  It doesn't

seem to have an electronic delay like some of the other GM cars had.

Thanks, John Spina

------------------------------

Date: Sat, 13 Jan 1996 15:48:20 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: 1/4 mile Dyno.....

Message-ID: <960113234819_76165.1654_GHM36-5@CompuServe.COM>

In digest 235 Scott Mueller wrote:

>>>. The quarter mile dyno does not lie!

<g> Scott.<<<

How very true. No matter how many numbers and chassis dyno figures you want to

crunch, it all comes down to that 1320 feet of asphalt. I can't wait until this

snow melts, so I can generate a few more timeslips.

Can I use you quote in the future Scott?<s>

Figuring out the rear wheel torque of my lawnmower,<BG>

Kurt

------------------------------

Date: Sat, 13 Jan 1996 16:11:54 PST

From: jsalmon@mindspring.com (John Salmon)

To: impala@east.xsis.xerox.com

Subject: 3.42 gears & PCM

Message-ID: <v01530500ad1da434e2c9@[168.121.55.70]>

Re:  the 3.42 GU5 gear swap discussion...

>c.) Can they reprogram the PCM correctly?

>Yes, the dealer can do this. They have to contact a GM Delco service center,

who can create the proper PCM program with the gear ratio change. Then the

dealer downloads this program into their Tech 1 Scan tool, and from there

downloads it into your car. Another alternative is to install a custom

aftermarket performance program such as those from Hypertech, Superchips,

Z-industries, etc. Scott.

Given the ratings I've seen in a couple of tests of JET's PCM, and

admitting that they may be inflated figures since the test was run on JET's

own dyno, and given that JET's pcm is cheaper than the Hypertech

programmer, can you achieve the same results using JET as you can with

Hypertech with regard to fan turn-on temperatures, rear gear swaps, shift

points & firmness, etc.?  I talked to a rep at Summit recently and he

recommended the JET PCM over Hypertech.  Said he had just recommended the

same to a Caprice owner who had already tried Hypertech and that he had

called the guy back a couple of weeks after the sale to see if the customer

liked the JET's performance (Now that's what I call a customer-first

attitude!) so he could know whether to recommend JET in the future.  He

said "you'll really only feel the Hypertech when you're at the drag strip,

where you'll feel the JET more even in daily driving."  I know JET offers a

fan switch kit and a cooler thermostat, but I haven't heard any mention of

reprogramming for the rear gears.  How much does this add to the cost of a

swap at a dealer?  What part numbers do you need to give them for the GU5

3.42 gears (if this has been in a previous post, I apologize but could it

please be reposted?)?

Also, killing another bird with the same email, has there been any

development on the Flowmaster cat-back kit or new Impala muffler some list

members were prototyping?  I'm dying for some new tunes from the exhaust

system, but I don't have $700 to throw at a Borla kit right now.

Thanks,

John Salmon

Data Warehousing Technical Support

personal: jsalmon@mindspring.com

work: john.salmon@bridge.bellsouth.com

'95 Dark Cherry Impala SS -- The SALMN8R

------------------------------

Date: Sat, 13 Jan 1996 17:11:28 PST

From: Don Hurter <don@sirius.com>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest  - administrivia

Message-ID: <199601140114.RAA19522@terra.sirius.com>

At the end of each mailing is the following:

------------------------------

End of IMPALA Digest XXX

************************

Can the list moderator tack on the URL for Dave Hardy's Impala SS page such as:

End of IMPALA Digest XXX

************************

for more Impala info, try

http://www.crl.com/~impala/

This will allow new list members to find more information, or else

quickly jump to the earlier list archives.

Thanks,

Don

------------------------------

Date: Sat, 13 Jan 1996 20:52:39 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Valve adjustments

Message-ID: <960114045239_73145.1566_FHD45-3@CompuServe.COM>

Mike,

>are the valves on the LT1 self-adjusting or one do have to adjust them

periodically.

There is no adjustment with most hydraulic lifter valvetrains, and the LT1

is no exception. There is a preload setting, but that is only done once when

the rockers are installed. You basically tighten the rocker nut down until

there is zero lash (no play), and then make one more turn on the nut. This

preload on the lifter prevents it from bottoming or topping out.

>Ever since I picked up my 96 about three weeks ago, I have noticed some

clicking sound coming from the left bank of the engine.

That is the nature of the beast! I have the same slight clicking on my LH

side as well, and I have heard it on others. The thick plastic 'vette

valvecovers are designed to help hide this natural noise, which is one of

the several reasons I recommend them.

>I start to suspect maybe my Impala is on its way to become a Lemon. While I

was bring in the car for the wheel alightment which I told you about around

two weeks ago, just as I arrived at the dealer, all the sudden the car

starts to mis-fire.  A quick check revealed that the spark plug wire had

fallen off my #1 cylinder.  All the service personal in the dealer thought

this is unbeliveable.

I hope they checked all of the other ones as well. I performed a slight

upgrade to my spark plugs and wires. First I installed the better '96

version spark plugs (AC 41-943) in place of the original ones that came in

the '94/'95 cars (AC 41-906). These newer plugs have less resistance, and

more platinum on the tip. Next I noticed that three of the spark plug boots

were covered by special metal heat shields. I purchased 5 more of these

shields, polished all of them (that boy is ill!), and slipped them on.

Besides protecting all of the expensive wires from the manifold heat, having

polished heat shields on all of them simply looks better! At the same time,

I coated all of the visible parts of my exhaust manifolds with Eastwood

stainless steel paint. This stuff makes the manifolds look like brand new

cast iron and they stay that way.

>Then, last week, while I was doing some highway driving I heard a rather

lound wind buffing sound coming from the driver side door. After a closer

examination revaled that the both weatherstrips running along the bottom

edge of the both doors on the driver side are about a inch short from the

edge!  This means another trip back to the dealer. Seriously I am really not

impressed with those glitches which seems to pop up contiously in my car. I

hope this is just a isolate incidence and won't occur again.

I have found numerous small glitches like this in my own car. My '94 was

missing the factory muffler heat shield above the LH muffler! It was

absolutely never installed. The LH rear passenger door was mis-aligned, as

was the rear fascia. The ball joints were torqued incorrectly, the trunk

light broke, there was a rattle, etc. I have corrected quite a few minor

problems like this, I have forgotten how many! I keep saying, it's only a

Chevy, not a Lexus.

Well, at least I can see they are improving. You should have seen the

assembly goofs and quality glitches on my '87 Buick Grand National! They all

had to have had the worst GM paint ever, and many little things were wrong,

missing, did not line up, rattled, etc. My '89 TTA had fewer problems, but

similar quality and build related problems nonetheless. The Impala was about

as good as my TTA. The new F-cars seem to be built much better than the SS

or my TTA, and I am looking forwared to getting one next year.

Last week I inspected a brand new '96 Impala at the local dealer, and from

right off the truck, the oil fill cap on the RH valvecover was missing! I

mean how could somebody forget that? The interior door handle screw was also

missing, the trim was not fully attached at the front of the rear wheel

opening, etc. This was stuff I found on a cursory inspection, if I owned it

I would find a whole bunch more. Sometimes I wonder about the folks at

Arlington. Good thing for GM I like their cars so much I put up with this

stuff. It would be far worse if I had to take it back to the dealer to

repair these glitches, in most cases they would make it worse, not better.

>ust out of curiousity, just how much money have you spend so far in

modifiying your SS?

I don't even know for sure, but probably around $3K so far, maybe more.

Scott.

------------------------------

Date: Sun, 14 Jan 1996 06:12:16 PST

From: BGordon401@aol.com

To: 73145.1566@compuserve.com

Cc: impala@east.xsis.xerox.com

Subject: Expertise Necessary to do some Mods.

Message-ID: <960114091215_41278861@mail02.mail.aol.com>

Scott...

First off, thank you for your message about pushrod wear w/ the 1.6 roller

rockers.

Which kind of leads me to my next concern.  I'm ready to order the parts for

the Corvette Injector covers, and also will probably be replacing  the valve

covers to take advantage of the noise reduction and "oil drippers" (I'm

assuming that 1995 Covette LT1 valve covers will fit).  If I'm going as far

as the covers, I might as well do the roller rockers since everything will be

open, which leads me to me question.

Since I've never attempted anything like this before, could I, or should I,

do the roller rocker mod myself?  This also applies to upgrading the 3.08

gears to 3.42.  I'd prefer do the work myself(sense of pride that I did it

myself), but my prudent side warns me not to get over my head.  I REALLY HATE

the idea of possibly messing up my car.  Eventhough I loathe the idea of

paying $300.00 for someone to install the gears or roller rockers, at times,

it's the best thing to do (just like the tailpipe upgrade, no way to bend

stainless myself).

My expertise level with cars is a basic backyard self-taught "jobber".  The

most difficult mod I've ever tackled was upgrading a stock 2bbl intake

manifold on a 1977 Pontiac 301 to a 4bbl manifold w/ edlebrock carb and

rebuilding the front end, including ball joints.

Your expert opinion, since you've "been there,and done that" would be greatly

appreciated.  Also, any special tools for these mods that are absolutely

necessary.  From what I understand, the gear area is rather involved.

Thanks again for taking your time to help me out with these problems and

questions!!

..Bruce

------------------------------

Date: Sun, 14 Jan 1996 09:31:47 PST

From: rcheek@one.net (Robert Cheek)

To: impala@east.xsis.xerox.com

Subject: Re: 3.5 inch hose

Message-ID: <199601141708.MAA32585@mail.one.net>

Way behind on the reply, but Troy wrote...

>Having run the 3.5" chrome pipe (cut/angled/welded) for some time, I've

>noticed it holds so much heat you can hardly touch it.  This defeats the cold

>air induction theory.  I would like to replace the pipe with something non

>metal and flexible in the same size.

Wouldn't a black hose absorb more heat, while the chrome tubing would

reflect more? I assume that's why it would feel so hot. Have you taken

it apart while it was "hot" to check to see if the inside was the same

temperature? Not flaming, just "chromed and curious"...

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers + "H" Pipe

------------------------------

Date: Sun, 14 Jan 1996 09:31:43 PST

From: rcheek@one.net (Robert Cheek)

To: impala@east.xsis.xerox.com

Subject: Headlight Alignment`

Message-ID: <199601141708.MAA32579@mail.one.net>

Just dug my car out from under about 1.5 ft of snow last night,

was updating the FAQ, and remembered that I wanted to adjust the

headlights. I asked a while back but no one answered: does anyone

know? (BTW, I got the vertical, but the horizontal doesn't seem to

move).

>1) Is the adjuster screw the _inboard_ one with the six-sided plastic cover

>on it immediately in front of the radiator? I cranked on this for

>a while and could tell if it was moving or not. It was dark and I

>couldn't see too well, but the light never seemed to move.  I loosened

>the Torx screws in the channel beforehand.  The manual says the adjuster

>is _outboard_ of the headlights. Is this right?  Could not find a

>good picture of the adjusting screws in the service manual.

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers + "H" Pipe

------------------------------

Date: Sun, 14 Jan 1996 09:31:45 PST

From: rcheek@one.net (Robert Cheek)

To: impala@east.xsis.xerox.com

Subject: BearTracker Installation (again)

Message-ID: <199601141708.MAA32582@mail.one.net>

Well, finally got out to install the BearTracker yesterday. I found a

place where I wanted the bracket, but then I tried to find out how

to plug the sucker into my radio antenna. What a PITA! Do I _really_

have to remove all of the dash panels just to get to the antenna

plug on the back of the radio? Is that what all of the other Beartracker

people have done? I noticed that there is a connection on the other

side of the firewall that I could splice into, but then I'd have to

route back through the firewall with the thick coax cable, and I'm

also afraid the Uniden "T" connector isn't as weatherproof as the

GM unit. Anyone?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers + "H" Pipe

------------------------------

End of IMPALA Digest 236

************************

IMPALA Digest 237

Topics covered in this issue include:

1) RE: Home plate removal

by "Harris, Scott" <sharris@xyplex.com>

2) Do it yourself?

by Scott Mueller <73145.1566@compuserve.com>

3) Door unlock mod

by Scott Mueller <73145.1566@compuserve.com>

4) Jet module

by Scott Mueller <73145.1566@compuserve.com>

5) Re: Flowmaster, new version

by Rock528@aol.com

6) JET and Rear change info

by Rock528@aol.com

7) Question for Scott Mueller/Corvette Valve Cvrs.

by Rock528@aol.com

----------------------------------------------------------------------

Date: Sat, 13 Jan 1996 23:57:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: Home plate removal

Message-ID: <30F9445A@corvette.xyplex.com>

Jeff,

There is a home plate cap available at your local grocery store.  Nestea

has a blue cap on their diet version that fits nicely.  It is almost

exactly the correct size.  You want to install it upside down with the

flat sitting flush to the inside of the elbow.  This will not only leave

a plain surface facing the world but, much more importantly, with the

flush surface inside will remove the vortexes and eddies that the

aluminum heater caps do not deal with.  There is an image of my engine

before Vette covers with this mod on the web page as

"ftp://ftp.crl.com/ftp/users/im/impala/images/imp_lt1.gif".  The blue

color of the cap does not stand out as much as the image indicates.  I am

still searching for cheap 1LE elbow or a black cap.

There should not be any heat issues in this placement so plastic is OK.

If enough heat reaches this to melt the plastic, your engine's on fire

and you have more than this to worry about.

Optimally you would want the 1LE elbow.  Next best option would be a plug

that fits flush internally with stepped OD holding it in place

externally.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Sun, 14 Jan 1996 11:06:43 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Do it yourself?

Message-ID: <960114190643_73145.1566_FHD24-1@CompuServe.COM>

>Since I've never attempted anything like this before, could I, or should I,

do the roller rocker mod myself?  This also applies to upgrading the 3.08

gears to 3.42.  I'd prefer do the work myself(sense of pride that I did it

myself), but my prudent side warns me not to get over my head.  I REALLY

HATE the idea of possibly messing up my car. Eventhough I loathe the idea of

paying $300.00 for someone to install the gears or roller rockers, at times,

it's the best thing to do (just like the tailpipe upgrade, no way to bend

stainless myself).

>My expertise level with cars is a basic backyard self-taught "jobber".  The

most difficult mod I've ever tackled was upgrading a stock 2bbl intake

manifold on a 1977 Pontiac 301 to a 4bbl manifold w/ edlebrock carb and

rebuilding the front end, including ball joints.

>Your expert opinion, since you've "been there,and done that" would be

greatly appreciated. Also, any special tools for these mods that are

absolutely necessary. From what I understand, the gear area is rather

involved.

This is a tough question to answer!! Sometimes I think to myself, it's like

asking what is the price of an F40 Ferrari, "If you have to ask, you can't

afford it." On the other hand, by asking you are showing caution and a

knowledge of your own capabilities, which are both very admirable traits

when it comes to auto tech work. Personally, I am of the notion that if I

have the service manual and the proper tools, I can do anything. I also have

the ear of at least one VERY experienced tech to help me verify a certain

approach or technique. I am always doing things I have never done before,

and learning tremendously from each experience.

I also know that there are often several ways to do a particular job. You

can do something in a cheap way, attempting to take shortcuts or re-use

existing parts where possible, or you can replace things that should be

replaced, and use new or upgraded ancillary components. For example, in the

case of the roller rockers, you will encounter valve float at about 4,750

rpm unless you change the stock valvesprings as well. Even so, I know that a

number of supposedly reputable performance companies that both sell and

install these rockers are telling people that they will work with the stock

springs. Their definition of "work" is obviously different from mine! I find

questionable advice and practices at every turn in this business, so I have

come to trust little unless I can verify and corroberate everything with my

own research and testing.

I don't have time today to finish the install instructions to the level of

detail I intend, but here are the basics. In the future, I will be writing

this up more completely, including all of the measurements, calculations,

tests, etc. that I performed. I'll break it down into several approaches:

1.) Install roller rockers only - This is not recommended as you will have

valve float at about 4,750 rpm, which is right before the stock factory

shift points. With the stock rockers, the stock springs have 80 lbs of seat

(valve closed) pressure, and 195 lbs. with the valve open. With the higher

lift due to the roller rockers, valve open pressure increases to only 203

lbs. with the stock springs, which is not enough. For comparision here are

the spring pressures on the LT1/LT4 engines:

B-car F/Y-car Y-car

LT1     LT1   LT4

================================

Valve Closed   80      85   102

Valve Open    195     262   260

My approach was to duplicate the LT4 settings as closely as possible. We

need more spring pressure, but only enough to do the job. Way more than

enough spring pressure just causes added wear. Note that the LT4 springs

would be great, but since they are a larger diameter you would have to

machine the spring seat for them to fit.

2.) Install roller rockers with upgraded valvesprings - This is recommended

as the upgraded springs will prevent float well beyond 5,500 rpm. The only

problem here was finding springs that would fit without machining the heads.

Also required was springs that could work without dampers, as there is no

room for them either without machining. I did find springs that would work

(K-Motion 700), however at the stock installed height, the pressure was a

little higher than I liked:

B-car LT1 w/1.6 rocker, K-700 springs:

Valve Closed:  115 lbs.

Valve Open:  311 lbs.

This will work fine, but is a little more pressure than what I wanted.

3.) Install roller rockers with upgraded valvesprings and keepers - By using

what are called offset keepers (locks), you can raise or lower the spring's

installed height, which also affects how far the spring is compressed.

Changing installed height affects both valve closed and open pressure.

Raising installed height lowers the pressure, while reducing installed

height increases pressure. In all cases you have to be aware of the spring's

free length as well as the length under coil bind. At no time should a

spring be compressed to less then .050 inches from coil bind, and in fact a

larger clearance is preferred.

What I did was to obtain a set of Crane machined keepers that are offset by

.050, raising the installed height from 1.700 inches to 1.750 inches. This

would have the net effect of reducing the pressures as follows:

B-car LT1 w/1.6 rocker, K-700 springs:

1.700   1.750

height  height

=========================

Valve Closed  115      94

Valve Open    311     289

This sounded good, however when I actually measured the keepers, they were

only offset by .035 from the stock ones, which had an actual installed

height of 1.695 inches. This meant that the calculations had to be revised:

1.695   1.730

height  height

=========================

Valve Closed  117     102

Valve Open    313     298

These are calculations based on actually measuring the engine, and not just

from specifications. I learned that often what is reported in a spec sheet

does not always come true in the real world. I purchase a spring height

micrometer as well as a spring pressure tester to validate all of my

measurements and information.

I also found that the keeper groove on the intake valve was higher than that

on the exhaust, such that the raised height keepers on the intake valves

would come only .008 inches from the washers on the rockers, which is not

enough clearance. So I had to grind .040 from the top of each of the keepers

used on intake valves for proper clearance. The exhaust valve grooves are

lower, and even the raised keepers are already .050 away from the washers.

The portion ground off of the keeper is above the retainer, and does not

interfere with the fit of the keeper between the retainer and valve stem.

Also, the Crane machined offset keepers are much stronger than the stamped

steel factory keepers.

The machined offset keepers are about $30, and it took me a few minutes to

grind .040 inches off of 16 of them for the intake valves. I used a bench

grinder with a vise to support the keepers for grinding, plus a micrometer

to check everything out. If this seems like too much, then I would recommend

simply re-using the stock keepers, as the slightly higher pressures should

not be a real problem. I just felt that for a small additional investment in

the offset keepers and a little grinding for clearance, that I could come

very close to my target pressures.

4.) Install roller rockers, remove and machine the heads for LT4 springs,

install LT4 springs and retainers - This would be ideal, and having the

heads machined for the larger LT4 springs costs only about $40. The real

problem here is that the heads have to be removed and replaced, which adds

considerably to the labor, not to mention the cost.

5.) Install roller rockers and aluminum LT1 heads with LT4 springs - If I

were to to remove the iron heads, I would most certainly install the

aluminum heads in their place. They are 25 lbs. lighter per side, and

already have the proper spring seat and guide machining for the LT4 springs.

6.) Install roller rockers and aluminum LT4 heads (with LT4 springs), and

matching LT4 intake - This would be nice, however the LT4 heads have raised

(larger) intake ports, which require the matching LT4 intake manifold as

well. At this point, you are basically converting your LT1 engine into an

LT4, but are still missing many special LT4 items like the stronger crank,

different pistons, different cam, different timing chain and gears,

different front pulley dampner, bigger injectors, etc. For just a little

more than the heads and intake alone, you can get...

7.) Drop in a crated, fully dressed LT4 factory complete engine! This would

be the ultimate, of course, and the fully assembled angine is available for

under $5K from the factory.

Basically, I recommend that you go with #2 (rockers and springs only), or #3

(rockers, springs, and offset keepers). You can get the springs and keepers

from Jegs or Summit.

To do these mods requires only a few special tools. First, don't even think

about it if you don't have the service manual. You will also need an air

compressor and a spark plug hole adapter to hold the valves up when changing

the springs. You will also need a valvespring compressor, and it must be the

small screw type design (not the lever design) so it can fit under the cowl

area. I recommend the one from Kent-Moore. This job will be VERY difficult

on the car for F-car owners due to the cowl in those cars! It is not that

bad at all on the Impala since there is so much room to work. The hardest

part on the Impala is getting to the #2 spark plug, and installing the air

compressor fitting in that hole.

You will need to jack the car up on stands to remove some of the plugs, at

least it is much easier that way. If you are going to grind the keepers, you

should have a micrometer, a feeler gauge, and a vise or other stable

platform to use when grinding.

BTW, since you have to remove and replace the plugs during this operation,

it makes sense to install the new upgraded '96 design spark plug (41-943),

which offeres lower resistance, faster starts, and more platinum on the tip.

Until I get a more complete writeup done, if anybody has any questions on

installing the rear end gears or roller rockers, I would be happy to help.

Scott.

------------------------------

Date: Sun, 14 Jan 1996 11:21:18 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Door unlock mod

Message-ID: <960114192117_73145.1566_FHD64-1@CompuServe.COM>

>Went out today and played around with one of the new 9C1's the local dealer

has for sale.  Interestingly, the drivers door acts just like the door on my

'95 Impala.  It will not unlock with the door handle.  I'm not sure if this

is a specially ordered option but it apparently is not standard on the '96

9C1 cars.

Indeed, SEO 6N1 (Driver's door handle lock release) is a $20 option, and was

not ordered on the car you saw. Scott.

------------------------------

Date: Sun, 14 Jan 1996 18:17:30 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Jet module

Message-ID: <960115021729_73145.1566_FHD24-1@CompuServe.COM>

John,

>can you achieve the same results using JET as you can with Hypertech with

regard to fan turn-on temperatures, rear gear swaps, shift points &

firmness, etc.?

No way! Most people don't seem to understand that the JET "chip" or "PCM" is

really no such thing. All it does is intercept the sensor signals on the

grey PCM connector, which contains almost exclusively sensor input data.

Here are some facts about the JET system that people must understand:

-It cannot possibly reprogram the PCM

-It cannot control the vehicle independent of the PCM

-It cannot operate the engine outside of PCM controlled parameters

-It cannot control the transmission

-It cannot control the fans

-It cannot control the air/fuel ratio

-It cannot control the spark advance

-All it can do is intercept and modify sensor data, "fooling" the PCM

The sensors it can mess with are the O2 sensors, IAT (Intake Air

Temperature) sensor, TP (Throttle Position) sensor, MAP (Manifold Air

Pressure) sensor, ECT (Engine Coolant Temperature) sensor, and Transmission

temperature range switches. Those are pretty much the only wires in the grey

PCM connector the JET device plugs into!

What this JET device does is to intercept these sensor readings, and to pass

different readings to the PCM in order to make the PCM compensate for the

different reading. For example, all PCMs are setup with internal spark and

fuel tables that vary the spark and fuel based on temperature, engine load,

and other sensor variables. These tables are programmed to richen the

air/fuel ratio if the ambient and engine coolant temperatures are low. So if

the JET device intercepts the IAT and ECT sensor readings, and adjusts them

lower before passing them to the PCM, then the PCM will be fooled into

selecting a lower temperature fuel calibration from the table that richens

up the mixture. Note that since this technique fools the PCM, it will also

fool a technician using a scan tool to troubleshoot a sensor problem. The

PCM will be reporting improper readings, and the technician will be

replacing sensors that are not bad.

Also note that contrary to some of the bogus hype that JET seems to be

passing along, their device does not seem to have a 20 MHz RISC

microprocessor, nor is it able to operate the independently of the PCM. They

seem to imply that their unit can advance the spark or change the fuel ratio

beyond what the PCM can normally do. Obviously this is false, as their unit

simply fools the PCM into reading different sensor values, and the PCM

software compensates accordingly. The PCM is still in full control of the

system, and only PCM pre-programmed parameters will be used for spark, fuel,

etc, according to what the sensor inputs indicate.

On the other hand, companies like Superchips, Hypertech, and Z-industries

can actually reprogram the PCM, altering the data tables so that differen

spark and fuel curves can be run. These companies can also re-set the

temperatures that the electric fans operate at, they can control the TCC

(Torque Converter Clutch) lockup speeds, alter the top speed cutoff, engine

rev limiter, change transmission shift points, shift firmness, and literally

everything else that is under PCM control. There is a big difference between

actually reprogramming the PCM and just intercepting some sensor data and

fooling the existing program to select different but pre-existing

parameters.

>I talked to a rep at Summit recently and he recommended the JET PCM over

Hypertech.

I have also talked with Summit reps and techs about different items and I

can assure you their advice is worth the price (free). They are there to

sell, and if you try to get anything other than facts, you will likely be

mislead, and they often have a hard time with just the facts.

>He said "you'll really only feel the Hypertech when you're at the drag

strip, where you'll feel the JET more even in daily driving."

The Hypertech unit is very noticible at part throttle. It was so much of an

improvement for me, that I would have recommended the device had there been

no full throttle improvements whatsoever. Other companies like Superchips

and Z-industries also reprogram the PCM, and make part throttle changes as

well.

>I know JET offers a fan switch kit and a cooler thermostat, but I haven't

heard any mention of reprogramming for the rear gears.

That's because they can't reprogram for different rear gears, in fact they

cannot reprogram anything! That is why they have to offer a fan switch kit

while the others can simply reprogram the temperature the PCM will turn on

the fans.

>How much does this add to the cost of a swap at a dealer?

A gear swap would go for about 5 hours of time at the dealer, so you would

have to base it on what they charge for an hour. The PCM program change

would also be based on time, and it only takes a few minutes to reprogram

the PCM.

>What part numbers do you need to give them for the GU5 3.42 gears (if this

has been in a previous post, I apologize but could it please be reposted?)?

Here are the parts needed for the GU6 3.42 (GU5 is 3.23) gear swap, besides

the gear, you will need at a minimum the following items:

Item/Description                   Part no.    Cost

-------------------------------------------------------

GU6 factory 3.42 gear set          #26041514   $ 273.24

20-tooth ABS reluctor              #26018945   $   7.45

pinion seal                        #26033578   $   9.07

cover gasket Fel-Pro               RDS 55028-1 $   4.75

2 qts. of (synthetic) diff. fluid  Mobil-1     $   6.99

GM limited slip additive           #1052358    $   4.28

Since you have to press on the pinion bearings, it is recommended that you

install new ones, rather than trying to press off the old ones and then

press them back on the new pinion. You may also need carrier side bearings

like I did (mine did not look great when I removed them with only 10K miles,

but that is not normal. Obviously you need to check out everything, and if

there are problems, replace the necessary additional parts.

The axle side bearings are normally fine, but I always recommend replacing

the seals. I also purchased an additional pinion crush collar, since they

are useable only once, and although the gear kit included one, having a

spare will be useful if you accidentally over torque the pinion nut. I also

recommend a new pinion nut, as they are a prevailing torque (crushed) type

fastener, which can loosen if used too many times.

Finally, shims are used to set the pinion depth and backlash, but you won't

know which shims to get until you take it apart to measure everything. Some

people will just re-use the original shims, but it is my experience (and

that of an associate who has done dozens of 8.5" rearends) that the original

shims are not always fully correct. I recommend that you use the proper

pinion depth gauge to check both depth and backlash, and compensate

appropriately with the proper shims. There are 4 different shim kits for

pinion depth, and 11 kits for setting backlash.

Here are the additional parts needed for a proper job (IMHO):

Item/Description                   Part no.    Cost

-------------------------------------------------------

Pinion front bearing               #9418356    $  35.17

Pinion rear bearing                #9413427    $  34.55

Pinion Nut                         #1260823    $   2.34

Pinion spacer (crush collar)       #1234726    $   2.49

Axle seals (2 reqd)                #554631     $  11.96 ($5.98 ea.)

Shim Kits (as needed, up to 2 reqd)            $  18 each

That should about do it! Scott.

------------------------------

Date: Sun, 14 Jan 1996 19:27:24 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Flowmaster, new version

Message-ID: <960114222723_61575344@mail02.mail.aol.com>

In a message dated 96-01-14 12:42:33 EST, you write:

>Also, killing another bird with the same email, has there been any

>development on the Flowmaster cat-back kit or new Impala muffler some list

>members were prototyping?

Had lunch with Razzorfl last week regarding this.  I dont know if he's caught

up on his email yet so I'll answer.  Flowmaster told him they are getting

ready to complete production on these early February, I believe.  Please

don't quote me to Flowmaster <g>.  Troy

------------------------------

Date: Sun, 14 Jan 1996 19:27:29 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: JET and Rear change info

Message-ID: <960114222721_61575332@emout06.mail.aol.com>

In a message dated 96-01-14 12:42:33 EST, you write:

>  I know JET offers a

>fan switch kit and a cooler thermostat, but I haven't heard any mention of

>reprogramming for the rear gears.

Having a Jet Stage II pcm device, I posed this question regarding

reprogramming for the rear end gears to JET directly a few weeks back.  Here

is their reply:

Troy,

"You can upgrade the stock ECU and still use the PCM (Jet's pcm) as long

as you dont let anyone mess around with the fuel and ignitions table in the

stock computer. The only change they need to make is for the gear ratio

calibration.

Thanks, JETCHIP"

Incidentally,

re: < I know JET offers a fan switch kit and a cooler thermostat>

Jet's cooling kit consists of a cooler temp sensor mounted in the left

forward head and wired into the fan turn-on relays.  They do not mess with

the thermostat.  I do notice that the fans kick on alot earlier and it does

not get hot in the FL summer at idle.  However, it does not show as cool on

the temp gauge as some of the local guys that are running hypertech's with a

160 thermo.  Troy

------------------------------

Date: Sun, 14 Jan 1996 19:27:25 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Question for Scott Mueller/Corvette Valve Cvrs.

Message-ID: <960114222724_61575363@mail06.mail.aol.com>

In a message dated 96-01-14 12:42:33 EST, you write:

>Which kind of leads me to my next concern.  I'm ready to order the parts for

>the Corvette Injector covers, and also will probably be replacing  the valve

>covers to take advantage of the noise reduction and "oil drippers" (I'm

>assuming that 1995 Covette LT1 valve covers will fit).

Scott Mueller, since youv'e already installed the corvette valve covers I'll

ask you this question.  I'm not sure of the alternator placement on the

Corvette but the right side valve cover on the Impala has some relief for the

back of the alternator.  Do the plastic valve covers have a similar

indentation for alt. clearance?  Some of the aftermarket chrome covers do not

and need a slight coaxing (hammerin).  Troy

------------------------------

End of IMPALA Digest 237

************************

IMPALA Digest 238

Topics covered in this issue include:

1) RE: auto lock/unlock features a thiefs dream ?

by CoolChevy@aol.com

2) 'vette valvecovers

by Scott Mueller <73145.1566@compuserve.com>

3) Re: IMPALA digest 236

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

4) FW: JEEPERS CREEPERS

by mbailey@rinet.hunton.com (Bailey, Martin)

5) Colored wax?

by "David Spitz" <david_spitz@vnet.ibm.com>

6) Re: Beartracker Install

by John_P._Kelly@troubbs.org (John P. Kelly)

7) Re:Beartracker Install

by Mfetto@aol.com

8) BearTracker Antennas - Reply

by John Pinkerton <jpinkert@sigg.com>

9) Re: Valve Spring Pressures

by BGordon401@aol.com

10) Scott's Mods

by David Hardy <dhardy@crl.com>

11) Re: 3.42 Gears

by BGordon401@aol.com

12) Re: PCM

by rcheek@one.net (Robert Cheek)

13) Re: IMPALA digest 236

by rcheek@one.net (Robert Cheek)

----------------------------------------------------------------------

Date: Sun, 14 Jan 1996 23:26:04 PST

From: CoolChevy@aol.com

To: impala@east.xsis.xerox.com

Subject: RE: auto lock/unlock features a thiefs dream ?

Message-ID: <960115022604_61696644@mail06.mail.aol.com>

Scott,

Please let me update you about the alarm unit i'm using. I'm an electronic

engineer and i'm of course aware of limitations as per design. I'm like you,

customizing something is one my favoured hobbies.

All your points are true and valid but not for this one, because :

a. the exact function is like : you started the engine (or turned the

ignition on) AND within 10 seconds all doors (hood and trunk too) are closed

AND you put the tranny into gear let's (say you actually MOVE the car -

motion sensor) THAN it will lock the doors automatically.

If any condition is missing, (the doors are not closed, hood/trunk open, no

vehicle movement) it will have a timeout and will NOT lock the doors. You

have to do it manually later on, but it will automatically open them for you

no matter what.

b. The thief could rock my car to check this feature out for about a hundreds

times and more and the horn will still sound with about 110dBa (even after

dis-connecting the battery) because the alarm unit is running of a second

battery (NiCad's) mounted somewhere inside the car. The additional alarm horn

is of course not mounted in the engine bay. Believe it or not, but if the

horn is ringing for more than 10 minutes, someone will call the police - at

least in the area where i used to live. Local record - two minutes 34 seconds

after my HiFi sets volumne was turned up to loud ! <g>

c. The starter disabling feature is not disabling the starter in my case it's

simply killing the ignition. So the Thief could (if he could manage to

overcome all other features so far)

crank the engine, but it will not start. I don't think a Thief having a rush

(no patience when the horn is making a hell of noise all the time) will

consider this car worth to be stolen anymore. If he/her decides to come back

at night and to tow it away, the motion sensor is waiting for him and the

remote battery will wait to. How many thiefes have the nerves to tow car

which is blinking the lights and sounding the horn(s) all the time ?

Hopefully not many !

Most of those feature have override switches, and in case of real emergency i

still have my standard GM remote to open the doors because i did not delete

it, i just added to it.

I would not dare to say, that this car can't be stolen but i think i could

make it as difficult as possible. Above locking features, saved my butt

already once during one of those unfortunate occurences encounters with a

Hispano gang in San Francisco.

Scott, many thanks about alerting or better pre-warning "Impala SS club"

members from the typical alarm "sales bull...". Your points are valid and

true and should help a lot of members in enabling them to ask "real"

questions to sales guys before handing over cash to purchase such a alarm

unit. Very often it turned out to be a nightmare for the innocent.

Heinz

------------------------------

Date: Mon, 15 Jan 1996 01:07:00 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: 'vette valvecovers

Message-ID: <960115090659_73145.1566_FHD11-1@CompuServe.COM>

Troy,

>I'm not sure of the alternator placement on the Corvette but the right side

valve cover on the Impala has some relief for the back of the alternator. Do

the plastic valve covers have a similar indentation for alt. clearance? Some

of the aftermarket chrome covers do not and need a slight coaxing

(hammerin).

The 'vette valvecovers do not have any special relief for the alternator

(which is mounted on the other side on the 'vette anyway), and yet they fit

perfectly without any "coaxing" whatsoever. The covers are symmetrical from

side to side, although both of them do have a slightly greater angle on the

front side than the rear, and neither front or rear sides are vertical. They

also seem to have more of a step-in from both the front and rear compared to

the stock stamped steel units. They do fit perfectly, and they are more

functional and look far better than the stock stamped steel covers.

If one were not going to do the roller rocker mod, then I most definitely

recommend the 'vette covers as they provide much better oiling to the stock

stamped steel rocker arms. The 'vette valvecover internal oil drippers are

strategically positioned to drip oil directly on to the rocker tip as well

as the pivot ball area. The stock rockers have no such added oiling

provisions.

Note that these drippers are of course totally unnecessary with the roller

rockers, in fact they are in the way physically and will have to be cut off

when installing the roller rockers. I was able to cut them off easily with a

dremel tool using a small cutoff wheel. The roller rockers feature needle

bearings on the fulcrum and the valve tip, which are both lubed directly

with pressurized oil through the pushrods. Scott.

------------------------------

Date: Mon, 15 Jan 1996 03:35:58 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 236

Message-ID: <Chameleon.960115063930.hollenbe@mustang.east.xsis.xerox.com>

On Sun, 14 Jan 96 12:39:35 EST  impala@east.xsis.xerox.com wrote:

>Date: Sat, 13 Jan 1996 17:11:28 PST

>From: Don Hurter <don@sirius.com>

>To: impala@east.xsis.xerox.com

>Subject: Re: IMPALA digest  - administrivia

>

>At the end of each mailing is the following:

>

>------------------------------

>

>End of IMPALA Digest XXX

>************************

>

>   Can the list moderator tack on the URL for Dave Hardy's Impala SS page such as:

>

>End of IMPALA Digest XXX

>************************

>for more Impala info, try

>http://www.crl.com/~impala/

>

>   This will allow new list members to find more information, or else

>quickly jump to the earlier list archives.

No can do.  That trailer is hard coded in the listproc software, and if I put any

other text there it would show up in the digest trailer for the other list I

administer.

I do point out the web and ftp sites to all new subscribers with their subscription

confirmation message, but that and the monthly how-to are about as good as I can

get.

==================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street, Lower Lobby, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

==================================================

------------------------------

Date: Mon, 15 Jan 1996 04:50:36 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (Impala1)

Subject: FW: JEEPERS CREEPERS

Message-ID: <1996Jan15.074100.1049.71309@rinet.hunton.com>

Scott Mueller wrote:

The Jeep 16" alloy wheels actually may weigh less than the monster 17"

Impala wheels, and likewise the tires. In any case, the differences here are

inconsequential.

>Also the circumference of the tires, depending on the option, is much

larger than the impala, which counteracts the effect of the 3.73 axle

ratio.

Yes, the Jeep tires are larger. Specifically, they are 28.40 inches in

diameter compared to the 27.04 inch diameter Impala tires. This is a

difference of only 5%. For comparison, the rear axle gear ratios differ by

17.4%.

I really am not taking this too seriously, but I think it is comparing

apples and oranges to compare the tire circumference 5% "size factor"  to

the 17.4% effect of the different axle ratios.   I agree that there is a

linear relationship between the torque multiplying effect of a 3.73 and a

3.08 rear axle, all other factors being the same.   The same cannot be said,

however, for ultimate effect on delivered torque of the difference in tire

sizes.  The reason for this is that the rotational inertia of a wheel/tire

varies with the square of the diameter of the tire.  There is not a linear

relationship between diameter and moment of inertia.

All other factors being the same, a 28.40 inch diameter tire *does* reduce

the torque multiplier effect of the rear axle by 5%, based on the size

difference alone.  However, there are at least two other significant

factors.  A 28.40 wheel/tire has a 1.05 squared, or 10%, higher rotational

moment of inertia than does a 27.04 inch diameter wheel/tire.  This

difference would also vary -- although linearly -- with any difference in

the mass of the wheel/tire.  Therefore, the ultimate effect of the bigger

diameter but maybe lighter Jeep wheel/tires is probably greater than 5%,

although I am not going to wade into the Serbonian Bog of attempting to make

the precise calculations.  I doubt, however, that it is an inconsequential

difference.

In the world of bicycling, substituting lighter wheels of a virtually

identical size but much lighter weight makes a dramatic difference in the

acceleration.  I think the same principle applies here.

------------------------------

Date: Mon, 15 Jan 1996 05:54:14 PST

From: "David Spitz" <david_spitz@vnet.ibm.com>

To: impala@east.xsis.xerox.com

Subject: Colored wax?

Message-ID: <96Jan15.055946pst."15078(15)"@alpha.xerox.com>

I'd like to wax my car... is a standard wax (like Maguire's) the best thing

to use on my black Impala, or would a colored (i.e., black) wax cause fewer

problems? I don't want a bunch of hard-to-remove wax stuck in various seams

(which could happen if I use a non-colored wax), but I also don't want to

hurt the paint job with an inferior paste (as some of the colored pastes are).

Thanks for any input,

David

------------------------------

Date: Mon, 15 Jan 1996 06:11:32 PST

From: John_P._Kelly@troubbs.org (John P. Kelly)

To: impala@east.xsis.xerox.com

Cc: rcheek@one.net

Subject: Re: Beartracker Install

Message-ID: <958914463.259712469@troubbs.org>

Rob,

This may not be an option for you, but about the same time I installed my

Beartracker, I was also installing a 3 watt/hands free adapter for my

flipphone.  I found a black spiral "cellular type" antenna that was a little

longer than my real cell antenna.  I placed them on the outside corners of

the rear window and all is well.

I put a coax "t" in the line so I can hook up a cb at a later time.  Radio

Shack was my source.

JPK

- John P. Kelly

--

For helpful Mac tips and tricks; useful files; product reviews; late breaking Mac news; and more, you're welcome to visit the Chicago Area Macintosh Area Users' Group at "http://www.cmug.org/cmug/"

------------------------------

Date: Mon, 15 Jan 1996 07:49:29 PST

From: Mfetto@aol.com

To: impala@east.xsis.xerox.com

Subject: Re:Beartracker Install

Message-ID: <960115104924_61803203@mail06.mail.aol.com>

Robert:

You will be able to get at the antenna connection for the radio via the glove

compartment. Drop the entire unit by removing the hex screws under the dash

(identical  for access to the wiring harness for the oil pressure mod).

It is tight back there, but you can now gain access to the connector.  NOTE:

The connector is the GM version of the Motorola plug - it is physically

smaller. The coupler (or Y connector) that you use will have to have a GM

male (to radio), GM female (to antenna), and whatever connector you are using

to get to the scanner.

NOTE 2: I'm not sure what you are using as a coupler, but if it is a straight

Y cable then what is being connected is a 75 ohm antenna input (radio) to a

75 ohm antenna lead to a 50 ohm antenna input (scanner). This is going to

result in an impedance mismatch which will degrade receive performance on

both units - particularly the A.M. reception.

I'm not sure how much performance you are looking for, or what bands you want

to receive on the scanner, but I have a simple interior rear deck-mounted

antenna for 460 MHz operation that operates well and is very un-obtrusive.

Let me know how the simple splitter operates...if it is a NOGO, then there

are other ways to get a decent receive.

------------------------------

Date: Mon, 15 Jan 1996 10:05:41 PST

From: John Pinkerton <jpinkert@sigg.com>

To: impala@east.xsis.xerox.com

Subject: BearTracker Antennas - Reply

Message-ID: <96Jan15.113934cst.26497@firewall.sigg.com>

Robert,

I have two seperate scanners in my car.  One is using glass mounted

antenna and the other is using a external antenna.

I did not use the car antenna because I want to be more precise on the

matching the antenna to the frequencies I was scanning for.

John

------------------------------

Date: Mon, 15 Jan 1996 09:51:36 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Cc: 73145.1566@compuserve.com

Subject: Re: Valve Spring Pressures

Message-ID: <960115125135_117125385@mail02.mail.aol.com>

Scott, you wrote:

>I also found that the keeper groove on the intake valve was higher than that

on the exhaust, such that the raised height keepers on the intake valves

would come only .008 inches from the washers on the rockers, which is not

enough clearance. So I had to grind .040 from the top of each of the keepers

used on intake valves for proper clearance.

This would mean 8 keepers had to be ground for the intake valves, right?

You then stated:

>The machined offset keepers are about $30, and it took me a few minutes to

grind .040 inches off of 16 of them for the intake valves.

Did you mean to say 8 were ground down .040 inches for the intake valves, or

did you find all 16 had to be ground because of the intake valves.

Since you had to grind the keepers, do the pressures below still apply.  I'm

under the assumption that with the keepers ground, the height is 1.730 with a

closed pressure of 102, which is right on target for the LT4, but a bit on

the high side for when the valve is open.  Will  the added 38 lbs of pressure

cause any problems that you can see?

1.695       1.730

height        height

=========================

Valve Closed  117     102

Valve Open    313     298

THANKS AGAIN for your help and advise.

.. Bruce / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Mon, 15 Jan 1996 10:34:28 PST

From: David Hardy <dhardy@crl.com>

To: impala@east.xsis.xerox.com

Subject: Scott's Mods

Message-ID: <Pine.SUN.3.91.960115103332.23372A-100000@crl12.crl.com>

Hey Scott...

What about the Night Vision Camera and Heads Up Display...

I want this mod report.....

I'm serious...

-Dave

dhardy@crl.com  %%%%%%%%%%%%%%%%%%%%%%%%%%%%%

"The thief is sorry he is to be hanged,

not that he is a thief" (Thomas Fuller, 1732)

------------------------------

Date: Mon, 15 Jan 1996 11:49:05 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Cc: 73145.1566@compuserve.com

Subject: Re: 3.42 Gears

Message-ID: <960115144905_117232556@mail02.mail.aol.com>

Scott, you wote:

>You may also need carrier side bearings

like I did (mine did not look great when I removed them with only 10K miles,

but that is not normal.

My dealer likes to "fool" everybody with higher than retail prices. For

instance, when I ordered the 9C1 front air deflectors for the brakes, they

wanted to charge over $4.00/ea.  Could you supply the p/n's for these parts

and the price YOU got for them, since I will be trying to make a stong

argument with my dealer for fair pricing.

BTW, what find of discount does your dealer give you and what's the bet way

to approach dealers to get this price break.

Thanks again for all the info you posted on the gears.  I will be going soon

to the dealership for them to do this job (once the tax returns are done).

More ammo you can proved, so all of us don't get "hosed" with the prices,

the better.

..Bruce / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Mon, 15 Jan 1996 17:35:09 PST

From: rcheek@one.net (Robert Cheek)

To: impala@east.xsis.xerox.com

Subject: Re: PCM

Message-ID: <199601160110.UAA23903@mail.one.net>

>Given the ratings I've seen in a couple of tests of JET's PCM, and

>admitting that they may be inflated figures since the test was run on JET's

>own dyno, and given that JET's pcm is cheaper than the Hypertech

>programmer, can you achieve the same results using JET as you can with

>Hypertech with regard to fan turn-on temperatures, rear gear swaps, shift

>points & firmness, etc.?

I was ready to buy the JET. It was only $250 from Summit vs. $350 for

the H-Tech. I read through the install in the January Car Craft. I then

looked at the connector on the PCM that it plugged into. There are indeed

some transmission control lines that it can control, but there is no way

it can compensate for rear gear changes or turn on the fans. The fan module

is separate (and more $$$, making it closer to the H-Tech). You could have

the dealer reprogram for the gears, though, and still use the JET. If it

gives you the performance they claim, I say go for it. I am skeptical.

BTW, I tend to believe the person who said that all the JET device did

to increase performance was to lower the IAT input to the PCM. Not that

I'm down on them for it, but the only things they have access to on the

plug that they go into are _inputs_ to the engine control, not _outputs_.

This sort of ruined there reputation for me since we've all seen the

president claim that they did _not_ do this publicly. I just don't trust

the device after that.

I know of at least one F-body owner (who is running very low 12's) that

has put a potentiometer on his IAT sensor. Anyone wanna try this instead

of the JET ;-)?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers + "H" Pipe

------------------------------

Date: Mon, 15 Jan 1996 17:35:01 PST

From: rcheek@one.net (Robert Cheek)

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 236

Message-ID: <199601160109.UAA23893@mail.one.net>

>The only thing I am concerned with, is that I do mostly highway driving, and

>have been

>reading about an annoying drone or note resonating thru the car at highway

>speeds.

>Is this your experience also, or does it have to do with where the H pipe is

>located

>in the exhaust system?

It seems like everyone has this drone, but just to varying levels of

loudness. I do, and I just changed the mufflers.  It's not bad, in fact

I like to keep it in that range 'cuz it sounds good there. I'm thinking

that the engine / exhaust manifolds just are particularly loud in that

range and the drone will be in direct proportion to the loudness of the

mufflers (or lack thereof ;-) that are on the car. Just something to think

about when switching mufflers...

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers + "H" Pipe

------------------------------

End of IMPALA Digest 238

************************

IMPALA Digest 239

Topics covered in this issue include:

1) Oil Sending Unit Resistor

by BGordon401@aol.com

2) New SS Owner

by Erda@aol.com

3) MSD ignition

by Scott Mueller <73145.1566@compuserve.com>

4) Rear Gears

by Scott Mueller <73145.1566@compuserve.com>

5) valvesprings

by Scott Mueller <73145.1566@compuserve.com>

6) Spark plugs

by Scott Mueller <73145.1566@compuserve.com>

7) Night Vision HUD

by Scott Mueller <73145.1566@compuserve.com>

8) Tire size difference

by Scott Mueller <73145.1566@compuserve.com>

9) Re: Colored wax?

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

10) Adj. Fuel Pressure reg's, MSD ignition

by stc@cypress.com (Steven Chapple / Field Sales Engineer)

11) List prices

by "Rush Luangsuwan" <rush@mcs.com>

12) Home Plate Stopper-Upper

by Erich Arndt <EARNDT@mednet.swmed.edu>

13) Mileage

by daryl@austin.ibm.com (Daryl Green)

14) Removal of Square Resonator

by Stewart McLellan <stewartm@magi.com>

15) No mufflers!

by Franklin Poole <franklin@intersurf.com>

----------------------------------------------------------------------

Date: Mon, 15 Jan 1996 21:50:27 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Subject: Oil Sending Unit Resistor

Message-ID: <960115234018_43130842@emout06.mail.aol.com>

Back in digest 79 from 12 Sept 95, I noticed this message concerning the

resistor for the oil sending unit.  To date, I have removed no less than 2

feet of tape from the harness behind the glove box, but have not yet found

the sucker.

The archive message stated:

>> The resistor is taped into the harness directly behind the glove box

area.> If you rotate the glove box forward (you should not have to remove

it), you> can access the main instrument panel harness. Carefully cut the

tape wrap,> and you should find the resistor soldered into the tan oil

pressure sending> unit wire. All you have to do is to clip one end of the

resistor, and solder> the two wire ends together. You can leave the resistor

there, but it will> not be a part of the circuit. Then tape it all back up

and you are done!> Thanks to Troy for finding the location of this sucker, I

gave up and ran a> separate wire myself! Scott

>Actually.....if you just clip the black wire to the resistor, you're done.

I did not even have to untape the whole thing.

>-Dave

Does this "fix" really work that easily??  If I continue to have no luck

finding the resistor, all I need to do is snip the black wire??  Anybody who

has done this care to give an opinion???

Bruce Gordon / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Mon, 15 Jan 1996 22:54:57 PST

From: Erda@aol.com

To: impala@east.xsis.xerox.com

Subject: New SS Owner

Message-ID: <960116005426_43196450@emout06.mail.aol.com>

Just found this group last week and I am so glad.  I had been eyeing the

Impala SS for many months and somehow a '96 Black Beauty was in the driveway

XMas morning.  Until now I havn't been able to find out squat outside of the

owners manual.  I'm green when it comes to performance mods and will

appreciate anything I can pickup re maintenace, problems, appearance &

keeping hands off.  For now I just drive around the block - real slow. Name

is Tom Woods and I'm in Chicago (South Side) also -

erda@aol.com

------------------------------

Date: Tue, 16 Jan 1996 00:59:23 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: MSD ignition

Message-ID: <960116085923_73145.1566_FHD26-4@CompuServe.COM>

Steve,

>Do you have any experience with or knowledge about adjustable fuel pressure

regulators and MSD ignition boxes for the Impala SS?

I have considered both mods, but am somewhat skeptical of them without real

dyno testing. I know that an adjustable regulator can help alot on

turbocharged engines like the Buick GN, but I'm not so sure it would really

help at all with the LT1. I feel the same about the ignition. It may help if

you were bolting on a supercharger, but on the stock LT1 I don't think

either higher fuel pressure or an MSD box will help much. In fact higher

fuel pressure will likely just make the car run too rich.

I think both mods (fuel and spark) are better done by modifying the PCM

program. You may want more fuel at WOT, and a little more spark advance all

around.

Now if anybody has any real testing that can verify an improvement from

these mods, I would be interested to hear! Scott.

------------------------------

Date: Tue, 16 Jan 1996 00:59:42 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Rear Gears

Message-ID: <960116085941_73145.1566_FHD26-8@CompuServe.COM>

>My dealer likes to "fool" everybody with higher than retail prices. For

instance, when I ordered the 9C1 front air deflectors for the brakes, they

wanted to charge over $4.00/ea.  Could you supply the p/n's for these parts

and the price YOU got for them, since I will be trying to make a stong

argument with my dealer for fair pricing.

I posted the part numbers and prices I paid in my previous message to the

list. If you have any questions on a specific item, just ask!

>what find of discount does your dealer give you and what's the bet way to

approach dealers to get this price break.

I get most things at standard trade (wholesale) pricing, except anything

that costs about $100 or more I get for 10% over dealer cost. That is the

lowest they are normally allowed to go. If you are purchasing a large number

of parts, I always recommend asking for the trade (wholesale) pricing, and

you will normally get it with no hassle. Scott.

------------------------------

Date: Tue, 16 Jan 1996 00:59:32 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: valvesprings

Message-ID: <960116085931_73145.1566_FHD26-6@CompuServe.COM>

Bruce,

>>The machined offset keepers are about $30, and it took me a few minutes to

grind .040 inches off of 16 of them for the intake valves.

>Did you mean to say 8 were ground down .040 inches for the intake valves,

or did you find all 16 had to be ground because of the intake valves.

Keepers are used in pairs for each valve, so there are 32 total, and 16 of

them were ground down to be used on the intake valves.

>Since you had to grind the keepers, do the pressures below still apply.

I'm under the assumption that with the keepers ground, the height is 1.730

with a closed pressure of 102, which is right on target for the LT4, but a

bit on the high side for when the valve is open. Will the added 38 lbs of

pressure cause any problems that you can see?

I don't think that the added 32 lbs over the LT4 spec will cause any

appreciable problems, especiallys since our heads are stronger cast-iron,

and we do not have the lighter LT4 hollow valves. With the heavier valves, a

little more pressure would likely be required to achieve the same results as

with the LT4. Scott.

------------------------------

Date: Tue, 16 Jan 1996 00:59:37 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Spark plugs

Message-ID: <960116085937_73145.1566_FHD26-7@CompuServe.COM>

Frank,

>I am on the Impals SS list but have and race a Monte Carlo SS. I am

interested in the 41-943 plugs you speak of. I assume they are AC's, no?

Could you fill me on them? Will they fit my L-69's heads? Did they change

the thread size on the LT1's?

They are the standard 5/8 tapered seat plugs for the smallblock, but they

are specifically designed for the LT1/LT4 heads. They will screw in to any

other smallblock head, but I don't know if the tip length, heat range, etc.

is correct. Compared to the 41-906 plugs used on the LT1 in '95 and earlier,

the newer 41-943 plugs have more platinum on the tip, the tip area is more

exposed, and they have less resistance for quicker starts. Scott.

------------------------------

Date: Tue, 16 Jan 1996 01:47:27 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Night Vision HUD

Message-ID: <960116094726_73145.1566_FHD28-4@CompuServe.COM>

Dave,

>What about the Night Vision Camera and Heads Up Display...

Yea, the numbers for Huges and Datavision (who market these devices) are

listed in the 9C1 factory brochure. How about you give them a call and give

us the full report! <g> I'd be interested in installing these items as well,

except I think you have to cut up the roof! Scott.

------------------------------

Date: Tue, 16 Jan 1996 01:47:24 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Tire size difference

Message-ID: <960116094724_73145.1566_FHD28-3@CompuServe.COM>

Martin,

>The same cannot be said, however, for ultimate effect on delivered torque

of the difference in tire sizes. The reason for this is that the rotational

inertia of a wheel/tire varies with the square of the diameter of the tire.

Inertia has no effect on torque. If you look over my previous calculations

carefully, you see that I factored the 5% tire size difference in the final

result, and it does not really have that much of an effect compared to the

gearing differences. My point is that the transmission and rear axle gears,

multiplied by the engine torque, determine what final rear wheel torque will

be. A taller or shorter tire can then have a final modifying effect since it

is in essence acting as the final gear connection to the road.

>there are at least two other significant factors. A 28.40 wheel/tire has a

1.05 squared, or 10%, higher rotational moment of inertia than does a 27.04

inch diameter wheel/tire.  This difference would also vary -- although

linearly -- with any difference in the mass of the wheel/tire. Therefore,

the ultimate effect of the bigger diameter but maybe lighter Jeep

wheel/tires is probably greater than 5%, although I am not going to wade

into the Serbonian Bog of attempting to make the precise calculations. I

doubt, however, that it is an inconsequential difference.

If you understand torque, you will also understand that it is a pure force

measurement that is independent of reciprocating mass, so the mass of the

wheel (engine, flywheel, transmission, driveshaft, axles, etc.) is

inconsequential. In fact if you really wanted to discuss reciprocating mass,

you left out quite a few other items that are much more important than the

wheels (engine, flywheel, trans, driveshaft, axles, etc.). Fortunately none

of this matters one bit when it comes to torque measurment and calculations!

<G> Scott.

------------------------------

Date: Tue, 16 Jan 1996 03:27:31 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: impala@east.xsis.xerox.com

Subject: Re: Colored wax?

Message-ID: <Chameleon.960116063522.hollenbe@mustang.east.xsis.xerox.com>

On Mon, 15 Jan 96 22:29:47 EST  impala@east.xsis.xerox.com wrote:

>Date: Mon, 15 Jan 1996 05:54:14 PST

>From: "David Spitz" <david_spitz@vnet.ibm.com>

>Subject: Colored wax?

>

>I'd like to wax my car... is a standard wax (like Maguire's) the best thing

>to use on my black Impala, or would a colored (i.e., black) wax cause fewer

>problems? I don't want a bunch of hard-to-remove wax stuck in various seams

>(which could happen if I use a non-colored wax), but I also don't want to

>hurt the paint job with an inferior paste (as some of the colored pastes are).

I think most of the colored waxes are more of a marketing ploy than they are

superior to "standard" waxes.  Personally, I use a product called Race Glaze,

which I think is one of the best synthetic waxes around.  It's a white liquid

that goes on easy, comes off easy, shines great, and has great durability.

Others on the list swear by other products.

My Impala is dark green gray metallic, but I've also used Race Glaze on a black

Toyota MR2 with similar success.  My own personal experience is that black cars

require more cleaning prep work before you apply the final coat of wax if you

want to maintain showroom shine.

------------------------------

Date: Tue, 16 Jan 1996 05:06:00 PST

From: stc@cypress.com (Steven Chapple / Field Sales Engineer)

To: impala@east.xsis.xerox.com

Cc: stc@cypress.com (Steven Chapple / Field Sales Engineer)

Subject: Adj. Fuel Pressure reg's, MSD ignition

Message-ID: <199601161306.FAA01308@mailb2.cypress.com>

Fellow Impala owners,

I have asked this question of Scott Mueller, "The Eternal Orb of Impala

Knowledge" but thought that someone on this list may have experience

with these mods:

Adjustable fuel pressure regulators and their affects on our LT1 engines.

MSD ignition boxes and affects on the LT1.

Any knowledge about or experience with the above would be greatly

appreciated.

Rgds,

Steve Chapple

stc@cypress.com

Black '95 w/mods

------------------------------

Date: Tue, 16 Jan 1996 06:49:47 PST

From: "Rush Luangsuwan" <rush@mcs.com>

To: impala@east.xsis.xerox.com

Subject: List prices

Message-ID: <m0tcChn-0005yUC@mars.mcs.com>

> ------------------------------

>

> Date: Mon, 15 Jan 1996 11:49:05 PST

> From: BGordon401@aol.com

> To: impala@east.xsis.xerox.com

> Cc: 73145.1566@compuserve.com

> Subject: Re: 3.42 Gears

> Message-ID: <960115144905_117232556@mail02.mail.aol.com>

>

> My dealer likes to "fool" everybody with higher than retail prices. For

> instance, when I ordered the 9C1 front air deflectors for the brakes, they

> wanted to charge over $4.00/ea.  Could you supply the p/n's for these parts

> and the price YOU got for them, since I will be trying to make a stong

> argument with my dealer for fair pricing.

I was under the impression that list pricing was the same among all dealers;

that they pulled it from a common database/price list.  My parts guy told me

that GM had a price increase across their entire line of parts.  I remember

last year the deflectors listing around $3 something, so the price may have

gone up.

>

> BTW, what find of discount does your dealer give you and what's the bet way

> to approach dealers to get this price break.

I just asked and they offered to give me 20% off.  Some other people in this

group have managed 30% off.  Another dealer was adamant in not giving me a

discount, period, no matter how much I argued about not paying list.  I guess

it depends upon the dealer, but I'm not sure exactly how it works.

---

Rush Luangsuwan

rush@mcs.com

'95 Blk SS

------------------------------

Date: Tue, 16 Jan 1996 07:47:00 PST

From: Erich Arndt <EARNDT@mednet.swmed.edu>

To: impala@east.xsis.xerox.com

Subject: Home Plate Stopper-Upper

Message-ID: <s0fb7453.015@MEDNET.SWMED.EDU>

'Mornin Folks!

Got an easy question.  There's been a lot of talk about stopping up the

hole for home plate with Nestea caps and the like.  I took the "Nestea

plunge"; worked great but I didn't like the look.

Does anyone have precise measured dimensions (diameter and depth)

that would be needed to totally fill in the hole?  I'm debating about having

a friend with access to a precision lathe turn a "hocky puck" filler with a

lip on top to hold it into place (the negotiated six-pack is MUCH cheaper

than a 1LE elbow).  Any suggestions on a good, thermally-stable, light,

turnable  material?  Would an epoxy-based casting stand up to daily

abuse?  Would metal like aluminum get too hot to be practical in the air

intake system?  Other ideas????

Thanks,

Erich Arndt

'94 Impala

------------------------------

Date: Tue, 16 Jan 1996 07:57:47 PST

From: daryl@austin.ibm.com (Daryl Green)

To: impala@east.xsis.xerox.com (impala)

Subject: Mileage

Message-ID: <9601161557.AA24976@augusta.austin.ibm.com>

Well, I'm about to hit 4000 miles on my '96 SS and I have noticed a

couple of things that I'd like to ask about.

1) Around town, it seems I get 16 MPG pretty much every time, all the

time.  I've run Citgo 93 octane and (for the last couple thousand

miles) Citgo 89 octane.  No difference in mileage.  Is this pretty

consistent with what others are seeing?  Also, is there any reason

to use the higher octane gas?  Would it result in any noticeable

improvement in power?

BTW, I got around 21 MPG on a 750 mile (Austin-OKC-Austin) highway

trip at an average speed of about 80 or so.  I've made that drive at

least 50 times and I'm here to tell you, these new speed limits make

a tremendous difference.  I'm a happy boy.  What used to be a six

hour drive (with stops) is under 5 1/2 now.  Of course, the Impala

may have more to do with that than the new limits. :-)

2) There is a sort of bass note that emanates from the exhaust system at

1500 RPM.  It sounds like a harmonic hum that just happens to occur

at that RPM rate.  Is this common?  I kind of like it.  If I made any

exhaust mods, I'd like that level of background rumble all the time.

I hope the Arlington gathering comes together.  I'd love to see and hear

some of the mods others have made so I wouldn't have to feel like I was

flying blind when choosing what to do.  I've pretty much decided to get

the Hennessey intake system first and then either exhaust mods or a PCM

chip.  Thank you, Scott, for exposing the JET chip.  I won't be getting

that.  Do you have a preference between the Z Industries and Hypertech

chips?  I'd appreciate any input anyone might have.

Daryl Green

daryl@austin.ibm.com

------------------------------

Date: Tue, 16 Jan 1996 09:09:12 PST

From: Stewart McLellan <stewartm@magi.com>

To: impala@east.xsis.xerox.com

Subject: Removal of Square Resonator

Message-ID: <30FBDBB8.4BEF@magi.com>

Hi Guys:

I was talking to a local plastic fabricator about the possibility of making a

replacement tube for the square resonator.  He did not have any 3.5" OD tubing which was prebent

or any curved pieces which didn't have the joint flanges installed.  What he did suggest was to

use 3" ID, black, ABS plumbing pipe (it's 3.5" OD) of the appropriate length, fill it full of

fine, dry sand to act as a mandrel and then gently heat the pipe in an oven or with a heat gun

until its soft enough to bend to the proper shape to replace the resonator.  Let the pipe cool

and trim the ends to the proper length.  A little of the proper black paint, either flat of semi

flat and the tube should drop into place and not radiate heat like the metal ones do.  I was

specific about questioning the ability of the ABS pipe to withdstand the temperatures under the

hood.  His reponse was that the pipe would be Ok unless the temp. got to well above 250 F.

All this is because I'm in the same boat as Jeff Gatcha.  I can't find a 3" pipe

blockoff at all.  I did find a neat 3" pipe vent.  It has louvers in it.  All you'd have to do

is find a means of sealing the louvers (some expanding foam would probably work) and them paint

it.  What does everyone think, a louvered cap on your intake elbow?

I keep reading about dealing with valve float if you install new rockers.  In a

previous digest I asked if anyone had any experience with a rev kit that fits between the

head and the lifters.  I found out Comp Cams makes it.  By using this setup, you wouldn't have

to worry about too much pressure over the nose collapsing the lifter or too little seat

pressure.  The stock springs would deal with the seat pressure appropriately and the rev kit

springs would keep the valves from floating.  Comments would be appreciated because I'm

considering this approach.

Re: Driving in the Snow on ZR4's - As any one who has lived in the hinter land of the

north, on either side of the border, will tell you, driving in the snow is an acquired SKILL.

The type, size and style of tires you are running will have an impact on your driving but if you

don't know how to drive in these conditions nothing you do will be easy.  Try 8 -10 inches of

heavy snow, in a driving snow storm, in a mid 70's TA shod with old style FR70-15 factory

Uniroyals.  It wasn't fun but I went over 400 miles and made it when 4x4's were in the ditch.

FWIW.

Regards, Stewart McLellan

------------------------------

Date: Tue, 16 Jan 1996 11:25:44 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: No mufflers!

Message-ID: <30FBFBB8.389E@intersurf.com>

Sunday I took the mate, mother-in-law, and two out of three toddlers (my

twins!) on a house hunting trip.  We drove 70 miles all afternoon and

had the windows up and down at different times.  With some careful

application of the throttle I was able to drive the entire day without

the wife or her mother making any comments about the noise level of the

car!  It's still somewhat loud mind you but for the $48 it cost to put

in an H pipe and replace the mufflers with 2" pipe I say it's a

steal!<g>

Tip of the day-----------------

While washing T-Rex on Saturday I noticed the trunk weatherstripping.

Just like on the Firebird only worse.  On my Firebird the weatherstrip

around that monster rear glass was glued together where the ends meet.

The ends meet right at the power deck release. They came apart and water

leaked into the car.  When I replaced it I put it in "upside down", or,

put the seam up at the top.  I also put silicone into the open channels

so that water couldn't get in and fill the gasket...

On T-Rex (the Impala) the gasket isn't even glued together.  Only held

with a couple of plastic inserts.  Water from the washing operation had

already worked around that seam and into the trunk (not much but any is

too much)

So what I did was remove the trunk weatherstrip (it's just a force fit

with a nominal amount of adhesive) and put the seam at the top.  I

filled the first inch or so of the seams with a new type of soft

silicone I found at Home Depot (in paint dept) and used superglue to

hold the seams together. Since there's little stress on the gasket it

should hold for a quite a while.  Meanwhile I've stopped a potential

leak into the trunk.

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

End of IMPALA Digest 239

************************

IMPALA Digest 240

Topics covered in this issue include:

1) gas mileage

by "David Spitz" <david_spitz@vnet.ibm.com>

2) Re: IMPALA digest 239

by Steve Das <sdas@sat.net>

3) Re: BearTracker install

by rcheek@one.net (Robert Cheek)

4) Re:Rear Gears and Valve Springs

by BGordon401@aol.com

5) Impalafest update

by Rock528@aol.com

6) Re: scanners

by Rock528@aol.com

7) elbow block off

by Rock528@aol.com

8) 6N1 door lock option

by Rock528@aol.com

9) Gas mileage

by Franklin Poole <franklin@intersurf.com>

10) Jeep Grand Cherokee

by "J.R. LEVITZ" <4everGreen@msn.com>

11) oil pressure sender mod

by jsalmon@mindspring.com (John Salmon)

12) Fuel press. reg.

by Scott Mueller <73145.1566@compuserve.com>

13) 6N1 Lock mod

by Scott Mueller <73145.1566@compuserve.com>

14) Misc. questions

by Scott Mueller <73145.1566@compuserve.com>

----------------------------------------------------------------------

Date: Tue, 16 Jan 1996 12:44:09 PST

From: "David Spitz" <david_spitz@vnet.ibm.com>

To: impala@east.xsis.xerox.com

Subject: gas mileage

Message-ID: <96Jan16.125553pst."16756(11)"@alpha.xerox.com>

Daryl,

The octane issue was discussed a bit a few months back. I think the

consensus was that you don't gain much by using a higher octane than

that which doesn't knock. So, if you don't knock at 89, don't bother

with 93. Of course, I suppose others will disagree and say that the

computer will reprogram itself for better advance with higher octane,

but I'd be suprised if you noticed anything other than a higher gas

bill. Probably the most important thing is to stick with one octane,

whichever way you go. Engines get "used" to a particular octane's

characteristics, I believe, and switching isn't a particularly good

thing to do.

Interesting what you say about gas mileage, though. I have found

that, no matter how I drive (fast/slow, city/highway), that I almost

never deviate from between 19mpg and 21mpg, with 400 miles on about

20 gallons (20mpg) being almost always what I get. In fact, when I

drive between Raleigh and New York or DC on cruise at 65-70 mph, I

always fill up at the same spots!

Needless to say, I was a little disappointed that the 25mpg EPA

rating on the sticker when I bought the car seems to be something

beyond the reach of my particular car. Well, not THAT disappointed.

It's worth kissing those Lexuses (Lexi?) and Infinity's goodbye!! :)

Regards,

David

------------------------------

Date: Tue, 16 Jan 1996 13:54:22 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 239

Message-ID: <199601162154.PAA03290@raven.sat.net>

Bruce Gordon wrote........

>>Actually.....if you just clip the black wire to the resistor, you're done.

> I did not even have to untape the whole thing.

>

>>-Dave

>

>

>Does this "fix" really work that easily??  If I continue to have no luck

>finding the resistor, all I need to do is snip the black wire??  Anybody who

>has done this care to give an opinion???

Yup, it's that easy.  As I recall, one end of the resistoe has two tan wires

connected and the other end has the black ground wire.  Clipping the black

wire gets it done.  Then if you want the gauge to read higher, go with the

120 ohm resistor in the place described in the later text.  I did this (in

fact, I developed it with the help of Gary Martin) and it works great.

Steve

------------------------------

Date: Tue, 16 Jan 1996 17:36:23 PST

From: rcheek@one.net (Robert Cheek)

To: impala@east.xsis.xerox.com

Subject: Re: BearTracker install

Message-ID: <199601170127.UAA13247@mail.one.net>

>You will be able to get at the antenna connection for the radio via the glove

>compartment. Drop the entire unit by removing the hex screws under the dash

>(identical  for access to the wiring harness for the oil pressure mod).

Will try that, and maybe finally get at that oil pressure mod too ;-)

Thanks much...

>NOTE 2: I'm not sure what you are using as a coupler, but if it is a straight

>Y cable then what is being connected is a 75 ohm antenna input (radio) to a

>75 ohm antenna lead to a 50 ohm antenna input (scanner). This is going to

>result in an impedance mismatch which will degrade receive performance on

>both units - particularly the A.M. reception.

The cable I have came with the BearTracker. Will see how well it works. I

really don't like additional antennas on the car: I'd rather shelf the

BearTracker than do that.

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers + "H" Pipe

------------------------------

Date: Tue, 16 Jan 1996 17:38:27 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Cc: 73145.1566@compuserve.com

Subject: Re:Rear Gears and Valve Springs

Message-ID: <960116203557_43991923@emout04.mail.aol.com>

Scott, you wrote:

>I posted the part numbers and prices I paid in my previous message to the

list. If you have any questions on a specific item, just ask!

You wrote previously in the digest w/ the part #:

> You may also need carrier side bearings like I did (mine did not look great

when I removed them with only 10K miles, but that is not normal.

The p/n and price on the carrier side bearings was what I meant to ask for.

If I neglected to do so, I apologise.  In the digest you wrote concerning

the rear gears, you mentioned front and rear pinion bearings.  I the event

that the carrier side bearings are in disrepair, it would be great to have an

idea of what price to negotiate.

>Keepers are used in pairs for each valve, so there are 32 total, and 16 of

them were ground down to be used on the intake valves.

Thanks for the clarification!!!

..Bruce / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Tue, 16 Jan 1996 18:43:06 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Impalafest update

Message-ID: <960116213837_62885283@mail02.mail.aol.com>

Just a brief update:

1.  The legal formalities of the assoc. are coming along.  Snow days in

Washington have slowed down the trademark info.

2.  Waiting on confirmation for the factory tour in Arlington from Don.

3.  Super Chevy is interested in us as a group also.  This will work nicely

at the Texas location as well as other spots throughout the country.  More

info as it is received.

4.  Need to find out actually how many people are interested in going to

Arlington/Dallas in April and how many will be driving.  I would hate for the

whole impala group to show up and show off a bunch of rental car mustangs.

<g>

email my impalaclub@aol.com address with your intentions for April.

Thank you.

Troy Willrick

Natl. Assoc. 94-6 Impala SS Owners

------------------------------

Date: Tue, 16 Jan 1996 18:44:53 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: scanners

Message-ID: <960116213854_62885536@emout04.mail.aol.com>

In a message dated 96-01-15 22:32:15 EST, you write:

>I have two seperate scanners in my car.  One is using glass mounted

>antenna and the other is using a external antenna.

just curious, but what are you guys listening to?   Troy

------------------------------

Date: Tue, 16 Jan 1996 18:45:24 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: elbow block off

Message-ID: <960116213830_62885190@mail02.mail.aol.com>

>Well, Ive finally gotten around to removing home Plate.  However I'm =

>finding it nearly impossible to find a 3" knockout made of aluminum (or =

>any metal for that matter).

Jeff, this has been on the list before but still a cosmetically acceptable

alternative..

In the interim of getting the right elbow, go to Home Depot in the gas hot

water heater section.  Get a 3" Tee Cap for about 2 bucks.  These are near

the gas hot water heater vent pipes.  It is the perfect size and it's

aluminum.  Be careful not to paint the surface that the clamp grips or else

will squeeze it out.  It even pays to rough it up there because the alum. is

slick.  Any air leak here will cause the engine to run rough.  A little

semigloss black matches nicely.  If you can't find one let me know.  I have a

few extras.  Troy

------------------------------

Date: Tue, 16 Jan 1996 18:44:51 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com, 73145.1566@compuserve.com

Subject: 6N1 door lock option

Message-ID: <960116213850_62885484@emout04.mail.aol.com>

Scott Mueller,

I've checked the 6N1 door opening parts list and secured the proper part

number for the lock rod.  I also noticed that there are a few more parts with

the 6N1 code attached, one being the door lock "module" for the '91-3 9C1.

Have you done this mod and are more parts needed to convert a non 6N1 door

lock than just the rod?  Thanx, Troy

------------------------------

Date: Tue, 16 Jan 1996 20:58:53 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Gas mileage

Message-ID: <30FC820D.24F@intersurf.com>

Daryl Green posted yesterday about his gas mileage. I've gotten a

consistent 14.75mpg over the total 2025 miles I've driven my car.  My

one "trip" netted me a 19mpg reading (only 45 miles one way though).

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Tue, 16 Jan 1996 20:58:52 PST

From: "J.R. LEVITZ" <4everGreen@msn.com>

To: "Impala List" <impala@east.xsis.xerox.com>

Subject: Jeep Grand Cherokee

Message-ID: <UPMAIL01.199601170458250939@msn.com>

There has been alot of talk about the Jeep Grand Cherokee. This is a very nice

vehicle.I would even purchase one. But, I do believe there is now chance in

_ell that it will run with an Impala.

If anyone ever gets beat by one:  1) check snow depth, 2) check for

supercharger, 3) check for nitrous bottle !!!!

RE: Discount parts pricing at dealership. Try giving them a reason to give

you a discount or wholesale pricing. Inform them that "just like a wholesale

account" you intend to make multiple purchases over a period of time.

Typically you know the part #'s. And that you can think of at least $500-1000

worth of parts you want in the near future.

If you stand on their side of the counter there are many reasons NOT to

give you a discount. For example, if you give them the impession that you are

going to "drill" them for twenty minutes over a $6 part, or, that you are

going to make them "jump through hoops" to complete the sale. Then certainly

you will not get your wholesale prices. After all, why should they work that

hard AND give you a discount when normally a wholesale phone call only lasts

30 seconds and nets them a volume purchase? Think about it.

Regards John

------------------------------

Date: Tue, 16 Jan 1996 21:03:55 PST

From: jsalmon@mindspring.com (John Salmon)

To: bgordon401@aol.com, impala@east.xsis.xerox.com

Subject: oil pressure sender mod

Message-ID: <v01530500ad21dd744028@[168.121.55.70]>

>Does this "fix" really work that easily??  If I continue to have no luck

>finding the resistor, all I need to do is snip the black wire??  Anybody who

>has done this care to give an opinion???

>Bruce Gordon / BGORDON401@aol.com

>1995 Cherry Metallic

Sorry you've had trouble with the mod, Bruce.  I wrote up some of the

instructions for it based on my experience and a hunch that most Impalas

would have used very similar wiring harnesses.  Obviously that isn't the

case.  All you do have to do to achieve the proper results is eliminate the

path to ground from the gauge through the resistor.  You can do this by

cutting the black/white stripe wire anywhere along its length since it only

runs from the resistor to the ground point in the passenger footwell.  I

would be cautious though, since in my car there are many black wires

converging at that point.  If you can somehow trace the tan wire from the

sender back through the firewall from under the hood, then check the

resistance from that wire back to the various wires at the grounding point

you can find the right wire by looking for the one with very close to 68

ohms resistance.  The same would be true from the other side (the tan wire

at the gauge itself) since the path to ground would go through the

resistor.

I'm no electrical engineer, so anybody who can make sure this is right care

to comment?  This is my best guess.

Obviously, YMMV on this mod...sorry about that. :-)

John Salmon

Data Warehousing Technical Support

personal: jsalmon@mindspring.com

work: john.salmon@bridge.bellsouth.com

'95 Dark Cherry Impala SS -- The SALMN8R

------------------------------

Date: Tue, 16 Jan 1996 22:09:12 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Fuel press. reg.

Message-ID: <960117060911_73145.1566_FHD49-10@CompuServe.COM>

Steve,

>I have been talking to a company here in Orlando called Dyno/Tune who's

main business is to maximize HP through tuning (i.e. fuel, pcm, etc. I

guess, they won't exactly say) and verify on a Dynojet Chassis Dynamometer.

They are supposed to be pretty reputable in getting significant HP gains out

of cars that have been "modified" by the owners (i.e. Hypertechs, exhaust,

intakes, etc).

>They recommended the adj regulator for the LT1 indicating that it is does

benefit from a little higher fuel pressure.

I would be very interested in hearing how this turns out. If it works, it is

indeed an easy and inexpensive mod. Please keep me informed! Scott.

------------------------------

Date: Tue, 16 Jan 1996 22:10:57 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: 6N1 Lock mod

Message-ID: <960117061056_73145.1566_FHD49-16@CompuServe.COM>

Troy,

>I've checked the 6N1 door opening parts list and secured the proper part

number for the lock rod.  I also noticed that there are a few more parts

with the 6N1 code attached, one being the door lock "module" for the '91-3

9C1. Have you done this mod and are more parts needed to convert a non 6N1

door lock than just the rod?

Well, my parts are in at the dealer, but I am teaching this week, and will

be gone in Boston teaching for the two weeks following. I will probably pick

up the parts this weekend, but unfortunately may not be able to get to the

mod until the second week in Feb. I will be working to get the car ready for

the big Chevy Vettefest on Feb. 24-25, plus we have the big Chicago

International (new car) show the week before. I plan on attending that one

several times to talk with some of the Arlington folks. Scott.

BTW, are there any Impala list members in the Boston area who would like to

hook up? I will be staying at the Le Meridian Hotel downtown on Franklin

street from Jan. 22 through Feb. 2, and will be working in the Nynex

building across the street for two weeks! Scott.

------------------------------

Date: Tue, 16 Jan 1996 22:09:47 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Misc. questions

Message-ID: <960117060946_73145.1566_FHD49-13@CompuServe.COM>

Rodger,

>Did you finish this?  Is it truly only a rod?  I have this in my 9C1 in the

driver side and was wondering about putting it in the passenger side.

If you want it on the passenger side, you will have to custom bend your own

rod, as it is only an SEO option for the driver's side. The 6N1 and other

parts are in at my dealer, unfortunately I am teaching for Unisys this week,

and won't be picking anything up until Friday or Saturday. I will be

teaching for two weeks in Boston starting Jan. 22nd, so I probably won't get

it installed until the beginning of February. I keep on forgetting that I

have to work sometimes to pay for all of my Impala parts! <G>

>Sounds like an aftermarket shifter would also work well.  Any inputs as to

quality of the OEM unit versus a good aftermarket one (Hurst and B&M come to

mind as the popular choices; any recommendations there?)

An aftermarket shifter would indeed work, but that is most definitely NOT my

style. I would never consider anything but the factory shifter myself!

>>I will actually be prototyping the Camaro cluster install very soon! Mike

Sundstrom has graciously consented to bringing over his father's new Camaro

for us to take apart. <g> We will be removing the cluster and doing a

prototype fitting in my Impala. This will give us an immediate pass/fail

indication of weather this is truely feasable or not. I have to do this to

prove if it can be done or not, so I can stop daydreaming about it! <g>

>When do you think that you'll get to this?  I am curious about the outcome

as I am already planning to get a '95 9C1 and this would be a definite mod

(my '91 9C1 has really spoiled me).

Yea, I know what you mean, the '91 has a killer instrument cluster, blows

the Impala away! Well, Mike and I did the prototype fitting, and indeed the

Camaro cluster WILL FIT in the Impala with a little work. Most importantly

the face of the cluster is almost a PERFECT match with the black surrounding

cluster bezel on the Impala. In other words, it will fit behind the stock

Impala cluster opening as if it was factory installed!

Now, before you get too excited <g>, there are several issues that must be

considered:

1.) Don't even consider the Camaro cluster unless you already have the floor

shifter. On a '96, obviously this is a given, but on an earlier '94-'95, you

must install the floor shifter first, or along with the Camaro cluster. The

Camaro cluster has no gear position indicator, which is on the '96 floor

console just as it is in the Camaro.

2.) The Camaro cluster lists for $750, I can get it brand new for just over

$500. You will have to cut the non-visible portion of the cluster quite

extensively as although the front part of the cluster is the right size, the

rear mounting area is too wide. All that will be cutoff is the white plastic

backing material which cannot be seen from the front. This will not affect

the circuitry or the face of the cluster in any way, but of course it will

be useless to a Camaro owner after you are done with it!

You must cut off all of the mounting tabs, cut the rear mounting section off

at the sides, and fabricate simple angle attaching brackets to mount it into

the Impala opening. You will also have to slightly relieve the area behind

and to the left of the cluster in the Impala cluster opening. These mods

will not prevent you from ever installing the Impala's original cluster

again, should you ever desire.

3.) An adapter harness will have to be constructed which mates the Impala

cluster connectors to the single Camaro cluster connector. This will consist

of a bundle of wires about a foot long, with the proper pins on each wire to

plug into the respective connectors. You then use the service manual wiring

diagrams to jumper the appropriate Impala wire to the appropriate Camaro

cluster wire.

4.) Although the Camaro cluster has much better gauges than the Impala, it

does lack a few features that the Impala cluster has. For example, the

Camaro cluster does not have the "Change Oil", or "Low Fuel" circuits (which

are contained in the Impala cluster itself), so those functions will be lost

completely. The change oil thing is not a problem for me as I have never

seen it light anyway, nor have I let the fuel go low enough to light the low

fuel light either. Personally, I would not miss those indicators at all!

The Camaro cluster also does not have the "Low Washer Fluid" indicator, but

it does have a "Low Trac" indicator that the washer fluid signal can be

wired to light up. The low trac indicator is for the Camaro traction

control, which is not present on the Impala. The other indicators will be

present, such as:

Impala Indicator       Camaro Indicator

--------------------------------------

SERVICE ENGINE SOON    same

CHECK GAUGE            CHECK GAUGES

PASS KEY FAULT         SECURITY

Low Coolant (graphic)  same

LOW OIL LEVEL          LOW OIL

ABS                    ABS INOP

AIR BAG                AIR BAG

Also, the Camaro cluster has a standard roll type odometer, with a separate

trip odometer and push to reset knob right in the cluster. The Impala has

ENG/METRIC and TRIP buttons on the instrument panel to the left of the

radio, both of these switches will be rendered non-functional with the

Camaro cluster installed. The ENG/METRIC switch is an on/off switch which

can be wired to control anything else you need a switch for, while the TRIP

switch is a momentary contact switch that can also be wired for some other

function as you desire.

5.) The bottom line is that it can be done, but it is definitely not a

project for the inexperienced! <g>

>Is this the overhead console from the Blazer?  This also might be a another

must-have mod to cover up the large aftermarket "dome" light that police

departments install (present car and the '95).  What else in the console and

what years have one?

The Blazer/S-10 console has an outdoor temperature indicator (not indoor),

and a digital compass, as well as a garage door opener holder, sunglasses

holder, misc. compartment, dome light, and both front and rear reading

lamps. Besides the Blazer version, there is also the Pontiac Trans

Sport/Lumina van version which may fit the Impala roof curve better. I have

not had a chance to check them out fully, but one or the other could be

installed with a little work!

>>Yes, the dealer can do this. They have to contact a GM Delco service

center, who can create the proper PCM program with the gear ratio change.

Then the dealer downloads this program into their Tech 1 Scan tool, and from

there downloads it into your car.

>I asked a dealer about this and they weren't aware that this could be done.

Do you an idea what a typical charge for this is? [Another '95 9C1 mod]

You probably need to find a more savvy dealer, or perhaps have the dealer

give you the number of the local Delco factory service center so you can

call them yourself. You might also try the Chevrolet Customer Assistance

Center (CAC) at 800-222-1020. They can help with technical matters on Chevy

products. Let me know what they tell you.

>>Yea, the numbers for Huges and Datavision (who market these devices) are

listed in the 9C1 factory brochure. How about you give them a call and give

us the full report! <g> I'd be interested in installing these items as well,

except I think you have to cut up the roof! Scott.

>Do you have the numbers handy; I'll try to get some info from them (or do

they only talk to police?).

They should talk to anybody, but tell them you install police equipment. To

find out about the SEO UV3 Nightsight option, call 214-995-7511. To find out

about the Hughes Datavision option, call 800-DATA-HUD. Let me know what you

find out! Scott.

------------------------------

End of IMPALA Digest 240

************************

IMPALA Digest 241

Topics covered in this issue include:

1) F-car intake elbow

by Scott Mueller <73145.1566@compuserve.com>

2) 9C1 Factory Brochure

by Scott Mueller <73145.1566@compuserve.com>

3) Rev Kit

by Scott Mueller <73145.1566@compuserve.com>

4) Side carrier bearings

by Scott Mueller <73145.1566@compuserve.com>

5) Mileage

by Erich Arndt <EARNDT@mednet.swmed.edu>

6) Custom Aluminum Work

by cohan02@cai.com (Andrew Cohen)

7) Re: Homeplate ?s

by Kurt Kemmling <76165.1654@compuserve.com>

8) Re: More Snow driving tips........

by Kurt Kemmling <76165.1654@compuserve.com>

9) Re: 9C1 Air Deflectors $$$

by Kurt Kemmling <76165.1654@compuserve.com>

10) Re: Best Wax/Polish for Dark Cherry????

by Kurt Kemmling <76165.1654@compuserve.com>

11) Re: IMPALA digest 240

by Slawkoz@aol.com

12) Octane/performance

by "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

13) Re: Gas Mileage

by John_P._Kelly@troubbs.org (John P. Kelly)

14) '96 finally built!

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

15) Re: IMPALA digest 239

by Derkiller@aol.com

16) Re: Impala Parts

by rcheek@one.net (Robert Cheek)

----------------------------------------------------------------------

Date: Tue, 16 Jan 1996 22:08:48 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: F-car intake elbow

Message-ID: <960117060848_73145.1566_FHD49-6@CompuServe.COM>

Erich,

>Does anyone have precise measured dimensions (diameter and depth)

that would be needed to totally fill in the hole?  I'm debating about having

a friend with access to a precision lathe turn a "hocky puck" filler with a

lip on top to hold it into place (the negotiated six-pack is MUCH cheaper

than a 1LE elbow).  Any suggestions on a good, thermally-stable, light,

turnable  material?  Would an epoxy-based casting stand up to daily

abuse?  Would metal like aluminum get too hot to be practical in the air

intake system?  Other ideas????

My guess is that if you guys all had the B-car and F-car intake elbows side

by side, and could see how much larger and less restrictive the F-car elbow

is on the inside diameter around the curve, you would forget all of this

messing around with coffee can lids, or precision lathe turning custom

fillers! We're talking better flow here, in addition to a much cleaner

appearance. <g> Scott.

------------------------------

Date: Tue, 16 Jan 1996 22:09:02 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: 9C1 Factory Brochure

Message-ID: <960117060901_73145.1566_FHD49-8@CompuServe.COM>

>Where (and how) did you get the 9C1 factory brochure?

I got it from a Chevy Dealer (Tom Todd, right in Wheeling where you live)!

They are not easy to come by, you have to find a dealer who specializes in

fleet sales to get one. Unfortunately they are all gone as far as I know.

>It sounds like an interesting addition to an Impala library.

It certainly is!

>If you really need to schmoooze to get one, might I be able to reimburse

you for a photocopy? I live in the Chicago area also (Wheeling) and am

planning on attending the Auto Show.

Unlike the previous year brochures which were separate for each vehicle, the

'96 Chevrolet Specialty Vehicles brochure covers all of the Chevy cop cars

and is pretty big, about 80 pages total. It covers the Caprice 9C1, Caprice

Wagon 1A2, Lumina 9C3, and Camaro B4C police packages.

What I am thinking about doing is taking the thing to Kinko's and having a

bunch of copies made up to take with me to the Chevy Vettefest at McCormick

Place on Feb. 24th and 25th, where I will be showing my Impala. I would like

to hand them out at the show to anybody who will cover the copying costs. So

besides seeing all those killer Chevies (including my '94 Impala), you have

another incentive to make it to the show! Scott.

------------------------------

Date: Tue, 16 Jan 1996 22:08:55 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Rev Kit

Message-ID: <960117060854_73145.1566_FHD49-7@CompuServe.COM>

>I keep reading about dealing with valve float if you install new rockers a

previous digest I asked if anyone had any experience with a rev kit that

fits between the head and the lifters. I found out Comp Cams makes it. By

using this setup, you wouldn't have to worry about too much pressure over

the nose collapsing the lifter or too little seat pressure.

A rev kit is really designed for extremely high lift racing applications

where you are running 500+ lb. springs, etc. Installation requires removal

of the intake manifold, possibly the heads, and may even require machining

of the block! Crane, Comp. Cams, and several other companies make them for

racing applications, but they seem totally inappropriate for this

application!

In the Impala, all you need are higher pressure springs like that used on

the LT1/LT4 engines. These are 100K+ mile durability engines, and have no

problems with collapsing lifters etc. running 260 to 300 lbs of spring

pressure (valve open). The springs I mentioned will work properly and their

installation does not require removing the heads, intake or anything else

except the valvecovers.

If you want to go all out, then really you should remove the heads and have

the spring pockets machined larger to fit the LT4 springs. Or better yet,

simply install fully dressed LT4 heads (and matching intake) when they

become available through the GM Performance Parts program in a few months.

Scott.

------------------------------

Date: Tue, 16 Jan 1996 22:09:54 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Side carrier bearings

Message-ID: <960117060953_73145.1566_FHD49-14@CompuServe.COM>

Bruce,

>The p/n and price on the carrier side bearings was what I meant to ask for.

If I neglected to do so, I apologise.

No problem, they are; Carrier side Bearings, #9420095, $24.26 each. Scott.

------------------------------

Date: Wed, 17 Jan 1996 06:45:28 PST

From: Erich Arndt <EARNDT@mednet.swmed.edu>

To: impala@east.xsis.xerox.com

Subject: Mileage

Message-ID: <s0fcb765.083@MEDNET.SWMED.EDU>

Daryl,

17 mpg on my '94 on 93 octane Mobil.  That's highway, city, or rush hour

(and a half).  Even cruising at 75-80 it's 17 mpg.

FWIW, the car *seems* to run better on 93 octane than it did on 89.  It

doesn't feel sluggish as it accelerates or clatter under load.

Erich Arndt

'94 Impala

------------------------------

Date: Wed, 17 Jan 1996 08:57:58 PST

From: cohan02@cai.com (Andrew Cohen)

To: impala@east.xsis.xerox.com

Subject: Custom Aluminum Work

Message-ID: <9601171657.AA10394@cai.com>

Hi everyone.

Well, my garage is looking better these days since I've got my Impala in it.

Unfortunately, all I get to do is wash and wax it because my Wife spends all

her time driving it. (I spend my time paying for it) Oh well.8-)

Here's a thought if research shows that the group needs some aluminum spinning

work done.  My friend Steve owns a small metal spinning shop and I've spoken

with him about doing small runs, custom work, etc. for folks in the group.  He

said no problem, and I told him I was going to post to the list, so here it is;

Able Metal Spinning

(516) 266-6229

I told him he could use my car for any measurements/prototyping, etc.  After

all, he's the one who recommended the SS as the ultimate 4 door family car.

(BTW, There's nothing in it for me)

-Andy

------------------------------

Date: Wed, 17 Jan 1996 09:50:26 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Homeplate ?s

Message-ID: <960117175025_76165.1654_GHM123-1@CompuServe.COM>

I have been following the discussion on "Homeplate" in the past few digests.

I am somewhat confused.

Is the "removal" of 'homeplate' the same as the "blocking" of 'homeplate'?

I thought I read where you can just block off the flow of air into the

triangular resonator with a Nestea or similar cap, and then replace 'homeplate',

leaving it to look factory original.(This is what I would like to do, if it is

possible)

Then there are those who opt for complete 'homeplate' removal, and then putting

a cap over the hole on the elbow, and painting it. I also have seen where some

of you have removed 'homeplate' and replaced the elbow with one from the 1LE

Camaro. (this makes a much "cleaner", factory look from what I have read). Most

then go on to replace the valvecovers and add the Corvette Fuel Injector covers.

Are these two separate procedures or am I totally "off base"<G>?

Thanks,

KK

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Wed, 17 Jan 1996 09:50:29 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: More Snow driving tips........

Message-ID: <960117175029_76165.1654_GHM123-3@CompuServe.COM>

In digest 239 Stewart McLellan wrote:

>>>Re: Driving in the Snow on ZR4's - As any one who has lived in the hinter

land of the

north, on either side of the border, will tell you, driving in the snow is an

acquired SKILL.

The type, size and style of tires you are running will have an impact on your

driving but if you

don't know how to drive in these conditions nothing you do will be easy.  Try 8

-10 inches of

heavy snow, in a driving snow storm, in a mid 70's TA shod with old style

FR70-15 factory

Uniroyals.  It wasn't fun but I went over 400 miles and made it when 4x4's were

in the ditch.

FWIW.<<<

Congratulations! Not only for making the trip,<s> but for saying what I have

been trying to say all along. Driving in the snow is definately a "SKILL" that

has to be learned. If you "know" how to

"go in the snow", you should be able to do it no matter what kind of tires or

vehicle you use.

Thank you very much for sharing your experience with the rest of us.

KK

'95 Dark Cherry Impala SS.....no mods........yet......stock ZR4's w/ 23000+

miles.....

------------------------------

Date: Wed, 17 Jan 1996 09:50:30 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: 9C1 Air Deflectors $$$

Message-ID: <960117175030_76165.1654_GHM123-4@CompuServe.COM>

>>>  I remember

last year the deflectors listing around $3 something, so the price may have

gone up.<<<

Just for a point of reference, I paid $4.05 each for the front brake cooling air

deflectors from the 9C1 police package just last week, 01/10/96. I know this is

the list price (retail), as I do not get a discount on any purshases under $50.

KK from CT

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Wed, 17 Jan 1996 09:50:32 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Best Wax/Polish for Dark Cherry????

Message-ID: <960117175031_76165.1654_GHM123-5@CompuServe.COM>

I am asking the members of this list what kind of wax and/or polish they use on

their Dark Cherry Impala SS's. Also what procedure do you use to apply and

remove the wax, ie hand or buffer, rotary or orbital???  Do any of you use any

special pre-wax cleaners? How about any after-wax sealers? What about using

glazes or scratch fillers/removers? What do you use to get the swirl marks out

of the finish, once you are done waxing the entire car?

I know these are a lot of questions, but once this winter is over, I want to be

able to get my Dark Cherry paint looking like it did when it left the showroom.

Before this winter, I used Meguires #20 Polymer Sealant on the car, without

waxing it, since it did not need any more wax at the time. It seems to be

holding up fairly well considering the severe winter we have been having here so

far. I wash the car every day.

Any and all methods and personal experiences are welcome. I want to know what

the BEST products and methods on protecting my Impalas finish are.

Thanks to all who reply to this post.

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Wed, 17 Jan 1996 09:36:45 PST

From: Slawkoz@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 240

Message-ID: <960117123644_63205333@mail06.mail.aol.com>

I haven't written to the list in some time... so here are my recent thoughts.

Has anyone else seen the "Fleeter Impala" article in Car Craft magazine?  It

claims to have added 40 rear-wheel hp to the car through JET mods... the

whole article seemed more like a JET advertisement, to tell you the truth.

They added the "Powr-Flo" filter (any advantage over the K&N unit?), JET

PCM, airfoil, and underdrive pulleys... their "before" and "after" rear-wheel

hp were 174.2 and 213.1, respectively.

Hmmm.

I still haven't taken my baby to the dealer... I'm waiting for this hellish

weather to stabilize.  I have gotten used to the snow, though.  The biggest

part is getting beaten by that 1989 Hyundai Excel with a 4-speed, just

because he can accelerate - and I can't!  <sob>

------------------------------

Date: Wed, 17 Jan 1996 00:51:58 PST

From: "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

To: impala <impala@east.xsis.xerox.com>

Subject: Octane/performance

Message-ID: <9601171809.AA2494@CServe-Hub4.inhouse.compuserve.com>

Daryl:

>From digest 239:

>> 1) ...I've run Citgo 93 octane and (for the last couple thousand

miles) Citgo 89 octane.  No difference in mileage.  Is this pretty

consistent with what others are seeing?  Also, is there any reason

to use the higher octane gas?  Would it result in any noticeable

improvement in power? <<

I have seen write-ups from several Automobile experts that have stated

(paraphrase):  "Using a higher octane gasoline than is required by the car

manufacturer for that model is a waste of money".  For a stock Impala SS, my

understanding is that it requires "regular" octane gas (89?, I think).  Based

on a whole ton of factors, including compression ratio, head operating temp,

etc., a given minimum octane rating is required.  With the stock SS, you are

running 10:1 compression ratio (ballpark, maybe a little higher?), which, in

most engines, would normally require a higher octane fuel.  However, thanks to

reverse flow cooling and its positive effect on the head temp, GM was able to

allow for the regular grade fuel in the B-cars.  This is not true for the F or

Y car LT1, since they run a higher compression ratio.

Anyway, the point is that as long as you are meeting the minimum requirement

for octane recommended by the manufacturer, you're fine.  Running a higher

octane fuel will have no detrimental effect, to my knowledge, but will

basically be a waste of money.  If you modify certain components (cam, heads),

this probably will impact the minimum octane requirement for your car.  Most of

the fuel companies (especially Amoco with their "Ultimate" brand) try to tell

you otherwise, to get you to spend the extra $.25 per gallon for the good

stuff, when you really don't need it.  Apparently, their advertising is

effective.  In poll results recently published in the Chicago Tribune's

Transportation section, 15% of people surveyed use the highest octane fuel

available even though their vehicle does not require it.  Apparently, despite

Amoco's claim that their Ultimate is the best for your car, the only difference

between it and the "regular" brand is the octane rating.  Both have the same

detergents and additives in them.

I WILL make the plug for the more premium BRANDS of fuel.  We don't have many

Citgos where I'm at, but I always stick with Amoco, Mobil and Shell (I'm sure

there are others, but these are the ones I trust in this area).  These three

tend to be about $.05 per gallon more than the cheaper brands at a given

octane, but my understanding is that this is due to the fact that their fuel

has better detergent additives, and better (i.e., lower) "moisture content"

standards that they need to abide by.  They also used to only sell pure

gasoline rather than the 10% ethanol blend, but Illinois law now forces

everyone to use the blends for environmental reasons.  I most frequently use

Mobil in all my vehicles, since you get double "GM points" on the credit card

at their stations (gotta save up for the rear-wheel drive Monte SS, in case GM

actually builts it <g>).  I used to use the cheapest around, but after a few

carb problems with other cars, I no longer will use these.

------------------------------

Date: Wed, 17 Jan 1996 09:46:16 PST

From: John_P._Kelly@troubbs.org (John P. Kelly)

To: impala@east.xsis.xerox.com

Subject: Re: Gas Mileage

Message-ID: <1961982.270889184@troubbs.org>

Although the Impailer is up on blocks right now (Chicago winters a little to

much for the HP!), I have kept detailed notes on my gas and mileage:

Current Average MPG:  15.5  (at 16,119 miles)(Mostly suburban traffic & some

highway)

Best MPG (tankfull):  24.1 MPG

Worst MPG (tankfull):  11.4 (pulling a trailer)

Gas Used:  Reg unleaded first 2k miles, installed Jet Chip "Stage 2"  now use

prem always.

Avg Gas Price (prem):  @ $1.40  (Suburbs of Chicago - mostly)

JPK

- John P. Kelly

--

For helpful Mac tips and tricks; useful files; product reviews; late breaking Mac news; and more, you're welcome to visit the Chicago Area Macintosh Area Users' Group at "http://www.cmug.org/cmug/"

------------------------------

Date: Wed, 17 Jan 1996 14:20:24 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Subject: '96 finally built!

Message-ID: <9601172228.AA19659@karloff.fstrf.org>

FYI,

The loaded '96, DCM, that I ordered on in Sep. '95 is on its' way!

The timeline so far is:

+ 25 Septermber 1995: Placed order with Dealer

+ 2 October 1995: Order accepted with Chevy (1 week delay)

+ 30 November 1995: Order preferenced (8 week delay)

+ 18 December 1995: Given production week (3 week delay)

+ 8 January 1996: Build week (3 week delay)

+ 17 January 1996: Given estimated delivery date of 31 January 1996

(Approx. 4 week delay)

+ 31 January 1996: Another new Impala SS will shed a few millimeters

of fine BFG rubber! <g>

BTW, if anyone's looking for a used 1990 Thunderbird, please let

me know - I need the $$ for the down payment! :)

- mike

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

Date: Wed, 17 Jan 1996 13:37:27 PST

From: Derkiller@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 239

Message-ID: <960117163720_63326381@emout04.mail.aol.com>

GREAT NEWS LETTER

REMOVE ME FROM LIST, REMOVE ME FROM LIST, REMOVE ME FROM LIST...

------------------------------

Date: Wed, 17 Jan 1996 15:42:49 PST

From: rcheek@one.net (Robert Cheek)

To: impala@east.xsis.xerox.com

Subject: Re: Impala Parts

Message-ID: <199601172333.SAA05789@mail.one.net>

In the digest I found...

>use 3" ID, black, ABS plumbing pipe (it's 3.5" OD) of the appropriate length,

and...

>blockoff at all.  I did find a neat 3" pipe vent.  It has louvers in it.

and...

>filled the first inch or so of the seams with a new type of soft

>silicone I found at Home Depot (in paint dept) and used superglue to

Do you get the feeling they should just break down and put in an

auto parts cross reference at Home Depot :-) :-)

It's getting late...

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers + "H" Pipe

------------------------------

End of IMPALA Digest 241

************************

IMPALA Digest 242

Topics covered in this issue include:

1) Re: MSD Ignition

by rcheek@one.net (Robert Cheek)

2) Re: Replacing the rectangular resonator

by rcheek@one.net (Robert Cheek)

3) '94 9C1

by Scott Mueller <73145.1566@compuserve.com>

4) P/N of the 1.6 Roller Rocker Modification

by BGordon401@aol.com

5) Our ZR4s as Snow Tires?

by BGordon401@aol.com

6) 1996 Impala Order &  misc ?

by Joe Palmer <laser@iquest.net>

7) Mystery connector

by "Rush Luangsuwan" <rush@mcs.com>

8) camaro cluster/console

by Rock528@aol.com

9) 1-LE intake elbow and differential

by "Jeff Gatcha" <foxhound@txdirect.net>

10) MPG

by "Jeff Gatcha" <foxhound@txdirect.net>

11) roller rockers

by Scott Mueller <73145.1566@compuserve.com>

12) 1LE Elbow and Scratch Out

by Erich Arndt <EARNDT@mednet.swmed.edu>

13) Rear O2 sensors question

by Scott Widmer <Scott.Widmer@cle.ab.com>

14) Re: Latest Info from Arlington

by Kurt Kemmling <76165.1654@compuserve.com>

----------------------------------------------------------------------

Date: Wed, 17 Jan 1996 15:42:42 PST

From: rcheek@one.net (Robert Cheek)

To: impala@east.xsis.xerox.com

Subject: Re: MSD Ignition

Message-ID: <199601172333.SAA05768@mail.one.net>

>I think both mods (fuel and spark) are better done by modifying the PCM

>program. You may want more fuel at WOT, and a little more spark advance all

>around.

I've heard way too many things about reliability problems out of both

the Crane and MSD boxes to consider them anymore. The stock OptiSpark

probably has enough juice to get a pretty good spark on a moderately

breathed on engine.

I was under the impression (I'm sure the Chou's are there to correct

me again if I'm wrong ;-) that the PCM does not use the O2 sensor during

WOT blasts, but relies on a static map and the other sensors. So, I would

think that moderate modifications could benefit from an adjustable regulator to

fine-tune WOT performance at the track. You could also fine-tune for

barometric and temperature changes.

As I mentioned in a previous post, I'm curious if JET's approach might be

appropriate here: recalibrate the engine and intake air temperatures to

get the correct A/F ratio using a potentiometer on the sensor leads.

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers + "H" Pipe

------------------------------

Date: Wed, 17 Jan 1996 15:42:50 PST

From: rcheek@one.net (Robert Cheek)

To: impala@east.xsis.xerox.com

Subject: Re: Replacing the rectangular resonator

Message-ID: <199601172333.SAA05797@mail.one.net>

I'm a little unclear about the motivation for looking for a curvy, black

plastic pipe to replace the rectangular resonator:

1.) Won't the curves just introduce unneeded turbulence into the airflow?

I know it won't be much, but still more than a straight pipe. What is

the motivation for the curvature? Alignment with the stock airbox?

2.) As far as black versus chrome, I think the chrome will reflect more

engine heat (thereby probably feeling warmer) than the black, which will

absorb heat and feel cooler (because of the heat transfer to the rushing

air through it). Am I missing something in this equation? This is from

a vague recollection of my heat transfer class. Or is the black just to

match the rest of the stuff in the engine compartment for a more stock

look?

3) To anyone thinking about using only hose for this, how would you

prevent it from collapsing on itself? Or is this not a problem with

the hose type, pressure differential, etc.?

I still like Ted Dinsmore's idea of the exhaust pipe, conical K&N, and

AIR breather for an economical, good looking, functional, and cheap

induction system. Just IMHO, though...

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers + "H" Pipe

------------------------------

Date: Wed, 17 Jan 1996 16:52:23 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: '94 9C1

Message-ID: <960118005223_73145.1566_FHD47-5@CompuServe.COM>

Dave,

>Did Chevy offer the Police packeage in 94?

You bet! My friend had two of them (he's down to one now), and I was only a

deposit away from ordering a '94 Caprice 9C1 LS with leather, every possible

luxury option, and every police option as well. It would have truly been a

one of a kind vehicle!

>Was it the current one with an LT1, and did it have fender skirts?

The skirts died in '92. All '94 and up Caprices had the LT1 optional (L99

standard) including both civillan and 9C1 versions.

>My impression form the ones in my Department are that they made the last

bunch of 93 9C1's with the 'ol 5.7 motor and then stopped, skipping the 94

year and producing the '95 in mid 94 with the Impala.

There were something like 60 thousand '94 9C1 cars produced!! They outsold

Ford 2 to 1 that year.

>Are they making 96 Police 9C1's...or is this production run limited to

Impala's?!  I just saw a '96 Caprice civilian model on the showroom floor.

They are definitely making '96 9C1s, however the full 9C1 production run has

been sold out since '95. Scott.

------------------------------

Date: Wed, 17 Jan 1996 17:59:55 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Cc: 73145.1566@compuserve.com

Subject: P/N of the 1.6 Roller Rocker Modification

Message-ID: <960117205846_63518840@mail06.mail.aol.com>

Scott, and fellow list members:

I would like to confirm the p/n involved for the roller rocker modification.

They are as follows:

1) Crane 1.6 Roller Rockers         p/n 10758-16          $300.00

( Set of 16 rockers and nuts )

2) K-Motion 1.265 dia.springs       p/n K-700               $40-50.00

( Set of 16 springs-115 lbs @ 1.700 closed )

3) Crane Machine locks (+.050)   p/n 99095-1            $32.00

( Set of 32 pcs/2 per cyl. )

4) Valve Seals (GM Stock)           p/n 10212810          $16.00

( 16 pcs @ $1.00 ea. )

The prices are very closed between Jegs and Summit, except for the K-Motion

springs, which I found a $10.00 difference.

Now to a curiousity question about the grinding of the locks for the intake

valves.  You took off .040 from the Crane locks, which come.050 over stock.

That leaves a .010 difference over the stock locks after grinding.

According to the pressures you gave in previous digests, that .010

difference of the Crane locks after grinding, compared to the stock locks,

would only make about 3lbs difference in pressure.  Which leads me to my

question(s).

Scott, would it make sense to use the Crane locks for the exhaust valves, and

the stock locks for the intake valves, given that there would be about a 2-3

lb difference in the target pressures for which you were looking??  Would the

difference in steel strengths between the stock and Crane locks create a

problem with the performance, or function, of the motor now, or down the

line?

Thanks for the clarification.

..Bruce / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Wed, 17 Jan 1996 19:50:40 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Subject: Our ZR4s as Snow Tires?

Message-ID: <960117224855_45265149@mail04.mail.aol.com>

Kurt wrote in digest 241:

>Driving in the snow is definately a "SKILL" that has to be learned. If you

"know" how to "go in the snow", you should be able to do it no matter what

kind of tires or

vehicle you use.

I couldn't pass this one up...:-)  Just a friendly commentary...

That means you could drive our Impala, or even a NASCAR Grand National Monte

Carlo, with slicks in a storm that drops 21" of snow overnight??  If I had

to, I could too, but I wouldn't choose to!!  There's an safety edge when you

drive narrower tires with a proper tread pattern that tend to "cut" through

the snow, i/o "floating" on the snow with a tires that have a substantially

wider surface area.  Anything can be done, if you want to try hard enough.  I

prefer to run with 15" tires, as the police choose to do,  with a more

agressive, and considerably deeper, tread pattern that eats through the snow.

As I posted in previous digests, I LOVE those 17" rims and the performance

the tires give, however, it is PRUDENT to forego this passion for several

months.  It's nicer to see the wheels in the garage i/o attached to the car

in a ditch.

BTW, we received over 70" of snow so far this Winter in N.E. Pa, and we still

have six good weeks left.

>Before this winter, I used Meguires #20 Polymer Sealant on the car, without

waxing it, since it did not need any more wax at the time. It seems to be

holding up fairly well...

This name brand is the same as I use.  There whole product line seems to hold

up very well under the worst of conditions.  In between waxings, I use the

spray detailing compound.  It sprays on as a clear liquid, and you just buff

it off.

..Bruce / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Wed, 17 Jan 1996 19:50:17 PST

From: Joe Palmer <laser@iquest.net>

To: impala@east.xsis.xerox.com

Subject: 1996 Impala Order &  misc ?

Message-ID: <30FDC379.45@iquest.net>

FYI,

Glad to see someone is getting somebody getting their new IMPALA.

Ordered mine on 9-20-95. And as of 1-12-96 it still wasn't preferenced.

Chevrolet says it will be built in Feb. ? Does anyone know if the

options could be holding up my order. I have the heat reflective

windshield on mine ?

On another note has anyone seen a SS with Tan interior. Theres a dealer

here in Indianapolis with on e on the lot.

P.S.

Whats the average wholesale price for a 95 SS with 11,000 miles I think

my dealer is trying to gouge me ?

Thanx

JOE

------------------------------

Date: Wed, 17 Jan 1996 21:03:15 PST

From: "Rush Luangsuwan" <rush@mcs.com>

To: impala@east.xsis.xerox.com

Subject: Mystery connector

Message-ID: <m0tcmVH-000IDUC@venus.mcs.com>

On the front of the car, dead center right in front of a radiator, there's

a two prong connector that comes from the wiring harness, just dangling

there.  Anyone have an idea of what it's for?

--

Rush Luangsuwan

rush@mcs.com

'95 Blk SS

------------------------------

Date: Wed, 17 Jan 1996 21:13:28 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: camaro cluster/console

Message-ID: <960117204456_63507600@emout06.mail.aol.com>

In a message dated 96-01-17 01:28:05 EST, you write:

>Also, the Camaro cluster has a standard roll type odometer, with a separate

>trip odometer and push to reset knob right in the cluster.

Scott M.,  sorry for all the questions..

with a roll type odometer, does the F body still retain a speedo cable or is

it electric by some means?

Will a speedo service center match the mileages?  Have you had better luck

with one location over another?

also, have you loosely priced the blazer console yet?  ball park?

<>>Re: the heads up display, wasn't there a Gran Prix a few years back that

did a factory HUD?

thanx, Troy

------------------------------

Date: Wed, 17 Jan 1996 22:55:05 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: 1-LE intake elbow and differential

Message-ID: <199601180649.AAA06396@legend.txdirect.net>

Scott wrote:

>My guess is that if you guys all had the B-car and F-car intake

>elbows side by side, and could see how much larger and less restrictive

> the F-car elbow is on the inside diameter around the curve, you would

>forget all of this messing around with coffee can lids, or precision lathe turning

>custom fillers! We're talking better flow here, in addition to a much

>cleaner appearance. <g> Scott.

Does anyone know if the JET Air Foil will fit inside the F-car elbow?

It doesn't screw into the throttle body or anything, it just sits in

the indents and is held (tightly) in place by the B-car elbow.

I suppose it would since both elbows are (or should be) the same

diameter to fit on the throttle body, but just wondering if anyone

knows for sure.

Also, does the 95 1-LE elbow have that sensor already in it just like

the Impala elbow? (the one at the bend)  I know previous posts have

said that it has a spot for that little breather hose but what about the

sensor? (what is it anyway?)  Sorry I dont know what they are called

but I don't have the service manuals yet. Mr. UPS man hasn't delivered

them yet, and I don't want to try to guess at names.

One last thing. I have a friend with a 95 SS that he bought back in

Dec 94. (he is in Australia till March or he would be hooked up to

the list already, but his car is still in MD).  He says his differential is

making a horrible grinding  noise whenever he makes sharp turns in a

parking lot. He has tried 2 bottles of the additive with no result.

The dealer he took it back to doesn't seem to know what is wrong .

His car has about 20k on it and the noise has gotten progressively

worse since about 17k. He said its so loud he is embarrassed to

park anywhere or take anyone with him in the car.  Could it be the

spacer problem someone alluded toin  earlier digests only worse?

If not what might it be?  I told him to make the dealer fix it under

warranty, but have them slip in the 3.42 gear while they had it apart.

Ideas and comments always welcome.

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala.  No SERIOUS Mods...YET...

------------------------------

Date: Wed, 17 Jan 1996 23:01:22 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: MPG

Message-ID: <199601180655.AAA06549@legend.txdirect.net>

Daryl,

I use 87 octane (sometimes 89), and have never gotten less then 17.5

mpg city (with a little hwy) and managed to get nearly 24 when I

moved from MD to TX. That with the trunk, back seat and passenger

seat packed to capacity with houshold junk. Ave speed was 72. My car

never feels sluggish, and I never get any ping either.  Then again I only

have 8,500 miles on it..

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala.  No SERIOUS Mods...YET...

------------------------------

Date: Thu, 18 Jan 1996 00:20:44 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: roller rockers

Message-ID: <960118082043_73145.1566_FHD26-4@CompuServe.COM>

>I would like to confirm the p/n involved for the roller rocker modification.

They are as follows:

1) Crane 1.6 Roller Rockers         p/n 10758-16          $300.00

( Set of 16 rockers and nuts )

2) K-Motion 1.265 dia.springs       p/n K-700               $40-50.00

( Set of 16 springs-115 lbs @ 1.700 closed )

3) Crane Machine locks (+.050)   p/n 99095-1            $32.00

( Set of 32 pcs/2 per cyl. )

4) Valve Seals (GM Stock)           p/n 10212810          $16.00

( 16 pcs @ $1.00 ea. )

Yes, everything there looks good, that is what I am using!

>Now to a curiousity question about the grinding of the locks for the intake

valves.  You took off .040 from the Crane locks, which come.050 over stock.

That leaves a .010 difference over the stock locks after grinding.

No! The crane offset locks move the groove for the valve +.050 inches

raising the retainer and the spring installed height by that amount. I then

shaved .040 off of the top of the retainer, which has no effect on the

groove position. I just made the retainers narrower to clear the valve

guided rocker washers.

Using a height micrometer, I was able to determine that although crane

advertises the locks at +.050, they were actually only +.030 from the stock

locks. This accounts for a noticible change in pressure, as indicated in my

previous messages. Scott.

------------------------------

Date: Thu, 18 Jan 1996 07:19:48 PST

From: Erich Arndt <EARNDT@mednet.swmed.edu>

To: impala@east.xsis.xerox.com

Subject: 1LE Elbow and Scratch Out

Message-ID: <s0fe10ec.087@MEDNET.SWMED.EDU>

Scott,

Thanks for the clarification on the Camaro elbow.  I had thought that the

1LE elbow and the Impala elbow were the same except for the gaping

hole in the top.  If the 1LE style flows significantly better, it would be

foolish not to change it out this down the road.

Kurt,

Everyone has their own preferences on waxes and car prep stuff.

Personally I like Meguiars products as a whole;  they have a

WONDERFUL selection of stuff that covers just about anything you may

need.

As for the scratches,  I use a product by Kit called "Scratch Out" (comes

in a yellow bottle at the local grocery store).  It seem to remove most

surface-type imperfections like key and ring scratches around the door

handles (it even removed paint deposited by a careless door-banger) as

well as any mild surface haze that may have developed.  It doesn't seem

to be very harsh.  If your scratches are very deep, that's a whole 'nother

animal.

My '94 is black (gee, who could've guessed?), so any little flaw shows

up like a neon sign; I suspect that the Black Cherry isn't that much

different about showing imperfections.  Whatever you do, spend a little

extra time and make sure that the car is REALLY CLEAN before you try

and do any scratch removal.

One other thing that's been tossed around the list lately is a product

called "the Absorber" for drying the car.  I bought one about 2 weeks

ago and have used it twice.  It cost me about $7 at the local K-Mart (a

whole lot cheaper than a decent sized chamois) and does a great job.

No spots, no streaks, no scratches.  I should have bought one a long

time ago!

FWIW,

Erich Arndt

'94 Impala

------------------------------

Date: Thu, 18 Jan 1996 11:25:48 PST

From: Scott Widmer <Scott.Widmer@cle.ab.com>

To: impala@east.xsis.xerox.com

Subject: Rear O2 sensors question

Message-ID: <199601181925.OAA03930@ppx.cle.ab.com>

Hi all,

Quick question about the O2 sensors aft of the catalytic converters on the '96

B-cars (Impalas):  Do they function merely as a switch to signal the PCM to set

a code when the cats' efficiencies are below set standards...or do they have

some effect on air/fuel mixture control as do the primary O2 sensors??  In other

words, if the rear O2 sensors detected "failing" converters (too little oxygen),

would the PCM lean out the mixture and/or retard timing in an attempt to

compensate, or would it just set a code??  Just curious.

BTW, what are the part numbers for those export pipes??

Thanks,

-Scott Widmer

srwidmer@cle.ab.com

'96 SS Black

------------------------------

Date: Thu, 18 Jan 1996 11:50:25 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Latest Info from Arlington

Message-ID: <960118195025_76165.1654_GHM148-1@CompuServe.COM>

Here is the latest "SPY" report from the insider spies at the Arlington Plant:

December 17, 1996 is the "official" cutoff date for production of Impala SS's.

They are currently working only one 8 hour shift, and plan to do so until the

cutoff.

They are producing 56 automobiles and hour.

1 out of every 6 automobiles is an Impala SS.

Therefore, 9.333 Impala SS's are being produced per hour.

This translates into 74.666 Impalas per day.

The weekly output of Impala SS's is 373.333.

There are aproximately 48 more weeks of production scheduled for automobiles

at the Arlington plant before the switchover to trucks.

So they should be able to produce another 17,920 '96 Impala SS's before the

cutoff.

I will try and find out how many '96 SS's have been built to date, so that we

can

estimate the total production for the 1996 model year.

By using the above equation, I estimate that they built 7,466.666 Impala SS's

so far to date, using a figure of 20 weeks of production prior to this week.

Therefore a ballpark production estimate for 1996 Impala SS models is 25,387

vehicles.

I would be curious to see how close this estimate is to the actual figures

released

by GM at the end of this year.

Please keep in mind that all of the above figures are my "Guesstimates", based

on

the most accurate info I have available to me today. I will keep everyone

updated on

these numbers, as more accurate information becomes available.

I have one other thing to relay to all of the list members who contacted me

privately

about finding out about their particular vehicles production dates and other

info they

have requested about any single Impala SS. I have been told by my contact that

requests

for info on a particular automobile cannot be processed. It would put my source

at risk

of being discovered, and for obvious reasons these requests can no longer be

filled.

I am sure we can all appreciate the efforts that this "inside spy" has gone

through, and

the helpful and valuable information that she/he has provided.

I am also pleased to announce that I have made contact with another individual

in the plant who has agreed to become an anonymous "Impala SSpy" for our list.

By having another person on the inside, we will now have corroberating

information, and also more accurate info available to us.

We may also have some "spy photos" in the near future from what I was told by

this

new insider. Let's all hope that all of these individuals continue to provide us

with the

latest information and pictures on the last year of production for this fine

automobile.

Kurt K

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

End of IMPALA Digest 242

************************

IMPALA Digest 243

Topics covered in this issue include:

1) FW: Stainless exhaust for intake

by Joe Reynolds <MGXAMERICA/RICHARDSON/JoeR%Micrografx@mcimail.com>

2) Various

by T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

3) HUD

by "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

4) Just joining in=?iso-8859-1?Q?=8A?=

by Perry Iannuzzi <madmacs@earthprobe.com>

5) Re: 1LE elbow

by jsalmon@mindspring.com (John Salmon)

6) Sample letter from South FL

by Rock528@aol.com

7) intake air sensor and elbow info

by Rock528@aol.com

8) cheap and effective intake

by Rock528@aol.com

9) Florida local

by Rock528@aol.com

10) export pipes

by Rock528@aol.com

11) Clarification of proper grinding for the 1.6 Crane Rockers

by BGordon401@aol.com

----------------------------------------------------------------------

Date: Thu, 18 Jan 1996 11:43:00 PST

From: Joe Reynolds <MGXAMERICA/RICHARDSON/JoeR%Micrografx@mcimail.com>

To: Impala <impala@east.xsis.xerox.com>

Subject: FW: Stainless exhaust for intake

Message-ID: <75960118194357/0004114862PL1EM@MCIMAIL.COM>

Last weekend I finished working with a piece of stainless 3.5" flexible

exhaust to use between the Camero intake elbow and the air sensor on my

Impala.  I'm real pleased with the look and the fact that I didn't have

to use anything but the stainless piece to fit between the two.  Anyone

else tried this?

I do have a bit of a bind at the elbow (the intake, not mine) due to the

exhaust not bending as much as I would've liked.  It did fit into the

sensor perfectly, and I polished it with a bit of steel wool to shine it

up.

The NAPA guys have been getting pushed around a bit out here, so I would

like to give my NAPA guy a kudo for getting this 3.5" gargantuan flex

hose to me, even though I only bought 2 feet and he had to order 10 to

get it from the warehouse.  (I didn't use two feet of the stuff, but I

like to have some extra for when I screw it up.) Rah Rah for Kevin and

the NAPA store in Richardson, TX!!

I would also like to know if anyone has any suggestions for rock chip

repair in the gorgeous paint job of my grey green Impala.  I have the

touch-up stuff, but as we all know, it's not the smoothest match of the

factory paint.

Thanks to all for their contributions to the list, and for Scott M.'s

expertise all of the time!!!

Joe Reynolds

Grey-Green

joer@micrografx.com

------------------------------

Date: Thu, 18 Jan 1996 12:26:45 PST

From: T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

To: IMPALA@east.xsis.xerox.com (Non Receipt Notification Requested)

Subject: Various

Message-ID: <0018ecymmsyw*/I=T/S=MAKICHUK/OU=FA.EPS/O=NHQ/PRMD=GC+DRHC.HRDC/ADMD=GOVMT.CANADA/C=CA/@MHS>

Hi all, I've spent the last month reading all the archives on the web site,

and am now ready to introduce myself.  I am the original owner of a 1988

9C1, with all the HD options on it but with few of the convenience items

that add weight and cost.  For the benefit of younger list members, I was

23 when I ordered the car.

I live in Ottawa, where many government sedans, and most Royal Canadian

Mounted Police (RCMP) cars are either current or previous generation

Caprices.  These cars all carry license plates that begin with FDM;

therefore, my marker is FDM 9C1.

I am planing to order a 1996 9C1 at the end of the summer, and hope to get

one of the last run produced.  Hopefully agents xxxx and xxxxx will be able

to find out when the last 9C1s will be built.  Sorry SS owners, it was a

tough choice.  My reasons were the $3,000 price difference, and the fear

that my wife, future child or dog would eventually damage such an

undeniably beautiful car.

1)  How do you aim the T84 export headlamps?  Do they aim in the same way

as described in SAE J599, i.e. with a screen at 25 feet.  Do they also have

the three points to mount a headlamp aiming device?  Not wanting to start

the DRL thread again, but has anyone considered hooking the 5 watt

positioning bulbs to a power source that is live in the run position.

2)  Posi clunking or booming noises while turning at slow speeds at light

to no throttle.  I have experienced this problem since the day I took

delivery of my 1988 9C1.  The dealer's response at the time was to put a

second bottle of additive in the differential.  It worked for about six

months.  Since then I have been using the "limited slip flushing procedure"

described in the 1988 service manual.

Essentially this is removing the cover and draining the warm lubricant,

reinstalling the cover and old gasket and refilling with new lube.  Raising

the rear wheels off the ground, and running the engine in drive for four

minutes at a speed not exceeding 30 mph.  Then draining again, replacing

the gasket and refilling with new lube and additive.

This is the procedure I use every time I change my axle lube, and it

eliminates the clunking for about 12,000 miles.  It is interesting, but I

knew someone who also owned a new 1988 9C1 with GU4 and G80, but he never

experienced these noises.

3)  BTW on the gasket, the correct and older design GM gasket was 3993593.

My parts guy was able to determine by computer search that there are none

left with any Canadian dealers, but that there is 200+ with US dealers.  He

was unable to get me one, but maybe someone on the list could inquire with

a US dealer and see if a 3993593 can be tracked down.  I bring this up

because I don't remember it having small rectangular slots like the Fel-Pro,

I think it was scalloped out.  If we had an original, maybe we could

modify the new (bad) 26016662 gasket to the older (proper) specs.  BTW when

I first showed my parts guy Scott M's gasket warning, he had never heard of

it, but knew from memory the part number for new axles.  Seems he's been

selling a few.

4)  Interesting to learn about SEO 7Y2 a $15 option that buys you a 25?

bulb installed in a pre-existing socket and wiring harness.  I found a

similar mod for 1990 and prior 9C1 owners.  These 9C1s were not available

with under dashboard courtesy lights; however, the sockets and wiring

harness are all in place.  All you need to do is install the two bulbs, 631

I think.  The sockets are immediately beneath the dash at the far left and

right corners.

5)  On why the rear wheels are closer to the front of the fender then the

back.  When the current generation Caprice was unveiled, I recall reading

in Motor Trend that the stylists wanted a slightly longer wheel base.

Since this would require significant suspension re-engineering from the

previous generation, it wasn't pursued.  As a result the stylists designed

the rhombus shaped cut outs to visually draw the rear wheels back.  I

suspect that when they rounded the cut outs for 1993, they were stuck with

their previous decision.  Their options would have been to redesign the

rear doors and centre the wheels, or save money and position the wheels

slightly forward.

6)  In digest 186, Erich mentioned that he heard B body production would

move up north.  This summer I read in one of the trade journals that

someone by the name of Chrysler wanted to buy the B body tooling and move

it to the closed Hyundai plant in Bromont Quebec.  From there he would

produce police vehicles exclusively.  The article explained that he wanted

all the tooling as he would combine the best parts.  The example of using

the Roadmaster roof on the Caprice was given.  This Mr. Chrysler was

complaining because GM would not agree to sell him the tooling.

7)  On the location of the trim build sheet.  In my 1988 9C1, I found one

stuck in the springs of the driver's seat, and another in the frame of the

rear seat.  I have a third, but can't remember where it came from.  Still

haven't found the main build sheet.

8)  Dexron synthetic?  I thought that when the international ban on whaling

came into effect in the early 70's they had to switch from whale blubber to

a synthetic, hence Dexron II.  Are not Dexron II, IIE and III already

synthetics?

9)  Quote of the month:  This week I went to a local tire shop, and asked

about blackwall GT+4 225 70HR15.  This is the tire my car came from the

factory with.  The guy only found a listing for white outline lettered.  I

asked him why he couldn't find a listing for the blackwall, since it's a

standard police tire.  His response was: "I can't sell 'em to you".  When I

asked how come, he said "cause it's illegal".

Tip Makichuk

45?17'22"N 75?51'11"W

------------------------------

Date: Thu, 18 Jan 1996 07:56:19 PST

From: "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

To: impala <impala@east.xsis.xerox.com>

Subject: HUD

Message-ID: <9601182232.AA0411@CServe-Hub4.inhouse.compuserve.com>

Troy:

>From digest 242...

>> Re: the heads up display, wasn't there a Gran Prix a few years back that

did a factory HUD? <<

I know it was an option on the '90 - '93 Olds Cutlass Supremes.  I have a '91,

but bought off the lot, and only absolutely wanted the 3.4 DOHC engine, other

than that I was flexible.  The one I bought didn't have HUD, but in retrospect,

I wish I had ordered it.  I do not think they offer this any more for the

Cutlass' - in fact, I think it was dropped for that model after '94.  I wish it

was an option on the Impala!

------------------------------

Date: Thu, 18 Jan 1996 10:13:12 PST

From: Perry Iannuzzi <madmacs@earthprobe.com>

To: impala@east.xsis.xerox.com

Subject: Just joining in=?iso-8859-1?Q?=8A?=

Message-ID: <30FE8DB8.3035@earthprobe.com>

Hi all,

I was quite impressed to find such a following for such a great =

autmobile. I've owned 48 cars in 22 years, but have never had so much =

attention paid to my 95 SS. People from 15 to 75 stop and ask "What =

kinda car is this"? pointing to the Big Black Demon parked at the =

pumps or at the traffic light. I've even had one fellow come uo to me =

and say "Hey Ford's really made the T-Bird look Great", you should've =

seen the look on his face when I informed him that this"Ford" was made =

by Chevy!

I bought the 95 with the impression that it was gonna be the last =

year, but to my surprise the 96's are here=8AI hope this is the last =

year because I've just ordered a 96 and put my 95 for sale (just gotta =

have the last one you know).

I love all the tech tips you guys are putting up here, really =

impressive to see the amount of data, facts & info you SS lovers (like =

myself) are sharing with us other owners.

We'll keep in touch and pass any info we come across with our SS.

Perry Iannuzzi

95 Black "Slammed" SS

------------------------------

Date: Thu, 18 Jan 1996 15:42:55 PST

From: jsalmon@mindspring.com (John Salmon)

To: impala@east.xsis.xerox.com

Subject: Re: 1LE elbow

Message-ID: <v01530501ad2435479cf7@[168.121.55.70]>

> Also, does the 95 1-LE elbow have that sensor already in it just like

> the Impala elbow? (the one at the bend)  I know previous posts have

> said that it has a spot for that little breather hose but what about the

> sensor? (what is it anyway?)  Sorry I dont know what they are called

> but I don't have the service manuals yet. Mr. UPS man hasn't delivered

> them yet, and I don't want to try to guess at names.

The sensor is the IAT (intake air temperature) sensor, and yes, it's

included in either of the Camaro elbows.  That's why the big rubber hose

costs upwards of $85.  The sensor is molded in from what I'm told...

Now, anybody found the '95 1LE hose's REAL GM part number?   The local

bonehead dealers can't seem to locate it on the computer ("uh, ya mean ya

want thishere rubber thingumbob?  Naw, ya can't get that, they don't make

but enough to go on the perduction cars.  Why wouldja wanna r'place that

there piece on your car, anyway?  Ya'd have ta cut a hole in the top to put

the triangular thingy back on, ya know?  'Scuse me, I gotta go help this

feller what wants a rebel flag on his pickup....")

Man, I get tired of this backwater town.  Alabama, the state that thanks

Mississippi for making us next to last on most every state list there is.

Harrumph.

John Salmon

Data Warehousing Technical Support

personal: jsalmon@mindspring.com

work: john.salmon@bridge.bellsouth.com

'95 Dark Cherry Impala SS -- The SALMN8R

------------------------------

Date: Thu, 18 Jan 1996 16:59:47 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Sample letter from South FL

Message-ID: <960118195907_200610892@emout04.mail.aol.com>

Here is a sampling of the stuff I get at the impalaclub address.  I'll call

it, "another satisfied customer".

Hello Troy!  This message is from john and camie george of cooper city

florida.  As a fellow law enforcement officer and also an impala owner i

wanted to share with you the additions that i have made to my 96 SS.  I

was able to obtain all of the corvette accesories and make the conversion

to the appearance of the enginge which has drawn alot of attention from

local car enthusiasts.  It has also drawn other impala owners out of the

woodwork. We frequent a cruise spot in davie, the juridiction that i work

in.  this cruise spot is an every friday night thing.  I regularly take

my 62 impala and Camie has occasionally brought the 96.  Last Friday,

Camie left the 96 at home, but at the cruise, there were 2 other Impala

SS's.  Bothe were 95's.  Not knowing either of these two owners, I

approached them and began to ask them about their cars, each

individually.  One of the cars had removed the filter and front square

box, but left the triangle.  I asked him why his car had the triangle and

the Impala that was there last week (meaning Camie) looked different

under the hood.  he began to explain to me that his car was not a 96 and

that the 96 came equiped witha corvette motor!  After listening to him

for a little while, I broke the news that the lady who was driving the 96

with the corvette motor was my wife and that he could have the same parts

on his engine.  I further let him know about the club and will provide

him with the application when I see him again.

John George

------------------------------

Date: Thu, 18 Jan 1996 16:59:12 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com, foxhound@txdirect.net

Subject: intake air sensor and elbow info

Message-ID: <960118195911_200610948@mail04.mail.aol.com>

Jeff,

>Does anyone know if the JET Air Foil will fit inside the F-car elbow?

Yes, it appears as if it is actually made for the F body elbow.  The B body

elbow doesn't have the proper upper indentation as the 1 LE or F body elbow.

I have the JET intake and have played with both elbows.  We have bantered

alot about the 1LE and the block off cap and I have to tell you it was money

well spent after I swallowed hard at the hundred bucks for the elbow.  The

1LE elbow is alot more professional looking especially if your doing the

complete engine dress.

>Also, does the 95 1-LE elbow have that sensor already in it just like the

Impala elbow?

Yes, this is why it is so expensive.  It is called the IAT, intake air temp

sensor.

BTW, here is the info regarding IAT resistance readings.  It would be easy to

fool the IAT pickup but I don't know what effect it will have on the O2

sensor?  Any ideas..

Troy. (original post from F list to follow)

Subject:  ATS Explained

From: Mike@Truppi-Kling.com

Date: 7 Nov 1995 13:15:04 -0500

ATS POT:  This one is not really needed, we were hunting a problem, and never

backed to backed it.  I would love someone to try it by it's self.  ATS (air

temp sender) is a thermal resistor,.  As temp changes the resistance changes.

We seemed to have a very lean condition, so this was an easy  way to make

the motor richer in little steps.  People had told me this changes the timing

curve too, I don't know.  I can tell you that if the inlet air is colder, you

should slightly richer.  So if you really want to try this, get a 0-15 turn

0-100K  5 volt variable resistor.  Plug this in place of the air temp sender.

Now you can make the computer "think" it is

colder.  Like I said, we did this to chase a problem, but I would like to

here what people find.  You asked, so here it is.

ATS resistance VS temp reading.

ohms			temp

1802			95

2238			86

2796			77

3520			68

4450			59

5670			50

7280			41

9420			32

12300			23 *

16180			14 *

21450			5   *

28680			-4

* places we have used

Michael C. Truppi (Mike@Truppi-Kling.com)

Truppi/Kling High Performance inc.

133 RT 22 East

Greenbrook NJ  08812   (908) 968 2030

------------------------------

Date: Thu, 18 Jan 1996 17:01:28 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: cheap and effective intake

Message-ID: <960118195906_200610869@mail02.mail.aol.com>

In a message dated 96-01-18 16:12:07 EST, you write:

>I still like Ted Dinsmore's idea of the exhaust pipe, conical K&N, and

>AIR breather for an economical, good looking, functional, and cheap

>induction system

If you look at Ted's setup, it is exactly like the one in the latest Super

Chevy (March) magazine. That manufacturer used stainless on their tube

instead.  Ted and the rest of us locally did the same thing with various

airbox variations, plus you can't beat the cost:  50.00 for the K&N and say

20.00 for the 3.5" chrome exhaust tip.  Kudos..Razzor    Troy

------------------------------

Date: Thu, 18 Jan 1996 17:14:46 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Florida local

Message-ID: <960118195908_200610913@emout05.mail.aol.com>

Local Impala Fellers,

Let's do Sanford meet on the last Saturday of January (27th) now that the wx

is back to the 70's (sorry for you guys that still can't open your garage

door, we were all there once).

Walmart/Wendy's on 17-92, South Sanford.  Cars and the band will be back.

Start at say 6:00 pm.

Take I-4 to Lake Mary Blvd to 17-92, North 1 block on right.  See you there,

Troy.

------------------------------

Date: Thu, 18 Jan 1996 17:14:50 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com, srwidmer@cle.ab.com

Subject: export pipes

Message-ID: <960118195912_200610967@emout05.mail.aol.com>

>BTW, what are the part numbers for those export pipes??

originally from Scott M.

-NM8   Exhaust System, Export Leaded Fuel

These are factory stainless steel pipes that directly replace the

catalytic converters on vehicles built for shipment overseas. They do not

include stainless steel bungs for the oxygen sensors, which I obtained

separately and welded on in the same locations as on the standard US

pipes. Note that these pipes are for export or off-road use only.  Scott

Each pipe has an O2 sensor.  Go to an exhaust system wholesaler in town.

They should have the round bungs to weld on your pipes.  Then drill the hole

and file slightly so the O2 sensor will fit.  There are also small baffles

that look like a shower drain in the aft end of the pipe. General consensus

is knock them out.   There is a noticeable sound difference with these pipes

vs. cats.  Troy

Export NM8 Leaded Fuel Only Front Exhaust Pipes:

Part #          Desc             List

--------------------------------------

10204015        PIPE ASM-        71.00

10204014        PIPE ASM-        71.00

12550283        GASKET-EX         5.45

10168551        GASKET-EX         5.45

I know what your thinking, and of course not, I would never knowingly do

anything that would be illegal.  I'm just real educated.  Troy

------------------------------

Date: Thu, 18 Jan 1996 17:52:57 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Cc: 73145.1566@compuserve.com

Subject: Clarification of proper grinding for the 1.6 Crane Rockers

Message-ID: <960118203421_120776191@emout05.mail.aol.com>

Scott, I REALLY HATE TO BE A PEST, but I'm a bit confused about the grinding.

So, please bear with me for a minute.  Locks and keepers have been used

synonymously throughout various digests, since no corrections have been

posted otherwise, I'm under the assumption they're the same.  Which, have

been used in different contexts several times than retainers.  So, we'll go

from there.

In digest 237, when going over you're install proceedure, you wrote in option

# 3:

>So I had to grind .040 from the top of each of the keepers

used on intake valves for proper clearance.

And...

>The machined offset keepers are about $30, and it took me a few minutes to

grind .040 inches off of 16 of them for the intake valves. I used a bench

grinder with a vise to support the keepers for grinding, plus a micrometer

to check everything out.

Now comes my confusion.  In our latest digest 242,  you responded to my

questions about grinding down the 16  Crane locks (keepers) by .040, as

posted above, by stating:

>No! The crane offset locks move the groove for the valve +.050 inches

raising the retainer and the spring installed height by that amount. I then

shaved .040 off of the top of the retainer, which has no effect on the

groove position. I just made the retainers narrower to clear the valve

guided rocker washers.

O.K., was it the 16 keepers (locks) that were shaved by .040, as posted in

digest 237, or the top of the retainers to make them narrower to clear the

rocker washers??

Maybe my question is off base, and if it is, I apologise.  However, now that

I'm ready to roll on this, the first step in a series of many modifications

over the next few months, I want to make sure I'm completely clear on

everthing for the proceedure to be done correctly the first time.

Thanks man!!

..Bruce / Bgordon401@aol.com

1995 Cherry Metallic

------------------------------

End of IMPALA Digest 243

************************

IMPALA Digest 244

Topics covered in this issue include:

1) Re:Gas Mileage

by lennie@earthlink.net (Lennie Dolphin)

2) Black bodies shed heat

by Scott Mueller <73145.1566@compuserve.com>

3) air filter

by Scott Mueller <73145.1566@compuserve.com>

4) P/N needed for 9C1 door handle

by Scott Mueller <73145.1566@compuserve.com>

5) Re: Oh no SNOW,here we go again<G>

by Kurt Kemmling <76165.1654@compuserve.com>

6) Black bodies shed heat

by Scott Mueller <73145.1566@compuserve.com>

7) misc stuff

by Scott Mueller <73145.1566@compuserve.com>

----------------------------------------------------------------------

Date: Thu, 18 Jan 1996 18:27:50 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: impala@east.xsis.xerox.com

Cc: David Spitz <david_spitz@vnet.ibm.com>

Subject: Re:Gas Mileage

Message-ID: <199601190227.SAA28607@iceland.it.earthlink.net>

David said:

>I think the consensus was that you don't gain much by using a higher octane

than that which doesn't knock. So, if you don't knock at 89, don't bother

with 93.

You know David, I've been 93 guy for sooo long that I hate to switch. I try

89 in my '91 Caprice a couple of times, starting pinging when I ran the air.

Mileage suffered also. I'll try it with the '95 SS "MONSTER MACHINE". Scott

M. gave an execellent technical explanation of the computer's octane table

and how it will adjust to the knock sensor.

>I have found that, no matter how I drive (fast/slow, city/highway), that I

almost never deviate from between 19mpg and 21mpg, with 400 miles on about

20 gallons (20mpg) being almost always what I get.

At 93, the best I can get is 16.5mpg with my foot "out of the tank". Haven't

takened the SS on the road yet. Might be time to try the 89 grade.

>It's worth kissing those Lexuses (Lexi?) and Infinity's goodbye!! :)

I always wondered if that Lexus that won't let me pass him, could read my

rear license plate <g>. Hope to see an Infinity in my travels to the store :-).

... Lennie

------------------------------

Date: Thu, 18 Jan 1996 18:59:53 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Black bodies shed heat

Message-ID: <960119025952_73145.1566_FHD39-6@CompuServe.COM>

Rob,

>As far as black versus chrome, I think the chrome will reflect more engine

heat (thereby probably feeling warmer) than the black, which will absorb

heat and feel cooler (because of the heat transfer to the rushing air

through it). Am I missing something in this equation? This is from a vague

recollection of my heat transfer class.

A black body is the most efficient heat radiator possible, so if you want

something to shed heat, paint it black! That is why radiators and other heat

exchangers are usually painted black. A shiny chrome or white body will

retain far more heat. Plastic or rubber is also a much better heat insulator

than metal. The use of black plastic or rubber in the induction system is no

accident, and is actually very good engineering. Personally, I think the

factory square plastic front mounted resonator is fine. If you really feel

the need, then seal the internal resonator ducting, but I really don't think

it creates much if any restriction.

The big restriction is in the airbox itself, and that is the area which

needs the most attention. A large 8" cone type K&N filter will add some

noticible HP, although I don't like the non-factory appearance. For a

factory look, use the stock type K&N filter and drill large holes in the

bottom of the airbox housing to let in more air. This will still not flow

nearly as well as the completely open larger cone filter, but it does pretty

good and looks completely stock. Scott.

Joe,

>On another note has anyone seen a SS with Tan interior. Theres a dealer

here in Indianapolis with on e on the lot.

I don't believe it came from the factory that way, the dealer probably

changed it. If you could get me the VIN number from that car, I would really

appreciate it. If indeed it is a genuine Impala with a Tan interior, that

would be extremely rare (as in maybe the only one in existance), and I could

confirm that via the VIN number! Scott.

Rush,

>On the front of the car, dead center right in front of a radiator, there's

a two prong connector that comes from the wiring harness, just dangling

there.  Anyone have an idea of what it's for?

That's for the front mounted ambient temperature sensor for the Buick

Roadmaster climate control system. As you can see, they use the same harness

for both the Roadmaster and the Caprice, even though that climate control

system is not available on the Caprice. Scott.

Troy,

>with a roll type odometer, does the F body still retain a speedo cable or

is it electric by some means?

All GM speedo/odos are now electric, including the mechanical roll type odo

in the Camaro.

>Will a speedo service center match the mileages?  Have you had better luck

with one location over another?

A Delco service center can change the mileage on the new cluster to match

the vehicle. I like D&B Radio (a big Delco service center) here in Addison

IL. That is who all of the Chicago area dealers use, and they are good.

>also, have you loosely priced the blazer console yet?  ball park?

Ball park I would say $300.

>Re: the heads up display, wasn't there a Gran Prix a few years back that

did a factory HUD?

The Grand Prix and Bonnie still have it as an option today ('96 models).

Scott.

Jeff,

>does the 95 1-LE elbow have that sensor already in it just like the Impala

elbow? (the one at the bend)

Yep, that is the MAT (Manifold Air Temperature) sensor, and in fact you get

a new one for free with the $70 F-car elbow. <g>

>He says his differential is making a horrible grinding noise whenever he

makes sharp turns in a parking lot. He has tried 2 bottles of the additive

with no result. The dealer he took it back to doesn't seem to know what is

wrong. His car has about 20k on it and the noise has gotten progressively

worse since about 17k. He said its so loud he is embarrassed to park

anywhere or take anyone with him in the car. Could it be the spacer problem

someone alluded toin  earlier digests only worse?

The real problem here is the dealer! I mean come on, they need to rip that

sucker apart and re-build it. The assembly tolerances in the 8.5" rear ends

I've seen leave a lot to be desired. The pinion depth (a critical setting)

was off .004" in my axle, which is actually quite a bit. I corrected the

depth problem by installing the correct shims when I installed the 3.42

gears.

>I told him to make the dealer fix it under warranty, but have them slip in

the 3.42 gear while they had it apart.

That is the best advice you could give him! Scott.

Scott,

>Quick question about the O2 sensors aft of the catalytic converters on the

'96 B-cars (Impalas): Do they function merely as a switch to signal the PCM

to set a code when the cats' efficiencies are below set standards...or do

they have some effect on air/fuel mixture control as do the primary O2

sensors??  In other words, if the rear O2 sensors detected "failing"

converters (too little oxygen), would the PCM lean out the mixture and/or

retard timing in an attempt to compensate, or would it just set a code??

As far as I understand, they are only to measure catalyst function and

efficiency, and the 2nd set of O2 sensors have nothing to do with the

air/fuel ratio. In other words, if you removed the cats, they would just set

a code. It may be possible to create an O2 simulator that could be used to

simulate the function of the second set of O2 sensors in case of cat

removal, for off road use only, of course. (John S. if you're listening what

do you think?)

>BTW, what are the part numbers for those export pipes??

Export NM8 Leaded Fuel Only Front Exhaust Pipes:

Part #    Desc               Cost

----------------------------------

10204015  PIPE ASM, LH       51.75

10204014  PIPE ASM, RH       51.75

Scott.

------------------------------

Date: Thu, 18 Jan 1996 19:00:04 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: air filter

Message-ID: <960119030003_73145.1566_FHD39-7@CompuServe.COM>

Mike,

>I remembered you mentioned that you have swtiched to the K&N air filter. I

am wondering did you also swap the whole system with a Hennessy unit or did

you just swap the filter.

I first used an 8" cone K&N filter directly connected to the MAF, completely

eliminating the stock airbox. This offers the best airflow possible, but did

not look factory original of course.

Since I decided to show the car, I wanted to re-install the factory airbox

for an original look, and then installed the factory style K&N filter in the

box. For more airflow, I drilled large holes in the side and bottom of the

airbox (which are not visible from above). This allows the stock type filter

to get much more airflow, but still would not be as good as no airbox with

the big cone filter. I decided to sacrafice a little performance for the

factory type look.

>The reason I am asking is that I saw a K&N commerical the other day which

is promoting their new "turboflow?" filter, it's a premenant filter which is

oil soaked I think and uses water to clean it when it gets dirty.

That's how all of the K&N filters work!

>Interestingly, they promoise a 15% increase in horse power just by merely

installing those filter.  If this is the case, then there's really no need

for the Henessy unit isn't it?  What do you think?

I think they are a little optimistic! You might get 15 HP more using the

large cone filter, and maybe 10 more using the stock type K&N filter in a

relieved (holes drilled) airbox. Scott.

------------------------------

Date: Thu, 18 Jan 1996 18:59:08 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: P/N needed for 9C1 door handle

Message-ID: <960119025907_73145.1566_FHD39-1@CompuServe.COM>

Mike,

>I am wondering can you give me the parts number for the 9C1 door handle kit

and how much did you paid for it?

I don't have the numbers as I have not even picked up the parts yet. I'm not

even sure I will have all of the necessary parts until I get them and see

how they fit!

>Also if you have the parts number for the front brake air duct handy I

would apprcieate it if you can give me that info as well.

That I have:

Part number   Price   Description

------------------------------------------------------

10220593    $ 2.84   Front LH air deflector (SEO 9C1)

10220594    $ 2.84   Front RH air deflector (SEO 9C1)

>Right now, my dealer still hasen't figure out how to get the parts number

from the RPO code and SEO code.

You need a new dealer, That info is literally staring them in the face as

they look at the catalog!! If they look at the proper group for the air

deflectors, they will see that there are standard and 9C1 versions, and they

are actually referenced that way right in the text. You do know that you can

get Chevy parts from any GM dealer. They are now all on computer systems and

any GM dealer can look up and sell parts for any GM make. Scott.

------------------------------

Date: Thu, 18 Jan 1996 19:59:46 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Oh no SNOW,here we go again<G>

Message-ID: <960119035945_76165.1654_GHM36-2@CompuServe.COM>

In Impala digest 242 Bruce Gordon wrote:

>>>That means you could drive our Impala, or even a NASCAR Grand National Monte

Carlo, with slicks in a storm that drops 21" of snow overnight??  If I had

to, I could too, but I wouldn't choose to!!  There's an safety edge when you

drive narrower tires with a proper tread pattern that tend to "cut" through

the snow, i/o "floating" on the snow with a tires that have a substantially

wider surface area.  Anything can be done, if you want to try hard enough.  I

prefer to run with 15" tires, as the police choose to do,  with a more

agressive, and considerably deeper, tread pattern that eats through the snow.

<<<

Yup, sure can.<s> I drive my TF Dragster during blizzards all the time. I tend

to prefer my TA Funny Car during ice storms though.

Seriously, I agree that anything can be done if you try. All I am doing is

relating just how easy it is to drive my Impala SS, in the snow, even during the

"Blizzard of 96", without even the slightest hint of being unsafe. If I felt I

was putting myself or my passengers or other drivers or my Impala SS in any more

danger just because I choose to drive with the factory supplied OEM, M+S rated

tires on it, I would park the car and walk. I have confidence that my Impala SS

and my driving skill will get me thru any snowstorm safely and without any

duress.

I see no so called "safety edge" in changing over to snows. Don't you think GM

would put a giant warning label in the Impala SS, "WARNING DO NOT DRIVE IN

SNOW!" if they thought that the OEM tires would not be sufficient? Especially in

these times when every moron with an attorney is ready to sue someone every time

they think they can blame some deep pockets for a mistake that they made. GM put

these tires on the car knowing full well that these cars would be driven in all

kinds of weather. I beleive this is why they chose a M+S (Mud and Snow) rated

performance tire, over a strictly summer performance tire like a GSC or whatever

they use on the Vette.

Here is a little more food for thought, that convinced me NOT to put snow tires

on my Impala SS.

An associate of mine has (had?) a 94 Impala SS. He put on a set of snow tires

with 15" steel rims this winter. The size of the tire is the exact size

recommended for Caprice's. He was involved in an accident this winter, during

the first major storm. The insurance company looked at the car, and asked him

why the original 17" tires and rims were not on the vehicle. He told them he

switched over to 15"rims and snowtires so his Impala SS would be safer in the

snow. (obviously not as safe as he hoped). The insurance company is denying his

claim because he "tampered with the original design of a safety device on the

vehicle as received from the factory." By switching from 17" rims to 15" rims

and narrower tires, he modified a part of the ABS system (considered a safety

device) which was designed for 17" rims and tires. He argued that the spare tire

was only a 15" tire. The insurance company's response was that the spare tire is

just that, a spare, which is to be used only to get you to the nearest repair

facility to repair the flat tire.

GM agrees, and is backing up the insurance company.

I hope this is the last time I have to explain my views and experiences with

driving in the snow with the tires that GM decided to put on my car. I am sure

that the engineers at GM spent lots of time contemplating what tires would be

put on this fine automobile. I know that GM does NOT want to open itself up to

any more lawsuits over anything. If these tires are good enough for all of the

engineers and attorneys at GM, they are good enough for me.

Kurt K

'95 Dark Cherry Impala SS.....no mods........yet......17" BFG M+S rated original

equipment tires.<BG>

------------------------------

Date: Thu, 18 Jan 1996 21:40:05 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Black bodies shed heat

Message-ID: <960119054004_73145.1566_FHD38-6@CompuServe.COM>

Rob,

>As far as black versus chrome, I think the chrome will reflect more engine

heat (thereby probably feeling warmer) than the black, which will absorb

heat and feel cooler (because of the heat transfer to the rushing air

through it). Am I missing something in this equation? This is from a vague

recollection of my heat transfer class.

A black body is the most efficient heat radiator possible, so if you want

something to shed heat, paint it black! That is why radiators and other heat

exchangers are usually painted black. A shiny chrome or white body will

retain far more heat. Plastic or rubber is also a much better heat insulator

than metal. The use of black plastic or rubber in the induction system is no

accident, and is actually very good engineering. Personally, I think the

factory square plastic front mounted resonator is fine. If you really feel

the need, then seal the internal resonator ducting, but I really don't think

it creates much if any restriction.

The big restriction is in the airbox itself, and that is the area which

needs the most attention. A large 8" cone type K&N filter will add some

noticible HP, although I don't like the non-factory appearance. For a

factory look, use the stock type K&N filter and drill large holes in the

bottom of the airbox housing to let in more air. This will still not flow

nearly as well as the completely open larger cone filter, but it does pretty

good and looks completely stock. Scott.

Joe,

>On another note has anyone seen a SS with Tan interior. Theres a dealer

here in Indianapolis with on e on the lot.

I don't believe it came from the factory that way, the dealer probably

changed it. If you could get me the VIN number from that car, I would really

appreciate it. If indeed it is a genuine factory original Impala with

a Tan interior, that would be extremely rare (as in maybe the only

one in existance), and I could confirm that via the VIN number! Scott.

Rush,

>On the front of the car, dead center right in front of a radiator, there's

a two prong connector that comes from the wiring harness, just dangling

there.  Anyone have an idea of what it's for?

That's for the front mounted outside ambient temperature sensor for

the Buick Roadmaster electronic climate control system. As you can

see, they use the same harness for both the Roadmaster and the

Caprice, even though the electronic climate control system is not

available on the Caprice. Scott.

Troy,

>with a roll type odometer, does the F body still retain a speedo cable or

is it electric by some means?

All GM speedo/odos are now electric, including the mechanical roll type odo

in the Camaro.

A Delco service center can change the mileage on the new cluster to match

the vehicle. I like D&B Radio (a big Delco service center) here in Addison

IL. That is who all of the Chicago area dealers use, and they are good.

>also, have you loosely priced the blazer console yet?  ball park?

Ball park I would say $300.

>Re: the heads up display, wasn't there a Gran Prix a few years back that

did a factory HUD?

The Grand Prix and Bonnie still have it as an option today ('96

models). It could probably be added to the Impala very easily. Oh no,

not another option to add... <g> Scott.

Jeff,

>He says his differential is making a horrible grinding noise whenever he

makes sharp turns in a parking lot. He has tried 2 bottles of the additive

with no result. The dealer he took it back to doesn't seem to know what is

wrong. His car has about 20k on it and the noise has gotten progressively

worse since about 17k. He said its so loud he is embarrassed to park

anywhere or take anyone with him in the car. Could it be the spacer problem

someone alluded toin  earlier digests only worse?

The real problem here is the dealer! I mean come on, they need to rip that

sucker apart and re-build it. The assembly tolerances in the 8.5" rear ends

I've seen leave a lot to be desired. The pinion depth (a critical setting)

was off .004" in my axle, which is actually quite a bit. I corrected the

depth problem by installing the correct shims when I installed the 3.42

gears.

>I told him to make the dealer fix it under warranty, but have them slip in

the 3.42 gear while they had it apart.

That is the best advice you could give him! Scott.

Scott,

>Quick question about the O2 sensors aft of the catalytic converters on the

'96 B-cars (Impalas): Do they function merely as a switch to signal the PCM

to set a code when the cats' efficiencies are below set standards...or do

they have some effect on air/fuel mixture control as do the primary O2

sensors??  In other words, if the rear O2 sensors detected "failing"

converters (too little oxygen), would the PCM lean out the mixture and/or

retard timing in an attempt to compensate, or would it just set a code??

As far as I understand, they are only to measure catalyst function and

efficiency, and the 2nd set of O2 sensors have nothing to do with the

air/fuel ratio. In other words, if you removed the cats, they would just set

a code. It may be possible to create an O2 simulator that could be used to

simulate the function of the second set of O2 sensors in case of cat

removal, for off road use only, of course. (John S. if you're listening what

do you think?) Scott.

------------------------------

Date: Thu, 18 Jan 1996 21:40:57 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: misc stuff

Message-ID: <960119054056_73145.1566_FHD38-11@CompuServe.COM>

Tim,

>I am planing to order a 1996 9C1 at the end of the summer, and hope to get

one of the last run produced.

I really think you are too late to order one, as in speaking to

Chevrolet directly as well as with the most knowledgable fleet dealer I

know, I have been informed that '96 9C1 production has been sold out

for some time now. Even if you were a legitimate law enforcement

organization trying to get one, they are simply sold out. Of course,

if you find out differently, please let me know, as I would be

interested in ordering one also! Perhaps the factory has reserved

orders for Canada, in that case I'd drive up to order one! <g> I hope

indeed that you can get a new one, but I don't think your chances are

very good.

>How do you aim the T84 export headlamps?  Do they aim in the same way as

described in SAE J599, i.e. with a screen at 25 feet.

That's basically the way I did it. Only I also make final adjustments by

feel and road testing.

>Do they also have the three points to mount a headlamp aiming device?

No they don't, and that is one reason they do not meet U.S. legal

requirements.

>Not wanting to start the DRL thread again, but has anyone considered

hooking the 5 watt positioning bulbs to a power source that is live in the

run position.

You could do that, but the 5w light is simply too dim to be visible during

the day. Of course I would never have DRLs on any of my vehicles, and will

disable any that come with them. Let's just say I am anti-DRL!

>This is the procedure I use every time I change my axle lube, and it

eliminates the clunking for about 12,000 miles.  It is interesting, but I

knew someone who also owned a new 1988 9C1 with GU4 and G80, but he never

experienced these noises.

If you are not using synthetic lube, you should! That will definitely help

in the noise and wear dept. I have found that the assembly of the 8.5" axles

seems to be very imprecise. Mine was off by .004" in pinion depth, which is

the most critical rearend setting. This could be responsible for noise, and

excessive wear. I corrected the problem when I installed the 3.42 gears. I

have a friend who has done literally dozens of 8.5" rearends in 9C1 cars,

and he says that this is a very common problem, that is they don't use the

precise sized shims they should, and instead pick a common size most of the

time.

>BTW on the gasket, the correct and older design GM gasket was 3993593. My

parts guy was able to determine by computer search that there are none left

with any Canadian dealers, but that there is 200+ with US dealers.

Thanks for the info, I'll definitely check out this gasket. If it is the one

with the scalloped ends, that might have been proper for the older 8.2"

corporate axle, but may not be correct for the 8.5". I will have to look

into that further. In any case, both Fel-Pro and Victor make the proper

gasket for the 8.5" axle with the oil holes. You should also know that for

many years Saginaw (American Axle) did not use gaskets at all, just silicone

sealer. It was when they went back to gaskets from using sealer that they

started having serious axle wear problems, and it has been due to the gasket

covering the oil holes. The sealer actually works great if you don't want to

get a gasket, and of course does not cover the oil holes.

>On why the rear wheels are closer to the front of the fender then the

back...

Interesting info on the wheelwell design and wheel placement. The placement

does look a little strange, but I agree that it was indeed designed that

way!

>This summer I read in one of the trade journals that someone by the name of

Chrysler wanted to buy the B body tooling and move it to the closed Hyundai

plant in Bromont Quebec.

The person was Dick Chrysler, and the car was to be called the "Cruiser".

Popular Science a year or so back had a big two page spread (picture) of the

thing, and detailed all of the (expensive) features it was to have. The

entire project was stillborn, and I must say it never had a chance of

happening, and was a pipe dream from the beginning. He would have never been

able to sell the car for a reasonable amount of money, and the govt. works

on low priced bids.

The main reason Chevy outsold Ford 2 to 1 in police cars was not that the

Chevy was so vastly superior to the Ford (although indeed we know it was!

<G>), but was because Chevy underbid Ford. That is also partly why the

Caprice is dead, although Chevy sold 60K+ 9C1 cars each year, they sold them

so cheap that they could not really make enough of a profit on them to

justify continuing production. Immediately after the last '96 9C1 orders

were accepted, Ford raised the Crown Vic cop car price by about $3K!

Unfortunately most departments will have to pay, since the front drive

Lumina is all Chevy has to counter offer, and front drive is a disaster for

police cars. The only hope for Chevy is the Tahoe police package, which I

think will be too big and expensive. I can't imagine using the huge Tahoe as

a cruiser!

>This week I went to a local tire shop, and asked about blackwall GT+4 225

70HR15.  This is the tire my car came from the factory with. The guy only

found a listing for white outline lettered.  I asked him why he couldn't

find a listing for the blackwall, since it's a standard police tire. His

response was: "I can't sell 'em to you".  When I asked how come, he said

"cause it's illegal".

That sounds like Bovine Scatology, as there is nothing illegal about

them! Besides, you should get P275/60-15 blackwalls for your '88 9C1.

That is what I have on my '88, and they not only fit perfectly, they

are the same diameter as the OEM tires, so your speedo will be

accurate. The monster blackwalls with the cop hubcaps really

accentuate the "Fed" look, and improve the handling as well! So does

the 1.5" diameter monster Herb Adams swaybar hanging under the rear

axle, in full view from behind. <g> Scott.

------------------------------

End of IMPALA Digest 244

************************

IMPALA Digest 245

Topics covered in this issue include:

1) keepers = locks

by Scott Mueller <73145.1566@compuserve.com>

2) Re: IMPALA digest 244

by Steve Das <sdas@sat.net>

3) Listproc Testing

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

4) Re: RPO code?

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

5) 6N1 door lock

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

6) JET's response to Scott Mueller

by Ray Berilla <writes4u@ganet.net>

7) 1LE F-Car Elbow

by Franklin Poole <franklin@intersurf.com>

8) National Assoc. 94-6 Impala SS/Regional contacts

by Rock528@aol.com

9) impala problem database

by Rock528@aol.com

10) Various

by Rock528@aol.com

----------------------------------------------------------------------

Date: Thu, 18 Jan 1996 21:40:36 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: keepers = locks

Message-ID: <960119054036_73145.1566_FHD38-10@CompuServe.COM>

Bruce,

>Scott, I REALLY HATE TO BE A PEST, but I'm a bit confused about the

grinding. So, please bear with me for a minute. Locks and keepers have been

used synonymously throughout various digests, since no corrections have been

posted otherwise, I'm under the assumption they're the same.

Yes they are indeed the same. Locks = Keepers, and Retainers = Caps.

>Now comes my confusion.  In our latest digest 242,  you responded to my

questions about grinding down the 16  Crane locks (keepers) by .040, as

posted above, by stating:

>>No! The crane offset locks move the groove for the valve +.050 inches

raising the retainer and the spring installed height by that amount. I then

shaved .040 off of the top of the retainer, which has no effect on the

groove position. I just made the retainers narrower to clear the valve

guided rocker washers.

I mistakenly said "retainers" here when I meant "keepers" (locks).

>O.K., was it the 16 keepers (locks) that were shaved by .040, as posted in

digest 237, or the top of the retainers to make them narrower to clear the

rocker washers??

Sorry for the confusion, I must have been tired! It is definitely 16 of the

32 locks which get ground down. The retainers are left alone. You are wise

to be absolutely positive about every move on something as important as the

valvetrain! Scott.

------------------------------

Date: Fri, 19 Jan 1996 05:38:42 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 244

Message-ID: <199601191339.HAA00400@raven.sat.net>

>Date: Thu, 18 Jan 1996 21:40:57 PST

>From: Scott Mueller <73145.1566@compuserve.com>

>To: <impala@east.xsis.xerox.com>

>Subject: misc stuff

>Message-ID: <960119054056_73145.1566_FHD38-11@CompuServe.COM>

>

>Tim,

>

>>I am planing to order a 1996 9C1 at the end of the summer, and hope to get

>one of the last run produced.

**************************************************************

I am told that the 9C1's I wrote about a week or so, the the local dealer

has for sale, are the last of the last.  In fact, the dealer ended up with

them because an agency ordered like 18 and they only made 7.  The agency

cancelled the order since they couldn't get all 18.  This dealer bought them

since they knew they would sell.  So, these 7 may well be the last of the last.

Steve

------------------------------

Date: Fri, 19 Jan 1996 08:15:23 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: Impala SS <impala@east.xsis.xerox.com>

Subject: Listproc Testing

Message-ID: <Chameleon.960119111609.hollenbe@mustang.east.xsis.xerox.com>

I'm having some trouble with the list processor.  This test message

is an attempt to figure out what's wrong.

==================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street, Lower Lobby, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

==================================================

------------------------------

Date: Fri, 19 Jan 1996 10:08:28 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Rock528@aol.com

Cc: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: Re: RPO code?

Message-ID: <Pine.SOL.3.91.960119095850.2518A-100000@netinfo.ubc.ca>

On Thu, 18 Jan 1996 Rock528@aol.com wrote:

> Hi Mike, the RPO is 6N1 and the door lever is pn 16622475 ($2.00=/-).  They

> also listed a few other 6N1 parts in the computer like the lock module.

> "module" sounds expensive..  I've asked scott if he did this mod yet to see

> what other parts are involved.  He is busy and won't get to it for a few

> weeks.  Let me know if you get into it and see other stuff is needed.  I'll

> look at the microfiche tomorrow also.  Troy

Thanks Troy for the info.  I just phoned up the dealer,

appearently in order for the whole 6N1 pacakge to work you need a bunch

of thing beside the rod which is the part you have listed above.  The

center piece for this a new door lock, which itself costs about $250 CDN,

the ball park for the whole package is about $280 cdn or so.  And since

they are stocked in the States and takes about two weeks to get to B.C. I

will pass it this time around.  Originally I planned to have them install

next Tues, when I will bring the car in for replacing the outsider

mirrors that were damaged due to vandalism.

BTW, just FYI, yesterday I discovered something was leaking

underneath the car.  The fluid is a very light pinkish fluid.  I

immeidately suspect it's the transmission that's leaking.  But when I

checked the power sterring oil box I realize the PS oil are just as light

pinkish as well.  I will lets you know what the heak is wrong with it

when I bring it in next week. Can you belive it, my car is less than 500

miles old, and it's been in service three times in the last three week already.

(One for alignment, one for rough idle which turned out to be spark plug

wire fell off, now for the leak) I wonder what's next?

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red

------------------------------

Date: Fri, 19 Jan 1996 10:13:08 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: 6N1 door lock

Message-ID: <Pine.SOL.3.91.960119100904.2518B-100000@netinfo.ubc.ca>

Hi everyone:

One more thing on the 6N1 door lock.  The 6N1 package poped up

under the regular RPO catagory in my dealer's computer.  So I guess for

those of you who are ordering your 96's you might try to order it and see

if the factory will accept it.  If they do it would be quite worth while

as the option is only about $25.  But to try to put them on later would

cost you about $250 just on parts alone.

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red

------------------------------

Date: Fri, 19 Jan 1996 11:02:15 PST

From: Ray Berilla <writes4u@ganet.net>

To: Impala list <impala@east.xsis.xerox.com>

Subject: JET's response to Scott Mueller

Message-ID: <199601191903.OAA26570@s1.GANet.NET>

-- [ From: Ray Berilla * EMC.Ver #2.5.02 ] --

I was curious to see what JET's response would be to Scott Mueller's

evaluation of their chip so I forwarded his comments to their representative

who can be contacted at JETCHIP@AOL.COM. Scott's comments and their response

follows:

------------------------------

SCOTT MUELLER said:

John,

>can you achieve the same results using JET as you can with Hypertech with

regard to fan turn-on temperatures, rear gear swaps, shift points & firmness

, etc.?

No way! Most people don't seem to understand that the JET "chip" or "PCM" is

really no such thing. All it does is intercept the sensor signals on the

grey PCM connector, which contains almost exclusively sensor input data.

Here are some facts about the JET system that people must understand:

-It cannot possibly reprogram the PCM

-It cannot control the vehicle independent of the PCM

-It cannot operate the engine outside of PCM controlled parameters

-It cannot control the transmission

-It cannot control the fans

-It cannot control the air/fuel ratio

-It cannot control the spark advance

-All it can do is intercept and modify sensor data, "fooling" the PCM

The sensors it can mess with are the O2 sensors, IAT (Intake Air Temperature

) sensor, TP (Throttle Position) sensor, MAP (Manifold Air Pressure) sensor,

ECT (Engine Coolant Temperature) sensor, and Transmission temperature range

switches. Those are pretty much the only wires in the grey PCM connector the

JET device plugs into!

What this JET device does is to intercept these sensor readings, and to pass

different readings to the PCM in order to make the PCM compensate for the

different reading. For example, all PCMs are setup with internal spark and

fuel tables that vary the spark and fuel based on temperature, engine load,

and other sensor variables. These tables are programmed to richen the

air/fuel ratio if the ambient and engine coolant temperatures are low. So if

the JET device intercepts the IAT and ECT sensor readings, and adjusts them

lower before passing them to the PCM, then the PCM will be fooled into

selecting a lower temperature fuel calibration from the table that richens

up the mixture. Note that since this technique fools the PCM, it will also

fool a technician using a scan tool to troubleshoot a sensor problem. The

PCM will be reporting improper readings, and the technician will be

replacing sensors that are not bad.

Also note that contrary to some of the bogus hype that JET seems to be

passing along, their device does not seem to have a 20 MHz RISC

microprocessor, nor is it able to operate the independently of the PCM. They

seem to imply that their unit can advance the spark or change the fuel ratio

beyond what the PCM can normally do. Obviously this is false, as their unit

simply fools the PCM into reading different sensor values, and the PCM

software compensates accordingly. The PCM is still in full control of the

system, and only PCM pre-programmed parameters will be used for spark, fuel,

etc, according to what the sensor inputs indicate.

On the other hand, companies like Superchips, Hypertech, and Z-industries

can actually reprogram the PCM, altering the data tables so that differen

spark and fuel curves can be run. These companies can also re-set the

temperatures that the electric fans operate at, they can control the TCC

(Torque Converter Clutch) lockup speeds, alter the top speed cutoff, engine

rev limiter, change transmission shift points, shift firmness, and literally

everything else that is under PCM control. There is a big difference between

actually reprogramming the PCM and just intercepting some sensor data and

fooling the existing program to select different but pre-existing parameters

.

>I talked to a rep at Summit recently and he recommended the JET PCM over

Hypertech.

I have also talked with Summit reps and techs about different items and I

can assure you their advice is worth the price (free). They are there to

sell, and if you try to get anything other than facts, you will likely be

mislead, and they often have a hard time with just the facts.

>He said "you'll really only feel the Hypertech when you're at the drag

strip, where you'll feel the JET more even in daily driving."

The Hypertech unit is very noticible at part throttle. It was so much of an

improvement for me, that I would have recommended the device had there been

no full throttle improvements whatsoever. Other companies like Superchips

and Z-industries also reprogram the PCM, and make part throttle changes as

well.

>I know JET offers a fan switch kit and a cooler thermostat, but I haven't

heard any mention of reprogramming for the rear gears.

That's because they can't reprogram for different rear gears, in fact they

cannot reprogram anything! That is why they have to offer a fan switch kit

while the others can simply reprogram the temperature the PCM will turn on

the fans.

-------------------------------------------

JET's Response:

Ray,

Ok lets get down to it........sometimes this stuff gets out of hand !!!

1.   I'm not going to get into a pissing contest with anyone over

Engineering designs !

Everyone has there own theories and at this point we believe ours works

the best If he thinks he      can do it better or prefers the competitions

device over ours than please buy one from them .          But please dont

try and to explain to other people how an Automotive computer works when you

really know nothing about it, we have 4 engineers that do that stuff

all day long, along with a           staff that does dyno testing and

development and the results to prove we have a pretty good idea      of what

were doing.

2.   Results are the bottom line! We have proven time and time again that

the PCM

outperforms the competition and after all is said and done thats

really what counts !

3. I WILL address the following BS ....there is no such thing as a lower

temperature fuel

calibration unless the car is in open loop (cold start), the PCM does

not effect Scan

tools or diagnostics of the car, we DO use a 20mhz RISC processor.,

because we

do not reprogram the stock computer we have to control fans externally

(again our

choice of design), and it is true that we can not change gear ratio

calibrations

because we dont reprogram, although the gear calibration can be done

with any

Tech 2 or CAMS machine at the dealership (or simply send it to us !)

4. Summit Racing employes a large staff of people that are well trained in

the products

they sell , although for the best tech info you should contact the

manufacturer.

5. "The Hypertech unit is very noticible at part throttle ".....The

Hypertech unit does NOTHING to the     fuel and ignition tables AT ALL, not

at part throttle , not at WOT period ! The only thing it does is     raise

rev limiter ( a useless mod in an LT1 they dont make any more power up there

, unless         highly modified) and turn on the fans at a lower temp !

This one we know to be a fact ...we look      at the competitions stuff,

they only change 5 bytes of info. You can get the same .15 in the 1/4

that the Hypertech unit does by installing a 160 thermostat, but make sure

you test it before the     car goes past 160 deg or it wont work !

If I can be of any further help please let me know....

Thanks, JETCHIP

------------------------------

Date: Fri, 19 Jan 1996 11:32:35 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: 1LE F-Car Elbow

Message-ID: <30FFF1D3.2B61@intersurf.com>

Several people have had trouble with the 1LE intake duct to replace the

one that works with the 'homeplate' on the 94-96 Impala SS.  First: my

information comes originally (of course) from Scott Mueller and I'm

adding my personal preferences and experiences.

part numbers

25147210	'94 1LE w/o vent nipple ($95.63 retail)

25147187	'95 1LE w/vent nipple	($155 retail!!!!)

Parts are described as "air cleaner outlet rear duct"

Both part numbers are currently (1/19/96) valid.

Just got off the phone with my man Hank and he says that the '95 part

number listed above is in his export section of the price list.  It does

not have prices but the part number 25147186 "with noise restriction"

(i.e. the cutout is cut out) lists as about $152.xx. I added couple of

bucks as a hedge.  I say DON'T EVEN THINK ABOUT THE '95 PART!

I say get the one without the vent nipple.  This allows you to pierce a

hole in the duct on the underside and hide that vent from view! Much

cleaner than having it sit on the top/side as yet another vacuum hose...

To install the vent from the distributor I chose the first rib from the

throttle body, picked a point about 20deg from the bottom toward the

driver's side and carefully drilled a hole (i forgot the size, exactly)

all the way through the rib that is small enough to hold the little blue

vent tube elbow tightly.  Then I trimmed the lenght of the vacuum hose

accordingly and connected it. Voila! One less vent hose in view!  Both

of the 1LE intake elbows come with the MAT installed. I went as far as

to remove it and compare part numbers, same on each.

On the bottom of the elbow (the 1LE) there is an attachment point for a

resonator (a tube) that is not cut out.  I suggest using a razor knife

or razor blade and carefully cutting that tube off flush with the rest

of the elbow.  This cleans up the bottom of the elbow and you're left

with a beautiful job, and an opportunity to tell your less informed

friends that "yes this car is equipped like factory!" which is mostly

true since these are factory parts!

Have fun!

If three people e-mail me with requests for these elbow I can get the

them at a 30% discount. That is $66.95 + $5 my effort + shipping to you.

Of course if you can talk you're local dealer into 30% like a good

dealer then that's always a better deal.

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Fri, 19 Jan 1996 17:05:02 PST

From: Rock528@aol.com

To: foxhound@txdirect.net, impala@east.xsis.xerox.com

Subject: National Assoc. 94-6 Impala SS/Regional contacts

Message-ID: <960119200108_201528546@mail02.mail.aol.com>

> I live in San Antonio and would be willing to be a Point of Contact if no

one else has offered....

Jeff,

I do have a couple in TX but it's a big state.  Basically, it is a point of

reference if someone wants to get together for a weekend show, etc.  You

could keep the names of SS owners in your area or start a regional club.

For example, I keep all of the local Central FL guys names and numbers in my

registry.  We get together about once a month and  I give a call to the

regulars and any new members.  To give you an idea of how popular this is we

started with 3 original impala owners last summer.  I now mail out over 220

newsletters.  You wouldn't believe how many times a newsletter can be faxed

before it is unreadable.  I get calls from guys (& girls) that have a copy of

a copy of a fax of a copy and want to get involved in our Impala SS

Association.

This is what I have so far for regional contacts:

Pick a region, state or a couple of smaller states and sign up as the

contact.

UNITED STATES:

NORTHEAST

MA, Harris, Scott, 508-582-6114

PA, Gordon, Bruce, 717-562-7868

NORTHWEST:

SOUTHEAST:

FL, Central - Willrick, Troy, 904-253-6638

SOUTHWEST:

CENTRAL NORTH:

IL, Oak Brook - Kelly, John P., 708-964-6499

CENTRAL SOUTH:

TX, Houston - Clyde Twine, 713-552-2172

TX, Arlington - Lambert, Don, 817-557-1529

TX, San Antonio - Gatcha, Jeff, 210-679-6870

LA, Baton Rouge - Franklin Poole, 504-338-0942

CANADA:

SWITZERLAND:

Jenni, Robert - jennir@thpo02.ch.unisys.com

AUSTRIA:

Kempf, Thomas, 011/43/5574/49710

SWEDEN:

Jernstrom, Anders, 46-8-971312

------------------------------

Date: Fri, 19 Jan 1996 17:06:13 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: impala problem database

Message-ID: <960119200111_201528597@emout04.mail.aol.com>

I am interested in logging specific oddball type problems on the

Impala/Caprice series.  Not the usual new car stuff but the more mechanical

things like the steering pump failures, rear differential problems and such.

If you have had a specific premature mechanical problem please email it to

the impalaclub@aol.com address and I will record it.  If anything, we can

build a track record of specific problems inherent to the Impala/Caprice

line.  Thanks, Troy

------------------------------

Date: Fri, 19 Jan 1996 17:19:08 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Cc: 73145.1566@compuserve.com

Subject: Various

Message-ID: <960119201629_121744541@mail06.mail.aol.com>

Hello,

Crane Cams:

I went to Crane Cams Factory R&D dept. today here in town.  I've been trying

to get them to use my car as a test vehicle for their new LT1 cam for

emission testing.  Talked about rockers, ignitions improvement and the LT4

heads.  Lots of good info.  One thing I didn't know was that the factory LT4

rockers will not fit on the LT1 heads.  The LT1 uses a 3/8 english studs and

starting with the LT4's, the studs are metric.  Keep in mind if your doing

head improvements.  Troy

6N1 Handle - Drivers Door Unlock

Inside driver door handle releases lock when opening drivers door.

Parts needed:  This is the best I can cipher from reading the headsplitting

microfiche:

1991-3     Door rod     pn 16622475

1994-6     Door rod     pn 16623261

1991-4     Lock Module (with pwr locks) pn 16625931 (incl. 2 clips, bolt)

1991-4     Lock Module (without pwr locks) pn 16625935 (incl. 2 clips, bolt)

1995-6     Lock Module pn 16630193 (I guess they only have pwr locks)

Note the split in years on the rod vs. the module.  No price on module.  Troy

BELL & HOWELL MICROFICHE

I called today reference GM parts on cd-rom.  They have a system but it

supposedly only works on their Bell & Howell computer system.  Back to the

microfiche, it's only eight bucks.  Last revision was November 1995.  Troy

TORSION BARS BY VSE

Matt Adams is taking over his dad's (Herb) business dba VSE by Matt Adams.

Look for a blurb in the next newsletter.  Front bar is 1 5/16" and rear is

1.5" as mentioned earlier.  He will discount for the group with enough

orders.  Hello tax return..  If anyone is interested in these, let me know

and we can preplan a bulk order for good pricing.  Regular price is 270.00

for the front and 220.00 for the rear complete.  Troy

------------------------------

End of IMPALA Digest 245

************************

IMPALA Digest 246

Topics covered in this issue include:

1) Lock module!!??

by David Hardy <dhardy@crl.com>

2) Production Date?

by Kurt Kemmling <76165.1654@compuserve.com>

3) TA cluster

by Scott Mueller <73145.1566@compuserve.com>

4) I'll be absent

by Scott Mueller <73145.1566@compuserve.com>

5) Jet rhetoric

by Scott Mueller <73145.1566@compuserve.com>

6) Stiffer rear springs

by Scott Mueller <73145.1566@compuserve.com>

7) Killer Swaybars!

by Scott Mueller <73145.1566@compuserve.com>

8) RE: IMPALA digest 245

by DScribner@hunter.com (Scribner, Dave)

9) FW: IMPALA digest 245

by mbailey@rinet.hunton.com (Bailey, Martin)

----------------------------------------------------------------------

Date: Fri, 19 Jan 1996 18:26:11 PST

From: David Hardy <dhardy@crl.com>

To: impala@east.xsis.xerox.com

Subject: Lock module!!??

Message-ID: <Pine.SUN.3.91.960119182437.12976B-100000@crl8.crl.com>

I understand the purpose of the rod to allow the unlocking of the door by

door handle...but why would we replace the lock module?   DO you need

that to make this mod?  or just the rod?

-Dave

------------------------------

Date: Fri, 19 Jan 1996 14:59:32 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: Mail List Impala SS <impala@east.xsis.xerox.com>

Subject: Production Date?

Message-ID: <960119225931_76165.1654_GHM64-1@CompuServe.COM>

I was wondering what the date that is stamped in black ink on the water pump

housing of my LT1 engine means? Is this the build date for the engine, or for

the vehicle?

If it is for the engine, where/how can I find the build date of my car?

Also, is there a difference between blocking homeplate and removing homeplate?

I asked this in a previous post, but got no response.

Thanks,

KK

'95 Dark Cherry Impala SS

------------------------------

Date: Sat, 20 Jan 1996 01:24:12 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: TA cluster

Message-ID: <960120092412_73145.1566_FHD30-5@CompuServe.COM>

Rush,

>Have you also looked at the current T/A cluster?  The top part of the

cluster seems like the same size/shape as the Impala, but it also goes

slightly below the top of the steering column on both the left and right

hand side.

Yes, I have checked and there is absolutely no way the TA cluster will fit

the Impala dash, it is not even close!

>If the only obstacle of installing the Camaro cluster is the outer casing,

couldn't you swap this with the current Impala cluster (haven't seen it so I

don't know if this is feasible). If this could be done, you could actually

rewire your connector from the inside, without having to build another

harness.

The Camaro cluster will fit the Impala dash perfectly from the front

perspective. The visible face is the same width, height, and contour as the

Impala cluster, they are almost a perfect match. The rear mounting portion

of the Camaro cluster would have to be trimmed quite a bit, but this would

not affect it's operation in any way. There is really no obstacle to

installing it other than the cost of the cluster itself, and your ability to

do some plastic cutting, bracket fabrication, and wiring up a jumper harness

from the Impala cluster connectors to the Camaro cluster. It would not be

possible to put the Camaro circuitry in the Impala cluster housing, as it is

wider in the rear, besides I would not want to cut up the Impala cluster

unnecessarily!

In any case, the cluster job will be secondary to installing the floor

shifter. I need to have access to a '96 Impala that I can get either on a

lift or some jackstands in order to do some research. The Camaro cluster

cannot be installed until the floor shifter is installed first! Scott.

------------------------------

Date: Sat, 20 Jan 1996 01:23:52 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: I'll be absent

Message-ID: <960120092351_73145.1566_FHD30-2@CompuServe.COM>

Mark,

>Will you still be in touch with the digest when you go to Boston??  I feel

sure, when you are unavailable is when I will need to pick your brain most.

Anyhow, have a safe trip.

Unfortunately I'll be out of touch for the entire two weeks. So far it seems

I will be meeting with Scott H. and Curt P. from the list while I am there!

I'm looking forwared to meeting them and anybody else who joins us,

hopefully that will prevent me from going into total SS withdrawl!

------------------------------

Date: Sat, 20 Jan 1996 01:24:22 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Jet rhetoric

Message-ID: <960120092422_73145.1566_FHD30-7@CompuServe.COM>

Ray,

>JET's Response:

>But please dont try and to explain to other people how an Automotive

computer works when you really know nothing about it, we have 4 engineers

that do that stuff all day long, along with a staff that does dyno testing

and development and the results to prove we have a pretty good idea of what

were doing.

If that product is the best their engineers can do, then they need to spend

a little more money on engineering than they do on slick marketing and hype.

I know pretty well how an automotive computer works, (or any computer for

that matter <g>), and according to what I hear I also seem to know the

limitations of their own product better than they do. I am also not the only

one with PCM knowledge or computer expertise that feels this way.

If you want to talk to people who really know the GM PCM system, and how to

modify it PROPERLY through recalibrating or reprogramming, rather than

making an end run around the PCM program by feeding it bogus sensor

information, then see Superchips, Hypertech, or Z-industries. JET on the

other hand seems to operate under the principle that marketing and sales

have priority over engineering, research and development. Scott.

------------------------------

Date: Sat, 20 Jan 1996 01:24:08 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Stiffer rear springs

Message-ID: <960120092407_73145.1566_FHD30-4@CompuServe.COM>

William,

>Scott, did you ever look into adding the Level Ride Control from a

Roadmaster or Fleetwood to the Impala?  The rear of my Impala sags a bit

(lots of heavy stereo gear in the trunk) and my fuel lines are scraping

against my differential housing.

Yes I did and I would not recommend it. The Impala has good DeCarbon shocks,

while the load levelling shocks are just a standard air shock. This might be

feasable if you get the 7,000 lb. GWVW air adjustable shocks from the

Limo/Hearse load levelling package. These would be very HD and might be as

stiff as the factory shocks.

Instead of the expensive factory load levelling system which requires the

use of non-performance air shocks, I wanted to use the premium factory SEO

8X3 Bilstien shocks. To increase the rear load carrying capability, I added

higher GVWR factory springs from the police package, along with Air Lift

bags in the springs. This makes my Impala sit with a very slight rake, the

amount of clearance in the rear wheelwell matches the front. I hate a saggy

look, and wanted to be able to carry heavy loads without bottoming out. The

stiffer springs make it so I can run the air bags virtually deflated with

normal loads, and the car still sits nice. If I add a heavy load, I can add

air to the bags, and the car will still sit normally.

I mounted the air fitting for the bags in a pre-existing hole behind the

license plate. The lines were run through the frame rails, in factory style

conduit, and the whole installation looks factory. As I prefer an even

stiffer ride, I am now experimenting with factory springs from a '90 Caprice

9C1, which had a higher rate than the '91 and up springs.

In summary, I'd recommend you install stiffer factory springs, Air Lift bags

in the springs, and finally Bilstien or Koni shocks for the ultimate in

damping. Scott.

------------------------------

Date: Sat, 20 Jan 1996 01:24:50 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Killer Swaybars!

Message-ID: <960120092449_73145.1566_FHD30-9@CompuServe.COM>

Mark, Dave, and all,

I have been getting some questions lately on installing the Herb Adams

swaybars, so here is a file on the subject:

One of the best mods anybody can make to their '94-'96 Impala SS (or for

that matter any '77 or up GM B-car) would be the installation of the Herb

Adams VSE (Very Special Equipment) stabilizer (anti-sway) bars. These

monster bars will make your Impala feel much more stable and corner

incredibly flat, you might actually think you are driving a Z28! The Impala

will feel like it lost 1,000 lbs with these bars installed, and the

cornering limits will be beyond what you ever could expect for a vehicle

this large.

Herb Adams is the original designer and engineer of the '69 Pontiac Trans

Am, and he also did the '73-'74 455 Super Duty Trans Ams as well. He quit

Pontiac around '74 seemingly over the bean counter and government related

politics that hamstrung the whole Super Duty project. His current focus is

racing cars, and most of his products are of the "some assembly required"

type. <g> You can get the Herb Adams swaybars either direct from Herb

Adams VSE (their number is 408-649-8423, tell them I sent you!) or from

Summit racing, however a better version of the front bar is only available

from Herb Adams VSE directly.

You can get the rear bar from Summit racing (216-630-0200) under the Herb

Adams/Moroso name, part number MOR-86516 ('77-up B-car, cost $166.69).

They also carry the front bar under part number MOR-86015, however they

only carry the bar with heim joint end links. These end links will rapidly

wear out and rattle in my experience, and they are very expensive to

replace. Instead, if you purchase this bar from Herb Adams VSE directly

you can get the bar modified for tie-rod end links (included). The tie-rod

end links are even stronger than the heim joints, and they are greasable

and do not wear out.

I would definitely NOT order the front bar through Summit as you will only

get the version with heim joint end links that wear out rapidly and once

they wear, they start to rattle. Replacing them is almost as expensive as

purchasing the whole bar. Instead you should get the front bar direct from

VSE modified for tierod end links, and it is only sold that way through

VSE directly. The bar has to have different ends welded on to accept the

vastly superior (greasable) tierod links.

As far as installation goes, you have to consider that the Herb Adams bars

come with no attaching hardware, and there are really no instructions to

speak of, so you are on your own. If you need instructions, use the

factory service manual or the information I am giving here as a guide,

since these bars install in the same manner as the factory bars. The big

difference is that you CANNOT re-use the stock mounting hardware, or at

least you SHOULD not. Here is what I recommend:

To install the Herb Adams front bar (33mm) you absolutely CANNOT re-use the

stock rubber bushings. These are designed for only a 30mm bar, and simply

will not fit around the 33mm bar. I don't recommend using polyurethane

aftermarket bushings either (such as supplied by Herb Adams in their

optional installation kit), as in my experiences the bar will abrade the

inner hole, wearing it oversize until the bar begins rattling around inside

the hard urethane bushings. Hard urethane bushings also tend to squeak a

lot, unless you are constantly lubricating them.

Instead I would recommend a factory replacement bushing from another GM

application. The stock bushings feature a 30mm hole, are made of soft

rubber, and have no fiberglass liner where the bar runs through them. I

recommend you replace these with factory 32mm or 33mm High Durometer (hard

rubber) bushings that have fiberglass liners. The liners prevent wear and

totally eliminate noise as the bar rotates in the bushings. These are

available under GM part number 14094388 (34mm) or 26039956 (32mm).

The 34mm bushings have exactly the same external dimensions as the

original 30mm soft rubber bushings used in the Impala, however I have

found that because of their high durometer (hard) rubber construction,

they are sometimes difficult to compress in the stock Impala clamps. If

you find this to be the case, then I recommend you use the smaller 32mm

bushings. The smaller bushings will still fit around the bar, although a

bit more tightly, and their smaller external dimensions will also fit

inside the stock clamps a little easier. Either of these seem to be tailor

made for installing the Herb Adams 33mm front bar in the Impala! These

bushings are from the F-car parts catalog, by the way. The 34mm bushing is

from an '85-'92 F-car, while the 32mm bushing is from a '93+ F-car with

the special 1LE suspension.

Now, on to the brackets and bolts. The bushings I just listed have the

same basic form factor of the stock bushings (except the larger hole for

the larger bar), so the stock brackets can be used. The bolts, however

should be changed. The original bolts are a metric M10x1.5 30mm prevailing

torque bolt. I do not like these bolts at all and absolutely will not

re-use them. The problem is the prevailing torque feature means they have

distorted threads that will strip the threads in the frame if installed

more than once, as well as making it very difficult to "feel" how tight

they are when you are installing them, causing you to easily strip the

threads. I recommend replacing the stock bolts with non-prevailing torque

bolts of the same size. These will tighten much more easily, and you will

know when they are getting too tight.

If your stock threads are not stripped, then I recommend an M10x1.5 25mm or

30mm standard (non-prevailing torque) thread bolt with a small internal hex

cap head. The small cap heads will easily clear the bracket, and you can

use a large (8mm) allen key or allen hex tool to tighten them. Conventional

hex head bolts do not leave enough room against the bracket to work

properly, and you cannot get a wrench or even a thin wall socket on them

easily.

The absolute best bolt for the job I have found is available from a

company called Totally Stainless (800-767-4781). Their motto is "In

Stainless we Trust, In Chrome we RUST!" <g> They specialize in nothing but

stainless steel fasteners, and have an excellent catalog that every car

crafter should have. They have high strength M10x1.5 25mm bolts with hex

socket cap heads in pure stainless steel which will never rust or corrode,

and which do not feature the distorted threads of the stock bolts. They

are available under part number 1-1273 (4 are required). Use locktite 242

(blue) to prevent them from loosening, and to act as a lubricant when they

are being installed. These are what I recommend if you have not stripped

your stock threads. Tell Totally Stainless I sent you!

If you strip the threads in the frame, you will have to either get an

M10x1.5 nut (also available from Totally Stainless) on the inside of the

frame rail (which is very easy to do on the RH side, but difficult on the

LH side), or install a "nutsert" type fastener. Nutserts are special nuts

that are installed through the frame holes, and then a special tool

(included when you buy them) is used to crush them against the inside of

the frame rail, where they will stay. Then you can use a conventional bolt

to tighten the bracket. In most cases, installing the nutsert will require

that you drill the frame holes out to a larger dimension, but in the end

you have a much stronger set of threads than the stock setup.

If you are using the nutsert approach, then I recommend a hex cap head bolt

in whatever size is required by the nutsert threads. A 3/8 version would be

the closest to the factory metric size and would fit the best. The bolt

should be a 3/8" diameter by about 1-1/4" long, and should feature the

small internal hex cap head.

For the rear bar, the stock 7/16" by about 1-5/16" long bolts are clearly

inappropriate. They feature only 1-1/4" of thread length after the 1/8"

thick washer is installed, and the Herb Adams rear bar is 1" thick at the

attachment points. This leaves only about 1/4" of thread to engage the nut

in the control arm bracket (absolutely not enough!). Instead I recommend

you use a longer bolt with the same 7/16" diameter as the stock one.

Absolutely the BEST bolt for the rear bars are the ARP Grade 8 stainless

steel bolts. These are available in a 7/16" diameter by 2-3/16" long

version, that leaves over 2" of thread after the 1/8" thick washers are

installed. These are also available from Totally Stainless as part number

2-0852 (4 are required). These bolts are absolutely a work of art, in fact

they are aerospace quality and just beautiful to look at. I have extra ones

in stock just so I can occasionally pick them up and admire them, but then

I am not normal! <G>

If there is a problem with these bars, it is the lack of included hardware

and poor to nonexistent installation instructions. Hopefully the

information I have provided here will help those of you who are considering

these bars as until you have driven a car with them, you don't know what

you are missing! Scott.

------------------------------

Date: Sat, 20 Jan 1996 07:17:24 PST

From: DScribner@hunter.com (Scribner, Dave)

To: impala@east.xsis.xerox.com (impala)

Subject: RE: IMPALA digest 245

Message-ID: <1996Jan20.090000.1442.13488@diablo.hunter.com>

Members and SS Staff,

Every day an incredible amount of information keeps coming by "digest".

I save the digests on my hard drive because there is barely enough time

to use the bathroom, maintain an affair, and get my job done at work.

Does anyone support archiving the "communications" by topic?

Maybe a seperate file we could keep on the home page?

I can't keep up.....as my job is taking up my entire life.  I live to

work and work to live.  No play makes Scrib a dull boy....but I'm still

wanting to play someday soon.

Someday when I want to go play....I'd like to access this information

more easily by topic.

Scrib

dscribner@hunter.com

------------------------------

Date: Sat, 20 Jan 1996 06:30:46 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (Impala1)

Subject: FW: IMPALA digest 245

Message-ID: <1996Jan20.092100.1049.75309@rinet.hunton.com>

I don't know whether this price refers to the front bars with TYROD ends or

heim joint ends, but I do know that this is a *very* high price for the rear

bar.  I paid only $129.95 for the rear bar from my local speed shop.  As for

"complete," as Scott M. has pointed out earlier, for the rear bar,"complete"

means a bar with two holes in either end..  Period.  There ain't nuthin'

else to it.

TORSION BARS BY VSE

Matt Adams is taking over his dad's (Herb) business dba VSE by Matt Adams.

Look for a blurb in the next newsletter.  Front bar is 1 5/16" and rear is

1.5" as mentioned earlier.  He will discount for the group with enough

orders.  Hello tax return..  If anyone is interested in these, let me know

and we can preplan a bulk order for good pricing.  Regular price is 270.00

for the front and 220.00 for the rear complete.  Troy

------------------------------

End of IMPALA Digest 246

************************

IMPALA Digest 247

Topics covered in this issue include:

1) Intro/96RearGasket

by Ray Lopez <impalass@mail.erols.com>

2) Re: IMPALA digest 246

by Slawkoz@aol.com

3) Re: Oil Leak

by BGordon401@aol.com

4) Clarification of Crane and stock locks "keepers"

by BGordon401@aol.com

5) valve keepers (locks)

by Scott Mueller <73145.1566@compuserve.com>

6) Re: IMPALA digest 246

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

7) SEO switches where lighter is...

by jsalmon@mindspring.com (John Salmon)

8) P/S pump leak

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

9) Re: New GM Performance Parts, & VOTE!

by Kurt Kemmling <76165.1654@compuserve.com>

10) 6N1 option

by Rock528@aol.com

11) Re: impala digests

by Rock528@aol.com

12) Fwd: information

by Impalaclub@aol.com

13) Re: Impala problems

by Impalaclub@aol.com

14) ATI parts ordered

by Mark Boggs <70004.173@compuserve.com>

----------------------------------------------------------------------

Date: Sat, 20 Jan 1996 17:24:48 PST

From: Ray Lopez <impalass@mail.erols.com>

To: impala@east.xsis.xerox.com

Subject: Intro/96RearGasket

Message-ID: <199601210124.UAA05751@mail.erols.com>

Hello, My name is Ray Lopez and I just purchased a new "BLACK" 1996 Impala SS.

While reading most of the FAQ, I became concerned with the possibility of my

new SS having the incorrect rear-differential gasket. I took the plunge and

investigated with only 660 miles on my 96 Impala SS.

Conclusion: "YES" my 96 SS with a build date of 10/95 did have the incorrect

gasket.  I have installed the Felpro gasket along with fresh GM oil/additive.

RAY L.

------------------------------

Date: Sat, 20 Jan 1996 18:20:34 PST

From: Slawkoz@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 246

Message-ID: <960120212017_202353350@emout06.mail.aol.com>

Help!  My '95 Impala seems to be choking on something.

It seems to only happen in third gear at 3/4 throttle... the car seems to

"choke", it loses power suddenly and jerks, and immediately launches itself

again.

I have observed this at between 70 and 80 mph.  It does not happen

constantly.  Trying to duplicate this effect results in it going away, but it

seems only temporary.  The only sound that I hear is the hiccup of the

engine.

Any suggestions at what this could be?  My car is stock, the only mods are a

K&N 33-2057 filter and a coffee lid on the intake elbow.

My fuel pump was replaced at roughly 10,000 miles.  I now have 12,XXX miles.

Should I go to the dealer?  I like to at least seem to know what I am talking

about when I go there, so does anyone have any suggestions as to what this

can be?

------------------------------

Date: Sat, 20 Jan 1996 18:43:34 PST

From: BGordon401@aol.com

To: mkyang@unixg.ubc.ca

Cc: impala@east.xsis.xerox.com

Subject: Re: Oil Leak

Message-ID: <960120214332_301395920@emout05.mail.aol.com>

Mike, you wrote:

>BTW, just FYI, yesterday I discovered something was leaking

underneath the car.  The fluid is a very light pinkish fluid.  I

immeidately suspect it's the transmission that's leaking.  But when I

checked the power sterring oil box I realize the PS oil are just as light

pinkish as well.  I will lets you know what the heak is wrong with it

when I bring it in next week.

F.Y.I.  I also found an oil leak, except that it was spraying around the air

conditioner compressor, and balancer.  I took it to the dealer, and they

wound up replacing the power steering pump, and some oil lines.  All covered

for no cost.  If you need more details, I;ll be more than happy to help.

..Bruce / BGORDON401@aol.com

1995 Cherry Metallic...That FINALLY has a real oil guage, after battling to

find the resistor for two weeks!! :-)

------------------------------

Date: Sat, 20 Jan 1996 18:45:37 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Subject: Clarification of Crane and stock locks "keepers"

Message-ID: <960120214511_301396594@emout04.mail.aol.com>

Scott, you wrote in Digest 245 in response to my question:

>I mistakenly said "retainers" here when I meant "keepers" (locks).

..and...

>It is definitely 16 of the 32 locks which get ground down. The retainers are

left alone.

THANKS MAN!!  I though I was going nuts for a moment.  I must have read the

digests at least 10 times trying to make sense of the proper proceedure to

use for the Crane 1.6 Roller Rocker Mod.

O.K., could you please give your valued opinion on my original question that

I posted

back on Digest 242 about using the stock locks for the intake valves instead

of grinding the Crane locks.  What I wrote to you at that time was:

>Now to a curiousity question about the grinding of the locks(keepers) for

the intake

valves.  You took off .040 from the Crane locks, which come +.050 over stock.

That leaves a +.010 difference over the stock locks after grinding.

According to the pressures you gave in previous digests, that +.010

difference of the Crane locks after grinding, compared to the stock locks,

would only make about 3 lbs difference in pressure.  Which leads me to my

question(s).

>Scott, would it make sense to use the Crane locks for the exhaust valves,

and

the stock locks for the intake valves, given that there would be about a 2-3

lb difference in the target pressures of the LT4 for which you were looking??

Would the difference in steel strengths between the stock and Crane locks

create a

problem with the performance, or function, of the motor now, or down the

line?

Well, there it is.  Let me know what you think about using a combination of

locks in the motor as I proposed about in Digest 242.

.. Bruce / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Sat, 20 Jan 1996 21:03:38 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: valve keepers (locks)

Message-ID: <960121050337_73145.1566_FHD39-1@CompuServe.COM>

>Scott, would it make sense to use the Crane locks for the exhaust valves,

and the stock locks for the intake valves, given that there would be about a

2-3 lb difference in the target pressures of the LT4 for which you were

looking?? Would the difference in steel strengths between the stock and

Crane locks create a problem with the performance, or function, of the motor

now, or down the line?

There would really be a 13 lb. difference in pressure between the stock

locks and the Crane +.050 locks, and that is based on my measurements that

the crane locks are in reality only +.030 different from the stock locks. If

they were actually +.050 as the spec indicates, the seat and open pressure

would have been reduced by 21 lbs.

I don't really think it would be a problem to use the stock keepers,

although the 13 lb. reduction in pressure from using the offset locks means

there is over 10% less pressure on the valve seat, which is definitely

better for long term durability. Remember the goal here is to increase

spring pressure enough to avoid valve float or seat bounce in the engine's

operational rpm range, and any more pressure than that is simply stressing

components unnecessarily.

Also it is worth noting that the Crane machined locks are much stronger than

the stamped steel locks, which may be important as we are approaching 300

lbs of nose (valve open) pressure. I guess that since I had the locks, and

as it only took 20 minutes to grind them down for clearance I feel I am

reducing stress and increasing durability. Scott.

------------------------------

Date: Sun, 21 Jan 1996 01:08:39 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 246

Message-ID: <Pine.SOL.3.91.960121010349.28986A-100000@netinfo.ubc.ca>

On Sat, 20 Jan 1996 impala@east.xsis.xerox.com wrote:

--------------------------------------------------------------------

>

> Date: Fri, 19 Jan 1996 18:26:11 PST

> From: David Hardy <dhardy@crl.com>

> To: impala@east.xsis.xerox.com

> Subject: Lock module!!??

> Message-ID: <Pine.SUN.3.91.960119182437.12976B-100000@crl8.crl.com>

>

> I understand the purpose of the rod to allow the unlocking of the door by

> door handle...but why would we replace the lock module?   DO you need

> that to make this mod?  or just the rod?

>

No, you need the lock module from the 6N1 package in

order for the whole thing to work.  This is what my dealer teels me

anyway, unless when Scott installs his package two weeks later and tells

us otherwise.

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red

------------------------------

Date: Sun, 21 Jan 1996 10:17:35 PST

From: jsalmon@mindspring.com (John Salmon)

To: impala@east.xsis.xerox.com

Subject: SEO switches where lighter is...

Message-ID: <v01530500ad27dcea2bbb@[168.121.55.70]>

I ordered and received the correct switch but I have been searching in vain

for a connector to mate with it.  To get the factory connector you have to

buy the whole SEO wiring harness ($800?  I may be crazy, but I'm not

stupid), or hack one out of a junked SEO car, which appears to be rarer

than hen's teeth.  Does anyone know who the OEM supplier for the SEO switch

connector is?  If I could get the connector casing I think I could put

together a makeshift harness.  It's an 8-pin by 1-column keyed

quick-disconnect.  I think it's the same connector used on all B-bodies for

the E/M and Trip switch to the left of the radio, so perhaps a junked

'94-up B-body would yield the right connector, but again getting the

harness from the factory or a Delco service center requires getting an

entire I/P harness.  And junked late-model B-bodies are hard to find

anywhere.  Scott M., you mentioned a JCWhitney kit for making up

factory-style connectors--all I've found are the usual run-of-the-mill

crimp-on connector kits, but nothing to pull and replace factory multi-pin

connectors.

Also, Scott mentioned that it would be wise to stock up on GM panel

fasteners.  Since I'm already planning an order through JCWhitney, does

anybody know if their GM-style fastener kit is the same as the one I'd get

from GM parts?  My local dealer(s) laughed at me when I asked for a parts

discount (I did so very tactfully and diplomatically).  So much for "most

GM dealers."  :-(

Thanks.

================================

John Salmon

BellSouth Usage Data Warehouse

personal: jsalmon@mindspring.com

work: john.salmon@bridge.bellsouth.com

'95 Dark Cherry Impala SS -- The SALMN8R

================================

------------------------------

Date: Sun, 21 Jan 1996 13:27:15 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: P/S pump leak

Message-ID: <Pine.SOL.3.91.960121132135.3990A-100000@netinfo.ubc.ca>

Hi everyone:

Ever since I posted my situation of fluid leak, I got quite a few

private responses suggesting it is likely to be the power sterring

pump that's causing the problem.  Appearently, quite a few members on

this this list are expereince this problem.  I am just wondering, are

those problems related to the P/S pump resulted from a design flaw, or

it's just so happen that the G.M. just so happens get a very high rate

of defective pumps from their part supplier.  I am also wondering, does

anyone have recurring problems after their pump are replaced?  I mean, if

the Arlington plant so happens to get a quite large number of defective

pumps, there's no gurnatee that the pump that your local dealer order

won't be defective either isn't it?

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red

------------------------------

Date: Sun, 21 Jan 1996 13:38:42 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: New GM Performance Parts, & VOTE!

Message-ID: <960121213841_76165.1654_GHM154-1@CompuServe.COM>

The February 1996 edition of Chevy High Performance magazine

has section that list 50 of the newest, hottest speed parts available

from the aftermarket for GM products. These were all shown at the

SEMA show in Las Vegas last October.

There is also a special pull-out section in the middle of the mag

titled Chevy Thunder. Inside this section are a bunch of little articles

and pictures of the latest parts from GM Motorsports. It is titled

Heavy-Duty Parts Update, and includes parts such as the new

LT4 head assy for upgrading LT1s to LT4 specs, new crate motors,

blueprinted cam & lifter kits and performance exhaust systems.

There is also a small write-up on the RWD Monte Carlo/Impala SS

that Chevy will hopefully make in '98.

Also check out the latest edition of Hot Rod, february 1996.

They have a feature article on the 460 HP V8 RWD Monte.

There is included in the article a Reader Ballot that you can

fill out and send in telling them if you would like Chevrolet

to build the RWD Monte Carlo/Impala SS in the near future.

Hot Rod will tabulate the results of this survey and forward

them to TPTB at GM, and hopefully they will listen.

So go out and pick up a copy, fill out the ballot, and VOTE!

Hopefully GM will listen, and you can say you had a part

in the building of this car, if GM decides to do so.

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Sun, 21 Jan 1996 14:10:02 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: 6N1 option

Message-ID: <960121171000_202902517@mail02.mail.aol.com>

In a message dated 96-01-20 20:23:35 EST, you write:

>I understand the purpose of the rod to allow the unlocking of the door by

>door handle...but why would we replace the lock module?   DO you need

>that to make this mod?  or just the rod?

Dave, without taking it apart, I don't know.  The module is what hits the

door striker bolt and contains the latching guts.  It must release the catch

when the rod is pulled where the normal module misses the release if the door

is locked.  The microfiche was very specific on which module to use with the

6N1 option. Just a guess, Troy

------------------------------

Date: Sun, 21 Jan 1996 14:10:02 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: impala digests

Message-ID: <960121171001_202902540@mail04.mail.aol.com>

In a message dated 96-01-20 20:23:35 EST, you write:

<I save the digests on my hard drive because there is barely enough time

to use the bathroom, maintain an affair, and get my job done at work.

<Does anyone support archiving the "communications" by topic?

Maybe a seperate file we could keep on the home page?

Dave,

I am working on cutting and pasting the table of contents from the digests

for the same reason.  If anyone else has already done this completely anal

project please advise.

Between saving lives and the Impala club, I have no life.  I lost my

girlfriend, got in trouble at work for using the county computer for impala

stuff, and have cut the paperwork out of going to the bathroom to save time.

On a serious note-----------------Everyone please check the subject area when

posting and replying to stuff.  Make it as detailed as possible so that we

can find the related threads later for topic research.  Thank you, Troy

------------------------------

Date: Sun, 21 Jan 1996 14:12:00 PST

From: Impalaclub@aol.com

To: impala@east.xsis.xerox.com

Subject: Fwd: information

Message-ID: <960121171159_402767065@mail04.mail.aol.com>

In a message dated 96-01-21 01:55:46 EST, hsharf@sirinet.net (Howard Sharf)

writes:

Guys, I've answered the club and common mod questions if someone could help

him with the 6 speed question. thanx, Troy

>Please send me any information about the impala club. I currentlyhave a

balck

>58 hardtop (first model) and I am now looking for a 96 (last model). I

>specifically  am interested if any one has an impala with a manual

>transmission and if it is possible to special order one with the camaro

>trans.  If not is there a good place to have one done as an after market

>change. What common modifications are being done  to get the performance and

>handling improved? Thanks for any info you can provide!

---------------------

Forwarded message:

From:	hsharf@sirinet.net (Howard Sharf)

To:	impalaclub@aol.com

Date: 96-01-21 01:55:46 EST

Please send me any information about the impala club. I currentlyhave a balck

58 hardtop (first model) and I am now looking for a 96 (last model). I

specifically  am interested if any one has an impala with a manual

transmission and if it is possible to special order one with the camaro

trans.  If not is there a good place to have one done as an after market

change. What common modifications are being done  to get the performance and

handling improved? Thanks for any info you can provide!

------------------------------

Date: Sun, 21 Jan 1996 14:12:02 PST

From: Impalaclub@aol.com

To: rush@mcs.com, impala@east.xsis.xerox.com

Subject: Re: Impala problems

Message-ID: <960121171201_301718442@mail02.mail.aol.com>

In a message dated 96-01-21 11:02:35 EST, you write:

>If this is the "Tilt and toot" fix, I can dig it up in the archives.  The

>person at the Customer assistance center really didn't seem too technical

>as they told me "We do not keep a record of service bulletins; You'll have

>to contact your dealer".

>

>I don't think my car does the tilt and toot.  How can I convince them that

>this will solve the problem of the insensitive horn?   I'll try going

>in the with the TSB number and see if it will help.  (right now the

>car is in storage, away from all the road salt.)

I had the same problem with a hard horn.  Brought it in last week in fact.

They adjusted it and now it tilts and toots.  I called my service writer and

gave him the tsb # and asked him to order the new membrane design introduced

in May 95.  The parts were expected to be avail in July 95.  Mine are on

order now.

Tech Serv Bulletin #53-32-05

Date July 09, 1995

Models:  94-5 Chevy Caprice/Impala SS

replacement parts:

pn 16757633  Wheel Assembly, Steering (Leather, Light Grey)

pn 16757619  SIR Module Assy, Light Grey

The TSB was in my December newsletter,

cc:  Listserv

Good luck, Troy

------------------------------

Date: Sun, 21 Jan 1996 14:22:02 PST

From: Mark Boggs <70004.173@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: ATI parts ordered

Message-ID: <960121222201_70004.173_FHT50-2@CompuServe.COM>

SSers,

The ATI and Art Carr parts were ordered on Friday.  I'm having the ATI kit

installed in my SS and the transmission is going to be rebuilt with Art Carr

components.  Lee Howie is going to get the kit(s) in, make sure everything is

there, setup a day for his transmission builder to come in and have me drop off

the car.  It should be done in two, three weeks tops.  He will only need the car

for three or four days.  He's going to install an MSD and tune the car for best

results.  I'm so excited...

I was reading through the digests and saw two posts on the rear end gear.  All

part numbers were the same, except the actual 3.42 gear.  One post refered to PN

26031116, while another post listed PN 26041514.  I don't have the parts fiche,

so I can't tell why the part numbers differs.  Which is the correct 3.42 gear

for my 95?

Also, AFR (Air Flow Research) has these interesting hydraulic roller lifters

(Hydra-Rev) that they claim can dramatically increase HP above 5500 rpm.  The

articles I read came from the June 95 issue of Hot Rod magazine and the April 95

issue of Car Craft.  This sounds like a natural addition to anybody who uses the

Hypertech, as it would add even more of a reason to let the car shift at a

higher point.  Has anybody heard of these? Will they work with the LT1? I'd

appreciate any opinions or comments on these lifters?

CYA Later...

-Mark

------------------------------

End of IMPALA Digest 247

************************

IMPALA Digest 248

Topics covered in this issue include:

1) Greetings

by jaking@holli.com (Jeremy King)

2) Re: IMPALA digest 241

by rcheek@one.net (Robert Cheek)

3) Re 3.42 gear P/N

by BGordon401@aol.com

4) Getting Trade Rates (wholesale) on parts

by BGordon401@aol.com

5) Impala SS vs. Taurus SHO

by sirjames@rmii.com (James W. Thompson)

6) Ridiculous

by "Rush Luangsuwan" <rush@mcs.com>

7) 3.42 gears

by Scott Mueller <73145.1566@compuserve.com>

8) Floor Shifter

by Scott Mueller <73145.1566@compuserve.com>

9) Springs & Air Lift bags

by Scott Mueller <73145.1566@compuserve.com>

10) Connectors

by Scott Mueller <73145.1566@compuserve.com>

11) I'll be back...

by Scott Mueller <73145.1566@compuserve.com>

12) 9C1 Caprices in West Columbia, Texas

by gbeaird@amoco.com

----------------------------------------------------------------------

Date: Sun, 21 Jan 1996 14:37:12 PST

From: jaking@holli.com (Jeremy King)

To: impala@east.xsis.xerox.com

Subject: Greetings

Message-ID: <9601212237.AA04971@mistletoe.holli.com>

Hi,

My name is Jeremy King of Kokomo IN, and I own a Black 96 Impala built the

week of Dec 11 95. The weather's kept it in the garage for most of the time

since I took delivery except for a thousand mile salt bath over the holidays

to break it in.

I was interested to hear about the differential gasket still being a problem

in OCT96. I will get mine checked and thanks for the tip. Maybe I'll try to

find out why GM is still building them like this. I also measured a small

(1/6 inch) difference between the 2 rear tires and their wheelwells.

------------------------------

Date: Sun, 21 Jan 1996 16:11:40 PST

From: rcheek@one.net (Robert Cheek)

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 241

Message-ID: <199601220003.TAA08087@mail.one.net>

>Has anyone else seen the "Fleeter Impala" article in Car Craft magazine?  It

>claims to have added 40 rear-wheel hp to the car through JET mods... the

>whole article seemed more like a JET advertisement, to tell you the truth.

> They added the "Powr-Flo" filter (any advantage over the K&N unit?), JET

>PCM, airfoil, and underdrive pulleys... their "before" and "after" rear-wheel

>hp were 174.2 and 213.1, respectively.

This was the article that almost sold me a JET: I figured 40 HP is 40 HP,

one way or another. However, I also realized a couple of things:

1)  The rep (jetchip@aol.com) said the device worked in a way that is

physically impossible with the connection that it plugs into. This

brings their credibility into suspicion, IMHO.

2)  The JET chip can't do things that you can do with other computer

mods, such as changing gear ratios and fan temps. This adds to

the cost, making it almost equal to any other PCM mod.

3)  174 HP seems _really_ low for rear wheel output in the Impala.

Since the tests were run on JET's dyno, I wonder if they "massaged"

the test (e.g. run the stock car while engine was very hot on

very poor fuel and a partially clogged air filter, for example). They

also ran dyno tests in another mag for some of their mods and achieved

very large gains. Possibly "ringer" PCM modules?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers + "H" Pipe

------------------------------

Date: Sun, 21 Jan 1996 18:04:39 PST

From: BGordon401@aol.com

To: 70004.173@compuserve.com

Cc: impala@east.xsis.xerox.com

Subject: Re 3.42 gear P/N

Message-ID: <960121210435_203101558@mail02.mail.aol.com>

Mark, you wrote:

> ... the actual 3.42 gear.  One post refered to PN 26031116, while another

post listed PN 26041514.  I don't have the parts fiche, so I can't tell why

the part numbers differs.

I was luck enough to recently establish a "Trader" rate (wholesale)

relationship with a very large local Chevy dealership.  I cross checked p/n

26041514 to see what the new, post Jan 1996, price is (roughly $20.00-30.00

more).  The 3.42 gears came up under this p/n.  I can't answer for what p/n

26031116 relates to, but it doesn't appear to be related to the gear-swap.

If it does, please respond back to let us all know.

..Bruce / BGORDON401@aol.com

------------------------------

Date: Sun, 21 Jan 1996 18:04:45 PST

From: BGordon401@aol.com

To: jsalmon@mindspring.com

Cc: impala@east.xsis.xerox.com

Subject: Getting Trade Rates (wholesale) on parts

Message-ID: <960121210437_203101599@mail06.mail.aol.com>

John, you wrote:

>My local dealer(s) laughed at me when I asked for a parts

discount (I did so very tactfully and diplomatically).  So much for "most

GM dealers."  :-(

Time to get a new dealer.  There is no reason that you were turned down for

this price break.  I recently established a "Trade" rate (wholesale)

relationship with a local Chevy dealership, thanks to some tips from the

folks on our list.

When you order parts, make sure you have the p/n handy (most of the time,

anyway), and ask what the "Trade Price" is.  They might ask if you own a

professional garage.  In my case, I told them "no, I do work in my own

garage."  It created no problem.  With the oil pressure sensor alone, it

meant the difference from $19.00 Trade to $25.00 list (using round numbers,

but you get the point).  In the future, the parts guy told me to just tell

his people that I "get Trade price with tax added."  He said professional

garages in Pa. don't get charged tax on parts.

I hope this info helps.

..Bruce / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Sun, 21 Jan 1996 18:39:00 PST

From: sirjames@rmii.com (James W. Thompson)

To: impala@east.xsis.xerox.com

Subject: Impala SS vs. Taurus SHO

Message-ID: <m0teCAD-000LC3C@rmii.com>

Has anyone dusted a Taurus SHO?  Are they worth the race? Does anuone have

specs for the F.O.R.D. ?   I have a friend at work who owns one and I would

like to have the facts before I open my big mouth....or race for pink slips.

Black '96 SS

Jim

sirjames@rmii.com

------------------------------

Date: Sun, 21 Jan 1996 23:21:35 PST

From: "Rush Luangsuwan" <rush@mcs.com>

To: impala@east.xsis.xerox.com

Subject: Ridiculous

Message-ID: <m0teGZL-0003kwC@mercury.mcs.com>

> An associate of mine has (had?) a 94 Impala SS. He put on a set of snow tires

> with 15" steel rims this winter. The size of the tire is the exact size

> recommended for Caprice's. He was involved in an accident this winter, during

> the first major storm. The insurance company looked at the car, and asked him

> why the original 17" tires and rims were not on the vehicle. He told them he

> switched over to 15"rims and snowtires so his Impala SS would be safer in the

> snow. (obviously not as safe as he hoped). The insurance company is denying his

> claim because he "tampered with the original design of a safety device on the

> vehicle as received from the factory." By switching from 17" rims to 15" rims

> and narrower tires, he modified a part of the ABS system (considered a safety

> device) which was designed for 17" rims and tires. He argued that the spare tire

> was only a 15" tire. The insurance company's response was that the spare tire is

> just that, a spare, which is to be used only to get you to the nearest repair

> facility to repair the flat tire.

> GM agrees, and is backing up the insurance company.

This is a ridiculous excuse!

Pretty soon you'll hear stories about insurance companies not covering your

claim because you disconnected your DRL because it as an "original design

of a safety device from the factory", or we can't cover your claim,

even though you weren't going over 150mph, you didn't have Z rated tires

that came from the factory, so that's a safety issue.  How far can this

go?

Anyway, just received a new catalog from Lingenfelter and it had in

insert dated 12/15/95 about their new 383 50 state legal engine:

350 cid LT1 packages include:

- CNC ported cynlinder heads & hand ported intake manifold

- Stainless steel one piece valves - 2.00"/1.56"

- Stainless steel full roller rocker arms - 1.6 : 1 ratio

- Heavy duty double valve springs with dampers

- Titanium valve spring retainers - lighter than steel

- Custom designed hydraulic roller camshaft

- 58 mm billet aluminum throttle body

- Borla 304 stainless steel exhaust system

- K&N air filter

- Professional installation for a factory installed appearance

383 cid include these additions:

- Custom forged aluminum pistons - 11.0:1 compression ratio

- Custom forged steel crankshaft - 3.750" stroke

- Billet steel connecting rods - 5.850" length

- File fit plasma moly piston rings

- Computer balanced rotating assembly

Automatic transmission (A/T) packages include:

- Custom torque converter

- Custom shift kit

- Revised shift points

SuperRam packages include:

- SuperRam intake manifold base

- SuperRam plenum/runner kit

- Custom designed hydraulic roller cam for use with SuperRam intake manifold

and runners

These engines come with a 2 year 24,000 mile warranty.

They had some performance data, but nothing specifically for an Impala.

440 BHP LT1 Camaro/Firebird Package

0 to 60mph in 4.2 seconds, 1/4 mile in 12.7 seconds, top speed 180+

1994-1995 Impala SS package prices:

50 state emission legal (E.O. D-333-4 & E.O. D-333-5)

383 cid A/T 425 BHP / 430 ft/lbs of torque		$16,450.00

383 cid A/T SuperRam 425 BHP / 467 ft/lbs of torque	$18,350.00

Seems pretty expensive, but it looks like serious power from some

serious amount of cash!

--

Rush Luangsuwan

rush@mcs.com

'95 Blk SS

------------------------------

Date: Mon, 22 Jan 1996 01:25:18 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: 3.42 gears

Message-ID: <960122092518_73145.1566_FHD34-2@CompuServe.COM>

Mark,

Congrats on the ATI purchase. Let us know how the install goes, and of

course how it runs afterwords!

>I was reading through the digests and saw two posts on the rear end gear.

All part numbers were the same, except the actual 3.42 gear. One post

refered to PN 26031116, while another post listed PN 26041514. I don't have

the parts fiche, so I can't tell why the part numbers differs. Which is the

correct 3.42 gear for my 95?

I don't know who posted the other number, but p/n 26041514 is the part

number for the "3.42 Gear Kit" I have installed in my car. It cost me

$273.24 at my dealer.

>Also, AFR (Air Flow Research) has these interesting hydraulic roller

lifters (Hydra-Rev) that they claim can dramatically increase HP above 5500

rpm.  The articles I read came from the June 95 issue of Hot Rod magazine

and the April 95 issue of Car Craft. This sounds like a natural addition to

anybody who uses the Hypertech, as it would add even more of a reason to let

the car shift at a higher point. Has anybody heard of these? Will they work

with the LT1? I'd appreciate any opinions or comments on these lifters?

These sound like what is commonly referred to as a fast bleed lifter, which

was pioneered by a company called Rhoads, and which by association are

commonly called Rhoads Lifters. Fast bleed lifters are designed to allow a

"big" cam to be used on a street car by bleeding down at lower oil

pressures, taking some of the lift and duration out of the cam. This allows

the car to idle better and produce more vacuum at lower rpms. When the revs

(and oil pressure and volume) are higher, they pump up and act like a normal

lifter giving the engine the full effect of the cam.

When these lifters bleed down, they tend to make a lot of noise due to the

excessive lash that develops in the valvetrain. This means they tick and

click, which is not something that sounds attractive on a nice new LT1

engine (or any street engine for that matter!). Not only do they sound bad,

but the ticking can sometimes trigger the knock sensors on computer

controlled cars, which causes the PCM to retard the timing, set trouble

codes, etc. Also the actual amount and rpm point of bleed down is difficult

to control, which may mean playing around with bearing clearances, oil

pressure (relief springs), and oil pump designs, to get the desired effect.

If indeed these lifters you are describing are a fast bleed type design, I

would not recommend them. If they are something else all together (what else

they could be I just don't know), then never mind, but my guess is these are

basically the old Rhoads lifter concept applied to the newer roller lifters.

Scott.

------------------------------

Date: Mon, 22 Jan 1996 01:25:08 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Floor Shifter

Message-ID: <960122092507_73145.1566_FHD34-1@CompuServe.COM>

>Sorry that I can't provide you with that '96 to look off of.  When you do

find someone with it, pay close attention to the shifter mechanism itself.

The parts catalog seemed very skimpy and vague on what parts came with the

console; even my parts guy wasn't sure if you got any of the hardware in the

console itself. (Or is there another way to find out what comes with an

assembly?)

I know how to read the parts catalogs pretty well, and especially so do the

parts people I deal with, and have already figured out what is needed for

the floor shift conversion. There are a few minor details to work out, but

most everything is pretty straightforward. I will be ordering the parts and

installing it soon!

>I have a feeling that you can still "re-use" most of the parts on a '94-'95

console, such as the arm rest, hinge, and screws without having to

repurchase them with the new '96.

Indeed, it seems as if the entire console lower exterior assembly, arm rest,

hinge, etc. can be re-used as they are the same as the '96 model (even

though their part numbers have changed).

>I hope that if you wanted to add the center console, it wouldn't require

any drilling in the floor.

There will be one small hole that has to be drilled to pass the shifter

cable through. A grommet will seal the hole, just as on the '96 models. I

need to inspect a '96 in order to determine the precise placement and size

of the hole. When I am done with the shifter install, it will look

absolutely as if it were factory installed just like on the '96. Of course

it will use all factory parts as well.

If I decide to retain my modified (added digital tachometer and temperature

gauge in the bezel) '94 instrument cluster, then I will also be able to

retain the gear position indicator in the cluster as well. In other words,

when I move the '96 style floor shifter, the collar on the column will also

move, as will both the gear indicator pointer on the column as well as the

one directly tied to the shifter in the console. A replacement collar will

be used on the column which does not have provisions for the column shifter

lever. This will all be done using factory parts!

Later if I decide to upgrade to the Camaro instrument cluster, I will then

lose the cluster mounted gear indicator needle, but of course will still

retain the one built-in to the '96 floor shifter. Scott.

------------------------------

Date: Mon, 22 Jan 1996 01:25:42 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Springs & Air Lift bags

Message-ID: <960122092541_73145.1566_FHD34-5@CompuServe.COM>

Rob,

>I realize you won't see this for a while, but could you give me some

approximate prices for this stuff, including the inflation assembly? I want

to get rid of the sag (which is the only exterior part of the car I don't

like ;-) if it's not too expensive.

The Air Lift bags can be purchased through JC Whitneys for about $60, and

the high GVWR factory springs are about $40 each. Scott.

------------------------------

Date: Mon, 22 Jan 1996 01:25:31 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Connectors

Message-ID: <960122092530_73145.1566_FHD34-3@CompuServe.COM>

John,

>I ordered and received the correct switch but I have been searching in vain

for a connector to mate with it.  To get the factory connector you have to

buy the whole SEO wiring harness ($800?...

This is not true, you can indeed purchase the proper connector. In fact the

right part number for this connector is in your service manual! Look in

section 8A-202 "Harness Connector Faces", this section shows every connector

in the car, gives the pinout, the application, and the part number. When you

purchase the connectors in this fashion you will still need the proper

terminals. The terminals can be purchased individually from the Standard

Parts catalog, from many parts stores, from Kent Moore tools, from Plews

tools, or from JC Whitney.

If you look in the Whitney catalog, they have a "computer connector/terminal

repair kit" (that may not be the exact name) for about $80 that has several

of the small terminal picks and tools, as well as a nice starter set of

terminals. I know that some of the terminals in this kit are used in the SEO

switch connector. Kent Moore also has a basic kit of different GM terminals

for about the same money, but it does not include any tools. Kent Moore

sells the individual tools separately, or a master GM connector tool and

terminal kit with everything for about $440. Of course this master kit would

be overkill unless you are constantly doing auto wiring projects like I am!

If you want to purchase the terminals individually, then you must first

identify the proper ones in the factory Standard Parts catalog (at your

dealer or available on 'fiche), or if you have a good local parts store,

they might stock these connectors in small bubble packs in an electrical

terminal section. I started with several of the different terminal kits, and

augment them or re-stock them with bubble pack purchases at a local parts

store, as well as individual selective purchases by looking them up in

Standard Parts.

Another source might be John Spina, who is on this list. He is an expert on

wiring harnesses in general, and has access to direct purchasing through

GM's Packard Electric division. He can get practially any terminal or

connector that is used in any GM vehicle.

>or hack one out of a junked SEO car, which appears to be rarer than hen's

teeth.

I try to make an annual trek to the local junkyards, just to pick up factory

hardware such as nuts, bolts, brackets, screws, retainers, clips, clamps,

connectors, switches, etc... Often time the connectors you are looking for

are used on many different GM makes and models, for various applications. I

just cut the connectors off of the donor cars, leaving as much wire with the

connector as possible.

>Does anyone know who the OEM supplier for the SEO switch connector is? If I

could get the connector casing I think I could put together a makeshift

harness.  It's an 8-pin by 1-column keyed quick-disconnect.

This connector is shown on page 8A-202-12 of the service manual, and is

listed as part number 12064862, 8-way F Metri-Pack 150 series, black. The "F

Metri-Pack 150 series" designates the actual types of terminals used in this

connector as Female Metri-Pack 150. That info will be needed when you go to

order the terminals.

>I think it's the same connector used on all B-bodies for the E/M and Trip

switch to the left of the radio, so perhaps a junked '94-up B-body would

yield the right connector...

This connector is listed in the service manual as being used for the

Instrument Cluster English/Metric Switch, Rear Compartment Lid Release

Switch (SEO), and the Remote Control Rear View Mirror (Buick).

>Scott mentioned that it would be wise to stock up on GM panel fasteners.

Since I'm already planning an order through JCWhitney, does anybody know if

their GM-style fastener kit is the same as the one I'd get from GM parts?

The GM style retainer kit you are referring to will have the exact same

retainer as used in the Impala/Caprice door panel, however there will only

be 8 of them, along with several other types of retainers for other

applications. That will not be enough of the proper retainers to do both

front doors.

>My local dealer(s) laughed at me when I asked for a parts discount (I did

so very tactfully and diplomatically).  So much for "most GM dealers."

Your dealer first told you that you could not get the connectors (wrong),

and then will not give you a discount. You need another dealer! I hope you

(and everybody else) realize that you can purchase Chevy parts from ANY GM

dealer! Try several of the other GM dealers in your area on your hunt for a

knowledgable, cooperative, and enthusiastic parts staff! Scott.

------------------------------

Date: Mon, 22 Jan 1996 01:38:01 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: I'll be back...

Message-ID: <960122093800_73145.1566_FHD24-1@CompuServe.COM>

I will be back in about 2 weeks, on February 3rd. I will respond to any mail

or Impala digests after that time. I look forward to meeting Scott H. and

Curt P. while I am in Boston! They will supply me with the necessary

transfusion of SS information which should keep the withdrawl symptoms to a

minimum. <G> TTFN, Scott.

------------------------------

Date: Mon, 22 Jan 1996 05:22:28 PST

From: gbeaird@amoco.com

To: impala@east.xsis.xerox.com

Subject: 9C1 Caprices in West Columbia, Texas

Message-ID: <199601221322.AA07086@interlock.amoco.com>

>>I am planing to order a 1996 9C1 at the end of the

summer, and hope to get

>one of the last run produced.

********************************************************

******

I am told that the 9C1's I wrote about a week or so, the

the local dealer

has for sale, are the last of the last.  In fact, the

dealer ended up with

them because an agency ordered like 18 and they only

made 7.  The agency

cancelled the order since they couldn't get all 18.

This dealer bought them

since they knew they would sell.  So, these 7 may well

be the last of the last.

Steve

Steve,

I drove one of these machines over the weekend.  Ran

great.  Handled great.  Indeed, it is a 9C1 Texas DPS

car.  I believe they still have all 7, although I only

saw the one I drove.  It was the weekend, around closing

time.  I am considering getting either one of these or

an Impala SS.  If I could find a 'new' '95 SS for a

price close to the 9C1s, I will probably go with that

and add the 9C1 options to it.

One question, though.  The 9C1s have a cable for the

light bar that goes through a grommet in the roof (just

over the front passenger's area).  Does anyone know

where someone could get a plug for this approximately

1in. diameter hole (in case I go for the 9C1)?  The

cable install was a factory option, I would hope the

factory has a correctly-sized plug for the hole should

the owner choose to remove the wiring.  Any ideas?

Thanks in advance.

Gene Beaird

gbeaird@amoco.com

------------------------------

End of IMPALA Digest 248

************************

IMPALA Digest 249

Topics covered in this issue include:

1) Insurance coverage denied!?

by John Pinkerton <jpinkert@sigg.com>

2) RE: Oh no SNOW,here we go again <G>

by "Rodger Cliff (216) 977-0036" <CLIFF#m#_RODGER@lims-a1.lerc.nasa.gov>

3) Dusted a SHO?

by gbeaird@amoco.com

4) Re:Tilt&toot

by John.Salmon@bridge.bst.bls.com

5) 9C1s for sale in West Columbia, Texas

by gbeaird@amoco.com

6) OEM Connectors

by jaking@holli.com (Jeremy King)

7) 9C1 vs. SS ABS

by "Rodger Cliff (216) 977-0036" <CLIFF#m#_RODGER@lims-a1.lerc.nasa.gov>

8) Re: Defective Steering Pump

by BGordon401@aol.com

9) 6NI prices

by "Rush Luangsuwan" <rush@mcs.com>

10) Relocating the Air Temp. Sensor on the Intake Elbow

by BGordon401@aol.com

11) Choking

by lennie@earthlink.net (Lennie Dolphin)

12) Resistor

by lennie@earthlink.net (Lennie Dolphin)

13) loose windshield

by "TODD M. MCMURRY" <73543.3717@compuserve.com>

14) Re: impala problem database

by Rock528@aol.com

----------------------------------------------------------------------

Date: Mon, 22 Jan 1996 09:34:11 PST

From: John Pinkerton <jpinkert@sigg.com>

To: impala@east.xsis.xerox.com

Subject: Insurance coverage denied!?

Message-ID: <96Jan22.113155cst.26498@firewall.sigg.com>

How can they deny coverage because of a tire change?  Does anyone

(Scott?) know if the standard caprice or 9C1 uses the same ABS system?  If

so, whats the problem with switching tires?  I have personally switched to

15" rims and 225x70x15 snowtires for winter and my ABS works fine.  I have

been into the ABS on may occasions and the car stoped straight and fairly

quickly.

Also, I contacted my insurance dealer (State Farm) and he stated that they

have never denied a claim based on this.  He stated that if it was attached to

the vehicle and you could produce a receipt of purchase he would repair the

vehicle and replace the parts.  Regardless of whether they were original

factory equipment or not.

I would definately fight this one, and then get a different insurance company.

Sounds like they're skimming a little too much off their profits.

=======

Previous message:

Date: Sun, 21 Jan 1996 23:21:35 PST

From: "Rush Luangsuwan" <rush@mcs.com>

Subject: Ridiculous

> An associate of mine has (had?) a 94 Impala SS. He put on a set of snow

tires

> with 15" steel rims this winter. The size of the tire is the exact size

> recommended for Caprice's. He was involved in an accident this winter,

during

> the first major storm. The insurance company looked at the car, and asked

him

> why the original 17" tires and rims were not on the vehicle. He told them he

> switched over to 15"rims and snowtires so his Impala SS would be safer in

the

> snow. (obviously not as safe as he hoped). The insurance company is

denying his

> claim because he "tampered with the original design of a safety device on

the

> vehicle as received from the factory." By switching from 17" rims to 15"

rims

> and narrower tires, he modified a part of the ABS system (considered a

safety

> device) which was designed for 17" rims and tires. He argued that the

spare tire

> was only a 15" tire. The insurance company's response was that the spare

tire is

> just that, a spare, which is to be used only to get you to the nearest repair

> facility to repair the flat tire.  > GM agrees, and is backing up the insurance

company.

This is a ridiculous excuse!

Pretty soon you'll hear stories about insurance companies not covering your

claim because you disconnected your DRL because it as an "original design

of a safety device from the factory", or we can't cover your claim, even though

you weren't going over 150mph, you didn't have Z rated tires that came from

the factory, so that's a safety issue.  How far can this  go?

--  Rush Luangsuwan			     rush@mcs.com

'95 Blk SS

------------------------------

Date: Mon, 22 Jan 1996 09:25:00 PST

From: "Rodger Cliff (216) 977-0036" <CLIFF#m#_RODGER@lims-a1.lerc.nasa.gov>

To: impala%east.xsis.xerox.com%smtp@lims-mrgate.lerc.nasa.gov

Subject: RE: Oh no SNOW,here we go again <G>

Message-ID: <01I0B7OSXHL68WXA30@lmsmgr.lerc.nasa.gov>

On Thu, 18 Jan 1996 19:59:46 PST, Kurt Kemmling <76165.1654@compuserve.com>

wrote:

>Here is a little more food for thought, that convinced me NOT to put snow

tires on my Impala SS.

An associate of mine has (had?) a 94 Impala SS. He put on a set of snow tires

with 15" steel rims this winter. The size of the tire is the exact size

recommended for Caprice's. He was involved in an accident this winter, during

the first major storm. The insurance company looked at the car, and asked him

why the original 17" tires and rims were not on the vehicle. He told them he

switched over to 15"rims and snowtires so his Impala SS would be safer in the

snow. (obviously not as safe as he hoped). The insurance company is denying his

claim because he "tampered with the original design of a safety device on the

vehicle as received from the factory." By switching from 17" rims to 15" rims

and narrower tires, he modified a part of the ABS system (considered a safety

device) which was designed for 17" rims and tires.  ... [text deleted]

>GM agrees, and is backing up the insurance company.

Kurt, please advise your friend to find out if GM said that using those tires

under the road conditions in which the accident occurred resulted in

significantly less braking performance than if he had not changed the tires.

GM will probably decline to respond to this statement because the tires may

actually be better in those conditions (not dry, good traction situations) and

the conditions in which the accident occurred would be difficult/impossible to

recreate.  Also that might imply that GM was endorsing that 15" wheel/tire set-

up and GM would not the liability from that.  The insurance company will have

no leg to stand on and must pay his claim.  If not tell him to get a lawyer and

sue his insurance company.

You could also ask someone here with a parts file to compare the abs component

numbers for the SS versus the 9C1 (especially the controller) and see if any

are different.

However, I would like to endorse the stock SS tires from an outsiders

perspective.  I did notice that during one of bad storms we had that I could

not keep up with an SS (with stock looking tires/rims) in my '91 9C1 with

235/70R-15 Eagle ST's (stock size listed on the door sticker).  Another person

at work commented how an SS kept up with him in his 4WD pick-up.  Sounds like

good tires to me.

Rodger Cliff		rodger.m.cliff@lerc.nasa.gov

------------------------------

Date: Mon, 22 Jan 1996 10:46:16 PST

From: gbeaird@amoco.com

To: impala@east.xsis.xerox.com

Subject: Dusted a SHO?

Message-ID: <199601221847.AA16560@interlock.amoco.com>

Has anyone dusted a Taurus SHO?  Are they worth the

race? Does anuone have

specs for the F.O.R.D. ?   I have a friend at work who

owns one and I would

like to have the facts before I open my big mouth....or

race for pink slips.

Black '96 SS

Jim

sirjames@rmii.com

Jim,

A stocker SHO against a stocker SS should be no contest,

but be careful about bragging rights until after you

run.  Never, never, NEVER, race for pink slips.  I have

a firend who had a 70 GTO, who one night was going to

race a pretty beat up camaro.  His last words before

getting into the car were "There's no WAY I can loose

this one".  Sure enough, his TH400 let go going into 2nd

and I lost $20.  I ran a warmed 76 T/A with my sleeper

77 T/A one night.  My car had a well prepared stock

looking .030 455.  I got beat by half a car.  I found

out a week later that my throttle mount had slipped,

letting the throttle open only about 3/4.  My normally

mid 13 second T/A was running probably high 14's when I

raced that guy.  There was no way I could loose that

$20, too.

What I'm saying is let the car talk, rib him after you

suck his headlights out. Let us knowhow bad you beat

him.

Gene Beaird

gbeaird@amoco.com

------------------------------

Date: Mon, 22 Jan 1996 02:03:47 PST

From: John.Salmon@bridge.bst.bls.com

To: Impala <impala@east.xsis.xerox.com>, Rush <rush@mcs.com>

Subject: Re:Tilt&toot

Message-ID: <m0teQac-000DjhC@bstfirewall.bst.bls.com>

I just had my car in at the dealer for all the pertinent TSB's and the

transmission recall.  They at first balked on the "tilt and toot" fix

unless the horn was actually having the problem.  I told him the horn

was hard to blow, which was a safety hazard in my opinion and would be

fixed by the TSB's recommended procedure, and he agreed to look up the

TSB and wound up ordering in a new complete steering wheel/airbag/horn

assembly from Memphis.  I noticed that the new wheel has two horn

symbols in the center, where the old one had the horn symbol in the

upper left.  The old airbag assembly had to pivot to press the horn

button, but now it appears they have moved the horn switch to right on

top of or underneath the airbag module, because I now can blow the horn

from both sides of the center seam right in the middle of the airbag pad

(like it should be!).  Now I can even tap out "shave and a haircut" if I

want to!

John Salmon

------------------------------

Date: Mon, 22 Jan 1996 11:50:10 PST

From: gbeaird@amoco.com

To: impala@east.xsis.xerox.com

Subject: 9C1s for sale in West Columbia, Texas

Message-ID: <96Jan22.124236pst."54403(10)"@alpha.xerox.com>

I just got through talking to one of the sales managers

at Bill Wiggins Chevrolet in West Columbia, Texas.  He

indicated that they had already sold 3 of the 7 Caprice

9C1s they had in stock.  If you are waiting, you had

better hurry, they are going fast.

Gene Beaird

gbeaird@amoco.com

------------------------------

Date: Mon, 22 Jan 1996 13:30:58 PST

From: jaking@holli.com (Jeremy King)

To: impala@east.xsis.xerox.com

Subject: OEM Connectors

Message-ID: <9601222130.AA08136@mistletoe.holli.com>

To: John @ impala:

A vast majority of the connectors in the Chevy B cars are GM Packard

Electric Division sourced. Once you get the part number from the dealer,

contact Pioneer Electric in (Cleveland or Cincy ?). One of the Pioneers in

Ohio stocks many PED connectors, while I beleive that the other outlets can

order them. If you are going to do any serious rewiring, I would suggest

getting the Packard data book (catalog), which will allow you in most cases

to match up a connector from pictures and part numbers in their book. If

Pioneer doesn't have the data book Packard may need to be contacted

directly. I think they are in Warren OH.

It may not be a bad investment for anyone doing wiring on GMs. Recent

outsourcing has resulted in a few vehicle-level connectors being

other-than-Packard, but most still are. Packard is now a unit of Delphi

Automotive (Delphi Signal Systems or something like that). Good Luck

------------------------------

Date: Mon, 22 Jan 1996 15:50:00 PST

From: "Rodger Cliff (216) 977-0036" <CLIFF#m#_RODGER@lims-a1.lerc.nasa.gov>

To: impala%east.xsis.xerox.com%smtp@lims-mrgate.lerc.nasa.gov

Subject: 9C1 vs. SS ABS

Message-ID: <01I0BL9LO0948WXA30@lmsmgr.lerc.nasa.gov>

After I responded to Kurt about the insurance company trying to

rip off an SS owner over using the 9C1 wheels/tires in the

winter, I thought about the reverse.  I am planning (in the

future) to pick up a used '95 9C1 and put the Impala SS

wheels/tires on it, especially for summer use.

Is there enough (any?) difference in the ABS components (between

a 9C1 and an SS) that I should consider some changes there also?

My best information is that this not a problem, as long as I keep

the rolling diameter of the tires close to the OEM set-up.

Anyone got any better info?

Rodger Cliff	 rodger.m.cliff@lerc.nasa.gov

------------------------------

Date: Mon, 22 Jan 1996 18:28:02 PST

From: BGordon401@aol.com

To: mkyang@unixg.ubc.ca

Cc: impala@east.xsis.xerox.com

Subject: Re: Defective Steering Pump

Message-ID: <960122210115_124236966@emout04.mail.aol.com>

Mike, you wrote:

> I am just wondering, are

those problems related to the P/S pump resulted from a design flaw, or

it's just so happen that the G.M. just so happens get a very high rate

of defective pumps from their part supplier.  I am also wondering, does

anyone have recurring problems after their pump are replaced?

I honestly can't answer you question concerning the defective condition of my

power steering pump.  There was a problem with the pump, and the dealership

replaced it w/o an argument.  BTW, the steering was "groaning" a bit when I

turned the wheel at a stand-still, which lead me to investigate the condition

further.

If the problem is from a design flaw, I will DEFINATELY be going back to the

dealership until The General fixs the flaw, or they'll be replacing the pump

every so often.  So far, however, I have had no problems once the pump was

replaced.  Everything looks good.  The "groaning" has not come back, and the

light oil spray in the steering pump area has not happened again.

Please note, I also had my oil lines that connect to the radiator replaced at

the same time.  Also at the dealer's cost, since they recommended it be done

along with the pump.  There might be a connection, but I'm not sure.

I hope this helps.  Write back if you need more information.

..Bruce / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Mon, 22 Jan 1996 19:35:20 PST

From: "Rush Luangsuwan" <rush@mcs.com>

To: impala@east.xsis.xerox.com

Subject: 6NI prices

Message-ID: <m0teZVx-0003kvC@mercury.mcs.com>

>

> > Hi Mike, the RPO is 6N1 and the door lever is pn 16622475 ($2.00=/-).  They

> > also listed a few other 6N1 parts in the computer like the lock module.

> > "module" sounds expensive..  I've asked scott if he did this mod yet to see

> > what other parts are involved.  He is busy and won't get to it for a few

> > weeks.  Let me know if you get into it and see other stuff is needed.  I'll

> > look at the microfiche tomorrow also.  Troy

>

> 	Thanks Troy for the info.  I just phoned up the dealer,

> appearently in order for the whole 6N1 pacakge to work you need a bunch

> of thing beside the rod which is the part you have listed above.  The

> center piece for this a new door lock, which itself costs about $250 CDN,

> the ball park for the whole package is about $280 cdn or so.  And since

> they are stocked in the States and takes about two weeks to get to B.C. I

> will pass it this time around.  Originally I planned to have them install

> next Tues, when I will bring the car in for replacing the outsider

> mirrors that were damaged due to vandalism.

Just visited my parts dealer.  On the two items for the 6N1 mod (if all

of them are needed), the rod itself was around $2 list, but the lock

module was somewhere around $110 list (eek!) .

I am hoping we won't need the module, since I'd rather by the $2 part,

than to purchase both!  We'll have to wait for Scott M's report.

Also had him check on the '95 1LE Intake Elbow.  The part number and

description were right in the computer, but when he hit the key to obtain

the price for the part, the number came up as "no good".  He cross

checked it with another list to see if the part number changed as they

usually do, and no luck there.  He proceeded to call some automated

number at GM and it said the part number was valid, along with a description

of the part.  He said that I'd have to call him back to tomorrow because

he'd have to talk to someone about what the price was.  It'd be interesting

to see if the list price he comes up with is the around the $150 price

which is floating around.

Evidently, there were supposed to be some liners for the cupholders

in the console of my '95, which were absent when I took delivery of the

vehicle. (I wonder who took them and why, or if they just forgot them

when they were building the car.)

Anyway, they're listed in the parts catalog as

10261527 LINER-F/F $2.10 (list each, as of Jan-96)

--

Rush Luangsuwan

rush@mcs.com

'95 Blk SS

------------------------------

Date: Mon, 22 Jan 1996 17:12:32 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Subject: Relocating the Air Temp. Sensor on the Intake Elbow

Message-ID: <960122201036_124187841@emout05.mail.aol.com>

O.K., all you "hungry for new modifications" people out there, I read

something to throw around for awhile.  It was suggested in an article, that

the air temp sensor that comes on the intake elbow be moved to a location

nearer to the air filter box.  Why? Because as the car gets warmed up,

especially in the summer, the sensor and the elbow get hot due to being

almost on top of the motor and insulated by the hood.  This creates a "lean

condition" because of a seemingly inflated air temp reading from the heated

sensor.  Which, in turn, forces the computer to refer to different air/fuel

tables.  If, however, the sensor is moved to where intake duct work is

cooler: i.e. by the air box, the computer gets a more accurate reading from

the sensor for a proper air/fuel mixture.

Any thoughts on this one??  If nothing else, it's an interesting idea.

..Bruce / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

Date: Mon, 22 Jan 1996 20:00:40 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: impala@east.xsis.xerox.com

Cc: Slawkoz@aol.com

Subject: Choking

Message-ID: <199601230400.UAA17501@iceland.it.earthlink.net>

Chris,

>Help!  My '95 Impala seems to be choking on something.

1.) Try replacing the fuel filter-might ta gotten some bad gas.(:

2.) Check for a loose plug and/or coil wire.

Hope this helps.<g>

... Lennie

------------------------------

Date: Mon, 22 Jan 1996 20:00:52 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: impala@east.xsis.xerox.com

Cc: BGordon401@aol.com

Subject: Resistor

Message-ID: <199601230400.UAA17511@iceland.it.earthlink.net>

Bruce:

>1995 Cherry Metallic...That FINALLY has a real oil guage, after battling to

>find the resistor for two weeks!! :-)

About ready to do battle with that wire harness to find that resistor.<g>

Was it behind the glove box?

... Lennie

------------------------------

Date: Mon, 22 Jan 1996 20:18:11 PST

From: "TODD M. MCMURRY" <73543.3717@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: loose windshield

Message-ID: <960123041810_73543.3717_FHD41-2@CompuServe.COM>

I have a 1996 black IMPALA SS which I am very proud of. I went to the factory

for a tour the day my car was made. it is pretty

awesome to see 35-40 SS"S roll off the assembly line every hour. Anyway back to

the reason for my message. I have had a few

problems with my car (1) front left speaker was bad and had to be replaced. (2)

both passenger doors had to be adjusted because

of miss alignment and wind noise, & Im not sure they are adjusted correctly yer

(3)  Wireing harness between transmission and body causeing a squeek. But the

big question is has anyone experienced a problem with a windshield pop when you

hit a bump?  It sounds like the windshield is slapping the frame, I would like

some feedback before I go and jerk the windshield out of my car for no reason.

`				Todd M. McMurry

Borger Texas

806/273-3781

73543.3717@compuserve.com

------------------------------

Date: Mon, 22 Jan 1996 22:10:07 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: impala problem database

Message-ID: <960123004609_302478331@mail06.mail.aol.com>

>. You might also be wise to send out a sample of

>the information you want (e.g. mileage, repair costs, symptoms, etc.)

I have been getting alot of information for the Impala SS Problem Database

from fellow list persons.  Please include the following when reporting your

problems to my impalaclub@aol.com address:

Car:

Year:

VIN:

Mileage:

Symptoms:

Problem:

Repair costs:

Dealer satisfaction in handling problem:

Thank you, Troy

------------------------------

End of IMPALA Digest 249

************************

IMPALA Digest 250

Topics covered in this issue include:

1) Shift Linkage Recall

by David Hardy <dhardy@crl.com>

2) 1-LE elbow

by "Jeff Gatcha" <foxhound@txdirect.net>

3) Re: roof holes

by Rock528@aol.com

4) Impalafest 96

by Rock528@aol.com

5) Re: Tilt&toot

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

6) Insurance Coverage Denied / Aftermarket

by DScribner@hunter.com (Scribner, Dave)

7) Differential Surgery

by Erich Arndt <EARNDT@mednet.swmed.edu>

8) Temperature Sensor Relocation

by Stewart McLellan <stewartm@magi.com>

9) The SS Emblem Trim Debacle

by Ed Runnion <erunnio@hubcap.clemson.edu>

10) Intake Elbow for SS

by Franklin Poole <franklin@intersurf.com>

11) Re: Racing SHO's

by Robert Cheek <rcheek@one.net>

12) 9C1, Tires and Rigidity?

by T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

13) Re: Air intake sensor

by Rock528@aol.com

14) DIFF GASKET

by Clyde Twine <clyde.twine@hok.com>

15) Re: IMPALA digest 249

by cohan02@cai.com (Andrew Cohen)

16) Re: Oil Sensor Resistor

by BGordon401@aol.com

----------------------------------------------------------------------

Date: Mon, 22 Jan 1996 22:32:47 PST

From: David Hardy <dhardy@crl.com>

To: Impala SS LIST <impala@east.xsis.xerox.com>

Subject: Shift Linkage Recall

Message-ID: <Pine.SUN.3.91.960122223116.21740C-100000@crl3.crl.com>

Somebody please email me the exact text of the recall notice I lost my

copy after I had the work done.  I need to add it to the web.

email to :

dhardy@crl.com

-Dave

dhardy@crl.com  %%%%%%%%%%%%%%%%%%%%%%%%%%%%%

"The thief is sorry he is to be hanged,

not that he is a thief" (Thomas Fuller, 1732)

------------------------------

Date: Mon, 22 Jan 1996 23:29:59 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: 1-LE elbow

Message-ID: <199601230724.BAA14791@legend.txdirect.net>

I had a friend of mine at Fuel Injection Specialties check with his

chevy parts guy to see if he could get me a 95 1-LE elbow at a

reduced price.

. The part number is in fact correct but the price doesnt show up.  He

called me today and said his parts guy told him the elbow is on its

way and will cost about $75.  I think retail price was somewhere

around $90-$100.  I was relieved to hear it wasnt $155.

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala. REAL Oil press gauge,

Muffler delete, Cone K&N ....so far....

------------------------------

Date: Tue, 23 Jan 1996 00:06:12 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: roof holes

Message-ID: <960123004612_302478350@mail04.mail.aol.com>

> Does anyone know where someone could get a plug for this approximately

>1in. diameter hole (in case I go for the 9C1)?

Gene,

Go to the radio shop (Motorola or similar) that does the police radio

installs/repair in your neighborhood.  Any LEO should know where it is.  They

should have plenty of hole plugs or you can use one of the antenna hole

plugs.  Troy

------------------------------

Date: Tue, 23 Jan 1996 00:49:06 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Impalafest 96

Message-ID: <960123015857_302503472@mail04.mail.aol.com>

Greetings,

I've been getting alot of email on interested persons for the upcoming natl.

meet in Dallas pending confirmation of the factory tour.  A little

preplanning on interstate routes and we could pick up members along the way.

There is safety in numbers when traveling through Ford country.

Florida guys will probably use I-10 accross FL, AL, LA, etc. into TX.  You

guys from IL could map your way and let other members know about where you'll

be on certain days. I have access to a voice recorder mailbox that could be

used for updates, times, etc.  What other star wars technology can we utilize

to coordinate meeting areas?

Thanks, Troy

------------------------------

Date: Tue, 23 Jan 1996 03:21:58 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: impala@east.xsis.xerox.com

Subject: Re: Tilt&toot

Message-ID: <Chameleon.960123062709.hollenbe@mustang.east.xsis.xerox.com>

On Tue, 23 Jan 96 01:12:03 EST  impala@east.xsis.xerox.com wrote:

>Date: Mon, 22 Jan 1996 02:03:47 PST

>From: John.Salmon@bridge.bst.bls.com

>To: Impala <impala@east.xsis.xerox.com>, Rush <rush@mcs.com>

>Subject: Re:Tilt&toot

>

>I just had my car in at the dealer for all the pertinent TSB's and the

>transmission recall.  They at first balked on the "tilt and toot" fix

>unless the horn was actually having the problem.  I told him the horn

>was hard to blow, which was a safety hazard in my opinion and would be

>fixed by the TSB's recommended procedure, and he agreed to look up the

>TSB and wound up ordering in a new complete steering wheel/airbag/horn

>assembly from Memphis.  I noticed that the new wheel has two horn

>symbols in the center, where the old one had the horn symbol in the

>upper left.  The old airbag assembly had to pivot to press the horn

>button, but now it appears they have moved the horn switch to right on

>top of or underneath the airbag module, because I now can blow the horn

>from both sides of the center seam right in the middle of the airbag pad

>(like it should be!).  Now I can even tap out "shave and a haircut" if I

>want to!

I just had my '95 in for the shift lever recall last week, and also asked my

dealership service department to look into the horn switch replacement.  The

service advisor wouldn't order the parts until they had a look at the horn.

I brought the car in and told him about the TSB and the redesigned switch.

After playing with the horn for a while he finally agreed to order the parts.

It'll be nice to have a horn that works when you need it!

==================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street, Lower Lobby, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

==================================================

------------------------------

Date: Tue, 23 Jan 1996 06:49:51 PST

From: DScribner@hunter.com (Scribner, Dave)

To: impala@east.xsis.xerox.com ('ImpalaClub')

Cc: DBowen@hunter.com (Bowen, Denny)

Subject: Insurance Coverage Denied / Aftermarket

Message-ID: <1996Jan23.082300.1442.13756@diablo.hunter.com>

No coverage because of 15" sno-tires and rims?

In hindsight....."who makes 17"/ 50 series snow-tires?".  ABS accuracy

has historically been based on tire diameter.  GM has issued charts for

vehicles such as light trucks with RWAL anti-lock brakes to "reprogram"

the computer for varying tire diameters.  If the 15" set of snow-tires

are within the acceptable tolerances of the factory wheel diameter then

ABS should not be effected. (Last sentence is an opinion.)

Tough (if not impossible) situation to fight.

Hunter Engineering is actively involved on the MAP (Motorist Assurance

Program)  MAP is a group of OEM's and repair groups working together to

write repair guidelines for the industry.  (see hunter.com, "Whats New")

We are currently writing the Brakes and ABS section of the "Guidelines

for Service Repairs".  Many OEM's, large service groups and repair shops

are asking for clarification on exactly this issue.  There are shops that

aren't even sure if they should perform ABS brake repairs on vehicles

with non-OEM size wheels!  We are waiting for some comment from the OEM's

and NHTSA on these issues.

The only real advice I can offer at this point is:

DON'T GET CAUGHT BETWEEN THE VEHICLE MANUFACTUER, INSURANCE COMPANIES AND

THE U.S. GOVERNMENT.

You can't win.  Nobody at this point will endorse anything besides OEM

fitment.  My question is why would anyone want to stick their neck out?

Good reason for a 17" snow tire that does not exist.

------------------------------

Date: Tue, 23 Jan 1996 07:16:46 PST

From: Erich Arndt <EARNDT@mednet.swmed.edu>

To: impala@east.xsis.xerox.com

Subject: Differential Surgery

Message-ID: <s104a7be.091@MEDNET.SWMED.EDU>

Ok, boys and girls;

I took my '94 into the dealership yesterday for a "grinding/klunking" noise

in the rear, particularly prevalent when turning.  For those of you old

fogies from the  '60s, it sounded EXACTLY like the old Detroit Locker

klunk except noisier.  This is the second visit to the dealer for that

particular problem; the first go-around the General did the bit with the

extra additive to no avail.  After running it for a week that way, the

problem continued with a new twist;  at a 20mph punch (twice), the rear

of the car shuddered so bad I thought it was going to come unglued at

the seams.

This time they're REPLACING "the carrier assembly and bearings".   I

have no idea why they're not just rebuilding the existing one.  Any

thoughts?

The car just turned 20k miles, and this is the first differential I've heard of

on the list that actually had to be replaced prematurely.  It's funny, on this

go-around the dealer gave me no static about using the FelPro gasket

instead of the GM...

My question to the experts on the list with a parts catalog handy is what

all is included in the carrier assembly for the '94 Impala and which

bearings might they be referring to?

Thanks,

Erich Arndt

'94 Impala (in surgery)

------------------------------

Date: Tue, 23 Jan 1996 07:20:52 PST

From: Stewart McLellan <stewartm@magi.com>

To: BGORDON401@aol.com

Cc: impala@east.xsis.xerox.com

Subject: Temperature Sensor Relocation

Message-ID: <3104FCD4.3BB2@magi.com>

Bruce:

Read your proposal in Digest 249.  FWIW, I think the plastic(?) of the elbow will

insulate the sensor from underhood heat.  The sensor is reading the temperature of the incoming

air flowing past it, not the temperature of the mount.  I stand to be corrected, but I don't

think that moving the sensor, even if you could and still maintain system integrity, would gain

anything.  You might see a degree drop in temp. but probably not enough to make the engine run

appreciably richer.

Regards, Stewart McLellan

-----------------------------------------------------------------------------

"Nothing in life is so exhilarating as to be shot at without result."

- Winston Churchill

-----------------------------------------------------------------------------

------------------------------

Date: Tue, 23 Jan 1996 07:40:34 PST

From: Ed Runnion <erunnio@hubcap.clemson.edu>

To: impala@east.xsis.xerox.com

Subject: The SS Emblem Trim Debacle

Message-ID: <199601231540.KAA27395@hubcap.clemson.edu>

Hey,

I had an interesting problem occur last week while washing "BMF" (a

Pulp Fiction reference I named the car after), my 1996 Impala SS.  While

getting the water off the car, I accidently snagged the corner of the last

"S" on the right-rear quarter panel and took off a 1 inch section of it!  As

the words "Oh S***" flew through my mind, I managed to find the little

piece of black plastic (Thank GOD!!).  My question now is, what is the

best way to re-attach the plastic to the car?  Obviously, I want it

to look as close as possible to new, and also to stay attached to the

car.  Any suggestions?

TIA

--

________        __

/_/_/_/_/       /_/   Ed Runnion, MSEE Student Clemson University

/_/___    ______/_/   Center for Semiconductor Device Reliability

/_/_/_/   /_/_/_/_/           Research

/_/_____  /_/___/_/   email   : erunnio@hubcap.clemson.edu

/_/_/_/_/ /_/_/_/_/   Homepage : http://www.clemson.edu/~erunnio/

------------------------------

Date: Tue, 23 Jan 1996 07:46:17 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Intake Elbow for SS

Message-ID: <310502C9.4A55@intersurf.com>

I wrote a message last week about this but it apparently went into the

bit bucket because I never saw it in a digest.  Anyhow---

The '94 intake elbow does not include the vent tube nipple for the

distributor air vent.  It carries a part number of 25147210 with a

retail price of $95.63.  My dealer gives me 30% discount for a price of

$66.94

The '95 elbow carries a part number of 25147187 and is listed as an

export part number.  This is why there is no price in their computer.

They can order the part if they want it.  My dealer assumes that this is

an export part because the resonator tube is sealed off.  We came up

with an approximate price of $152 retail or $106 with discount.

My friendly advice is: go with the '94 part and hide the distributor

vent connection.  Here's what to do:

On the first rib from the throttle body, about 20deg to the driver's

side from the bottom, use a drill bit just smaller than the blue vent

tube elbow and bore a hole through the rib.  You can then push the blue

vent tube into the hole and hide "yetanothervacuumtube" from view.

You'll need to trim the vacuum tube to length (about three inches I

think).  When finished you'll have a factory looking intake elbow and a

hidden distributor vent tube.

The bottom of the elbow has an oval tube protruding from it.  That is

the resonator connection, which you'll notice is not opened.  Careful

work with a razor knife or razorblade will allow you to cut off the tube

and leave a clean surface on the bottom.  A little mistake won't cost

you much because about the only place you can see that spot is from

underneath the car.

My dealer gave me a proper description of these parts as:

"air cleaner outlet rear duct" and

"air cleaner outlet rear duct (export)"

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Tue, 23 Jan 1996 08:25:53 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Re: Racing SHO's

Message-ID: <199601231625.LAA00668@shell.one.net>

> A stocker SHO against a stocker SS should be no contest,

> but be careful about bragging rights until after you

> run.

Be sure to race him from a stop, not a rolling start, as

the SHO's like the higher RPM ranges and the Impala is big

on low end torque. You might not win a rolling race, especially

since he has the 5 speed and you need to rely on the auto

to downshift appropriately (which has let me down a few times).

The front drive SHO will also have more problems getting off

the line because of the weight transfer to the rear wheels.

Good luck,

------------------------------

Date: Tue, 23 Jan 1996 10:02:37 PST

From: T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

To: IMPALA@east.xsis.xerox.com (Non Receipt Notification Requested)

Subject: 9C1, Tires and Rigidity?

Message-ID: <0018jjnhlrht*/I=T/S=MAKICHUK/OU=FA.EPS/O=NHQ/PRMD=GC+DRHC.HRDC/ADMD=GOVMT.CANADA/C=CA/@MHS>

>'96 9C1 production has been sold out for some time now

Well as second choices go, a 96 Impala SS built on or before December 17,

1996, isn't much of a compromise!  :-)

>you should get P275/60-15 blackwalls for your '88 9C1.  That is what I

have on my '88, and they not only fit perfectly, they are the same diameter

as the OEM tires.

Scott, these tires sound radical, but I'm not sure that they are exactly

the same diameter as the 225 70 15.  They do seem perfect to upgrade the

235 70 15 used from 92? to 95 on the 9C1.  FWIW Chevy has gone back to the

225 70 15 size for the 96 9C1, and dropped the GT+4, a tire used in this

application since 1988, and gone with the Eagle RSA.

Can someone explain how the current engineering/marketing trend of

referring to the rigidity of unibody cars, relates to full frame vehicles

like the B body?  I know that the Impala SS and the 9C1 sport "increased

gage of certain frame members".  So what gives, are the unibodies stiffer

than the B body, and just what is the resonate frequency (Hz) of the

SS/9C1?

Tip Makichuk

"I can't sell 'em to ya, cause there illegal"

------------------------------

Date: Tue, 23 Jan 1996 12:57:21 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Air intake sensor

Message-ID: <960123155717_404448386@emout06.mail.aol.com>

In a message dated 96-01-23 01:19:10 EST, you write:

> If, however, the sensor is moved to where intake duct work is

>cooler: i.e. by the air box, the computer gets a more accurate reading from

>the sensor for a proper air/fuel mixture.

>

>Any thoughts on this one??  If nothing else, it's an interesting idea.

Bruce, a few digests back I posted an article from the F body list that shows

the various tempuratures and the resistance values for the air intake sensor.

Mike Marx (mbmarx@aol.com) recently experimented with putting a fixed value

in there simulating an outside air temp of about 40 or 50.  Other than idle

changes I'm not sure of what other results he had.. (Mike?? are you out

there?)  Troy

------------------------------

Date: Tue, 23 Jan 1996 13:48:07 PST

From: Clyde Twine <clyde.twine@hok.com>

To: impala@east.xsis.xerox.com

Subject: DIFF GASKET

Message-ID: <9601232148.AA05257@portal.hok.com>

I took the beast in to have the fuel pump checked out (making a loud whining

noise) and to have the scheduled 7500mi service done on the differential. I

had previously purchased the Fel-Pro replacement gasket, Mobil 1 oil and

additive to do the job myself, but I figured since the fuel pump needed

looking at I might as well let them do the diff also. They wound up

replacing the fuel pump...defective from the factory I guess. Now for the

fishy part.  The bill showed them charging me for two quarts of GM diff oil.

When I asked what they did with the Mobil 1, they said they used it also.

The service tech said that the diff held 3 quarts of oil plus the additive.

He also said that Mobil makes GM's synth diff oil so there was no problem

with mixing the two. If someone can tell me that this all sounds o.k., I'll

stop wondering if they actually replaced the gasket.  How would I be able to

tell other than checking for "clean spots" on the bolts or something?  I

cant think of any reason they would have for not replacing it, but I dont

think I would have any recourse when the thing locks up at ~70K.

ct

------------------------------

Date: Tue, 23 Jan 1996 14:17:46 PST

From: cohan02@cai.com (Andrew Cohen)

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 249

Message-ID: <9601232217.AA22938@cai.com>

John Salmon wrote:

> I just had my car in at the dealer for all the pertinent TSB's and the

> transmission recall.  They at first balked on the "tilt and toot" fix...

Could someone point me to a list of the TSBs', or if possible email or

post them to the list?  My dealer says that the car (95 SS) hasn't been

recalled for anything, and I know that just ain't so.  Thanks.

Andrew Cohen

cohan02@cai.com

95' Cherry Dark Cherry

------------------------------

Date: Tue, 23 Jan 1996 16:33:28 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Oil Sensor Resistor

Message-ID: <960123192848_125165045@mail04.mail.aol.com>

In a message dated 96-01-22 23:01:41 EST, Lennie write:

>About ready to do battle with that wire harness to find that resistor.<g>

>

>Was it behind the glove box?

Yah man. The wire harness is DIRECTLY behind the glove box latch.  Your best

way to get to it is to pull the gray plastic dash off that surrounds the

glove box.  You will notice the little plastic harness connector tabs/nipples

coming through by the metal glove box latch.  Squeeze the tabs, push them

through the other side, reach your fingers up behind the latch (a real tight

fit) and pull the harness out to where you can work on it.  The resistor is

wrapped inside the electrical tape.

I had a hard time with the harness until I pulled the gray plastic dash out

to give myself more room to work.  Just gently pull on the panel until it

pops off.  It held by snap in connectors.

Good luck!!  If you need any more help, drop me a line.  Glad to help.

..Bruce / BGORDON401@aol.com

1995 Cherry Metallic

------------------------------

End of IMPALA Digest 250

************************