IMPALA Digest 301

Topics covered in this issue include:

1) motorweek reviews camaro SS & the 6 spd impala SS

by Steve Elias <eli@cisco.com>

2) RE: Manual tranny

by "Harris, Scott" <sharris@xyplex.com>

3) Digest 300 - mystery 300 HP Impala

by David Hardy <dhardy@crl.com>

4) New '96 coming!!

by Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

5) Speedo error

by "J.R. LEVITZ" <4everGreen@msn.com>

6) New Member!

by ZTMP50A@prodigy.com ( DAVID NMN)

7) Gas Bib

by gader@interaccess.net (Gary Lader)

8) Thanks Super SS Members.

by Loren Peterson <equ_rino@cencom.net>

9) Arlington, TX home page

by Loren Peterson <equ_rino@cencom.net>

10) Re: Crane roller rockers

by Kerry Bohannon <75511.1220@compuserve.com>

11) Steering Knuckle???

by Steve Das <sdas@sat.net>

12) Re: Wounded Shamu

by Robert Cheek <rcheek@one.net>

13) Vericom Computer

by Steve Das <sdas@sat.net>

14) Another interesting Problem

by Ed Runnion <erunnio@hubcap.clemson.edu>

15) Re: Manual Transmission Convert

by Rock528@aol.com

16) Clyde Twine... where are you?

by Steve Das <sdas@sat.net>

----------------------------------------------------------------------

Date: Tue, 20 Feb 1996 11:23:12 PST

From: Steve Elias <eli@cisco.com>

To: impala@east.xsis.xerox.com, f-body@cisco.com

Subject: motorweek reviews camaro SS & the 6 spd impala SS

Message-ID: <199602201923.OAA08105@cisco.com>

hi,

in case you f-body & impala folks can view the show "Motorweek" on

your local cable/antenna/whatever, they do a re-review of various John

Moss custom impala SS cars on this week's show.  also they review the

96 Camaro SS, or 96 Camaro Z28 SS, whatever is the actual name

of the car...

/eli

------------------------------

Date: Tue, 20 Feb 1996 02:06:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Cc: "'les@makro.com'" <les@makro.com>

Subject: RE: Manual tranny

Message-ID: <312A29C7@corvette.xyplex.com>

Les,

That sounds like to good of a deal!  I would thoroughly check this guys

credentials.  Has he done this before?  I read of a few being done by a

shop (in Atlanta?) and they had to modify the floor in order to fit the

tranny.  They then installed a different console and seats because the

originals wouldn't fit after the floor/tunnel mods.  The cost was in the

$4,000 ball park I believe.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Tue, 20 Feb 1996 12:38:27 PST

From: David Hardy <dhardy@crl.com>

To: impala@east.xsis.xerox.com

Subject: Digest 300 - mystery 300 HP Impala

Message-ID: <Pine.SUN.3.91.960220123632.2613B-100000@crl5.crl.com>

I thought that there were a few limited edition Impala's billed as Dale

Earnhart Impalas that were sold from the Intimidators dealership...Some

one posted that here once didn't they?

-Dave

dhardy@crl.com  %%%%%%%%%%%%%%%%%%%%%%%%%%%%%

"The thief is sorry he is to be hanged,

not that he is a thief" (Thomas Fuller, 1732)

------------------------------

Date: Tue, 20 Feb 1996 14:45:01 PST

From: Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

To: impala@east.xsis.xerox.com

Subject: New '96 coming!!

Message-ID: <312A4EED.7C40@lfwc.lockheed.com>

Just heard at 14:20 today that the body of my SS will be leaving the

paint shop within the next two (2) hours.  The chassis has already had

some operations completed, and the two should be mated tomorrow.  What

fun!! Should be interesting!  Stay tuned. Later.

Mark

Black '96 being built now!

------------------------------

Date: Tue, 20 Feb 1996 13:17:25 PST

From: "J.R. LEVITZ" <4everGreen@msn.com>

To: impala@east.xsis.xerox.com

Subject: Speedo error

Message-ID: <UPMAIL01.199602202323150516@msn.com>

Bruce Gordon writes in digest 300..................

For the rest of you that want to do the 3.42 swap, but are concerned with the

"MONSTER" speedometer reading error, it accounts for a 10mph difference.  My

wife paced me with her Bonneville, and when I was going 65, her speedo read

55.  So, this monster turned out to be a "Grover" instead.  And yes, the

shift points are higher without the proper PCM calibration,............

Yeah, another gear convert!!!!   The speedo error is not that big at 65mph

and as you stated, it can be driven until your calibration is done. But, just

to be exact, the speedo error is not linear. It is more like exponential. Why

don't you go out for a high speed run and report to us a 180mph blast!! I know

of an F-body owner who reported his speedo reading 140 when he was acctually

going only 100mph. He knows because it was at the drag strip. Gears were

changed from 2.73 to 3.73.

Regards John

------------------------------

Date: Tue, 20 Feb 1996 15:31:46 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: New Member!

Message-ID: <097.01664586.ZTMP50A@prodigy.com>

Hi all!  Glad to be aboard.  Hope this goes to the correct locale..

<G>  I have a  '96 Black SS.  For now, it is completely stock, but

have plans to correct this soon.  Has anyone added a supercharger to

their SS?  That seems to be the path I want to go, CARB certification

withstanding.

Thanks!

Dave.

------------------------------

Date: Tue, 20 Feb 1996 15:47:01 PST

From: gader@interaccess.net (Gary Lader)

To: impala@east.xsis.xerox.com

Subject: Gas Bib

Message-ID: <199602202347.SAA16508@sparky.interaccess.net>

I checked with my local dealer (they are supposed to have the largest parts

dept. in Central FL) and they have no clue how to find the gas bib.  Can

anyone help me out with the part number?

Gary

------------------------------

Date: Tue, 20 Feb 1996 15:55:59 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Thanks Super SS Members.

Message-ID: <312A5F8F.1BD0@ns.cencom.net>

Thanks!

I really appreciated all the help in getting my act together regarding

posting without attaching the universe.

Also, while I promised to die if I screwed up the last post, I was

unable to keep my promise as I could not bear to leave my Dark Cherry

Beauty.  I hope my Wife never figures out how much I love that SS.

Thanks again, and hope to see ya in TEX!

Loren

DCM96

------------------------------

Date: Tue, 20 Feb 1996 16:18:31 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Arlington, TX home page

Message-ID: <312A64D7.437C@ns.cencom.net>

Hi SSers

Anyone going to Arlington, TX in April?  If you are, you might like to

tune in to Arlington's home page: http://www.dfw.net/~acvb/acvb.html

It has a lot of info, but not as much as these pages!

Loren

DCM96

------------------------------

Date: Tue, 20 Feb 1996 16:43:14 PST

From: Kerry Bohannon <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Crane roller rockers

Message-ID: <960221004313_75511.1220_GHJ122-1@CompuServe.COM>

Got my Crane roller rockers, K Motion springs and ground down keepers installed

last night. Scott thanks for your help. It's obvious when you do the

installation that you need to grind .40 off the top of the intake keepers or the

spacers on the side of the roller will hit the top of the keepers. After asking

so many questions and not understanding why you need to do this it became

apparent quickly.

I do notice improvement in power but roads have been too wet to really get on it

for a good road test.

Some bad news: as we were applying air pressure to keep the valves up. We used a

leak tester where you put in 100 psi on one gauge with the air regulator and a

second gauge is used to see leakage or blowby. Well I have a problem (nothing to

do with the roller rockers).

Number:

One	2psi

Three	4psi

Five	2psi

Seven	2psi

Two	4psi

Four	2psi

Six	20psi	Shi..........

Eight	18psi	Shi..........

This may also explain why I feel my gas mileage (15mph all the time while other

SS's get 18mph) is not up to par as well as sometimes I feel the engine runs

rough. I did bring it to the dealer twice with this complaint and was told

everything was fine. So I need to set up a time with the dealer to explain my

delima. This really stinks as there must be something serious with these two

cylinders or valves and convincing the dealer I have a problem is not the

easiest either. I did save all the plugs and marked the cylinder it came out of

and #6 and 8 are different than the rest in appearance. I hope this will help.

Looking at a new short block? or head work?

Well,  "cars are fun but dealers can be a pain!"

I'll keep you posted on my SS's condition and results.

Curt P.

------------------------------

Date: Tue, 20 Feb 1996 16:50:35 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: Steering Knuckle???

Message-ID: <199602210049.SAA12590@raven.sat.net>

>So the moral of the day is, next time when you have wheel off for

>tire, you may want to check your caliper to see if it's loose and check

>if the clearence is out of spec.

>

Mike,

I give up, what part do you mean by "steering Knuckle"???

------------------------------

Date: Tue, 20 Feb 1996 17:57:58 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Re: Wounded Shamu

Message-ID: <1.5.4b11.32.19960221015758.006892a8@mail.one.net>

>Anyway, I've heard black is the toughest to match because

>of the sun bleaching it. I hope I don't have to have the whole hood

>repainted has anyone else had similar body work done on the "Black" keelor

>whale.

I had my whole hood replaced :-(  It looks fine: no problems matching. I

had always heard that black was easy: white is always one shade or another

off.

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Tue, 20 Feb 1996 19:03:07 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: Vericom Computer

Message-ID: <199602210301.VAA22177@raven.sat.net>

Well, the latest addition to the "boy's toys" arrived today and I must say,

the Vericom Computer is OUTSTANDING!!  Went out tonight and played around

with myself and one of my 200# friends in the car along with a full tank of

gas (~130#).  I must say I am VERY impressed with the repeatability of the

computer.  The data available is nothing short of amazing.  I intend to go

out this weekend, weather permitting, and do some controlled runs for the

record and I will post the results.  One of the amazing tests we did tonight

was a 60 to 0 braking test.  How about 132 feet (dry concrete) without

really getting deep into the ABS??? Not bad if you ask me!  Anyway, stay

tuned for the results and I must say, if any of you lads were thinking of

getting one of theses while the initial deal is on, jump for it, if you like

cars, you will LOVE this thing.

Steve...sdas@sat.net

------------------------------

Date: Tue, 20 Feb 1996 21:43:49 PST

From: Ed Runnion <erunnio@hubcap.clemson.edu>

To: impala@east.xsis.xerox.com (Impala)

Subject: Another interesting Problem

Message-ID: <199602210543.AAA13557@hubcap.clemson.edu>

Hey,

Just got the car back from the dealer this afternoon after

being in the shop all day.  I had an interesting part break on

my 3 month old '96 named "BMF" : the driver's door handle broke!

It broke somewhere inside the door, and I had to go thru one of

the other doors and pull on the inside handle to open the

driver's door (I am 6'4", crawling myself over the center

console was NOT an option :^).  The dealer here in Houston could

not find a door handle to match the car anywhere in the Houston

area (and that my friends is a LOT of GM dealers) so they put a

temporary Silver Caprice handle on the door so I could use it

until the part comes in in a week or so.  Can we say UGLY part,

I thought so :^).  Well, in three months ownership I have had

these problems, not too bad but not a Lexus either:

1. Driver's door light switch disconnected from factory.

2. Gas Tank shield loose from factory

3. Developed a small windshield creak over bumps, I am not

even gonna worry about this one, it is a GM after all...

4. Busted Driver's door handle.

Other than that, the car has been great and even hauled a 2500

lb trailer of my worldly possessions from SC to TX behind it.

It was sorta fun having more acceleration power WITH the trailer

than most empty econoboxes :^).

Well, enough rambling.  If you have a door handle break, you are

not alone but expect to "stick out" for a while.

Ed from Houston in the ugly door handled 96 SS named BMF

--

________        __

/_/_/_/_/       /_/   Ed Runnion, CU BSEE 1994, MSEE 1996

/_/___    ______/_/   Flash Memory Reliability Engineer

/_/_/_/   /_/_/_/_/   Texas Instruments, Houston TX USA

/_/_____  /_/___/_/   email   : erunnio@hubcap.clemson.edu

/_/_/_/_/ /_/_/_/_/   Homepage : http://www.clemson.edu/~erunnio/

------------------------------

Date: Tue, 20 Feb 1996 21:56:01 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Manual Transmission Convert

Message-ID: <960221005600_427834469@emout08.mail.aol.com>

Hi all,

Can someone contact Doug Rippie Motorsports?  I'm not sure of their address,

but per Chevrolet, Rippie is the one that got all the tooling from the 6

speed install in their prototype.  He is supposed to do the swap for about

5K.  Can someone do the homework on it and report back??

Troy

NAISSO

------------------------------

Date: Wed, 21 Feb 1996 10:42:52 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: Clyde Twine... where are you?

Message-ID: <199602211842.MAA00149@raven.sat.net>

Sorry to do this to the net but I've lost Clyde.

Clyde, your system is not accepting responses.  FIX IT so we can arrange for

Friday night!!

Steve

------------------------------

End of IMPALA Digest 301

************************

IMPALA Digest 302

Topics covered in this issue include:

1) 15,000 Mile Report

by Mfetto@aol.com

2) Impalafest 96 Update/Hotels

by Rock528@aol.com

3) Newsletter Update

by Rock528@aol.com

4) Doug Rippie 6-Speed Info

by Erich Arndt <EARNDT@mednet.swmed.edu>

5) Leakdown test results

by Scott Mueller <73145.1566@compuserve.com>

6) Re:Steering Knucle???

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

7) Re: IMPALA digest 301

by Derkiller@aol.com

8) Re: Lokar shifter

by Curt Platteborze <75511.1220@compuserve.com>

9) Re: Crane Roller rockers

by Curt Platteborze <75511.1220@compuserve.com>

10) New '96

by Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

11) Vericom Performance Computer

by Steve Das <sdas@sat.net>

12) #301-Vericom Cptr./Clyde.....

by Fast94SS@aol.com

13) Popping/Banging Noises While Turning

by Robert Cheek <rcheek@one.net>

14) Torque Converters

by Robert Cheek <rcheek@one.net>

15) Last ditch warranty work

by Robert Cheek <rcheek@one.net>

16) Re: Arlington Communications

by Rock528@aol.com

----------------------------------------------------------------------

Date: Wed, 21 Feb 1996 13:48:16 PST

From: Mfetto@aol.com

To: impala@east.xsis.xerox.com

Subject: 15,000 Mile Report

Message-ID: <960221164815_328970029@mail06.mail.aol.com>

1994 Impala 'SS in service 10 June, 1994

15,000 miles on 15 Feb. '96

Oil/filter changes every 3,000 miles - Havoline3 5W-30

Average fuel mileage: 17.5 mpg, mixed driving.

Mods: Oil Pressure Sender - no bias change.. I like the dynamic range.

Fel-Pro gasket along with fluid change at 8,000 miles.

225/15 M&S Firestones for winter, baby moon hubcaps

Service: Rear end, chassis lube...that's it. My service dept. is average.

Accidents: One, new hood & front grille (hit a van with one of those stupid

ladders on the back door, who needs a ladder on a van ???)

Quirks: No squeaks, leaks or loose parts.

Complaints: Electric antenna (hate 'em); interior, instumentation lacks some

of the extras that my Bonneville SSE had.

Overall comments: Runs great, reliable, good in the snow. Get a lot of

positive comments on it, especially at Watkins Glen.

Hope info is useful,

Mike

------------------------------

Date: Wed, 21 Feb 1996 13:54:42 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Impalafest 96 Update/Hotels

Message-ID: <960221165441_150009166@emout04.mail.aol.com>

Here is a tentative list of Arlington Hotels we are working on for the April

meet.

Can one of you TX guys hammer them for a corporate or government rate and get

back to me??  Covered and security for the parking ares are priority, beds

and showers are optional...

Radisson Suite Hotel

700 Avenue H East

Arlington, TX   817-640-0440

Comfort Inn/Metroplex Apartment Suites

1601 E. Division

Arlington   817-261-2300

Country Inn

1075 Wet n' Wild Way

Arlington   817-261-8900

Holiday Inn

Hwy 360 North at Brown Blvd.

Arlington   817-640-7712

Lexington Suite Hotel

Hwy 360 North at Brown Blvd.

Arlington   817-640-4444

Fairfield Inn

I-30 @ Hwy 360,  2500 E. Lamar Ave.

Arlington 817-649-5800 or 800-228-2800

******

Also, here is a update for the tentative schedule:

Thursday: Arrive Arlington

Friday: GM tour 12 noon., car wash day for show.

Friday pm:  local drags or social at Don's Lambert's house.

Friday late pm:  Bail Razzor out of facility for doing burnouts on Don's

street.

Saturday:  Motorplex car show.

Saturday pm: Local cruise if we can find one or we'll make one.

Sunday:  Motroplex car show, giveaway items at club raffle.  (Secret Stuff)

Monday: Go home.

Any other neat ideas???

thanx in advance

Troy

------------------------------

Date: Wed, 21 Feb 1996 13:54:50 PST

From: Rock528@aol.com

To: Mbmarx@aol.com, stc@cypress.com, RAZZORFL@aol.com,

Subject: Newsletter Update

Message-ID: <960221165449_150009261@emout06.mail.aol.com>

Well, all of those post it notes are off my screen and the latest Impala SS

newsletter is a wrap.  Goes to printer tomorrow and snail mail Friday.

Thanks for your patients.  Troy

------------------------------

Date: Wed, 21 Feb 1996 14:19:17 PST

From: Erich Arndt <EARNDT@mednet.swmed.edu>

To: rock528@aol.com, impala@east.xsis.xerox.com

Subject: Doug Rippie 6-Speed Info

Message-ID: <s12b4665.006@MEDNET.SWMED.EDU>

Howdy Crew (and Troy in particular),

I just got off the phone with Peter Sikkila, the shop manager at Doug

Rippie Motorsports (phone: (612)559-7605).

He indicated to me that they have done *exactly one* 6-speed

conversion on a '95 Impala for an unidentified customer in New York-- no

'94s.  His understanding was that the prototype was done by Roush

(fact or fiction?).

Before he had to go, I did get a few sketchy details of the conversion

that he did on the '95 (BTW, he actually did the conversion himself).  In

order to make the conversion work, he used a combination of F and

Y-car parts.  The floor of the car had to be substantially modified and a

custom crossmember was fabricated.  If he was to do one again, it

would cost about $7500.00 (MEGA-ouch!).

I know the info's kinda thin, but hopefully it's enough of a starting place

for someone with access to the right parts catalogs.

FWIW,

Erich

------------------------------

Date: Wed, 21 Feb 1996 14:35:24 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Leakdown test results

Message-ID: <960221223523_73145.1566_FHD49-2@CompuServe.COM>

Curt,

>Got my Crane roller rockers, K Motion springs and ground down keepers

installed last night. Scott thanks for your help. It's obvious when you do

the installation that you need to grind .40 off the top of the intake

keepers or the spacers on the side of the roller will hit the top of the

keepers.

Great, I'm glad this part of the install went well.

>Some bad news: as we were applying air pressure to keep the valves up. We

used a leak tester where you put in 100 psi on one gauge with the air

regulator and a second gauge is used to see leakage or blowby. Well I have a

problem (nothing to do with the roller rockers).

Ouch! You and/or the shop you used should be commended for having the

foresight to do a leakdown test while installing the springs. I think

everybody should do that as it really does help you find problems.

The high readings on adjacent cylinders 6 and 8 usually indicate a problem

with the head gasket seal between the cylinders. This is not really a

serious problem but will require that the head gasket be replaced on that

side. Tell this to the dealer and demand satisfaction!

Now personally, if I had to pull the heads, I would order up a set of the

new aluminum LT4 heads and intake and bolt them on instead! The roller

rockers you got will work perfectly with the LT4 heads, plus you would be

removing 50 lbs of weight from the front of the car. The LT4 heads are

available complete with valves and springs, and are listed as such in the

new GM performance parts catalogs, which just arrived at your dealer about a

week ago. Since the head gasket probably has to be replaced anyway, you

could tell your wife that you will be "forced" to change the heads as well.

<g> Don't show her this message, of course! <BG> Scott.

------------------------------

Date: Wed, 21 Feb 1996 15:12:07 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: Re:Steering Knucle???

Message-ID: <Pine.SOL.3.91.960221150803.6850A-100000@netinfo.ubc.ca>

Hi Steve:

The steering Kuncle (sp. that's what they put down on the

ordering sheet for the parts) I am referring to is the metal plate where

the break caliper mounts.  So I guess when they say the clearence between

the the caliper is out of spec, I guess they either meant the mount holes

are too big so the caliper can't be bolt down tightly (this is probably

the case) or the space between the caliper and the metal plate is too

wide. (This guess makes less sense)

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red

------------------------------

Date: Wed, 21 Feb 1996 15:14:33 PST

From: Derkiller@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 301

Message-ID: <960221181431_150083237@emout07.mail.aol.com>

great information

please remove me from mailing list, fifth request, or reply with problem

------------------------------

Date: Wed, 21 Feb 1996 15:32:45 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Lokar shifter

Message-ID: <960221233244_75511.1220_GHJ161-2@CompuServe.COM>

I sent away for a catalog you may be interested in. This company makes shifters

for the 4L60E transmission. I called and talked to a guy named Chuck. This

shifter has a lot of positioning ability to mount possibly in the 94/95 SS. I

will be looking at the trany this weekend possiblly and keep you posted on how

it may fit. I have also checked with B&M and Hurst and they don't have anything

for the Impala.

For those interested the address is:

Lokar Performance Products

10924 Murdock Drive

Knoxville, TN 37922

Tel: 423-966-2269

Curt P.

------------------------------

Date: Wed, 21 Feb 1996 15:32:42 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Crane Roller rockers

Message-ID: <960221233242_75511.1220_GHJ161-1@CompuServe.COM>

Just to let you know My wife Kerry Bohannon keeps forgetting to change the name

on compuserve when she's finished with the computer.

The Crane post #301 was really from Curt Platteborze Waltham, MA.

------------------------------

Date: Wed, 21 Feb 1996 17:34:50 PST

From: Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

To: impala@east.xsis.xerox.com

Subject: New '96

Message-ID: <312BC83A.A7F@lfwc.lockheed.com>

Just heard my car has been assembled.  I'm told it made all the way

through assembly and testing with ZERO deficiencies. Next is final

assembly, then out to the "big" parking lot, then to the dealer, then

finally home.  After waiting since October, maybe I'll take possession

next week!  Stay tuned. Later.

Mark

Black '96 - No Mods, except I got the gas bib already.

------------------------------

Date: Wed, 21 Feb 1996 16:05:12 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: Vericom Performance Computer

Message-ID: <199602220003.SAA20779@raven.sat.net>

Guys,

Last night, I reported my Vericom Computer had come in and I was really

impressed with it.  Several guys E-Mailed me today and wanted more info and

suggested I repost an article to the list so here goes....

The Vericom Performance Computer is the "standard" for most of the

automobile magazines and performance groups for VERY accurate testing of

your car's performance both in acceleration, braking and cornering ability.

In addition, it will provide you with rear wheel horsepower as well as drag

at any speed!  With one full throttle run for 1/4 mile (or more) it can tell

you an amazing amount of information.  I have made up a sheet to use to

record the stuff it gives me and I have copied that below to give you an

idea.  The whole thing is based on a VERY accurate accelerometer and believe

me, it works.  My background is Physics and this thing is based on very

solid principles of Physics.

Fuel Injection Specialties in San Antonio, Texas is becomming a dealer and

WAS giving a really good deal on the computer for a short time.  I don't

know if they still are but I suggest you call them at 210-654-0774 and ask

for Donald.  They have an E-mail address if you want to use it.  Contact

them at; Fuel Injection Specialties <fuelinjection@fuelinjection.com>.  The

list price of this little jewel is $493.00 but their deal was much better

than that.  I love it and I know if you like messing with cars, you would

love this thing. I say this and believe me, I get nothing out of it except

the satisfaction of seeing you guys get a really neat tool.  Go for it!!

VERICOM PERFORMANCE RECORD SHEET

Vehicle:_________________ Date:________________ Time:_______________

Run#:_______

TIME

1/4 Mile___________

1/8 Mile___________

330 Ft...___________

60 Ft.....___________

30 Ft.....___________

15 Ft.....___________

0-60......___________

0-30......___________

Reaction Time______

SPEED

1/4 Mile___________

1/8 Mile___________

330 Ft...___________

60 Ft.....___________

30 Ft.....___________

15 Ft.....___________

DISTANCE

To 60 MPH ________

To 30 MPH ________

G FORCE

1/4 Mile____________

1/8 Mile____________

330 Ft...____________

60 Ft.....____________

30 Ft.....____________

15 Ft.....____________

Peak G_________@__________MPH

Peak G_________@__________Feet

Peak HP________@__________MPH

***NOTE:***

Rear Wheel HP= Peak HP X Car Weight

Peak HP is specified in HP/1000 Lbs.

------------------------------

Date: Wed, 21 Feb 1996 16:29:20 PST

From: Fast94SS@aol.com

To: impala@east.xsis.xerox.com

Subject: #301-Vericom Cptr./Clyde.....

Message-ID: <960221192918_228025303@emout05.mail.aol.com>

Steve says (excerpted):

..."boy's toys" arrived today....Went out tonight and played around with

myself....You will love this thing.

Clyde, your system is not accepting responses. FIX IT so we can arrange for

Friday night!!

*******

Hi Steve:

Sounds kinda' interesting, maybe,....Do you have anything for us "girlz"??

BTW, count me in for the double duet Friday night, if, you can fix me up,

and she has alot of money, and likes fast cars. Please advise when and where

<G> DJ.

------------------------------

Date: Wed, 21 Feb 1996 17:52:22 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Popping/Banging Noises While Turning

Message-ID: <1.5.4b11.32.19960222015222.0067f068@mail.one.net>

Did we ever come to a conclusion as to what causes the popping

and banging noises from the front end during hard turns? I'd like

to add the cause to the FAQ, if we have a definite answer (or maybe

just several speculations).

On a related note, my car just seems to hate to turn at max lock. It

shivers and bucks considerably. Is this just a fat-tire thing, or

is it something I should be worried about? I just had an alignment,

and it's always done this (it has gotten no better and no worse).

Thanks,

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Wed, 21 Feb 1996 17:52:17 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Torque Converters

Message-ID: <1.5.4b11.32.19960222015217.0067cf04@mail.one.net>

I just wanted to get this stuff straight:

1. Stock torque converter == 1800 RPM stall (free ;-). Correct?

2. Precision Industries converter  == 2500 RPM stall ($750). Correct?

3. 86(?) Vette converter == ???? RPM stall ($???).

4. Regardless of stall speed, the converter will lock up for normal

crusing under PCM control, even if under stall. Correct?

Lastly, could anyone who has installed a non-stock converter e-mail

me with their impressions. How difficult is the install? What

sort of labor rates would I expect? Any "gotchas" I should watch

out for?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Wed, 21 Feb 1996 17:52:20 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Last ditch warranty work

Message-ID: <1.5.4b11.32.19960222015220.0067882c@mail.one.net>

I'm within about 3K of the end of my 36K warranty. Does anyone

have suggestions as to what I should put on my "final wish list"

for warranty coverage? Would it be prudent to have the dealer do

a thorough inspection of the car (possibly for extra $$$) to make

sure everything looks OK?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Wed, 21 Feb 1996 20:02:34 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Arlington Communications

Message-ID: <960221230233_228229949@emout04.mail.aol.com>

Scott & all,

the marine VHF is a great suggestion.  I was thinking cb's but they are such

a PIA.  Are you going to do a magnetic antenna outside or on the rear shelf?

The Florida group will most likely leave on Wed am, staying overnight

somewhere West of Mobile, AL.  The second 10 hour trip on Thursday will bring

us into Arlington around dinner time.

I have spoken with Don (don69z28@aol.com) in Arlington and he is checking

prices on lodging.  I will post this stuff next week sometime.  Pull up those

Arlington web sites for Super Chevy and Texas Motorplex for maps or wait till

my mar/april newsletter.  I will put them in there.

We tentatively decided to stay in the Arlington area and drive to the

Motorplex and Dallas if needed.  Any other suggestions?

Also, pick a VHF radio channel somewhere in the middle of the band for best

reception. Any ideas?  It would be nice for all the Impala's in town to have

reliable communication between them.

Let's start laying down the groundwork so it is not a mad scramble come

April.

Troy

------------------------------

End of IMPALA Digest 302

************************

For interested SS Owners

Loren

DCM96

X-POP3-Rcpt: equ_rino@ns

Return-Path: <Shelley@acvb.org>

Received: from gateway.acvb.org by ns.cencom.net (5.x/SMI-SVR4)

id AA15086; Thu, 22 Feb 1996 12:31:00 -0600

Received: from acvb.org (pc4.acvb.org [172.16.0.4]) by gateway.acvb.org (8.6.11/8.6.9) with SMTP id LAA04018 for <equ_rino@ns.Cencom.net>; Thu, 22 Feb 1996 11:44:30 -0500

Date: Thu, 22 Feb 1996 12:28:26 -0800 (PST)

From: Shelley <Shelley@acvb.org>

To: equ_rino@cencom.net

Subject: Re: Request for Information

X-Sender: Shelley@localway

In-Reply-To: <199602210006.SAA06159@www.dfw.net>

Message-Id: <Pine.PCP.3.91.960222122503.3954B-100000@[172.16.0.4]>

Mime-Version: 1.0

Content-Type: TEXT/PLAIN; charset=US-ASCII

Content-Length: 958

X-Mozilla-Status: 0011

I have forwarded your request to the Arlington Visitors Information

Center, they will send you the information you requested.  In the

meantime, the GM factory is located in Arlington and the Dallas Motorplex

is about 30 miles from Arlington.  The average temperature in April for

Arlington is around 75 degrees.

Shelley                         817/265-7721

Sales & Admin Support           800/433-5374

Arlington Convention        fax 817/265-5640

and Visitors Bureau, Inc.

1250 E. Copeland Rd, #650

Arlington  Texas  76011

Url Address:  http://www.acvb.org/welcome/

On Tue, 20 Feb 1996 equ_rino@ns.Cencom.net wrote:

> Name    Loren Peterson

> Address180 W. Fremont St.

>

> City   Galesburg

> State  IL

> Zip    61401-1735

> Phone  309-343-2010

> Fax    309-343-0204

> email  equ_rino@ns.Cencom.net

> Inf    Hotel Information

> Nearness to GM B Factory

> Distance to Dallas Motorplex

> Ave. temps. in mid April

> Thanks a ton!

> --

>

IMPALA Digest 304

Topics covered in this issue include:

1) Re: Vericom Computer

by Kurt Kemmling <76165.1654@compuserve.com>

2) IMPALA digest 302

by ZTMP50A@prodigy.com ( DAVID NMN)

3) Edelbrock P/Ns

by Ray Lopez <impalass@erols.com>

4) leak down test

by jeffd@accessus.net (Jeff Dawson)

5) Production date

by Scott Mueller <73145.1566@compuserve.com>

6) Down-to-the-wire warranty work

by BGordon401@aol.com

7) "Popping" Sound

by BGordon401@aol.com

8) F-body Elbow

by BGordon401@aol.com

9) polished wheels

by dhflores@iwl.net (Debbie/Henry Flores)

10) Art Car vs. TransGo

by dhflores@iwl.net (Debbie/Henry Flores)

11) Re: Blowby

by Curt Platteborze <75511.1220@compuserve.com>

12) Re:Popping/Banging Noises While Turning

by lennie@earthlink.net (Lennie Dolphin)

13) help again - swaybars

by Rock528@aol.com

14) Ramblings

by mbailey@rinet.hunton.com (Bailey, Martin)

15) RE: Drilled discs

by "Harris, Scott" <sharris@xyplex.com>

----------------------------------------------------------------------

Date: Thu, 22 Feb 1996 15:42:31 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Vericom Computer

Message-ID: <960222234231_76165.1654_GHM102-1@CompuServe.COM>

In digest 301 Steve Das wrote:

>>> if any of you lads were thinking of

getting one of theses while the initial deal is on, jump for it, if you like

cars, you will LOVE this thing.<<<

What exactly is this deal? Are you selling them?

Thanks.

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Thu, 22 Feb 1996 15:51:16 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: IMPALA digest 302

Message-ID: <097.01848218.ZTMP50A@prodigy.com>

Rob, why not tell the dealer about the noise you hear in the front

end along with any other symptoms that may be troubling you?  (You

could even make up a few about the car just to have them check it for

free if you can do without the car for a bit).  Also, to you or

anyone else that has the gas bib....Is it a stock part from GM, what

the part # is, etc.  I'd be interested in adding this to the car

since I hate having gas dribble onto the paint.  Thanks in advance!!!

Dave in So. Cal, stock (so far) '96 Black SS.

------------------------------

Date: Thu, 22 Feb 1996 17:42:00 PST

From: Ray Lopez <impalass@erols.com>

To: impala@east.xsis.xerox.com

Subject: Edelbrock P/Ns

Message-ID: <199602230142.UAA15539@mail.erols.com>

I recieved the 1996 Edelbrock Automotive Performance Catalog today and these

are some items of interest for the LT1/Impala SS:

1-Performer LT1 Intake Manifold

p/n2107 available Spring 1996, no price in consumer price list.

"This unique design has equal legnth runners that are longer than stock for

incredible torque and HP gains.  Prototype testing indicate 17ft/lbs of

torque over stock at 4000 rpm."

2-Edlebrock/BBK twin LT1 Throttle Bodies

p/n3807 52mm no price in consumer price list.

p/n3808 58mm no price in consumer price list.

3-Performer LT1 Cylinder Heads

p/n6095 complete pair $1482.95

p/n6094 bare pare $1090.95

"Performer LT1 cylinder heads added over 20 hp, increased valve sizes (2.02"

intake, 1.60" exhaust). Will work with stock or aftermarket self-guiding

roller rocker arms.

Note: An Edelbrock Performer FMP 2000 (Flash memory programmer) is required

when installing on 94-95 vehicles. Contact Edlebrock or check instruction

sheet for more information." ((Looks like we'll have another programmer

avail soon)).

4-Performer-Plus LT1 Camshaft

p/n2108 available May 1996 $287.95

"Use only stock ratio rocker arms.  Do not use dual springs."

same note as for heads apply.

5-TES Headers

p/n6603 no price in consumer price list.

no available date mentioned.

I ordered their catalog by contacting them at:

http://www.edelbrock.com

Please note that I quoted the above out of their catalog...

RAY L.

9 SS 6

------------------------------

Date: Thu, 22 Feb 1996 17:01:06 PST

From: jeffd@accessus.net (Jeff Dawson)

To: impala@east.xsis.xerox.com

Subject: leak down test

Message-ID: <199602230101.TAA00508@mtvernon1.accessus.net>

In response to Kerry's inconsistant leak down test it looks like there is

considerble carbon build up. As to a cause, that is a good question. The

condition of the plugs is where to find the answer though. If this were my

SS I would check the #6 & #8 plug wires first. The injectors for these

cylinders might have a problem also. The problem might be fixed with the

X-66 cleaner I read about a few issues ago without pulling the head. That

would be a quick and dirty fix for what might be the problem.

Jeff Dawson 95 DCM SS no mods yet

------------------------------

Date: Thu, 22 Feb 1996 17:29:53 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Production date

Message-ID: <960223012953_73145.1566_FHD40-1@CompuServe.COM>

Erich,

>In one of the digests you indicated that your Impala was early '94

production.  I'm trying to figure out just how early (or late) production my

'94 is.  After factoring for the 100001, my car's unit number is 71,815

(built in 4/94 according to the sticker on the driver's door; I'm assuming

that it's reliable). Do you have any suggestions on how to go about figuring

this out?

You can get the actual production date from the parts counter of your

dealer. Ask them to use the Dealer Communication System (DCS), which is a

computer terminal they all have. If they run what is called a Service System

report, which given the VIN, will tell you the Julian date of vehicle

production, date the car was sold, as well as the first and current retail

owners. They can also print a report of every option that is factory

installed on a car using this system.

For example, my SS VIN ends in 152370 (# 52,370 off the line), and the DCS

report shows a Julian build date of 4074 (day 74 of '94), which means my

car was built on March 16th, 1994. Other places you can verify the

production date would be the driver door sticker which only identifies the

month, mine says 03/94. Even better is the Trim build sheet, which is placed

under the passenger seat. Mine indicates that the seat trim was built late

(18:33 hours) on March 14th. Also the LH rear of my frame is stenciled with

a 3/04/94 assembly date. The frames are built by Dana corp. and shipped to

the plant "Just In Time" for assembly. You'll notice that the stencil is

upside down, as the frames are built upside down and then flipped.

One thing to note is that it apparently takes something like 20 hours to

assemble an Impala from start to finish, so if they are working only one

shift (as they are now for example) then it will take two or three work days

to complete the car. The build date given on the DCS report would be when

the car was finished.

Let's see if we can figure out who has the oldest (or lowest serial number)

new generation Impala SS! All of you with '94 models, please check your VIN

and report to the list. I think that '94 Impala SS production actually

started in February of '94, so are there any '94s out there built before

March 16, 1994 or with a VIN of less than 152370?

>Also, in an earlier digest, you mentioned that Crutchfield had Infinity

speakers that are a "drop-in" for the stock?  I'm looking at replacing mine,

Infinity makes good stuff, and the fact that I wouldn't have to cut anything

DEFINITELY has it's attractions.  Do you have any details or can you steer

me in the right direction?

That would best be answered by others on the list, or you can call

Crutchfield and they can set you up. I understand they know exactly what

speakers drop in, and they have wiring adapters so they just plug into your

stock speaker connectors with no cut or spliced wires.

>I'm looking forward to the April meet.  It sounds like it's really shaping

up to be quite an event.  I also read in the March Super Chevy that ten of

the Super Chevy events are going to be broadcast on ESPN and ESPN-2 (see

p.17).  I wonder if the Texas Motorplex event is one of them???

Now that I don't know, but it sure will be a great show! Scott.

------------------------------

Date: Thu, 22 Feb 1996 19:17:42 PST

From: BGordon401@aol.com

To: rcheek@one.net

Cc: impala@east.xsis.xerox.com

Subject: Down-to-the-wire warranty work

Message-ID: <960222221741_151219129@emout10.mail.aol.com>

Rob, you wrote:

>I'm within about 3K of the end of my 36K warranty. Does anyone

have suggestions as to what I should put on my "final wish list"

for warranty coverage? Would it be prudent to have the dealer do

a thorough inspection of the car (possibly for extra $$$) to make

sure everything looks OK? <

For what it's worth, by service manager told me that is you ever had a

problem with the drive-train up to about 50,000 miles, you can still have GM

foot the bill AS LONG AS YOUR PERSISTANT ENOUGH and are willing to make a

couple of calls to the service center.  They did have a 50,000 mile waranty

up until a couple of years ago.  He has done work for people "free" when they

were outside the guildlines of the waranty. Personally, I'm a living example

of it when a front suspension spring broke on my 1991 Caprice at 55,000

miles.  He made a call, and GM covered me for it!!  The General can be swayed

with the right kind of pressure.  This is MHO.

..Bruce

------------------------------

Date: Thu, 22 Feb 1996 19:17:33 PST

From: BGordon401@aol.com

To: rcheek@one.net

Cc: impala@east.xsis.xerox.com

Subject: "Popping" Sound

Message-ID: <960222221732_151219111@emout08.mail.aol.com>

Rob, you wrote:

>Did we ever come to a conclusion as to what causes the popping

>and banging noises from the front end during hard turns? I'd like

>to add the cause to the FAQ, if we have a definite answer (or maybe

>just several speculations).

>On a related note, my car just seems to hate to turn at max lock. It

>shivers and bucks considerably. Is this just a fat-tire thing, or

>is it something I should be worried about? I just had an alignment,

>and it's always done this (it has gotten no better and no worse).<

Well, for your first question, I'll give you my experience.  Since I've

checked the torque specs on the upper and lower ball joints, the "popping"

noises have stopped. (knock on wood)  The steps I used are as follows:

The torque specs of the ball joint's knuckle nuts were indeed off.  The worst

was the driver's side.  Both the upper and lower nuts were BACKED OFF at the

factory to line up the cotter pins through the knuckle.   My torque wrench

moved about 30-40 degrees in rotation before "clicking"  into the proper

specs. The RH side of the car, however, was pretty close to where it should

have been.  My wrench "clicked" right away, so my concern was that they

weren't over tightened.  I backed the nuts off, then retorqued them.  It

seems that the person on the RH side during construction did things the right

way.  The person on the LH side, however, needed a refresher class on

quality.

PLEASE NOTE...different people have reported findings, so check all four ball

joints with a quality torque wrench.  Grease when finished because there have

been reports of ball joints coming off the factory line with NO grease.

The torque specifications I used was as follows:

For the upper nut, the proper torque spec is 61 ft-lbs and requires a 7/8"

socket.  As per specs, I tightened until my torque wrench "clicked", then

continued the turn until the next notch in the nut came clear for the cotter

pin to go through the knuckle.  I made sure not to exceed 60 degrees of

additional rotation.

For the lower nut, the proper torque spec is 83 ft-lbs and requires a 15/16"

socket.  Just like the upper nut procedure, I tightened until the "click" and

continued to rotate until the cotter pin was cleared, and didn't exceed 60

additional degrees.

I have found most of the problems that have been posted for the Impalas apply

to my car.  I think it would benefit everyone to check their ball joint

torque specs as well.

This was a bit longer than I wanted, but I hope this helps with the "popping"

question.  So far, it has worked for me.

..Bruce

------------------------------

Date: Thu, 22 Feb 1996 19:17:20 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Cc: 73145.1566@compuserve.com

Subject: F-body Elbow

Message-ID: <960222221719_151219149@mail06.mail.aol.com>

Scott, you wrote:

>Don't believe those HP and torque claims for a second! Many of us have these

devices installed, but it is more on the basis of "it probably couldn't

hurt, but probably does not really do much for performance".

A much better gain would be realized by using the larger diameter F-car

intake elbow, and I would not even rate that at anywhere near a 10 HP gain

either! Scott.<

Your REALLY said a mouth full here.  I installed the export f-body elbow last

week, and I can notice the difference.  Well, at least the PCM can for sure.

Under HEAVY pedal driving (with the new 3.42 gears an all) :-), the car

pings a bit.  Obviously the knock sensor is working O.T. to tell the PCM to

run under a difference air/fuel mixture. I'm sure this "pinging" business

will be going away shortly.

.Bruce

------------------------------

Date: Thu, 22 Feb 1996 20:17:59 PST

From: dhflores@iwl.net (Debbie/Henry Flores)

To: impala@east.xsis.xerox.com

Subject: polished wheels

Message-ID: <96Feb22.202126pst."15087(12)"@alpha.xerox.com>

I recently spoke to a shop regarding polished wheels and he indicated that

the upkeep on polished wheels is substantial. Are there any fellow members

who have heard the same? He made it sound like I needed to wax/clean the

wheels every two weeks or they would stain.  Sounded a bit strange.

------------------------------

Date: Thu, 22 Feb 1996 20:22:00 PST

From: dhflores@iwl.net (Debbie/Henry Flores)

To: impala@east.xsis.xerox.com

Subject: Art Car vs. TransGo

Message-ID: <96Feb22.202525pst."15132(12)"@alpha.xerox.com>

I called every person at both shops and they both seem to talk bad about

each other.  I finally called some transmission shops and it appeared that

Transgo was the shift kit of choice.  Consequently, to all you impala

brothers who steered me in the direction of the TransGo to begin with----I

humbly apologize.  You were all correct in your assessments.  I stand before

you in shame.  I am installing the kit this weekend. I will take it out to

the track for some baseline testing and report back.  Thank you everyone for

your information.

------------------------------

Date: Thu, 22 Feb 1996 20:53:08 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Blowby

Message-ID: <960223045307_75511.1220_GHJ105-1@CompuServe.COM>

Scott M, Scott H,

Thanks for your comments. I will bring the car this weekend to redue the tests

again. I sure hope it's not serious. What a pain! I'll keep you posted to my

findings.

Curt P.

------------------------------

Date: Thu, 22 Feb 1996 21:06:11 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: Robert Cheek <rcheek@one.net>

Cc: Impala <impala@east.xsis.xerox.com>,

Subject: Re:Popping/Banging Noises While Turning

Message-ID: <199602230506.VAA12683@finland.it.earthlink.net>

Rob:

>Did we ever come to a conclusion as to what causes the popping and banging

noises from the front end during hard turns?

Scott Hollenbeck and I have been working on the problem.

My BEAST: Lower ball joints out of spec...no gease. Gonna check the steering

linkage and stabilizer too.

His BEAST: Upper ball joints dry...no gease.

Both beast seem to have been cured!!

>On a related note, my car just seems to hate to turn at max lock. It

shivers and bucks considerably. Is this just a fat-tire thing, or is it

something I should be worried about? I just had an alignment, and it's

always done this (it has gotten no better and no worse).

My BEAST: LH lower ball joint was under spec...wheel would actually move.

Would shake and rattle during hard turns...front-end alignment was

worthless. Shake and rattle gone, waiting to order my Bilstein 9C1 shocks

before I do another alignment. Forget Mr. GM, he never found the problem.

Will take the thing to a frond-end specialist this time.

Hope this helps!!

... Lennie

------------------------------

Date: Fri, 23 Feb 1996 02:53:56 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: help again - swaybars

Message-ID: <960223055355_330645370@emout05.mail.aol.com>

hi,

Tax money is in and I zapped the file of everyone that was interested in the

Herb Adams Sway Bars.  If your interested in them real soon please email me

right away.  Thanks, Troy.

------------------------------

Date: Fri, 23 Feb 1996 04:08:03 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (Impala1)

Subject: Ramblings

Message-ID: <1996Feb23.065900.1049.102286@rinet.hunton.com>

1.   Monday morning, after installing the cone K&N, I had to go to the

airport for a business trip.  On a stretch of an isolated access road,  on

the way there

I decided to put it to the wood and see if the filter made any

seat-of-the-pants difference.  Where before I was only able to reach 118-120

before having to back off for a curve in the road, this time I hit 136 and

FAST!!   The sound of the engine sucking in through the K&N, moreover,

filter was just AWESOME.  It will be very interesting this spring to see

what the drag strip results are for folks with this mod and exhaust

modifications.

2.   9C1 Deflectors -- a side benefit of this mod: now the car fits on drive

on ramps.

Before the side deflectors made the car sit too low to fit, at least on

mine.

3.   9C1s for sale!!  While riding back to the airport this Wednesday in

Richmond, VA in a '89 Ford cab that obviously was an ex-cop-car, I asked

the driver where he got it.  He told me that a guy in Dinwiddie, VA (sp?)

buys Ford and Chevy cop cars from auctions all over the place, fixes them up

as

needed and sells them.  He paid $2300 for a loaded Ford Crown Vic and

thought the Chevys went for a similar price.  He couldn't remember the name

of the place.  I'm not from VA so I can't fill in the blanks.   Anyone from

VA that knows what place this guy is talking about.  The cabbie said that

the guy had a whole field of cars for sale.  And you can test drive them!

Martin Bailey

mbailey@hunton.com

Filtercharged '95 GG with VSE bars and Ultraflows.  3.42 coming.

------------------------------

Date: Thu, 22 Feb 1996 20:22:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: Drilled discs

Message-ID: <312DCDB4@corvette.xyplex.com>

Cross drilling a brake rotor does NOT improve braking and can NOT give

you a shorter stopping distance UNLESS the stock setup is hampered by

"gassing."  The drilled holes (or newer "dimples") allow any gas released

by the pads to escape instead of forming a barrier which limits friction.

The holes actually reduce the area covered by the pad reducing braking

efficiency a small amount.  As the rotors heat cycle, spider web like

cracks will appear around the holes.  These small cracks typically do not

going to hurt anything unless they grow into larger cracks penetrating

through the rotor.  The new dimple type would prevent this phenomenon.

In some instances there is a strange noise emitted by these rotors when

in use - kind of a whistling noise.

If you ever need to get a set of drilled rotors turned, good luck finding

a shop willing to risk breaking their lathe bits.  You will probably have

to pay big bucks and have them ground instead of turned.

Other than these issues - they do look cool.

Scott Harris

sharris@xyplex.com

------------------------------

End of IMPALA Digest 304

************************

IMPALA Digest 305

Topics covered in this issue include:

1) Re: Fuel Bib Part Number

by thaxton@kscdm6.ksc.nasa.gov

2) Thanks to the LIST

by Loren Peterson <equ_rino@cencom.net>

3) Springs

by John Latrobe <racerx@jagunet.com>

4) Re: popping sounds

by Robert Cheek <rcheek@one.net>

5) "Popping" Noises

by Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

6) Build dates

by Ray Berilla <writes4u@ganet.net>

7) Vericom unit

by "Jeff Gatcha" <foxhound@txdirect.net>

8) Gas bib P/N

by "Jeff Gatcha" <foxhound@txdirect.net>

9)

by Jeff Osmer <josmer@awod.com>

10) Caprice bumper cover

by John.Salmon@bridge.bst.bls.com

11) Digest 303

by Franklin Poole <franklin@intersurf.com>

12) Re: Popping/Banging noises

by "Tim M. Killmeyer" <74144.103@compuserve.com>

13) Re: Lurker finally speaks!

by "Tim M. Killmeyer" <74144.103@compuserve.com>

14) SS...Sewer Pipe Equipped

by Loren Peterson <equ_rino@cencom.net>

15) Fwd: Day at the drag strip

by Impalaclub@aol.com

16) Re: Plant Tour

by Impalaclub@aol.com

----------------------------------------------------------------------

Date: Fri, 23 Feb 1996 06:39:55 PST

From: thaxton@kscdm6.ksc.nasa.gov

To: impala@east.xsis.xerox.com

Subject: Re: Fuel Bib Part Number

Message-ID: <9602231439.AA06905@kscdm6.ksc.nasa.gov>

>Also, to you or anyone else that has the gas bib....Is it a stock

part >from GM, what the part # is, etc.

The fuel bib is indeed a GM part.  Part Number: 10186225

I heard it costs about $12 now.  This was in the last Impala club

newsletter.

Eric

Merritt Island, FL

------------------------------

Date: Fri, 23 Feb 1996 08:07:03 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Thanks to the LIST

Message-ID: <312DE627.5717@ns.cencom.net>

Hi Folks,

Just finished the Felpro gasket/Mobile 1 etc. job.  Piece of cake thanks

to ya'll!  Never would of known a damn thing had it not been for the

list.  Checked the front suspension joints and most were dry.  My car

has just over 4,100 miles on it, and maybe now the rear will last a long

long time.

Texas or bust.

Loren DCM96

cut box/K&N

air foil

Flowmasters

Air silencers disabled...soon to go

p.s. My parts guy wanted me to give him a ride around the block when I

picked up the felpro yesterday.  He was purely astounded at the giddie

up an go.

------------------------------

Date: Fri, 23 Feb 1996 08:30:19 PST

From: John Latrobe <racerx@jagunet.com>

To: "'impala@east.xsis.xerox.com'" <impala@east.xsis.xerox.com>

Subject: Springs

Message-ID: <01BB01EC.42550B80@dlup-t.jaguNET.com>

Has anyone installed aftermarket springs on their Impala?

Did you install them yourself?  The manual makes it sound fairly easy.

Jack up the car, remove the shock and steering knuckle, compress

the spring, remove the lower control arm bolts, remove spring.

I think someone quoted the Eibachs at $280.  Are these just stiffer

or do they also lower the car?

JCW carries Suspension Techniques springs for $70/pair. Is anyone

familiar with this brand?  Half the price sounds twice as good ;-)

BTW, JCW also has 9004 bulbs on sale.

83CD0801A    55/100 watt    7.99

83CD0802R    80/100 watt    8.49

Thanks for any info,

John Latrobe<racerx@jagunet.com>

'96 Impala SS, Black

------------------------------

Date: Fri, 23 Feb 1996 09:56:16 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Re: popping sounds

Message-ID: <199602231756.MAA20097@shell.one.net>

(Massive digest response...)

> Rob, why not tell the dealer about the noise you hear in the front

> end along with any other symptoms that may be troubling you?

I told the dealer(s) about the popping sound. The first dealer lubed the

springs, and I think that's what the second one did, too, but the sound

stayed gone after that. Maybe he lubed the ball joints, which may have

been the real culprit.

> For what it's worth, by service manager told me that is you ever had a

> problem with the drive-train up to about 50,000 miles

<znipz>

Sounds like a lot of this is having a great service manager! I don't think

any of the places that I took my Impala to would go to bat for me like that.

I want to make sure that I get everything that may be out of sorts taken

care of during this last visit under the written warranty.

> My BEAST: LH lower ball joint was under spec...wheel would actually move.

> Would shake and rattle during hard turns...front-end alignment was

> worthless. Shake and rattle gone, waiting to order my Bilstein 9C1 shocks

> before I do another alignment. Forget Mr. GM, he never found the problem.

> Will take the thing to a frond-end specialist this time.

My alignment was just redone with no change. I will definitely have the

the ball joints checked! I need them to check 1) torque settings and 2)

amount of grease on both the upper and lowers, right?

--

Rob Cheek

rcheek@one.net

------------------------------

Date: Fri, 23 Feb 1996 11:55:48 PST

From: Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

To: impala@east.xsis.xerox.com

Subject: "Popping" Noises

Message-ID: <312E1BC4.505C@lfwc.lockheed.com>

Re: digest 304, rcheek@one.net, "popping" noises

FYI, one person tightens all four ball joints with a torque wrench.  The

quality, calibration, and settings of the torque wrenches may come into

question, as might the treatment of those wrenches.  One wrench is used

for uppers, another for lowers.  Also, since the ball joints have a

tapered shaft, are you "knocking" the ball joint loose from the control

arms after you loosen the nuts?  If not, you may not be changing the load

on the ball itself because the tapered shaft had been drawn into the hole

under the higher torque.  If you don't remove the previously torqued load

on the ball, will you have premature wear?  Maybe something to think

about and discuss.  I agree with you that the ball joints may be dry even

though they are supposed to be lubed from the supplier.  There is no

"lube rack" on the assembly line.  Hope this info helps. Later.

Mark

------------------------------

Date: Fri, 23 Feb 1996 10:12:49 PST

From: Ray Berilla <writes4u@ganet.net>

To: Impala list <impala@east.xsis.xerox.com>

Subject: Build dates

Message-ID: <199602231819.NAA05437@s1.GANet.NET>

>>Let's see if we can figure out who has the oldest (or lowest serial number

) new generation Impala SS! All of you with '94 models, please check your

VIN and report to the list. I think that '94 Impala SS production actually

started in February of '94, so are there any '94s out there built before

March 16, 1994 or with a VIN of less than 152370?<<

Haven't checked any of the other info yet but the serial number on my '94

SS ends in 164564.

Ray B.

------------------------------

Date: Fri, 23 Feb 1996 10:46:34 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Cc: fuelinjection@fuelinjection.com

Subject: Vericom unit

Message-ID: <199602231846.MAA01226@legend.txdirect.net>

> In digest 301 Steve Das wrote:

>

> >>> if any of you lads were thinking of

> getting one of theses while the initial deal is on, jump for it, if you like

> cars, you will LOVE this thing.<<<

>

> What exactly is this deal? Are you selling them?

Kurt

> '95 Dark Cherry Impala SS.....no mods........yet......

The Vericom computer was/is being offered by Fuel Injection Specialties

for Approx $430 for a short time.  Retail is around $500. (Steve can

correct me if Im wrong)  I was just up there and  Donald has 2 left in stock

from his original order.

I cant speak for them so I dont know if they will be selling the units

at that price after those 2 are gone or if they  are still offering

that price.  Donald can be reached at (210) 654-0774 or you can

E-mail him at fuelinjection@fuelinjection.com.

Mention Steve's name, mine or the Impala list for the special

pricing  (If it is still available).

I have CC'd Donald  this message so he may post with more accurate info.

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala. REAL Oil press gauge, H-pipe with

Muffler delete, 1-LE elbow w/3.5"chrome pipe and Cone K&N,

Jet Air Foil, RED Bow-Tie, 9C1 deflectors, Upgrade speakers, Gas bib,

Vette Engine Dress, Z-pcm, 3.42 gear, tach inbound....

------------------------------

Date: Fri, 23 Feb 1996 10:57:37 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Gas bib P/N

Message-ID: <199602231857.MAA01626@legend.txdirect.net>

>From digest 304....

>  Also, to you or anyone else that has the gas bib....Is it a stock part from

>GM, what the part # is, etc.  I'd be interested in adding this to the car

>since I hate having gas dribble onto the paint.  Thanks in advance!!!

> Dave in So. Cal, stock (so far) '96 Black SS.

Fuel bib is a GM part.  P/N 10186225  Costs between $9 and $15

depending on how much your parts guy likes you.

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala. REAL Oil press gauge, H-pipe with

Muffler delete, 1-LE elbow w/3.5"chrome pipe and Cone K&N,

Jet Air Foil, RED Bow-Tie, 9C1 deflectors, Upgrade speakers, Gas bib,

Vette Engine Dress, Z-pcm, 3.42 gear, tach inbound....

------------------------------

Date: Fri, 23 Feb 1996 11:21:39 PST

From: Jeff Osmer <josmer@awod.com>

To: impala@east.xsis.xerox.com

Message-ID: <199602231921.OAA07673@sumter.awod.com>

First post.. hope it works  I have a Black 96 with 2500 miles and no mods.

Love the car the only problem is while backing it makes the most horrible

noise.  Has any one else had this problem?   Any help or suggestions will be

appreciated.

Thanks

Jeff

|===================================================================|

|Jeffery D. Osmer		||	*  *			    |

|josmer@compusult.com		||	  >			    |

|Compusult Inc			||	\==/		   	    |

|===================================================================|

|"No free man shall ever be de-barred the use of arms.    	    |

|the strongest reason for the people to retain their right          |

|to keep and bear arms is as a last resort to protect themselves    |

|against tyranny in government. "	Thomas Jefferson	    |

|===================================================================|

------------------------------

Date: Fri, 23 Feb 1996 09:06:55 PST

From: John.Salmon@bridge.bst.bls.com

To: Impala Ss <impala@east.xsis.xerox.com>

Subject: Caprice bumper cover

Message-ID: <H00001820585c456>

>A couple of weeks ago someone posted some info about a company that

made the front spoiler for the Custom Caprice Wagon in the current Jeg's

Catalog. I cant find it in my archives, could you repost it!!!

Thanks

JP Kelly

That would've been me....

from Digest 276:

<snip>

Santini Auto & Paint did the custom (very trick) front bumper, but there

is a major drawback--you have to replace the entire stock front bumper

including the metal reinforcements and all the fed-mandated 5mph bumper

stuff too.  Santini wanted to make very sure I understood that the

bumper wouldn't provide the same safety level anymore. <snip>

JEG's 1-800-345-4545

Bell Tech 209-445-1602

Santini 714-891-8895

Suspension Techniques 909-465-1020

------------------------------

Date: Fri, 23 Feb 1996 12:09:01 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Digest 303

Message-ID: <312E1EDD.6E8@intersurf.com>

anybody else's diget 303 strange?  Mine doesn't even have the headers at

the top!  Only one message in the digest...

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Fri, 23 Feb 1996 17:53:52 PST

From: "Tim M. Killmeyer" <74144.103@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Popping/Banging noises

Message-ID: <960224015351_74144.103_BHR68-2@CompuServe.COM>

Rob,

When I took my Impala infor the noise i took along all the info about the ball

joints and the dealer checked them out. They were fine and in spec according to

them.

It turned out to be that the coil spring on the left front was not seated right.

They lubed it and all has been fine since.

Tim

Black '95 - tinted rear and back quarter windows

Alpine 6 CD changer and amp in trunk.

Boston Accoustic speakers all around

CB, four hole lighter for scanner and cellphone

no mechanical mods 'till I autocross it in stock class

a few more times. (should I sneak in a K&N?)

------------------------------

Date: Fri, 23 Feb 1996 17:53:43 PST

From: "Tim M. Killmeyer" <74144.103@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Lurker finally speaks!

Message-ID: <960224015342_74144.103_BHR68-1@CompuServe.COM>

Hi All,

I've been around for here about a month or so but had no strong urge to add

anything since I had already got a lot of info on Compuserve. Today though, I

felt I just had to write. I took the snow shoes off the Impala and put on the

summer treads. What a difference! I only had the car about two and a half months

before I put on some Firestone Town & Country, H100 rated, 225-70-15s on some

black steel rims. Baby Moons completed the picture. It was nice out on the

highway, headlights on in the left lane - people just moved right out of the

way. But on the local streets  everybody slowed down to 5 mhp under the speed

limit whenever I pulled out behind them. The tires (and the car) were great in

all the snow we got here in Pittsburgh though.

I bought the black '95 last September. I had a choice of a couple of demos at

different dealerships (all driven by the owners of the dealerships, but used as

demos when the need arose). Most had around 3000 miles or so on them. But one

dealer also had one on their used car lot. They had just picked it up at a GM

auction and claim it was driven by some GM exec. It had 5200 miles and was about

$1,500 than what I could get the others for. It also had the rear and both

quarter windows tinted black. The original driver had also installed an Alpine

6-CD changer and amp in the trunk with Boston Accoustic speakers all around. You

can crank it up all the way with nary a whimper. And he also had a hot wire

running up to the drivers visor for a radar detector. (The detector was not

there but I've bought a Uniden since. Haven't passed a cop with radar since I

bought it.)

As anxious as I have been to try some of the mods I've seen bandied about, I've

decided to wait just a little bit longer so that I can autocross it in the stock

class a few more times. Last October I took it to the place where I've

autocrossed my previous vehicles. They put me in the V8 class with Camaros,

Firebirds, Mustangs, and one Lincoln Mark VII.  14 vehicles in all, it was the

largest class at the event. I came in sixth, bested by only a few hundredths of

seconds by the other places. The Lincoln actually won that day (a great driver).

So I figure if the Lincoln can win, with a little more practice I should be able

to do it too. If you guys (and gals?) haven't tried to autocross your Impalas

yet - GO FOR IT!  - it's a blast.

Anyway, after I got the summer treads put on, I wash the car and took a trip

downtown to pick up a few things. It still turns heads. Especially with the

stock wheels and tires back on.

That's all for now, just had to say - COME ON SPRING!  YEAH!

Tim

Black '95 - tinted rear and back quarter windows

Alpine 6 CD changer and amp in trunk.

Boston Accoustic speakers all around

CB radio, four hole lighter adapter for scanner and cellphone

no mechanical mods  ('cept for Fel-Pro gasket) 'till I autocross it in stock

class

a few more times. (should I sneak in a K&N?)

------------------------------

Date: Fri, 23 Feb 1996 19:21:42 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: SS...Sewer Pipe Equipped

Message-ID: <312E8446.7A8D@ns.cencom.net>

Thanks to I dont remember who, I am now sewer pipe equipped.  Painted

them black, except I added a rubber connecting tube from Ace hardware

which replaces the black rubber and clamp at the entrance to mass air.

It is about 6 inches long.  Really does look pretty normal, and with the

cut out airbox/k+n it sounds great.

I temporarily blocked home plate entrance with a painted can bottom till

I can get to Lowe's tomorrow.  I have a chevy emblem which I will put on

top.

Thanks again sewer pipe benefactor.

Loren DCM96

Tex or bust

------------------------------

Date: Fri, 23 Feb 1996 21:57:54 PST

From: Impalaclub@aol.com

To: impala@east.xsis.xerox.com

Subject: Fwd: Day at the drag strip

Message-ID: <960224005753_331509611@mail02.mail.aol.com>

Hey guys, please help this fellow SS owners.  Sounds like he needs a new

service dept.  Troy

---------------------

Forwarded message:

From:	JHNS81A@prodigy.com (MR GREG L SULLIVAN SR)

To:	impalaclub@aol.com

Date: 96-02-23 15:29:47 EST

I recently spent a day at Pomona Drag Strip in Southern California.

My Impala is a Black '96, Chromed wheels, factory cassette player

with 12 CD changer. The car is completely stock.  On my first 1/4

mile run I got an ET of 15.350 seconds at 90.45 MPH.  On my second

run the tires went up in smoke.  I think a previous car had leaked

something on the track.  Most magazines tests I have read put the SS

1/4 mile times at 15.4 seconds.  I am curious what is the best ET

anyone has seen or done with a completely stock SS.

I too am getting the popping from the left front wheel during firm

braking at slow speeds.  Dealer service is horrible!  Every time I

pick the car up after warranty repairs they have added a new ding or

scratch.

My SS has 7500 miles and gets 17 to 19 MPG.  Mobil 1 10w-30 is in the

crankcase, with a FRAM Filter.

Also, the drivers seat rocks back and forth during medium stops and

starts.  Dealer says it's normal.  Has anyone else noticed this?

There is an air hose connected to nothing next to my battery.  Has

anyone else noticed this hose?  The dealer says it's normal.  I don't

trust them at all.  Any feedback would be great.

Thx, Greg   (JHNS81A@Prodigy.com)

------------------------------

Date: Fri, 23 Feb 1996 21:57:48 PST

From: Impalaclub@aol.com

To: jerryp@stl-06lssc.army.mil, impala@east.xsis.xerox.com

Subject: Re: Plant Tour

Message-ID: <960224005746_152187484@emout06.mail.aol.com>

In a message dated 96-02-23 14:44:46 EST, you write:

>Is there a minimum age for participating in the plant tour?  I have a 16

>year old boy (but he is a tall one) who wants to see the plant.  OK, or

>has anyone checked on this.  There may be others who might be interested

>in this.

Kids are fine as long as they are leashed.  I asked the factory directly on

this one after several inquires.  They do many school field trips so they're

used to kids.  Troy

------------------------------

End of IMPALA Digest 305

************************

IMPALA Digest 306

Topics covered in this issue include:

1) Re: flowmaster

by Rock528@aol.com

2) Trade Pricing and New Orders

by Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

3) Re: IMPALA digest 297

by RAZZORFL@aol.com

4) Re: IMPALA digest 296

by RAZZORFL@aol.com

5)

by RAZZORFL@aol.com

6) Re: IMPALA digest 303

by RAZZORFL@aol.com

7) Home plate, etc

by ZTMP50A@prodigy.com ( DAVID NMN)

8) Electronic Heated Windshield

by Loren Peterson <equ_rino@cencom.net>

9) In France, Miss Car

by "David Spitz" <david_spitz@vnet.ibm.com>

10)

by Steve Das <sdas@sat.net>

11) Re: Response to Mark Boggs

by ML3001@aol.com

12) RE: Day at the drag strip

by "Harris, Scott" <sharris@xyplex.com>

13) Carwash in Arlington...

by Matt Platts <mplat@airmail.net>

14) 96 Part # ?

by "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

15) Radar Detectors

by Jim Ganninger <jtgstl@home.stlnet.com>

16) Vette engine dress

by "Jeff Gatcha" <foxhound@txdirect.net>

17) Mysterous Socket?

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

18) local car show

by dhflores@iwl.net (Debbie/Henry Flores)

19) Re: Lurker...

by Fast94SS@aol.com

20) Impala Sales Brouchure availability?

by Loren Peterson <equ_rino@cencom.net>

----------------------------------------------------------------------

Date: Fri, 23 Feb 1996 21:53:16 PST

From: Rock528@aol.com

To: 70004.173@compuserve.com, impala@east.xsis.xerox.com, Mbmarx@aol.com,

Subject: Re: flowmaster

Message-ID: <960224005313_152185157@mail02.mail.aol.com>

In a message dated 96-02-23 21:59:06 EST, you write:

>Is there any news on the flowmaster cat-back setup???

>-Mark

Yes, I spoke with Flowmaster a couple of days ago.  They revised the muffler

that they are going to use in the "American Thunder" cat back system.  It is

a 3 chamber muffler and is about 4 inches thick.  They did change it slightly

since the prototype version  Razzor got back in November.  They improved it

to remove some of the resonance with some of their earlier mufflers.  They

did decide to go stainless possibly after our input a few weeks back.  Thanx

to all that responded.  It uses 2.25" pipe from the cats back.  It should be

out in early March.  They are working on machinery to bend the stainless.  I

should have a set of mufflers on my doorstep in a week or so and will let you

know how they are.  Troy

------------------------------

Date: Fri, 23 Feb 1996 07:22:46 PST

From: Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

To: impala@east.xsis.xerox.com

Subject: Trade Pricing and New Orders

Message-ID: <312DDBC6.436A@lfwc.lockheed.com>

Tried the trade order request on two dealers yesterday.  First one,

Jerry's in Weatherford, TX., sold me the part at trade/fleet without

hesitation.  (Ask for Willis.)  Special ordered two parts from my dealer

(no pre-pay).  Asked for trade price.  Salesman said, "we don't normally

do that."  I didn't respond, and he asked if I had a tax number on file.

I told him I had one, but it was for a different business.  We left it

at that, but I'll press the issue when the parts come in and it's time to

pay.

BTW, my dealer told me yesterday that future sales for ALL Impalas will

be at STICKER!!  I guesss that's what happens when a football player buys

a car dealership.  The dealership is Troy Aikman Auto Mall in Fort Worth.

Big whoopie doo.

My car should finally come in today or tomorrow.  Only a short transport

from Arlington.  Since I ordered it in October, I'm more than ready!!!

Later.

Mark

Black '96 - no mods

I've already got the service manuals and the gas bib.

------------------------------

Date: Fri, 23 Feb 1996 22:37:27 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 297

Message-ID: <960224013727_230023222@mail04.mail.aol.com>

In a message dated 96-02-18 20:08:27 EST, you write:

> I was hoping that someone

>out there with a service manual or CD or something could find me the part

>number of the different (enhanced speed compensated) cassette deck so that

>I can phone the local dealer and see if they can give me a price.

Don't know that this is what you need but here goes.

Option Code ULO $55.00 additional is the SCV Cassette radio

RAZZORFL.

------------------------------

Date: Fri, 23 Feb 1996 22:37:26 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 296

Message-ID: <960224013726_230023216@mail02.mail.aol.com>

In a message dated 96-02-17 20:07:32 EST, you write:

>Just thought you all would appreciate that.

>

>

>

Don't you just LOVE IT!!!!!!!!!!!!!!! Thanks for sharing this with us!

RAZZORFL.

------------------------------

Date: Fri, 23 Feb 1996 22:37:28 PST

From: RAZZORFL@aol.com

To: <impala@east.xsis.xerox.com>

Message-ID: <199602240637.BAA15019@emout05.mail.aol.com>

d 96-02-21 14:23:45 EST, you write:

>checked with my local dealer (they are supposed to have the largest parts

>dept. in Central FL) and they have no clue how to find the gas bib.  Can

>anyone help me out with the part number?

>

>

#10186225

RAZZORFL.

------------------------------

Date: Fri, 23 Feb 1996 22:37:23 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 303

Message-ID: <960224013722_230023237@emout07.mail.aol.com>

In a message dated 96-02-22 18:41:27 EST, you write:

>

>I want to paint my calipers.  Any advice on method/type of paint?

>

>

>I've read Steve and Razzor's PPG suggestions, but am wondering what

>colors are available?  RED?  Red would be pretty cool.....

>

>-Dave

FASTRED  DELTRON #DBU73412 and the same type paint in black is what Steve and

I used on our's. This works, looks great and has had NO BAD results. I

painted the main part red and the fixtures black. Steve and I also painted

our plenums red JUST LIKE THE LT4 with NO BAD results, again

This paint applies easily and cleans upn easily with laquer thinner.

On our black '95 SS it does look pretty cool -----ask the guyes that have

seen them!

RAZZORFL.

------------------------------

Date: Fri, 23 Feb 1996 23:04:34 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: Home plate, etc

Message-ID: <097.01961632.ZTMP50A@prodigy.com>

Hey guys, since I'm new to all this, could someone explain to me what

the "home plate" is?  How much is involved with removing it, what are

the benefits, and what is required to install a K&N filter?  I hear

about "cut" airboxes.  Is this all necessary?  Thanks!

Dave in So Cal (Black '96 SS, tinted windows-so far)

------------------------------

Date: Sat, 24 Feb 1996 05:23:28 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Electronic Heated Windshield

Message-ID: <312F1150.32FC@ns.cencom.net>

Fellow SSers

Are there any experts out there on the heated windshield?  Back when

Taurus was introduced, I read and also heard from a Ford Employee that

the windshield blocked radar signals, thus your radar detector is pretty

much useless unless you have a remote antenna.  Also, the police radar

supposedly bounces up from the windshield instead of back to the police

unit.  This is supposed to make the car close to invisible.  Any

comments?

I have some nice Pioneers in the rear deck which improve the sound buku.

This was not too much of a job, but I am intimidated by the door panels,

but would like to change them out too.  Anyone did it?  Any suggestions?

I have 4 Polk Audios that came out of my '94 Z28.  They are round and I

believe something like 5 1/2", not sure though.  I wonder if they would

fit?

Loren DCM96

Sewer Pipe Equipped ...."SPE" from now on

TEXAS OR BUST

------------------------------

Date: Sat, 24 Feb 1996 05:16:16 PST

From: "David Spitz" <david_spitz@vnet.ibm.com>

To: impala@east.xsis.xerox.com

Subject: In France, Miss Car

Message-ID: <96Feb24.052747pst."14523(9)"@alpha.xerox.com>

This list is a lifeline for me. I'm currently on assignment in Nice,

France (yes, could be worse :), where I'm driving a 0.75 liter (!)

diesel Fiat. I think I could knock this car over if I sneezed too

hard. You could mistake the tires for Cheerios if they weren't black,

they're so small. And I can adjust the passenger side mirror without

removing my seat belt. This is no Impala SS.

When I get back in August, I have two months before I get married.

Which means I have two months to do all the modifications I want

to do (primarily stereo, plus some of the simpler engine mods like

removing home plate, and the red bowtie on the grill). I really like

good stereos, so I'll post the results when I'm done. I plan on only

doing "reversible" modifications so I can always return to a purely

stock Impala.

Anyway, no point to this posting except to say I love this list and

I wish I were around to make it to the plant tour. Long live big

cars, big engines, cheap gas (!), mom, and apple pie!

Regards,

David

------------------------------

Date: Sat, 24 Feb 1996 06:42:27 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: 74144.103@compuserve.com

Message-ID: <199602241441.IAA10235@raven.sat.net>

And he also had a hot wire

>running up to the drivers visor for a radar detector. (The detector was not

>there but I've bought a Uniden since. Haven't passed a cop with radar since I

>bought it.)

Tim, Check out the label on your widshhield.  If you have the heat

reflecting windshield, the wire for the radar detector is simply weighting

down the car... as is the detector itself.  The heat reflecting windshield

is virtually a wall to RF in the microwave frequency range.  I didn't

believe it but I did some testing and the detector is virtually useless

behind that windshield.  In fact, I sent my Passport 5000 off to Remote

Systems in Minnesota and I am proud to say that I am going out to my shop

allmost as we speak to install the world's first, wide band, laser, fully

remote, radar detector in my car.  I will post the results to the group

after the install is done.  Check that windshield and if you have the "good

one", drive carefully when in the vicinity of police protection.

Steve....sdas@sat.net

------------------------------

Date: Sat, 24 Feb 1996 07:06:35 PST

From: ML3001@aol.com

To: impala@east.xsis.xerox.com

Cc: 70004.173@compuserve.com

Subject: Re: Response to Mark Boggs

Message-ID: <960224100634_430513202@emout08.mail.aol.com>

Mark, You wanted more info on my modifications:

>supercharged 94

The supercharger is Carrol-Vortec Kit

Boost: 6 - 12 pound depending on pulley.  I run 8 to stay emissions legal.

>w/ headers

AS&M - The Best.  I cut off .6 tenths on the 1/4 mile with just the headers.

>tpi wedge - what is this?

This is an airfoil that shapes the air flow directly into the TPI unit on our

engine.

>cold air intake - how does it effect supercharger performance?

It adds 30 horsepower to a supercharged engine.

>Reprogrammed computer - who did the programming?

Z Industries has a custom program for Geoff and the Carrol-Vortec Kit.

Good Luck with any future mods!

Mike Lonergan

------------------------------

Date: Fri, 23 Feb 1996 21:20:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Cc: "'JHNS81A@prodigy.com'" <JHNS81A@prodigy.com>

Subject: RE: Day at the drag strip

Message-ID: <312F2DB3@corvette.xyplex.com>

The best time of a "stock with minimal mods" Impala SS I've seen is a

14.64 seconds at 94.5 MPH.  My personal best is 14.71 at 95.3 with

obvious traction problems.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Sat, 24 Feb 1996 08:31:17 PST

From: Matt Platts <mplat@airmail.net>

To: "'Impala'" <impala@east.xsis.xerox.com>

Subject: Carwash in Arlington...

Message-ID: <01BB02A3.5831BCC0@fw04-22.ppp.iadfw.net>

I just wanted to volunteer my driveway for carwash use if anyone in =

interested.  I live abou 7 or 8 miles south of the factory and plan on =

attending the tour with the group (even though I don't have an Impala =

:).  I can handle several cars...  one in the driveway... one in the =

neighbor's driveway...  two in the street (it's a quiet neighborhood)... =

well, you get the picture.  I would hate to see anyone having to go to =

one of these spray-your-car-clean places (not that anyone on this list =

would).  Let me know if you are interrested.  I figure that there are =

enough of us in Arlington that we can get all of the cars washed on =

Friday evening.

-Matt Platts

817-472-6076 (home)

------------------------------

Date: Sat, 24 Feb 1996 03:44:00 PST

From: "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

To: impala <impala@east.xsis.xerox.com>

Subject: 96 Part # ?

Message-ID: <312F4ED0@po5.cp.unisys.com>

I ordered a '96 Imapla SS on Dec 8th 95, for $23,900 + tax and lic., and it

was built this past week.  Actually they built two, the dealer thought the

order was missed and reordered another in the end of December.  So two

identical cars are coming my way! Both fully loaded  black beauties.  Any

way the question is has anyone done the corvette valve cover / injector

cover / 1LEreplacement on a '96? are the part #'s on the web page

applicable?  Especially the magnesium valve covers # 10108605 &6 and the

elbow # 25147210, they seem to be the most spendy items and there were a

couple of other #'s listed.  Since I haven't seen a '96 yet I can't compare

the howto write-ups to a vehicle.  I want to order all the parts before the

car arrives so I get an extra 10% discount using the GM charge card.  I

would also order the 3.42:1 gears if I felt confident that 20 years from now

I could reload the PCM properly for them if needed.  I have 5 other vehicles

three with 3.73:1 ratios and one with 3.23. Two of which are equipped with

700R4 xmissions, that overdrive takes care of anything negative about lower

gears, and they sure make the cars /   van snappier.

I         Don Fischer

'96 SS Built and on its way

'30 Ford 5wdw cpe w/ '67 vette 427 th400 3.73 rr

'64 Belair 2dr sedan 409 700R4 3:23 rr

'67 Camaro SS 350 th350 3.73rr

'86 Beauville van 305 700R4 3.73 rr

An olds wagon 4 kids and a wife who all love cars! (motorcycle and two

'deere tractors)

------------------------------

Date: Sat, 24 Feb 1996 10:10:40 PST

From: Jim Ganninger <jtgstl@home.stlnet.com>

To: impala@east.xsis.xerox.com

Subject: Radar Detectors

Message-ID: <199602241810.MAA00306@home.stlnet.com>

Has anyone hard wired a radar detetor into their beast yet?  I have an old

Maxon that I had hard wired into my old '90 Cutlas by stripping the

cigarette lighter end off and plugging the positive into the radio fuse,

and screwing the negative onto a screw to use as a ground.   This had worked

great for years.  Now I put the Maxon set up into my wife's Achieva and it

works fine,  but I bought a brand new BEL 735IP last week and tried to do

the same trick on the Imp and I am getting no power.    Anyone have any ideas?

Thanks.

Jim Ganninger

'95 Dark Cherry , FelPro gasket

------------------------------

Date: Sat, 24 Feb 1996 10:56:33 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Vette engine dress

Message-ID: <199602241856.MAA15453@legend.txdirect.net>

For those of you who have done the vette valve/injector covers,  has

anyone found a hose that will replace the one that sits on top of the

valve cover on the RH side if the engine?  It looks so much better

without it sitting on top of the cover, plus it interferes with the

oil fill cap.    Seems if it just bent the other way it would be perfect.

Havent been able to find a pre-formed hose that will work and the

regular hose I have seems to kink too much.    Any suggestions?

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala. REAL Oil press gauge, H-pipe with

Muffler delete, 1-LE elbow w/3.5"chrome pipe and Cone K&N,

Jet Air Foil, RED Bow-Tie, 9C1 deflectors, Upgrade speakers, Gas bib,

Vette Engine Dress, Z-pcm, 3.42 gear, tach inbound....

------------------------------

Date: Sat, 24 Feb 1996 13:45:43 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: Mysterous Socket?

Message-ID: <Pine.SOL.3.91.960224134442.8815A-100000@netinfo.ubc.ca>

Hello Everyone:

Today while I was looking underneath the dash I noticed there is

mysterious socket located to the left of the ash tray.  The socket is

there for without any appearent function.  Does anyone have any idea what

is the function of this socket?  And does this socket also exist in

94-95 cars?

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red

------------------------------

Date: Sat, 24 Feb 1996 14:57:23 PST

From: dhflores@iwl.net (Debbie/Henry Flores)

To: impala@east.xsis.xerox.com

Subject: local car show

Message-ID: <96Feb24.150050pst."14478(14)"@alpha.xerox.com>

Steve Das and I recently went to a local Houston area mall parking lot where

many hot rodders park and rope off a section and conduct an impromptu car

show.  Steve and I parked our Black 95 Impalas side by side with the hoods

up and we had a substantial number of people come over and admire both cars.

These vehicles are simply gorgeous the general public thinks so also. From

the seasoned hot rodder to the budding high school hot rodder, they all came

over to grab a look. Boys, I must tell you....having two of them lined up

together looked wonderful.  I can only imagine what Arlington will look like.

Henry in Houston

------------------------------

Date: Sat, 24 Feb 1996 16:25:08 PST

From: Fast94SS@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Lurker...

Message-ID: <960224192507_430811236@emout07.mail.aol.com>

Hi Tim:

Welcome to the Impala list! It's a bit uppity, unless you are an auto techno

nerd, but there are alot of useful tips, etc.  As a transplanted Yankee in

texAssville, I couldn't help notice your snow tire change over (god, do I

miss the winters up north!) information.  I've previously scanned the

archives, and memory tells me that somebody has said that snow tires are not

needed on the Impala SS up North in winter...whats the story on this???  Glad

to see that you acknowledge us "(and girls)", however, I can't imagime my 2

ton Impala in an autocross?

Donna Jenkins (DJ) '94 used SS...

------------------------------

Date: Sat, 24 Feb 1996 17:56:42 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Impala Sales Brouchure availability?

Message-ID: <312FC1DA.70D1@ns.cencom.net>

Hi,

Anyone got a line on where we can purchase or obtain 94-95-96 sales

brochures?  They are very hard to find!

Loren

DCM96 SPE

------------------------------

End of IMPALA Digest 306

************************

IMPALA Digest 307

Topics covered in this issue include:

1) Vericom Computer Results

by Steve Das <sdas@sat.net>

2)

by Steve Das <sdas@sat.net>

3) ALDL Connector

by Steve Das <sdas@sat.net>

4) Remote Systems Radar Detector

by Steve Das <sdas@sat.net>

5) Radio parts number

by glen.novitsky@nashville.com

6) Re: ALDL Connector

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

7) Touch Up Paint

by "Thomas L. Pierce" <TLPierce@msn.com>

8) Re: Unknown Benefactor

by "Thomas L. Pierce" <TLPierce@msn.com>

9) F Y I

by "Thomas L. Pierce" <TLPierce@msn.com>

10) Missing Digests

by "David P. Fenzl" <dfnzl@erols.com>

11) WANTED

by Impalaclub@aol.com

12) Re: early 94 model

by Rock528@aol.com

13) Re:  exhaust hose.

by Rock528@aol.com

14) Re: Springs

by Rock528@aol.com

15) Re: local clubs

by Rock528@aol.com

16) Re: us vs. them

by Rock528@aol.com

17) Re: heat reflective windshield secrets.

by Rock528@aol.com

----------------------------------------------------------------------

Date: Sat, 24 Feb 1996 18:52:56 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: erunnio@infohwy.com, dhflores@iwl.net, foxhound@txdirect.net,

Subject: Vericom Computer Results

Message-ID: <199602250251.UAA22638@raven.sat.net>

Well, as promised, I went out today and made two runs (both ways) and

averaged the data supplied by the Vericom Computer.  This thing is really

amazing with respect to the data it gives.  I was a little disappointed in

the performance but the humidity was literally 100% and I was hauling 15

gallons of gas that weighs about 90 pounds so I guess that isn't too bad.

The typical times at the track will run about .3 seconds better than this

because the Vericom starts counting as soon as it sees .2 G's where as at

the track, you have about 1-1.5 foot roll out which cuts off about .3 second

according to Vericom.  I have seen this verified in some magizines so I

guess it's so.  Anyway, here is the data from a BONE STOCK (engine wise)

Impala with about 5500 miles on the clock.

VERICOM PERFORMANCE RECORD SHEET

Vehicle: 1995 Impala    Date: 02/24/96    Time: 1800 CST  Air Temp: 60F

Relative Humidity: 100%   Fuel gallons: 15   Car Mileage: 5350

TWO RUN AVERAGE

TIME

1/4 Mile  15.84

1K Ft...  13.35

1/8 Mile  10.325

330 Ft... 6.875

60 Ft.... 2.725

30 Ft.... 1.91

15 Ft.... 1.36

0-60..... 7.495

0-30..... 2.855

SPEED

1/4 Mile  89.45

1K Ft...  82.55

1/8 Mile  72.5

330 Ft... 56.85

60 Ft.... 28.85

30 Ft.... 21.25

15 Ft.... 15.7

DISTANCE

To 60 MPH: 383.5 Ft.

To 30 MPH: 66 Ft.

G FORCE

1/4 Mile   .125

1K Ft...   .125

1/8 Mile   .155

330 Ft...  .235

60 Ft..... .41

30 Ft..... .435

15 Ft..... .525

Pk G: .6045 @ 3.1 MPH

Pk G: .6045 @ 1 Feet

PkHP:  53.5 @ 45.6 MPH

***NOTE:***

Rear Wheel HP= Peak HP X Car Weight

Peak HP is specified in HP

Hope this is of interest to you guys.

Steve....sdas@sat.net

------------------------------

Date: Sat, 24 Feb 1996 19:04:18 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: Jim Ganninger <jtgstl@home.stlnet.com>

Message-ID: <199602250302.VAA23301@raven.sat.net>

>Has anyone hard wired a radar detetor into their beast yet?

Yup, Just finished installing a Passport 5000 Remote in my '95.  Got power

from the fuse panel on the driver's side.  There are several blank fuse

spots in there with power to one side of the "hole" and a blank on the

other.  I soldered the wire from the detector to one leg of a fuse and

insert it in one of these holes.  Works great.  Incidentally, if you have

the heat reflecting windshield, you are waisting your time... the detector

won't work.

Steve

------------------------------

Date: Sat, 24 Feb 1996 19:08:58 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

Subject: ALDL Connector

Message-ID: <199602250307.VAA23564@raven.sat.net>

Hello Everyone:

Today while I was looking underneath the dash I noticed there is

mysterious socket located to the left of the ash tray.  The socket is

there for without any appearent function.  Does anyone have any idea what

is the function of this socket?  And does this socket also exist in

94-95 cars?

Mike,

Sounds like you are looking at the ALDL connector.  It is used to get the

diagnostics from your cars computer and to communicate with it if necessary.

There is not anything from the car normally plugged in there.

Steve

------------------------------

Date: Sat, 24 Feb 1996 19:36:10 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: erunnio@infohwy.com, clyde.twine@hok.com, dhflores@iwl.net,

Subject: Remote Systems Radar Detector

Message-ID: <199602250334.VAA24817@raven.sat.net>

Today I tackled the job of installing the Remote Systems, Passport 5000

remote conversion unit.  What they did was take my Passport 5000 and modify

it so that the antenna assembly goes out behind the grill and the display

goes inside the car.  This unit is VERY nicely assembled.  The receiver

electronics with antenna(s) is in a sealed ("O" ring?), very rugged black

plastic box that mounts with a single 5/16 stud.  It took me about 3 hours

to decide on the best place for the box.  I ended up with it on the right

side, right next to the front end crush detector.  I fabricated an aluminum

mount out of 1 inch by 1/4 inch aluminum bar stock.  One end is bolted to

the radiator core support and a 90 degree bend produces the flat surface for

the detector mount.  The unit mounted with the lenses looking right out

through the grill.  The really important one is the laser detector window.

This must be mounted with a clear view of the road ahead.  The other window

is for the X and K band detectors and can be blocked by the plastic grill

with no effect.  In my mounting, both lenses are clear.

The cable from the unit has to go into the car.  This is NO FUN!!  I ran the

cable through the large factory harness protector on the left inner fender

and then to the well known foam plug behind the left fender.  The cable that

has to go through comes with a plug on the end to plug into the display

unit. It is impossible to get this plug through the firewall without making

a BIG hole so I ordered a spare connector and cut the supplied one off and

installed the new one after getting the cable into the car.  This takes a

little know-how so if anyone decides to do this, let me (or Remote Systems)

know and I will give you a little help.  I was pushing a large (~3/8)

plastic tube through the foam plug from inside the car when the foam plug

POPPED OUT!!!  It snaps into the hole from the outside (under the hood) and

let me tell you, it is NO FUN to get it back in once it pops out.  It was

easier to get the cable through once it was out but there has to be a better

way!!!

The display unit is a small unit about 1 1/4 high, 2 1/2 wide and about 5/8

thick.  I cut a slot in the black bezel around the instrument cluster and

fed the connector for the display through.  Then I mounted the display with

the supplied double-sticky foam tape over the slot.  The ribbon cable for

the display is not visible and the unit looks like it belongs there.

Let me tell you, the windshield was blocking ALL the RF as now I get the

"normal" warnings around town.  If anybody is looking for a top quality

remote detector for their car, this is the way to go.  If anybody decides to

do this job, I can save you a LOT of time since I have "plowed the ground".

I will be glad to help.

Steve Das.... sdas@sat.net

------------------------------

Date: Sat, 24 Feb 1996 16:00:40 PST

From: glen.novitsky@nashville.com

To: impala@east.xsis.xerox.com

Subject: Radio parts number

Message-ID: <96Feb24.220237pst."14906(11)"@alpha.xerox.com>

Its been requested:

I> > I was hoping that someone

I> >out there with a service manual or CD or something could find me the part

I> >number of the different (enhanced speed compensated) cassette deck so that

I> >I can phone the local dealer and see if they can give me a price.

The easiest thing to do is go to your dealer, or any GM dealer in your area

and ask them who services their speedo clusters and radios needed for

repair.   Each town usually has at least one shop/dealer.  Here in

Nashville its Rickwood Radio.  If, as I think your refering to, you already

know the dealer, they should have the BEST book on ALL parts for GM cars

from Clusters to "main brains" to radios to CDs, etc for them to pick out

the radio you need....surely if they are a true Delco dealer they will have

this book (which comes complete with pictures!)....the only problem is they

charge full retail AND they must install it ($50) or they wont honor the

warranty...and guess what, their the only one in town who services

them!!!!....thats when I called Atlanta Speedo...the number is in the

archives....start locally though, as they will give you the part and model

number. If the first dealer won't give the service name to you, try

another!!

-Glen

------------------------------

Date: Sat, 24 Feb 1996 22:20:43 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Steve Das <sdas@sat.net>

Cc: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: Re: ALDL Connector

Message-ID: <Pine.SOL.3.91.960224221936.16173A-100000@netinfo.ubc.ca>

>

> Mike,

> Sounds like you are looking at the ALDL connector.  It is used to get the

> diagnostics from your cars computer and to communicate with it if necessary.

> There is not anything from the car normally plugged in there.

>

> Steve

>

Thanks Steve, and Dave for the info.

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red

------------------------------

Date: Sun, 25 Feb 1996 03:15:08 PST

From: "Thomas L. Pierce" <TLPierce@msn.com>

To: impala@east.xsis.xerox.com

Subject: Touch Up Paint

Message-ID: <UPMAIL09.199602251115300687@msn.com>

Hi,

Does anyone know where I can find touch up paint for the grey plastic dash

pieces - my dash has several places where the grey has flaked off to reveal

the black plastic beneath - Thanks in advance.

Tom 94 Stock (yeah, right) No, Honestly it is, I promise...

------------------------------

Date: Sun, 25 Feb 1996 06:35:35 PST

From: "Thomas L. Pierce" <TLPierce@msn.com>

To: impala@east.xsis.xerox.com

Subject: Re: Unknown Benefactor

Message-ID: <UPMAIL09.199602251438530851@msn.com>

>Thanks to I dont remember who, I am now sewer pipe equipped.  Painted them

black, except I added a rubber connecting tube from Ace hardware...

Loren,

Well, um, err, actually Loren, I posted the original message about this 'fix'

after a trip to Home Depot. In fact, I've still got it in the trunk.

I really didn't think it would be taken as gospel.

Let me know how it holds up under the heat...

Tom 94 Stock (yeah, right) No, Honestly it is, I promise...

------------------------------

Date: Sun, 25 Feb 1996 07:26:29 PST

From: "Thomas L. Pierce" <TLPierce@msn.com>

To: impala@east.xsis.xerox.com

Subject: F Y I

Message-ID: <UPMAIL09.199602251526440740@msn.com>

Guys and Gals,

Found this on the Internet in a homepage for a auto buying service:

"If the automobile you want is not found on the screen "Automobiles Available

at or Below Dealer Invoice", then ACS can arrange for the purchase of this

vehicle at a price only slightly above dealer invoice. In fact, the only

automobiles that ACS is unable to acquire at fantastic savings are BMW model

M3s, Range Rovers, Chevrolet Impala SS, and Saturns."

Interesting group, eh?

Tom 94 Stock (yeah, right) No, Honestly it is, I promise...

------------------------------

Date: Sun, 25 Feb 1996 07:48:23 PST

From: "David P. Fenzl" <dfnzl@erols.com>

To: "'Impala Digest'" <impala@east.xsis.xerox.com>

Subject: Missing Digests

Message-ID: <01BB036E.D1025EE0@ppp92.erols.com>

I never received Digests 302 and 303.  Could someone please forward them to me?  I haven't seen them in the archives yet.

Dave F., green-gray '96 built and scheduled for delivery this week.

dfnzl@erols.com

------------------------------

Date: Sun, 25 Feb 1996 08:14:58 PST

From: Impalaclub@aol.com

To: impala@east.xsis.xerox.com

Subject: WANTED

Message-ID: <960225111458_332495598@emout04.mail.aol.com>

I need to correct some addresses for the newsletter mailing list.  If the

following persons are on this email list, please contact me asap.  Thank you,

Troy

Burrow, Bobby

Chick, Greg

Dawson, Jeff

Powers, Richard

Tavarez, Beinvenido

------------------------------

Date: Sun, 25 Feb 1996 10:00:46 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: early 94 model

Message-ID: <960225130046_332560975@emout04.mail.aol.com>

In a message dated 96-02-24 01:01:05 EST, you write:

> I think that '94 Impala SS production actually

>started in February of '94, so are there any '94s out there built before

>March 16, 1994 or with a VIN of less than 152370?<<

How about 1G1BN52P1RR150816.  It belongs to Cliff Foster of Kissimmee, FL

from my registry list.  For all of you on my mailing list please send me your

VIN, Year and Color.  I have a lot of holes in the registry info.  Troy

------------------------------

Date: Sun, 25 Feb 1996 10:01:21 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re:  exhaust hose.

Message-ID: <960225130120_332561044@emout10.mail.aol.com>

In a message dated 96-02-24 21:01:15 EST, you write:

>For those of you who have done the vette valve/injector covers,  has

>anyone found a hose that will replace the one that sits on top of the

>valve cover on the RH side if the engine?

Jeff, Are you speaking of the hose coming off the R exhaust manifold?  GM

changed design of this on the later 95 versions.  The later ones screw onto a

coupling that is inserted into the manifold rather that screwing directly

into the manifold.  Anyway the hose is more molded on the later versions and

sits flatter on the manifold.  I've removed that funky hose covering and

polished the manifold fittings.  Anal, I know.   Troy

------------------------------

Date: Sun, 25 Feb 1996 10:00:45 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Springs

Message-ID: <960225130044_332560951@mail02.mail.aol.com>

In a message dated 96-02-24 01:01:05 EST, you write:

>Has anyone installed aftermarket springs on their Impala?

>Did you install them yourself?

Yes & Yes

>The manual makes it sound fairly easy.

Yes, they are easy.  The toughest part is knocking loose the lower ball

joint.  A five pound sledge persuaded it fairly easy.

>Jack up the car, remove the shock and steering knuckle, compress

>the spring, remove the lower control arm bolts, remove spring.

Yes, no spring compressor was needed.  They came right out.  The lwr. A frame

had a long swing on it.

>

>I think someone quoted the Eibachs at $280.  Are these just stiffer or do

they also lower the car?

Both.  Lowered my car 1.5 inches in the front and about the same in the rear,

maybe slightly less.  We can get the springs for $199.00 a set (4).  Info is

in my newsletter that will be shipped Monday morning.

Troy

------------------------------

Date: Sun, 25 Feb 1996 10:00:55 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: local clubs

Message-ID: <960225130053_332561055@mail04.mail.aol.com>

In a message dated 96-02-24 21:01:15 EST, you write:

>These vehicles are simply gorgeous the general public thinks so also. From

>the seasoned hot rodder to the budding high school hot rodder, they all came

>over to grab a look. Boys, I must tell you....having two of them lined up

>together looked wonderful.  I can only imagine what Arlington will look

like.

Agreed!!  Our show last night only had 20 Impalas lined up.  Last month we

had 32 in a row.  And your right they do attract attention.  I can send you

some club flyers to hand out.  Sounds like your ready to start a regional

Impala group.  Send me some regional info and I will add it to the

newsletter.  I wish more folks would send local information and meeting

places.  When I'm on vacation, I could bring along a copy and figure out

where the Impalas meet.

Forget Disney World, I'M GOING TO ARLINGTON!! <g>

Trying not to be "uppity", Troy

------------------------------

Date: Sun, 25 Feb 1996 10:00:47 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: us vs. them

Message-ID: <960225130047_332560986@emout05.mail.aol.com>

In a message dated 96-02-24 01:01:05 EST, you write:

> But on the local streets  everybody slowed down to 5 mhp under the speed

limit whenever I pulled out behind them.

Know the feeling, try driving a truck with a light on the roof and a star on

the door.  What a royal pain.  Everyone thinks your pacing  THEM.  What a

guilt ridden society we live in.  All I'm trying to is get to the the local

choke and puke, and all "they" think about is not going 2 mph over the speed

limit.

Working traffic is for kids. <G> my 0.02

------------------------------

Date: Sun, 25 Feb 1996 10:01:13 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com, Mbmarx@aol.com, stc@cypress.com,

Subject: Re: heat reflective windshield secrets.

Message-ID: <960225130113_332561022@emout08.mail.aol.com>

>Are there any experts out there on the heated windshield?  Back when

>Taurus was introduced, I read and also heard from a Ford Employee that

>the windshield blocked radar signals, thus your radar detector is pretty

>much useless unless you have a remote antenna.

I didn't put much faith in this one but we have a auto glass company owner in

our Fl group and he verifies that the metalized film in the heat reflective

windshield does affect radar detector performance.

> Also, the police radar supposedly bounces up from the windshield instead of

back to the police unit.  This is supposed to make the car close to

invisible.

This is absolutely true.  In fact, if you ever photograph your car you'll

notice it doesn't even show up in the pictures.  Troy

ps - you said the "F" word again.. that's a buck in the cuss jar!  Tell him,

Razzor!

------------------------------

End of IMPALA Digest 307

************************

IMPALA Digest 308

Topics covered in this issue include:

1) Manifold hose

by "Jeff Gatcha" <foxhound@txdirect.net>

2) Re: IMPALA digest 307

by dstabb@epix.net (David Stabb)

3) re:  ULO radio option code and price

by ZTMP50A@prodigy.com ( DAVID NMN)

4) IMPALA digest 307-FYI on prices

by ZTMP50A@prodigy.com ( DAVID NMN)

5) VIN Numbers

by Robert Cheek <rcheek@one.net>

6) Want to buy

by Bngwip@aol.com

7) new fax number

by Rock528@aol.com

8) ImpalaFest announcement

by Impala SS <impala@crl.com>

9) Washington D.C. Area Owners

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

10) Going to Arlington from the East.

by les@makro.com (Les Kolodziej)

11) Houston area group

by Clyde Twine <clyde.twine@hok.com>

12)

by Thomas.Drake-1@pp.ksc.nasa.gov

13) Digest 302 and 303

by Franklin Poole <franklin@intersurf.com>

14) valve covers

by Franklin Poole <franklin@intersurf.com>

15) Right side A.I.R. hose

by Franklin Poole <franklin@intersurf.com>

16) Re: Radar/Laser Detector & NMA

by Kurt Kemmling <76165.1654@compuserve.com>

17) '96 Sales Brochure

by Erich Arndt <EARNDT@mednet.swmed.edu>

----------------------------------------------------------------------

Date: Sun, 25 Feb 1996 13:40:22 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Cc: Rock528@aol.com

Subject: Manifold hose

Message-ID: <199602252142.PAA26324@legend.txdirect.net>

In a message dated 96-02-24 21:01:15 EST, you write:

>>For those of you who have done the vette valve/injector covers,  has

>>anyone found a hose that will replace the one that sits on top of the

>>valve cover on the RH side if the engine?

>Jeff, Are you speaking of the hose coming off the R exhaust manifold?  GM

>changed design of this on the later 95 versions.  The later ones screw onto a

>coupling that is inserted into the manifold rather that screwing directly

>into the manifold.  Anyway the hose is more molded on the later versions and

>sits flatter on the manifold.  I've removed that funky hose covering and

>polished the manifold fittings.  Anal, I know.   Troy

Yes, there is a metal piece that comes up off the front of the RH Exhaust

manifold then goes back 1/2 way down the manifold toward the rear.

My hose then makes a  turn (toward the engine) and goes over the

valve cover all the way BACK to the front of the engine, around

the oil fill cap and over to a metal pipe/fitting at the front of

the manifold, just about right where it started..

As it is now, the hose covers 3/4 of the RH Valve cover. It would

look MUCH better if it would just bend the other way and route around

the dipstick over near the AC condenser and then connect.  My car was built in

June 95, Kind of in the middle so it could go either way I guess.

Will the later hose get it off the valve cover, or cover it less?

How about the 96's?

I havent seen one on a lot that I could take a look at.

IF this later hose would work, I dont know how could I get

it.  Most of the parts people in this town can barely find

their way to work let alone a hose in an engine compartment that was

changed midway through the production cycle......yikes...

I also went to a few dealers in the area and all are asking AT LEAST

Sticker to order an Impala.   If there is one on the lot  (and I ve

seen only 1 since October 95!!)   you prob will pay dearly....

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala. REAL Oil press gauge, H-pipe with

Muffler delete, 1-LE elbow w/3.5"chrome pipe and Cone K&N,

Jet Air Foil, RED Bow-Tie, 9C1 deflectors, Upgrade speakers, Gas bib,

Corvette Valve & Injector covers.

Comming soon:  Z-pcm, 3.42 gear, and a Tach

------------------------------

Date: Sun, 25 Feb 1996 15:53:36 PST

From: dstabb@epix.net (David Stabb)

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 307

Message-ID: <v02130501ad56a2f6f702@[199.224.67.87]>

Hello- I just joined the list, so pardon me for answering to the digest-

Having purchased a new 1995 Impala SS a few weeks ago (yes, a leftover, in

dark cherry red that had been over the dealer's usual limit, having been

delivered last July 3 to the dealer for a July 4th "Red Chevy" parade

promotion) I noted

Glen Novitsky's comments about the Cassete deck.

I've got this deck on my car, and it probably won't be there long. I miss

my CD player, and it seems pretty difficult to add a CD to the car while

still using the factory head unit. Unless someone knows how to get a real

schematic of the Delco unit including pinouts with voltages, etc?  The

dealer supplied a "radio diagram" which sure isn't the same thing.

Anyway, I salvaged some of the audio components from my old (1988, 170K

mile) Corsica that I totalled in the first hours that the PA roads were

legally open after the blizzard of 96. So I'm sitting with 2 Nakamichi

subwoofers, a 200 watt Alpine Amp, A 60 Watt Concord Amp, crossovers, etc..

that I'd like to move into the SS. Probably will go for a new head unit,

which means the factory speed

compensated cassette will be history.

The worst thing about the SS sound is that there is just no life or

ambience. Has anybody experimented with a good tweeter or small monitor

speaker placement that would bounce off the windshield?

And Glenn- If you were looking for a factory cassette player, drop me mail,

and we can discuss it. ;-)

>Date: Sat, 24 Feb 1996 16:00:40 PST

>From: glen.novitsky@nashville.com

>To: impala@east.xsis.xerox.com

>Subject: Radio parts number

>Message-ID: <96Feb24.220237pst."14906(11)"@alpha.xerox.com>

>

>Its been requested:

>

>I> > I was hoping that someone

>I> >out there with a service manual or CD or something could find me the part

>I> >number of the different (enhanced speed compensated) cassette deck so that

>I> >I can phone the local dealer and see if they can give me a price.

____|)avid             Good Morning, Good Afternoon, Good Evening.

How difficult it is to tell when

dstabb@epix.net               you're locked in a tiny room with

dstabb@bix.com                     no windows and artificial lights.

------------------------------

Date: Sun, 25 Feb 1996 16:44:09 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: re:  ULO radio option code and price

Message-ID: <097.02074805.ZTMP50A@prodigy.com>

Hi..just check my order book for the radio.  For the speed

compensated volume option with cassette, the code given is correct

(ULO),  and I just wanted to add that invoice on the radio is $47.30.

Prices may tend to be higher if you have the dealer order the part

for you.

Dave in So Cal.  96 Black SS

------------------------------

Date: Sun, 25 Feb 1996 16:59:48 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: IMPALA digest 307-FYI on prices

Message-ID: <097.02076304.ZTMP50A@prodigy.com>

Hey gang..this maybe old news, but since I'm new here, thought I

could help.  Impala SS invoice with all the available options (the

heat reflective W/S, CD, and 1SE pkg, and engine heater) is $23,788.

22, list being $25,712.00   This includes the 3% holdback the dealer

gets and the dealer advertising costs (which is 1% of holdback amount.

)  When I ordered my car, everyone thought production would end very

soon for the plant conversion.  Since this is not the case and

production will run thru Dec '96, Impalas should be bought less than

list.  Anyway, hope this info is useful!

Dave in So Cal (Black '96 SS)

------------------------------

Date: Sun, 25 Feb 1996 19:19:05 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: VIN Numbers

Message-ID: <1.5.4b11.32.19960226031905.0066f84c@mail.one.net>

My 1994 SS has 152025 as the last 6 digits. Anyone earlier than that?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Sun, 25 Feb 1996 19:51:47 PST

From: Bngwip@aol.com

To: impala@east.xsis.xerox.com

Subject: Want to buy

Message-ID: <960225225147_431594358@emout09.mail.aol.com>

I was wondering if anyone has changed out their factory wheels and tires and

would like to sell the stock SS rims and tires. I think they would look cool

on my 71 Chevy PU.

Why does the 1996 SS brochure say the car comes with volt and oil pressure

gauge but they don't?

In a previous posting, I stated I ordered a Black 96 on Feb. 9th. From what I

have read, it looks like it will take alot longer than the dealer indicated

to get my car.

Does anyone know if anyone has installed LT-4 heads on their SS? How about

LT-1 Aluminum heads? Any thoughts on performance gains?

I have been having trouble with my server so I have started using my aol

account for e-mail temporarily. Phoenix Data Systems in Houston has started

making improvments to their system. (yeah... Right!)

Brett

------------------------------

Date: Sun, 25 Feb 1996 21:28:47 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: new fax number

Message-ID: <960226002846_231285650@emout06.mail.aol.com>

John Kelly and all;

I gots me one of those new dedicated facks lines - 904-258-3779.   Troy

------------------------------

Date: Sun, 25 Feb 1996 21:38:08 PST

From: Impala SS <impala@crl.com>

To: Impala SS List <impala@east.xsis.xerox.com>

Subject: ImpalaFest announcement

Message-ID: <Pine.SUN.3.91.960225213638.19489A-100000@crl5.crl.com>

Come see the Arlington ImpalaFest official web announcement!

Submit your RSVP if you'd like!

http://www.crl.com/~impala/impalafest.html

-Dave

impala@crl.com %%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%%%%%%%%%%%%%% The Impala SS Information HEADQUARTERS!

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

"Is that a Lexus or something?"

------------------------------

Date: Mon, 26 Feb 1996 03:53:04 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: Impala SS <impala@east.xsis.xerox.com>

Subject: Washington D.C. Area Owners

Message-ID: <Chameleon.960226065530.hollenbe@mustang.east.xsis.xerox.com>

I'm thinking about formally starting a regional NAISSO club here in the

metropolitan Washington, D.C. area.  I know there are several other folks

on this list in the area, so what do y'all think?  Would there be enough

interest to get together maybe once a month or once every other month?

Drop me a line via e-mail or phone and let's see what we can put together.

==================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street, Lower Lobby, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

==================================================

------------------------------

Date: Mon, 26 Feb 1996 06:00:13 PST

From: les@makro.com (Les Kolodziej)

To: impala@east.xsis.xerox.com

Subject: Going to Arlington from the East.

Message-ID: <199602261400.JAA06557@supernews.login.net>

Is anyone of you driving to Arlington from the East Coast? I live in

Montreal, Canada and I'm thinking of going, but is too much of a trip to do

it alone.  It would probably take obout 25 hrs from here, so we could get a

few cars together and drive down. I'd be willing to meet somewhere on the

way, southern New York comes to mind. BTW, when is this thing on exactly,

April 11, I've looked through previous Impala digests and could not find the

date. E-mail me or call (514)421.4544 if interested.

------------------------------

Date: Mon, 26 Feb 1996 06:45:18 PST

From: Clyde Twine <clyde.twine@hok.com>

To: impala@east.xsis.xerox.com

Subject: Houston area group

Message-ID: <2.2.16.19960226084302.3eb760ba@hou.hok.com>

The Houston area group had what I believe was its second meeting this past

Sunday.  What a blast!  We were only three strong, but you could have sworn

there were three prototype exoctics lined up in Steve's drive based on the

numbers of admirers.  Henry brought fajitas and Steve pulled rank for the

last half-pound.  He kept yelling something about a rule that the host gets

the last choice :)

Anyway, we took Henry's beast (he had recently added the 3.42 gears and the

Hypertech I believe) out for some runs with Steve's new Vericom.  What a

choice piece of equipment!  Ill let Steve explain the details to those who

are unfamiliar with the computer.  Too make a long story short, it is truly

amazing to see how a common interest can bring together three virtual

strangers for a fantastic afternoon of male bonding...the beer had nothing

to do with it :)  Did I mention that Henry and I had to eat vegetarian

fajitas while Steve...

I cant wait until April!

BTW, we talked Steve's wife into taking a photo of the beasts.  Maybe he can

post it to the web site.

ct

------------------------------

Date: Mon, 26 Feb 1996 09:52:00 PST

From: Thomas.Drake-1@pp.ksc.nasa.gov

To: impala@east.xsis.xerox.com

Message-ID: <9602261448.AA02304@attila.ksc.nasa.gov>

> Hi,

> Anyone got a line on where we can purchase or obtain 94-95-96 sales

> brochures?  They are very hard to find!

> Loren

In case your Chevy dealer is too stingy with the promo material (which you

paid for).  You

can get a '96 Impala SS brochure by calling 1-800-222-1020 (very busy, keep

trying).

Ted Drake

Titusville, FL

'96 Black Impala SS (still in Arlington, assembly finishes tomorrow)

------------------------------

Date: Mon, 26 Feb 1996 08:19:48 PST

From: Franklin Poole <franklin@intersurf.com>

To: dfnzl@erols.com

Cc: Impala List Server <impala@east.xsis.xerox.com>

Subject: Digest 302 and 303

Message-ID: <3131DDA4.4212@intersurf.com>

>I never received Digests 302 and 303.

>Could someone please forward them to me?

>I haven't seen them in the archives yet.Dave, use the following command to retrieve any archive you need.

You didn't save the instructions from listproc did you?<g>

mail to : listproc@east.xsis.xerox.com

in the body say ONLY! >>>     get impala digest 303

Gee I hope I told him right!<gg>

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Mon, 26 Feb 1996 08:20:05 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Cc: fischerw@ihub.ea.unisys.com

Subject: valve covers

Message-ID: <3131DDB5.2B83@intersurf.com>

I don't know where it's coming from but the valve covers for the 'vette

are PLASTIC and not magnesium.  Man you don't want a big chunk of

magnesium under your hood if you don't need it!  Ask any pilot about

those magnesium alternators with the explosive bolts....

Get the valve covers, and don't order the gaskets, or grommet, they now

come installed on the covers.  Get the F-car duct w/out the vent nipple

and make the necessary hole yourself.  Email me off-line if you need

specifics.

Valve covers 10108605 and 606

studs  10108674 (x8),

cap    10108694 and

bolts  10108675 (x8)

F ducts 25147210 (about $95 retail)  some dealers have trouble finding

this because I believe it's an export part.

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Mon, 26 Feb 1996 08:19:56 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Right side A.I.R. hose

Message-ID: <3131DDAC.2EF9@intersurf.com>

>For those of you who have done the vette valve/injector covers,  has

>anyone found a hose that will replace the one that sits on top of the

>valve cover on the RH side if the engine?  It looks so much

>better.(edit)

I'm working on a modification for that AIR hose.  It loops around

crazily and then connects to the exhaust manifold.  The '96 Trans Am

has a more direct route, not over the valve cover but won't mate with

our exhaust manifold.  When I get it figured out I'll post the fix!

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Mon, 26 Feb 1996 08:37:02 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Radar/Laser Detector & NMA

Message-ID: <960226163702_76165.1654_GHM107-1@CompuServe.COM>

In digest 306 Steve Das wrote:

>>> I am proud to say that I am going out to my shop

allmost as we speak to install the world's first, wide band, laser, fully

remote, radar detector in my car. <<<

Sorry guy, but I have had one of these installed in my car

since September of 1994, so I guess you can't claim the prize.<BG>

I had one of the designer/engineers at Cincinnatti Microwave make

a custom unit for me. I also had them integrate one of those early

warning construction site monitors into the unit, I can't recall the exact

name for the beacons that most states now use at CS's to warn motorists

to slow down. This unit gives me an unbelievable range. I now have plenty of

time to let off the gas, and cruise by the Highway Patrol at 55 and smile.<S>

BTW, I am a member of the NMA (National Motorists Association). I was

wondering how many of you guys/gals are? This is the organization

that is responsible for repealing the National 55 MPH speed limit.

They are also the reason why I can now use a Radar detector "legally"

here in my home state of CT, where it was illegal to even own a radar detector.

It is also the only organization that fights for motorists rights.

I suggest that it you are at all serious about driving, you should

support this fine group. You can join by calling 1-800-882-2785.

You can also join via the WWW site they have. Set your web browsers to:

http://www.msn.fullfeed.com/nma/

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Mon, 26 Feb 1996 09:20:53 PST

From: Erich Arndt <EARNDT@mednet.swmed.edu>

To: equ_rino@cencom.net, impala@east.xsis.xerox.com

Subject: '96 Sales Brochure

Message-ID: <s1319852.033@MEDNET.SWMED.EDU>

Loren,

Save your money for your mods.  I've got a couple of '96 sales

brochures.  I can either mail you one, or you can get it from me in April.

By the way,  does ANYONE have a brochure for a '94 that they'd be

willing to part with????

Thanks,

Erich Arndt

'94 Impala SS (black, of course)

9C1 deflectors

Intake mods pending...

Dallas, TX

------------------------------

End of IMPALA Digest 308

************************

IMPALA Digest 309

Topics covered in this issue include:

1) Transmission stays in first in "2" position

by rcheek@one.net (Robert Cheek)

2) Chevy-Vettefest

by Scott Mueller <73145.1566@compuserve.com>

3) .

by "TODD M. MCMURRY" <73543.3717@compuserve.com>

4) [Fwd: Re: volt and pressure guage - digest 308]

by Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

5) LT1 decals/emblem

by "Jeff Gatcha" <foxhound@txdirect.net>

6) Revell Impala Model

by Jerry Phillips <jerryp@stl-06lssc.army.mil>

7) Re: IMPALA digest 308

by Erik - Bertrand <bertran5@scully.egr.msu.edu>

8) Re: IMPALA digest 308

by Rock528@aol.com

9) Re: Diagnostic Plug

by Kurt Kemmling <76165.1654@compuserve.com>

10) Houston Area Impala Group  AKA: Clyde Lies!!!

by Steve Das <sdas@sat.net>

11) Worlds first Remote Radar Detector

by Steve Das <sdas@sat.net>

12)

by Steve Das <sdas@sat.net>

13) Re:Washington D.C. Area Owners

by lennie@earthlink.net (Lennie Dolphin)

14) .

by "TODD M. MCMURRY" <73543.3717@compuserve.com>

15) Earliest Impala Yet..

by Greg Zarkis <zarkman@ix.netcom.com>

16) p/n prices 96 SS for sale

by "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

----------------------------------------------------------------------

Date: Mon, 26 Feb 1996 10:21:26 PST

From: rcheek@one.net (Robert Cheek)

To: impala@east.xsis.xerox.com

Subject: Transmission stays in first in "2" position

Message-ID: <199602261821.NAA21185@mail.one.net>

To those with the "Z" PCM,

Have any of you tried using the "2" position of the gear selector?

My car seems to grab first instead of second in this position. I want

to determine if it is the "Z" causing this or if something else is wrong

with my car. If I select "2", either by downshifting while driving at

about 15-20MPH or by starting from a stop, I always get first and it

show no signs of shifting (I went up to at least 3500 RPM at light

throttle).

Thanks for your help,

--

Robert Cheek - Random Corporation, Cincinnati, Ohio

rcheek@one.net

------------------------------

Date: Mon, 26 Feb 1996 10:29:07 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Chevy-Vettefest

Message-ID: <960226182906_73145.1566_FHD49-2@CompuServe.COM>

Martin,

>I looked high and low for you and that big black Impala SS at the Chevy

Vettefest to no avail.  Sure hope nothing serious caused you to miss this

event.  Went to the information desk to check to see what space you had been

assigned, and the guy said "he's got to be here, you're about the third or

fourth person that came asking for him".

Last week was not good for me (or my wife!). On Friday as I was preparing to

leave for the show, my wife doubled over in pain, with what turned out to be

an ectopic pregnancy! This required immediate emergency surgery (she was

bleeding internally), and I was at the hospital most of the day on Friday

and early Saturday. The procedure was completed using a laproscope, which

meant no large incision, and she was able to go home on Saturday. Everything

went extremely well, and she will be fine!

I'm sorry I had to miss the show, as I would have been the ONLY Impala

there! The person I know locally who has a Vortech supercharged Impala was

supposed to be there but he had transmission problems and could not drive

the car. The Morgan's One-Lap Impala was supposed to be there also, but he

could not make it either.

Although my car was not there I did make it to the show at opening on Sunday

with Mike Sundstrom to pass out the Impala club literature. We spent almost

the whole day there. They had a table where club and car show info could be

placed for people to pickup, and we left several hundred copies of various

club information pieces. We talked with people during the show, and

everybody had already seen the literature we had left, so hopefully we get

some new members!

>The weather looks like it's trying to get more favorable and we'll probably

get together at Strat's or whatever.

I'll definitely be cruising at Strat's and Fluky's on Wed., Fri, and

Saturday nights when I can!

>I wanted to ask if there was any truth to what I'm hearing about the heat

reflecting windshield being a deterent to using a radar detector through it?

Yup, the A23 Heat reflective windshield will reflect radar waves, which

means that your detector will not work behind it. If you have the A23

windshield, then you must use a remote mounted radar detector. Scott.

------------------------------

Date: Mon, 26 Feb 1996 11:03:42 PST

From: "TODD M. MCMURRY" <73543.3717@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: .

Message-ID: <960226190342_73543.3717_FHD40-1@CompuServe.COM>

I have a queston about the incorrect rear differential gasket, If the rear end

isn't leaking will it harm anything & how.

also were do aquire the mobile 1 rear end oil and what is the part #.

Also I talked to the people at A & A specialties, they are the people who make

front & rear spoilers & ground affects.

They said they would have a groung affects kit for the 94-95-96 IMPALA SS in

about 90 days .  Could be interesting.

Todd McMurry

96 BLACK SS

Front air deflectors, truck mat,

corvette engine dress, K&N filter&air charger,

gas bib, silicone hoses, police strobes F&R.

hard wired radar detector & police scanner,

exuast & hypertech on the horizon.

------------------------------

Date: Mon, 26 Feb 1996 13:09:57 PST

From: Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

To: impala@east.xsis.xerox.com

Subject: [Fwd: Re: volt and pressure guage - digest 308]

Message-ID: <313221A5.6C42@lfwc.lockheed.com>

X-Mozilla-Status: 0001

Message-ID: <31321C03.53D4@lfwc.lockheed.com>

Date: Mon, 26 Feb 1996 12:45:55 -0800

From: Mark Hawthorne <mahawthorne@lfwc.lockheed.com>

Organization: Training and Development

X-Mailer: Mozilla 2.0 (Win16; I)

MIME-Version: 1.0

To: Bngwip@aol.com

CC: impala@east.xsis.xerox.com

Subject: RE: volt and pressure guage - digest 308

X-URL: http://www.lfwc.lockheed.com:8001/

Content-Type: text/plain; charset=us-ascii

Content-Transfer-Encoding: 7bit

Brett,

Read the brochure carefully.  It probably has a disclaimer

somewhere.  The brochures are printed up long before the cars go into

production.  The 1996 brochure claims that the interior light is supposed

to come on when you use the remote to unlock the doors.  I was told by a

factory rep that that option was only for the Park Avenue?, but the

reference wasn't removed from the brochure.  One guy actually gave his

Impala back because this feature didn't work!  If the wiring is there ,

maybe this option could be activted with a little engineering.  Whadda

you say?  Anyone out there up to the challenge in figuring out what it

takes to get the dome light to come on when you use the remote to unlock

the doors?? (Aside from opening the door)  Later.

Mark

Stock Black '96

Finally take possession today - waited 4 months and 10 days!!!

------------------------------

Date: Mon, 26 Feb 1996 11:09:24 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: LT1 decals/emblem

Message-ID: <199602261908.NAA05250@legend.txdirect.net>

In the March Issue of Motor Trend (Raw Power) there is a picture of

Enkei wheels on a ZR-28 on pg 130. On that car you can see  an emblem

or decal consisting of a Red  Chevy Bow-Tie and an LT1 in silver.

Has anyone seen these decals anywhere, magazine, shop etc?

Thought they might look good on the injector covers or something.

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala. REAL Oil press gauge, H-pipe with

Muffler delete, 1-LE elbow w/3.5"chrome pipe and Cone K&N,

Jet Air Foil, RED Bow-Tie, 9C1 deflectors, Upgrade speakers, Gas bib,

Corvette Valve & Injector covers.

Comming soon:  Z-pcm, 3.42 gear, and a Tach

------------------------------

Date: Mon, 26 Feb 1996 11:28:21 PST

From: Jerry Phillips <jerryp@stl-06lssc.army.mil>

To: impala@east.xsis.xerox.com

Subject: Revell Impala Model

Message-ID: <96Feb26.112821pst."16508(9)"@alpha.xerox.com>

I have not seen anything related to this so far, so it may be of

interest.  The 1996 Revell catalog includes a 1/25 kit of the Impala SS.

It is numbered 85-6399, with a UPC of 0-3144506399-2.  It is 1/25 scale

and is one of the SnapTite series.  This is the catalog description:

"The upscale version of the popular Chevy Caprice, this one has all the

bells and whistles coupled with beautiful new styling touches.  Highly

detailed snap kit with high performance engine.  Skill 2."  The picture

is of an actual car rather than an assembled kit, so it is impossible to

tell what color the plastic is.  There are no release dates in the

catalog.  The hobby store owner checked his monthly distributor's sheets

(what is now available for him to order in for stock) and there is

nothing through February concerning the Impala.  Talk to your hobby

retailer and let them know you want them to order your kits as soon as

they are available.

Jerry Phillips

St Louis

------------------------------

Date: Mon, 26 Feb 1996 11:30:49 PST

From: Erik - Bertrand <bertran5@scully.egr.msu.edu>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 308

Message-ID: <Pine.SOL.3.91.960226142422.28035C-100000@scully.egr.msu.edu>

Hey Guys,

I am new at this, I just joined your "club" this morning.

I am an ex-GM einineer who is on a leave of absence getting his

masters in mechanicel engineering.  Ayone looking for 96's to buy,

there are plenty of them here.  I mean plenty.  "Bud Kouts" (lansing)

has one on the lot, "Merollis Chevrolet" (Detroit) has 10 black last

time I was there, etc.

Has anyone changed their gear ratio over to 3.42's yet?  I

have the 92-94 police ring and pinion just waiting to go in.  Also,

looking for any PCM calibrating software.  The guys out at Milford

were reluctant to change my PCM over from 3.08's to 3.42's.

------------------------------

Date: Mon, 26 Feb 1996 13:12:31 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 308

Message-ID: <960226161230_432080392@emout07.mail.aol.com>

In a message dated 96-02-26 12:47:17 EST, you write:

>The Houston area group had what I believe was its second meeting this past

>Sunday.  What a blast!  We were only three strong, but you could have sworn

>there were three prototype exoctics lined up in Steve's drive

Love to Hear it.......   Hey Razzor, the virus is spreading...<g>

Troy

NAISSO

------------------------------

Date: Mon, 26 Feb 1996 13:23:33 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Diagnostic Plug

Message-ID: <960226212332_76165.1654_GHM110-1@CompuServe.COM>

In digest 306, Mike Yang wrote:

>>>Today while I was looking underneath the dash I noticed there is

mysterious socket located to the left of the ash tray.  The socket is

there for without any appearent function.  Does anyone have any idea what

is the function of this socket?  And does this socket also exist in

94-95 cars?<<<

This is where the dealer plugs his diagnostic computer to interface with the PCM

in your car.

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Mon, 26 Feb 1996 13:36:15 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: anthony@sat.net, Clyde Twine <clyde.twine@hok.com>,

Subject: Houston Area Impala Group  AKA: Clyde Lies!!!

Message-ID: <199602262135.PAA24990@raven.sat.net>

I feel the necessity to respond to the gross overstatements made about the

second meeting of the Houston Area Impala Lovers (HAIL!!)  My comments ar in

caps below.

>The Houston area group had what I believe was its second meeting this past

>Sunday.  What a blast!  We were only three strong, but you could have sworn

>there were three prototype exoctics lined up in Steve's drive based on the

>numbers of admirers.

THIS PART IS TRUE.  THERE WERE CARS STOPPING IN THE STREET TO ADMIRE THREE

VIRTUALLY IDENTICAL CARS.  IT WAS INTERESTING TO NOTE THE LITTLE "FACTORY

BUMPS" HERE AND THERE IN THE FINISH.... EXACTLY THE SAME ON ALL THREE CARS.

THAT SAYS SOMETHING FOR MASS PRODUCTION.

Henry brought fajitas and Steve pulled rank for the

>last half-pound.  He kept yelling something about a rule that the host gets

>the last choice :)

THIS IS A BOLD FACE LIE!  THERE WASN'T ANYWHERE NEAR HALF A POUND LEFT AFTER

CLYDE PASSED THROUGH THE LINE... ABOUT 5 TIMES!!!

>

>Anyway, we took Henry's beast (he had recently added the 3.42 gears and the

>Hypertech I believe) out for some runs with Steve's new Vericom.  What a

>choice piece of equipment!  Ill let Steve explain the details to those who

>are unfamiliar with the computer.

HENRY TOLD ME IF I TOLD ANYONE ABOUT THIS, HE WOULD KILL ME AND NEVER BRING

THE FAJITAS AGAIN SO I AIN'T TALKIN'!!

Too make a long story short, it is truly

>amazing to see how a common interest can bring together three virtual

>strangers for a fantastic afternoon of male bonding...the beer had nothing

>to do with it :)

BEER?  WAS THERE BEER THERE???

Did I mention that Henry and I had to eat vegetarian

>fajitas while Steve...

LIAR, LIAR, PANTS ON FIRE!!!

>

>I cant wait until April!

ME EITHER.  ANY OF YOU GUYS THAT CAN'T FIGURE OUT A WAY TO GET THERE ARE

REALLY GOING TO MISS A PARTY!!!

>

>BTW, we talked Steve's wife into taking a photo of the beasts.  Maybe he can

>post it to the web site.

I WILL SEE IF I CAN GET THE PHOTOS DIGITIZED.  I DON'T KNOW HOW TO DO THAT

BUT AS CREATIVE AS CLYDE IS, I'LL BET HE CAN FIGURE OUT HOW TO DO IT.

STEVE

------------------------------

Date: Mon, 26 Feb 1996 13:44:31 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: Resys@aol.com

Subject: Worlds first Remote Radar Detector

Message-ID: <199602262144.PAA25722@raven.sat.net>

>From: Kurt Kemmling <76165.1654@compuserve.com>

>In digest 306 Steve Das wrote:

>

>>>> I am proud to say that I am going out to my shop

>allmost as we speak to install the world's first, wide band, laser, fully

>remote, radar detector in my car. <<<

>

>

>Sorry guy, but I have had one of these installed in my car

>since September of 1994, so I guess you can't claim the prize.<BG>

>I had one of the designer/engineers at Cincinnatti Microwave make

>a custom unit for me. I also had them integrate one of those early

>warning construction site monitors into the unit, I can't recall the exact

>name for the beacons that most states now use at CS's to warn motorists

>to slow down. This unit gives me an unbelievable range. I now have plenty of

>time to let off the gas, and cruise by the Highway Patrol at 55 and smile.<S>

>Kurt Kemmling <76165.1654@compuserve.com>

Kurt,

Well DARN!!  Steve (the President of Remote Systems) told me I was getting

the first.  I think I'll send it back!!!

Steve

------------------------------

Date: Mon, 26 Feb 1996 13:11:32 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: dstabb@epix.net (David Stabb)

Message-ID: <199602262111.PAA23296@raven.sat.net>

>Glen Novitsky's comments about the Cassete deck.

>

>I've got this deck on my car, and it probably won't be there long. I miss

>my CD player, and it seems pretty difficult to add a CD to the car while

>still using the factory head unit. Unless someone knows how to get a real

>schematic of the Delco unit including pinouts with voltages, etc?  The

>dealer supplied a "radio diagram" which sure isn't the same thing.

WRONGO David,

You have the ONLY head unit that there is a FACTORY 12 disc changer for.  It

goes in the trunk with a harness to the head unit.  You need to talk to some

of the guys that have done it with an upgraded set of speakers and a little

more amplifier.  I have yet to hear any of the Impala guys bad-mouth this

setup.  The Delco CD connects to the head unit you have and ALL the

functions appear on your head unit.  Nothing else to buy.  Look into this a

little more before you rip out your casette deck with speed compensated

volume.  It ain't that bad!  I have talked to several guys that are much

more into audio than I and they recommended keeping the setup I described

and upgrading the speakers and amp.  Just a thought.

Steve...sdas@sat.net

------------------------------

Date: Mon, 26 Feb 1996 14:28:06 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

Cc: Impala <impala@east.xsis.xerox.com>

Subject: Re:Washington D.C. Area Owners

Message-ID: <199602262228.OAA21444@finland.it.earthlink.net>

Scott:

>I know there are several other folks on this list in the area, so what do

y'all think?

I'm IN!!

>Would there be enough interest to get together maybe once a month or once

every other month?

Let's take a vote. I'll vote for once a month.

... Lennie

------------------------------

Date: Mon, 26 Feb 1996 15:39:08 PST

From: "TODD M. MCMURRY" <73543.3717@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: .

Message-ID: <960226233907_73543.3717_FHD77-1@CompuServe.COM>

Does anyone have an agenda for the show in Arllngton, Will there be a cruise

friday night or saturday night?

What time is the tour, do you have to have a reservation for the tour

------------------------------

Date: Mon, 26 Feb 1996 16:34:32 PST

From: Greg Zarkis <zarkman@ix.netcom.com>

To: impala@east.xsis.xerox.com

Subject: Earliest Impala Yet..

Message-ID: <199602270034.QAA11935@ix2.ix.netcom.com>

>March 16, 1994 or with a VIN of less than 152370?<<

Well, how about 146330?? I believe mine was built in early March '94 so I

doubt it was one of the first ones.

BTW, Anyone have any carpet cleaning tips?  I just went through 2 months of

snow, slush and passengers who like to jump into cars with dripping, muddy

feet.  I've already established that the mats(especially the driver's side)

are not worth salvaging.  (Part ##'s please??)  Also, does anyone have

leather care preferences?  In my rear seat I have a scratch from what I can

figure was a pen in someone's back pocket. (Maybe I should stop giving rides

to anyone who asks... hmmmmm) Any ideas on repair/preventative maintenence?

Thanks for stopping and taking a look...

Greg

'94 No Home Plate, Disgusting Interior, and Bald Tires.

Oh yea, Black (of course)

P.S. Troy - The info's on the way......

------------------------------

Date: Mon, 26 Feb 1996 10:41:00 PST

From: "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

To: impala <impala@east.xsis.xerox.com>

Subject: p/n prices 96 SS for sale

Message-ID: <313253C4@po5.cp.unisys.com>

I just purchased some dress up parts, listed below are the list and net

prices that I paid ( each).  The dealer never batted an eye about the price

or ordering them it was just business as usual for him.  If anyone can fill

in the missing p/n I'd appreciate it.

This is the same dealer that ordered two identical Black SSs.  If anyone is

interested Black w/all options windshield block heater and cd you can get it

for the same price I paid $23,962.00 call Southview Chev at 612 451-2211 ask

for Mike Skog, mention my name ( Don Fischer) and I get $50.00.

Part Number    Qty       Description                   list net

Rqd

10108605  1    valve cover Corvette plastic

10108606  1    valve cover Corvette plastic                 108.00    82.08

10108694  1    Cap Assy                      3.22 2.45

10108674  8    valve stud                         2.60 1.98

10108675  8    Bolts                              1.00 0.76

25147210  1    1LE elbow F body (duct asm) no a/c           95.63     72.68

10186225  1    Fuel bib  (shield-F)                    12.15      9.23

10201491  1    oil press sender police                 26.55     20.18

10208794  1    injector cover                     10.35     7.87

10208795  1    injector cover

10229207  1    emission brkt Z28 LT1                   3.35 2.55

10214035  1    pwr brake hose                     7.25  5.51

10108672  4    mtg studs                     1.40 1.06

11514607  1    Bolt for brkt                      0.32 0.24

?    1    1/4  to 5/8  fitting

?    1    1/4  brake booster valve

10031003  4    bulb  door sw floodlight                1.65 1.25

10161510  20   door panel retainer clips 6 per door     20per pkg     0.21

0.16

3053190        1    thermostatic switch      wrong p/n ?

Trunk mat

Engine oil cooler  external police  7P8

power steering oil cooler   police  7L9

front brake air deflector     police

green silicon hoses opt 1T1

------------------------------

End of IMPALA Digest 309

************************

IMPALA Digest 310

Topics covered in this issue include:

1) Buddy-L Impala

by "Jeff Gatcha" <foxhound@txdirect.net>

2) RSVP at web site

by "Jeff Gatcha" <foxhound@txdirect.net>

3) Steering Wheel Replaced

by MK2112@aol.com

4) First Time Contributor

by "Walter H. Wease, Jr." <74020.63@compuserve.com>

5)

by dkempa@compu.net (Dennis Kempa)

6) Interior Lights with Remote

by Robert Cheek <rcheek@one.net>

7) RH injector hoses

by Scott Mueller <73145.1566@compuserve.com>

8) Magnesium valvecovers

by Scott Mueller <73145.1566@compuserve.com>

9) How to Start a Regional Group/NAISSO History

by Rock528@aol.com

10) Re: bow tie/LT1 emblem

by Rock528@aol.com

11) WANTED MEET PHOTOS

by Rock528@aol.com

----------------------------------------------------------------------

Date: Mon, 26 Feb 1996 17:27:18 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Buddy-L Impala

Message-ID: <199602270126.TAA22801@legend.txdirect.net>

A relative just gave me the little 1/24 scale buddy-L  Impala as a

belated birthday gift.  This darn thing is NEAT!!!    Love the

burnout.   Kind sounds like 1/24th of my car.   ; )

Only one problem......Its BLACK?!?!?

KIDDING.....IM JUST KIDDING.......

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala. REAL Oil press gauge, H-pipe with

Muffler delete, 1-LE elbow w/3.5"chrome pipe and Cone K&N,

Jet Air Foil, RED Bow-Tie, 9C1 deflectors, Upgrade speakers, Gas bib,

Corvette Valve & Injector covers.

Comming soon:  Z-pcm, 3.42 gear, and a Tach

------------------------------

Date: Mon, 26 Feb 1996 17:35:50 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: RSVP at web site

Message-ID: <199602270135.TAA23204@legend.txdirect.net>

Hey, is anyone else having problems with the RSVP for the Arlington

trip at the Impala homepage?    When I hit it, my computer just locks

up and does nothing.   Where does the mail go to,  says

impala@crl.com. at the bottom but  Where/who is that? Dave H.?  Troy?

Rob?  Or is it someone else?

tks

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala. REAL Oil press gauge, H-pipe with

Muffler delete, 1-LE elbow w/3.5"chrome pipe and Cone K&N,

Jet Air Foil, RED Bow-Tie, 9C1 deflectors, Upgrade speakers, Gas bib,

Corvette Valve & Injector covers.

Comming soon:  Z-pcm, 3.42 gear, and a Tach

------------------------------

Date: Mon, 26 Feb 1996 18:07:34 PST

From: MK2112@aol.com

To: impala@east.xsis.xerox.com

Subject: Steering Wheel Replaced

Message-ID: <960226210733_333990887@emout08.mail.aol.com>

Just had my steering wheel and module changed.  Before, I would almost have

to punch the horn pad to honk the horn, now just a slight tap does it.  It's

not just a tilt and toot symptom... If anyone has this problem I suggest

heading to your dealer, there is a bulletin on the steering wheel but not a

recall!

You have to ask for it to be done.  It took my dealer an hour to do it.

See yas...

Mike

Black '95

------------------------------

Date: Mon, 26 Feb 1996 18:28:38 PST

From: "Walter H. Wease, Jr." <74020.63@compuserve.com>

To: Impala Club Digest <impala@east.xsis.xerox.com>

Subject: First Time Contributor

Message-ID: <960227022838_74020.63_FHR97-1@CompuServe.COM>

Greetings from a first time contributor reading digests since #219

I bought my '94 Impala in Charleston in June of '94  shortly after the

Darth Vader ads appeared in Popular Science and other Motor Mags.  I had problems

initially with a broken AC, a  broken heater, drivers door switch and motor broke

twice, PS pump leaked, rear end noise, tilt and toot, stuck accelerator,

missing radio escutcheon "Impala SS", poor fit on doors, trunk leaked,

buttons on floor pads broke, upholstery on drivers seat  torn.  Loved the

acceleration.  Toward Oct 95, the electrical system started failing and the

dealer couldn't fix it.  Went to  another dealer (while it was raining) and

traded up to a '96.   The rain masked the dent which I got in the driver's

door when an olds lost a hubcap which hit me.

My '96 has problems also.  I had the wrong gasket on the  differential which I

found out thanks to the Club.  The dealer and GM would not replace a Genuine GM

part with an aftermarket part  (The Felpro).  I paid a local mechanic to do it

for me.  I have also had some experience with mufflers.  I put three chamber

Flow Masters on my '94.  Really a nice tone.  I switched to Hookers on my '96

and they are quieter but still bark when I put my foot in the tank.  I also put

Quad Chrome pipes on the tailpipes for  effect.  My '96 has ill fitting doors

and the trunk was off-centered.  Dealer fixed with little urging.  Also, there

was a popping noise when turning small circles at slow speeds which indicate one

of the identified problems discussed in the digest.  This Sunday I removed my wheels

and retorqued the upper and lower ball joints per the specs published herein.  I

failed to knock loose the bolt prior to retorquing but, with a combination of

making the adjustments and supplying fresh grease to the joints, the noise stopped.

Once again I am indebted to those of you who discovered this problem and the probable

fix.  I was under the front disk brake, parked on my wife's side of the garage

when she entered the driveway and hit her door switch.  I was helpless to keep the

overhead door from hitting my car as I had forgotten to activate the lock.  Luckily,

the door had a rubber strip on it and hit my rear window but didn't scratch anything.

I think that from here on, I'm going to go to the professional garages.

IMHO, I have found the information valuable yet hard to follow.  Its format lets

me see the FAQ's of which I would like to see the answer to immediately but I have to

wait for the everpresent ">" which repeats the question and then proceeds to

answer it.  If there is a better way, I don't know it.

I want to perform several of the mods which have been done but I guess I'l looking

for a how-to synopsis for each one before I tackle them.  Incidentally, has anyone

identified the $0.59 dongle eliminator which "All Chevy:" used to cover the hole

left by removal of home plate?  If there is a digest which gives a step-by-step, for

replacing the homeplate with the Camaro elbow, I would appreciate being pointed to it.

Also a step-by-step for the silicone hose replacement (is '96 same as '94?).

Probably the best way for me to stay up with the digests was through a WordPerfect

Macro which I wrote to shorten the length of the digests from an average of twelve

pages to eight or nine.  I format the text so the wasted spaces, carriage returns,

tabs, and hyphens are removed and the test is formatted in paragraph form which

flows better when printed out.  Also, CompuServe recommends that E-Mail originated

on Compuserve which is intended for Internet be "Send as shown" instead of

"Reformattable"

We would all be surprised to find the amount of space taken up by comments such

as "the thief", "the coffee", and "Blazin' black 96 Impala SS and luvin't it while

floggin' the f*ords".  If I didn't feel the same, I wouldn't subscribe but there

are certainly a lot of 22KB digests loaded with a lot of extraneous info.  Why

should I complain... this one's way too long.

Has anyone tried the Nology Plugs and wires featured in the Jan "All Chevy"?  If

they're no good, I'd appreciate an E-Mail telling me so.  Also, Has anyone opted for

the Keystone Colorado Custom Wheels and Michelin MXX3 18 (or are they 20) inch tires?

I do have one hint which may be old hat to you all as you have been reading the

digest since #1: A single sheet 1/8" acrylic shield plate is available to protect

the licence plate from Trail Blazers.  I used the old type which covered back and

sides of the plate but had to elongate the frame holes so that the door would fold

down low enough to refuel.  I also use DuraShine.  I used Liquid Glass and Zaino

Brothers Show Car Polish on my '94 but DuraShine is a lot more care free and the

rain and dirt seem to sheet off instead of stay with the beads.  Again, let me

know if I'm hurting the finish.

One last request:  As I have a set of '94 manuals which I would be willing to ship

anywhere CONUS for $55.

Also, I think that it would benefit us all if, on a region by region basis, we could

catalog the reputable shops where we can get reliable modificaitons done at a not-too-

steep price.  One place locally here in Charleston where excellent exhaust work can be

done is Muffler Masters in Summerville.  I think that I will have to go to Charlotte

for my 3.42 swap out which I hope includes the chip work too.  I'll have to research

the digests to find out who of you had this done in Charlotte.

Speed through the Low Country,

Buz Wease.

------------------------------

Date: Mon, 26 Feb 1996 18:25:00 PST

From: dkempa@compu.net (Dennis Kempa)

To: impala@east.xsis.xerox.com

Message-ID: <m0trF6x-0005mEC@marki.compu.net>

First time caller...sound the horns!!

My VIN is 151916, 94 (obviously)

Just bought it used after MONTHS or searching for one that I could afford.

I paid 19,500 for it with CD, remote keyless, twilight sentinal etc.  It had

46k on the clock but I bought the GM warranty with high tech option.  Did I

get hosed??  Too late now I guess.  What are the going rates for these

machines.  I jumped at the 19,500 because when you can find them they

average 22-25000 used.  I am in Clarksville TN (40 mi N of Twang Town).

I have seen alot of you have installed the K&N cone filter and have perked

my interest.. What are the benefits, costs, and hassles with installation.

Also, how can I tell if mine has the stealth mode (a.k.a. heated windshield)

Dennis Kempa

94 no known mods)

------------------------------

Date: Mon, 26 Feb 1996 20:39:33 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Interior Lights with Remote

Message-ID: <1.5.4b11.32.19960227043933.006863d4@mail.one.net>

>The 1996 brochure claims that the interior light is supposed

>to come on when you use the remote to unlock the doors.  I was told by a

>factory rep that that option was only for the Park Avenue?, but the

>reference wasn't removed from the brochure.

Not only that, but the owner's manual (in '94 at least) claims that

this works also! I had my car in the Goodwrench Goons looking at my

car for the better part of two days trying to figure out why they

didn't work!

>If the wiring is there ,

>maybe this option could be activted with a little engineering.  Whadda

>you say?  Anyone out there up to the challenge in figuring out what it

>takes to get the dome light to come on when you use the remote to unlock

>the doors?

It's been done by several list members. Check out the Keyless Entry

Modifications in http://w3.one.net/~rcheek/technical/technical.html

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Mon, 26 Feb 1996 21:16:42 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: RH injector hoses

Message-ID: <960227051641_73145.1566_FHD50-5@CompuServe.COM>

Jeff (and Franklin),

The '94 and early '95 cars have the best air injector hose setup. This

version does not loop around and cover the RH valvecover like the later ones

do! It not only looks much better, but also makes removing the valvecover

much easier as well, since that fitting does not even get in the way.

There are several other cosmetic advantages to the '94 engine compartment,

for example the brake lines run from the ABS controller down along the frame

under the steering shaft, and not by the LH exhaust manifold like on the '95

and '96 cars. This makes changing spark plugs (and valvesprings) on the LH

side MUCH easier (just ask Mike S. <g>)! The less obtrusive air injector

hose and brake lines really make the '94 engine compartment look cleaner.

Also, the '94 cars have a much nicer looking oil dipstick that says "Engine

Oil" on it and which also has a rubber dipstick wiper built into the end of

the tube. The '95 and later ones consist of a simple yellow ring with no

dipstick wiper.

If you want to clean up the engine appearance some, you can easily fit the

'94 dipstick and tube w/wiper as well as the nicer looking and less

obtrusive air injector hose and fittings on any '95 and later Impala. The

brake lines would be too difficult to change, and the '95-'96 uses a

different ABS controller anyway. Scott.

------------------------------

Date: Mon, 26 Feb 1996 21:16:50 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Magnesium valvecovers

Message-ID: <960227051649_73145.1566_FHD50-6@CompuServe.COM>

Franklin,

>I don't know where it's coming from but the valve covers for the 'vette are

PLASTIC and not magnesium.  Man you don't want a big chunk of magnesium

under your hood if you don't need it!  Ask any pilot about those magnesium

alternators with the explosive bolts....

Actually, the '92 LT1 Corvettes all came with Magnesium valvecovers. In fact

Magnesium valvecovers came on all 'vettes from '87 through '92. In '93 they

switched the LT1 Corvette (but not F-car) engines to plastic valvecovers to

minimize valvetrain noise.

The magnesium valvecovers are actually lighter than the plastic ones, but

unless coated (I would recommend Jet-Hot) or painted they stain very easily.

I checked on the price once, and I think they are actually CHEAPER than the

plastic covers! I would consider getting some and Jet-Hot coating them for

use on the Impala, that way they would look great and never stain. One

problem is that I don't know if they are tall enough to clear the Crane 1.6

roller rockers. I do know that they have internal oil drippers which would

have to be ground off, which would really be fun to do with a high speed die

grinder! "That would be cool, heh heh hehheh" "Fire! Fire! heh hehheh heh"

<g> Scott.

------------------------------

Date: Mon, 26 Feb 1996 22:47:09 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com, stc@cypress.com, RAZZORFL@aol.com,

Subject: How to Start a Regional Group/NAISSO History

Message-ID: <960227014707_334278331@emout05.mail.aol.com>

February 27, 1996

How to Start a Regional Group/NAISSO History

Go get a cup of coffee and come back while I go through the steps in what we

have done so far to get this project off the ground.

I'll start first by stating that today is a milestone in the National

Association of 94-6 Impala Owners Group (N.A.I.S.S.O.).   Membership to our

94-6 Registry broke 300.  Today, I have also mailed out just under 300

newsletters of our Issue Number 4.  You will see some nice changes to the

newsletter this issue.  I'll reserve comment until you get it in the mail.  I

have been refining this newsletter process for the last four months.

Everything experienced with rapid growth has happened.  Printing problems,

computer problems, mailing labels falling off, etc.  Future issues will now

come to you with near perfect photos.  You will start to see the photos in

the Mar/April issue along with some new features.  Please send in some nice

color (glossy finish) photos if you want your ride in the newsletter, along

with a description.  I am interested in anything different you have done.

Whether it be an engine mod, wheel change, or some fancy trim additions.

Send in the unusual stuff.  If you have a bone stock '95 SS in black, we can

all picture what it looks like.<g>

HISTORY

Back in early May 95 I joined this news group of Impala enthusiasts along

with the 3 Scott's, i.e., Mueller, Harris and Hollenbeck.  I kind of stumbled

on it when the most interesting stuff was whether or not the 96's were going

to be built and there was rumor of a floor shifter.  Shortly after that I got

together locally with Razzor, Rocket and about twelve others from the Central

Florida area for our first ever Impala meet.  There were about 14 Impalas

that night.  That was the start of the virus.

Since that night I realized that we had the start of something very

addicting.  Everywhere we have met since then, people have gathered and

pulled in to see our rides.  The group dwindled slightly from the first night

and for the next few months until we realized who the core members of our

local chapter were going to be.  We now call it the "core five".  We would

meet regularly once or twice a month dovetailing on other local car shows.

Putting on your own event is nice to do until someone twists an ankle

walking through the parking lot and decides to sue your little club.  This is

why we continue to meet at already established local car club events.  It

puts the liability on them.

Many of the older street rod owners gave us the cold shoulder at first.  They

did not want to see new iron at the shows.  After a few months they started

to warm up to us and come over to look at the cars and what we have done.

Once they realized that we acted professionally and were into perfecting an

already nice machine, we met common ground.  In the last couple of months,

they look forward to us coming to the shows and when 20 - 30 Impalas show up,

it gets everyone's attention.  Last Saturday, we had an owner of a beautiful

'62 Impala SS 409 want to start hanging out with our Impala group.  My how

the tides change.

GROWTH

Our turning point was the Turkey Run in Daytona back in November 95.  This is

an annual 5,000 show car event and has about 65,000 spectators every

Thanksgiving weekend.  At first, the Daytona Beach Street Rods said no more

grocery getters, simply because they ran out of room in the infield of

Daytona International Speedway.  This show has continually outgrown every

location it has been at for the last 10 or 15 years.  We were able to

convince them that we were a formal club just to get in.  Hence, the birth of

the National Association of 94-6 Impala SS Owners.  Our corral area was about

as far out in right field as you could get.  We showed about 8 Impalas then

and were absolutely mobbed with people for two days.  Impala

owners/enthusiasts came out of the woodwork.  We must have talked to hundreds

of people about the corvette dress, engine mods and exterior treatments.

Since then, registry has boomed.  I must get 6 calls every day from people

all over the country that have a copy of a copy of a newsletter, a flyer

someone stuck in their window or another friend of Razzor!!

Starting a club for fun can be a nice thing.  This usually lasts until you

start spending every waking moment on the phone, writing newsletters & email,

and distributing newsletters to over 300 per issue all over the US, Canada

and Europe.  It is only because I am single with no dependants and work

midnights at saving Bike Week casualties that I can continue this without

getting thrown out of my house.  My social life recently sits in the garage

under cover. <g>

Back in July, I was writing a local one page thing to include the addresses

of the guys in the local club as a point of contact.  This is how the

registry got started on our end.  Starting in August 95, I put out the rear

end gasket notice that Scott Mueller developed.  There was a lot of local

people here that didn't have access to a computer and much of this

information.  Heck, there are some here that just started walking upright.

Many enjoyed the information and the newsletter got its start by being kind

of a technical source.  Lots of part numbers, prices and how-to's.  Many

thanks again to all that have contributed especially Scott Mueller.

LOCAL CHAPTERS

I have been asked many times recently about local chapters.  The ground work

is now in place to start formalizing local groups or chapters.  I will revise

the club application and handout brochure to show a point of local contact in

your area.  When I get it done in a couple of days, I will send it to Dave

Hardy for posting to the http://www.crl.com/~impala site and include it in

the next newsletter issue.  Any paperwork that has the approved NAISSO name

on it, has the written authority to use the trademarked Impala deer and

script logo from General Motors.  It must also have the written verbiage from

Chevrolet on it.  You will see this notice on all of my paperwork.  I have

set up and trademarked our club name, improved the registration record

keeping and have made many contacts within Chevrolet.

The strength in our national group is in the numbers.  This is why I tried to

push for single registry and newsletter source initially.  We are now over

300 members strong in distribution and membership.  I will say here that I

still only have a 52% paid subscription rate. You can do the math on how much

the printing and shipping is on a 350 issue, ten page newsletter, every other

month (Ouch).  I continued to send to the unpaid members this month because

it is starting to become clear of the benefits of a large bodied national

group.   We are beginning to get recognition from many aftermarket parts

vendors who see our group as a possible customer base.  With this recognition

comes volume discounts.  I included a vendor discount sheet in our newsletter

today.  It is relatively new, but we have some significant savings on many of

the aftermarket products we are interested in.

I also included the regional contact list we have talked about before on this

email list.  All it takes is for someone in your area to grab the ball and

run with it.  Start scouting out the local car cruises and just show up.  If

you've done the corvette engine dress, you will immediately attract

attention.  Then all it takes is WORD OF MOUTH plus handing out a club flyer

on every Impala you see.  Razzor hands them car to car on the Interstate.

This is how we end up with 20 to 30 Impalas at our small monthly shows.

Our newest member stopped by on the way home from the dealer with her new 96

with about 9 miles on it.  I can supply you with the local Impala's in your

area from my registry base.  Don't forget to submit any club or local

information you would like to see in the newsletter, i.e., monthly meeting

places, phone numbers, etc.  You will soon have a friend in every city in

this country.  (except maybe for that phord factory place).

If your ever in Daytona, USA please feel free to contact me at 904-253-6638

or my new fax line 904-258-3779.   I hope to see many of you in Texas for the

National Meet. I have busy trying to get some nice door prize giveaways for

the show.  If you can't make it to Texas, please come to our Speedway

Spectacular at the Daytona Beach International Speedway on March 23-4, 1996.

Email me for registry information.  This is a 2,000 entry vehicle show plus

lots of other neat stuff.  You may even get to watch Razzor go into lockup

when he spins those BFG's past the steel belts and makes sparks fly out. <g>.

FUTURE

As the registry and membership improves, I would like to add some nice clear

Lucite club decals, a new membership Impala pin, and more regional meets.

Next month I hope to take MC and Visa for the club merchandise (jackets,

hats, tshirts,,,)  I am trying not to bite off more than my expenses will

allow by jumping into a hard cover newsletter.  You get my drift.  This

project has worked very nicely with slow steady growth.  Any ideas for the

future and newsletter improvements are welcome.

To give you an example of how well our group is received, I will share with

you a note I received today from Chevrolet's C.E.O. Mr. Jim Perkins.

'Dear Troy, .....I surely appreciate your interest in Chevrolet.  We can

never have enough "boosters".  I have sent your card(s) to our Impala Brand

Manager as well as our Communications Director....Thanks for what you are

trying to do with the 1994-1996 Impala Owners Group...

Your friend, Jim'

Very Cool, huh?

Comments or feedback is always appreciated,

Troy C. Willrick

National Association of 94-6 Impala SS Owners (N.A.I.S.S.O.)

1648 Taylor Road, #202

Daytona Beach, FL 32124

email: impalaclub@aol.com

------------------------------

Date: Mon, 26 Feb 1996 23:21:04 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: bow tie/LT1 emblem

Message-ID: <960227022103_334295711@emout04.mail.aol.com>

In a message dated 96-02-26 19:46:24 EST, you write:

> On that car you can see  an emblem

>or decal consisting of a Red  Chevy Bow-Tie and an LT1 in silver.

>Has anyone seen these decals anywhere, magazine, shop etc?

>Thought they might look good on the injector covers or something.

Yes, Ecklers Corvette in Titusville/Melbourne area of FL has them.  Email

Razzorfl@aol.com for their number and price. He has it on his car.  Troy

------------------------------

Date: Mon, 26 Feb 1996 23:21:04 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: WANTED MEET PHOTOS

Message-ID: <960227022104_334295717@emout05.mail.aol.com>

>>BTW, we talked Steve's wife into taking a photo of the beasts.  Maybe he

can

>>post it to the web site.

This is exactly what I need for the NATIONAL NEWSLETTER!!  Send me some in a

glossy finish.  Troy

on another note - I think you left the "r" out of beasts.

------------------------------

End of IMPALA Digest 310

************************

IMPALA Digest 311

Topics covered in this issue include:

1) Arlington Update

by Rock528@aol.com

2) It's Just Not My Day

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

3) RE: volt and pressure guage - digest 308

by Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

4) Radar/Heat Reflective Windshield

by Kelly Pratas <72537.655@compuserve.com>

5) Club dues

by "Jeff Gatcha" <foxhound@txdirect.net>

6) list subjects

by jaking@holli.com (Jeremy King)

7) Re: radar detector

by Kurt Kemmling <76165.1654@compuserve.com>

8) Hybrid Impala?

by "David P. Fenzl" <dfnzl@erols.com>

9) Re: IMPALA digest 306

by "Tim M. Killmeyer" <74144.103@compuserve.com>

10) Re: digest -able?

by "Tim M. Killmeyer" <74144.103@compuserve.com>

11) Re: Tire pressure

by "Tim M. Killmeyer" <74144.103@compuserve.com>

12) Re: Floor mats

by "Tim M. Killmeyer" <74144.103@compuserve.com>

13) Arlington

by bngwip@mail.phoenix.net (Brett A. Rudolph)

14) Parts is Parts

by bngwip@mail.phoenix.net (Brett A. Rudolph)

----------------------------------------------------------------------

Date: Mon, 26 Feb 1996 23:21:05 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Arlington Update

Message-ID: <960227022105_334295729@emout06.mail.aol.com>

>Does anyone have an agenda for the show in Arllngton, Will there be a cruise

>friday night or saturday night?

>What time is the tour, do you have to have a reservation for the tour

Reservation for tour, YES

Friday's factory tour is noon o'clock and lasts for 1.5 hours.  No photos

inside.

Friday afternoon is open or we may do a car detail somewhere.

We are looking for a phone mail system to coordinate message service.  Does

anyone

have access to an 800 number that we can use for a mail box message system??

Tentatively we are looking at some local drags on friday night or a social at

one of the guys homes.  Depending on numbers.

We may get our fill of drags on Sat/Sun at the Motorplex so maybe the social

is better.

Saturday night we will find a cruise or make one!!

This is all subject to change if something better comes along.

Sincerely,

Troy, your cruz director

------------------------------

Date: Tue, 27 Feb 1996 03:42:48 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: Impala SS <impala@east.xsis.xerox.com>

Subject: It's Just Not My Day

Message-ID: <Chameleon.960227064805.hollenbe@mustang.east.xsis.xerox.com>

The day just can't start off any worse...

I'm doing some kitchen remodeling, using my garage to do some of the

painting prep work.  I have several newly painted moulding pieces

lying on the floor between my Impala and my wife's rugrat hauler.

So this morning I start to back out of the garage, making sure I

don't run over any of my newly painted chair rail -- and I wack

the driver's side view mirror on the garage opening as I back through

the door!

Luckily it looks like I just broke the painted covers for the mirror

assembly; hopefully these parts can be purchased pre-painted.  Now

why doesn't some GM engineer design a mirror that folds back in both

directions?

==================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street, Lower Lobby, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

==================================================

------------------------------

Date: Mon, 26 Feb 1996 12:45:55 PST

From: Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

To: Bngwip@aol.com

Cc: impala@east.xsis.xerox.com

Subject: RE: volt and pressure guage - digest 308

Message-ID: <31321C03.53D4@lfwc.lockheed.com>

Brett,

Read the brochure carefully.  It probably has a disclaimer

somewhere.  The brochures are printed up long before the cars go into

production.  The 1996 brochure claims that the interior light is supposed

to come on when you use the remote to unlock the doors.  I was told by a

factory rep that that option was only for the Park Avenue?, but the

reference wasn't removed from the brochure.  One guy actually gave his

Impala back because this feature didn't work!  If the wiring is there ,

maybe this option could be activted with a little engineering.  Whadda

you say?  Anyone out there up to the challenge in figuring out what it

takes to get the dome light to come on when you use the remote to unlock

the doors?? (Aside from opening the door)  Later.

Mark

Stock Black '96

Finally take possession today - waited 4 months and 10 days!!!

------------------------------

Date: Tue, 27 Feb 1996 06:44:13 PST

From: Kelly Pratas <72537.655@compuserve.com>

To: Impala SS Mail <impala@east.xsis.xerox.com>

Subject: Radar/Heat Reflective Windshield

Message-ID: <960227144412_72537.655_EHM102-1@CompuServe.COM>

>From my recent experience, it seems that some radar detectors do work

in the Impala with a heat reflective windshield.  Every morning, on my

way to work, there is a predictable speed trap. My Solo is mounted between

the headliner and the top of the rear-view mirror.  The warning begins about

1/4 to 1/2 a mile ahead of the trap.  I believe the warning time is reduced but

it is still there.  I plan to test this detector in my wife's Blazer to compare

warning times.

Comments?

Kelly

------------------------------

Date: Tue, 27 Feb 1996 09:14:09 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Club dues

Message-ID: <199602271713.LAA24175@legend.txdirect.net>

I'm sure  you have all read Troy's little story about how the club

formed and where it currently stands..   In it,  you all saw that the

collection rate for the club dues is only 52%.  Im also sure that

everyone means to pay their membership dues, but like most things

we put on the back burner, we sometimes never get around to sending

out that check.

I'm asking that all those who haven't gotten around to sending Troy their

dues to please  make a concerted effort to do so.   He has spent a lot of

time and money getting this project off the ground, the least we can do is

send him our measly $10 to pay for the newsletter.  Just think of it

as another mod for the big boy in the garage.

Jeff Gatcha

Foxhound@txdirect.net

1995 GGR

San Antonio Tx

------------------------------

Date: Tue, 27 Feb 1996 09:36:02 PST

From: jaking@holli.com (Jeremy King)

To: impala@east.xsis.xerox.com

Subject: list subjects

Message-ID: <9602271736.AA15458@mistletoe.holli.com>

No offense intended, but I did not subscribe to the Impala line to hear

about peoples' ectopic pregnancies. Maybe that is better information for a

health-care chat line.

Do you know how to send a personal message from AOL ?

jaking

96 Black

------------------------------

Date: Tue, 27 Feb 1996 18:51:48 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: radar detector

Message-ID: <960228025147_76165.1654_GHM177-2@CompuServe.COM>

In digest 309 Steve Das wrote:

>>>Kurt,

Well DARN!!  Steve (the President of Remote Systems) told me I was getting

the first.  I think I'll send it back!!!

Steve<<<

Good idea.<BG>

Kurt

'95 Dark Cherry Impala SS...

------------------------------

Date: Tue, 27 Feb 1996 19:13:16 PST

From: "David P. Fenzl" <dfnzl@erols.com>

To: "'Impala Digest'" <impala@east.xsis.xerox.com>

Subject: Hybrid Impala?

Message-ID: <01BB0560.D0744260@pppc229.erols.com>

I was at a small shopping center yesterday and parked next to a dark

cherry SS.  Naturally I had to scope it out.  I noticed it had bucket seats

and a console, but the shift lever was on the steering column.  Were any

built like that, or did someone make a mod to a '95?  I hung around for a

few minutes looking for the owner/driver to ask about the car, but had to

leave before finding out.

I just got home with my brand-new '96 green-gray beast.  It now has 13

miles on the odometer.  I ordered it on 16 Dec 95, and it arrived at the

dealer last nite.  Preps and pre-delivery inspection were finally complete

at 6pm tonite -- the service dept. first prepped a bunch of cabs that came

in with my Impala.

Dave F.

dfnzl@erols.com

------------------------------

Date: Tue, 27 Feb 1996 20:17:28 PST

From: "Tim M. Killmeyer" <74144.103@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: IMPALA digest 306

Message-ID: <960228041727_74144.103_BHR71-3@CompuServe.COM>

Hi DJ,

Thanks for the welcome.

>> unless you are an auto techno

nerd, but there are a lot of useful tips, etc.<<

Well, I've always considered myself more of a "driver" than a "piston head."

I'm sure some people will get a chuckle when I tell you that I just getting a

handle on the fact that the higher the gear ratios go, the faster the car will

go (with the corresponding drop in gas mileage). I just tell the mechanic that I

need first and or second gear to wind out a little longer and let them make the

necessary changes. But the stuff I learned here is really impressive.

>> memory tells me that somebody has said that snow tires are not

needed on the Impala SS up North in winter...whats the story on this???<<

>From everything I read in the Automobile Forum on Compuserve (before I joined

the list) "the wider the tire the worse in the snow." I was also interested in

keeping those nice looking wheels out of the salt, slush, and grime. It's also

my belief that if you are going to go anywhere like the ski slopes, or travel

back or forth to work real early or real late, in a RWD vehicle, snow tires are

the only way to go.

>>however, I can't imagime my 2

ton Impala in an autocross?<<

Since I started to autocross about 15 years ago, I've always told people that

they should take their car, no matter what kind it is (not 4x4s or vans, etc.),

and autocross it just to see how it handles. You can learn a lot about what a

car will do in tricky situations on the road. You would be very surprised at how

the Impala handles in a four wheel power slide and such. I much prefer autocross

to drag racing (except at a stop light) but that's just me. I don't plan on

attending every event with the SS (unless I'm winning 8-) ), but a couple of

times a year should be fun.

(Question: I've seen a couple of HIGHLY different numbers with regards to the

Impala's front to rear weight ratio. I think the larger rear should give it a

better ratio than Camaros, Firebirds, and the like. Does anybody have a

definitive answer?)

>>god, do I

miss the winters up north!<<

You missed a beaut this year! It does make you appreciate Spring though.

Good drivin' to ya,

Tim

------------------------------

Date: Tue, 27 Feb 1996 20:17:24 PST

From: "Tim M. Killmeyer" <74144.103@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: digest -able?

Message-ID: <960228041724_74144.103_BHR71-2@CompuServe.COM>

Say people,

has anyone figuired out a good way to glean the messages you want to keep out of

a digest and canning the rest? My "in box" is getting way too full and I hate to

save 16 messages so I can keep the one with the trunk mat part number. Know what

I mean?  8-)

Tim

Black '95 - tinted rear and back quarter windows and smoked Ventvisors

Alpine 6 CD changer and amp in trunk.

Boston Accoustic speakers all around

CB radio, four hole lighter adapter for scanner and cellphone

No mechanical mods  ('cept for Fel-Pro gasket) 'till I autocross it in stock

class a few more times. (should I sneak in a K&N?)

------------------------------

Date: Tue, 27 Feb 1996 20:17:16 PST

From: "Tim M. Killmeyer" <74144.103@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Tire pressure

Message-ID: <960228041715_74144.103_BHR71-1@CompuServe.COM>

Do most of the people on the list run the thirty pounds of pressure in their

tires like it says on the door or is there a better way?

Tim

Black '95 - tinted rear and back quarter windows and smoked Ventvisors

Alpine 6 CD changer and amp in trunk.

Boston Accoustic speakers all around

CB radio, four hole lighter adapter for scanner and cellphone

No mechanical mods  ('cept for Fel-Pro gasket) 'till I autocross it in stock

class a few more times. (should I sneak in a K&N?)

------------------------------

Date: Tue, 27 Feb 1996 20:17:36 PST

From: "Tim M. Killmeyer" <74144.103@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Floor mats

Message-ID: <960228041735_74144.103_BHR71-5@CompuServe.COM>

Greg,

I just took my Impala through this sloppy winter using those black floor mats

that are supposed to hold a quart of water or something like that. They have

deep grooves and did catch a ton of water. I've got a few spots to clean up at

one edge, but for the most part, they saved my hide (my carpet hide that is).

Tim

------------------------------

Date: Mon, 26 Feb 1996 09:31:25 PST

From: bngwip@mail.phoenix.net (Brett A. Rudolph)

To: impala@east.xsis.xerox.com

Subject: Arlington

Message-ID: <Chameleon.960226093717.bngwip@dialin.phoenix.net>

I wonder if my SS will make it in time for Super Chevy? Oh well. If not,

I'll drive my ole Pick up. At least it's a Chevy. Ummm....That is if my

new engine is finished by then. Hmmm...

I got an engine from a late model Trans-Am. 350 TPI. Soon to be a 383

TPI. W00H00!  Do you have to drive your SS to go on the factory tour or

will anything do? (except furds)

I'll be there even is I have to rent something. I go just about every

year.

Brett

------------------------------

Date: Tue, 27 Feb 1996 22:54:17 PST

From: bngwip@mail.phoenix.net (Brett A. Rudolph)

To: impala@east.xsis.xerox.com

Subject: Parts is Parts

Message-ID: <Chameleon.960227225636.bngwip@dialin.phoenix.net>

Has anyone compiled a list of Part #'s for the GM parts available like the

ducts, red bowtie, etc... If so, is it posted on the SS home page or

somewhere? Thanks!!!

Brett

------------------------------

End of IMPALA Digest 311

************************

IMPALA Digest 312

Topics covered in this issue include:

1) Re: Correction

by Rock528@aol.com

2) RE:  Radar/Heat Reflective Windshield

by "Scott Blum" <rsblum@directmedia.com>

3)

by Steve Das <sdas@sat.net>

4) Stereo Options

by Thomas.Drake-1@pp.ksc.nasa.gov

5) Reference to "Personal" Messages

by Erich Arndt <EARNDT@mednet.swmed.edu>

6) Broken mirrors

by Mike Sundstrom <102366.175@compuserve.com>

7) list subjects

by Mike Sundstrom <102366.175@compuserve.com>

8) RE: Hybrid Impala?

by "Harris, Scott" <sharris@xyplex.com>

9) personal info

by Scott Mueller <73145.1566@compuserve.com>

10) Radar & Accident

by Mikado95@aol.com

11) TSB update...

by Scott Mueller <73145.1566@compuserve.com>

12) Chevy-vette fest

by roy@astro.ocis.temple.edu (Roy L. White)

13) Measley $10.00

by JGreen5275@aol.com

14) ARRRGGGHH!!

by mbailey@rinet.hunton.com (Bailey, Martin)

15) List Topics

by "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

16) Tire Pressure

by "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

----------------------------------------------------------------------

Date: Wed, 28 Feb 1996 01:21:05 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Correction

Message-ID: <960228042104_233181266@emout06.mail.aol.com>

> time and money getting this project off the ground, the least we can do is

>send him our measly $10 to pay for the newsletter.  Just think of it

>as another mod for the big boy in the garage.

I don't mean to be greedy but I had to raise it to $20.00 annually back in

January.  State and federal trademark fees, bank accts., phone line, monthly

mailbox service, tax license, county occupational license, and a 10 page

newsletter now done on a printing press.

You can get the money back in savings if you buy any of the performance parts

from the vendors on the discount sheet inserted in the newsletter.  BTW, I

got the latest Paddock catalog and they had our satin SS jacket for 70.00.

Ours is still 45.95 for the same jacket.  There's your 20 dollars right

there.  Thanks again for your support, Troy

ps - thanks Jeff for the plug.

------------------------------

Date: Wed, 28 Feb 1996 03:27:37 PST

From: "Scott Blum" <rsblum@directmedia.com>

To: impala@east.xsis.xerox.com

Subject: RE:  Radar/Heat Reflective Windshield

Message-ID: <n1386625923.57240@smtp.directmedia.com>

Kelly you asked,

>From my recent experience, it seems that some radar detectors do work in the

Impala with a heat reflective windshield.  Every morning, on my way to work,

there is a predictable speed trap. My Solo is mounted between the headliner

and the top of the rear-view mirror.  The warning begins about 1/4 to 1/2 a

mile ahead of the trap.  I believe the warning time is reduced but it is still

there.  I plan to test this detector in my wife's Blazer to compare warning

times.

>>Comments?

I mistakenly ordered my '96 with the SunGuard windshield before the thread

about the effects to radar detection began herein.  Although I quickly tried

to change my order, it had already been preferenced--meaning, I was told,

sorry, too late...#%$@!

Running the same detector, a Passport, in both cars (w/ & w/o the SunGuard),

over the same stretches of road, I can easily tell a marked loss in warning

distance.  In fact, such a difference, I'd go so far as to say, the SunGuard

may completely block detection and it only goes off after the 'rays' come

through the side or back windows.  It's that bad.

A word for anyone considering ordering a new SS or you are still in a position

to change your order, do yourself a favor and delete the SunGuard option.  I

now, like many of the others who made this mistake, now face the added expense

of buying a new remote detector, paying the ridiculous cost to have our

present units converted (Steve, Kurt), or replacing the windshield with the

standard issue.  None of which seem particularly appealing to me.

To further make my point, some have even wondered if the SunGuard windshield

actually makes your car more visible to radar.  Although I don't think anyone

has taken a gun to prove or disprove this theory, it sure sounds likely.

Perhaps a good test for one of our police members?

One last note, maybe one of our southern members might address, does the

SunGuard even make that much difference for what it is intended to do?  Being

someone that is VERY anal about cold A/C in the summer, does the SunGuard

really help noticeably?  This is what sold me, obviously before the sacrifices

came to my attention.  With my luck, it doesn't help at all!!!

Whew...Glad I finally got that off my chest (sorry for filling an entire

digest!).  I'm better now...

My $0.02,

-Scott B.

'96 SS, Black, 80w/100w HL's, 9C1 Deflectors, FelPro Diff Gasket, Upgraded

Speakers, ONE Gas Bib...So far.

------------------------------

Date: Wed, 28 Feb 1996 05:45:31 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: Kelly Pratas <72537.655@compuserve.com>

Message-ID: <199602281344.HAA04900@raven.sat.net>

>>From my recent experience, it seems that some radar detectors do work

>in the Impala with a heat reflective windshield.  Every morning, on my

>way to work, there is a predictable speed trap. My Solo is mounted between

>the headliner and the top of the rear-view mirror.  The warning begins about

>1/4 to 1/2 a mile ahead of the trap.  I believe the warning time is reduced but

>it is still there.  I plan to test this detector in my wife's Blazer to compare

>warning times.

>

Kelly,

I found my Passport 5000 to be virtually dead.  After remoting it, the

warnings are back to nornal.  It would OCCASIONALLY beep but I think it was

reflections.  There was no comparison to the unit remoted or behind regular

glass.  You are SURE you have the heat reflecting windsheild aren't you?

SD

------------------------------

Date: Wed, 28 Feb 1996 09:11:00 PST

From: Thomas.Drake-1@pp.ksc.nasa.gov

To: impala@east.xsis.xerox.com

Subject: Stereo Options

Message-ID: <9602281406.AA11131@attila.ksc.nasa.gov>

I am about to take delivery of my new '96 Impala and

have seen many people with upgraded amps, CD

changers, and speakers.  I ordered the CD player option

and I have read that I cannot add the CD changer to it.  I have

a few amps, eq., and speakers from my previous car that I

would like to use.  The factory unit does not sound as full as

my previous system.  Does the factory CD player provide

pre-amp outputs?  If so great.  If not what capability is there

for upgrade with the factory unit?

Ted Drake

'96 Black (on it's way to daddy)

Mods - got a bunch of parts in my garage waiting

------------------------------

Date: Wed, 28 Feb 1996 06:23:09 PST

From: Erich Arndt <EARNDT@mednet.swmed.edu>

To: impala@east.xsis.xerox.com

Subject: Reference to "Personal" Messages

Message-ID: <s13411a0.096@MEDNET.SWMED.EDU>

To all the members of the list, please forgive my propagating this flame.

Jeremey,

Please understand that this list is dedicated to people sharing a

common interest -- the Impala in this case.  That also implies that we care

not only about our cars, but ABOUT EACH OTHER.  Scott M. is a revered

member of this list and is one of the major forces that is responsible for

its existence, not to mention the fact that he has probably forgotten more

about the late model Chevy than most of us could ever hope to know.  He

is dealing with a very difficult situation and has EARNED our respect and

support.

Scott M.,

We (most of us anyway)  hope your wife is doing well.

Thanks for the patience,

Erich Arndt

------------------------------

Date: Wed, 28 Feb 1996 07:26:56 PST

From: Mike Sundstrom <102366.175@compuserve.com>

To: Impala List <impala@east.xsis.xerox.com>

Subject: Broken mirrors

Message-ID: <960228152655_102366.175_EHR101-2@CompuServe.COM>

Scott,

>and I wack

the driver's side view mirror on the garage opening as I back through

the door!  Luckily it looks like I just broke the painted covers for the mirror

assembly; hopefully these parts can be purchased pre-painted.  Now

why doesn't some GM engineer design a mirror that folds back in both

directions?

My driver's side got wacked off at my local mechanic.  My car is dark cherry, so

I knew I was in trouble.  The mirrors are a paint to match item.  If your car is

black, just get a new mirror, it should be a perfect match.  I didn't think I'd

be happy with a paint job, so I replaced both sides with black mirrors.  I

actually like the look better, it gives the car more of a muscle car look.  The

mirrors are around $90 each (not bad if you consider they've got the motor and

heater built in).

Mike

------------------------------

Date: Wed, 28 Feb 1996 07:26:55 PST

From: Mike Sundstrom <102366.175@compuserve.com>

To: Impala List <impala@east.xsis.xerox.com>

Subject: list subjects

Message-ID: <960228152655_102366.175_EHR101-1@CompuServe.COM>

Jaking,

>No offense intended, but I did not subscribe to the Impala line to hear

about peoples' ectopic pregnancies. Maybe that is better information for a

health-care chat line.

Do you know how to send a personal message from AOL ?

No offense intended, but i did not subscribe to the Impala line to have

questions asked about how to use E-mail.  Maybe that is better asked on a

computer chat line.

Mike

------------------------------

Date: Tue, 27 Feb 1996 22:46:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: Hybrid Impala?

Message-ID: <313486F6@corvette.xyplex.com>

>>I noticed it had bucket seats and a console, but the shift lever was on

the steering column.  Were any built like that, or did someone make a mod

to a '95?

Sounds like a stock 95 to me, they were all built like that.  If the

shifter is your concern, only 96's have floor console shifters.  But we

have cup holders.

Scott Harris

sharris@xyplex.com

95 Dark Cherry, Z PCM, muffler delete + H,

ventilated air box w/K&N, Nestea cap - diet of course ;-)

[Not a permanent sig file - just want to keep up with all you yahoos!]

------------------------------

Date: Wed, 28 Feb 1996 08:50:18 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: personal info

Message-ID: <960228165018_73145.1566_FHD46-1@CompuServe.COM>

Dean (and all),

>No offense intended, but the part of Scott's message that had to do with

his wife totaled one paragraph. He was also mentioning it in reference to

why he was not at the Chevy show in Chigago, which many people were planning

to attend to get a look at his car.

Yes, I apologize if the personal info was out of line, but as you indicated

the main reason I posted it was to explain the absence of my car at the

Chevy-Vettefest. I could have just said "medical problems", but that might

have raised additional questions and besides it is just my nature to be

technically more detailed than that! Most people might not have said

anything at all, but I feel that we have all become friends here, much more

so than on other lists I have been on. Scott.

------------------------------

Date: Wed, 28 Feb 1996 09:57:03 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: Radar & Accident

Message-ID: <960228125702_155640128@mail04.mail.aol.com>

>My Solo is mounted between

>the headliner and the top of the rear-view mirror.

We use a Solo in our Impala too, (no heat-reflective windshield).

I've talked to Escort representives as well as Chevy dealers, and they've

said the worst place for a radar detector is where you have yours mounted.

Even if you don't have a heat-reflective windshield, the upper-most part of

the glass is tinted anyway, and causes some loss in performance of the

detector.

Also, last week, my father parked the Imp in a Oak Brook hotel parking lot,

and when he came out, the Imp had a sizable dent between the driver's door

and the front wheel.   He was furious, but what could he do?   So, he just

got in and drove home.    When he got home, the Oak Brook police called to

give him the name of the guy who hit the car--it turned out that the guy who

hit it called the police and told them everything, so that he could pay for

it.    My dad called him and he agreed to pay--the initial assessment is

about $400 to completely restore the fender.   I'm glad there are SOME people

out there with a concious!!

Randy

1995 Green-Gray (still in the shop)

------------------------------

Date: Wed, 28 Feb 1996 10:10:58 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: TSB update...

Message-ID: <960228181057_73145.1566_FHD61-2@CompuServe.COM>

There is a new TSB that will affect us Impala owners. It was just released

and I don't have the TSB number right now, but it officially covers only the

9C1 cars, even though the Impala suspension is essentially the same.

It seems that the cops have been having problems with the swaybar brackets

and how the brackets attach to the rear lower control arm, and how the

swaybar then attaches to the brackets. I can tell you from personal

experience that this is a problem area, especially with the monster 1.5"

Herb Adams rear swaybar installed. I have literally ripped apart two sets of

brackets on my '88 9C1 with the HA bar! The first one I tore up I attributed

to the fact that the original brackets were weakened by rust, as the huge

bar actually pulled the attaching nut right through the steel bracket. The

second time I sheared off the side bolts which attach the bracket to the

control arm.

I have not had any similar problems on my Impala, however that is probably

because I used ARP grade 8+ stainless steel hardware for both the side

bracket bolts as well as the longer bolts to hold the bar to the bracket.

Also, the Impala was new and the bracket was of course not rusted like on my

'88 9C1.

In any case, this new TSB addresses this problem with a completely new rear

lower control arm design (including high durometer bushings) that has

different holes, a different swaybar attaching bracket, and long bolts with

nuts that go completely through the control arm from end to end. The new

arms, brackets and bolts result in a much stronger setup than before. This

would be especially recommended if you are using a larger rear swaybar.

The TSB refers to a kit p/n 10289786 which includes everything to convert

from the old setup to the new one, including 2 rear lower control arms,

brackets, bolts, etc. Your dealer may tell you that this number is no good,

but that is only because it is brand new. Since the TSB only refers to the

9C1 cars, you will have to pay for this one yourself if you want to make the

change, but it is a bargain and only costs about $40 (less than a single

control arm purchased separately!). Scott.

------------------------------

Date: Wed, 28 Feb 1996 06:28:12 PST

From: roy@astro.ocis.temple.edu (Roy L. White)

To: impala@east.xsis.xerox.com

Subject: Chevy-vette fest

Message-ID: <v01540b01ad5a04bfe8e2@[155.247.43.22]>

Scott Mueller: Best wishes for a speedy recovery for your wife! I was at

the Chevy-Vette fest and looked for you. It was a great show, with lots of

chevys (no new stuff) and vettes. Some of the custom work is incredibly

artistic.

>Last week was not good for me (or my wife!). On Friday as I was preparing to

>leave for the show, my wife doubled over in pain, with what turned out to be

>an ectopic pregnancy! This required immediate emergency surgery (she was

>bleeding internally), and I was at the hospital most of the day on Friday

>and early Saturday. The procedure was completed using a laproscope, which

>meant no large incision, and she was able to go home on Saturday. Everything

>went extremely well, and she will be fine!

p.s. jaking@holli.com (Jeremy King) Get a life, you heartless bastard!

Roy L. White <'95 Impala SS; '75 Vette, L-48 4 speed>

------------------------------

Date: Wed, 28 Feb 1996 10:41:09 PST

From: JGreen5275@aol.com

To: impala@east.xsis.xerox.com

Subject: Measley $10.00

Message-ID: <960228134107_233438589@mail02.mail.aol.com>

I have a question about what Jeff Gatcha said.  He said we should at least

send Troy the measley $10.00 for the newsletter.  I paid 20.00 last Saturday

to Troy.  How much is it 10 or 20? If 10 then I'd like a refund if 20 cool

I'm paid up. Another  thing why did Jeff get off for 10 if it is 20?  Also if

it is 20 then why don't Jeff send troy the other measley 10.

------------------------------

Date: Wed, 28 Feb 1996 10:50:12 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (Impala1)

Subject: ARRRGGGHH!!

Message-ID: <1996Feb28.134000.1049.106563@rinet.hunton.com>

After several weeks of trying to locate/fix a popping sound somewhere up

front (tightened ball joints, etc), I took the car to the dealer.  As I

pulled into the service bay, I discovered that my left rear tire had about

10lbs.  I picked up a nail on the way.  The dealer fixed the flat, performed

the "park" recall, gave me a complimentary alignment and is trying to fix

the noise.

What effect, if any, does a plug have on the performance of a tire/its

ability to withstand supersonic speed?  I'm a little reluctant to 'let her

eat till I find out.

Martin Bailey

Wounded '95 GG with K&N Cone, UltraFlows, and Herb Adams WonderBars.

------------------------------

Date: Wed, 28 Feb 1996 06:00:35 PST

From: "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

To: impala <impala@east.xsis.xerox.com>

Subject: List Topics

Message-ID: <9602282025.AA4468@CServe-Hub4.inhouse.compuserve.com>

Jeremy:

>From digest #311:

>> No offense intended, but I did not subscribe to the Impala line to hear

about peoples' ectopic pregnancies <<

I can tell you a little background on the reason Scott Mueller posted this to

the list, and that some *were* interested in it.  Scott had originally planned

to show his car at Chevy/Vette Fest last weekend here in Chicago at McCormick

place, which is a pretty large event of its kind.  I was out of town for the

week, but was sure to get to the fest to see his '94 Impala, which was sure to

win an award.

When I got to the show, I asked at the info booth where he would be located,

and went to that spot.  He wasn't there.  Right then and there, I said to the

folks I was with:  "This isn't good - I hope nothing serious has happened, and

at worst he got caught on an emergency out-of-town consulting assignment or

something."  Unforturnately, it did turn out to be a family emergency for

Scott.  I (and others who attended the show) am glad he let us know what was

up, and that it appears all is well now for him & the family.  Remember, this

is no longer just an Impala chat group -- it's become an extended family for

some of us<g>!

Now the Chicago/Midwest local group (informal at this point) is awaiting for

the weather to behave and for Scott & others to start showing their vehicles at

some of the smaller local meets come springtime.

Jeff

'96 DCM, no mods, 590 mi., in garage 'till I can install that Felpro & synth.

fluid

------------------------------

Date: Wed, 28 Feb 1996 05:36:28 PST

From: "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

To: impala <impala@east.xsis.xerox.com>

Cc: "74144.103" <74144.103@compuserve.com>

Subject: Tire Pressure

Message-ID: <9602282036.AA4886@CServe-Hub4.inhouse.compuserve.com>

Tim:

>> Do most of the people on the list run the thirty pounds of pressure in their

tires like it says on the door or is there a better way? <<

I picked up my '96 DCM last Friday night in Cinncinnatti on my way home from

Boston (I live in Chicago).  Since I didn't have the tire gauge with me (and it

was pretty late), I drove her home without checking the tire pressure from the

factory/dealer.  The next morning, it was the first thing I did.  To my

suprise, there was at least 40 lbs. in each tire!  Needless to say, I set 'em

down to 30 psi (cold - add 4 lbs. if warm).  I have always stuck with the

sticker on the door for the stock tires on a car, and it has done me well so

far.  I put 60K mi. on a set of Michellins (sp?) that came with my '91 Cutlass

by sticking with the door sticker pressure.

BTW, I will be posting later this week my excellent experience with using ACS

(an auto broker) to purchase my vehicle.  Car buying has never been so easy,

and I was *very* pleased with the price!

Jeff

------------------------------

End of IMPALA Digest 312

************************

IMPALA Digest 313

Topics covered in this issue include:

1) RE: plugged tire

by "Harris, Scott" <sharris@xyplex.com>

2) Mirror

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

3) Re: Reference to "Personal" Messages

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

4) Dues for newsletter

by Franklin Poole <franklin@intersurf.com>

5) The Wandering Beast

by lennie@earthlink.net (Lennie Dolphin)

6) 95 w/console

by Franklin Poole <franklin@intersurf.com>

7) Arlington Show

by Loren Peterson <equ_rino@cencom.net>

8) Technical Information on GM Chemical and Maintenance Products

by T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

9) Re: DEADBEAT ALERT!!!!!!!!!!!!!!!!!

by Kurt Kemmling <76165.1654@compuserve.com>

10) Re: Beavis & Butthead

by Kurt Kemmling <76165.1654@compuserve.com>

11) Radar, etc.

by Kelly Pratas <72537.655@compuserve.com>

12) Re: digest -able?

by lennie@earthlink.net (Lennie Dolphin)

----------------------------------------------------------------------

Date: Wed, 28 Feb 1996 03:30:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Cc: "'mbailey@rinet.hunton.com'" <mbailey@rinet.hunton.com>

Subject: RE: plugged tire

Message-ID: <3134C974@corvette.xyplex.com>

>>What effect, if any, does a plug have on the performance of a tire/its

ability to withstand supersonic speed?

A plug seriously weakens the tire to the point where many tire

manufacturers have stated that Z,V & H rated tires are downgraded to an S

rating.  Some went so far as to say plugging is not an acceptable repair

method.  I would have them pull the tire off the car, dismount the tire

and put in a proper patch from the inside, remount the tire, spin balance

it, then put it back on the car.  This should cost $20.  If you want to

be more secure than that, i.e. you regularly drive well in excess of the

speed limit (at a track of course), shell out the $137 and buy a new

tire.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Wed, 28 Feb 1996 13:35:54 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

Cc: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: Mirror

Message-ID: <Pine.SOL.3.91.960228132923.20726D-100000@netinfo.ubc.ca>

Well Scott:

You will be glad to know that you are not the first one who has

the problem with the mirror.  I had the unfortunate situation like yours

happened to me about a month ago.  3 days right after my right hand side

mirror was vandalized.  So from my experience I can tell you that unless

you didn't chip away any plastic on the mirror housing, the whole mirror

would have to be replaced.  The only things on the mirror assembly that

can be replaced separatedly are the glass, and the triangular moulding

piece.  The mirror assembly cost $157 (cdn) trade (insursance comany

rate).  They come from the factory primed but unpainted (the painting

labour is 0.9 hour and the installation labour is 1 hour according to the

Mitchell's guide).  The whole thing cost me around $350 cdn with tax.

So, depend on your insurance coverage, it might be worthwhile for you to

claim it.  BTW, if you have a 94, the mirror housing assembly is actually

more expensive.  As I recalled, the cost was around $200 cdn.

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red

------------------------------

Date: Wed, 28 Feb 1996 13:52:13 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Subject: Re: Reference to "Personal" Messages

Message-ID: <9602282155.AA27506@karloff.fstrf.org>

At 12:41 28/2/96 PST, you wrote:

>------------------------------

>

>Date: Wed, 28 Feb 1996 06:23:09 PST

>From: Erich Arndt <EARNDT@mednet.swmed.edu>

>To: impala@east.xsis.xerox.com

>Subject: Reference to "Personal" Messages

>Message-ID: <s13411a0.096@MEDNET.SWMED.EDU>

>

>To all the members of the list, please forgive my propagating this flame.

>

>Jeremey,

>      Please understand that this list is dedicated to people sharing a

>common interest -- the Impala in this case.  That also implies that we care

>not only about our cars, but ABOUT EACH OTHER.  Scott M. is a revered

>member of this list and is one of the major forces that is responsible for

>its existence, not to mention the fact that he has probably forgotten more

>about the late model Chevy than most of us could ever hope to know.  He

>is dealing with a very difficult situation and has EARNED our respect and

>support.

>

>Scott M.,

>    We (most of us anyway)  hope your wife is doing well.

>

>Thanks for the patience,

>                             Erich Arndt

>

>------------------------------

I don't speak up too much, although I've been around since the beginning,

but I must voice to Scott M. that my thoughts are with you and your wife.

At the risk of sounding "mushy," not only have I gained invaluable experience

about my Impala and cars in general, but have enjoyed the comeraderie shared by

all of us Impala lovers.  The personal discussions that are carried on

in-between pure gear-head, shop-talk discussions are what give this list its'

character!

(Flame on)

Now, I wouldn't go as far as calling Jeremy a "heartless bastard," <G> but

I would say that if you're not happy with the content, unsubscribe.  If you

just want pure facts, part numbers, etc... use the Web pages, or call Chevy.

(Flame off)

Thanks to the list for all the info so far.  For a description of my own

solution to the mod of removing "home plate" check out my web page at:

http://stewart.fstrf.org/~calanan

It's not anything revolutionary, just another (IMHO, better <G>) option

to using a plastic lid.

- mike

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

My '96 Impala SS: "It goes fast, FAST!"

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

Date: Wed, 28 Feb 1996 14:14:19 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Dues for newsletter

Message-ID: <3134D3BB.4B5@intersurf.com>

>I'm asking that all those who haven't gotten around to sending Troy their

>dues to please  make a concerted effort to do so.   He has spent a lot of

>time and money getting this project off the ground, the least we can do is

>send him our measly $10 to pay for the newsletter.  Just think of it

>as another mod for the big boy in the garage.

Troy has had to raise the rate to  $20US which is still reasonable.  Troy and I

emailed each other the other day and I advised him to CUT YOU OFF AT THE KNEES if

you hadn't paid your dues!!!

Get out that check book and write the man a check! Today!

NAISSO

1648 Taylor Road #202

Daytona Beach, FL  32124

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Wed, 28 Feb 1996 14:42:28 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: Impala <impala@east.xsis.xerox.com>

Subject: The Wandering Beast

Message-ID: <199602282242.OAA26139@finland.it.earthlink.net>

Ever wonder why the beast wanders when it hits one of those ruts or dips

while crusing the boulevard?

I just retorqued the sway bar insulator bushing nuts to 18lb-ft. WOW!! What

a difference:

- increased positive feel in the steering wheel

- rides better

- improved handling and maneuverability in the straightways and curves

- best of all "No Wandering"

Hope this helps!!

... Lennie

------------------------------

Date: Wed, 28 Feb 1996 14:51:47 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: 95 w/console

Message-ID: <3134DC83.16FA@intersurf.com>

>I was at a small shopping center yesterday and parked next to a dark

>cherry SS.  Naturally I had to scope it out.  I noticed it had bucket seats

>and a console, but the shift lever was on the steering column.  Were any

>built like that, or did someone make a mod to a '95?

If you just saw a stock '95 they had a console, just no shifter.  Same split

bench/bucket arrangement.

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Wed, 28 Feb 1996 14:55:45 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Arlington Show

Message-ID: <3134DD71.2657@ns.cencom.net>

I would like to encourage those in charge to let us know where home base

will be in Arlington.  As busy as that area is, it wouldnt be bad to

make individual reservations a.s.a.p.  Time is close and I am excited!

Loren DCM96 SPE

Texas or bust

------------------------------

Date: Wed, 28 Feb 1996 13:23:39 PST

From: T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

To: IMPALA@east.xsis.xerox.com (Non Receipt Notification Requested)

Subject: Technical Information on GM Chemical and Maintenance Products

Message-ID: <0070hbhzdvxn*/I=T/S=MAKICHUK/OU=FA.EPS/O=NHQ/PRMD=GC+DRHC.HRDC/ADMD=GOVMT.CANADA/C=CA/@MHS>

As mentioned in one of my previous postings I have some technical

information on GM chemical products.  There seems to be some interest in

this so I'll post a sample.  The information comes from a GM of Canada

Publication <Chemical and Maintenance Products Product Information>.  This

publication is strictly intended for use at the dealer level, and contains

various information on GM chemical and maintenance products.  The

Publication numbers are as follows: 180-0M-791 is the original February

1985 124 pp. publication; -791A is the November 1988 128 pp. revision;

-791B is the binder to hold 791A; -791C is the dividers for 791A; -791D is

the spine and cover to fit into 791B; and -791E is the 1992 update for

791A.  I own 791, 791A and 791B.

I have good contacts at DGN, GM of Canada's publishing house, and tried

just about everything to get -C, -D and especially -E, but it seems that

very few were ever printed, and they're long gone.  I even called Helm in

the US, but they had no idea what I was talking about.

This is the listing for Cleens - X66 - Top Engine Cleaner that was recently

discussed in the digest.

-------

CARBURETOR AND COMBUSTION CHAMBER/CLEENS/CLEANER

Part Numbers:  993026  525 g Aerosol (CLEENS)  (875)

992872  500 ml (CLEENS)  (875)

Description:  Clear, amber liquid, distinct aromatic odour.

Physical Properties:

Boiling Point:  Above 212'F

Vapour Pressure:  Less than 1mm Hg

Solubility in Water:  Partly Miscible

Specific Gravity:  0.913

Volatile Fraction:  70%

Viscosity at 40'C:  55

Flash Point:  Above 135'C (COC)

Contains:  Aromatic Solvents, Butyl Cellosolve, Water, Detergent, Oil and

Soap.

Uses:  Helps to remove gum, varnish, and other deposits from the induction,

combustion, and exhaust areas.  Then, helps keep deposits within acceptable

limits.  See the diagram attached for engine areas that benefit from the

conditioner.

Directions For Use:  Use conditioner with every tune-up and/or 15,000 kms.

1.  Run engine until warm.

2.  At medium-fast idle, spray the product directly into the carburetor

venturi without stalling.

3.  When can is empty, choke off air supply immediately and turn off

ignition.  Wait for 10-15 minutes.

4.  Restart and blow out the deposits.

Benefits To Use:

- Easy to use and quickly eliminates deposits.

- Restores compression losses caused by deposits.

- Contains rust preventative and is non-toxic to skin.

- Will not harm painted surfaces under normal conditions.

Clean-Up, Storage, and Safety Data:

Spills:  Scrape up and place in disposal containers.

Waste Disposal:  Incinerate.

Storage:  Keep from freezing.  Avoid storing near sources of heat.  Provide

at least 2 feet clearance from ceiling heat sources when stacked on

pallets.

Extinguishing Media:  Carbon dioxide, dry chemical, foam.

Inhalation:  Move to fresh air.

Eye Contact:  Wash with water until irritation stops.

Skin Contact:  Wash with water.

Ingestion:  Do not induce vomiting, contact physician.

CASE QUANTITIES:  525 g - 12  500 ml - 12

<There is a one page diagram featuring a cross section of an engine with

the following text.>

ENGINE AREAS THAT BENEFIT MOST FROM TOP ENGINE CLEANER

1.  CARBURETOR LINKAGES AND EXTERIOR-CARBURETOR CHOKE PLATE.

2.  VENTURI (AIR INLET)

3.  THROTTLE PLATE

4.  INTAKE MANIFOLD

5.  INTAKE VALVE AND VALVE GUIDE.

6.  COMBUSTION CHAMBER AND SPARK PLUG.

7.  PISTON TOP AND COMPRESSION RING AREA.

8.  HEAT EXHAUST VALVE

9.  CATALYTIC CONVERTER

-------

I believe that most, if not all, of the chemical products sold by GM of

Canada are also available from GM in the US.  The part numbers will most

likely be different because Canadian law requires the measurements to be in

metric, and the label to be in both English and French.

If anyone is still interested after reading this WOB, E-mail me directly.

Maybe I could have photo copies made of my 791A, and for a small charge ...

Tip Makichuk

"I can't sell 'em to ya, cause they're illegal"

------------------------------

Date: Wed, 28 Feb 1996 15:47:30 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Cc: Troy Willrick <Rock528@aol.com>

Subject: Re: DEADBEAT ALERT!!!!!!!!!!!!!!!!!

Message-ID: <960228234729_76165.1654_GHM98-1@CompuServe.COM>

In digest 310 Troy wrote:

>>>  I will say here that I

still only have a 52% paid subscription rate. <<<

This is really SAD and EMBARRASING!!!!!!!!!!!!!!!!!!!!

Come on all you big spenders................get out that

checkbook and send Troy that 10 bucks. It is a small

price to pay, and you can't argue that you already haven't

received many many times that in invaluable information alone.

Maybe if you printed the names of those who still haven't

paid, it will embarass them into it, but I hope you would

not have to resort to such a tactic Troy, as everyone should

just step up and pay their fair share.

I can hear the squeaking of those crusty dusty billfolds

all the way up here in CT.

Troy, you deserve a great big applause for all of the

work you have done to make the NAISSO what it is today.

To put it plainly, there would be no club today if it were not for you.

You have devoted countless hours of your time and who

knows how much of your own

money and deserve the recognition for what you have

done, and for what you are planning to do in the future.

That letter from Jim Perkins is so cool. Congratulations.

It must feel good to get some recognition from Chevrolet,

and by writing this, I just want you to know that I also very

much appreciate what you are doing for us Troy.

I am sure the very reasonable $10 you ask for barely covers

your printing and mailing costs. I can't beleive that almost

half of the people reading this expect the rest of us, to pay their way.

So this is a call, to all of those who have been coming

along for the free ride. You surely can afford a measly

$10 to help defray some of Troy's costs. If you can't, then

it's time to get off the gravy train right here.

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Wed, 28 Feb 1996 15:47:33 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Beavis & Butthead

Message-ID: <960228234732_76165.1654_GHM98-2@CompuServe.COM>

>>> "That would be cool, heh heh hehheh" "Fire! Fire! heh hehheh heh"

<g> Scott.<<<

Hey Beavis...............That was cool...!!!!!!------<slap><BG>

------------------------------

Date: Wed, 28 Feb 1996 15:49:19 PST

From: Kelly Pratas <72537.655@compuserve.com>

To: Impala SS Mail <impala@east.xsis.xerox.com>

Subject: Radar, etc.

Message-ID: <960228234918_72537.655_EHM69-1@CompuServe.COM>

Randy, you wrote:

)) We use a Solo in our Impala too, (no heat-reflective windshield).

I've talked to Escort representives as well as Chevy dealers, and they've

said the worst place for a radar detector is where you have yours mounted.

Even if you don't have a heat-reflective windshield, the upper-most part of

the glass is tinted anyway, and causes some loss in performance of the

detector.

Perhaps more explanation in in order.  I definitely have the Sungate heat-

reflective windshield -- stamped in the lower passenger side (thanks Steve Das

for advising how to tell for sure).  I very rarely use a radar detector in my 96

Impala

because I have yet to take a road trip.  I have used a Solo in my 93 K5 Blazer

and found that it generally gives adequate warning, not spectacular but okay.

For the last two weeks there has been a predictable speed trap on my way to

work.  Every weekday, same place, same time so I have had an excellent op-

portunity to test the Solo in the Impala.  Every day, without fail, the Solo

warns

of the radar between 1/4 and 1/2 a mile.  I have not tried to maximize the

effec-

tiveness of the detector mount -- I simply wedge it between the headliner and

the rearview mirror.

My original post was meant to let some of you guys know that at least one Impala

owner is having some success with a detector AND the heat reflective windshield.

Once I am able to test the Blazer under the same conditions I'll let you know

the

results.

Scott, you wrote:

))I could have just said "medical problems", but that might

have raised additional questions and besides it is just my nature to be

technically more detailed than that! Most people might not have said

anything at all, but I feel that we have all become friends here, much more

so than on other lists I have been on. Scott.

There are many of us on this maillist and other automobile forums who are very

indebted to you for your unselfish posts and sharing of your technical

knowledge.

When someone like yourself takes the time and expends the effort to help those

he hasn't even met, it becomes readily apparent what kind of person you are.  I

am glad to have made your acquaintance through this list and consider you

more than just another email address.  I hope your wife continues to do well.

Kelly

------------------------------

Date: Wed, 28 Feb 1996 15:53:51 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: "Tim M. Killmeyer" <74144.103@compuserve.com>

Cc: Impala <impala@east.xsis.xerox.com>

Subject: Re: digest -able?

Message-ID: <199602282353.PAA08770@iceland.it.earthlink.net>

Tim said:

>has anyone figuired out a good way to glean the messages you want to keep

out of a digest and canning the rest?

Yes, I copy the messages I want, paste it into a new message and print it

out for my "Technical Beast Notebook", then trash the rest. You can always

go to the impala home page read the historical archived digests to get

additional STUFF if you need it.

>Do most of the people on the list run the thirty pounds of pressure in their

tires like it says on the door or is there a better way?

No, I do the option 35lbs like the 9C1s do...the thing handles much better

and less drag at the "go fast speeds", in my opinion. Usually get 50K out of

a set of 235/70-15 Michelins.

Hope this helps!!

... Lennie

------------------------------

End of IMPALA Digest 313

************************

IMPALA Digest 314

Topics covered in this issue include:

1) Re:"Personal" Messages

by lennie@earthlink.net (Lennie Dolphin)

2) Re:ARRRGGGHH!!

by lennie@earthlink.net (Lennie Dolphin)

3) Herb Adams/Bilstein

by lennie@earthlink.net (Lennie Dolphin)

4) Re: Various

by Rock528@aol.com

5) WE ARE UP! FUEL INJECTION SPECIALTIES

by Fuel Injection Specialties <fuelinjection@fuelinjection.com>

6) Sunguard Windshield vs. Radar Detector

by Steve Das <sdas@sat.net>

7) Searching the archives

by Ray Berilla <writes4u@ganet.net>

8) Oil viscocity choice

by "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

9) Newsletter Dues

by bngwip@mail.phoenix.net (Brett A. Rudolph)

10) New 1996 Impala Order

by SManey0830@aol.com

11) RE: Personal Messages....

by John Spina <caspers@miso.wwa.com>

12) 2nd Gear Selection

by Robert Cheek <rcheek@one.net>

13) Brake Pads

by Robert Cheek <rcheek@one.net>

14) Re: Stereo Options post by Tom Drake

by dstabb@epix.net (David Stabb)

15) Drag Racing & Weight

by Jerry Phillips <jerryp@stl-06lssc.army.mil>

16) Popping sound in Caprice/Impala

by lennie@earthlink.net (Lennie Dolphin)

----------------------------------------------------------------------

Date: Wed, 28 Feb 1996 15:53:54 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: Erich Arndt <EARNDT@mednet.swmed.edu>

Cc: Impala <impala@east.xsis.xerox.com>

Subject: Re:"Personal" Messages

Message-ID: <199602282353.PAA08776@iceland.it.earthlink.net>

Erich:

>Scott M. is a revered member of this list and is one of the major forces

that is responsible for its existence, not to mention the fact that he has

probably forgotten more about the late model Chevy than most of us could

ever hope to know.  He is dealing with a very difficult situation and has

EARNED our respect and support.

RIGHT ON!! I second that...he is Chevy God and spiritual leader...when he

writes we read!<g>.

... Lennie

------------------------------

Date: Wed, 28 Feb 1996 15:53:57 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: mbailey@rinet.hunton.com (Bailey, Martin)

Cc: Impala <impala@east.xsis.xerox.com>

Subject: Re:ARRRGGGHH!!

Message-ID: <199602282353.PAA08786@iceland.it.earthlink.net>

Martin said:

>The dealer fixed the flat, performed the "park" recall, gave me a

complimentary alignment and is trying to fix the noise.

If the ball joints didn't do it, tell the dealer to look at the pitman arm

which attaches to the power steering gear box...might need torquing and/or

lubing. Also, have him check the attachment bolts for the gear box, they

might need retorqing too. That should do it.<g>.

>What effect, if any, does a plug have on the performance of a tire/its

ability to withstand supersonic speed?

Picked what a screw about a month ago...still goes fast with no problems.

... Lennie

------------------------------

Date: Wed, 28 Feb 1996 15:53:43 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: Scott Mueller <73145.1566@compuserve.com>

Cc: Impala <impala@east.xsis.xerox.com>,

Subject: Herb Adams/Bilstein

Message-ID: <199602282353.PAA08762@iceland.it.earthlink.net>

Scott:

Talked to Totally Stainless today. Catalogue is on its way. Can't wait to

see that work of art, as you called them bolts.<g>.

Greer sent me this fast mail on an aftermarket vendor who has the Bilsteins

at a much lower price than Mr. Goodwrench. I called them, they said YES,

they are the same ones that go on the 9C1. Greer was "Right On" with this one.

Can this be the "REAL STUFF"?

================================================================================

Date:     Mon,  5 Feb 96 12:34:37 PST

From: "Greer Murphy"    <Greer.Murphy@Forsythe.Stanford.EDU>

To: lennie@earthlink.net

Don't buy those Bilsteins  from the dealer!

I got mine from W.M. Saunders 203-723-6907 for $68.67

each.

By the way, I checked directly with Bilstein, and they make only ONE set of

shocks for B-body cars.  You get the same shock regardless of whether you

buy it through GM or aftermarket.

The shocks are great!

Greer Murphy

94 Impala SS

Stanford, CA

================================================================================

... Lennie

------------------------------

Date: Wed, 28 Feb 1996 16:02:04 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Various

Message-ID: <960228190203_233704786@emout07.mail.aol.com>

>>> No offense intended, but I did not subscribe to the Impala line to hear

>about peoples' ectopic pregnancies <<

I know you are waiting for something clever and believe me, I wrote it.  I

have adopted the 24 hour rule just like when I look at a new car.  I don't

send it for at least 24 hours and by that time I've defrosted.  It was good

though...I try hard at being a cunning liguist.

Regarding the 10 or 20 dollar amount.  I hope the post today cleared it up.

After the first of the year, I had to raise it due to all the expenses

mentioned.  One late mortgage payment convinced me.  We are still at a 52%

pay rate but I expect it will improve after this newsletter.

Picked up a new member today from Hawaii.  Some of the police officers out

there use their own Impalas for work on a reimbursement mileage type deal.  I

also did some sniffing around and found the Impala badge should reappear in

about 99 or 2000 on a Lumina platform.  If you quote me, I'll deny it. <g>

Troy

ps - I'm glad I am not the only one who thought we were actually becoming

friends.  Working midnights and flying unconscious drunks to the hospital all

night long, there's not too many other intelligent people I get to speak to.

;->

------------------------------

Date: Wed, 28 Feb 1996 16:18:58 PST

From: Fuel Injection Specialties <fuelinjection@fuelinjection.com>

To: impala@east.xsis.xerox.com

Subject: WE ARE UP! FUEL INJECTION SPECIALTIES

Message-ID: <19960229001857953.AAA223@FIS>

To All Impala Owners,

Fuel Injection Specialties is finally up on the Net.

Check out our homepage at www.fuelinjection.com

Fuel Injection Specialties

http: www.fuelinjection.com

email: fuelinjection@fuelinjection.com

phone: 1-210-654-0774

fax: 1-210-654-0775

------------------------------

Date: Wed, 28 Feb 1996 14:31:48 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: "Scott Blum" <rsblum@directmedia.com>

Subject: Sunguard Windshield vs. Radar Detector

Message-ID: <199602282231.QAA09849@raven.sat.net>

>I mistakenly ordered my '96 with the SunGuard windshield before the thread

>about the effects to radar detection began herein.  Although I quickly tried

>to change my order, it had already been preferenced--meaning, I was told,

>sorry, too late...#%$@!

>A word for anyone considering ordering a new SS or you are still in a position

>to change your order, do yourself a favor and delete the SunGuard option.  I

>now, like many of the others who made this mistake, now face the added expense

>of buying a new remote detector, paying the ridiculous cost to have our

>present units converted (Steve, Kurt), or replacing the windshield with the

>standard issue.  None of which seem particularly appealing to me.

Scott,

Living it the South of Texas, I can tell you that the Sunguard windshield is

a BARGAIN.  I can go out in the afternoon with the car sitting in the 95+

degree temperatures and in the bright sun and the dash is simply the

temperature of the inside of the car... that is to say, it is warm but not

really too hot.  Go to my wife's silver Buick with the "normal" windshield

and a very similarly colored dash pad and the dash in that car WILL BURN

YOU!!!  Been there, done that!!  I have no doubt that the Sunguard

windshield is a good deal in terms of the long term life of the (very

expensive) dash pad.  I will gladly pay the expense of the remote detector

to save the dash pad and lower the interior temperature of the car in the

summer.  Incidentally, I know people that complain about the new air

conditioner's not cooling as well as the old Freon 12 units.  Well, the AC

in my Impala seems to work just fine... in South Texas mind you.  I don't

know that it is because of the windshield but it ain't hurting either, I'll

tell you that for sure.  Another side benefit to the remote detector is not

having to worry about leaving it to attract a thief,  Small benefit maybe,

but none the less, a benefit.

I think you are making a poor recommendation when you suggest people should

not order the Sunguard windshield.  If it is offered on any car I order in

the future, I'll get it for sure!!

Stete....sdas@sat.net

------------------------------

Date: Wed, 28 Feb 1996 18:11:24 PST

From: Ray Berilla <writes4u@ganet.net>

To: Impala list <impala@east.xsis.xerox.com>

Subject: Searching the archives

Message-ID: <199602290218.VAA05399@s1.GANet.NET>

>>has anyone figuired out a good way to glean the messages you want to keep

out of a digest and canning the rest?

>Yes, I copy the messages I want, paste it into a new message and print it

out for my "Technical Beast Notebook", then trash the rest. You can always

go to the impala home page read the historical archived digests to get

additional STUFF if you need it.<

Speaking of which, has anyone looked into the possiblity of adding some

sort of search mechanism on one of the Impala websites for the archives?

Maybe something like Yahoo or Lycos where you could just type in a keyword?

Dave, Rob how difficult would this be? Thanks.

Ray B.

------------------------------

Date: Wed, 28 Feb 1996 11:00:11 PST

From: "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

To: impala <impala@east.xsis.xerox.com>

Subject: Oil viscocity choice

Message-ID: <9602290240.AA3562@CServe-Hub4.inhouse.compuserve.com>

List members:

I have already decided to go with Mobil-1 synthetic in the SS based on comments

from others that have used it with great success.  My question is with regard

viscocity choice:  The manual recommends 5W-30 or 10W-30.  I will probably opt

for the 5W-30, since it also says to use this on the oil cap.  Any comments

from fellow members of the list?

Also, I have been using AC/Delco "regular" oil, 10W-30, in my '91 Cutlass

Supreme (now has 60K mi.) since day 1 of owning the vehicle, but would like to

now switch to a synthetic.  It has the same viscocity recommendations as the

Impala, as well as 5W-30 on the oil fill cap.  Do you think I will have any

problems if I now switch to the lighter 5W-30?  Any comments would be

appreciated!

They also now make a version of Slick50 oil treatment called "Synthetic", and

recommend its use if you are using synthetic in the car.  I know Mobil-1 used

to say not to use Slick50 if you are using Mobil-1, since it adds no value, but

I wonder if the new "Synthetic" version changes this premise.  Again, any

comments on oil additives such as Slick50 would be appreciated.

Jeff

'96 DCM, no mods

------------------------------

Date: Wed, 28 Feb 1996 21:16:53 PST

From: bngwip@mail.phoenix.net (Brett A. Rudolph)

To: impala@east.xsis.xerox.com

Subject: Newsletter Dues

Message-ID: <Chameleon.960228212837.bngwip@dialin.phoenix.net>

I haven't taken delivery of my SS yet and was wondering about joining your

NAISSO. Do I have to have the car first or just the 20 bucks? ;-)

Still need to sell my 94 Formula Firebird if anyone knows anyone who is

interested. 19K miles. $15,200. What a bargain!  ( I wish my SS was here

already ) :-(

I have a line on a Manufacturer whois talking with me about making shorty

headers for the SS. They will not be so cumbersom as the Arizona Speed and

Marine headers. ( I Hope! ) I do like the looks of their cold air kit that I

saw in an ad in Chevy High Performance.

In the same magazine, I saw an ad for 49 and 50 state legal LT-4 Conversions

for the SS from a company called Doug Rippie Motorsports. Anyone heard

anything good or bad about this procedure?

Brett

------------------------------

Date: Wed, 28 Feb 1996 19:28:10 PST

From: SManey0830@aol.com

To: impala@east.xsis.xerox.com

Subject: New 1996 Impala Order

Message-ID: <960228222810_156138980@emout10.mail.aol.com>

This is a great thing going here! Just ordered a black 1996 last week (called

and deleted the heat reflective windshield after reading the info here). I

just found out today it won't be built until June (it's an GM employee option

1 order). Anyone else order option 1? How long did it take? Kinda like to

have my car now...

------------------------------

Date: Wed, 28 Feb 1996 19:25:55 PST

From: John Spina <caspers@miso.wwa.com>

To: 'Multiple recipients of list' <impala@east.xsis.xerox.com>

Subject: RE: Personal Messages....

Message-ID: <01BB0623.7DB422C0@vh2-043.wwa.com>

Scott M explains.....

Yes, I apologize if the personal info was out of line,.......

No Scott, you're not out of line at all.  In fact, I was scouring the

'Vettefest looking for you and your SS. A mere explanation as you wrote was

all I needed to understand your absence at the 'fest. I don't believe an

apology is in order here.  Anyone that reads the digests even occasionally

will attest to SCOTT M. and his incredible knowledge and input that we all

have come to appreciate here on the list.  Scott, hope all is well at home

and will see you soon in the cruising circuit here in Chicago. John Spina

------------------------------

Date: Wed, 28 Feb 1996 19:32:24 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: 2nd Gear Selection

Message-ID: <1.5.4b11.32.19960229033224.0066bcc4@mail.one.net>

I tried monkeying around with the shifter while I was driving today

to no avail. There was no way I could get second gear. I had it at

a point where just a little movement shifted up to D, and the tranny

almost immediately shifted through second and third at the throttle

I was giving it. I'll have the dealer look at it, but I'm afraid I'll

get hassled about the "Z" PCM if I tell them, and that they will try

to reprogram (or replace!) it if I don't. I know there have been some

discrepancies between the 94 and 95 programs related to the transmission

(the Performance Shift Program comes to mind). Could it be related

to this?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Wed, 28 Feb 1996 19:32:22 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Brake Pads

Message-ID: <1.5.4b11.32.19960229033222.00664440@mail.one.net>

I'm about to need to change brake pads in the Impala. Should I run

with the stock pads, or are there some 9C1 parts that I might want

to upgrade to? Aftermarket pads? Also, should I change brake fluid

due to the added stress of ABS? I've got about 33K on it now.

Thanks,

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Wed, 28 Feb 1996 20:45:39 PST

From: dstabb@epix.net (David Stabb)

To: impala@east.xsis.xerox.com

Subject: Re: Stereo Options post by Tom Drake

Message-ID: <v02130502ad5ad9d5a956@[199.224.67.82]>

I'm going through the same process. For factory sound, call your dealer and

get the name of the Delco rep. The Delco people have information about all

GM radio gear and do installations as well.

>From what I understand, only the Cassette Player with Speed Compensated

Volume has the capability of running a changer in the trunk directly. Of

course, there are other changer options, such as getting a changer that

broadcasts FM and picking up the signal with your tuner, but that is a

compromise. Also most aftermarket changers will saddle you with some weird

remote and stick-on control setup that only looks good in their catalogs.

So, I am sticking with factory Delco head unit and the factory 12 disc

changer from Delco (actually Pioneer). For once, I'm glad I didn't *insist*

on the CD player being swapped for the SCV cassette that came in the car.

As to the rest of your story. None of the Delco equipment has standard

RCA-out. In fact, it's real rough to work with Delco equipment and other

add ons. Your stereo installer shop should be able to find an adapter,

however (Mine did, CC Hi-FI in Lancaster, PA). This is necessary because

the Delco outputs a medium level signal, that is too strong for standard

inputs, but not strong enough for a standard attenuator. The unit I saw is

a blue box about 1x1.5x3 inches, and has two pots on it to adjust

attenuation. It will give standard RCA out.

As far as sound, the stock speakers are not very good. Quite muddy to my

ears, and there is little ambience. Speaker placement in the lower door

area doesn't help much. There is no room for a kickpanel placement. What

I've decided to do is to get rid of all the stock speakers, and put in

something better, probably 6 inch rounds in the back (you can get them to

fit the 6x9 oval holes). Rounds have a cleaner sound than ovals. I'll put

some smaller rounds in the doors, and will add some tweeters, probably

KEF's on the dash. These will probably angle toward the center of the car,

but we may experiment with a facing window placement before anchoring them.

The only concern with this is that they will be closer to the ear than the

door speakers, so stereo image may be slightly inconsistend. By

reverse-phasing the back speakers, we should be able to minimize the

effects of this and create an open ambience (old trick, Dyna did it on 4

channel vinyl records back in the 70's).

For the back, (at least) I'll split the input signal with a Nakamichi

active crossover that I rescued from my old car, and send the middle and

high end to the 6 inch rounds. There's room for my rescued 8 inch Nakamichi

subwoofers to go close (desireble, you get acoustic coupling, and

effectively a bigger speaker) together on the back deck. Haven't decided if

we'll need to do anything about the carpet on the ledge, or if it won't

matter much to the bass. The subs will be powered by a 200 watt Alpine. The

6 inch rounds (or perhaps the fronts) by my rescued 60 watt Concord amp.

Still deciding if I need another amp in this setup.

Fortunately, my better half (who says the SS is too *BIG*) also wants the

sound upgraded. :-)

____|)avid             Good Morning, Good Afternoon, Good Evening.

How difficult it is to tell when

dstabb@epix.net               you're locked in a tiny room with

dstabb@bix.com                     no windows and artificial lights.

------------------------------

Date: Wed, 28 Feb 1996 21:09:09 PST

From: Jerry Phillips <jerryp@stl-06lssc.army.mil>

To: impala@east.xsis.xerox.com

Subject: Drag Racing & Weight

Message-ID: <96Feb28.210909pst."15942(11)"@alpha.xerox.com>

I have a question for the accomplished drag racers.  It seems that the

key to a quick e.t. is traction, hooking up as soon as possible.  Is it

best to remove the spare tire, which is located in the trunk and aids in

the front/rear weight balance situation, or remove the spare to decrease

the overall vehicle weight?  I have always considered e.t. and more

importantly who gets to the other end first paramount over top speed.

Once you hook up, weight is bad, but what is the optimum situation from

roll out to hook up.  All of this is predicated on stock tires and

street legal configuration.

Jerry Phillips

St Louis

No fancy signature line, some of them seem to run over 80 characters and

really screw up the hard copy of the digests.

------------------------------

Date: Wed, 28 Feb 1996 21:22:41 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: Impala <impala@east.xsis.xerox.com>

Cc: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

Subject: Popping sound in Caprice/Impala

Message-ID: <199602290522.VAA23407@finland.it.earthlink.net>

Here are my findings in two separate cars.

'91 Caprice:

SYMPTOM: Popping sound when turning steering wheel lock-to-lock

CAUSE: 3 attachment bolts for the power steering gear box out of spec

REMEDY: Torque to spec and WD40 the pitman shaft

RESULTS: Never heard the sound again

'95 Impala:

SYMPTOM: Popping sound when turning steering wheel lock-to-lock

CAUSE: LH/RH lower ball joints out of spec and dry

REMEDY: Torque to spec and lube

RESULTS: Haven't heard the sound in about 3 weeks

Hope this HELPS!!

... Lennie

------------------------------

End of IMPALA Digest 314

************************

IMPALA Digest 315

Topics covered in this issue include:

1) Re: radar detectors

by Rock528@aol.com

2) Subscription price

by "Jeff Gatcha" <foxhound@txdirect.net>

3) RE: 2nd gear and brake fluid

by "Harris, Scott" <sharris@xyplex.com>

4) RE: snake oil

by "Harris, Scott" <sharris@xyplex.com>

5) RE: Drag Racing & Weight

by "Harris, Scott" <sharris@xyplex.com>

6) many opinions about many posts

by "Jeff Gatcha" <foxhound@txdirect.net>

7) speakers

by "Jeff Gatcha" <foxhound@txdirect.net>

8) Springs/Bars

by mbailey@rinet.hunton.com (Bailey, Martin)

9) RE: Sunguard Windshield vs. Radar Detector

by "Scott Blum" <rsblum@directmedia.com>

10) RE: Sunguard Windshield vs. Radar Detector

by Steve Das <sdas@sat.net>

11) Re: Stereo Options

by John.Salmon@bridge.bst.bls.com

12) Sunguard Windshield

by klin@morgan.com (R.Kevin Lindsay)

13) "A" Pillar Tweeters

by klin@morgan.com (R.Kevin Lindsay)

14) Springs

by Scott Mueller <73145.1566@compuserve.com>

15) Bilstein shocks

by Scott Mueller <73145.1566@compuserve.com>

----------------------------------------------------------------------

Date: Wed, 28 Feb 1996 22:31:16 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: radar detectors

Message-ID: <960229013115_234039683@mail04.mail.aol.com>

here is an interesting observation:  I haven't used a radar detector in at

least 12 years.  I haven't gotten a ticket or pulled over in that long

either.  I do drive somewhat spirited being aware of the normal max speed

before a hard copy gets issued.  Recently I figured the Impala does like to

creep up there, so I'll get me one of the battery powered nuclear radiation

detector devices.  Now without changing my driving habits from before, I've

gotten nailed twice by instant on radar in the last week.  It's like gettin

shot in the back of the head. If you don't know its coming, it don't hurt so

much...   Troy

------------------------------

Date: Wed, 28 Feb 1996 23:04:29 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Subscription price

Message-ID: <199602290703.BAA21036@legend.txdirect.net>

>From JGreen:

>I have a question about what Jeff Gatcha said.  He said we should at least

>send Troy the measley $10.00 for the newsletter.  I paid 20.00 last Saturday

>to Troy.  How much is it 10 or 20? If 10 then I'd like a refund if 20 cool

>I'm paid up. Another  thing why did Jeff get off for 10 if it is 20?  Also if

>it is 20 then why don't Jeff send troy the other measley 10.

When I subscribed back in August or September of 95, the cost was

$10.   If Troy would like me to send him another $10, to make me even

with the rest of the group,  I will be happy to.  I think the

newsletter is a valuable source of info and I look forward to getting

it. I also think supporting the Impala club/organization is also a

worthwhile endeavor, and am more than willing to pay my fair share.

Didnt mean to open up a can of worms.   Troy, let me know.

Actually I may just send you the money anyway, then I can look my

fellow Impala owners in the eye(s) knowing I paid like all the rest.

Jeff Gatcha

Foxhound@txdirect.net

1995 GGM

------------------------------

Date: Wed, 28 Feb 1996 13:21:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Cc: "'rcheek@one.net'" <rcheek@one.net>

Subject: RE: 2nd gear and brake fluid

Message-ID: <313554B4@corvette.xyplex.com>

Rob,

Did your car shift normally before you swapped the PCM?  Do you have

anyone local that you can swap with for a test?  Does the dealer have a

94 there to swap PCMs?  If you can prove it is the PCM I'm sure that Z

would fix it for you.  It still sounds like a linkage adjustment to me.

As for brake pads - I've heard that the stock Impala pads are very good

and the best available for the calipers.  Flushing the brake fluid is

more related to moisture absorption than "ABS stress."  Unless you

autocross or race at a road course track (is Mid-Ohio near you?) and heat

up the brakes pretty well flushing the system every 2 years and bleeding

each corner on the off year is a good rule.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Wed, 28 Feb 1996 13:21:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: snake oil

Message-ID: <313554B3@corvette.xyplex.com>

Before you think about using ANY oil additives read the write ups all

over the web on them.  I don't have any of the URLs handy but I'm pretty

sure there's one on the F-Body site.  A web search for "snake oil" should

turn up a few.  The stuff IS slick - slick marketing hype!  Dupont who

makes Teflon (the supposed engine coating in Slick 50) has said that it

can not function as an engine oil additive for years.  Mobil 1 is such a

good lubricant that I don't see what do you would expect to gain.

The difference between 5-30 and 10-30 Mobil 1 seems immeasurable.  I

wouldn't worry about using either one in anything other than a rotary

engine.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Wed, 28 Feb 1996 13:20:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Cc: "'jerryp@stl-06lssc.army.mil'" <jerryp@stl-06lssc.army.mil>

Subject: RE: Drag Racing & Weight

Message-ID: <313554B1@corvette.xyplex.com>

Weight-wise, removing the spare is not as important as arriving at the

track with the gas tank at 1/8 (or less).  Just guessing but I'd say the

spare weighs the same as a couple of gallons of gas.  While you're

preparing to run at the track, it wouldn't hurt to have some Sunoco Ultra

94 or equivalent in the tank instead of wimpy 87.  In addition to all of

the mods frequently mentioned here, for the track you should try pulling

out the MAF screen and pulling the PCM fuse for a minute to "clear the

codes."

More important than weight is engine temperature and air density.  If you

run on a nice cool night with low humidity, high pressure AND let the car

cool down with the hood open for an hour before you run, you're going to

see an easy couple tenths improvement over a warm daytime run.  I assume

you already have a 160 t-stat and 175-180 fans somehow.

As for getting it out of the hole - you'll have to see what works for

your car.  When mine was stock launching around 1,300 with a slow

throttle squeeze got the best 60 ft (2.1) and 1320 times.  But when the

mufflers "fell off" there was a little too much power and I went up to

2.2's.  The track closed before I could try starts from idle.  Some

friends of mine with automatic LT1 Vette's swear by idle starts.

Also, don't even think about playing with tire pressures.  Traction seems

to lessen at 25 psi and less so I've been leaving them at the 34 I use on

the street.  I talked to a few tire people and they all said that low

pressure in a wide radial will actually take traction away from the

center of the tire because it starts bowing up off the road.  I'd love to

get a set of the BFG Comp T/A DOT drag radials!

Good luck and please post your results.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Wed, 28 Feb 1996 23:41:20 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: many opinions about many posts

Message-ID: <199602290740.BAA21671@legend.txdirect.net>

Scott, you need to apologise for NOTHING!

I would like to echo the sentiments of Erich and many others here on

this list.  Many of you are more than an E-mail addresses, you are

friends, and this list is more than an info source for part numbers

and ideas.  Its almost like a family, to me anyway.  Scott, I too

wish your wife a speedy recovery.   Like Troy, I have a 24 hr rule

which has saved me a bunch of times.  Glad I didnt post what I really

thought of Jeremys insensitive post, with congress passing that

internet law and all....

About the tires:  I was keeping them at 32 cold, but increasing them

to 35 cold has made a noticeable improvement in handleing and the car

doesnt seem sluggish as it did before.  Got a thrust angle alignment

at the dealer and the alignment tech said to keep em around 35 psi

cold.

About the Heat-resistant Windshield, I agree with Steve.  South Texas

is a mighty hot place at times and that windshield does keep the heat

down to a great extent.  Parked next to each other on the same 100

deg day, my wifes Honda is a blast furnace,  the Impala, a toaster.

Heat does terrible things to a dashpad and interior,  I would order it

again in a minute.

Mobil 1 10w-30 vs 5w-30.   5w-30 might get you a few more feet per

gallon. Otherwise I dont think it makes that much difference which

one you use.  Both will perform admirably.   As for Slick 50, I

personally wouldnt use it.  I dont think you will gain any more

benefit than using the straight Mobil-1, and the potential for harm

exists if you believe some of those reports out there. Why take the

chance.   As for switching to synthetic in your 91 Olds, I think that

it would be fine.  I know several people myself included,  who have

switched  to Mobil-1  after 30-50,000 miles on late model cars

(85 and newer) with no ill effects.   I have heard it is different with

older cars ( 60's-70's) but I dont have any experience with that.

If Im off base on this one Im sure someone will let me know.

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala. REAL Oil press gauge, H-pipe with

Muffler delete, 1-LE elbow w/3.5"chrome pipe and Cone K&N,

Jet Air Foil, RED Bow-Tie, 9C1 deflectors, Upgrade speakers, Gas bib,

Corvette Valve & Injector covers.

Comming soon:  Z-pcm, 3.42 gear, and a Tach

------------------------------

Date: Wed, 28 Feb 1996 23:47:49 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: speakers

Message-ID: <199602290747.BAA21769@legend.txdirect.net>

Kicker makes a 4x6 speaker that fits in the doors of the Impala.

They have an angled tweeter that aims up and toward the driver

instead of at your feet.  They sound really good and aren't that

expensive.     Just a thought

Jeff Gatcha

Foxhound@txdirect.net

San Antonio Texas

1995 Green-Grey Impala. REAL Oil press gauge, H-pipe with

Muffler delete, 1-LE elbow w/3.5"chrome pipe and Cone K&N,

Jet Air Foil, RED Bow-Tie, 9C1 deflectors, Upgrade speakers, Gas bib,

Corvette Valve & Injector covers.

Comming soon:  Z-pcm, 3.42 gear, and a Tach

------------------------------

Date: Thu, 29 Feb 1996 05:13:07 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (Impala1)

Subject: Springs/Bars

Message-ID: <1996Feb29.080100.1049.107320@rinet.hunton.com>

Has anyone on the list installed the Herb Adams Wonderbars  AND springs that

lower the car? I know Scott Mueller has reported installing the bars and

springs that RAISE the car but I am curious as to the combination of the

huge rear sway bar and a 1.5" lowered ride height due to springs.  I am

contemplating the Eibach/Suspension Techniques springs.  I am concerned,

however, about placing that rear bar any closer to the ground; it already

drags occasionally in some cases with stock height.

Has anyone tried this and, if so, do you have any comments?  Also any

comments on the Eibachs vs. Suspension Techniques?  Other than the obvious

price difference.

BTW, I ordered the rear control arm fix kit today from my dealer per Scott

M's suggestion.  I paid $44 so my dealer likes me but doesn't love me.

Martin Bailey

mbailey@hunton.com

------------------------------

Date: Thu, 29 Feb 1996 03:17:33 PST

From: "Scott Blum" <rsblum@directmedia.com>

To: "Steve Das" <sdas@sat.net>

Cc: impala@east.xsis.xerox.com

Subject: RE: Sunguard Windshield vs. Radar Detector

Message-ID: <n1386539904.28851@smtp.directmedia.com>

Steve,

>Living it the South of Texas, I can tell you that the Sunguard windshield is

a BARGAIN.  I can go out in the afternoon with the car sitting in the 95+

degree temperatures and in the bright sun and the dash is simply the

temperature of the inside of the car... that is to say, it is warm but not

really too hot.  Go to my wife's silver Buick with the "normal" windshield

and a very similarly colored dash pad and the dash in that car WILL BURN

YOU!!!  Been there, done that!!  I have no doubt that the Sunguard

windshield is a good deal in terms of the long term life of the (very

expensive) dash pad.  I will gladly pay the expense of the remote detector

to save the dash pad and lower the interior temperature of the car in the

summer.  Incidentally, I know people that complain about the new air

conditioner's not cooling as well as the old Freon 12 units.  Well, the AC

in my Impala seems to work just fine... in South Texas mind you.  I don't

know that it is because of the windshield but it ain't hurting either, I'll

tell you that for sure.  Another side benefit to the remote detector is not

having to worry about leaving it to attract a thief,  Small benefit maybe,

but none the less, a benefit.

>I think you are making a poor recommendation when you suggest people should

not order the Sunguard windshield.  If it is offered on any car I order in

the future, I'll get it for sure!!

Do you, by chance, have anything to do with PPG? (Just kidding, of course).  I

thank you for the feedback on how well the SunGuard works.  Previously, no one

has mentioned if it in fact made a percievable difference, hence my request

for input from our southern members.  Up here in Chicago, we frankly will be

waiting a few more months for our 'investment' to pay off.

This revelation obviously puts a new twist on weighing this option.  Perhaps

once we get some feedback on the enlarged radar visibility concern, we'll have

a complete picture for all to weigh the pro's and con's in consideration of

their geographic location prior to ordering.  Thanks again for the

feedback--can't wait now for July...

Thanks,

-Scott B.

'96 SS, Black, 80w/100w HL's, 9C1 Deflectors, FelPro Diff Gasket, Upgraded

Speakers, ONE Gas Bib...So far.

------------------------------

Date: Thu, 29 Feb 1996 05:51:36 PST

From: Steve Das <sdas@sat.net>

To: "Scott Blum" <rsblum@directmedia.com>

Cc: impala@east.xsis.xerox.com

Subject: RE: Sunguard Windshield vs. Radar Detector

Message-ID: <199602291351.HAA11192@raven.sat.net>

>This revelation obviously puts a new twist on weighing this option.  Perhaps

>once we get some feedback on the enlarged radar visibility concern, we'll have

>a complete picture for all to weigh the pro's and con's in consideration of

>their geographic location prior to ordering.  Thanks again for the

>feedback--can't wait now for July...

No problem.  Just one's mans observation.  Hang in there, you'll love the car.

Steve

------------------------------

Date: Thu, 29 Feb 1996 06:36:32 PST

From: John.Salmon@bridge.bst.bls.com

To: Impala Ss <impala@east.xsis.xerox.com>

Subject: Re: Stereo Options

Message-ID: <H0000182059b1ac2>

I have had a system installed in my Impala w/CD player head unit.  As to

line level outs, no it doesn't have them, but a $29 part called an

AudioLink is all you need.  You run speaker-level input (which is

roughly 6-9V) to the Audiolink, and you get between 1.7 and 1.8V

preamp-level line outs.  I had mine done with two Audiolinks so I'd have

fader control (front speaker outs to one, rears to another).

In my system, from there RCA cables connect to two Soundstream Reference

405 5-channel amps (one left, one right) with both amps bridged into 3

channels.  each channel goes to a speaker.  Front stage is handled by 2

KEF Uni-Q 6 1/2" coincident-source drivers, rear fill is done with 2 KEF

Uni-Q 6x9's, and sub duties are handled by 2 Soundstream SPL 12 subs in

sealed 1 cu.ft. boxes on each side of the trunk.  The amps are mounted

with the passive crossovers for the KEF's on a MDF (medium-density

fiberboard) panel to the back of the rear seat area.  The factory spare

is still in place, and so is the cargo net.  Everything was built around

keeping 90% of trunk space plus all the factory mounting points for

everything.  The system would be rated in IASCA's 151-300W class, but

effective 14.4V RMS power is 110Wx4 + 200Wx2 = 840W.  It definitely has

headroom.

According to my installer (William Morris at Custom Radio in Columbus,

GA), the 6 1/2" coaxes will drop in with no problem.  He had to cut out

a half-moon shaped piece of metal about 2" x 1/2" from each side of the

front speaker hole to get clearance for the basket, but depth was no

problem.  He mounted the speaker on a 1/2" piece of MDF, then mounted

that to the door, mainly to get the acoustic benefits of the MDF (more

solid mount than the door sheet metal).  The rear speakers fit perfectly

in the factory holes, and are even inside the factory foam baffles.  The

system plays incredibly low (I've tried the Telarc "Chiller" disc, track

1, which has super-loud recordings of live thunder, and it sounds so

real the dogs run for cover).  It's also very tight, not a boom system

by any means.  But the pipe organ notes around 16Hz in track 2 of

Chiller really shake the car.  The Uni-Q drivers do the best job I've

ever heard keeping off-axis response clean and smooth (and you're almost

always off-axis from car speakers).

Make sure if you install your system or somebody else does that you

carefully set the gains on every downstream component from the factory

head unit if you go that way... William used an oscilloscope to set

everything to clip simultaneously, which did wonders for the little bit

of noise that was in the system (it was popping and whirring through the

front speakers whenever the CD track buttons were used, but it's silent

now).  The electronic volume control on the factory head is the only

annoyance--you can't tell when it's up to 3/4 volume or the clipping

point except by ear.  Also make sure your amps are very well grounded.

Another thing William did to improve dynamics from the amps was to

upgrade the factory ground strap from the battery to the car body from

the tiny 12-14 gauge factory wire to a 1/0 gauge wire (look at your

thumb--that's about the size of it).  Make sure your power leads are

fused within 18 inches of the battery, too.  Any wire directly connected

to the battery must be fused to prevent a fire should the wire short to

ground (in a wreck or from chafing against a hole in the firewall, etc).

Hope some of this helps.  I've got some more CD's to listen to, see ya!

-- John

jsalmon@mindspring.com

John.Salmon@bridge.bellsouth.com

------------------------------

Date: Thu, 29 Feb 1996 07:28:30 PST

From: klin@morgan.com (R.Kevin Lindsay)

To: impala@east.xsis.xerox.com

Subject: Sunguard Windshield

Message-ID: <9602290728.ZM9238@morgan.com>

I am planning to replace my windshield with the Sunguard. That option wasn't

generally available in '95 and I suspect that it will cool things down in the

summer. Here in L.A., last summer, the dash top got VERY hot and heated up the

whole car quite a bit. I just hope that the insurance replacement guys that

come to the house to replace the windshield are able to get the Sunguard OK.

Radar is not a big deal here--not that many people have detectors. The CHP

doesn't use radar on the fwys in So Cal--in fact, any car pulled over attracts

so much attention that it slows down the fwy a lot. It's as if people have

never seen a police car before!

Kevin

------------------------------

Date: Thu, 29 Feb 1996 07:47:01 PST

From: klin@morgan.com (R.Kevin Lindsay)

To: impala@east.xsis.xerox.com

Subject: "A" Pillar Tweeters

Message-ID: <9602290747.ZM9252@morgan.com>

This posting is in response to David Stabb's posting re his sound system plans:

You might think about adding tweeters inside the A pillars. They remove easily.

My installer used a metal template to drill a circular pattern of holes and

installed the tweeter inside of the pillar. The tweeters solve my perceived

imaging problem in my system. I needed some sound in front of me as the door

speaker location is quite low.

Kevin

------------------------------

Date: Thu, 29 Feb 1996 09:46:20 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Springs

Message-ID: <960229174619_73145.1566_FHD89-4@CompuServe.COM>

Martin,

>BTW, are you aware of any differences between the Eibach springs and the

Suspension Techniques springs other than price.  I am planning to make this

my next move.

I am not sure what differences there are, if any, between these springs.

Perhaps Eibach even makes the for ST? The info you would need to determine

this is the spring rate including whether they are variable (progressive)

rate or not, and the approximate height change from stock. Let us know what

you find out. Scott.

------------------------------

Date: Thu, 29 Feb 1996 09:46:34 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Bilstein shocks

Message-ID: <960229174633_73145.1566_FHD89-5@CompuServe.COM>

Lennie (and Greer),

>By the way, I checked directly with Bilstein, and they make only ONE set of

shocks for B-body cars.  You get the same shock regardless of whether you

buy it through GM or aftermarket.

There are actually two sets of Bilstein shocks available for the B-cars,

including the Impala. Bilstein lists the shocks as different versions for

the '77-'90 cars and the '91-96 cars, although of course these are

completely interchangeable under most circumstances.

Here are the intended applications and Bilstein part numbers:

Application    Front  Rear   Notes

------------------------------------------------------

'77-'90 B-car  #1104  #0929  OEM only

'91-'96 B-car  #1516  #1517  Also Used w/Chevy SEO 8X3

The shocks they specify for the '91-'96 models were specifically developed

to Chevy SEO 9C1 (police) platform engineering parameters, and are

actually the same (except for cosmetics) as those sold through GM under

the SEO 8X3 option on the 9C1 Caprice. They are all painted yellow, but

the factory 8X3 versions have black boots and Delco-Bilstein stickers and

a broadcast code sticker while the OEM Bilstein ones have blue boots and

regular OEM Bilstein stickers. The OEM versions are much cheaper than the

8X3 versions purchased through GM!

The 1516/1517 shocks feature firmer compression settings than the

1104/0929 shocks, which help control the car better when it is going over

large bumps, uneven roads, or when it is coming down from being airborne.

The 1516/1517 shocks are also progressively damped, which means they have

a variable rate that offers less resistance to light inputs, and much

greater resistance to large inputs. This gives a more comfortable ride,

especially over rough roads, but still offers greater control when the

situation demands. The 1104/0929 shocks are a linear damped shock, and do

indeed "feel" firmer, especially with lighter inputs.

Another issue is the 1517 rear shock was designed for 1" longer travel

compared to the 0929 shock. This was also specifically at GM's request,

primarily to facilitate tire changing on the '91-'92 Caprice 9C1 with the

skirted rear wheel openings. Obviously on the '93 and up Caprice or the

Impala which have radiused rear wheel openings, this is not an issue.

Here are the specifications on the Bilstein B-car shocks:

Bilstein B-car Front shocks:

P/N   Rebound / Compression  Notes

----------------------------------------------------------------------

'77-'90  #1104    4275 / 1200         Linear rate

'91-'96  #1516    1100 / 1495         Progressive rate

Bilstein B-car Rear shocks:

P/N   Rebound / Compression  Notes

----------------------------------------------------------------------

'77-'90  #0929    2135 / 1035         Linear rate

'77-'90  #1517     780 / 1040         Progressive rate, 1" longer travel

Bilstein commented that the 1516/1517 shocks are one of the only ones they

have done where the compression rate is higher than the rebound. These

were done specifically for Chevy at the SEO 9C1 platform engineer's

request. This does give them more of a "built-in float" than the 1104/0929

shocks. It helps cushion an impact (high compression), but the light

rebound makes the car feel somewhat floaty. As I understand it, these

specs are in Newtons per mm.

I am currently running the SEO 8X3 (1516/1517 shocks) in my Impala, also

with stiffer rear springs and the Herb Adams swaybars. I also have the

1104/0929 shocks on my '88 9C1 and they definitely feel much stiffer

overall and a little less floaty. I may end up changing to the 1104/0929

shocks on my Impala as well, after I first try another set of rear

springs. I like the stiffer 1104/0929 shocks, but I must say that drinking

a cup of coffee without chipping my teeth in my 9C1 is much more difficult

than in the Impala! <g>

If you are really into stiff shocks (I am!), then you might also try the

Koni adjustable shocks. Set these on one of the stiffer settings, and you

can forget about ever using the cupholders, plus you will need to go over

the car periodically to tighten all of the nuts and bolts that shake

loose! <g>

BTW, Bilstein did mention to me that they could do custom valving, but for

the cost ($55 per shock) I don't think I will do that for the Impala!

Scott.

------------------------------

End of IMPALA Digest 315

************************

IMPALA Digest 316

Topics covered in this issue include:

1) Re: Scott Mueller's personal post

by Kurt Kemmling <76165.1654@compuserve.com>

2) Re: Dues Increase

by Kurt Kemmling <76165.1654@compuserve.com>

3) IRC

by bngwip@mail.phoenix.net (Brett A. Rudolph)

4) RE: IMPALA digest 311

by "Thomas L. Pierce" <TLPierce@msn.com>

5) rear bar height

by Scott Mueller <73145.1566@compuserve.com>

6) Bilstein Shock Part # site

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

7)

by BGordon401@aol.com

8) Re: IMPALA digest 314

by Mikado95@aol.com

9) Searchable Impala Archives

by Robert Cheek <rcheek@one.net>

10) Bucket Seats

by "David P. Fenzl" <dfnzl@erols.com>

11) No Holds Barred Road Race?

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

12) Hello to all!

by Stephen Furr <skfurr@raleigh.ibm.com>

----------------------------------------------------------------------

Date: Thu, 29 Feb 1996 10:19:17 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Scott Mueller's personal post

Message-ID: <960229181916_76165.1654_GHM51-4@CompuServe.COM>

In digest 312 Scott Mueller writes:

>>>Yes, I apologize if the personal info was out of line, but as you indicated

the main reason I posted it was to explain the absence of my car at the

Chevy-Vettefest. I could have just said "medical problems", but that might

have raised additional questions and besides it is just my nature to be

technically more detailed than that! Most people might not have said

anything at all, but I feel that we have all become friends here, much more

so than on other lists I have been on. Scott.<<<

Scott,

No need to apologize. The wacker that wrote the flame probably

still hasn't paid his dues, so take it FWIW.<BG>

BTW, I agree with you about all of becoming friends, not just

anonymous members of a mailing list. You have contributed

more to this club than most of us ever will, myself included.

I hope your wife is doing better, and also hope you will tell

us when things like this happen. If the individual that objected

to your post has a problem with this, I have a simple solution:

==========UNSUBSCRIBE===================

Take care Scott,

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Thu, 29 Feb 1996 10:19:08 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Cc: Troy Willrick <Rock528@aol.com>

Subject: Re: Dues Increase

Message-ID: <960229181908_76165.1654_GHM51-1@CompuServe.COM>

In digest 312 Troy writes:

>>>I don't mean to be greedy but I had to raise it to $20.00 annually back in

January.  State and federal trademark fees, bank accts., phone line, monthly

mailbox service, tax license, county occupational license, and a 10 page

newsletter now done on a printing press.<<<

Troy,

You don't have to justify the price increase. I paid my 10 bucks as soon as

I found this list, and always thought it was too low. Even at $20.00, those of

us that are true members can see the value without making you feel like

you have to explain it to us.

You are obviously a very generous and sincere individual, and just want

to make everyone feel like they are getting their money's worth. I and many

others on this list know how much time and effort you have and are putting

into keeping this club together. There will always be freeloaders that will

complain about things, even when they haven't paid a thin dime, and you

can't please them no matter what Troy. Just keep doing what you are

doing. The dead wood will eventually just float away, and this club will

contain only those willing to support and contribute to it's betterment.

Thank you Troy for a great job.

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Thu, 29 Feb 1996 14:26:49 PST

From: bngwip@mail.phoenix.net (Brett A. Rudolph)

To: impala@east.xsis.xerox.com

Subject: IRC

Message-ID: <Chameleon.960229143023.bngwip@dialin.phoenix.net>

Do any of you guys ever talk on irc or aol? If anyone is interested,we should

get together and do a live ImpalaSS channel chat thing sometime. and discuss

Arlington or Super Chevy or Jeremy's lack of good sense or what ever. Just a

thought.

Brett

------------------------------

Date: Wed, 28 Feb 1996 13:41:06 PST

From: "Thomas L. Pierce" <TLPierce@msn.com>

To: impala@east.xsis.xerox.com

Subject: RE: IMPALA digest 311

Message-ID: <UPMAIL09.199602292252090686@msn.com>

Tim M. Killmeyer sez:

>>I've always told people that they should take their car...and autocross it

just to see how it handles. You can learn a lot about what a

car will do in tricky situations on the road.

Amen to that, Tim - it's suprising how many people don't know the limits of

the vehicles they are driving.

>>>Question: I've seen a couple of HIGHLY different numbers with regards to

the Impala's front to rear weight ratio.

Last time I was at the autocross, I had the beast corner weighed - here are

the numbers (with a 220 pounder in the drivers seat)

Left Front 1246	Right Front 1188

Left Rear  1049	Right Rear 993

Front Total 2334	Total Rear 2042

Total Weight	4376

Front to Rear Ratio	53:47

As I'm sure you know from your years of autocrossing, one of the important

weight distribution ratios (especially for effect on handling) is the LF/RR to

RF/LR ratio ('squareness') - The Impala SS corner ratios are as follows: LF/RR

2239 and RF/LR 2237. This is astonishing 'square' and contributes greatly to

the amazing handling (all things considered) of the beast...

While I've got you here, what tire pressures do you use for autocross

(assuming that you're running stock tires)? I've seen as high as 45 psi

recommended, but I usually run around 33...

Tom 94 Stock (yeah, right) No, Honestly it is, I promise...

------------------------------

Date: Thu, 29 Feb 1996 15:55:03 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: rear bar height

Message-ID: <960229235502_73145.1566_FHD58-2@CompuServe.COM>

Martin,

>I am contemplating the Eibach/Suspension Techniques springs. I am

concerned, however, about placing that rear bar any closer to the ground; it

already drags occasionally in some cases with stock height.

The swaybar height off of the ground is not a factor of the springs at all,

in fact that is entirely controlled by wheel and tire diameter. Whether you

run tall, short, or no springs at all have no effect on where that bar is in

relation to the ground. Scott.

------------------------------

Date: Thu, 29 Feb 1996 17:23:12 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: Bilstein Shock Part # site

Message-ID: <Pine.SOL.3.91.960229172118.28091A@netinfo.ubc.ca>

Hi:

Just found this site which will give you the parts number of

Bilstein Shocks for every car.  Although admitedly, their price aren't

the best.  Still worth while to check it out.

http://catalog.com/susq/bcars.htm

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red

9C1 Air deflactors, Gas Bib

------------------------------

Date: Thu, 29 Feb 1996 18:25:57 PST

From: BGordon401@aol.com

To: <impala@east.xsis.xerox.com>

Message-ID: <199603010225.VAA05918@emout05.mail.aol.com>

my .02 worth about the "issue" on being personal.

Like Troy, I need 24 hours to digest information before passing judgement.

I was ALWAYS UNDER THE IMPRESSION THAT YOU CONFIDE IN FRIENDS about important

issues in your life??  We are all friends here, right??

Talking about our cars is great, no doubt, but there are personalities behind

them---primarily ourselves.  One goes with the other.

BTW Scott, you were NOT our of line, and there is no need to apologize to us.

THANK YOU for giving us a chance to listen!!

I feel better now.

As far as the 10 or 20 dollar amount for the Impala SS Club newsletter, let's

all pay and do our fair share.  What does this money boil down to???  Doing

without ONE extra case of beer for the year.  Come on, you can save much more

than this by doing one mod with the vendors listed in the newsletter.

Once again, my .02 worth

..Bruce

------------------------------

Date: Thu, 29 Feb 1996 20:46:54 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 314

Message-ID: <960229234653_234851690@mail04.mail.aol.com>

If you have the SunGuard windshield, how *exactly* can you tell?

How does the lower passenger side glass read, *exactly* ?

My 1995 Monte Carlo Z34 says "shaded", but I don't believe this to mean

"SunGuard".

Please help me!!

Randy

------------------------------

Date: Thu, 29 Feb 1996 22:16:29 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Searchable Impala Archives

Message-ID: <1.5.4b11.32.19960301061629.006b6ef4@mail.one.net>

>Speaking of which, has anyone looked into the possiblity of adding some

>sort of search mechanism on one of the Impala websites for the archives?

>Maybe something like Yahoo or Lycos where you could just type in a keyword?

>Dave, Rob how difficult would this be? Thanks.

It would be very difficult for me to do it from one.net, as the archives

are stored on Daves machine. I'd be willing to code up the HTML/CGI to

do it, though, if Dave is willing to host it but doesn't have time to. Dave?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Fri, 1 Mar 1996 02:47:33 PST

From: "David P. Fenzl" <dfnzl@erols.com>

To: <impala@east.xsis.xerox.com>

Subject: Bucket Seats

Message-ID: <01BB0732.9A576A80@ppp39.erols.com>

<....bucket seats and a console with the shift lever on the column????  ALL

94-95 Impalas are that way!! >

I appreciate Andy Cohen, Steve Das, Frank Poole, Scott Harris, and others

setting me straight on front seat/shift lever configurations.  That's why I

read the Digest every day -- to learn.  Somehow I had the impression that

'94s and '95s had bench front seats.  The bum dope was planted in my brain

by a less-than-knowledgeable salesman who said that "console, shifter, and

bucket seats are new for  '96".  My misperception was reinforced by John P.

in Digest 260, who wrote that the Impala is a "4-door 6-passenger" car,

while the Maxima SE is a "4-door 5-passenger" car.  (Remember the tirade

that was set off a few weeks ago by Fred and his Maxima?)  I guess John P.

carries around four real skinny people in his back seat, or else two very

close friends in the right front passenger seat.

<If the shifter is your concern, only 96's have floor console shifters.

But we have cup holders.>

Scott, are you pulling my leg?  My '96 has a cup holder.

Dave F., DGGM (holds driver plus 4 pax) w/bucket seats, floor shifter, and

cup holder

dfnzl@erols.com

------------------------------

Date: Fri, 1 Mar 1996 04:13:02 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: Impala SS <impala@east.xsis.xerox.com>

Subject: No Holds Barred Road Race?

Message-ID: <Chameleon.960301072340.hollenbe@mustang.east.xsis.xerox.com>

The April 1996 issue of Mustang Monthly (no snide remarks,

please -- I'm also a Classic Mustang fanatic) includes an

interesting editorial description of an event that is supposedly

taking place later this year:

"Here's the deal.  Somewhere in southeast Montana (we are reluctant

to divulge the exact location) on April 1, there is rumored to be

a clandestine gathering of the country's top high-performance pony

cars -- Mustangs, Camaros, and Firebirds.  The venue: the open stretches

of the Big Sky Country.  The objective: survive a no-holds-barred, no

speed limits enforced round-trip rally from Montana's eastern plains

through the Rocky Mountains.  The prize: bragging righst the likes of

which this sport has never seen.  The first challenger who returns

to the starting point without careening over a mountainside takes

top dog honors."

Remember that Montana recently eliminated speed limits on many (if not

all) rural highways.

Supposedly this competition if by word of mouth invitation.  The editor

did not divulge his source, but suggested that word would somehow get

around if you are considered "worthy".  I would hope that some Impala

driver could be considered "worthy".

Has anyone else heard of this?  Sounds like it would be a blast to drive

in or watch.

==================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street, Lower Lobby, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

==================================================

------------------------------

Date: Fri, 1 Mar 1996 02:33:44 PST

From: Stephen Furr <skfurr@raleigh.ibm.com>

To: impala <impala@east.xsis.xerox.com>

Subject: Hello to all!

Message-ID: <9603011533.AA0994@rtpnsi01.raleigh.ibm.com>

I've tried to avoid the common newbie pitfalls, by:

1)  Reading the FAQ, and

2)  Reading all (phew!) of the archives.

I'm fascinated by the incredible amount of interest, and

the high S/N ratio on this list.

My name is Stephen Furr.  I live in Raleigh, N.C.

I am 25, and work for IBM.  I just ordered my '96 Black

Impala SS, and am anxiously awaiting delivery.

Now my question:

The dealer that I ordered from sold me the car for exactly

$800 off of sticker.  From reading the archive, this may

or may not have been a good deal.  Seeing as nobody in

the area has a car in stock, I'm figuring that I did pretty

good.  The car that I ordered was previously ordered as

part of the dealers allocation on November 3.  The

car has yet to be preferenced.  When I placed my order

(this week,) the dealer said that he would update the

allocation order to "sold vehicle" status and that this

would cut my delivery time to under 4 weeks.  From

reading the archive, it seems that I'm looking at least

eight weeks, since the order hasn't been preferenced

yet.  The dealer is also willing to "stock-transfer" one

for me, but does not know where one is.

Can anyone help?  This really bites.  If anyone knows

where I can find a car like the one I ordered, I'd be

eternally grateful!

1996 Chevrolet Impala SS

41U Black

1SE Preferred Equipment Group

UN0 CD Player

A23 Heat reflective windshield

I'd prefer NOT to have VK3 (Front License Plate Bracket)

as in N.C. we don't use front plates, and I don't want holes

drilled in my bumper!

Thanks again for any suggestions.

Stephen K. Furr

------------------------------

End of IMPALA Digest 316

************************

IMPALA Digest 317

Topics covered in this issue include:

1) Heat Reflective Windshield

by ZTMP50A@prodigy.com ( DAVID NMN)

2) Airbox enhancements

by ZTMP50A@prodigy.com ( DAVID NMN)

3) LT-4 Heads

by Godley@aol.com

4) Introduction

by Erik - Bertrand <bertran5@mulder.egr.msu.edu>

5) Monthly How-To: Please Read and Save

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

6) '96 purchase

by Scott Mueller <73145.1566@compuserve.com>

7) Valvoline Duraguard etc. etc. etc.

by Loren Peterson <equ_rino@cencom.net>

8) Wholesale pricing

by Loren Peterson <equ_rino@cencom.net>

9) 2nd Gear Start

by Scott Mueller <73145.1566@compuserve.com>

10) Re: IMPALA digest 316

by John Mackanics <jmack@chelsea.ios.com>

----------------------------------------------------------------------

Date: Thu, 29 Feb 1996 10:37:00 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: Heat Reflective Windshield

Message-ID: <097.02493160.ZTMP50A@prodigy.com>

Randy, the terminolgy for the reflective windshield says : "Sungate

Shaded Solar Ray" on the passenger side.

Hope this helps!

Dave in So Cal..'96 Black SS

------------------------------

Date: Thu, 29 Feb 1996 10:40:39 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: Airbox enhancements

Message-ID: <097.02493358.ZTMP50A@prodigy.com>

Hi All.  I was wondering if all the adjustments to the airbox, baffle,

and home plate were worth the trouble in performance gains.  I know

cleaning the engine compartment up is cool with the addition of the

Corvette pieces, but to cut the airbox, etc, is there realized gains??

Has anyone dyno'd the car before and after?  I know the benefits of

cold air for increase in power, but does it show with these changes?

Thanks!

Dave in So Cal, '96 Black SS, tinted windows so far.

------------------------------

Date: Fri, 1 Mar 1996 10:58:30 PST

From: Godley@aol.com

To: impala@east.xsis.xerox.com

Subject: LT-4 Heads

Message-ID: <960301135828_337902874@emout04.mail.aol.com>

This message was relayed to me via my Tuner

Doug Rippie is the one that said, to a personal friend of mine, that the LT-4

heads would decrease the power if you did not change the cam accordingly

(what profile of cam I do not know) on the Impala SS the increased flow would

decrease the midrange torque and slow the cars down and that portingthe stock

cast iron cylinder heads was "the way to go" (DR).  What he means is more

volume lowers air velocity at lower RPM's thereby reducing torque.  To get

the full effectiveness of the increase in flow you need to turn more RPM's

(another benefit is the 40 or so lbs. of weight taken off the car plus the

increased thermal transference of aluminum.)  Remember the LT-4 only makes

330 net HP.  Not very impressive for "the only heads to run"  Don't get me

wrong, I do think the LT-4 heads are "kickin" but when they costs $900.00

plus not including port work or parts eg., valves, springs, etc., for minimal

gains over a stock set or worked heads I don't think it's worth it.  Remember

the formula for torque is: the smallest amount of area with the highest

amount of flow.  Chevrolet has used this formula quite well on 1985-1992

Tuned Port Injected Engines 305 CID =330Ft.Lbs, 92F-Body 350= 350 Ft.Lbs,

etc.  Torque is how much work gets done, horsepower is how fast it gets done.

Tipping the scales at 4400Lbs. with driver SHAMU needs truck fulls of torque

to get her moving.  This is something it already has.  As the work load

decreases (the weight needed to be moved is reduced), the amount of torque

needed to move it also decreases as is the case of the 3300 lb Vette, 3400 lb

Z-28 or the nimble 3000 LX mustang.

"The Hammer R.V."

MARK

95 DCM IMPALA

------------------------------

Date: Fri, 1 Mar 1996 11:25:10 PST

From: Erik - Bertrand <bertran5@mulder.egr.msu.edu>

To: impala@east.xsis.xerox.com

Subject: Introduction

Message-ID: <Pine.SOL.3.91.960301141640.1346A-100000@mulder.egr.msu.edu>

Hey Guys,

I forgot to introduce myself the first time

I wrote it to the bulletin board.  So I will do it now,

Name:		Erik Bertrand

Age:		29

Occupation:	Full time masters student (x chevy mechanic and

GM engineer)

Car:		95 Dark Cherry With the usual options

9C1 rims and tires

Real oil gauge

K&N air filter

Fabricated 3.5" stainless induction system

(aka no resonators)

Hobbies:	Wife, Car, Hockey, Drag Racing

(not necessairly in that order)

Location:	Lansing, Michigan.

That's a quick intro to me, now where do I send my dues?

I know I will get a response from that one!

------------------------------

Date: Fri, 1 Mar 1996 12:09:56 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: Impala SS <impala@east.xsis.xerox.com>

Subject: Monthly How-To: Please Read and Save

Message-ID: <Chameleon.960301151035.hollenbe@mustang.east.xsis.xerox.com>

The impala list is managed by an automated list processor program.

All interaction with the list processor can be acomplished by sending it e-mail

messages.  It's e-mail address is "listproc@east.xsis.xerox.com".  When sending

a message to listproc, you should ALWAYS leave the subject field of your note

blank!  listproc will reject messages with what it thinks are suspicious

subjects, so leaving the field blank is ALWAYS safe.  If you can't leave the

subject blank it's OK to use some harmless character like a period.

The e-mail address for impala is "impala@east.xsis.xerox.com".  Please do NOT

send admin requests to the group, because most people don't care if you wish

to subscribe, unsubscribe, or whatever.  Messages sent to impala should

include a subject.

Messages for the list are distributed in digest mode only.  That means messages

are collected and formatted as they are received.  Digests are sent when 500

lines of text have been received or 24 hours has past since the last digest

was sent.  Live mail delivery is no longer an option -- when mail delivery

problems occur (believe me, they occur OFTEN when you're trying to send mail

to some 300 people) they cause messages to sit in my corporate gateway,

consuming processing cycles every so often in redelivery attempts.  After

a while the gateway was getting clogged with messages waiting for redelivery,

which got in the way of delivering business mail, which got my gateway

administrators attention, which got my hand slapped.  So, this is the best

I can do from within Xerox.

When responding to a message, do NOT include ANY of the mail headers from the

original message in your response if you send it to the list.  The list

processor will reject messages that include certain headers in the message body

as part of a loop detection mechanism.  The message ID and listprocessor ID

fields seem to be included in many of the messages I've seen rejected by

listproc.

Common commands:

To unsubscribe:

unsubscribe impala

(Note that you do NOT include any form of your name with this command!)

To receive the FAQ:

get listproc/impala impala-faq

(Actually, this command can be used to retrieve any file from the archive.

Just replace "impala-faq" with the name of the file you want and you'll be

OK)

To see what's in the archive:

index impala

To receive a list of subscribers:

review impala

As always, if you have a question you can usually have it answered pretty

quickly by dropping me a mail note.  Please don't send a note to the whole list

asking questions like "is the list working?"

I receive "cc" copies of almost all error messages created when mail can't be

delivered to you for whatever reason.  I usually ignore the first such error,

because network problems are often resolved in a short amount of time.

However, if I receive a flood of messages over a day or two that tells me

there's a problem with you receiving mail, I will temporarily postpone your

subscription.  This prevents mail from being delivered to you, but maintains

your subscription.  If you think you've been postponed, send this message to

listproc:

set impala mail digest

Remember, you can always set your mail preferences at any time should you wish

to change from postpone to digest format or vice-versa.

Some mail delivery errors will cause you to be automatically removed from the

list.  If this happens, you must resubscribe as follows:

subscribe impala <Your Name>

where <Your Name> should be replaced with your REAL name, not your e-mail

address.  For example, a person named Louis Chevrolet would subscribe like this:

subscribe impala Louis Chevrolet

Finally, a few words about what is acceptable message content and what

isn't.  The list is here for discussions concerning Chevrolet Impalas,

both late model and early model, though the current membership is

mostly late model enthusiasts.  All topics related to the cars are welcome.

Profanity in any form is not acceptable.  Personal attacks are not

acceptable.  I realize we will not always agree on all points, but this

is intended to be an open forum where people can ask questions and

express opinions without being attacked by other list members.  I will

remind individuals of this policy the first time I notice something

offensive.  Repeated problems after being reminded will result in

revocation of your list subscription.

Until next month,

Scott Hollenbeck

=========================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street

Lower Level, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

=========================================================

------------------------------

Date: Fri, 1 Mar 1996 12:37:02 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: '96 purchase

Message-ID: <960301203701_73145.1566_FHD39-1@CompuServe.COM>

Stephen,

>I just ordered my '96 Black Impala SS, and am anxiously awaiting delivery.

>The dealer that I ordered from sold me the car for exactly $800 off of

sticker.  From reading the archive, this may or may not have been a good

deal.

If you would like to pusave approximately an additional $1,000 then you drop

your order with the dealer and call Tarry or Jennifer at ACS (800-223-4882,

tell them I sent you). You can order an Impala for a few hundred over

invoice through them (which is close to $2,000 below sticker) and even have

it shipped to your current dealer for delivery! In fact, you can request a

printout of the deal from ACS, and take that back to your dealer and tell

them to match it or you'll just order it through them. Either way, you will

get the car for the ACS price.

BTW, Jeff S. here on the list just bought an Impala through ACS, my cousin

in CA has a '96 on order, plus I have purchased a '95 Trans Sport and have a

'96 Jeep GC on order through them. They are great to deal with, and I'm sure

Jeff S. will add his comments to the same effect.

>I'd prefer NOT to have VK3 (Front License Plate Bracket) as in N.C. we

don't use front plates, and I don't want holes drilled in my bumper!

VK3 is not an orderable or non-orderable option, it is an automatic option

that is either included on a state by state basis, or in fact it may be

included on all Impalas. Fortunately, the front license plate bracket is

always shipped in the trunk, and must be dealer installed. Your mission is

to stop the guy who preps the new cars at the dealer from drilling the nose

of your incoming Impala and installing the bracket. As delivered, the Impala

has no holes in the front bumper for the bracket, however there is one

dimple to guide the placement of the drill bit when the holes are drilled by

the dealer.

When I bought my car, I told the dealer that he/she had better not install

that bracket (which requires drilling holes in the nose of the Impala!) or I

will walk. Here in IL we are supposed to have front license plates, but I

have not had them on several of my cars for many years now. Several dealers

have told me that the bracket comes already installed (WRONG!), or that they

must install it under IL state law (also WRONG, that is the vehicle owner's

responsiblity, not the dealers). I simply repeat my statement, "Drill a

single hole in my car, install that bracket, and I will walk, it is your

choice." My rule is that if holes must be drilled, the plate stays off. The

Impala front end looks so smooth without that ugly plate bracket spoiling

it. Scott.

------------------------------

Date: Fri, 1 Mar 1996 12:47:19 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Valvoline Duraguard etc. etc. etc.

Message-ID: <31376257.63B2@ns.cencom.net>

Greetings to fellow members of the best list...bar none,

I have always used name brand oil in my cars (non synthetic). I change

around 2,500 mi.  Never really saw the need for synthetic.  When

Valvoline came out with the semi-synthetic I decided to use it in my new

V10 Ram, and now in my '96 Impala.  If I believe what I have read, it is

kind of best of both worlds.  Benefits of synthetic but cost not too

much more $ than reg. oil.  I have not put any additive in the oil and

of course with under 5,000 wouldnt want to anyway.  Is there an additive

out there that is worth considering?  I know the O.M. says NO NO NO.

Any comments would be appreciated.  I would consider switching to Total

synthetic if someone could show me the BIG benefit.

Also a question was asked recently about wax and it was not talked of

much.  I use Durashine cause I like the sheeting...yes SHEETING action

because it doesnt seem to leave beads which collect dirt and then dry to

spots.  Its not petro based, and I like that too.  Any comments?

Also, I helped organize the worlds first F%#@ Taurus SHO convention in

Peoria, IL a few years back.  We had a rep from Amoco speak to us.  I

could or maybe someone else has better connections, but he put on a

super seminar.  Anyway, do we want him to speak to us during convention,

or are we doing any of that.  We had a series of seminars which factory

reps etc. spoke.  Just wondering out loud.

Lastly..........Good luck and God speed the Wifes recovery....you know

who I'm talking to.  99.9% are for ya, and in this day in age, that aint

bad.   I think the .1% arent against ya, they just care for numero uno.

nuff said.

Loren DCM 96

p.s. I am going to have my custom decal guy make me us a set of small

fender decals for the Arlington Show.....SP Equipped.......ie Sewer Pipe

Anyone wanting decals for any reason or purpose let me know and I can

get them for you.  He can scan anything on paper to make 'em.

TEXAS OR BUST

------------------------------

Date: Fri, 1 Mar 1996 12:54:55 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Wholesale pricing

Message-ID: <3137641F.1F0F@ns.cencom.net>

SSers

One more thing, my local Chevy Dealer just signed me up for wholesale

pricing.  I gave him my IL tax #.  If there is anyone around here (IA

IL) that needs anything and arent getting a price break, let me know and

I will get it at my cost, whatever it is.  Also are there any Impala

owners in Western Iowa or central IL that would want to get together?

Any convoys to TEXAS?  Any X SHO Owners that came to Peoria Convention?

Loren DCM96 SPE

Lets get sideways

------------------------------

Date: Fri, 1 Mar 1996 15:06:09 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: 2nd Gear Start

Message-ID: <960301230609_73145.1566_FHD23-3@CompuServe.COM>

Scott,

>Rob says his car used to start in second gear if the selector was in the 2

position, like a Ford.  This must be a 94 only calibration since my 95 and

all the other 95 & 96's I've driven start in first and upshift to second

like a GM.  Do you know if this is correct?  Are 94's different? Was it in

only "some" 94's?

According to the '94, '95, and '96 service manuals, as well as the

Hydra-Matic 4L60-E Technician's Guide, ALL 4L60-E transmissions feature 2nd

gear start! In the Tech. Guide, it states under Explanation of Gear Ranges;

"2 - Manual Second gear range prevents the transmission from operating in

any gear other than second. This allows the transmission to start moving the

vehicle in second gear which may be helpful under slippery conditions such

as snow or ice. Manual Second also provides additional engine compression

braking by not allowing the transmission to shift above second gear. Note:

Some vehicle applications will downshift into first gear when the

transmission is in Manual Second under heavy throttle conditions at low

speed."

I can verify that my '94 does start and remain in 2nd gear when in Manual

Second range. According to all of the service and technical literature, all

4L60-E transmissions should operate this way, but maybe there was a design

change for '95? Can everybody with a '95 or '96 verify whether their Impala

starts in 2nd Gear when the shifter is in the Manual 2nd range? Scott.

------------------------------

Date: Fri, 1 Mar 1996 15:09:20 PST

From: John Mackanics <jmack@chelsea.ios.com>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 316

Message-ID: <313783A0.3421@chelsea.ios.com>

impala@east.xsis.xerox.com wrote:

>

>                             IMPALA Digest 316

>

> Topics covered in this issue include:

>

>   1) Re: Scott Mueller's personal post

>         by Kurt Kemmling <76165.1654@compuserve.com>

>   2) Re: Dues Increase

>         by Kurt Kemmling <76165.1654@compuserve.com>

>   3) IRC

>         by bngwip@mail.phoenix.net (Brett A. Rudolph)

>   4) RE: IMPALA digest 311

>         by "Thomas L. Pierce" <TLPierce@msn.com>

>   5) rear bar height

>         by Scott Mueller <73145.1566@compuserve.com>

>   6) Bilstein Shock Part # site

>         by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

>   7)

>         by BGordon401@aol.com

>   8) Re: IMPALA digest 314

>         by Mikado95@aol.com

>   9) Searchable Impala Archives

>         by Robert Cheek <rcheek@one.net>

>  10) Bucket Seats

>         by "David P. Fenzl" <dfnzl@erols.com>

>  11) No Holds Barred Road Race?

>         by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

>  12) Hello to all!

>         by Stephen Furr <skfurr@raleigh.ibm.com>

>

> ----------------------------------------------------------------------

>

> Date: Thu, 29 Feb 1996 10:19:17 PST

> From: Kurt Kemmling <76165.1654@compuserve.com>

> To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

> Subject: Re: Scott Mueller's personal post

> Message-ID: <960229181916_76165.1654_GHM51-4@CompuServe.COM>

>

> In digest 312 Scott Mueller writes:

>

> >>>Yes, I apologize if the personal info was out of line, but as you indicated

> the main reason I posted it was to explain the absence of my car at the

> Chevy-Vettefest. I could have just said "medical problems", but that might

> have raised additional questions and besides it is just my nature to be

> technically more detailed than that! Most people might not have said

> anything at all, but I feel that we have all become friends here, much more

> so than on other lists I have been on. Scott.<<<

>

> Scott,

>

> No need to apologize. The wacker that wrote the flame probably

> still hasn't paid his dues, so take it FWIW.<BG>

> BTW, I agree with you about all of becoming friends, not just

> anonymous members of a mailing list. You have contributed

> more to this club than most of us ever will, myself included.

> I hope your wife is doing better, and also hope you will tell

> us when things like this happen. If the individual that objected

> to your post has a problem with this, I have a simple solution:

>

> ==========UNSUBSCRIBE===================

>

> Take care Scott,

>

> Kurt

>

> '95 Dark Cherry Impala SS.....no mods........yet......

>

> ------------------------------

>

> Date: Thu, 29 Feb 1996 10:19:08 PST

> From: Kurt Kemmling <76165.1654@compuserve.com>

> To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

> Cc: Troy Willrick <Rock528@aol.com>

> Subject: Re: Dues Increase

> Message-ID: <960229181908_76165.1654_GHM51-1@CompuServe.COM>

>

> In digest 312 Troy writes:

>

> >>>I don't mean to be greedy but I had to raise it to $20.00 annually back in

> January.  State and federal trademark fees, bank accts., phone line, monthly

> mailbox service, tax license, county occupational license, and a 10 page

> newsletter now done on a printing press.<<<

>

> Troy,

>

> You don't have to justify the price increase. I paid my 10 bucks as soon as

> I found this list, and always thought it was too low. Even at $20.00, those of

> us that are true members can see the value without making you feel like

> you have to explain it to us.

> You are obviously a very generous and sincere individual, and just want

> to make everyone feel like they are getting their money's worth. I and many

> others on this list know how much time and effort you have and are putting

> into keeping this club together. There will always be freeloaders that will

> complain about things, even when they haven't paid a thin dime, and you

> can't please them no matter what Troy. Just keep doing what you are

> doing. The dead wood will eventually just float away, and this club will

> contain only those willing to support and contribute to it's betterment.

>

> Thank you Troy for a great job.

>

> Kurt

>

> '95 Dark Cherry Impala SS.....no mods........yet......

>

> ------------------------------

>

> Date: Thu, 29 Feb 1996 14:26:49 PST

> From: bngwip@mail.phoenix.net (Brett A. Rudolph)

> To: impala@east.xsis.xerox.com

> Subject: IRC

> Message-ID: <Chameleon.960229143023.bngwip@dialin.phoenix.net>

>

> Do any of you guys ever talk on irc or aol? If anyone is interested,we should

> get together and do a live ImpalaSS channel chat thing sometime. and discuss

> Arlington or Super Chevy or Jeremy's lack of good sense or what ever. Just a

> thought.

>

>                                        Brett

>

> ------------------------------

>

> Date: Wed, 28 Feb 1996 13:41:06 PST

> From: "Thomas L. Pierce" <TLPierce@msn.com>

> To: impala@east.xsis.xerox.com

> Subject: RE: IMPALA digest 311

> Message-ID: <UPMAIL09.199602292252090686@msn.com>

>

> Tim M. Killmeyer sez:

>

>         >>I've always told people that they should take their car...and autocross it

> just to see how it handles. You can learn a lot about what a

> car will do in tricky situations on the road.

>

>         Amen to that, Tim - it's suprising how many people don't know the limits of

> the vehicles they are driving.

>

> >>>Question: I've seen a couple of HIGHLY different numbers with regards to

> the Impala's front to rear weight ratio.

>

> Last time I was at the autocross, I had the beast corner weighed - here are

> the numbers (with a 220 pounder in the drivers seat)

>

> Left Front 1246 Right Front 1188

>

> Left Rear  1049 Right Rear 993

>

> Front Total 2334        Total Rear 2042

>

> Total Weight    4376

>

> Front to Rear Ratio     53:47

>

> As I'm sure you know from your years of autocrossing, one of the important

> weight distribution ratios (especially for effect on handling) is the LF/RR to

> RF/LR ratio ('squareness') - The Impala SS corner ratios are as follows: LF/RR

> 2239 and RF/LR 2237. This is astonishing 'square' and contributes greatly to

> the amazing handling (all things considered) of the beast...

>

> While I've got you here, what tire pressures do you use for autocross

> (assuming that you're running stock tires)? I've seen as high as 45 psi

> recommended, but I usually run around 33...

>

> Tom 94 Stock (yeah, right) No, Honestly it is, I promise...

>

> ------------------------------

>

> Date: Thu, 29 Feb 1996 15:55:03 PST

> From: Scott Mueller <73145.1566@compuserve.com>

> To: <impala@east.xsis.xerox.com>

> Subject: rear bar height

> Message-ID: <960229235502_73145.1566_FHD58-2@CompuServe.COM>

>

> Martin,

>

> >I am contemplating the Eibach/Suspension Techniques springs. I am

> concerned, however, about placing that rear bar any closer to the ground; it

> already drags occasionally in some cases with stock height.

>

> The swaybar height off of the ground is not a factor of the springs at all,

> in fact that is entirely controlled by wheel and tire diameter. Whether you

> run tall, short, or no springs at all have no effect on where that bar is in

> relation to the ground. Scott.

>

>

> ------------------------------

>

> Date: Thu, 29 Feb 1996 17:23:12 PST

> From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

> To: Impala SS Mailing List <impala@east.xsis.xerox.com>

> Subject: Bilstein Shock Part # site

> Message-ID: <Pine.SOL.3.91.960229172118.28091A@netinfo.ubc.ca>

>

> Hi:

>

>         Just found this site which will give you the parts number of

> Bilstein Shocks for every car.  Although admitedly, their price aren't

> the best.  Still worth while to check it out.

>

>         http://catalog.com/susq/bcars.htm

>

>        Have a nice day!

> Mike Yang

> University of British Columbia

> 1996 Impala SS Dark Cherry Red

> 9C1 Air deflactors, Gas Bib

>

> ------------------------------

>

> Date: Thu, 29 Feb 1996 18:25:57 PST

> From: BGordon401@aol.com

> To: <impala@east.xsis.xerox.com>

> Message-ID: <199603010225.VAA05918@emout05.mail.aol.com>

>

>  my .02 worth about the "issue" on being personal.

>  Like Troy, I need 24 hours to digest information before passing judgement.

>

> I was ALWAYS UNDER THE IMPRESSION THAT YOU CONFIDE IN FRIENDS about important

> issues in your life??  We are all friends here, right??

>

> Talking about our cars is great, no doubt, but there are personalities behind

> them---primarily ourselves.  One goes with the other.

>

> BTW Scott, you were NOT our of line, and there is no need to apologize to us.

>  THANK YOU for giving us a chance to listen!!

>

> I feel better now.

>

> As far as the 10 or 20 dollar amount for the Impala SS Club newsletter, let's

> all pay and do our fair share.  What does this money boil down to???  Doing

> without ONE extra case of beer for the year.  Come on, you can save much more

> than this by doing one mod with the vendors listed in the newsletter.

>

> Once again, my .02 worth

>

> ..Bruce

>

> ------------------------------

>

> Date: Thu, 29 Feb 1996 20:46:54 PST

> From: Mikado95@aol.com

> To: impala@east.xsis.xerox.com

> Subject: Re: IMPALA digest 314

> Message-ID: <960229234653_234851690@mail04.mail.aol.com>

>

> If you have the SunGuard windshield, how *exactly* can you tell?

> How does the lower passenger side glass read, *exactly* ?

> My 1995 Monte Carlo Z34 says "shaded", but I don't believe this to mean

> "SunGuard".

> Please help me!!

>

> Randy

>

> ------------------------------

>

> Date: Thu, 29 Feb 1996 22:16:29 PST

> From: Robert Cheek <rcheek@one.net>

> To: impala@east.xsis.xerox.com

> Subject: Searchable Impala Archives

> Message-ID: <1.5.4b11.32.19960301061629.006b6ef4@mail.one.net>

>

> >Speaking of which, has anyone looked into the possiblity of adding some

> >sort of search mechanism on one of the Impala websites for the archives?

> >Maybe something like Yahoo or Lycos where you could just type in a keyword?

> >Dave, Rob how difficult would this be? Thanks.

>

> It would be very difficult for me to do it from one.net, as the archives

> are stored on Daves machine. I'd be willing to code up the HTML/CGI to

> do it, though, if Dave is willing to host it but doesn't have time to. Dave?

> --

> Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

> 1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

> "Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

>

> ------------------------------

>

> Date: Fri, 1 Mar 1996 02:47:33 PST

> From: "David P. Fenzl" <dfnzl@erols.com>

> To: <impala@east.xsis.xerox.com>

> Subject: Bucket Seats

> Message-ID: <01BB0732.9A576A80@ppp39.erols.com>

>

> <....bucket seats and a console with the shift lever on the column????  ALL

> 94-95 Impalas are that way!! >

>         I appreciate Andy Cohen, Steve Das, Frank Poole, Scott Harris, and others

> setting me straight on front seat/shift lever configurations.  That's why I

> read the Digest every day -- to learn.  Somehow I had the impression that

> '94s and '95s had bench front seats.  The bum dope was planted in my brain

> by a less-than-knowledgeable salesman who said that "console, shifter, and

> bucket seats are new for  '96".  My misperception was reinforced by John P.

> in Digest 260, who wrote that the Impala is a "4-door 6-passenger" car,

> while the Maxima SE is a "4-door 5-passenger" car.  (Remember the tirade

> that was set off a few weeks ago by Fred and his Maxima?)  I guess John P.

> carries around four real skinny people in his back seat, or else two very

> close friends in the right front passenger seat.

>         <If the shifter is your concern, only 96's have floor console shifters.

>  But we have cup holders.>

>         Scott, are you pulling my leg?  My '96 has a cup holder.

>

> Dave F., DGGM (holds driver plus 4 pax) w/bucket seats, floor shifter, and

> cup holder

> dfnzl@erols.com

>

> ------------------------------

>

> Date: Fri, 1 Mar 1996 04:13:02 PST

> From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

> To: Impala SS <impala@east.xsis.xerox.com>

> Subject: No Holds Barred Road Race?

> Message-ID: <Chameleon.960301072340.hollenbe@mustang.east.xsis.xerox.com>

>

> The April 1996 issue of Mustang Monthly (no snide remarks,

> please -- I'm also a Classic Mustang fanatic) includes an

> interesting editorial description of an event that is supposedly

> taking place later this year:

>

> "Here's the deal.  Somewhere in southeast Montana (we are reluctant

> to divulge the exact location) on April 1, there is rumored to be

> a clandestine gathering of the country's top high-performance pony

> cars -- Mustangs, Camaros, and Firebirds.  The venue: the open stretches

> of the Big Sky Country.  The objective: survive a no-holds-barred, no

> speed limits enforced round-trip rally from Montana's eastern plains

> through the Rocky Mountains.  The prize: bragging righst the likes of

> which this sport has never seen.  The first challenger who returns

> to the starting point without careening over a mountainside takes

> top dog honors."

>

> Remember that Montana recently eliminated speed limits on many (if not

> all) rural highways.

>

> Supposedly this competition if by word of mouth invitation.  The editor

> did not divulge his source, but suggested that word would somehow get

> around if you are considered "worthy".  I would hope that some Impala

> driver could be considered "worthy".

>

> Has anyone else heard of this?  Sounds like it would be a blast to drive

> in or watch.

>

> ==================================================

> Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

> Manager, Software Systems Engineering

> Xerox Special Information Systems

> 1901 North Moore Street, Lower Lobby, Suite 5

> Arlington, Virginia, USA 22209

> (703) 284-3865

> ==================================================

>

> ------------------------------

>

> Date: Fri, 1 Mar 1996 02:33:44 PST

> From: Stephen Furr <skfurr@raleigh.ibm.com>

> To: impala <impala@east.xsis.xerox.com>

> Subject: Hello to all!

> Message-ID: <9603011533.AA0994@rtpnsi01.raleigh.ibm.com>

>

> I've tried to avoid the common newbie pitfalls, by:

>

> 1)  Reading the FAQ, and

> 2)  Reading all (phew!) of the archives.

>

> I'm fascinated by the incredible amount of interest, and

> the high S/N ratio on this list.

>

> My name is Stephen Furr.  I live in Raleigh, N.C.

> I am 25, and work for IBM.  I just ordered my '96 Black

> Impala SS, and am anxiously awaiting delivery.

>

> Now my question:

>

> The dealer that I ordered from sold me the car for exactly

> $800 off of sticker.  From reading the archive, this may

> or may not have been a good deal.  Seeing as nobody in

> the area has a car in stock, I'm figuring that I did pretty

> good.  The car that I ordered was previously ordered as

> part of the dealers allocation on November 3.  The

> car has yet to be preferenced.  When I placed my order

> (this week,) the dealer said that he would update the

> allocation order to "sold vehicle" status and that this

> would cut my delivery time to under 4 weeks.  From

> reading the archive, it seems that I'm looking at least

> eight weeks, since the order hasn't been preferenced

> yet.  The dealer is also willing to "stock-transfer" one

> for me, but does not know where one is.

>

> Can anyone help?  This really bites.  If anyone knows

> where I can find a car like the one I ordered, I'd be

> eternally grateful!

>

> 1996 Chevrolet Impala SS

> 41U Black

> 1SE Preferred Equipment Group

> UN0 CD Player

> A23 Heat reflective windshield

>

> I'd prefer NOT to have VK3 (Front License Plate Bracket)

> as in N.C. we don't use front plates, and I don't want holes

> drilled in my bumper!

>

> Thanks again for any suggestions.

>

> Stephen K. Furr

>

> ------------------------------

>

> End of IMPALA Digest 316

> ************************Stephen

The dealers in New Jersey in my area have some Impalas in stock. They

all have the front plate though and you can expect $500-750 over

invoice. My 96 was ordered on Dec 5 and came in today. 3-1-96

John

------------------------------

End of IMPALA Digest 317

************************

IMPALA Digest 318

Topics covered in this issue include:

1) Not me, no way

by Loren Peterson <equ_rino@cencom.net>

2) 1.6 rockers

by Robert Cheek <rcheek@one.net>

3) 1965 Ford Wagon

by ajcohen@li.net (Andrew Cohen)

4) Re: IMPALA digest 316

by Darren Buck <dwb0522@okway.okstate.edu>

5) PCM

by Stinkyg@aol.com

6) Another Digest quoter!

by Ray Berilla <writes4u@ganet.net>

7) Front Bumper Holes

by BGordon401@aol.com

8) Re: 2cd Gear Starts

by Kurt Kemmling <76165.1654@compuserve.com>

9)

by dhflores@iwl.net (Debbie/Henry Flores)

10) Re: IMPALA digest 316

by "Tim M. Killmeyer" <74144.103@compuserve.com>

11) Impala purchase

by Franklin Poole <franklin@intersurf.com>

12) Airbox cutting, homeplate removal, etc.

by Franklin Poole <franklin@intersurf.com>

13) Impala Prices

by Franklin Poole <franklin@intersurf.com>

14) is my car stuck in performance mode??

by Franklin Poole <franklin@intersurf.com>

15) Supercruise through Baton Rouge

by Franklin Poole <franklin@intersurf.com>

16) Introduction

by Jim Paulsgrove <mpaul@nando.net>

----------------------------------------------------------------------

Date: Fri, 1 Mar 1996 16:51:00 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Not me, no way

Message-ID: <31379B74.634B@ns.cencom.net>

I didnt do it this time!

Loren DCM96 SPE

------------------------------

Date: Fri, 1 Mar 1996 16:56:28 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: 1.6 rockers

Message-ID: <1.5.4b11.32.19960302005628.006f6378@mail.one.net>

For anyone interested in the Crane 1.6:1 rockers, I got this

from a friend on the f-body list:

>X-POP3-Rcpt: rcheek@mail.one.net

>Return-Path: jca@fibercom.com

>Date: Wed, 28 Feb 96 12:25:13 EST

>From: jca@fibercom.com (James (Chris) Akers)

>To: rcheek@one.net

>Subject: 1.6 rockers

>

>Hey Rob,

>  Thought you might be interested...  there is currently

>a group purchase going on for the Crane Gold 1.6 full roller

>rockers.  I have them on my LT1 Z and they were a breeze to

>install with no other mods necessary.  Anyway, the group price

>is $255 a set + shipping.  You might want to pass this along

>to the Impala list.  They are $300 in Summit and Jegs.

>

>Call Craig at Pro Pony to get on the list.  800-776-4432

>

>jC. '95 Z28 M6 - the snot rocket - 12.92@107.4

>

>ps. I don't gain anything from this group purchase since

>I already have mine!

>_________________________________________________________

>James C. Akers    Broadband Software Group    Roanoke, VA

>jca@fibercom.com    Litton-FiberCom Inc.    (703)342-6700

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Fri, 1 Mar 1996 16:59:10 PST

From: ajcohen@li.net (Andrew Cohen)

To: impala@east.xsis.xerox.com

Subject: 1965 Ford Wagon

Message-ID: <v01530503ad5d4512c093@[199.173.74.115]>

(ha ha)

Now that I have your attention...

A newer Mercedes pulled up next to me on my left at a light.  I had about a 1/4

mile to get over to the left lane for my turn.  Not thinking too much, the light

went green and I started off "smartly" to get over to the left.  At the same

instant, I heard the squeal of rubber slipping and out of the corner of my

eye, I

could see this benz *smoking* the tires!  Well, I don't beat on my Impala, but

this was now a question of pride... I punched it.  Blew (I mean blew!) his doors

off!  I gave him a wave from everyone as I passed him too!  I have to say, it

was fun.  Just think of it... I could probably have 3 Impalas for what he paid.

That's 3 times the fun; one for the wife, one for the girlfriend, and one

for me!

---

Troy, any news on the HA bar group purchase?  Bushings?  Stainless bolts?

---

Oh, can someone please confirm; the crackling sound going over bumps coming from

around the rear view mirror area... that's a loose windshield (pop)?  What

should

I do if it is?  Dealer?, or is this a do it right yourself item?

---

BTW, I just love seeing those 2 part newsletters; email 101.

---

One last thing;  does anyone actually work at Xerox?  If so, I've got a

question.

TIA.

-Andy

Cherry 95' Mods: The bib, and Hot Rods to Hell on VHS in the glove compartment.

------------------------------

Date: Fri, 1 Mar 1996 14:50:34 PST

From: Darren Buck <dwb0522@okway.okstate.edu>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 316

Message-ID: <13796f40@Okway.okstate.edu>

Stephen,

Congrats on putting an order in for a GREAT car!  The dealer

should be able to do a search on the computer and locate one

at another dealer very easily.  If he can't, find another

dealer.  Also, when I ordered my car, I made a deal for $500

over dealer invoice.  When the car came in, he showed me the

invoice and I wrote a check for $500 more.  He still made

the 3% holdback also which was plenty.  Finally, you might

opt for the cassett deck and buy the factory CD changer for

the trunk like some others on the list.  If I had known, I

would have.  Good luck.

Darren Buck

'96 Dark Cherry

------------------------------

Date: Fri, 1 Mar 1996 17:18:23 PST

From: Stinkyg@aol.com

To: impala@east.xsis.xerox.com

Subject: PCM

Message-ID: <960301201822_235527085@mail06.mail.aol.com>

I need some help deciding which PCM to purchase.  If any of you have

installed either JET's or the PCM from evergreen perf. in your impala, please

let me know the differences(pros and cons) between the two.  You may either

respond via the"list" or directly to me.  Thanks!

Steve

Stinkyg@aol.com

Awaiting delivery of black '96 soon.

------------------------------

Date: Fri, 1 Mar 1996 17:35:10 PST

From: Ray Berilla <writes4u@ganet.net>

To: Impala list <impala@east.xsis.xerox.com>

Subject: Another Digest quoter!

Message-ID: <199603020142.UAA00055@s1.GANet.NET>

>>The dealers in New Jersey in my area have some Impalas in stock. They

all have the front plate though and you can expect $500-750 over

invoice. My 96 was ordered on Dec 5 and came in today. 3-1-96

John<<

John, better put on your asbestos underwear. You're gonna take some heat

for quoting the entire digest!

Hey Loren, don't you feel better now? <BG>

Ray B.

------------------------------

Date: Fri, 1 Mar 1996 18:14:10 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Cc: 73145.1566@compuserve.com

Subject: Front Bumper Holes

Message-ID: <960301211409_235566798@emout04.mail.aol.com>

Scott, you wrote:

>>I simply repeat my statement, "Drill a

single hole in my car, install that bracket, and I will walk, it is your

choice." My rule is that if holes must be drilled, the plate stays off. The

Impala front end looks so smooth without that ugly plate bracket spoiling

it. Scott.<<

Unfortunately, some of us didn't have the  chance to catch the prep mechanic

before doing the drilling.  Now that's it's done, what is the best way to

hide/patch the holes??  I was basically told that "it's too late to fix it."

..Bruce

------------------------------

Date: Fri, 1 Mar 1996 18:28:37 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: 2cd Gear Starts

Message-ID: <960302022837_76165.1654_GHM71-1@CompuServe.COM>

In digest 317 Scott Mueller writes:

>>> Can everybody with a '95 or '96 verify whether their Impala

starts in 2nd Gear when the shifter is in the Manual 2nd range? Scott.<<<

Scott,

My 95 starts in second gear, and stays there unless I really punch it, then it

downshifts to 1st. I am glad that GM finally did this, as Ford trannys have been

this way for years. This has helped me countless times this winter, when

starting from stop, on a hill, or during extremely icy conditions. I also use

2cd gear when driving down steep hills, to avoid having to use the brakes to

stay within a reasonable speed range.

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......

------------------------------

Date: Fri, 1 Mar 1996 19:13:23 PST

From: dhflores@iwl.net (Debbie/Henry Flores)

To: impala@east.xsis.xerox.com

Message-ID: <96Mar1.191657pst."14606(1)"@alpha.xerox.com>

Scott Mueller writes:

>>>Yes, I apologize if the personal info was out of line, but as you indicated

the main reason I posted it was to explain the absence of my car at the

Chevy-Vettefest. I could have just said "medical problems", but that might

have raised additional questions and besides it is just my nature to be

technically more detailed than that! Most people might not have said

anything at all, but I feel that we have all become friends here, much more

so than on other lists I have been on. Scott.<<<

Scott,

No need to apologize. I just came back into town and realized what happened.

How dare this individual!!!!  Let me assure you that I consider you to be a

friend and knowing a bit more about your life is an honor. A little self

disclosure that is shared with friends creates long lasting friendships.  I

hope your wife is doing fine. The both of you will be in my prayers.  Now

then, for that insensitive primate who thought he would be cute for posting

his intelligence I say may UNSUBSRIBE.  We really don't need your kind.

Please find another group who would better fit your profile. Perhaps

1-800-IDIOT!!  I also hope your LT1 cracks a block, throws a rod, burns oil,

detonates, and stops when you are out of warranty

Henry in Houston

------------------------------

Date: Fri, 1 Mar 1996 19:19:45 PST

From: "Tim M. Killmeyer" <74144.103@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: IMPALA digest 316

Message-ID: <960302031944_74144.103_BHR42-1@CompuServe.COM>

Tom,

>>Last time I was at the autocross, I had the beast corner weighed - here are

the numbers (with a 220 pounder in the drivers seat)

Left Front 1246	Right Front 1188

Left Rear  1049	Right Rear 993

Front Total 2334	Total Rear 2042

Total Weight	4376

Front to Rear Ratio	53:47

As I'm sure you know from your years of autocrossing, one of the important

weight distribution ratios (especially for effect on handling) is the LF/RR to

RF/LR ratio ('squareness') - The Impala SS corner ratios are as follows: LF/RR

2239 and RF/LR 2237. This is astonishing 'square' and contributes greatly to

the amazing handling (all things considered) of the beast...<<

Thanks for the info, that must be why it's so much fun throwin' that baby around

the course. You really do feel under control even though you're driving such a

big beast.

>>While I've got you here, what tire pressures do you use for autocross

(assuming that you're running stock tires)? I've seen as high as 45 psi

recommended, but I usually run around 33...<<

Last time out (mid to high 60 degrees October day) I started out using about 42

lbs all the way around.  If I remember right, (I've got to start keeping a log)

I ended up with about 35 lbs in the back and 40 in the front. I was working on

trying to get more traction and keeping the rear end from sliding out too much

on a particular 120 to 130 degree turn.

One of other threads is discussing second gear starts. I turned my best times

when I started in second gear. Going by the other thread, I can't prove that I

was starting or staying in second gear, but it -seemed- to me that I DID have

less wheel spin off the line and more importantly less wheel spin when I slowed

down for a sharp turn and then tromped it while exiting the turn. I was losing

valuable time with my tires spinning, but when I started and left it in second

gear, the tires seemed to grip faster and got me on my way. What a rush after my

TR8 and Escort GT.  I've never driven anything so fast on an autocross course.

I'm REALLY going to have to change my style.

Have you (or anybody) ever tried second gear starts in the SS while auto -xing?

And if you haven't, would you consider it? Why or why not?

Tim

'gotta miss the first auto-x of the year to go to an uncles 72 birthday party.

can't get to one 'til April 14   8-(

------------------------------

Date: Fri, 1 Mar 1996 20:19:16 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Cc: skfurr@raliegh.ibm.com

Subject: Impala purchase

Message-ID: <3137CC44.791A@intersurf.com>

Stephen,

don't fret the deal!  yeah you MIGHT be able to get some dealers

to give you an enormous discount, but I doubt it at this late

date.  I've already heard (probably untrue) romours that the

Impala SS's will be "ordered out" by June.  Some dealers are

already asking sticker.  A local dealer asked me (jokingly) if I

could help him get a couple...

Also don't worry about the plate on the front. It comes on every

car and is "installed" in the trunk.  Simply threaten the sales

manager with death by dismemberment if he allows service to even

THINK about disfiguring your nice new car!!!!!!<G>

all my posts for 7 days are going to the circle file so if you

get this stephen and don't see it on the digests, please echo me

back...

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Fri, 1 Mar 1996 20:23:50 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Airbox cutting, homeplate removal, etc.

Message-ID: <3137CD56.64D7@intersurf.com>

David,

Regarding removal of homeplate, and cutting airboxes you'll

notice the change in the seat of the pants.  the aftermarket

wouldn't be able to sell those $250 kits for very long if they

just looked cool. Cool is cheap, performance is valuable!

The airbox and removal of homeplate on my '96 made a noticeable

difference and I haven't even gotten the K&N filter installed

yet.  Even better was the removal of the mufflers<g>.  Ask Scott

Harris.  We be bad when the leather is on the carpet! You

haven't heard a sound like that ever probably!  I turn heads

from two blocks when I'm on an onramp with it to the floor!

go for it!

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Fri, 1 Mar 1996 20:30:11 PST

From: Franklin Poole <franklin@intersurf.com>

To: skfurr@raliegh.ibm.com

Cc: Impala List Server <impala@east.xsis.xerox.com>

Subject: Impala Prices

Message-ID: <3137CED3.5E32@intersurf.com>

whoops!  Scott Mueller to the rescue again.  Ignore my earlier

post Stephen Furr and take Scott's advice.  If Mr. Mueller says

the world is flat then you'll be running off the edge soon!

CALL ACS and get your car cheap!!!!!!

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Fri, 1 Mar 1996 20:40:36 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: is my car stuck in performance mode??

Message-ID: <3137D144.754B@intersurf.com>

If this reaches the digest it'll be the first in a week!!

Shamu my 2TONLT1 seems to be stuck in what would be called

performance mode shifting.  She won't go to fourth gear until

52mph no matter what and won't "lock up" until 61mph.  I have

stock gears and stock PCM programming.  The only serious

performance mods are the fresh air airbox cutout, no homeplate,

no muffler thing.

Any ideas?

Another minor thing.  Frequently I hear a loud TICK sound from

the vicinity of the fuse box on the left side of the dash. Any

clue?

My service manual is now 5 weeks from order and still not here!

I ordered through my local dealer ($15 discount dontjaknow) and

he is freaked out.  We have 4 of us waiting for manuals...

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Fri, 1 Mar 1996 20:45:47 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Supercruise through Baton Rouge

Message-ID: <3137D27B.7A17@intersurf.com>

For those of you going to Arlington I'm working on some fun

stuff when you stop by Baton Rouge on the way (those of you on

the Gulf South tour<g>).  A local restaurant (seafood, cajun) is

interested in hosting us and the local All Star Chevrolet has

expressed "some" interest in hosting us for a little fun show.

I'm also going to beat up a couple of local hotels for nice

discounts if enough floridiots (uh floridians) come over this

way for the trip<G>.

The show may or may not be appropriate depending upon how tired

everyboy will be and the lateness of the arrivals.

Those of you local to the list (baton rouge area) contact me and

we'll get together soon to scare the locals.  I'm planning to go

to the local SuperCruise night sponsored by Oldies 98 on

Saturday march 2 at 5:00 till.  I'm moving the office this

weekend so I could be held up. (whoops, personal information,

I'll delete that).

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Fri, 1 Mar 1996 21:47:30 PST

From: Jim Paulsgrove <mpaul@nando.net>

To: impala@east.xsis.xerox.com

Subject: Introduction

Message-ID: <1.5.4b11.16.19960302005053.25cfdbd2@parsifal.nando.net>

Hello Everyone,

I've been lusting over the SS since it was reintroduced in '94.  A lady with

whom I work with at Data General Corp. purchased a '94 at her son's urging.

Her son picks up his SS next week.

My 90 SHO (I won't use the f word) has been a good ride and fun to drive but

my 30+ years of driving experience assumes that it's the rear wheels that do

the propelling.  FWD vehicles may do okay in the snow but I haven't had much

success in doing donuts with it.

The SS has been on order for a few weeks and is expected in early April.

Thanks to this list I will be contacting my dealer ASAP to make sure that

there are no tag holes drilled in the front since plates are only issued for

the rear here in NC.

The Arlington gathering sounds like a lot of fun and will be a magnificant

sight.  Maybe next year...Hopefully someone will take some pics and put them

on the net for downloading.

I look forward to the dialog on this list in addition to the information

being shared.

Cheers to all....Jim

P.S. Rumor has it that a dealer in Jasper Florida has a few Impala's on his

lot that will go for $100 over invoice.

------------------------------

End of IMPALA Digest 318

************************

IMPALA Digest 319

Topics covered in this issue include:

1) Bumper holes

by Scott Mueller <73145.1566@compuserve.com>

2) Roller rockers

by Scott Mueller <73145.1566@compuserve.com>

3) Re: Impala Prices

by dstabb@epix.net (David Stabb)

4) What are the Best Performance Mods?

by dstabb@epix.net (David Stabb)

5) 2nd Gear starts

by Steve Das <sdas@sat.net>

6) My excellent buying experience with ACS

by "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

7) 2nd Gear Start

by Steve Das <sdas@sat.net>

8) Second gear starts

by "J.R. LEVITZ" <4everGreen@msn.com>

9) Another RECALL!!

by BGordon401@aol.com

10) NAISSO CLARIFICATION/Various

by Rock528@aol.com

11) Re: IMPALA digest 318

by Mbmarx@aol.com

12) Factory Speakers

by T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

13) Waiting for Parts

by T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

14) Floor Mats

by T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

----------------------------------------------------------------------

Date: Fri, 1 Mar 1996 23:41:52 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Bumper holes

Message-ID: <960302074151_73145.1566_FHD30-1@CompuServe.COM>

Bruce,

>Unfortunately, some of us didn't have the  chance to catch the prep

mechanic before doing the drilling. Now that's it's done, what is the best

way to hide/patch the holes?? I was basically told that "it's too late to

fix it."

It certainly can be fixed! There are two ways to go, either have it

professionally repaired or do a quick fix yourself. For a professional

repair, they will probably grind the area a little, put in some flexible

repair material, and then shape it, sand it, prime and paint. If done by a

competent shop, when finished there would be no evidence that it was ever

repaired. Unfortunately this would probably cost several hundred dollars.

If you want a cheaper fix, remove the screws and push the hole closed as

much as possible from behind if you can. If the car is black, then you might

dab a little black silicone sealer in there to smooth it. This will only be

detectable at very close range. If the car is other than black, then try

dabbing some touchup paint in the hole to fill it. This will also be fairly

undetectable unless you are looking for it. Let me what you end up doing and

how well it works. Scott.

------------------------------

Date: Fri, 1 Mar 1996 23:42:01 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Roller rockers

Message-ID: <960302074200_73145.1566_FHD30-3@CompuServe.COM>

Rob,

Your friend from the F-car list said:

>there is currently a group purchase going on for the Crane Gold 1.6 full

roller rockers. I have them on my LT1 Z and they were a breeze to install

with no other mods necessary.

Based on the measurements I made, I sure hope that he installed the LT4 or

other compatible valvesprings when he installed the 1.6 rockers! If not he

may be in for some serious trouble down the road, as the stock LT1 springs

are very close to coil bind with the 1.6 rockers. If he breaks a spring, the

valve will drop into the combustion chamber, and his engine will be

seriously damaged. Scott.

------------------------------

Date: Sat, 2 Mar 1996 06:10:00 PST

From: dstabb@epix.net (David Stabb)

To: impala@east.xsis.xerox.com

Subject: Re: Impala Prices

Message-ID: <v02130502ad5df936eff8@[199.224.67.48]>

Everybody's been talking about prices paid for their Impalas. My 95 was a

leftover in January 96, with less than 100 miles on the clock. With the

Impala home page's technical FAQ in hand and a copy of Edward's, I

negotiated an at-invoice price, with the dealership performing all of the

service, including the most serious gasket hole fix before pickup.

They also put in a new power steering pump, as indicated by the FAQ.

For comparison, the dealer offered me a 96 with the CD player (now I'm glad

I didn't go for it - my 95 has that SCV cassette deck that will control a

changer in the trunk) for $1300 over invoice. He said that there was no way

he would go lower on a 96 - they were in too much demand and he knew he

would sell it for at least that amount. I believe him.

With the kind of driving I do, I really prefer the column shifter and

larger console, despite the sporty look and feel of the floor shifter. I do

miss having a tach, but won't go for an add-on.

____|)avid             Good Morning, Good Afternoon, Good Evening.

How difficult it is to tell when

dstabb@epix.net               you're locked in a tiny room with

dstabb@bix.com                     no windows and artificial lights.

------------------------------

Date: Sat, 2 Mar 1996 06:36:11 PST

From: dstabb@epix.net (David Stabb)

To: impala@east.xsis.xerox.com

Subject: What are the Best Performance Mods?

Message-ID: <v02130503ad5e0b1522f0@[199.224.67.48]>

Fellow ImpalaListers-

Can we develop a menu of the best, most cost-effective performance mods?

Starting with things that are warranty-safe, of course. And in the order

to do them. As a new Impala owner, I'd like a wish list program.

In no particular order, I suspect the following might make the list:

Cold-Air Kit

HyperTech Power-Programming Kit for the computer chip

Dr. Jacobs' ignition

Vortec shift kit

Smaller torque converter

Better exhaust headers from ???

Aftermarket head/valves from ???

Aftermarket cam/lifters

Blueprint the engine, stroke it, bore it out to a 383

Sell the mouse, buy an aftermarket rat.

Well, the last IS a bit extreme ;-)

____|)avid             Good Morning, Good Afternoon, Good Evening.

How difficult it is to tell when

dstabb@epix.net               you're locked in a tiny room with

dstabb@bix.com                     no windows and artificial lights.

------------------------------

Date: Sat, 2 Mar 1996 08:18:08 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: 2nd Gear starts

Message-ID: <199603021616.KAA18201@raven.sat.net>

Well I went out and tried to get "shifty" with my '95 today and all I can

tell you is that if I put the lever in the 2 position, it certainly feels

like it is shifting from 1st to second.  The service manual says it will

"start and stay in second gear" but I'm betting the thing is really starting

in 1st and shifting to 2nd.  According to the shop manual, the convertor

clutch is not suposed to lock in second so I don't think that is it but I'll

guarantee it is shifting.  I tried it several times and it is

unmistakeable... it's shifting.

Anybody else got an opinion?

Incidentally, I'll bet those with a tach (my ANALOG tach isn't here yet from

Summit) can verify it is shifting.  Any takers out there???

Steve

------------------------------

Date: Sat, 2 Mar 1996 04:34:44 PST

From: "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

To: impala <impala@east.xsis.xerox.com>

Cc: "73766.2604" <73766.2604%cishub%cserve@notes2.compuserve.com>

Subject: My excellent buying experience with ACS

Message-ID: <9603021837.AA2885@CServe-Hub4.inhouse.compuserve.com>

Stephen (and list members):

>> The dealer that I ordered from sold me the car for exactly $800 off of

sticker.  From reading the archive, this may or may not have been a good deal <<

I have been wanting to do a detailed post to the list on my pleasant buying

experience with using ACS (Automobile Consumer Service, I think) since I picked

up my car 2/23/96, but have been too busy this week.  They are based out of

Cinncy.  Let me begin with the usual disclaimer:  I have no ties or financial

kickbacks from this company.  Also, I will apologize for the length of this

post in advance.

In late September of '95, I began looking into the purchase of a '96 SS.  I

went to many of the local Chicagoland dealers, and was very disappointed with

their reaction to my wanting to purchase a '96.  Most of them said, "sticker,

nothing less".  Several of them tried the, "ya know, they're only gonna build

5,000 SS's, and production stops January '96, so you better order today, blah

blah blah..." and other garbage like that.  One dealer even had the nerve to

say that their only way to take orders on the '96 SS was to have people submit

bids, and they would take the highest one out of a group when they "built up".

I tried going through Chevy dealers with whom my employer has arranged

discounts.  Best I could get was 2% below retail.  This was starting to get old

very fast.

I then tried a few different broker services, one through a consumer club I

belong to.  Same old story, not much off of retail.  Some wouldn't even tell me

how much they could get the car for until I paid them a fee.  Forget that

noise!  Then, I checked with the Guru of Automobiles himself, Scott Mueller.

He said, "Give Tarry at ACS a call.  I think he will give you a deal you can't

beat, and they are very professional about the whole thing" (I'm paraphrasing,

Scott, but it was something to this effect).

I knew if Scott was endorsing them, they must be good.  Scott, while also being

very knowledgeable, demands goods service, or he will not endorse a product or

service from what I know of him (I mean this as a compliment, Scott).  For ACS

to get his blessing was reason enough to use the service, although I was a

little leary of using an auto broker, since I had not done this before.

When I heard that Impala production (this was in Oct. '95) would take place

through August of '96, I decided I would order the car around early December in

hopes of a March or April delivery.  This was desired for a number of reasons

which I will not go into here.  At the time, many on the list were talking of

3+ months of order-->delivery date time

I called ACS in early December after hearing through the grapevine that they

had some pretty sweet deals on SS's.  The way they (and some other brokers)

work is this:  They have buying power with several dealerships of all kinds of

makes/models of automobiles due to the volumes they can do each year.  When I

called ACS, I told them that I wanted a DCM '96 SS, and pretty much knew the

options I wanted.  They sent me a detailed listing, with retail and invoice

prices, of the car and all its available options.  I called and ordered the car

on Dec. 6.  They told me I would pay invoice + $X over invoice as dealer profit

+ $Y ACS fee.  That's how they work -- they will tell you a price, no haggling

or walking out of dealerships and coming back later, etc involved.  I ran the

numbers for what I paid, and this was at least $1,000 below what anyone offered

at the local Chicagoland dealerships.  I only had to pay a small portion of the

ACS fee up front as down payment, and a down payment (on my credit car) to the

dealership.  They dealer, BTW, never charged the down payment through -- he

just kept the info in case I renegged on the deal.

I had three options for delivery to Chicago:  (1) For about $175, the car would

be put on a truck from the Cinny dealer and be delivered to a local Chicagoland

dealer of my choice; (2) For a fee, I could have someone at ACS drive it up and

then fly back to Cinncy, or (3) I could pick up the vehichle myself in Cinncy.

They even had a voucher available on ComAir for $75 that would get me a flight

from Chicago to Cinny.  I chose option 3, but ended up being able to stop on

Cinncy on the way home from one of my business trips.  I just did not want to

have to deal with any Chicagoland Chevy dealers, so I didn't like option (1).

I got word January 4 that the car would probably be available some time in

February.  Late January, I found out that the car had been assigned a build

date of February 5.  It was built, and shipped from Arlington on Feb. 8.  The

preliminary invoice was sent to me that next week.  They anticipated arrival at

the dealer some time around the 16th - 23rd of Feb.  In the mean time, I found

out last minuet that I had to go out of town (Boston) from the 19th to the

23rd.  The cheapest flight on that late notice was to change planes somewhere,

so I used Delta, and changed in Cinncy, in anticipation that the car would

arrive while I was gone.  It worked out best for both the company I work for

and for me!

Turns out, the car arrived on the 22nd, and the dealer prepped it that day.

Tarry of ACS met me at the gate at the airport on the 23rd, all the paperwork

in hand.  I even made a last minuet change in my flight, which was no problem

for him.  It took less than 15 min. to review and sign everything, and I was in

the car!  I didn't even have to give him a ride back to ACS.  I was homebound

right from the airport.  Every number we agreed on when the car was ordered the

6th of December was on the purchase agreement down to the bone, no hidden

charges!  The trip from Cinncy to Chicago was a nice break in period, although

I do admit it was hard to stay below 55, and also vary the speed quite a bit

and ignore cruise control.

All in all, this has been the best experience I have ever had with purchasing

an automobile!  Tarry and Jenny of ACS are extremely professional.  You will

not be hassled to get rust protection, warranties, or other add-ons (none of

this even gets mentioned).  I think they can even arrange financing, although I

did not do this, so check with them.  They even put up with me calling them

constantly and asking tons of questions.  In reality, I could have just placed

the order Dec. 6, waited for them to tell me when it was ready, and picked it

up.  They apparently have received a write up in the March edition of Money

Magazine, although I haven't had a chance to read it.  If anyone is interested,

call them at 800-223-4882, and tell them Scott M. sent you (I can't take

credit, since I heard of this through Scott!).  And, yes, they will leave the

liscense plate braket off, if you desire.  Also, no dealership stickers or

emblems of any kind were affixed to the vehicle.

Jeff

'96 DCM

------------------------------

Date: Sat, 2 Mar 1996 11:04:16 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com, lavera.twine@hok.com, erunnio@infohwy.com,

Subject: 2nd Gear Start

Message-ID: <199603021902.NAA27445@raven.sat.net>

More on the ability to start off in second gear.... I went by my local

Pontiac dealer today (I was looking at a new Suburban) and was looking at a

new Firebird.  They have a button behind the shifter that is labeled "Second

Gear Start".  Seems if it was already a feature of the transmission, they

wouldn't have a button so labeled.  I tested my '95 again and it will take a

lot to convince me that it isn't starting in first and shifting to second.

I don't care what the manual says!  I can definitely feel it.

Steve

------------------------------

Date: Sat, 2 Mar 1996 11:17:08 PST

From: "J.R. LEVITZ" <4everGreen@msn.com>

To: "Impala List" <impala@east.xsis.xerox.com>

Subject: Second gear starts

Message-ID: <UPMAIL01.199603021921450003@msn.com>

My '95 Impala always starts in first regardless of gear selection. And, with

the exception of first gear which "governor upshifts" holds all gears to the

rev limiter.

How about the rest of you!

Regards John

------------------------------

Date: Sat, 2 Mar 1996 12:21:05 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Subject: Another RECALL!!

Message-ID: <960302152103_338901956@emout08.mail.aol.com>

O.K., folks we have another recall on our hands.

The recall is under safety campaign no. 96C14 for the ball joints.

Unfortunately, I only have a response card from the dealer, not the

"official" notice on GM letterhead.

Does anybody have any specifics on this recall??

..Bruce

------------------------------

Date: Sat, 2 Mar 1996 14:08:30 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: NAISSO CLARIFICATION/Various

Message-ID: <960302170830_236084105@emout05.mail.aol.com>

Hi all,

I have been getting alot of complementary mail on the club and I thank you.

There is a confusing issue however.  The email digests are separate from our

formed group of the National Assoc. of 94-6 Impala SS Owners dba N.A.I.S.S.O.

NAISSO includes the Impala SS registry, the hardcopy (snail mail)

newsletter, optional SS merchandise, performance parts club discounts and so

on.  This is what the subscription dues are applied to.

The email digests continue to be operated (free) by the very generous Scott

Hollenbeck and Xerox Company.  Scott deserves a lot of credit for putting up

with as much hassle as the newsletter produces.  Thank you and I just wanted

to make sure all the new folks understand.

Secondly,

>	Just found this site which will give you the parts number of

>Bilstein Shocks for every car.  Although admitedly, their price aren't

>the best.  Still worth while to check it out.

Club members can get Bilsteins for $65.00 per shock.  Info is in the new

newsletter.

Next,

Remember to make up some kind of sheet or board of your car's modifications

if you are going to show it at the Texas show.  Scott, please include a cd

rom with yours like the computer book..<g>.

Lastly,

On personal topic (god forbid), I put my life on hold and stole a couple of

days at a great little cabin up on the Suwannee River.  Upon return I had

over fifty impala email messages...this is spreading like a virus.

It's time to go fly again..500,000 bikers in town and mucho trauma.

Troy

------------------------------

Date: Sat, 2 Mar 1996 14:12:11 PST

From: Mbmarx@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 318

Message-ID: <960302171210_338991563@emout04.mail.aol.com>

In a message dated 96-03-02 00:50:37 EST, you write:

>Shamu my 2TONLT1 seems to be stuck in what would be called

>performance mode shifting.  She won't go to fourth gear until

>52mph no matter what and won't "lock up" until 61mph.  I have

>stock gears and stock PCM programming.  The only serious

>performance mods are the fresh air airbox cutout, no homeplate,

>no muffler thing.

>

>Any ideas?

When I activated the "Performance mode shifting" on my PCM (terminal

#13 grey plug) I had the same results, only my car would NOT go into

converter lock up at any speed. The 4th gear shift was at 52 mph and all

other shift points were raised. I would think if this circuit were grounded

internally, the "mode" would constantly go in and out. You may want to check

the connection at this plug and the usual inspections.

John Spears are you out there? Can you shed any light on this PMS

thing... I could'nt resist.<g>

Scott M.... is there any thing in the tranny breakdown showing any additional

solenoids that may be associated with this feature, causing

problems, or maybe missing from the B-body version ?.

Also, my '95 will NOT pull away in second gear regardless of throttle

position. It pulls away in first, hits second and stays until I shift out of

second. I've tried it with the A/C on/off, VERY light throttle etc.

Any thoughts as to why some do and some cars don't? My build

date is 2/95.

Mike Marx

95 Black, Usual mods

------------------------------

Date: Sat, 2 Mar 1996 14:18:34 PST

From: T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

To: IMPALA@east.xsis.xerox.com (Non Receipt Notification Requested)

Subject: Factory Speakers

Message-ID: <0071phfrrdht*/I=T/S=MAKICHUK/OU=FA.EPS/O=NHQ/PRMD=GC+DRHC.HRDC/ADMD=GOVMT.CANADA/C=CA/@MHS>

>Any one out there know if the enhanced cassette had the same speakers as

the base unit?

Dave:

According to the 1996 Delco Canada Sound Facts book, all sound systems

installed in the 1996 Impala and Caprice have the same front and rear

speakers.  Only the UM7 has different, downgraded, speakers.  Since the

sound systems were carried over from 1995, I don't see why this wouldn't

also apply to your '95.

Tip Makichuk

"I can't sell 'em to ya, cause they're illegal"

------------------------------

Date: Sat, 2 Mar 1996 14:20:11 PST

From: T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

To: IMPALA@east.xsis.xerox.com (Non Receipt Notification Requested)

Subject: Waiting for Parts

Message-ID: <0070phfrrdht*/I=T/S=MAKICHUK/OU=FA.EPS/O=NHQ/PRMD=GC+DRHC.HRDC/ADMD=GOVMT.CANADA/C=CA/@MHS>

>steering knucles which unfortunately have to come from the U.S. (thus a

two week wait).

Mike:

FWIW GM of Canada has a special program for the delivery of parts.  It's

called VIP for Very Important Part.  Under VIP, GM will deliver any part

from anywhere to your dealer in just two days.  It's primarily intended for

vehicles that are out of service because of a part, but if you put the

squeeze on your dealer, they may extend this service to you.

Tip Makichuk

"I can't sell 'em to ya, cause they're illegal"

------------------------------

Date: Sat, 2 Mar 1996 14:25:30 PST

From: T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

To: IMPALA@east.xsis.xerox.com (Non Receipt Notification Requested)

Subject: Floor Mats

Message-ID: <0070xndjlftl*/I=T/S=MAKICHUK/OU=FA.EPS/O=NHQ/PRMD=GC+DRHC.HRDC/ADMD=GOVMT.CANADA/C=CA/@MHS>

>Anyone have any carpet cleaning tips?  I just went through 2 months of

snow, slush and passengers who like to jump into cars with dripping, muddy

feet.  I've already established that the mats(especially the driver's side)

are not worth salvaging.

Greg:

While not really a carpet cleaning tip, you may want to consider my

approach.  When my car was new, I bought four Goodwrench rubber mats.

Although these don't hold as much water as the 'Pant Saver' style, they do

have the GM logo molded into them <G>.  Every fall, on November 1, I

install my four snow tires, winter wiper blades, and replace the factory

carpet mats with the four rubbers.  On April 1, I repeat this procedure,

and reinstall my summer equipment.  Hope this helps!

Tip Makichuk

"I can't sell 'em to ya, cause they're illegal"

------------------------------

End of IMPALA Digest 319

************************

IMPALA Digest 320

Topics covered in this issue include:

1) Re: 2nd Gear Start

by Steve Das <sdas@sat.net>

2) Wish List

by jeffd@accessus.net (Jeff Dawson)

3) 2nd gear starts

by BGordon401@aol.com

4) PMS problems

by john spears <speartec@iquest.net>

5) Something New at Burger King

by Loren Peterson <equ_rino@cencom.net>

6) 2nd Gear Start

by Scott Mueller <73145.1566@compuserve.com>

7) Personal stuff

by Scott Mueller <73145.1566@compuserve.com>

8) Re: 2nd gear starts

by Steve Das <sdas@sat.net>

9) Testing my posting ability

by Franklin Poole <franklin@intersurf.com>

10) transmission broke !

by ML3001@aol.com

11) SS prices

by "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

12) Re: IMPALA digest 310

by RAZZORFL@aol.com

13) Re: IMPALA digest 319

by RAZZORFL@aol.com

14) Re: IMPALA digest 314

by RAZZORFL@aol.com

15) Re: IMPALA digest 317

by RAZZORFL@aol.com

16) 2nd gear

by "Jeff Gatcha" <foxhound@txdirect.net>

17) Wheel centers

by "Jeff Gatcha" <foxhound@txdirect.net>

----------------------------------------------------------------------

Date: Sat, 2 Mar 1996 14:59:32 PST

From: Steve Das <sdas@sat.net>

To: Ed Runnion <erunnio@infohwy.com>

Cc: impala@east.xsis.xerox.com (Impala SS List)

Subject: Re: 2nd Gear Start

Message-ID: <199603022258.QAA09498@raven.sat.net>

At 04:49 PM 3/2/96 -0600, Ed Runnion wrote:

>>

>> More on the ability to start off in second gear.... I went by my local

>> Pontiac dealer today (I was looking at a new Suburban) and was looking at a

>> new Firebird.  They have a button behind the shifter that is labeled "Second

>> Gear Start".  Seems if it was already a feature of the transmission, they

>> wouldn't have a button so labeled.  I tested my '95 again and it will take a

>> lot to convince me that it isn't starting in first and shifting to second.

>> I don't care what the manual says!  I can definitely feel it.

>>

>

>I just tried mine in the parking lot : it is DEFINITELY starting

>in first and shifting to second when in the "2" position.  Looks

>like they may have removed this feature in some 95's and the

>96's (mine is a 96).

>

Tell you what ED, I'll bet they are ALL the same and the other guys just

can't tell it's shifting.

Steve

------------------------------

Date: Sat, 2 Mar 1996 15:13:53 PST

From: jeffd@accessus.net (Jeff Dawson)

To: impala@east.xsis.xerox.com

Subject: Wish List

Message-ID: <199603022313.RAA07348@mtvernon1.accessus.net>

Sorry for the lenth of this message but I wanted to share my wish list with

the folks who would most understand. This list is not carved in stone so

comments and observations will be most appreciable. Stage 1 of this list in

in progress as this is being written. Still waiting on the Walker exhaust,

but all other parts of 1st stage are here or coming. I am a "blue collar

worker" so this will take awhile to finish. Stage 3 will be done about the

time the SS is paid off and warranty is gone. Hope no one shorts out their

monitor while reading this so keep a napkin or hanky at the ready. Remember

drool and electricity don't mix. I found a local garage to do the rear

change. R & R Motorsports in Benton IL. The owner is knowlegable and did not

ask why I wanted to change. With a little luck and no suprises (3 kids) I

will have stage 1 done in time for Street Machine Nats here in DQ. =09

P.S.=20

Alot of these parts are going to come from LPE. John was nice enough to send

me a catalog for free.  Three days after asking for one on his web page it

was in my mailbox! This kind of service must be rewarded!=20

Impala Project List=09

Stage 1

1. Walker cat back exhaust with Utra Flow mufflers

2. 3.42 rear axle gear

3. K & N air filter in opened up stock airbox

4. Hypertech PCM

5. 160=B0 thermostat

Stage 2

1. Edelbrock-BBK 52mm throttle body {58mm?}

2. Edelbrock Intake

3. A. S. & M. Headers {LPE, Edelbrock?}

4. 1LE intake elbow

Stage 3

1. Edelbrock Heads {possibly Trick Flow?}

2. Crane 1.6 ratio roller rockers=20

3. cam {undecided, LPE, Edelbrock or TPIS ZZ9?}

4. 2500 rpm stall converter

Stage 4

1. Herb Adams F & R sway bars

2. Bilstien shocks

3. Hotchkis trailing arms

4. Eibach Springs

*notes:

At the end of stage 3 there should be roughly 325 hp at the rear wheels.

Any more power will have to be gained with the increase of cubic inches(383)

or Paxton supercharger($$$). 1/4 mi. times should be low 14's or high 13's.

Jeff Dawson 95 DCM still stock

------------------------------

Date: Sat, 2 Mar 1996 17:42:01 PST

From: BGordon401@aol.com

To: sdas@sat.net

Cc: impala@east.xsis.xerox.com

Subject: 2nd gear starts

Message-ID: <960302204201_339150601@mail02.mail.aol.com>

Steve, you wrote:

>>but I'm betting the thing is really starting

in 1st and shifting to 2nd.  According to the shop manual, the convertor

clutch is not suposed to lock in second so I don't think that is it but I'll

guarantee it is shifting.  I tried it several times and it is

unmistakeable... it's shifting.

Anybody else got an opinion?<<

I'll second your findings.  I have a 1995 SS with the stock programming and

no mods to the tranny or linkage.

If I put her in the "2" position, she DEFINATELY starts in 1st, and then

shifts into 2nd and stays there until rev limit.  I was hoping to find that

she started in 2nd and stayed there, but I was disappointed to find I was

wrong.

As far as why, I'm in the fog.  It appears that the shift lock works for

some, but not for others in the 1995 model year.  Hopefully, somebody will

come up with the answer, b/c running with 3.42 gears, my tires would stick

much better if the 2nd gear start worked.

..Bruce

------------------------------

Date: Sat, 2 Mar 1996 17:40:00 PST

From: john spears <speartec@iquest.net>

To: impala@east.xsis.xerox.com

Subject: PMS problems

Message-ID: <m0tt2mx-004XLyC@iquest.net>

Mike and Franklin,

We may have to take this PMS discussion to a medical chat line!<BG>

Seriously though, my guess on Franklins car is that you are having some type

of transmission failure, but that is only a guess as I am not a transmission

specialist. Did you try to hook up the Perf Mode Shift feature that I

described in an earlier digest? As far as I know Mike Marx is the only one

that actually hooked it up on the 95 SS calibration; and it was with less

than favorable results. It did not act like it does on my 94 Trans Am

calibration. It sounded like maybe just a partial calibration or something

on the 95.

Scott M...you mentioned in digest 319 about serious trouble using the 1.6

roller rockers with stock valvesprings. Are your measurements based on the

iron head valvespring or the aluminum head valvespring? Or do they use the

same valvespring? I am using the Crane 1.6 roller rockers on my 91 "SS"

wagon with the aluminum heads and stock springs. This setup with the 94 T/A

PCM program has a WOT upshift of 5800 RPM. I have noticed no problems, but

would like to know if I am flirting with disaster!         John.

------------------------------

Date: Sat, 2 Mar 1996 17:44:54 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Something New at Burger King

Message-ID: <3138F996.3D59@ns.cencom.net>

Hi All,

Well not in Burger King exactly, but while waiting in line at the drive

thru, my Wife made an interesting discovery.  On our '96 anyway, if you

push the ash tray forward (inside drop down door) low and behold there

is a cup/can holder.  Somehow we had missed this feature.  Glad She

found it cause she wished we had 94/95 console for the cup holders.  Now

were both happy.

Loren DCM96 SPE

Lets get sideways

TEXAS or bust

p.s. Chewed up a SHO on I74.....no contesto

------------------------------

Date: Sat, 2 Mar 1996 18:18:23 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: 2nd Gear Start

Message-ID: <960303021823_73145.1566_FHD60-1@CompuServe.COM>

Mike,

>is there any thing in the tranny breakdown showing any additional solenoids

that may be associated with this feature, causing problems, or maybe missing

from the B-body version ?.

Not really. There was a slight change in the '95+ 4L60-E transmissions with

regards to the torque converter and the Torque Converter Clutch (TCC)

circuit. In these models there is an extra PWM (Pulse Width Modulated)

solenoid that controls the application of the TCC, softening the engagement.

The '94 and earlier transmission do not have this solenoid. The TCC in the

converter was also slightly re-designed for '95. None of these things should

have any effect on the 2nd Gear Start capability, but it seems like many '95

models lack this feature.

It sounds to me like there may have been either a valve body design or PCM

calibration change partway through the '95 model year, as some '95 have the

2nd Gear Start feature while others seem to lack it. All '94s have this

feature, but what about the '96 models?

I wonder how many different PCM calibrations there are for '94, '95, and

'96, and whether any of these are documented to remove the 2nd Gear Start

function?

Mike S. and I were testing his Impala the other night with a '94 F-car PCM

calibration (including the bigger F-car MAF), and it ran great! The shifting

was dramatically improved, as was the operation of the TCC. We were

expecting problems because the '94 calibration did not have the code to

operate the new PWM TCC solenoid in Mike's '95 transmission, but everything

seemed to work fine. Unfortunately we did not think to check if the 2nd Gear

Start function now worked with the F-car calibration, it definitly does not

function in Mike's car with the stock calibration, which is an early '95

build. We will probably swap in the F-car PCM again to check it out. Scott.

------------------------------

Date: Sat, 2 Mar 1996 18:18:35 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Personal stuff

Message-ID: <960303021834_73145.1566_FHD60-4@CompuServe.COM>

Jeremy K. wrote me and very graciously apologized for the comment he made

earlier on the list. I just wanted everybody to know as I would really like

him to feel welcome here again. There are no hard feelings, and let's

consider this matter closed! Scott.

------------------------------

Date: Sat, 2 Mar 1996 18:55:10 PST

From: Steve Das <sdas@sat.net>

To: BGordon401@aol.com

Cc: impala@east.xsis.xerox.com

Subject: Re: 2nd gear starts

Message-ID: <199603030253.UAA22076@raven.sat.net>

>As far as why, I'm in the fog.  It appears that the shift lock works for

>some, but not for others in the 1995 model year.  Hopefully, somebody will

>come up with the answer, b/c running with 3.42 gears, my tires would stick

>much better if the 2nd gear start worked.

>

Bruce,

I'm betting that they all work the way yours and mine do.  Several of the

other guys have written to me confirming the same thing you found.  It will

be interesting to see if anybody can verify that their car truly starts in

second.  I'm betting not.  Thanks for the response.

Steve

------------------------------

Date: Sat, 2 Mar 1996 19:39:10 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Testing my posting ability

Message-ID: <3139145E.3611@intersurf.com>

sorry to do this but my posts have all been disappearing for 8 days now!

--

Franklin Poole, Jr.

Praecedo, Inc.

================================================

It's not how fast you go, it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Sat, 2 Mar 1996 19:54:31 PST

From: ML3001@aol.com

To: impala@east.xsis.xerox.com

Subject: transmission broke !

Message-ID: <960302225429_436630756@emout05.mail.aol.com>

o.k. ineed some opinions please,i blew my transmission last week but i dont

know exactly why. i just had installed the transgo shift kit and i was racing

around town for the first decent chicago day in months. after a couple of

hours of driving, my car will shift from  1-2 fine but when it should go into

third the engine just revs freely as if the car were in nuetral,but if i

stomp the gas at that point the car will downshift and really go, but no

matter what it, will not go into third. i have had the trans apart 3 times

now to check my installation or for any obvious problems, everything seems

fine though.i have heard that the third gear clutch pack in the 4L60E is

small in diameter and could be a weak point? before you doubt the integrity

of a solid and proven transmission, let me tell you my car is an early 94

with 30000 miles on it also a carrol/vortec supercharger kit, 3 extra fuel

injectors, hi vol fuel pump, cold air induction w/ t.p.i. airfoil, as&m

headers,& custom 2-1/2 cat-back exhaust w/dynomax ultra flow mufflers. it has

other goodies too, but you can see i am abusing this transmission.

PERFORMANCE TECHNICAL CENTER in elk grove village built this car with me so

he is replacing it with a new 4L60E at no cost. warranties are not my

problem. i just wonder if i wrecked it with the shift kit install, the

horsepower, or it was just one of those things. i know 6 other people with

this same engine and transmission combo supercharged, and higher mileage with

no problems. the good part of this is, i can sneak in a new vigilante torque

converter ! that ought to max out the visa card (and home equity loan and

every other source of credit i could find to feed my car addiction)

P.S. has anyone tried the new baer racing, alcon brakes for our cars

yet, I NEEEEED THOSE TOO !!!!

------------------------------

Date: Sat, 2 Mar 1996 15:26:00 PST

From: "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

To: impala <impala@east.xsis.xerox.com>

Subject: SS prices

Message-ID: <31392E07@po5.cp.unisys.com>

Jeff,

You write

>They told me I would pay invoice + $X over invoice as dealer

profit .

So what was the bottom line price less license and tax?

'96 on the train some where so they tell me.

Don Fischer

------------------------------

Date: Sat, 2 Mar 1996 21:31:07 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 310

Message-ID: <960303003107_339314123@emout10.mail.aol.com>

In a message dated 96-02-27 02:24:38 EST, you write:

>Comming soon:  Z-pcm, 3.42 gear, and a Tach

>

>

Call me if you would like some suggestions on your choice of tach's and their

mounting.

RAZZORFL.

------------------------------

Date: Sat, 2 Mar 1996 21:30:16 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 319

Message-ID: <960303003016_339314043@mail02.mail.aol.com>

In a message dated 96-03-02 17:50:21 EST, you write:

> I do

>miss having a tach, but won't go for an add-on.

>

>

>

If your interested, I can tell you how I added a Auto Meter Tach to my Blk

'95 so it looks like it sits on the dash just above the left part of the

insrtument area with NO additional holes! In fact it has become somewhat of a

trademark of mine.

RAZZORFL.

------------------------------

Date: Sat, 2 Mar 1996 21:31:27 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 314

Message-ID: <960303003125_339314108@emout09.mail.aol.com>

In a message dated 96-02-29 00:26:56 EST, you write:

>>What effect, if any, does a plug have on the performance of a tire/its

>ability to withstand supersonic speed?

>

>

Please trust me on this one!

If you talk to anyone that is knowledgable in the tire industry they will

tell you that pluging a tire, especially a redial tire is not done anymore!

The proper way to repair a flat today is to dismount the tire and patch it

from inside. A tire plug can damage the radial cords and/or allow water and

other elements to damage the tire.

RAZZORFL.

------------------------------

Date: Sat, 2 Mar 1996 21:30:18 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 317

Message-ID: <960303003018_339314065@mail04.mail.aol.com>

In a message dated 96-03-01 18:16:51 EST, you write:

>Has anyone dyno'd the car before and after?  I know the benefits of

>cold air for increase in power, but does it show with these changes?

>

>

If your interested in 5 (five) additional REAR WHEEL HORSEPOWER gains PLUS,

then please call me!?!

RAZZORFL.

------------------------------

Date: Sat, 2 Mar 1996 22:22:58 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: 2nd gear

Message-ID: <199603030622.AAA07892@legend.txdirect.net>

My car behaves just like  Steve Das's on this one.

When my car is in #1,  starts in 1st gear and stays there

When  it is in #2, it starts in 1st gear, then shifts to 2nd, and

stays there.

So mine has no 2nd gear start.   June 95 prod date if that makes any

difference.

God, grant me the courage to change the things I can,

the serenity to accept the things I can't,

And the wisdom to hide the bodies of the people who really

pissed me off.

Jeff Gatcha

1995 GGM

Foxhound@txdirect.net

San Antonio Texas

------------------------------

Date: Sat, 2 Mar 1996 22:47:36 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Wheel centers

Message-ID: <199603030647.AAA08383@legend.txdirect.net>

Does anyone know or can anyone find out the Part Number for the

center caps on the Stock Impala wheels?    I had a rock or something

hit my left rear wheel and it nicked and scratched the center cap a

good bit.  I would like to get a new one and would like to have the p/n

when I order it.

If anyone is confused, it is the plastic cap with the Impala symbol

on it that covers the lug nuts on the stock wheels.

Thanks

God, grant me the courage to change the things I can,

the serenity to accept the things I can't,

And the wisdom to hide the bodies of the people who really

pissed me off.

Jeff Gatcha

1995 GGM

Foxhound@txdirect.net

San Antonio Texas

------------------------------

End of IMPALA Digest 320

************************

IMPALA Digest 321

Topics covered in this issue include:

1) 2nd Gear Start

by Scott Mueller <73145.1566@compuserve.com>

2) 1.6 rockers and springs

by Scott Mueller <73145.1566@compuserve.com>

3) '96 tranny in 2nd strarts in 1st

by Loren Peterson <equ_rino@cencom.net>

4) Re: IMPALA digest 320

by RAZZORFL@aol.com

5) Re: IMPALA digest 320

by RAZZORFL@aol.com

6) Airbox Mods

by Robert Cheek <rcheek@one.net>

7) Re: 2nd gear starts

by BGordon401@aol.com

8) IMPALA digest 320

by JHNS81A@prodigy.com (MR GREG L SULLIVAN SR)

9) Boy was I beat!-I love this car!

by aas@riker.neoucom.edu (Anuj A. Shah)

10) Re: '96 Impala's for $100 over invoice!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

by Kurt Kemmling <76165.1654@compuserve.com>

11) Latest Vericom Performance Results

by Steve Das <sdas@sat.net>

12) HD AC Oil filter?

by Loren Peterson <equ_rino@cencom.net>

13) Re: 2nd gear starts

by Steve Das <sdas@sat.net>

14) Valvesprings

by Scott Mueller <73145.1566@compuserve.com>

----------------------------------------------------------------------

Date: Sun, 3 Mar 1996 01:45:44 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: 2nd Gear Start

Message-ID: <960303094544_73145.1566_FHD30-2@CompuServe.COM>

Steve,

>I'm betting that they all work the way yours and mine do.  Several of the

other guys have written to me confirming the same thing you found.  It will

be interesting to see if anybody can verify that their car truly starts in

second.  I'm betting not.

I can assure you that all of the '94 cars do feature the 2nd Gear Start, at

least mine does! What I have found out is that this is directly controlled

by the PCM program and is not a function of the mechanical transmission

assembly itself. So Chevy apparently changed the calibration for '95 and '96

B-cars to remove this function, which is too bad as it is very useful in the

winter or in wet driving conditions. I wonder if either the Hypertech,

Z-industries (Evergreen), or Superchips PCM recalibrations could add this

feature back for the '95-'96 cars? I also wonder if the 2nd Gear Start

function is present in the '94 F-cars also, and perhaps removed in '95-'96?

Scott.

------------------------------

Date: Sun, 3 Mar 1996 01:45:37 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: 1.6 rockers and springs

Message-ID: <960303094536_73145.1566_FHD30-1@CompuServe.COM>

John,

>you mentioned in digest 319 about serious trouble using the 1.6 roller

rockers with stock valvesprings. Are your measurements based on the iron

head valvespring or the aluminum head valvespring? Or do they use the same

valvespring? I am using the Crane 1.6 roller rockers on my 91 "SS" wagon

with the aluminum heads and stock springs. This setup with the 94 T/A PCM

program has a WOT upshift of 5800 RPM. I have noticed no problems, but would

like to know if I am flirting with disaster!

The problem with coil bind depends primarily on which cam, rocker and

valvespring you are using. For example, if you are using an F/Y-car LT1 with

aluminum heads, stock F/Y-car valvesprings, stock F/Y-car cam, and then you

install the 1.6 rockers, you will be squeezing the springs to only 0.030

from coil bind!! This is WAY too close for comfort, and will likely result

in a busted valvespring at high rpms, and subsequently a dropped valve. The

solution is to install LT4 springs which will be 0.155 away from coil bind,

an extremely generous clearance.

If you are using the B-car cam, B-car valvesprings, and then install the 1.6

rockers, there will be no problem with coil bind (0.092 away from bind),

however you will be rpm limited such that valve float will occur at about

4900 to 5000 rpm. Different (stiffer) valvesprings (I suggest K-Motion #700)

must be selected to increase the rpm before float.

If you are using the B-car cam with aluminum F/Y-car heads with F/Y-car

valvesprings, and then install 1.6 rockers, there will no problem with

either bind clearance (0.62 away from coil bind) or valve float, however I

would still recommend installing the LT4 valvesprings as they are much

better than the LT1 valvesprings and they only cost about $80 for the set.

Scott.

------------------------------

Date: Sun, 3 Mar 1996 04:38:21 PST

From: Loren Peterson <equ_rino@cencom.net>

To: "impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: '96 tranny in 2nd strarts in 1st

Message-ID: <313992BD.2A89@ns.cencom.net>

My '96 takes off in 1st no matter where the shifter is located.

Loren DCM96 SPE

------------------------------

Date: Sun, 3 Mar 1996 06:43:45 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 320

Message-ID: <960303094343_436807330@emout07.mail.aol.com>

In a message dated 96-03-03 01:56:52 EST, you write:

>p.s. Chewed up a SHO on I74.....no contesto

>

>

DON'T YOU JUST LOVE IT!

RAZZORFL.

------------------------------

Date: Sun, 3 Mar 1996 06:45:00 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 320

Message-ID: <960303094457_436807341@emout09.mail.aol.com>

In a message dated 96-03-03 01:56:52 EST, you write:

>Does anyone know or can anyone find out the Part Number for the

>center caps on the Stock Impala wheels?

Here you go Jeff,

G.M. #9592229 bought this one for $20.13 + tax. I sell Automotive equipment

for my living and am fortunate enough to have a G.M. Dealer that likes me. He

has agreed to sell me my parts at 10 over their cost, the above price

reflects the disc.

A number of us in Fl. have "found" extra tires and rims(mags) and use them as

spares. To go along with my other mags I had my extra mag polished and am

about to mount in my trunk today. Therefore I needed an extra center cap to

go along with my new spare.

RAZZORFL.

------------------------------

Date: Sun, 3 Mar 1996 06:56:30 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Airbox Mods

Message-ID: <1.5.4b11.32.19960303145630.006d8140@mail.one.net>

>Hi All.  I was wondering if all the adjustments to the airbox, baffle,

> and home plate were worth the trouble in performance gains.  I know

>cleaning the engine compartment up is cool with the addition of the

>Corvette pieces, but to cut the airbox, etc, is there realized gains??

I was looking through the new Summit catalog and came across an ad

for an aftermarket intake for a '93 Camaro. The interesting part is

that they said they were talking to a guy at GM who's job it is to

make intakes and exhaust restrictive to the point of getting the

"correct" amount of horsepower out of the engine. Looking at all of

the baffles and the teeny air inlet hole for the Impala, I don't

doubt that this is true at all. Just thought it was kinda neat if GM

does do this, because it makes "hopping them up" all that much easier.

They're even making their own intake and exhaust pieces for the

Camaro now, so I'm guessing there is something to be gained.

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Sun, 3 Mar 1996 07:13:42 PST

From: BGordon401@aol.com

To: sdas@sat.net

Cc: impala@east.xsis.xerox.com

Subject: Re: 2nd gear starts

Message-ID: <960303101341_158768240@mail02.mail.aol.com>

In a message dated 96-03-02 21:54:30 EST, you write:

> It will

>be interesting to see if anybody can verify that their car truly starts in

>second.  I'm betting not.  Thanks for the response.

Steve, please let me know if someone come up with a good write-up for the 2nd

gear start.  If we can get the proper information together, our dealers will

be getting lots of SSer visiting them soon.

..Bruce

------------------------------

Date: Sun, 3 Mar 1996 08:34:17 PST

From: JHNS81A@prodigy.com (MR GREG L SULLIVAN SR)

To: impala@east.xsis.xerox.com

Subject: IMPALA digest 320

Message-ID: <013.02621256.JHNS81A@prodigy.com>

I just thought I would jump on and say how much I enjoy reading all

of these letters about the Impala.

I have a '96 Black SS with 7800 miles.  The windows are tinted, the

wheels are chromed, the 12 CD player is in the trunk playing through

the factory (SCV) cassette player.  Mobil 1 10w-30 is in the

crankcase. Otherwise, the car is completely stock.

About a month ago I took the Black Beast to the drag strip.  On my

first run I got a 15.350 ET at 90.45 MPH.  Spare tire was in the

trunk with about 1/4 tank of gas.  Tire pressures were left at 33 PSI.

My second run was a disaster.  The track must have had some oil or

water residue on it as I encountered serious wheel spin.  By the time

it got traction, it added over 1 second to my ET.  There were a

couple of other SS's there racing also.  I talked with both of the

owners.  One guys SS was completely stock and he turned 15.359 (MPH

unknown).  The other SS had "some modifications", but the owner

wouldn't say what.  He turned a 14.79 ET in the quarter.

My SS does share some common "characteristics" with some of the other

SS's I have read about:

1)  No matter what I do, the car always starts out in !st gear.  I

also saw the part in the service manual that says 2nd gear starts are

available - not true!

2)  The rear end makes a groaning sound while backing up and turning

at the same time.  (dealer says, "meets factory spec")

3)  The left front brake caliper makes clicking and popping noises

when braking at slow speeds.

4)  The driver's seat rocks back and forth during firm braking or

accelleration.  (Dealer says, "meets factory spec".)

Recently I had a little "race" on a country highway (late at night,

no traffic).  The other car was a new (300 HP Northstar) Cadillac El

Dorado Touring Coupe (ETC).  As we took off from the light, I punched

it first and he didn't seem to be interested, so I immediately let

off.  Just as I let off, he punched it.  So I punched it again but it

was too late.  He had gotten enough of a jump on me that I couldn't

catch him.  We went to about 80 MPH.  Needless to say I was a little

disappointed.  My father happens to have an ETC also, which he has

taken to the drag strip.  His best ET was 15.4 seconds.  It is so

closely matched to the SS that it comes down to who punches it first.

Happy motoring!

Greg Sullivan, (Southern California)  '96 Black, no mods. (except

window tint and chrome wheels).

------------------------------

Date: Sun, 3 Mar 1996 09:22:30 PST

From: aas@riker.neoucom.edu (Anuj A. Shah)

To: impala@east.xsis.xerox.com

Subject: Boy was I beat!-I love this car!

Message-ID: <9603031722.AA00330@riker.neoucom.edu>

To all:

I just had to share this story with others.  This is, I guess, a

turnaround from most of the "dragging" stories that are posted...

I was driving to my medical school in my 1990 Beretta GTZ (a great car for

me) on good'ol I-76 in Ohio going my usual 75-80 mph, when this dark

Cherry Impala flashed across my rear view mirror.  My heart started

pumping like a fish out of water.  He came around to my right, and I

couldn't just let him pass without a fight (Even though I knew I had no

chance in hell).  He played with me until about 90 mph, after which he

blew me out of the water so fast, that he left a vision of twin tailpipes

dancing in my head.  I have two things to say after this:

1) Whoever this guy was, I want to shake your hand.

2) I'M GOING TO OWN A 96 BLACK IMPALA SS SOMEDAY!!

- I guarantee it!!

Anuj A. Shah			aas@riker.neoucom.edu

NEOUCOM				(216)-677-3922

1537 Whitehall Blvd.

Kent, Ohio 44240                 1990 Chevy Beretta GTZ

1975 Chevy Impala SportCoupe

(sky-blue with white vinyl top)

------------------------------

Date: Sun, 3 Mar 1996 09:47:20 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: '96 Impala's for $100 over invoice!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Message-ID: <960303174719_76165.1654_GHM96-2@CompuServe.COM>

In digest 318 Jim Paulsgrove wrote:

>>>P.S. Rumor has it that a dealer in Jasper Florida has a few Impala's on his

lot that will go for $100 over invoice.  <<<

Jim,

For the benefit of all of the new members to this list and to all who may have

forgotten,

Lee Chevrolet in New Britain, CT is taking factory orders for 1996 Impala SS's

for

$100 over invoive. This is a fact, no rumor. You can reach them at 860-229-0345.

Ask for John Gallicchio and tell him I sent you. He will take very good care of

you.

Kurt Kemmling

'95 Dark Cherry Impala SS....bought from Lee Chevrolet for $100 over

invoice........September 1994..

------------------------------

Date: Sun, 3 Mar 1996 09:50:47 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com, lavera.twine@hok.com, erunnio@infohwy.com,

Subject: Latest Vericom Performance Results

Message-ID: <199603031749.LAA05363@raven.sat.net>

I did some more testing with the Vericom today and found some interesting

things.  The last time I ran the car, the humidity was 100% (heavy fog).

This time, it is 42% with the temperature at 63 degrees.  I won't bore you

with all the numbers except the pertinant ones.  If anybody wants all the

data, let me know and I'll send it direct.  In addition, I made a pair of

runs WITHOUT the air filter just to see if that made any difference.  It

did... the car slowed down!  All numbers are the average of two runs, both

directions on the same road to average out wind.  This is a bone stock

Impala as far as the running gear goes.  Enjoy!

Steve

100% RUN        42% RUN         W/O FILTER

TIME

1/4 Mile        15.84           15.765          15.84

1/8 Mile        10.325          10.285          10.335

0-60.....       7.495           7.4             7.49

0-30.....       2.855           2.83	        2.86

SPEED

1/4 Mile        89.45           90.05           89.55

1/8 Mile        72.5	        73.05           72.75

60 Ft....       28.85	        29.05           28.9

DISTANCE

To 60 MPH:      383.5 Ft.       377.5           382.5

To 30 MPH:      66 Ft.          64.5            65.5

Interesting to note that the car without the air filter performed about the

same as the car in 100% relative humidity.  The air box in this car is

stock.. no extra holes drilled.  I know a filter will straighten out the air

(I'm an OLD drag racer roo) but I don't believe with the path the air has to

take in the Impala, that this is a factor.  Obviously, I did not run the car

long enough for the computer to relearn the operating conditions but none

the less, I find the data interesting.

Incidentally, I did a stopping distance test from 60 on an asphalt road and

the results are pretty impressive.  How about 60 - 0 in 123 feet and 2.95

seconds!

Later,

Steve...sdas@sat.net

------------------------------

Date: Sun, 3 Mar 1996 09:58:30 PST

From: Loren Peterson <equ_rino@cencom.net>

To: "impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: HD AC Oil filter?

Message-ID: <3139DDC6.7CE@ns.cencom.net>

I know F%*# makes a Heavy Duty FL1A oil filter.  Does GM make a Heavy

Duty version of the PF52?  Is it necessary?  I have went so far as to

use Fram which is I am sure a great oil filter, but kinda like to stick

with the Mfg. own filters.  Is plain ole STP oil treatment worth a s%#*?

I've used it off an on thru the years.  Anyone heard any studies which

gave an answer if the stuff is good/bad or whatever?

Loren DCM96 SPE

flowmasters

TS-A6970 Pioneers rear deck

MM6520 for the doors....next.....soon....COURAGE!

cut air box/K+N

Hypertech foil

LETS GET SIDEWAYS!

Texas or Bust

------------------------------

Date: Sun, 3 Mar 1996 10:16:45 PST

From: Steve Das <sdas@sat.net>

To: BGordon401@aol.com

Cc: impala@east.xsis.xerox.com

Subject: Re: 2nd gear starts

Message-ID: <199603031815.MAA06708@raven.sat.net>

At 10:13 AM 3/3/96 -0500, BGordon401@aol.com wrote:

>In a message dated 96-03-02 21:54:30 EST, you write:

>

>> It will

>>be interesting to see if anybody can verify that their car truly starts in

>>second.  I'm betting not.  Thanks for the response.

>

>Steve, please let me know if someone come up with a good write-up for the 2nd

>gear start.  If we can get the proper information together, our dealers will

>be getting lots of SSer visiting them soon.

>

Bruce,

I'm sure there will be much more posted on the list.  Hang in there for the

latest!

Steve

------------------------------

Date: Sun, 3 Mar 1996 10:22:33 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Valvesprings

Message-ID: <960303182232_73145.1566_FHD62-1@CompuServe.COM>

Rob,

>Nope, he's running the stock springs. I told him that we're all changing

springs to the K-Motion units, and that you've found some valve float

problems with the stock springs. He replied saying that he regularly runs it

up to 6200 with those springs and his driven over 2K street miles with them

and has had no problems.

If he has an F/Y-car LT1 with the stock springs, then valve float is not the

problem, however COIL BIND is! Us B-car LT1 owners do NOT have a clearance

problem (we are 0.092 inches from bind with stock valvesprings and 1.6

rockers), however our weaker valvesprings will float at 5000 rpm, which is

why I recommend the K-Motion #700 springs. The K-Motion valvesprings are not

appropriate for an aluminum head (F/Y-car) LT1.

The combination of an otherwise stock F/Y-car LT1 and the 1.6 roller rockers

results in the valvesprings being compressed to within 0.030 inches of coil

bind, which is way too close for comfort! He can rev all he wants without

float, but he is dangerously close to binding up the whole valvetrain

(especially if he gets the engine hot and the clearances get smaller), not

to mention that compressing a spring that close to bind makes it much more

likely to fail, and such failure will trash the engine.

One should never install either a higher lift cam or rockers without

checking spring, pushrod, rocker, etc. clearances but of course people do it

all the time. Fortunately for F/Y-car owners the solution is simple, just

bolt in a set of factory LT4 valvesprings ($80). This will put the F/Y-car

LT1 0.155 inches away from coil bind, which is a very generous clearance

indeed.

>Also, do you have any insight into this second gear transmission thing? Is

it even possible that the PCM can cause this, or is the transmission

physically forced into second when the lever is in the "2" position? John

Levitz said you had the tranny manuals and might be able to shed some light

on this.

Yes, the PCM definitely controls the 2nd Gear Start function, and not the

transmission valve body. This is covered in the '94-'95 (but NOT '96)

Service Manuals under "Transmission Diagnosis", where the Manual Second gear

hydraulic flow is shown. This is also covered in s similar section in the

4L60-E Technician's Guide. Here it states:

"First Gear Prevented - The prevention of First gear is controlled

electronically by the PCM through shift solenoid "A" (the 1-2 shift valve

solenoid). The PCM keeps shift solenoid "A" de-energized regardless of

vehicle operationg conditions when the PSA (Pressure Switch Assembly)

signals Manual Second Gear range. This keeps signal "A" fluid (against the

1-2 shift valve) exhausted and spring force holds the 1-2 shift valve in the

upshifted position"

The '94 Impala PCM software indeed has this function, but apparently this

feature has been removed from the '95 and '96 PCM calibrations! One of the

aftermarket PCM programmers such as Hypertech, Z-Industries (Evergreen), or

Superchips may be able to add that function back in to the newer models.

BTW, I found out in researching this that GM has been fooling around with

the Service manuals! Apparently the additional OBDII stuff has caused them

to remove other information. For example, the '96 manuals for B/F/Y cars

completely lack the "Transmission Unit Repair" section that details

dissasembling and servicing the transmission!! Also all of the '96 manuals

have a really wimpy "Transmission Diagnosis" section that now lacks all of

the oil flow diagrams showing the state of oil flow and solenoid valve

engagement in each of the ranges and transmission modes. To be fair, the '96

manuals are stamped "Preliminary", so perhaps they will be adding this

missing info back in.

It almost seems as if they are ready to make the transmission a

non-rebuildable assembly, which is simply removed and replaced. This type of

thing has happened before, for example all of the older '70s and '80s

manuals used to detail things like re-building the alternator, starter

motor, A/C compressor, and now the manuals don't show how to do this, they

just want you to replace these items. Perhaps the automatic transmission is

going this way also?

Fortunately all of the 4L60-E transmission information is available to me in

the '94-'95 manuals, as well as the separate 4L60-E Technician's Guide. The

'94-'95 manuals feature a detailed Transmission Diagnosis section, as well

as the Transmission Unit Repair section, which essentially shows how to

completely dissassemble and rebuild the trans.

The 4L60-E Technician's Guide features theory of operation, a complete parts

breakdown, power flow diagrams, and complete hydraulic circuit diagrams for

every possible mode and condition. This guide has fantastic graphics in

color and would be ideal for anybody wanting to learn automatic transmission

function (which I find fascinating myself). You can get the Tech. Guide

direct from Helm (800-782-4356), same as the Service Manuals.

They also have available a killer 25" x 19" full color poster showing a

cutaway illustration of the 4L60-E complete with part call-outs, as well as

a set of 4 color wall charts which show exploded views of the transmssion

and grahpically show parts interaction. These are designed for use in

teaching transmission servicing and repair. Scott.

------------------------------

End of IMPALA Digest 321

************************

IMPALA Digest 322

Topics covered in this issue include:

1) SEO Part Numbers and Prices

by "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

2) Sunroof Installation

by Loren Peterson <equ_rino@cencom.net>

3) Need part numbers

by Loren Peterson <equ_rino@cencom.net>

4) Re: GM performance Parts

by Rock528@aol.com

5) Welcome

by Rock528@aol.com

6) Traction! I need Traction!

by lavalle@nwlink.com (Russ LaValle)

7) Front Brake Popping

by mbailey@rinet.hunton.com (Bailey, Martin)

8) 2nd gear start verified!

by glen.novitsky@nashville.com

9) Re: 2cd gear starts

by Kurt Kemmling <76165.1654@compuserve.com>

10) Re: 2cd gear starts

by Kurt Kemmling <76165.1654@compuserve.com>

11) Re: 2cd Gear Starts

by Kurt Kemmling <76165.1654@compuserve.com>

12) Re: disappearing posts?

by Kurt Kemmling <76165.1654@compuserve.com>

13) availability - space saver spare tires

by Loren Peterson <equ_rino@cencom.net>

14) Re: IMPALA digest 316

by LFriend121@aol.com

15) Re: IMPALA digest 321

by Joe Cotter <jcotter@edge.net>

16) Re: IMPALA digest 321

by Mbmarx@aol.com

----------------------------------------------------------------------

Date: Sun, 3 Mar 1996 04:44:00 PST

From: "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

To: impala <impala@east.xsis.xerox.com>

Subject: SEO Part Numbers and Prices

Message-ID: <3139E935@po5.cp.unisys.com>

Here is another list of part numbers, and updated prices, I purchased these

parts on 3/1/96.  MUCHO thanks to Scott Mueller for the part numbers.   The

list prices went up by 5% from the ones Scott send me and the discount is

about 19%.  Absolutely no hassle from the dealer about the export Headlight

assys.

SEO 1T1 Silicone Rubber Hoses:

Qty     Part #          Desc                          List       Net

------------------------------------------------------

1       10260863        HOSE ASM         37.50   30.38

1       10197619        HOSE ASM         57.75   46.78

1       10256544        HOSE ASM         58.00   46.98

1       10258505        VALVE ASM        92.67   75.06

1       10219447        HOSE-COOL         9.15    7.41

SEO 7L9 Power Steering Fluid Cooler:

Qty     Part #          Desc                         List   Net

------------------------------------------------------

1       26036034        PIPE ASM-        27.00   21.87

1       2091638         CLAMP -LO         1.35    1.09

SEO 7P8 Engine Oil Cooler (Oil-to-Air):

Qty     Part #          Desc                          List     Net

------------------------------------------------------

1       10226222        COOLER AS      75.25  60.95

1       10225895        PIPE ASM-         11.10   8.99

1       10225896        PIPE ASM-        11.10   8.99

1       10260341        HOSE ASM-      43.25   35.03

1       10219157        BRACKET-E         1.00   0.81

2       15966491        CLIP                         1.85   1.50

3       12337917        NUT,RR BP         0.92    0.75

3       20351035        SCREW                0.73    0.59

SEO B48 Trunk Mat:

Qty     Part #          Desc                      List       Net

------------------------------------------------------

1       10211580        MAT TRUNK        75.25   60.95

Export T84 Capsule Headlamps:

Qty     Part #          Desc                            List      Net

------------------------------------------------------

1       16519237        CAPSULE A       256.00  207.36

1       16519238        CAPSULE A       256.00  207.36

2       6288471         CONNECTOR         0.86    0.70

6       6294068         TERMINAL               0.47    0.38

On a dreaded personal note my 17 year old daughter wants to take the, yet to

be delivered , SS to prom She says it would be " way cool" over renting a

limo which a lot of kids pool together to do.  I guess it would be a good

memory maker!

Don Fischer

'96 it's on the train up!

------------------------------

Date: Sun, 3 Mar 1996 11:43:16 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Sunroof Installation

Message-ID: <3139F654.48AD@ns.cencom.net>

Hey all,

I was conversing with one of ya'll recently about power sunroof

installation, but my records are lost.  Can anyone tell me about your

experience with sunroof installation?  What did the interior look like

after installation?  The headliner is contoured and I wondered what it

might look like after installation.  Dont want a weird chopped up deal.

Thanks in advance.

Loren DCM96

SPE

Lets get sideways

Texas or bust

------------------------------

Date: Sun, 3 Mar 1996 12:31:04 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Need part numbers

Message-ID: <313A0188.711D@ns.cencom.net>

SSers

I am getting a shopping list together for some parts I will order soon.

Does anyone have the part number(s) for silicon heater/radiator hoses?

How about #'s for police wiper arms?  There was some discussion earlier,

but I am not sure which or what to do, because two prices were given.

Which F-car elbow is recommended to replace our holey SS air intake

elbow (pt.#)?

Pleeeeeze, any word on home base, Arlington??

Loren DCM96 SPE

Lets get sideways......we shall

P.S. Hope some of those SS's from hell with all those killer mods show

up at Arlington!!!!!

------------------------------

Date: Sun, 3 Mar 1996 14:06:26 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: GM performance Parts

Message-ID: <960303170624_339802574@mail04.mail.aol.com>

In a message dated 96-03-03 13:28:38 EST, you write:

> Just thought it was kinda neat if GM

>does do this, because it makes "hopping them up" all that much easier.

>They're even making their own intake and exhaust pieces for the

>Camaro now, so I'm guessing there is something to be gained.

I have been in recent contact with the manager of GM performance parts.

They are very interested in our group for feedback of upcoming Impala

aftermarket stuff like intakes and other items not yet defined.  He will

contact us when they are ready.  I was able to get a couple of GMPP banners

for our Dallas show area.  Troy

------------------------------

Date: Sun, 3 Mar 1996 14:08:05 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Welcome

Message-ID: <960303170619_339802598@emout07.mail.aol.com>

> I have two things to say after this:

>

>		1) Whoever this guy was, I want to shake your hand.

>

>		2) I'M GOING TO OWN A 96 BLACK IMPALA SS SOMEDAY!!

>                   - I guarantee it!!

and with a flash of blinding light, yet another Impala purist was born.

Welcome to our world.

Troy <g>

------------------------------

Date: Sun, 3 Mar 1996 14:59:52 PST

From: lavalle@nwlink.com (Russ LaValle)

To: impala@east.xsis.xerox.com

Subject: Traction! I need Traction!

Message-ID: <v01530500ad5f60a0251d@[206.129.23.39]>

I didn't see a good consensus about whether removing the spare helps more

in the overall weight reduction department when removed than it does in the

weight distribution area when installed.

I've have mine out sonce I think overall weight reduction is more

important. Am I right? That's what seems to be the philosophy when going to

the strip with 1/4 tank of gas.

How about relocating the battery to the trunk? Any problems with this?

I'd like to lighten the front as much as possible. Aluminum heads would be

nice. I'm sure replacing all the resonators & air box with the Callaway

honker has helped by a few pounds.

What else can be done to improve traction? Traction bars? Gears? Different

tires (that last almost as long)?

Thanks,

Russ

-------------------------------------------------------------------------

Speed Is Life.

-------------------------------------------------------------------------

------------------------------

Date: Sun, 3 Mar 1996 14:52:45 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (impala)

Subject: Front Brake Popping

Message-ID: <1996Mar03.174100.1049.109830@rinet.hunton.com>

It has been written:

>3)  The left front brake caliper makes clicking and popping noises

when braking at slow speeds.

I had the very same problem.  After working through all the suspension and

steering bits and pieces, I gave up and took it to the dealer.  It was an

improperly lubricated slide pin for the front brake caliper.   It's fine

now.

Martin Bailey, mbailey@hunton.com or mbbatty@ix.netcom.com

------------------------------

Date: Sun, 3 Mar 1996 09:13:01 PST

From: glen.novitsky@nashville.com

To: impala@east.xsis.xerox.com

Subject: 2nd gear start verified!

Message-ID: <96Mar3.152515pst."14733(15)"@alpha.xerox.com>

Steve,

I> From: Steve Das <sdas@sat.net>

I> Bruce,

I> I'm betting that they all work the way yours and mine do.  Several of the

I> other guys have written to me confirming the same thing you found.  It

will

I> be interesting to see if anybody can verify that their car truly starts

in

I> second.  I'm betting not.  Thanks for the response.

I>

WELL, tried out the Second Gear start on my 95 **Caprice** LT1 today.....I

can VERIFY the fact that when the selector is in "2", it **starts**  in

second, and stays in second until you hit the rev limiter.....It will even

"upshift" from 1st to second while **slowing** under 10 m.p.h. to zero if

you shift from D or OD to second!!!!!

Whatever I do the car WILL NOT go into first when its in 2.  Maybe, and

help us out here Scott, the owners books are wrong for the Impalas, and the

second gear starts are only good for the 2.93 rear-end Classics as many of

these (not mine, thankfully!) do not come with the limited slip as ALL

Impalas do!!! Could be the ECM at work here!!  Just a thought!!!! I'm about

to make a purchace of a Hypertech programmer...As soon as I get it and do

the reprogramming and I'll see if the 2nd gear start goes away.

-Glen

p.s., wheres the cheapest place to order a programmer?  All responses will

be appreciated!!

------------------------------

Date: Sun, 3 Mar 1996 15:52:54 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: 2cd gear starts

Message-ID: <960303235253_76165.1654_GHM62-2@CompuServe.COM>

In digest 321 Scott writes:

>>>I can assure you that all of the '94 cars do feature the 2nd Gear Start, at

least mine does! What I have found out is that this is directly controlled

by the PCM program and is not a function of the mechanical transmission

assembly itself. So Chevy apparently changed the calibration for '95 and '96

B-cars to remove this function, which is too bad as it is very useful in the

winter or in wet driving conditions<<<

Scott,

Did you see my post in the digest right after you first asked the question? I

have an early build '95

VIN sequence # 1030xx. It was built in late July, early August 1994. I noticed

soem other differences

from my car and other '95's also, such as minor underhood details and whatnot.

I know my car starts in 2cd gear from a stop when I place the shifter in that

position.

I mentioned in my earlier post about how this has helped tremendously this

winter.

Do you think the early '95s still had this feature, since I am sure there is

always some

production overlap between model years. I know my '90 F-250, again an early

build, has many differences from my friends '90 F-250 late build. What do you

think Scott?

Kurt

Early '95 Dark Cherry Impala SS...VIN Sequence 1030xx.....starts in 2cd

gear......

------------------------------

Date: Sun, 3 Mar 1996 15:52:56 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: 2cd gear starts

Message-ID: <960303235255_76165.1654_GHM62-4@CompuServe.COM>

In digest 320 Steve Das writes again:

>>> It will

be interesting to see if anybody can verify that their car truly starts in

second.  I'm betting not.  Thanks for the response.<<<

Steve,

I am still willing to bet you. How about pink slips?<G>This way I can have one

'95 that starts in 2cd gear, and one that doesn't. (for all of you that may

think I am ranking on Steve, I am not, he and I know we are only kidding around

as we have gone thru this before, right Steve...;-)

KK

'95 Dark Cherry Impala SS...early build...1030xx...VIN Sequence #...

.starts in 2cd gear..and I'll bet on it.

------------------------------

Date: Sun, 3 Mar 1996 15:52:57 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: 2cd Gear Starts

Message-ID: <960303235257_76165.1654_GHM62-5@CompuServe.COM>

In digest 320 Steve Das writes:

>>>Tell you what ED, I'll bet they are ALL the same and the other guys just

can't tell it's shifting.<<<

Steve,

Bet! I'l put up $1000. Are you covered?<G>

I have driven Fords for a long time, and have also driven cars with automatics

that I have added Trans-Packs to. I know the difference between 2cd gear starts

and 1st gear starts. My August 1994 built '95 Impala SS starts in second gear

when I put it in 2cd from a standing start. I can force it to downshift if I

floor it, but with a normal takeoff, it starts in 2cd and holds that gear.

Maybe it is only a feature of the '94's and early '95s, but I'll take you up on

that bet if your game.<G>

Kurt

'95 Dark Cherry Impala SS....starts in 2cd gear when I want it to..........

------------------------------

Date: Sun, 3 Mar 1996 15:52:52 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: disappearing posts?

Message-ID: <960303235252_76165.1654_GHM62-1@CompuServe.COM>

In digest 320 Franklin Poole writes:

>>>sorry to do this but my posts have all been disappearing for 8 days now!<<<

Franklin,

Just to let you know, I have seen about 8 or 9 of your posts in this list in the

past week. Maybe for some reason you have turned off the option to see your own

messages in your email software? I know some programs offer this option so you

do not have to read your own post if you do not want to. I hope this helps.

Kurt

'95 Dark Cherry Impala SS...starts in 2cd gear all the time now!.see what you

did Steve!<G>

------------------------------

Date: Sun, 3 Mar 1996 16:03:45 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: availability - space saver spare tires

Message-ID: <313A3361.538F@ns.cencom.net>

Are spacesaver spare tires readily available from wrecking yards?  What

type of cars did the space saver spare tire originally come in?

Is this the best place to obtain these spares?  Has anyone purchased new

from dealer?  Cost?

Thanks,

Loren DCM96 SPE

------------------------------

Date: Sun, 3 Mar 1996 17:19:07 PST

From: LFriend121@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 316

Message-ID: <960303201906_159170340@emout04.mail.aol.com>

In a message dated 96-03-01 12:56:10 EST, you write:

>

>Date: Thu, 29 Feb 1996 14:26:49 PST

>From: bngwip@mail.phoenix.net (Brett A. Rudolph)

>To: impala@east.xsis.xerox.com

>Subject: IRC

>Message-ID: <Chameleon.960229143023.bngwip@dialin.phoenix.net>

>

>Do any of you guys ever talk on irc or aol? If anyone is interested,we

should

>

>get together and do a live ImpalaSS channel chat thing sometime. and discuss

>Arlington or Super Chevy or Jeremy's lack of good sense or what ever. Just a

>thought.

>

>

Good idea.  I am new to this club, (and have sent in my 20)  I would be

interested in talking.  I'm into about 7,000 miles on my chevy (black '95).

The thing is just getting better and better all the time.  I get more people

wanting to talk about it when they see it.  I also have a '63 SS.  Sometimes

I drive the '95 to the swap meets instead of the '63 with just about the same

number of people looking at it.

------------------------------

Date: Sun, 3 Mar 1996 17:23:59 PST

From: Joe Cotter <jcotter@edge.net>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 321

Message-ID: <2.2.32.19960304012359.006f2c20@edge.net>

Any truth to the rumor a couple of issues ago that said something about the

factory issuing a recall on Impala relating to the front end? I took mine to

my friendly Nashville dealer and they swore up and down they had never had a

problem like this before. But since they didn't even have an SS on the lot,

my guess is they don't sell too many....

Joe Cotter

1996 Green/Grey Impala SS

He's not a cop, he's from Ohio!

------------------------------

Date: Sun, 3 Mar 1996 17:35:35 PST

From: Mbmarx@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 321

Message-ID: <960303203219_339997199@emout07.mail.aol.com>

In a message dated 96-03-03 13:29:58 EST, you write:

Scott,

>

>It almost seems as if they are ready to make the transmission a

>non-rebuildable assembly, which is simply removed and replaced. This type of

>thing has happened before, for example all of the older '70s and '80s

>manuals used to detail things like re-building the alternator, starter

>motor, A/C compressor, and now the manuals don't show how to do this, they

>just want you to replace these items. Perhaps the automatic transmission is

>going this way also?

>

This is in fact the case, GM's small front wheel driver will have a NON

serviceable

transmission for '97.  Tranny will be totally sealed with no drains, pan

etc.

Fluid is supposed to last the life of the transmission. I read this in one of

the rags and they had some pictures of this unit.

Is this scary or what!

You indicated that you flashed  94 f-body code to a b-body pcm. Was there any

difference in performance? I know you used the larger MAF so

this comes into play also. Were the shifts firmer? shift later? etc.

The differences would be interesting to see.

Mike Marx

95 Black, usual mods

------------------------------

End of IMPALA Digest 322

************************

IMPALA Digest 323

Topics covered in this issue include:

1) Performance Testing

by dhflores@iwl.net (Debbie/Henry Flores)

2) Re: IMPALA digest 319

by LFriend121@aol.com

3)

by LFriend121@aol.com

4)

by Steve Das <sdas@sat.net>

5) Head Hunt

by dhflores@iwl.net (Debbie/Henry Flores)

6) Newbie checks in ...

by Scott Hollums <ash@antec.com>

7) Re: Texas trip

by Curt Platteborze <75511.1220@compuserve.com>

8) Re: Texas travel guide

by Curt Platteborze <75511.1220@compuserve.com>

9) Re: Moonroof

by Curt Platteborze <75511.1220@compuserve.com>

10) LT4 valvesprings

by Scott Mueller <73145.1566@compuserve.com>

11) Re: Front Brake Popping

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

12) RE: HD AC Oil filter?

by "Harris, Scott" <sharris@xyplex.com>

13) PLEASE help me

by Mikado95@aol.com

14) Various Part Numbers

by Franklin Poole <franklin@intersurf.com>

15) Car show, etc.

by Franklin Poole <franklin@intersurf.com>

16) Arlington cruise

by Franklin Poole <franklin@intersurf.com>

17) Shamu won't go into 4th

by Franklin Poole <franklin@intersurf.com>

----------------------------------------------------------------------

Date: Sun, 3 Mar 1996 17:34:06 PST

From: dhflores@iwl.net (Debbie/Henry Flores)

To: impala@east.xsis.xerox.com

Subject: Performance Testing

Message-ID: <96Mar3.173741pst."14568(8)"@alpha.xerox.com>

Gentlemen,

I went out to Houston Raceway  park this afternoon to test the mods on my

Impala.  The beast ran 14.943 at 93.8 mph.  I will tell you that with the

3.42 gearing you must feather it off the line or you will not hook up even

on a sticky track surface.  My first run was a 15.5 at WOT immediately from

the green light.  It smoked the rears for quite a way thus the poor time.

However, during the second run I feathered the throttle off the green and it

hooked up early (I actually heard the chirp)  and then I stood on it all the

way down.  I am still having problems with the 2-3 shift. It still hangs

there and I do believe that with the sift kit in place I can cut at least

one more tenth.

My current Performance Mods: Borla exhaust, 52mm throttle body with air

foil, Hennessey Cold Air induction, Power Pulley, Hypertech Programmer, 3.42

gears, Jacobs Ultra Team Ignition System, Enlarged Mass Air Housing,

I find my track performance data interesting because of an article I

recently read in a Super Chevy Mag.   They profiled an Impala and their data

indicated that with the Hennessey intake alone it reduced times by 3 tenths!

Heck, my Impala needed the Hypertech Programmer, the Hennessey unit and the

Borlas in order to reduce the same 3 tenths at the track (previous track

test).  Do you think I have a gremblin somewhere?  In that same article,

they then installed the Jet Module and the car was in the mid 14's!! Now

then, that is pretty stout considering the mods they made.  Either I have a

significantly poorly tuned car or that is one bad Impala in the article.  Do

any of you have tips on tuning my beast?  I have yet to remove the homplate

baffle--could this be the trick?  The car was rather hot since I drove it 30

miles to the track and pulled into the staging lanes--could this be the

trick? Should I purchase a Pratt and Whitney F-16 afterburner? Fellows, I

need your expertise.

I am going to install the Transgo shift kit this week and take it back out

for baseline testing.  I will also index the spark plugs to unshroud

combustion spark. I will use a new set of plugs and then take her back out.

What else would you suggest besides blowers and nitros?

Henry in Houston

------------------------------

Date: Sun, 3 Mar 1996 17:39:38 PST

From: LFriend121@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 319

Message-ID: <960303203937_159187044@emout08.mail.aol.com>

In a message dated 96-03-02 17:50:23 EST, you write:

>: Sat, 2 Mar 1996 06:36:11 PST

>From: dstabb@epix.net (David Stabb)

>To: impala@east.xsis.xerox.com

>Subject: What are the Best Performance Mods?

>Message-ID: <v02130503ad5e0b1522f0@[199.224.67.48]>

Actually this sounds good to me.  I am interested if anyone has tried any of

the performance mods with the new computer upgrade that is being advertized.

I will be looking into this in the next couple of months.

------------------------------

Date: Sun, 3 Mar 1996 17:54:02 PST

From: LFriend121@aol.com

To: <impala@east.xsis.xerox.com>

Message-ID: <199603040154.UAA19171@emout05.mail.aol.com>

96-03-03 01:53:00 EST, you write:

>

>If your interested, I can tell you how I added a Auto Meter Tach to my Blk

>'95 so it looks like it sits on the dash just above the left part of the

>insrtument area with NO additional holes! In fact it has become somewhat of

a

>trademark of mine.

>

>RAZZORFL.

I am interested.  I want a tach but do not want something that looks bad.

------------------------------

Date: Sun, 3 Mar 1996 18:53:21 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: Kurt Kemmling <76165.1654@compuserve.com>

Message-ID: <199603040252.UAA00912@raven.sat.net>

>In digest 320 Steve Das writes:

>>>>Tell you what ED, I'll bet they are ALL the same and the other guys just

>can't tell it's shifting.<<<

>

>Steve,

>

>Bet! I'l put up $1000. Are you covered?<G>

>I have driven Fords for a long time

WOW, now there's something to be proud of!!  <G>

And I thought we weren't going to use dirty words (F%$#) here.

Maybe it is only a feature of the '94's and early '95s, but I'll take you up on

>that bet if your game.<G>

OK, OK, so I would have lost the bet.  Wonder when they changed it and

why??? Any body out there know the reasons???

Steve (the loser...this time)

------------------------------

Date: Sun, 3 Mar 1996 19:27:43 PST

From: dhflores@iwl.net (Debbie/Henry Flores)

To: impala@east.xsis.xerox.com

Subject: Head Hunt

Message-ID: <96Mar3.193120pst."14460(15)"@alpha.xerox.com>

Gentlemen,

I am looking for a set of LT1 heads for my Impala.  I just got back from the

track and I want more power ................Argh, Argh, Argh, Argh.

Nevetheless, a local performance shop indicated that if I could find a set

of Camaro Aluminum heads, he can do his majic to them for about $1200.  He

offers these prices on a exchange basis. Do you think this is a wise choice

or should I return to Air Flow Research or Lingenfelter?

Henry in Houston

------------------------------

Date: Sun, 3 Mar 1996 19:30:57 PST

From: Scott Hollums <ash@antec.com>

To: impala@east.xsis.xerox.com

Subject: Newbie checks in ...

Message-ID: <199603040330.WAA17210@antec.com>

Hi all,

I have been following this list for about 3 weeks now.  I thought that I should

introduce myself.  My curiosity about the car was peeked from the moment

that I heard that Chevy was building them.  I test drove a car in early '95,

fell in love with it, and knew that I had to have one.  I finally bought my

black beast in August, straight off the lot at a dealership here in Atlanta.

I have never enjoyed owning a car this much before.  But I am sure that I am

preaching to the choir here.  I only wish that girls stopped and looked at

the car as much as the guys do <g>.  Oh, well, I guess this is mostly a guy

thing.  (Wouldn't it be a great world if our cars really were as much of a

babe magnet as us guys think they are? <BG> Please, no flames about me being

a sexist, male chauvinist pig.)

I am not much of a gear-head so I haven't made any mods to my car.  I did

take it in to the dealer last week for it's 6000 mile oil change.  Thanks to

this list I had a list of things to be checked on the car.  When I called to

make the appointment to bring the car in I told the service rep about what I

had read in this list and on the Impala SS home page.  He wanted to read the

articles so I printed them out and took them with me.  I had the

differential gasket replaced (I bought a Fel-Pro gasket and took it in with

the car).  I had the exhaust hanger bolts checked (all were tight and none

missing).  I had the steering knuckles checked (proper torque).  The dealer

(Jim Ellis Chevrolet) did all this work for NO CHARGE!  Changing the diff

gasket, including replacing the fluid, was listed on my invoice as

"warranty" work.  Yes, I think that I will be going back to this service

department in the future.

This introduction is getting a bit long so I'll end for now.

Thanks to everyone.

Scott Hollums

'95 Black, Fel-Pro

------------------------------

Date: Sun, 3 Mar 1996 19:43:32 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Texas trip

Message-ID: <960304034331_75511.1220_GHJ99-2@CompuServe.COM>

New England residents

I changed my mind about making the trip to Dallas.

I want to get together with anyone from New England and put together a SS convoy

to play tag with along the way to Dallas.

We might as well make the most fun out of this long ride.<BAG>

Anyone interested please Email me direct so we don't clog up the digest.

Curt Platteborze

75511.1220@compuserve.com

------------------------------

Date: Sun, 3 Mar 1996 19:43:30 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Texas travel guide

Message-ID: <960304034329_75511.1220_GHJ99-1@CompuServe.COM>

To All,

There is a free 272 page travel guide for Texas available to anyone who who

requests it. I found it in the AAA magazine.

Telephone:

1-800-888-8539 ext. 485

------------------------------

Date: Sun, 3 Mar 1996 19:43:35 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Moonroof

Message-ID: <960304034334_75511.1220_GHJ99-3@CompuServe.COM>

Loren,

I had an American Sunroof installed before I took my Impala home from the

dealer.

Details:

A bit pricey ($850.00) but it's totally automatic with vent and a wind shield

that pops up. Push the open button and the vent closes and the roof opens full.

Both can be jogged to the desired opening 1/4-1/2 etc.... Also they saved the

upholstered top so that you can slide it closed to keep the sun out, but when

you push the open button it pulls the gray top back with it. The controls look

good and are mounted in the headboard. Also when you turnoff the engine the unit

closes automatilly or push the lock button to keep it open to vent the car when

it's locked. Top of the line. By the way I was extremely concerned that the roof

would have scratches or the inside would be a mess and neither! Also has not

leaked a drop. The installation was clean and looks factory.

Here's the address for ASC:

American Sunroof Company

Direct Sales

18900 Walnut Street

Southgate, MI 48195-9800

Tel: 800-327-8054

Call to find a installer near you.

Good luck,

Curt P. 95 Blk

------------------------------

Date: Sun, 3 Mar 1996 17:22:37 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: LT4 valvesprings

Message-ID: <960304012237_73145.1566_FHD59-1@CompuServe.COM>

John,

>Are you saying that the LT4 springs are a drop in on my aluminum LT1 heads.

I thought I had read somewhere (maybe in one of your posts?) that

re-machining is necessary for the LT4 springs. Yes or no?

The LT4 springs will drop right on the aluminum LT1 heads with no machining

required. However, to use the LT4 springs on the cast-iron B-car heads will

require that the spring pockets be machined to a larger diameter. The

smaller diameter K-Motion #700 springs will drop right on the B-car

cast-iron heads with no spring seat machining required. Scott.

------------------------------

Date: Sun, 3 Mar 1996 19:51:22 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: Re: Front Brake Popping

Message-ID: <Pine.SOL.3.91.960303194841.5025A-100000@netinfo.ubc.ca>

It has been written:

>3)  The left front brake caliper makes clicking and popping noises

when braking at slow speeds.

>I had the very same problem.  After working through all the suspension and

>steering bits and pieces, I gave up and took it to the dealer.  It was an

>improperly lubricated slide pin for the front brake caliper.   It's fine

>now.

>Martin Bailey, mbailey@hunton.com or mbbatty@ix.netcom.com

The problem with mine was diagonised as a loose steering knuckle.  And

they need to be replaced  Although I am not sure if this is the right

diagonose or not since theose replacement parts has yet to arrive from

the states yet.

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red (9C1 Air Deflactors, Gas Bib)

------------------------------

Date: Sun, 3 Mar 1996 10:28:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: HD AC Oil filter?

Message-ID: <313A7266@corvette.xyplex.com>

Loren,

First let me say that I'm glad to see you got email lessons.  ;-)

As far as I have found, the only "heavy duty" GM oil filter is the PF-35L

which does for F-bodies but does not fit the B-bodies.  It has a

synthetic element (totally unrelated to synthetic oil) which filters out

finer particles.  You could install a remotely mounted filter and use

this filter, or the FL1A, but in the stock location you are stuck with

the smaller diameter PF52.

STP Oil Treatment?  Only 2 words - Mobil One!

Scott Harris

sharris@xyplex.com

------------------------------

Date: Sun, 3 Mar 1996 20:38:49 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: PLEASE help me

Message-ID: <960303233848_236957781@mail06.mail.aol.com>

I sent in a request to this list about 4-5 digests ago, but I never got any

response.

Could someone please tell me what *exactly* to look for on the windshield to

tell if it's a SunGuard or not?   Thanks.

Randy

------------------------------

Date: Sun, 3 Mar 1996 21:19:27 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>, equ_rino@cencom.net

Subject: Various Part Numbers

Message-ID: <313A7D5F.4FB8@intersurf.com>

>SSers

>I am getting a shopping list together for some parts I will order soon.

>Does anyone have the part number(s) for silicon heater/radiator hoses?

>How about #'s for police wiper arms?  There was some discussion

>earlier,

>but I am not sure which or what to do, because two prices were given.

>Which F-car elbow is recommended to replace our holey SS air intake

>elbow (pt.#)?

>Pleeeeeze, any word on home base, Arlington??

>Loren DCM96 SPE

>Lets get sideways......we shall

>P.S. Hope some of those SS's from hell with all those killer mods show

>up at Arlington!!!!!

Loren,

I'll send you what you want late Monday.  We'll see you in Arlington.

somebody will be there with a twin turbocharged, intercooled,

supercharged, 4.11 rear gear, nitrous injected beast and we can all

watch it go up in smoke on the starting line, traction limited!

--

Franklin Poole, Jr.

Praecedo, Inc.

================================================

It's not how fast you go, it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Sun, 3 Mar 1996 21:36:11 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Car show, etc.

Message-ID: <313A814B.4C8@intersurf.com>

Wayne Nelson and I went to the local (sort of) monthly car show in

Gonzales, LA sponsored by a Baton Rouge radio station on Saturday.

We had a constant mob around our cars until I left at about 8:00PM.

I took the babies (twins) and my three year old and the wife to prove

that this is not only a high performance car but a cool family ride too!

I photocopied 30 or so copies of the front page of the NAISSO newsletter

from December and added my name and company address as well as the

NAISSO address and $20 dues info on the back.  Twenty copies were picked

up and at least 10-15 were interested in getting together with us in the

Baton Rouge area.  Wayne and I talked about a once a month or bimonthly

event.

I've appointed myself the Baton Rouge Area NAISSO Representative.

I embarassed a 5.0 Mustang w/his girlfriend on the two lane onramp on

the way home! (w/wife and THREE kids and two strollers)

A friendly young chap at the show had the most highly modified SS in the

Baton Rouge area there.  His SS has the ASM intake, 52mm throttle body,

ASM intake, ASM headers (UGLY!!!) and chrome ASM(?) injector covers as

well as somebody's 2.5inch exhaust.  Just before the show he installed

the TOP GUN nitrous system.

Last week (60deg) he ran on the local strip and turned just 14.99 with

all those mods.  Am I right in thinking that the larger intake, larger

throttle body, etc. is actually impeding his airflow?!?  He hasn't

gotten into the engine so he's got stock heads and valves, and rockers.

How about some input from one of you more informed Automotive Engineer

types like Scott M.  I've always heard that too large a throttle body

for example is just like a too large carburetor, you can't flow enough

air with a stock long block to take advantage...

--

Franklin Poole, Jr.

Praecedo, Inc.

================================================

It's not how fast you go, it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Sun, 3 Mar 1996 21:38:34 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Arlington cruise

Message-ID: <313A81DA.6341@intersurf.com>

How many who are cruising through BatonRouge, or would come up to meet

those cruising through would like a crawfish boil (with hamburgers as

backup) for your reception into Red Stick?

Email me direct.  I'll work this week on some hotel discounts, and I've

got some new ideas if I boil crawfish (with help, Wayne??).

--

Franklin Poole, Jr.

Praecedo, Inc.

================================================

It's not how fast you go, it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Sun, 3 Mar 1996 21:43:18 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Shamu won't go into 4th

Message-ID: <313A82F6.398E@intersurf.com>

Those of you with '96 SS's check for me.  Accelerate gently through all

the gears and note when you hit 4th and when you get lock up.  I'm

beginning to think my atypical gas mileage may be caused by an anomaly

and not my wild man driving habits.  It's beginning to really annoy me

not getting the thing to go into 4th by any means until 52mph, and she

still won't hit lock up until 60.  It's repeatable.

email me direct, so I'll know if I should go to the dealer.

PS: maybe Scott Mueller's info about the "preliminary" information in

the '96 service manuals explains the 5 week wait my friends and I are

having to receive ours.  MAYBE they're being revised.....It's really a

pain having you other guys quote pages from the manual and me having to

imagine the darn thing......

--

Franklin Poole, Jr.

Praecedo, Inc.

================================================

It's not how fast you go, it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

End of IMPALA Digest 323

************************

IMPALA Digest 324

Topics covered in this issue include:

1) Oil additives

by Franklin Poole <franklin@intersurf.com>

2) 2nd Gear Start

by Scott Mueller <73145.1566@compuserve.com>

3) Space Saver Spare

by Scott Mueller <73145.1566@compuserve.com>

4) F-car PCM program

by Scott Mueller <73145.1566@compuserve.com>

5) Re: Mistake

by Rock528@aol.com

6) NAISSO Flyer

by Rock528@aol.com

7) Florida to Dallas express, via Baton Rouge

by Rock528@aol.com

8) NEWSLETTERS

by Rock528@aol.com

9) Intake mods etc

by "Jeff Gatcha" <foxhound@txdirect.net>

10) Re: .

by Curt Platteborze <75511.1220@compuserve.com>

11) Impala Pricing, etc... (was Re: Hello to all!)

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

12) Heat Reflective Windshield

by Kelly Pratas <72537.655@compuserve.com>

13) SunGate windshield

by Scott Mueller <73145.1566@compuserve.com>

14) heads

by Scott Mueller <73145.1566@compuserve.com>

----------------------------------------------------------------------

Date: Sun, 3 Mar 1996 21:47:14 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Oil additives

Message-ID: <313A83E2.276D@intersurf.com>

>?with the Mfg. own filters.  Is plain ole STP oil treatment worth a s>%#*?

>I've used it off an on thru the years.  Anyone heard any studies which

>gave an answer if the stuff is good/bad or whatever?

You'll be bombarded on this one my friend but the short answer is:  You'll never

legitimately need any oil additive in any properly running engine.  Change the

oil at 3,000 miles and use your choice of oils (many owners use Mobil 1, but I

use Castrol) and always change the filter when you change the oil.  You'll never

have an oil related failure.

--

Franklin Poole, Jr.

Praecedo, Inc.

================================================

It's not how fast you go, it's how you go fast!!

$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

------------------------------

Date: Sun, 3 Mar 1996 21:59:58 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: 2nd Gear Start

Message-ID: <960304055957_73145.1566_FHD23-1@CompuServe.COM>

Kurt,

>Did you see my post in the digest right after you first asked the question?

I have an early build '95 VIN sequence # 1030xx. It was built in late July,

early August 1994. I noticed soem other differences from my car and other

'95's also, such as minor underhood details and whatnot.

Yes, the early '95 models have some of the features of the '94s including

the cleaner looking RH air injector hose, the extra coolant hose from the

resevior to the radiator, and maybe some other things as well.

>I know my car starts in 2cd gear from a stop when I place the shifter in

that position. I mentioned in my earlier post about how this has helped

tremendously this winter. Do you think the early '95s still had this

feature, since I am sure there is always some production overlap between

model years.

Since your car definitely has it, and is an early '95 production model, and

several other '95 owners have reported also having the 2nd Gear Start

feature, I must conclude that this was a feature that was removed during '95

model year production. There are usually several different PCM code

revisions during a model year, so perhaps the earliest one(s) had the 2nd

Gear Start function like the '94s. Scott.

------------------------------

Date: Sun, 3 Mar 1996 22:00:08 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Space Saver Spare

Message-ID: <960304060007_73145.1566_FHD23-2@CompuServe.COM>

Loren,

>Are spacesaver spare tires readily available from wrecking yards?

Yes.

>What type of cars did the space saver spare tire originally come in?

All GM B/D-cars came with the same space saver spare. This would include the

Caprice, Impala, Roadmaster, and Fleetwood which were originally equipped

with the N65 (Space Saver spare) option.

>Is this the best place to obtain these spares?  Has anyone purchased new

from dealer?  Cost?

The best place would indeed be a junkyard, just be sure to get one that has

never been used. Also be sure to get the cosmetic carpeted cover that goes

with it.

If you were to purchase the space saver spare new, you should know that you

cannot purchase the tire from a GM dealer, only the 16x4 steel wheel and

shorter carpeted cosmetic cover. The Goodyear Convenience Spare T145/80D16

tire can be purchased from any Goodyear dealer.

BTW, the N81 "Full-Size spare" that is included with the Impala is of course

not really full sized at all! It also sits 9 inches tall in the trunk,

wasting space IMHO. The space saver spare sits only 5.75 inches tall in the

trunk, saving 3.25 inches of space over the spare you have now, plus it

weighs less. I have the space saver spare (taken from a brand new Fleetwood)

in my Impala. Scott.

------------------------------

Date: Sun, 3 Mar 1996 22:00:14 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: F-car PCM program

Message-ID: <960304060013_73145.1566_FHD23-3@CompuServe.COM>

Mike,

>You indicated that you flashed  94 f-body code to a b-body pcm. Was there

any difference in performance?

It sure felt better, and according to back to back vericom testing the stock

F-car calibration performed the same as the Hypertech modified stock

calibration.

Although the quarter mile times and speeds were the same, the F-car

calibration FELT much better, it seemed like it had more midrange power, and

the biggest difference was in the shifting which was night and day better

than the existing Impala calibration. The shifts were much crisper, part

throttle shifts were later, TCC operation was crisper and re-engagement was

much quicker after getting off and on the throttle, etc. Upshifting,

downshifting and TCC operation were much much better than the standard

Impala calibration.

I suspect the times would have been even better with the F-car program,

however the F-car WOT shifts were a little late for the B-car engine. I

think the ultimate would be an F-car calibration modified with stock Impala

WOT shift points, and lower temp. fan operation. This also gives you a

functional "performance shift" button! Scott.

------------------------------

Date: Sun, 3 Mar 1996 22:52:59 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Mistake

Message-ID: <960304015259_340276121@mail02.mail.aol.com>

> my 17 year old daughter wants to take the, yet to be delivered , SS to

prom.....

PROM QUEEN + SILLY GIRLFRIENDS + CHAMPAGNE = $25,000 EMESIS BASIN.

besides, you'll never get the high heel marks out of the

headliner..........just kidding

Troy <g>

------------------------------

Date: Sun, 3 Mar 1996 23:52:21 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: NAISSO Flyer

Message-ID: <960304025220_159509101@emout04.mail.aol.com>

I have changed the impala handout flyer and uploaded it to SuperDave Hardy

for the web site.  It is in a zipped format from an original Wordperfect 6.0

document.  There are 2 flyers per 8.5x11inch page.  If there is a better way

to do it, let me know.  Dave should have it available in a day or so via the

WWW or FTP site.  The club application is forthcoming.  Thanx, Troy

------------------------------

Date: Mon, 4 Mar 1996 00:07:24 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Florida to Dallas express, via Baton Rouge

Message-ID: <960304030723_437450418@emout10.mail.aol.com>

In a message dated 96-03-04 00:48:15 EST, you write:

>How many who are cruising through BatonRouge, or would come up to meet

>those cruising through would like a crawfish boil (with hamburgers as

>backup) for your reception into Red Stick?

Franklin, count us in as a 1/2 way stop for Wednesday night prior to the

show, GUARRRONNTEEE.

Troy & company.

------------------------------

Date: Mon, 4 Mar 1996 00:07:43 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: NEWSLETTERS

Message-ID: <960304030742_437450413@emout09.mail.aol.com>

I am curious when the newsletters start arriving around the country.  Please

let me know from the East coast to the West and Canada and Europe (Thomas).

Thank you.

They were shipped from Daytona Beach, FL in bulk mail last Tuesday.

Troy

------------------------------

Date: Mon, 4 Mar 1996 00:28:48 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Intake mods etc

Message-ID: <199603040828.CAA17939@legend.txdirect.net>

I have a question for you gearheads out there..

Since making my "homemade" hennesy type intake (3.5 tube with cone

K&N)  and taking the mufflers off the car, I kind of thought it was a

bit slower off the line and poss slower in the 0-60 range.  I

definately have picked up more horsepower, as in stock form I could

barely break the wheels loose, but now I can smoke em for about 15-20

feet, however I still think it isnt as fast off the line.  When it

comes to speeds in excess of 60mph, the car flies. No doubt about it,

the mods have helped it breathe and it will prob outgun a stock

impala and a lot of Rustangs  from 60 on well past 100mph. I

certainly have trounced a bunch of F#&#s on the highway.   I think

opening up the intake  has  caused me to lose a bunch of torque.  Ive

heard that installing TOO oversized a throttle body (52 or 58mm) will

reduce the ammt of torque the motor produces, so IF this is true, why

not the same case with the intake mods?   Im really not sure if it is

my immagination or if my car is indeed slower. It has been so long

since I have driven a stock impala that I cant tell, and I dont have

a track around where I can make some runs with the car.  All I have

to test it with is a stopwatch but that doesnt allow me to concentrate on

getting a good launch and the nearest Vericom is 4 hrs away (right

Steve) <BG>

Is my line of reasoning correct?  or am I way off in my thinking?

Might it be the lack of mufflers?  Should I go to a reg old

aftermarket system like the walker or flowmaster?  Anyone have any

opinions on which system might be better (walker vs Flowmaster)  or

arent they out yet?

Will a PCM allow the motor to take full advantage of the intake and

exhaust mods (if I'm not already)?   As soon as I find someone competant

to put my 3.42 in (difficult in THIS area), I will be getting the Z-pcm.

I also noticed that while I dont have a 2nd-3rd gear shift hang like

some of you, my car DOES take a long time to shift from 1st to 2nd at

WOT. I noticed it in stock form (both in the test car I drove and the

one I bought)  but it is more pronounced now with the mods.  Its

almostas if 2nd is too tall or it is just taking its own sweet time getting

fully into 2nd.   Im hoping the PCM mod will fix this.

At WOT Id like it to kick me in the A$$ when it shifts.

Just wondering.  Im not at all sorry I have done my mods  as 95% of my

driving is done at 70mph right where the mods have helped the most.

Just thinking out loud and wondering what you all thought.

Thanks, and sorry about the length.

God, grant me the courage to change the things I can,

the serenity to accept the things I can't,

And the wisdom to hide the bodies of the people who really

pissed me off.

Jeff Gatcha

1995 "Modified" DGGM

Foxhound@txdirect.net

San Antonio Texas

------------------------------

Date: Mon, 4 Mar 1996 03:52:14 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: .

Message-ID: <960304115213_75511.1220_GHJ46-1@CompuServe.COM>

Hi ,

I would like to gat a list of all the current members on the list.

review impala

------------------------------

Date: Mon, 4 Mar 1996 06:10:37 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Cc: Stephen Furr <skfurr@raleigh.ibm.com>

Subject: Impala Pricing, etc... (was Re: Hello to all!)

Message-ID: <9603041414.AA23435@karloff.fstrf.org>

At 09:52 01/3/96 PST, you wrote:

>------------------------------

>

>Date: Fri, 1 Mar 1996 02:33:44 PST

>From: Stephen Furr <skfurr@raleigh.ibm.com>

>To: impala <impala@east.xsis.xerox.com>

>Subject: Hello to all!

>Message-ID: <9603011533.AA0994@rtpnsi01.raleigh.ibm.com>

>

>I've tried to avoid the common newbie pitfalls, by:

>

>1)  Reading the FAQ, and

>2)  Reading all (phew!) of the archives.

>

>I'm fascinated by the incredible amount of interest, and

>the high S/N ratio on this list.

>

>My name is Stephen Furr.  I live in Raleigh, N.C.

>I am 25, and work for IBM.  I just ordered my '96 Black

>Impala SS, and am anxiously awaiting delivery.

>

>Now my question:

>

>The dealer that I ordered from sold me the car for exactly

>$800 off of sticker.  From reading the archive, this may

>or may not have been a good deal.  Seeing as nobody in

>the area has a car in stock, I'm figuring that I did pretty

>good.  The car that I ordered was previously ordered as

>part of the dealers allocation on November 3.  The

>car has yet to be preferenced.  When I placed my order

>(this week,) the dealer said that he would update the

>allocation order to "sold vehicle" status and that this

>would cut my delivery time to under 4 weeks.  From

>reading the archive, it seems that I'm looking at least

>eight weeks, since the order hasn't been preferenced

>yet.  The dealer is also willing to "stock-transfer" one

>for me, but does not know where one is.

>

>Can anyone help?  This really bites.  If anyone knows

>where I can find a car like the one I ordered, I'd be

>eternally grateful!

>

>1996 Chevrolet Impala SS

>41U Black

>1SE Preferred Equipment Group

>UN0 CD Player

>A23 Heat reflective windshield

>

>I'd prefer NOT to have VK3 (Front License Plate Bracket)

>as in N.C. we don't use front plates, and I don't want holes

>drilled in my bumper!

>

>Thanks again for any suggestions.

>

>Stephen K. Furr

>

Group,

Sorry for taking up the space, but I thought this might interest others

(or newcomers) with the same question.

I don't have any advice or contacts for you to use in trying to get the same

car at a better price, but the following information may help give you a

point of reference if someone else can.

Here are the stats on purchasing my 1996 DCM Impala:

Vehicle @ Invoice: $22,331

Options @ Invoice:

1SE  Impala SS Sedan Preferred Equipment Group 1    $421

includes 6-way adjuster power passenger

seat, remote keyless entry w/power trunk

opener, and twilight sentinel

K05  Engine Block Heater                             $17

UN0  Radio                                          $133

includes electronically tuned AM/FM stereo

radio w/ seek/scan, automatic tone control,

CD player, digital clock, theft lock and

speed compensated volume; see package

A23  Heat Reflective Windshield                      $45

Destination: $590

Total Invoice: $23,537

I paid a total of $23,900 (plus tax) to a buying service for the factory order.

Local dealers that had some SS's in-stock wouldn't even COME DOWN to sticker!

I was told that the reason for getting the savings that I did was that I

opted to place a factory order, as opposed to having the buying service

search the country for an existing SS; It would seem that you should

see a greater savings that only $800.

Although it took a while the wait was well worth it, especially for the

savings.

The timeline was:

+ 25 Septermber 1995: Placed order with Dealer

+ 2 October 1995: Order accepted with Chevy (1 week delay)

+ 30 November 1995: Order preferenced (8 week delay)

+ 18 December 1995: Given production week (3 week delay)

+ 8 January 1996: Build week (3 week delay)

+ 17 January 1996: Given estimated delivery date of 31 January 1996

(Approx. 4 week delay)

+ 31 January 1996: Estimated Pick-up Date

+ 1 Febuary 1996: Actual Pick-up Date from a local dealer. (This was

a courtesy delivery from a dealer in Nevada.)

Total time: 18 weeks.

Good luck in your search!

- mike

------------------------------

Date: Mon, 4 Mar 1996 07:20:09 PST

From: Kelly Pratas <72537.655@compuserve.com>

To: Impala SS Mail <impala@east.xsis.xerox.com>

Subject: Heat Reflective Windshield

Message-ID: <960304152008_72537.655_EHM37-1@CompuServe.COM>

) I sent in a request to this list about 4-5 digests ago, but I never got any

) response.

) Could someone please tell me what *exactly* to look for on the windshield to

) tell if it's a SunGuard or not?   Thanks.

Randy,

Mine says SUNGATE Solar Ray Guard -- lower corner, passenger side.

Kelly

------------------------------

Date: Mon, 4 Mar 1996 07:33:32 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: SunGate windshield

Message-ID: <960304153331_73145.1566_FHD61-1@CompuServe.COM>

Randy,

>I sent in a request to this list about 4-5 digests ago, but I never got any

response.

Yes you did! Dave in So. Cal responded to you with the answer in digest 317.

>Could someone please tell me what *exactly* to look for on the windshield

to tell if it's a SunGuard or not?

I'll save you the trouble of looking for that response <g>, basically it

says "SunGate Shaded Solar Ray" on the lower RH (passenger) side if you have

the A23 Heat Reflective windshield. Of course another way to verify if your

car came with this originally is to look for the A23 RPO (Regular Production

Option) code on the SPID (Service Parts ID) label on the underside of the

decklid. Scott.

------------------------------

Date: Mon, 4 Mar 1996 07:34:04 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: heads

Message-ID: <960304153404_73145.1566_FHD61-2@CompuServe.COM>

Henry,

>I went out to Houston Raceway  park this afternoon to test the mods on my

Impala.  The beast ran 14.943 at 93.8 mph...

>My current Performance Mods: Borla exhaust, 52mm throttle body with air

foil, Hennessey Cold Air induction, Power Pulley, Hypertech Programmer, 3.42

gears, Jacobs Ultra Team Ignition System, Enlarged Mass Air Housing,...

>I find my track performance data interesting because of an article I

recently read in a Super Chevy Mag.   They profiled an Impala and their data

indicated that with the Hennessey intake alone it reduced times by 3 tenths!

Heck, my Impala needed the Hypertech Programmer, the Hennessey unit and the

Borlas in order to reduce the same 3 tenths at the track (previous track

test).  Do you think I have a gremblin somewhere?

Forget about ET, that is a factor of traction. Your MPH figure of nearly 94

indicates around 4 to 5 mph higher (40 to 50 HP) than stock! I'd say you are

doing real well. This improvement is exactly what you should expect from the

exhaust and air filter improvements that you made, as I'd factor each of

those mods as a 20 HP increase individually, or 40 HP (4 mph) for both. The

larger throttle body and air foil are probably not doing anything for you

(especially the air foil - those are a gimmick), and probably neither is the

ignition system. The stock throttle body flows more than enough compared to

the other areas of the induction system, and the stock ignition is fine in

your rpm range. Those mods did not get you anything, but then they did not

hurt either.

The "Power Pulleys" also did not get you anything, and in fact may hurt

ignition voltage plus you will be discharging your battery at idle with any

accessories on. This has been verified by performing some measurements with

these installed on an Impala, the maximum output capability drops to below

50 amps at idle, which is not enough to run a full load of accessories, in

other words, the headlights will dim at idle.

I would return to the stock pulleys right away! Those pulleys are a hoax as

any electrical engineer could tell you. For example, the alternator in the

Impala is a 140 amp unit putting out 13.8 volts, which would be 1,932 watts

assuming it was running at full output (of course it runs probably at half

output or less most of the time). At full tilt, that corresponds to 2.591

horsepower to run the alternator. I know it is not 100% efficient, so let's

round it up to 3 HP to drive the alternator at full tilt.

Now let's assume that when you are making your quarter mile run you don't

have any accessories on like the A/C fan, headlights, rear defogger, stereo,

etc. <g> Let's assume that the alternator is putting out about half of it's

rated power (actually it should be less), which takes only about 1.5 HP to

run. Now let's assume your pulley slows it down by 1/3 (I bet it does not

slow it down that much), which would save you a whopping 0.5 HORSEPOWER!!

What else does the belt run?, how about the A/C compressor. I don't know

about you, but when I am making runs down the quarter, I usually turn the

A/C off, at least for the run. <g> Actually, you don't have to do that, as

the PCM will automatically cut off the A/C compressor clutch at WOT for you,

thus insuring that it won't be taking any of your power.

The belt also runs the power steering pump, but that does not take very much

power at all to run, especially if you are not turning! <G> I bet it takes

about a quarter of a HP to run it when going straight, so the pulley change

would save you about 1/12 of a HP (1/3 of 1/4) there, not much to write home

about!

Unlike other engines, the LT1 drives the water pump from the cam, and the

pulleys don't come into play here. That is a blessing in disguise for those

running these pulleys, as otherwise their engines may overheat at idle. Also

the LT1 features electric fans, so no gain there.

I can assure you that whoever designed these "Power Pulleys" is not an

engineer! <g> As the most basic calculations prove, at best you would save

about 1 HP (or less), at the cost of reduced system ignition voltage and a

potentially discharged battery.

>In that same article, they then installed the Jet Module and the car was in

the mid 14's!!

Be aware that the magazines usually report "corrected" times, which is bogus

IMHO. The actual times would likely have been much less, and again you

really did quite well. Forget ET and focus instead on the HP (mph) your car

is generating.

>I have yet to remove the homplate baffle--could this be the trick?

Certainly the stock elbow is much more restrictive than the F-car unit. With

all you have spent on the other induction pieces, you really should treat

yourself to the F-car 1LE elbow!

>The car was rather hot since I drove it 30 miles to the track and pulled

into the staging lanes--could this be the trick?

Engine temperature, ambient temperature, humidity, etc. all play a HUGE part

in your times. Much bigger perhaps than any single mod you could do. That is

another reason the magazine times often look so good, they often apply

correction factors for altitude, temperature, and humidity. I would rather

just see the real time, and then a report on the ambient conditions so I

could draw my own conclusions. You will generally find that you will run

your best times after a 30 minute cooldown at the track.

>I am looking for a set of LT1 heads for my Impala.  I just got back from

the track and I want more power ................Argh, Argh, Argh, Argh.

Nevetheless, a local performance shop indicated that if I could find a set

of Camaro Aluminum heads, he can do his majic to them for about $1200.  He

offers these prices on a exchange basis. Do you think this is a wise choice

or should I return to Air Flow Research or Lingenfelter?

Get the new factory LT4 heads instead! They are now available much more

cheaply than has been reported before as a complete assembly under a single

part number (#12363287). This includes the heads, special larger lightweight

hollow stem (sodium filled exhaust) valves, the special LT4 springs, and

standard screw in LT1 style rocker studs.

These heads are completely assembled and ready to bolt on. They come with

the LT1 style adjustable studs instead of the shouldered non-adjustable

studs used on the LT4. This allows you to use any type of rocker (I would

recommend the Crane 1.6 roller rockers) instead of the expensive and

non-adjustable factory (Crane manufactured) rockers. The problem with the

factory type LT4 roller rockers is that they will only work with the LT4 cam

as they are non-adjustable for lash. The aftermarket Crane roller rockers

will work on these new factory upgrade LT4 heads as they come with the

proper adjustable (non-shouldered) LT1 type studs.

These heads come with the very expensive larger LT4 valves, which are not

only larger than the standard LT1 valves, but they are lighter as well! This

is due to the fact that they are hollow, and feature undercut heads. The

exhaust valves are filled with sodium/potassium, which helps to transfer

heat and keep them cool. These lighter valves allow the engine to rev much

higher without floating, even with 1.6 rockers.

These heads feature recontoured intake ports that are 0.100 inches higher

than the standard LT1 ports, so you need a matching LT4 intake manifold as

well. If you really want to go all out, send both the heads and intake to

Lingenfelter for CNC machining. I would trust Lingenfelter's CNC porting

before your local tuner (does your tuner have CNC equipment?), plus I

believe Lingenfelter's price is better! Scott.

------------------------------

End of IMPALA Digest 324

************************

IMPALA Digest 325

Topics covered in this issue include:

1) IMPALA digest 317 -Reply

by John Pinkerton <jpinkert@sigg.com>

2) RE: is my car stuck in performance mode??

by John Latrobe <racerx@jagunet.com>

3) Re: Second Gear Starts

by John Latrobe <racerx@jagunet.com>

4) Curt's sun roof

by "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

5) Check yours! and questions (was Re: '96 rattling)

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

6) '94-'95 Program Evaluation Car

by Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

7) Space Saver Tires

by Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

8) Jim Perkins

by Jim Ganninger <jtgstl@home.stlnet.com>

9) Intake (was Re: heads)

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

10) Power pulleys

by Scott Mueller <73145.1566@compuserve.com>

11) cost of performance

by Scott Mueller <73145.1566@compuserve.com>

12) Newbie Impala questions...

by Stephen Furr <skfurr@raleigh.ibm.com>

----------------------------------------------------------------------

Date: Mon, 4 Mar 1996 08:15:01 PST

From: John Pinkerton <jpinkert@sigg.com>

To: impala@east.xsis.xerox.com

Subject: IMPALA digest 317 -Reply

Message-ID: <96Mar4.100001cst.26497@firewall.sigg.com>

.....4L60-E transmissions should operate this way, but maybe there

was a design change for '95? Can everybody with a '95 or '96 verify

whether their Impala starts in 2nd Gear when the shifter is in the Manual

2nd range? Scott.

I have tested this feature and my 95 does not start in second gear.

Furthermore, if I place the car in First gear and mash on the gas - when I

reach apx. 50MPH the car will automatically shift to second.  Even

though the gear indicator still reads first.  I don't know if it will continue to

shift into third or OD if I keep going.

John Pinkerton

95 DCM, no mods.

------------------------------

Date: Mon, 4 Mar 1996 08:28:05 PST

From: John Latrobe <racerx@jagunet.com>

To: "'impala@east.xsis.xerox.com'" <impala@east.xsis.xerox.com>

Subject: RE: is my car stuck in performance mode??

Message-ID: <01BB09BD.FF437320@dlup-a30.jaguNET.com>

>Shamu my 2TONLT1 seems to be stuck in what would be called

>performance mode shifting.  She won't go to fourth gear until

>52mph no matter what and won't "lock up" until 61mph.  I have

>stock gears and stock PCM programming.  The only serious

>performance mods are the fresh air airbox cutout, no homeplate,

>no muffler thing.

The same thing happened to me and I haven't done any mods, yet :-)

Two dealers basically told me I was full of it, the car was supposed to

act that way. After I showed them the shift chart from the manual they

agreed to replace the PCM. It shifted exactly the same afterwards.

After 1 month, the 4th gear shift occurred correctly 50% of the time.

A month after that, it shifted right about 90%, and now (3 months) it

shifts correctly all the time.

According to the manual, only two inputs affect the shift points:

the throttle position sensor and the vehicle speed sensor.

John Latrobe

'96 Impala SS, Black

------------------------------

Date: Mon, 4 Mar 1996 08:28:00 PST

From: John Latrobe <racerx@jagunet.com>

To: "'impala@east.xsis.xerox.com'" <impala@east.xsis.xerox.com>

Subject: Re: Second Gear Starts

Message-ID: <01BB09BD.FAE80840@dlup-a30.jaguNET.com>

>I can verify that my '94 does start and remain in 2nd gear when in Manual

>Second range. According to all of the service and technical literature, all

>4L60-E transmissions should operate this way, but maybe there was a design

>change for '95? Can everybody with a '95 or '96 verify whether their Impala

>starts in 2nd Gear when the shifter is in the Manual 2nd range? Scott.

Some of you may remember the tranny problems I had a few months back where I couldn't shift into 4th below 50 mph. Also, with the shifter in

second, the tranny always started out in 1st. The dealer eventually

replaced the PCM which didn't fix either problem. I was told by the

dealer that the 4th gear problem was caused by my driving habits.

When I asked about the second gear start, they said Chevy told them

this might not work on all cars. (WHAT??)

John Latrobe

'96 Impala SS, Black

------------------------------

Date: Mon, 4 Mar 1996 02:19:25 PST

From: "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

To: impala <impala@east.xsis.xerox.com>

Subject: Curt's sun roof

Message-ID: <9603041629.AA4469@CServe-Hub4.inhouse.compuserve.com>

Loren:

>> I had an American Sunroof installed before I took my Impala home from the

dealer. <<

I was recently out in the Boston area, and was able to meet up with Curt to see

his SS.  I can second his comment that the sun roof he had installed looks

totally factory, actually better than some factory sun roofs I have seen on

other cars!  As Curt described in the previous post, it has all the goodies you

want in a power sun roof.  I know in the '91 Cutlass Supreme that the factory

sun roof ate up 3" of the interior head room.  This was not the case with

Curt's, in fact it seemed to add a little head room when open.  This is a mod I

am currently looking into, based on seeing how nice it looked on Curt's SS.

Jeff

'96 DCM

------------------------------

Date: Mon, 4 Mar 1996 09:06:55 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Cc: calanan@karloff.fstrf.org

Subject: Check yours! and questions (was Re: '96 rattling)

Message-ID: <9603041710.AA28073@karloff.fstrf.org>

Regarding the message below that I sent to the group in early February

about feeling a rattling through the console shifter:

>Date: Wed, 07 Feb 1996 11:11:50 -0500

>To: Impala SS

>From: "Michael C. Calanan" <calanan@fstrf.org>

>Subject: '96 rattling

..edit...

>

>The _only_ defect I've been able to find is a light clunking or rattling

>that can be felt through the console shifter as I'm driving.  The shifter

>itself is solid, and not loose - I can feel the clunking through it.  It feels

>like something is loose under the car...

..edit...

I was helping a friend install new polygraphite front-end bushings (just the

sway bar, and end links - from PST) in his Monte Carlo SS over the weekend,

and I decided to take my first in-depth look under the beast.  I wanted to

compare suspension designs, and take at look at how easy/hard it would be to

upgrage my bushings, and possibly the sway bar.  While underneath (and greasy

and dirty by then) I figured I would crawl the length of the car and inspect

every inch, keeping in mind the well-known defects to look for (like hanger

bolts, etc...).

What follows is a list of "discoveries:"

My main finding (and the reason for the "Check yours!" subject line)

was a loose nut that connects the shifting linkage to the tranny.

(Excuse me if my terminology is wrong, but I don't yet have the shop

manuals!)  The nut was only 3 turns away from falling off!!  I don't

know the proper torque, so I torqued it to the generic "down."

Once it was tightened, the rattling I felt through the shifter was gone.

Anyone know the proper torque?

Starting from the engine, heading back:

1) Although it's hard to confirm, I think that none of the grease fittings

were lubed from the factory/dealer.  If I twist the lower tie-rods in

the front, I can feel grinding at the ball joints.  My grease-gun is empty

and I'd like to know (without starting a

"my-grease-is-better-than-your-grease" war <G>) what type of grease would

be best to use.  Any recommendations?

2) Jammed between the left wheel well and some piping, I found two little

plastic caps, one red, one green, about 3/4" high & 1/2" in diameter.  I

have no idea what these could be from, but they look like they could have

capped a grease-gun, or glue-gun.

3) Fully surrounding the brake lines was a standard rubber band; the lines

pass though the center of the band.  It must have been on before the build,

so I left it on for posterity. <g>

4) The right-side sway bar bushing was not installed centered within the

clamp - it looks like over time this could cause major deflecting.

The scary thing, in comparing the quality of the PST parts we installed in

the Monte SS vs. stock, is that not only do the stock parts rust quickly,

but they're very frail-looking.  The end-link bolts themselves are very small

in diameter, just like the '88 Monte SS's (at least compared to PST's) and

after seeing his just snap off after some spirited driving <G>, I'm

sure my much heavier (and faster) beast could easily crack those suckers off.

Anyone know of good aftermarket front-end bushings for the '96?  I'll

be calling PST today (4 mar 96), but I'm not sure if polygraphite really

is the way to go.

5) I had been hearing some "tinny" rattling at idle, which sounded like a loose

heat shield.  Although the bolts were tight, a heat shield that covers an

area of about 12" x 10" next to the oil filter, "above" the oil pan in the

centerline of the chassis was rattling where it met the underside of the

tranny.  I removed it, stuck on some weatherstripping, and reattached it.

Rattling gone.  I hope the weatherstripping doesn't melt.

6) All exhaust hanger bolts were on and tight.

7) The right rear shock looks like it's mounted at a severe angle from the

trailing arm (?), even though it's attached tightly and the bolt looks

straight through the arm itself.  Also, the rubber bushing where the bolt

passes through is very deflected.  The left side's shock is not exactly

90 degrees from the arm, but the angle is not as severe as the right

side.

Even though there was nothing major wrong, other than the linkage nut, I'd

have to suggest checking out your beast's underside if you have not already

done so.  It truly is a bonding experience. :)

I've got some other questions now that I've inspected, but I'll bring them

up in another, shorter message.

Thanks,

- mike

God, I love this car!

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

My '96 Impala SS: "It goes fast, FAST!"

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

Date: Mon, 4 Mar 1996 11:14:01 PST

From: Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

To: impala@east.xsis.xerox.com

Subject: '94-'95 Program Evaluation Car

Message-ID: <313B40F9.1503@lfwc.lockheed.com>

Yesterday, I stopped at the local "Stop and Rob" to fill up, and what do

my eyes behold stopping on the other side of the gas island?  Another

black "SS".  The city in which I live is only 9,800 citizens strong, so

seeing a third "SS" surprised me. (Previously, I knew of only one other

in town.) In the spirit of comraderie, I struck up a conversation.  This

guy told me he works in the Tooling Department at the plant in Arlington,

and his car was a program evaluation car.  He picked it up with 3,00

miles on it, and it now has 25K. We got in a conversation about tires,

and he didn't seem to know Goodrich was the single supplier.  BTW, I

checked the tread depth on his tires and they had plenty to go before the

treadwear bars start showing.  He told me his wife drives the car, and he

just gets to fill it up for her.  I told him he should get his own. He

asked about some of the options on my '96, then we parted company. No big

deal - I just thought some of you might be interested in hearing the

story.

------------------------------

Date: Mon, 4 Mar 1996 11:13:43 PST

From: Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

To: impala@east.xsis.xerox.com

Subject: Space Saver Tires

Message-ID: <313B40E7.17C5@lfwc.lockheed.com>

Loren,

Why would you want a space saver spare?  Just curious.

Mark

------------------------------

Date: Mon, 4 Mar 1996 09:58:15 PST

From: Jim Ganninger <jtgstl@home.stlnet.com>

To: impala@east.xsis.xerox.com

Subject: Jim Perkins

Message-ID: <199603041758.LAA15943@home.stlnet.com>

I was talking with my brother-in-law last night at a family function.  He

owns a Chevrolet dealership here in St. Louis.   I was telling him about the

list and he was giving me trouble, saying the Impala is for "gray-haireds".

He said out of about 12 Impala sales over the last three years,  I was the

only one under the age of 50 to buy one(I am 27).   I told him he was crazy

and later I figured out he was just trying to push my buttons, although his

sales stats were true.

Anyway, he started to tell me about how Jim Perkins is being pushed out

of Chevrolet.  Evidently,  there is some disagreement on the way things

should be done, in particular the decision to discontinue the

Caprice/Impala.  So GM is going to combine some of the operations of Pontiac

and GMC, and they are going to replace Perkins with the guy from Pontiac.

This is to be effective May 1st.   He said that most of the dealers really

like Perkins and that he got Chevy pointed in the right direction again.

Perkins didn't want to retire till Chevy was #1,   but the General had

different plans.

I don't know if this is old news to list members or not,  but I have

been reading the list for a couple of months and don't remember seeing

anything.

I now have a stupid question for list members,   what octane is

everyone using?   I know the manual says at least 87,   I have only filled

up once so far.   Just gets driven around town.  Is alternating octanes bad

for the car?

Thanks,

Jim Ganninger

------------------------------

Date: Mon, 4 Mar 1996 09:52:05 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Subject: Intake (was Re: heads)

Message-ID: <9603041755.AA06212@karloff.fstrf.org>

At 07:39 04/3/96 PST, you wrote:

>------------------------------

>

>Date: Mon, 4 Mar 1996 07:34:04 PST

>From: Scott Mueller <73145.1566@compuserve.com>

>To: <impala@east.xsis.xerox.com>

>Subject: heads

>Message-ID: <960304153404_73145.1566_FHD61-2@CompuServe.COM>

>

>Henry,

>

..edit...

>>I have yet to remove the homplate baffle--could this be the trick?

>

>Certainly the stock elbow is much more restrictive than the F-car unit. With

>all you have spent on the other induction pieces, you really should treat

>yourself to the F-car 1LE elbow!

I know that the P/N for the 1LE has been posted a gazillion times for

'94-'95 Impalas, but what is the P/N for the corrensponding intake that

fits a '96, if it's not different.  I'd like to "do the right thing" and get

an OEM intake as opposed to hacking the stock one up.

>Scott.

Thanks,

- mike

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

My '96 Impala SS: "It goes fast, FAST!"

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

Date: Mon, 4 Mar 1996 10:51:57 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Power pulleys

Message-ID: <960304185157_73145.1566_FHD72-1@CompuServe.COM>

There is one fatal flaw in my calculations on the supposed power savings

from the bogus "Power Pulleys"! I stated that the CS-144 140 amp alternator

used in the Impala running under maximum load would put out 1932 watts (140

amps times 13.8 volts), which is about 2.5 HP, and which I rounded up to 3

HP figuring natural losses and inefficiency. This is fine. Then I stated

that during normal vehicle operation with most accessories off, running a

quarter mile, I would expect the alternator to be running at a 50% load, in

other words putting out about 70 amps and consuming a maximum of 1.5 HP.

This is fine also.

I then stated that if you reduce the alternator speed by 1/3 by using a

bogus "Power Pulley", that you would save about 0.5 HP (1.5 times 1/3). This

is where I screwed up!

Actually, an alternator will put out sufficient current to match the load,

as long as it is running beyond a certain minimum speed (just above idle).

In other words, if the vehicle electrical system is drawing 70 amps, than no

matter whether you run the alternator at full speed or at 1/3 less than full

speed, it will still be putting out 70 amps and drawing about 1.5 HP!! In

other words, these bogus "Power Pulleys" are actually worth no HP gain at

all, and the only effect you will get is that when you are at idle, the

alternator will no longer be running at sufficient speed to carry anything

higher than a 50 amp load. The Impala alternator system is designed to put

out between 72-108 amps at idle!

In other words, the only noticible effect is that while idling at night with

the headlights and several other accessories on, you will notice the lights

dim as you begin to discharge the battery. In fact, when you do eventually

drive away from the light, the alternator will then be running under an

INCREASED load (drawing MORE HP) as it now has to replace the charge in the

battery that was lost while idling!!

Let me just re-iterate what a total waste of time these bogus "Power

Pulleys" are! Some of them are also made from soft aluminum (for looks),

which wears in contact with the belt. This is atavistical engineering, or

performance evolution in reverse. I'd like to see the credentials of any

engineer who would admit to designing them! <G> Don't take this personally

if you have them installed on your car, as it is easy to be taken in by the

huckster claims made for these devices. Some of the ads for these things

claim they are worth 10 to 15 HP! I would sure like to see the creative

mathematics they used to come up with those totally bogus figures! I think

these pulleys are one of the biggest performance shams being sold today.

There, I feel better now that I finally have that off my chest. Sorry but

those things have been bugging me for a while! <g> Scott.

------------------------------

Date: Mon, 4 Mar 1996 10:52:21 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: cost of performance

Message-ID: <960304185221_73145.1566_FHD72-3@CompuServe.COM>

Steve,

>I don't know if Henry communicated to the list that he finally got

Hypertech to state what their tested horsepower improvement was... how about

8 typically?  Not much of an improvement for over 300 bucks in my mind. If

you need the speedo calibration, I guess it's a good deal but other than

that, I can think of a lot of ways to spend $300.00.

I agree (I think!) I factor the Hypertech at 10 HP, and I don't think there

is really much more power in the PCM. That power comes at the expense of

increased advance and the requirement for premium fuel. Adding more spark

and fuel than necessary will only make a properly calibrated engine

detonate, and run too rich, sending black smoke out the tailpipe (white if

you blow a head gasket <g>), not to mention damage the cats (if you still

have 'em, that is <g>).

On the other hand, I do recommend PCM changes not for the raw horsepower,

but for the improved driveability instead. That is one reason I like the

Hypertech, but especially liked the F-car PCM, it seemed much more

driveable, the midrange torque seems improved, and the transmission shifting

is dramatically better. I would guess that it also has 10 HP over the stock

PCM calibration, based on vericom measured testing. Unfortunately even it is

not perfect, as the F-car shift points are too high for the B-car LT1

engine, and the fan activation temps need to be reduced.

You have to be careful when you compare mods based on $ per HP. There are

many other useful mods that do not generate more HP, but they improve the

car nonetheless. The 3.42 gears would be the best example of that. I mean

they cost $500 to $750 installed, and you get 0 HP from them, but the car

sure is quicker! The '86 'vette torque converter is the same thing, better

driveability, quicker off the line, but no more actual HP.

Anyway, $300 does sound like a lot for only 10 HP, but that is about what 10

HP goes for, actually I think it is somewhat of a bargain! The 1.6 roller

rockers will set you back $400 with the new springs and keepers, and they

are only worth about 10 HP as well, plus they are much more difficult to

install than the Hypertech. You might get 10 to 20 HP from a $40 K&N cone

air filter, which is of course a performance bargain (that is unless you

spent $250 for a Hennesy or other "brand" $40 K&N filter)! An exhaust can

get you another 10 to 20 HP, but can cost anywhere from $50 to simply

replace your stock mufflers with straight pipe, to $800 or more for a custom

2.5" stainless system. The latter would get you your 20 HP, which is about

$40 per HP, which seems to be about the going rate.

LT4 heads would get another 30 HP, at a cost of probably $1,500 total, which

is about $50 per HP. There you would also get a 50 lb. weight savings. Add

in CNC machining, and both the HP and the cost per HP goes up. A cam is

relatively inexpensive ($150 or so), but the labor to install it will be

$300 or more, so for about $450 you might get another 20 HP. Boy this

performance stuff sure can get expensive, but is it ever FUN! <G> See you in

Arlington, Scott.

------------------------------

Date: Mon, 4 Mar 1996 00:18:52 PST

From: Stephen Furr <skfurr@raleigh.ibm.com>

To: impala <impala@east.xsis.xerox.com>

Subject: Newbie Impala questions...

Message-ID: <9603041908.AA0536@rtpnsi01.raleigh.ibm.com>

Now that I've gotten through the purchasing nightmare,

I thought I'd ask a couple of real questions!

When I was looking to buy the car, it was sitting in the

dealers service bay at idle with the hood up.  I reached

in and blipped the throttle a couple of times, using the

throttle body's accelerator linkage.  All of a sudden, it

stuck at WOT.  It sat there and banged against the

rev limiter for maybe 2 seconds before I reached in

and turned the key off.  Depressing the accelerator

pedal caused the throttle to "unhang."  I was unable

to reproduce this using only the pedal.  Is there

something in the linkage that I could have mistakenly

gotten bound up?  Is this normal???  Is this possibly

related to the campaign referred to in the archives

for the '94-'95 cars?  (It wasn't very cold that night,

maybe 55 degrees.)

Also, I noticed a very slight "shimmy" that felt like

it was coming from the front end during my test

drive.  It only occured at around 55-65 MPH

(I didn't get up any faster.)  I made it a "must-fix"

before I would accept the car.  Any ideas?

I'll find out this afternoon (when I pick up the

car) if they figured it out.

Thanks again!

Stephen Furr

------------------------------

End of IMPALA Digest 325

************************

IMPALA Digest 326

Topics covered in this issue include:

1) My sincerest thanks!

by Stephen Furr <skfurr@raleigh.ibm.com>

2) RADAR Detector

by "Parker, Terry" <tparker@atlanta.glenayre.com>

3) Was I Foiled?

by VSI!BP!Madsto@vrsy.attmail.com

4) FW: Bushings

by mbailey@rinet.hunton.com (Bailey, Martin)

5) RE:  2nd Gear Start

by ZTMP50A@prodigy.com ( DAVID NMN)

6) 1LE Intake elbow part numbers, again

by Franklin Poole <franklin@intersurf.com>

7) PST Polygraphite Bushings.

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

8) Power Pulleys

by Franklin Poole <franklin@intersurf.com>

9) Trunk Rattle

by bertran5@egr.msu.edu

10) Vericom on the WWW

by Steve Das <sdas@sat.net>

11) 2nd Gear Starts

by Ray Lopez <impalass@mail.erols.com>

12) 96 Service Manual

by ahavill@vcnet.com (Andrew Havill)

13) Re: Leakdown test

by Curt Platteborze <75511.1220@compuserve.com>

14) Re: Trans Go shift kit and vette converter

by Curt Platteborze <75511.1220@compuserve.com>

15) Re: IMPALA digest 322

by RAZZORFL@aol.com

16) Re: IMPALA digest 325

by Destiny Miller <destinym@lark.cc.ukans.edu>

----------------------------------------------------------------------

Date: Sun, 3 Mar 1996 17:07:24 PST

From: Stephen Furr <skfurr@raleigh.ibm.com>

To: impala <impala@east.xsis.xerox.com>

Subject: My sincerest thanks!

Message-ID: <9603041913.AA0668@rtpnsi01.raleigh.ibm.com>

Thanks to the several members that offered advise and information

in my search for a '96 Impala.  In the end, I found a '96 Black SS at

a local dealer with exactly the options I wanted, including the front

license plate bracket still unattached and laying in the trunk!

The car was ordered by another customer, and arrived at the

dealership two weeks ago.  Just Friday, the customer bailed out

as he had been trying for two weeks to secure financing, and

finally couldn't.  The car had been sitting on the lot with a

SOLD sign in it for those two weeks.  I walked up at 8:30 PM

Friday night, and bought it.  I didn't get as great of a deal as

I had hoped for, but since I didn't have to wait for 18-24 weeks,

I guess it was worth it.  (I still got $700 off of sticker.)

Picking up my freshly detailed car Monday afternoon.

Thanks again for all of your help!

Stephen Furr

------------------------------

Date: Mon, 4 Mar 1996 14:57:00 PST

From: "Parker, Terry" <tparker@atlanta.glenayre.com>

To: Impala <impala@east.xsis.xerox.com>

Subject: RADAR Detector

Message-ID: <313B4B26@gvnsmtp>

Hey Fellow SSers,

This weekend I performed the best modification to my baby yet. I installed a

Whistler 3000 wireless remote triband RADAR detector. It took about 1 hour

after a trip to radio shack.

I installed the antenna/receiver left of the air bag sensor that is below

the hood latch so that it gets a good look through the grill at all of our

esteemed law enforcement personnel.  Just a couple of small holes in the

radiator support and self threading screws and appropriate wiring and the

front was done. Next I removed the ashtray bracket and used the wires for

the lighter to power the console (it is a little larger that a pager) and

reinstalled the cup-holder/ashtray bracket and I was done. The console fits

in my cup holder in the above mention bracket and when I close my

ashtray/cup holder it is completely stealth. I could just unplug the console

and take it with me when I leave my SS, but unless I plan to valet park when

I travel I'll just close the ashtray.

I already have enough trouble trying to keep my foot out of the floor as it

is. Now I can feel better about those WOTs.

Does anyone have experience with the Whistler 3000? Is it any good?  I only

paid $50 for it from a guy that had it installed in his Jag.

Can't wait till I cruise down I-20 out of Atlanta towards the ocean at WOT!.

In the words of RAZZOR... Don't you just  LOVE IT!!!! ;>)

Regards,

Terry Parker

Atlanta, GA USA

95 Dark Cherry, Someone Stole Home Plate, Chrome 3.5", RED Bow-tie

"I am bound upon a wheel of fire"  (king lear)

------------------------------

Date: Mon, 4 Mar 1996 06:09:00 PST

From: VSI!BP!Madsto@vrsy.attmail.com

To: impala@east.xsis.xerox.com

Subject: Was I Foiled?

Message-ID: <MS-MAILG-3.00-Note-vrsy-Madsto-0825968229>

Hey Scott big fan!

You wrote in #324

>The larger throttle body and air foil are probably not doing anything for

you

>(especially the air foil - those are a gimmick), and probably neither is

the

>ignition system

Alright so air foils are pretty much bunk? I also got done reading the Super

Chevy mag in which they do a test with the bolt on's to a 94vette and they

had the TPI air foil on it plus TPI air filter with plug wires. Any way I

was figuring since I was soon  going ahead with the 1LE elbow, 13" chrome,

and cone I would throw the air foil in there.

Has anybody had improvements they could attribute to the airfoil or should I

just return after I get it?

Tom

95 Black (stored til spring)

------------------------------

Date: Mon, 4 Mar 1996 12:38:30 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (Impala1)

Subject: FW: Bushings

Message-ID: <1996Mar04.152700.1049.110509@rinet.hunton.com>

Anyone know of good aftermarket front-end bushings for the '96?  I'll

be calling PST today (4 mar 96), but I'm not sure if polygraphite really

is the way to go.

Whether you are talking about new bushings for your existing swaybar, new

swaybars or control arm bushings, I think it would be hard to beat Herb

Adams/Matt Adams Very Special Equipment's products.  I have installed their

swaybars and there is simply no comparison to the stock ride.   While the SS

wasn't a lumbering brute before installation and while it's no gokart

afterwards, it corners MUCH better with these monster bars.  Moreover, Scott

M. has made the installation a relative cinch with his detailed

instructions.  I am in the process or ordering the front control arm

bushings on the theory that if some is good, more is better!

BTW, you can buy Herb Adams universal front sway bar replacement bushings

through your local speed shop as Moroso part number 87100 or call Herb Adams

direct at (408)649-8423.  I know nothing about this product, although once

you see the HA swaybars you'll never go back.

Martin Bailey

mbaile@hunton.com

mbbatty@ix.netcom.com

------------------------------

Date: Sun, 3 Mar 1996 13:40:31 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: RE:  2nd Gear Start

Message-ID: <097.02719666.ZTMP50A@prodigy.com>

Hi All!.  After reading all the posts on the 2nd gears starts, I

tried it on my '96.  With the shifter in '2', it definitely starts

from first and shifts to second and stays there.  You definitely feel

the 1-2 upshift and can see the rpms drop also on the tach also.

Take care

Dave in So Cal.  '96 Black SS, tinted windows, and soon to be drilled

Airbox..<BG>

------------------------------

Date: Mon, 4 Mar 1996 13:58:58 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: 1LE Intake elbow part numbers, again

Message-ID: <313B67A2.53D4@intersurf.com>

>I know that the P/N for the 1LE has been posted a gazillion times for

>'94-'95 Impalas, but what is the P/N for the corrensponding intake that

>fits a '96, if it's not different.  I'd like to "do the right thing" and get

>an OEM intake as opposed to hacking the stock one up.

same for '96 (I have one)

94 elbow (cheap one) 25147210 (no vent nipple, make your own and hide it)

95 elbow (get raped) 25147187 (has vent nipple, $60 more expensive at retail)

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Mon, 4 Mar 1996 14:04:40 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Cc: calanan@karloff.fstrf.org

Subject: PST Polygraphite Bushings.

Message-ID: <9603042208.AA22290@karloff.fstrf.org>

I called PST (1 800 247 2288) to see if they have polygraphite

sway bar bushings and end links for my '96, and they do carry them.

The clamps and bushings for the sway bar are $25 / pair, and the

end links (complete - bushings + hardware) are $25 / pair.

My questions are (again, hopefully without starting a war <G>) :

Are the polygraphites worth it for our B-body's?  The difference was great

for my friend's '88 Monte SS, but that's a whole other (smaller) ball

of wax.  Would just upgrading those two areas make a difference?

And, does anyone have the most up-to-date information on Herb Adams'

sway bars + kit for the '96 Impala's (pricing, reviews, etc...) or at least

a phone number?  I may just go for the gusto if it's worth it.

Thanks,

- mike

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

My '96 Impala SS: "It goes fast, FAST!"

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

Date: Mon, 4 Mar 1996 14:23:33 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Power Pulleys

Message-ID: <313B6D65.142D@intersurf.com>

>Let me just re-iterate what a total waste of time these bogus "Power

>Pulleys" are! Some of them are also made from soft aluminum (for looks),

>which wears in contact with the belt. This is atavistical engineering, or

>performance evolution in reverse. I'd like to see the credentials of any

>engineer who would admit to designing them! <G> Don't take this personally

>if you have them installed on your car, as it is easy to be taken in by the

>huckster claims made for these devices. Some of the ads for these things

>claim they are worth 10 to 15 HP!

You are right as far as your statements go but I think there

is an underlying issue involving the physics.  I'm not a physicist

or really qualified to give a dissertation on the matter but

I think I can spout the gist of it:  with a smaller driving pulley

and a larger driven pulley you are indeed lessening the effort

necessary for the driving pulley to turn the driven pulley.

Think about it.  On your old ten speed bike you could shift down

into first and go straight up the side of a mountain with relatively

little effort.  You still weighed 150lbs, and you still put out 1.5HP

(or whatever) but you climbed the hill.  Shift into high and try that.

I think the same thing will pass through on the so called power

pulleys.  I'll agree that I'd have to see a dyno test to believe

10-15HP but I think there could well be some (however negligable)

advantage during drag racing or (very) high speed work.  Since

my 2TONLT1 is only turning about 2.2 grand at 80MPH I doubt

there would be a measurable mileage increase though.

Does this make sense or am I out to lunch with the power pulley people?

Any physics majors out there???

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Mon, 4 Mar 1996 14:24:49 PST

From: bertran5@egr.msu.edu

To: impala@east.xsis.xerox.com

Subject: Trunk Rattle

Message-ID: <9603042224.AA16330@egr.msu.edu>

Hey Guys,

Has anybody experienced a minor rattle in the trunk, only when you

hit washboard type of small bumps?  I get this minor klunk-klunk-klunk that

sounds like a piece of pressboard hitting metal lightly, and it is driving

me up the wall!!  Also, I seem to be getting rotor pulsation.  Has anyone

else warped their rotors (either front or rear)?  Thanks for the input!

Erik Bertrand

------------------------------

Date: Mon, 4 Mar 1996 15:34:18 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: Vericom on the WWW

Message-ID: <199603042332.RAA03145@raven.sat.net>

Hey guys,

If you are interested, Vericom now has a page on the web.  You'll find them

at; http://users.aol.com/vcomputers

This is courtsy of Fuel Injection Specialties.

Later,

Steve

------------------------------

Date: Mon, 4 Mar 1996 16:26:53 PST

From: Ray Lopez <impalass@mail.erols.com>

To: impala@east.xsis.xerox.com

Subject: 2nd Gear Starts

Message-ID: <199603050026.TAA18648@mail.erols.com>

Fellow SS owners:

I have a 96 Impala SS with a build date of 10/95, I took the car out today

to verify this "question of second gear starts", well from a complete stop

and the lever in second, the SS starts in first then shifts to second. I did

this a couple of times with the same results.  The question: Is it supposed

to work this way?

Today on the way home from work my wife drove the SS home, as I was trying

to follow her in her Dodge Caravan ES, a 95 F#&D Garden-Snake with a nice

looking blond pulled up beside the SS, the blond decided she no longer

wanted to play after my wife and her got into an acceleration test. My wife

is well aware to show who rules if any of these F#&Ds pull up and want to

play, I guess this Garden-Snake decided to go into hibernation.

Ray & Angie

9 SS 6

------------------------------

Date: Mon, 4 Mar 1996 17:06:09 PST

From: ahavill@vcnet.com (Andrew Havill)

To: impala@east.xsis.xerox.com

Subject: 96 Service Manual

Message-ID: <v01540b00ad61424f179a@[205.228.248.45]>

Waiting for me on the doorstep today was my brand spanking new 96 Service

Manual.  I thought

I'd mention this since several other list members have made comments about

delays in receiving

theirs.  I ordered mine 7 days ago direct from Helm (while waiting on hold

for 20 minutes in

order to speak to a rude, gum chewing order-taker) - it arrived today.

I've only spent a few minutes with it so far, interestingly it mentions the

oil pressure gauge and voltmeter, neither of which is in fact on the 96SS.

Kind of makes me wonder......  Briefly looked through the transmission

section to see if there is any mention of a 2nd gear start, but didn't see

anything.  I may have missed it though.

For those of you who ordered yours  from Helm, and haven't received it yet,

all I can suggest is to call them up and give them *heck*.

Andy   96 DCM, home plate-ectomy.

------------------------------

Date: Mon, 4 Mar 1996 19:33:45 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Leakdown test

Message-ID: <960305033344_75511.1220_GHJ121-2@CompuServe.COM>

Had the leak down test done today and saw some improvement. Cylinder #6 was only

at 4% at 100PSI.

Unfortunately #8 was at 20%. Also with the vent hose removed from the valve

cover there was about 20PSI coming out. Like there was a tiny fan blowing

through the hose. This confirms a ring problem with # 8 cylinder. I will redu

the test this Sat and if it's the same, remove the Crane rockers and replace the

old valve covers.

I will leave the springs and keepers as they are and bring the partially mauled

beast to the dealer. I assume if and when they realize what the problem is  will

replace the block or entire engine. So I don't want to hassle getting the Crane

parts off after the fact. The springs and keepers will stay as they are not that

expensive to replace and will require more time than it's worth to remove them.

Curt P

------------------------------

Date: Mon, 4 Mar 1996 19:33:42 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Trans Go shift kit and vette converter

Message-ID: <960305033341_75511.1220_GHJ121-1@CompuServe.COM>

Installed the Trans Go kit and vette converter last Friday night. One word

"POWER".

This setup is amazing with stiff hard shifts and 1-2 barking till you want it to

stop. "Even with my impaired #8 cylinder."

The shift kit took a bit more time than I thought and two of the stock

components you had to reinstall backwards. I kept looking at the instructions

with disbelief and called Trans Go's tech line to confirm this possible diagram

error.  Yes you do mount them upside down from stock. I used all three washers

for the hardest shifts.

The Vette converter went in with out a hitch. Not sure where I picked up the

most gain but It's done now.

We removed the cats for easier access to the trany which made the whole job go

fast. I'm not sure I would have wanted to do any of this without a lift. The

Trans Go video sure makes the job for the shift kit look easy with just a

creeper under the car.

I was also able to tighten any loose bolt and nut I saw and also installed the

front air brake deflectors.

Can't wait for Saturday to retest the #8 cylinder and know for sure what's up. I

believe I already know the answer. Also it's going to be a pain talking to the

dealer about this and pushing for a new block or whatever. I'm sure not looking

forward to this.

Keep you posted!

What about an SS convoy from New England???????

Curt P.

------------------------------

Date: Mon, 4 Mar 1996 19:40:23 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 322

Message-ID: <960304224022_438217443@emout07.mail.aol.com>

In a message dated 96-03-03 20:39:17 EST, you write:

>Does anyone have the part number(s) for silicon heater/radiator hoses?

>How about #'s for police wiper arms?  There was some discussion earlier,

>but I am not sure which or what to do, because two prices were given.

>Which F-car elbow is recommended to replace our holey SS air intake

>elbow (pt.#)?

#10260863  upper hose asm$36.25

#10197619  lower hose asm$55.75

#10256544  heater hose asm$56.00

#10258505  heater valve asm$91.75

#12522853  coolant hose      $12.45 not required on later '95's

#10219447  resvr. hose         $8.85 Thanks to troy for this info

wiper arms #5049730 order one for each side if you like.

elbow #25147210 This will require you to put a new hole for your dist. hose I

put mine under the elbow to help clean up the appearance.

RAZZORFL.

------------------------------

Date: Mon, 4 Mar 1996 20:05:01 PST

From: Destiny Miller <destinym@lark.cc.ukans.edu>

To: impala@east.xsis.xerox.com

Cc: Multiple recipients of list <impala@east.xsis.xerox.com>

Subject: Re: IMPALA digest 325

Message-ID: <Pine.OSF.3.91.960304215837.31387A-100000@lark.cc.ukans.edu>

Jim's MESSAGE:

> I was talking with my brother-in-law last night at a family

function.  He> owns a Chevrolet dealership here in St. Louis.   I was telling him about the

> list and he was giving me trouble, saying the Impala is for "gray-haireds".

> He said out of about 12 Impala sales over the last three years,  I was the

> only one under the age of 50 to buy one(I am 27).   I told him he was crazy

> and later I figured out he was just trying to push my buttons, although his

> sales stats were true.

FYI--

I'm a 26 year old female. Proud owner of a black 1995 Impala SS.

This is

my first car and I purchased it just last July. It only has 3,800 miles

on it so far, but I plan on taking a few road trips this summer with

other teacher friends.

Can't wait for summer,

Destiny

Kansas City

------------------------------

End of IMPALA Digest 326

************************

IMPALA Digest 327

Topics covered in this issue include:

1) Re:2nd gear start

by BGordon401@aol.com

2) 96 C 14 Recall Notice

by BGordon401@aol.com

3) Impala Pricing

by starone@gnn.com (Jack)

4) Re: IMPALA digest 326

by RAZZORFL@aol.com

5) Re: IMPALA digest 326

by RAZZORFL@aol.com

6) Space Saver spare

by Scott Mueller <73145.1566@compuserve.com>

7) Off the *true* subject

by Mikado95@aol.com

8) LT4 heads

by Scott Mueller <73145.1566@compuserve.com>

9) Hello [Newbie Post]

by Christopher Massa <massa@intac.com>

10) Grey Hairs

by dstabb@epix.net (David Stabb)

11) Newbie Impala questions...

by Stephen Furr <skfurr@raleigh.ibm.com>

12) "gray-haireds"

by jeffd@accessus.net (Jeff Dawson)

13) Power pulleys

by Steve Das <sdas@sat.net>

14) list of problems

by "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

15) Funny story

by David <71223.325@compuserve.com>

----------------------------------------------------------------------

Date: Mon, 4 Mar 1996 20:14:54 PST

From: BGordon401@aol.com

To: 76165.1654@compuserve.com

Cc: impala@east.xsis.xerox.com

Subject: Re:2nd gear start

Message-ID: <960304231452_160348451@emout09.mail.aol.com>

Kurt, you wrote:

>> It was built in late July, early August 1994. I

noticed

soem other differences

from my car and other '95's also, such as minor underhood details and

whatnot. <<

My SS has a build date of Dec. 1994 for my 1995 model.  I can assure you that

my 2nd gear start doesn't work.  (RATS!!)  Sometime between your date and my

date they must have converted the PCM programming to reflect the model year.

..Bruce

------------------------------

Date: Mon, 4 Mar 1996 20:13:44 PST

From: BGordon401@aol.com

To: jcotter@edge.net

Cc: impala@east.xsis.xerox.com

Subject: 96 C 14 Recall Notice

Message-ID: <960304231344_160348478@mail02.mail.aol.com>

Joe, you wrote:

>>Any truth to the rumor a couple of issues ago that said something about the

factory issuing a recall on Impala relating to the front end?<<

I can assure you that the RECALL notice I have is true.  It came on an index

card, which is the same as the last recall for the shift indicator.  It reads

as follows:

OFFICIAL NOTICE

NATIONAL TRAFFIC AND MOTOR VEHICLE SAFETY ACT - 1966

CAMPAIGN NO.  96 C 14

SERIAL NO. 1G1.........................

Chevrolet Motor Division notified you previously that your Chevrolet listed

above requires a safety campaign in compliance with the National Traffic and

Safety Act, for the safety of you and your family-contact us immediately to

make an appointment to have this correction completed.  Bring this card with

you.

Failure to correct this may result in notification of the Department of

Transportation-License and Registration Division.

THIS CAMPAIGN CAN BE PERFORMED AT ABSOLUTELY NO CHARGE AT OUR DEALERSHIP OR

ANY AUTHORIZED CHEVROLET DEALER.

TOM HESSER CHEVROLET/BMW INC.

ETC..., ETC...

If you all haven't received a notice yet, you probably will shortly.  I don't

have the specifics of the recall, b/c the form letter from Chevrolet has not

come yet.  I haven't had the time to make that call to the dealer either.

I'll try to call tomorrow if time permits.

..Bruce

------------------------------

Date: Mon, 4 Mar 1996 15:11:45 PST

From: starone@gnn.com (Jack)

To: impala@east.xsis.xerox.com

Subject: Impala Pricing

Message-ID: <199603050413.XAA04983@gnn-2.gnn.com>

I'm new to this list so I thought I'd introduce myself and chirp in on the

pricing data.  I have a 96 SS Black.  No Mods yet but I have a mechanic

checking in on things and he will make suggestions.

I ordered the car 10/16 and picked it up 2/16.

Details on price:

1BL19      22330.58

1SE          421.40

K05           17.20

UN0          133.30

A23           44.72

Advertising   62.81

Dest. Charge 590.00

Dealer Prof   49.00

Bottom Line 23649.01  (No hassle except the wait)

Jack  96 Black

------------------------------

Date: Mon, 4 Mar 1996 20:34:24 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 326

Message-ID: <960304233422_438271349@emout04.mail.aol.com>

In a message dated 96-03-04 23:09:23 EST, you write:

>Can't wait till I cruise down I-20 out of Atlanta towards the ocean at WOT!.

>

>In the words of RAZZOR... Don't you just  LOVE IT!!!! ;>)

>

>

God love him!!!!!!!!!!!!!!!!!!!!!!!

RAZZORFL.

------------------------------

Date: Mon, 4 Mar 1996 20:34:25 PST

From: RAZZORFL@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 326

Message-ID: <960304233424_438271363@emout06.mail.aol.com>

In a message dated 96-03-04 23:09:23 EST, you write:

>Terry Parker

>Atlanta, GA USA

>95 Dark Cherry, Someone Stole Home Plate, Chrome 3.5", RED Bow-tie

>

>

Someone Stole Home Plate......................I LIKE THAT!

RAZZORFL.

------------------------------

Date: Mon, 4 Mar 1996 21:25:17 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Space Saver spare

Message-ID: <960305052516_73145.1566_FHD73-4@CompuServe.COM>

Kevin,

>In digest 324, you noted that all GM B/D-cars come with the same space

saver spare tire.  I don't have a recent Chevrolet manual, however, I was

wondering if the 'base' Caprices and Roadmasters come with 11 inch front

discs and 9 inch rear drum brakes? If so, the bolt pattern will be different

than the larger 12 inch discs and 11 inch drums that are standard on wagons

and Fleetwoods.

All of the B/D-cars get the same space saver spare tire and wheel. Also, no

matter whether it is a base Caprice, an Impala or a Fleetwood, and no matter

what brakes are installed, the wheel bolt pattern is the same 5x5" bolt

circle on all of them.

>BTW, my Fleetwood, built April '95, shifts to second immediately when the

selector is placed to '2', and starts in second gear from rest.

Well, the Caddy definitly has the 2nd Gear Start, but they tend to do their

own thing with respect to PCM programming. For example you also have

Traction Control, plus factory 3.42 gears! Those LT1 Fleetwoods with the

heavy duty Cadillac trailer tow (V4P - 3.42 rear axle) are total sleepers,

as few people know about them, and they have a ton of luxury options not

available on the Impala as well. In fact that car is so much of a sleeper,

I'd probably go through withdrawl symptoms due to the lack of being noticed

compared to the Impala! It must be fun dusting totally unsuspecting Mustang

GTs and SHOs with a Fleetwood! <g> LT1, LT1, she's so fine my LT1... <BG>

Scott.

------------------------------

Date: Mon, 4 Mar 1996 21:36:50 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: Off the *true* subject

Message-ID: <960305003649_341354868@mail06.mail.aol.com>

I was wondering if anyone knew or wanted to make suggestions to me on the

modifications I should perform on my 1995 Monte Carlo Z34.   I'd like less

body roll in turns, and more torque/horsepower overall.   It has a 3.4 liter

V-6 rated at 210 horses and 215 ft-lbs. torque.    Would Herb Adams sway-bars

be a good investment?  A true dual exhaust?   Maybe I should call Jon Moss

and ask if I can trade mine for the 467 HP prototype of the Monte Carlo SS?

=)

Also, has anyone heard anything of the U.S. government taking Amoco to court?

The government claims that Amoco is falsely advertising the Ulimate gasoline

as being superior, and supposedly Amoco has admitted to it.   Anybody know

the whole story?

Thanks,

Randy

1995 Green-Gray Impala SS

1995 Monte Carlo Z34

------------------------------

Date: Mon, 4 Mar 1996 21:24:49 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: LT4 heads

Message-ID: <960305052448_73145.1566_FHD73-1@CompuServe.COM>

Mark,

>I'm interested in your comments regarding the LT4 head on the Impala SS.

I'm under the impression that the LT4 intake/head combination is setup to

make peak HP and torque at higher RPM's. In effect, if you install the LT4

setup, you hurt the bottom end of the Impala SS motor. Is this an incorrect

statement?

Yes and no. The LT4 actually seems to have as much or more torque on the

bottom end as the LT1, the main difference is that it just keeps revving!

BTW, factory specs show the LT4 engine producing peak torque only 500 rpm

higher than the LT1.

>If the head/intake combination does hurt low-end performance, could you fit

(port) the stock LT1 intake to the LT4 heads?

The intake ports will not match between an LT4 head and an LT1 intake. While

you might be able to grind the manifold ports to raise them enough to get it

to work passably, it would be easier to purchase the LT4 intake which is

already properly matched. Besides, the LT4 intake comes pre-painted that

killer red color. <g> I'd have to get the LT4 spark plug wires 'cause they

are red too. Scott.

------------------------------

Date: Mon, 4 Mar 1996 21:51:00 PST

From: Christopher Massa <massa@intac.com>

To: impala@east.xsis.xerox.com (Chevy Impala Digest)

Subject: Hello [Newbie Post]

Message-ID: <199603050551.AAA24240@mailhost.intac.com>

Hello all,

Well, I subscribed to this list because I am interested in getting an Impala.

Unfortunately, I have to wait until I get a job, which should be soon.

I would like to let all of you know that I have enjoyed reading the info

that this list has given me. From mods to things to look out for, etc.

First off, Where do I send my $20 to?? I like to be part of that newsletter.

Second, I must of missed it somewhere, but what are the 9C1 deflectors??

Are these for the front brakes to keep the dust getting to the wheels??

If so, how come there aren't any for the rears? Since it is a 4 wheel disc.

Gosh, there are so many things I wanted to talk about, but can't think of them

at this time. Damn!

Hmm, Is anyone from NJ?? I was wondering how to go about setting up a

NJ chapter of the NAISSO. Also, how do I become a member of hte NAISSO?

I know I should wait until I get hte car, but my friend just bought a '94.

I won't be able to make the Impala Fest, have the EIT test that 20th. It's a 12 hour test. [Tell me about it!]

I think the first mod will be the audio system, since I'm a big car audio buff.

I think I'll wait a year till I touch the "home plate" and the exhaust.

Sorry for making this a long post for a Newbie. But I guess I'll wait a little

while to add some more comments.

Christopher Massa

No job, No car, No mods, Have the money though :)

------------------------------

Date: Tue, 5 Mar 1996 03:47:56 PST

From: dstabb@epix.net (David Stabb)

To: impala@east.xsis.xerox.com

Subject: Grey Hairs

Message-ID: <v02130500ad61d8a41049@[199.224.67.48]>

My Chevy dealer checked his sales stats for the Caprice and the Impala SS.

Average age buyer for Caprice is 68.

For the Impala SS 41.

I'm 41, and have some grey hairs. The Impala helps.

Last night, I was driving at about 70 or so on a quiet 4 lane, and a

hot pickup blasts by me at about 90. What the hell, I've not let the

beast out yet, having just had it a month. I nail it, and sweep by him

on my way to 115 mph. My exit is coming up, so I back off. Very stable,

and he *was* surprised. Plenty of room left to experiment above that

speed, if I ever feel stupid.

Our CEO has an Audi A6 Quattro. Nice car. I'll never be foolish enough

to pass the CEO, but I do like the motto on the Audi home web page,

"If you're still in control, you're not going fast enough."

____|)avid             Good Morning, Good Afternoon, Good Evening.

How difficult it is to tell when

dstabb@epix.net               you're locked in a tiny room with

dstabb@bix.com                     no windows and artificial lights.

------------------------------

Date: Mon, 4 Mar 1996 16:32:00 PST

From: Stephen Furr <skfurr@raleigh.ibm.com>

To: impala <impala@east.xsis.xerox.com>

Subject: Newbie Impala questions...

Message-ID: <9603051212.AA0127@rtpnsi01.raleigh.ibm.com>

My thanks to Scott Harris.  He hit the nail right on the head.

The dealer found a wheel out of balance (lost weight in shipping?)

and rebalanced the front wheels.  No more shimmy.

Picked up the car tonight.  Man, this car is awesome.  Already have

nearly 200 miles on it.  I've owned a long list of really cool hardware

('87 GTA, two '88 Vettes, '87 GN, '90 Q45) and have to say that

this is right up there with the GN in fun-factor!  I know I shouldn't

be playing yet with so few miles on the clock, but I can't help it.

I've got to drive SLOWLY tomorrow.  I promise!  :-)

I've got one more question:  I've noticed what sounds like the

normal GM TPI fuel injector ticking noise...  It is barely noticeable

when driving the car around town.  With the windows up, I can

here it when I'm into the throttle at all.  When I lift, it goes away.

I suspect that the ECM stops firing the injectors when you close

the throttle, therefore the noise stops.

Again, I suspect this is WAD (working as designed) so I'll live

with it.  Is this something unique to the SFI V8??  I don't remember

it being so prevalent on the TPI engines of the past (L98.)

Stephen Furr

------------------------------

Date: Tue, 5 Mar 1996 04:09:23 PST

From: jeffd@accessus.net (Jeff Dawson)

To: impala@east.xsis.xerox.com

Subject: "gray-haireds"

Message-ID: <199603051209.GAA17747@mtvernon1.accessus.net>

>the Impala is for "gray-haireds"!

The nerve of this young whipper snapper! Why just as soon as I can get out

of my EZ-Lift Chair I'm going to drive up there and beat him with my cane!

Jeff Dawson 95 "year of SS not age" DCM

------------------------------

Date: Tue, 5 Mar 1996 05:46:17 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Subject: Power pulleys

Message-ID: <199603051345.HAA09985@raven.sat.net>

>I think the same thing will pass through on the so called power

>pulleys.  I'll agree that I'd have to see a dyno test to believe

>10-15HP but I think there could well be some (however negligable)

>advantage during drag racing or (very) high speed work.  Since

>my 2TONLT1 is only turning about 2.2 grand at 80MPH I doubt

>there would be a measurable mileage increase though.

>

>Does this make sense or am I out to lunch with the power pulley people?

>Any physics majors out there???

Gosh, I hate it when this happens.... you asked for a Physics major and now

I have to step up.... the degree was a long time ago but...... Scott wins.

His logic is sound and, although I didn't check his math, his theory is dead

on.  About the only thing I think he left out of the equation is the

"turning losses" of pumping a belt.  However, although this is not a zero

number, the difference attributable to the "Power Pulleys" has to be

virtually unmeasureable.  Probably the largest gain a Power Pully would give

you is at the water pump... but we don't have that to drive so I'd save my

money for some other goodie.  If you were looking for the difference on the

track or with a Vericom, I don't think you would see the difference.  I will

bet on this one... you certainly won't see any mileage difference... and you

guys know I don't bet easily... any more!!

Steve

------------------------------

Date: Tue, 5 Mar 1996 00:02:00 PST

From: "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

To: impala <impala@east.xsis.xerox.com>

Subject: list of problems

Message-ID: <313C4A41@po5.cp.unisys.com>

I ordered an SS on Dec. 8th and the dealer called yesterday with a delivery

date of March 13th.  The dealer said he would check out known problems

posted on this list during the pre delivery physical. I know a number of you

have posted lists of problems and cures, I believe Mike Yang had a very

extensive one,  would you please email them directly to me so I could put

together a list for the dealer?  If anyone else has need for it I will post

the results.

That archive search engine would sure come in handy!  It would alleviate a

lot of posts for questions on part numbers etc., and ones like this.

ps I posted a  list of silicon hose part numbers and current prices in

digest #322

Don Fischer

------------------------------

Date: Tue, 5 Mar 1996 06:18:36 PST

From: David <71223.325@compuserve.com>

To: <Impala@east.xsis.xerox.com>

Subject: Funny story

Message-ID: <960305141835_71223.325_GHL93-1@CompuServe.COM>

Hi all !

I was wondering if anyone from the left coast is headed

to Dallas for the show?    I'm in Tucson, Az, and would love

to be able to hook up with some others driving out.

Funny story:

had to take a cab yesterday from the airport to my office.

When I walked up to it, it looked rather strange,

.

It was a white Caprice, but this cab owner had put on

the Impala grill, trunk emblem, and rear quarter panel lettering.

And, all these pices were in Dark Cherry!

Ok...so decidng to play along, durinmg the trip to my

office, I ask plainly," is this one of those new

Impala SS's I've been hearing about?"

The cabbie emphatically said, "yes!  it is!!"

The rest of the conversation:

Dave: Wow!  What kind of motor?

Cabbie:  Lt-1 just like in the Corvette.

Dave:    That sound neat.  Hey, I didn't think they made

these in White.

Cabbie:  They came out with about 100 for the taxi cabs.

Dave:   Well it sure is a neat car.

After arriving at office and paying the cabbie, I ask if I can see

under the hood.

Dave:  Hey, I've never seen a 'vette engine, can I take a look?

Cabbie:   Sure..hold on.

Cabbie opens the hood..it's NOT an Lt-1 Caprice....

After a couple minutes, Cabbie gets back in to leave:

Dave: oh, one more thing.

Cabbie: Yes?

Dave:  If you're gonna pretend to have an SS, at least know what you're

talking about.  If you want to see a real one..come by my office

again some time.

Cabbie drove off wit a half confused, half embarrased look on his face.

Cheers,

David Curtis

'96 DCM   Callaway Intake, Exhaust, HA Bars, Corvette engine dress.

------------------------------

End of IMPALA Digest 327

************************

I'll try this one again.

Mark Hawthorne wrote:

>

> Anyone come up with any mods to the factory switch for the flash to pass which was discussed a couple of weeks ago?

X-Mozilla-Status: 0001

Message-ID: <313C9FA7.5F95@lfwc.lockheed.com>

Date: Tue, 05 Mar 1996 12:10:15 -0800

From: Mark Hawthorne <mahawthorne@lfwc.lockheed.com>

Organization: Training and Development

X-Mailer: Mozilla 2.0 (Win95; I; 16bit)

MIME-Version: 1.0

To: impala@east.xsis.xerox.com

Subject: flash to pass

X-URL: http://www.crl.com:80/~impala/mailing.html

Content-Type: text/plain; charset=us-ascii

Content-Transfer-Encoding: 7bit

Anyone come up with any mods to the factory switch for the flash to pass

which was discussed a couple of weeks ago?

IMPALA Digest 329

Topics covered in this issue include:

1) Arlington

by Bngwip@aol.com

2) Corvett Injector covers for sale

by Franklin Poole <franklin@intersurf.com>

3) Re: flash to pass

by Rock528@aol.com

4) Re: Armageddon

by Rock528@aol.com

5) Re: NAISSO requests

by Rock528@aol.com

6) 96 Cooling Fan

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

7) 9C1 Replacement Trailing Arms

by Stewart McLellan <stewartm@magi.com>

8) 96 Service Manual

by "Andy Havill" <Andy.Havill@amgen.com>

9) Model 9C1's for each state.

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

10) Ram Air systems

by randomtech@earthlink.net ( Random  Technology)

11) Room Chat on AOL

by bngwip@mail.phoenix.net (Brett A. Rudolph)

12) Orders

by Joe Palmer <laser@iquest.net>

13) rattle in trunk area

by MeeKyung73@aol.com

14) Re: 2cd gear starts

by Kurt Kemmling <76165.1654@compuserve.com>

15) Pro side of Air Foil and pulley

by "J.R. LEVITZ" <4everGreen@msn.com>

16) Re: IMPALA digest 328

by JGreen5275@aol.com

17) Service Manuals

by "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

18) Re:  Home plate/airbox improvements

by ZTMP50A@prodigy.com ( DAVID NMN)

----------------------------------------------------------------------

Date: Tue, 5 Mar 1996 12:41:59 PST

From: Bngwip@aol.com

To: Impalaclub@aol.com, impala@east.xsis.xerox.com

Subject: Arlington

Message-ID: <960305154158_341889404@mail04.mail.aol.com>

Hey,

Tell John Moss if he will build it, I will come. I'm still waiting for

mine! It was ordered from Ron Craft Chevrolet in Baytown, Tx. on or about the

9th of Feb. A friend of mine who is not on the list but has a 95 SS will be

coming from Houston with me. If mine is not here yet, I will ride with him.

Brett

------------------------------

Date: Tue, 5 Mar 1996 12:47:28 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Corvett Injector covers for sale

Message-ID: <313CA860.44F2@intersurf.com>

Oops!  My local counter man Hank was ordering some parts for some of you

guys on the list and ordered an extra set of the Corvette Injector

covers by mistake.  Twenty bucks ($20) will get the pair shipped to you

via UPS.  Email me direct and they're yours.  Help me out here 'cause he

is sure helping us out.

Reminder:  If you can't get your dealer to help you, mine will give

Impala SS special orders a 30% discount.  Email me for details.

Injector covers:  10208794 and 10208795 $10 each, factory new in box.

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Tue, 5 Mar 1996 12:49:50 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: flash to pass

Message-ID: <960305154949_238256870@emout10.mail.aol.com>

In a message dated 96-03-05 15:20:46 EST, you write:

>Anyone come up with any mods to the factory switch for the flash to pass

>which was discussed a couple of weeks ago?

>

>

I spoke with a new member in Hawaii last week who was working  on this.

Seems that an earlier Pontiac SSE model had a Flash option and he was

looking at the switches.  One had to be unriveted and put on the impala's

headlight dimmer switch mounted base.  Anyone want to tackle this one??  Troy

------------------------------

Date: Tue, 5 Mar 1996 12:50:47 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Armageddon

Message-ID: <960305155046_238257546@emout10.mail.aol.com>

In a message dated 96-03-05 09:24:50 EST, you write:

> Dave:  If you're gonna pretend to have an SS, at least know what you're

>         talking about.  If you want to see a real one..come by my office

>         again some time.

Dave, great work...  It will take much work to weed out the imposters.

Following the  apocalyptic fuel wars of 1999, many will pretend to have

LT1's to gain status among the species.  The weak, of course, will drive fwd

phords.   Rock

------------------------------

Date: Tue, 5 Mar 1996 12:50:40 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: NAISSO requests

Message-ID: <960305155039_238257526@emout08.mail.aol.com>

In a message dated 96-03-05 09:24:50 EST, you write:

> I was wondering how to go about setting up a

>NJ chapter of the NAISSO. Also, how do I become a member of hte NAISSO?

email me direct at impalaclub@aol.com.  Troy

------------------------------

Date: Tue, 5 Mar 1996 13:09:17 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: 96 Cooling Fan

Message-ID: <Pine.SOL.3.91.960305130845.20909A@netinfo.ubc.ca>

Hello Scott and Troy and whoever else is interested:

While checking under the hood today, I checked out an old request

by Scott about the cooling fan.  As it turned out in my 96 I had the

150/240 Watt set up like the one under Troy's hood.  The parts number is

22137068 and the code is DD.  So I guess only a small portion of the 95

got the 150/100 watt DF set up.

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red (9C1 Air Deflactors, Gas Bib)

------------------------------

Date: Tue, 5 Mar 1996 13:29:04 PST

From: Stewart McLellan <stewartm@magi.com>

To: impala@east.xsis.xerox.com

Subject: 9C1 Replacement Trailing Arms

Message-ID: <313CB220.407B@magi.com>

Hi Guys:

Someone (Scott M.?) recently posted something regarding 9C1 replacement training arms

which will hold the HA sway bars better.  I can't find it in my files.  Can someone repost it

for me?

I finally got the VIN for a European SS that I was looking for.  Thomas Kempf was kind

enough to give me the VIN for a non-emissions '95 SS he had for a bit.  The dealer has now

ordered the T84 capsules for me.  GM Canada had to give in.  Horray for the list members.

Still waiting to get a production date for my black '96.  Its about 21 weeks so far with

probably another 4 or 5 to go.

Regards, Stewart

=====================================================================

"Nothing in life is so exhilarating as to be shot at without result."

- Winston Churchill

=====================================================================

------------------------------

Date: Tue, 5 Mar 1996 14:20:02 PST

From: "Andy Havill" <Andy.Havill@amgen.com>

To: "Impala Mailing List" <impala@east.xsis.xerox.com>

Subject: 96 Service Manual

Message-ID: <n1386085796.32554@amgen.com>

Scott M., you wrote:

BTW, you should have received a two volume manual set. Did the volume 2 =

book

you received have the word "Preliminary" stamped on the cover? When I got =

my

'96 Service manuals, Vol. 1 has a purple cover and appears complete, while

Vol. 2 has a white cover and has "Preliminary" stamped on it. The '96 =

F-car

manuals came the same way (Vol. 2 with a white cover and "Preliminary"), =

and

the '96 Y-car ('vette) manual I got was a single volume in white with the

"preliminary" designation.

*********

Yes, Vol.2 does have the word "Preliminary" stamped across a white cover.  =

The

transmission section DOES state that the 2 position starts out and stays =

in 2nd gear.

It seems that most of the Chevy figures in the manual are Caprice =

specific, in that

they show the Caprice dashboard, and the column mounted shifter.  I have =

not looked

through it carefully yet.  Believe it or not, I actually *read* these =

things, rather

than just looking up what I need!

Anyway, with 3500 miles on the odo, it is time for me to meet Mr. =

Goodshaft, with

the usual list of complaints:  grinding from differential on backing =

turns, popping from

front suspension, poorly adjusted rear doors.  I think I will also mention =

that the car

does not start in 2nd gear as specified in the service manual - that =

should cause them

to scratch their heads for awhile.

Andy

------------------------------

Date: Tue, 5 Mar 1996 14:26:29 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Subject: Model 9C1's for each state.

Message-ID: <9603052230.AA15743@karloff.fstrf.org>

For you 9C1-heads:

I found a model, 1:43 scale & made of metal, of the 9C1 state police cars

for each state at a local drugstore (Vix).  The maker is "Road Champs,"

and the title of the series is "Caprice State Police Series."

The packaging is a flat, bright-red bordered card (6.5" x 8.5") with

the car attached in a clear plastic bubble.  Each car has plastic windows

(fronts are "rolled down") and plastic gumballs (some all red, some all

blue [no pun intended], and some mixed colors).

They have a generic "State Police" 9C1, and then one for each state in the

(apparently) proper color-scheme and logoing.  The front doors open (to reveal

the *yuck* bench seating <g>), and the trunk opens as well.  I found mine

(NY state) with the model number of 6430, at $2.98.

I don't know what floodgates I might open, but I woulnd't mind making a trip to

Vix and picking up some for any list members interested that can't find these

in their area.  The only caveat: reinbursement for price (NYS sales tax is 7%)

and shipping (however you'd want them shipped) - of course I'd do it for free!.

Let me know via email if you're interested.  Note: they don't always

have every state's 9C1 on-hand.

- mike

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

My '96 Impala SS: "It goes fast, FAST!"

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

Date: Tue, 5 Mar 1996 14:28:59 PST

From: randomtech@earthlink.net ( Random  Technology)

To: impala@east.xsis.xerox.com

Subject: Ram Air systems

Message-ID: <v01530501ad622732e4c9@[153.37.16.185]>

We're new to the list and hopefully joining will better enable us to

answer questions of Impala owners. We've been getting a lot of phone calls

about our soon-to-be-released Ram Air system for the latest and greatest

Impalas. We should have a prototype within a week to 10 days and hopefully,

they'll be ready to ship before the end of the month. The system consists

of a scoop that fits behind the front air dam, a complete air cleaner

housing and all the ducting and hardware needed to connect everything. The

only part of the original system that is be retained is the rubber elbow

that clamps on the throttle body. A new center air dam piece will also be

supplied so the original one won't have to be cut for the scoop. Target

price is in the $250-$275 range. I'll be happy to answer any questions.

------------------------------

Date: Tue, 5 Mar 1996 16:00:43 PST

From: bngwip@mail.phoenix.net (Brett A. Rudolph)

To: impala@east.xsis.xerox.com

Subject: Room Chat on AOL

Message-ID: <Chameleon.960305161040.bngwip@dialin.phoenix.net>

Hello SS'ers

I would like to propose an aol Chat night sometime this week. Anyone who

is interested should sign on and go to the Impala SS room. Click on member

rooms then click on create member room then type in Impala SS and click OK.

(If there is an easier way to do it, I don't know about it.) I think we

should try for Thursday Night at 10PM EST. If anyone has a better idea,

post it. I think this would be fun and several of us would get to know each

other more or less before Super Chevy. Hope to see you there!!!

Brett

------------------------------

Date: Tue, 5 Mar 1996 15:18:25 PST

From: Joe Palmer <laser@iquest.net>

To: Impala Page <impala@east.xsis.xerox.com>

Subject: Orders

Message-ID: <313CCBC1.78A2@iquest.net>

I have read with great interest a the mail about the time from ordering

to delivery. According to the printouts at my dealer orders placed in

late September would be built in March. This holds true for my 96 in

that it was  ordered on September 20 and was built on February 29. And

is on the truck and on its way to Indiana. Sold my 95 SS on January 22

and am really anticipating the arrival of the new BEAST. Another person

I work with order his in the first or second week of October and took

delivery 2 weeks ago but he didn#t have the heat reflecting windshield.

Think that might have held mine being preferenced. So I wonder if the

reports of order placed in December and being delivered in March are

actually car the dealer already had in the system. Not that it#s really

that important !!

Joe Palmer BLACK 96 on the way

------------------------------

Date: Tue, 5 Mar 1996 15:26:44 PST

From: MeeKyung73@aol.com

To: impala@east.xsis.xerox.com

Subject: rattle in trunk area

Message-ID: <960305182643_342064068@emout09.mail.aol.com>

Erik

I had the same problem. I struggled to find the problem.

I found out it was a missing muffler bracket, I quess it fell off?

Try shaking your muffler tips, the muffler hits the heat shield making the

noises

------------------------------

Date: Tue, 5 Mar 1996 16:03:10 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: 2cd gear starts

Message-ID: <960306000309_76165.1654_GHM27-2@CompuServe.COM>

In digest 327 Bruce Gordon writes:

>>>My SS has a build date of Dec. 1994 for my 1995 model.  I can assure you that

my 2nd gear start doesn't work.  (RATS!!)  Sometime between your date and my

date they must have converted the PCM programming to reflect the model year.<<<

Bruce,

Yup, I agree. At least we have narrowed down the dates somewhat. I think they

probably kept using the '94 PCM in the '95s until they ran out, and began using

the '95 PCM. I have no idea why they would delete this feature, as I said, it is

great to have. I just assumed that GM finally implemented 2cd gear starts

because of Ford's success with it. My F-250 has 4.11 gears, a 3 speed C6

automatic, and I don't know what I would do without 2cd gear starts with that

setup! It has the 460 big block, and 2cd gear starts help to buffer some of

those 450 ft lbs of torque when I need to.

I wonder what GMs reasoning was for this change? They must have planned on

keeping it because the manuals state that they (95's) have this feature.

I wonder if you can add this feature back into the PCM with the Hypertech

programmer? Maybe one of the Z-PCM's has this feature in it?

If it is possible to add it, I would recommend doing so, as it is a handy

feature to have.

TTYL,

Kurt

'95 Dark Cherry Impala SS.....no mods........yet......but it does start in 2cd

gear.........

------------------------------

Date: Tue, 5 Mar 1996 17:20:10 PST

From: "J.R. LEVITZ" <4everGreen@msn.com>

To: "Impala List" <impala@east.xsis.xerox.com>

Subject: Pro side of Air Foil and pulley

Message-ID: <UPMAIL01.199603060120010162@msn.com>

OK Scott M., Grab your sword and put on your mask. It is time to joust!!<BG>.

Just MHO with the intent of letting the list members decide for themselves.

What the heck soured your milk so bad about air foils? My personal

experience and observations are quite contrary to yours. I consider the air

foil a  "must" for any throttle body equipped car.

Let me be specific, I am referring to the Air Foil made by T.P.I.S. and

also sold by Hypertech. This is the one that bolts very smoothly to the

throttle body.

There is dyno proof that they work. Could the manufacturer really sell a

bogus part for over ten years? I put one on my personal Impala and I swear it

was noticeably better. This is the same response I get from my customers. This

alone justifies the price. It is cheap, it is easy to put on, and it works!

They ( TPIS) claim 10-12 Hp. I would agree this is optimistic and a best

case scenario. Even if it is only 5-8 Hp I still would give the air foil a

"buy" recommendation.

Regarding pulleys. Another buy recommendation. BUT, I warn all my customers

that they are a "compromise" part. You will notice a performance gain and the

associated voltage drops at idle. If you have a killer stereo do not buy an

underdrive crank pulley!  If you want the most out of your car, this is a part

that again, is cheap , is easy to install, and works. Just be aware that there

will be charging problems, but ONLY at idle.  Another observation, it does not

affect the performance of your A/C. It will still blow cold .

Take two Impalas' with equal performance, now add to one the air foil and

crank pulley. Now bet on the winner of the race. It is pretty clear who will

win!

Regards John Levitz

------------------------------

Date: Tue, 5 Mar 1996 18:48:06 PST

From: JGreen5275@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 328

Message-ID: <960305214805_439124482@mail04.mail.aol.com>

I,m interested in the trip to Arlington and probably will go, how about an

interary? is there anybody out there that has a copy of the latest

newsletter? I sure would like to see a copy.Maybe someone can make a copy and

fax one to me. My fax is 407 323-3615 thanks.

------------------------------

Date: Tue, 5 Mar 1996 14:17:00 PST

From: "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

To: impala <impala@east.xsis.xerox.com>

Subject: Service Manuals

Message-ID: <313D12CE@po5.cp.unisys.com>

I ordered my service manuals, direct from Helm (800-782-4356),  on  Thurs.

Feb. 29th and received them today Tues. March 5th. Book 2 is white and

stamped preliminary across its front. They cost $95 for the set.  Inside it

does say that the xmission flow diagrams are located in the transmission

unit repair manual.

I have all the Chevrolet car and truck service manuals from 1957 to 1970,

my dad was a Chevy mechanic, and they are invaluable references.

Don Fischer

------------------------------

Date: Mon, 4 Mar 1996 20:32:38 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: Re:  Home plate/airbox improvements

Message-ID: <097.02857808.ZTMP50A@prodigy.com>

Hey all...I was investigating the above.  After removing homeplate,

which is very easy, I noticed all the cables and wires are exposed.

What do you do about this?  It really leaves mess to be seen.  I now

have seen the light re: the inlet opening and the "cap" placement.

In fact, the whole air induction system is just plain ridiculous.  No

wonder everyone wants to remove the baffle, etc.  <G>.  As much as I

would like to add the Callaway cold air induction system, what I

really want to do is add a Vortech or Paxton Supercharger system that

would make the cold air system obsolete afterwards.  This is probably

a year aways though.  Also, per the instructions given here, the hole

to be drilled in the airbox says on the front left side.  On my '96,

thats where the factory opening is.  On the right side, there is a

bracket that mounts the box to the car.  There is some room there,

but 1.25" might be pushing it.    Any comments?  I think the easiest

way to go overall for my case is a 3.5" chrome pipe, the 1LE inlet

and the K&N until I can supercharge this baby.  Sorry for the

length!

Dave in So Cal...'96 Black SS.  No Kills, so Far....<BG>

P.S.  BTW, I submitted a note to Motor Trend regarding our cars and

the Club.  I got a response back that says it might make print in an

upcoming issue.  <G>

------------------------------

End of IMPALA Digest 329

************************

IMPALA Digest 330

Topics covered in this issue include:

1) Torque Converters

by stc@cypress.com (Steven Chapple / Field Sales Engineer)

2) ram air intake

by "Jeff Gatcha" <foxhound@txdirect.net>

3) Motor Trend

by "David Spitz" <david_spitz@vnet.ibm.com>

4) herb adams bars

by Impalaclub@aol.com

5) Flash to pass

by Scott Mueller <73145.1566@compuserve.com>

6) re-designed control arms

by Scott Mueller <73145.1566@compuserve.com>

7) air foils and pulleys

by Scott Mueller <73145.1566@compuserve.com>

8) cooling fans

by Scott Mueller <73145.1566@compuserve.com>

9) Engine dressup

by Scott Mueller <73145.1566@compuserve.com>

10) Information.

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

11) 9C1 Models by state

by "Michael W. Martin" <mwmartin@lanmail.rmc.com>

12) New Groupie

by jserko@terrabyte.net (John Serko)

13) power pulley --fact or friction?

by Ed Walters <75763.466@compuserve.com>

----------------------------------------------------------------------

Date: Tue, 5 Mar 1996 20:46:24 PST

From: stc@cypress.com (Steven Chapple / Field Sales Engineer)

To: impala@east.xsis.xerox.com

Subject: Torque Converters

Message-ID: <199603060446.UAA13023@mailb2.cypress.com>

Hello again,

Razzor and I are now fully TransGo'd.  Awesome shift kit for $200

installed!  We are now ready to get torqued.  Should we go with the

general's '86 Vette converter (per Scott Mueller) or one of the other

performance models (i.e. Vigilante, etc)?

Any experience/info/prices and p/n's would be appreciated.

Thanks,

Steve Chapple  (ROCKET)

'95 blk w/many mods

"Let the big dog eat!"

------------------------------

Date: Tue, 5 Mar 1996 21:21:31 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: ram air intake

Message-ID: <199603060520.XAA28028@legend.txdirect.net>

>From digest 329

> We're new to the list and hopefully joining will better enable us to

>answer questions of Impala owners. We've been getting a lot of phone calls

>about our soon-to-be-released Ram Air system for the latest and greatest

>Impalas

I'll bite,   will I have any problems with it  sucking up water in the usual

South texas downpours when there is like a foot of water on the road?

God, grant me the courage to change the things I can,

the serenity to accept the things I can't,

And the wisdom to hide the bodies of the people who really

pissed me off.

Jeff Gatcha

1995 "Modified" DGGM

Foxhound@txdirect.net

San Antonio Texas

------------------------------

Date: Wed, 6 Mar 1996 00:19:05 PST

From: "David Spitz" <david_spitz@vnet.ibm.com>

To: impala@east.xsis.xerox.com

Subject: Motor Trend

Message-ID: <96Mar6.002119pst."16719(7)"@alpha.xerox.com>

Dave, or anybody else, if Motor Trend does in fact produce an article

about this club/list, please please please somebody save me a copy.

I won't be back in the U.S. until August and I'd hate to miss this!

thanks,

David

'95 Black Impala, mods: one small dent caused by some jerk in the snow.

(BTW, my dealer said he could fix this for $65. No paint lost, apparently

they heat and "massage" the panel (rear quarter) from behind to remove

the dent. Anybody have thoughts on this? I'd prefer not to have to repaint

and such... but it's a big enough dent that I don't think it can be sucked

out.)

------------------------------

Date: Wed, 6 Mar 1996 00:31:40 PST

From: Impalaclub@aol.com

To: impala@east.xsis.xerox.com

Subject: herb adams bars

Message-ID: <960306033139_161485234@mail06.mail.aol.com>

This is a tentative price for the bars:

Front sway bar, regular 270.00   discount $216.00

add 50.00 for the tie bar ends instead of the heim joint.  I haven't

negotiated this yet.

Rear bar 1.5 inch dia,  regular 220.00   discount 176.00

plus free freight which is probably about 30-50 bucks due to their weight.

Troy

------------------------------

Date: Wed, 6 Mar 1996 02:07:49 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Flash to pass

Message-ID: <960306100748_73145.1566_FHD43-4@CompuServe.COM>

Troy

>>Anyone come up with any mods to the factory switch for the flash to pass

which was discussed a couple of weeks ago?

>I spoke with a new member in Hawaii last week who was working  on this.

Seems that an earlier Pontiac SSE model had a Flash option and he was

looking at the switches.  One had to be unriveted and put on the impala's

headlight dimmer switch mounted base.  Anyone want to tackle this one??

A better application to borrow from might be the Caddy Fleetwood. I am

pretty sure that they have the flash to pass feature, and the D-car

Fleetwood is about as close to the B-car as you can get! If anybody figures

this one out, post it on the list. Scott.

------------------------------

Date: Wed, 6 Mar 1996 02:07:46 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: re-designed control arms

Message-ID: <960306100746_73145.1566_FHD43-3@CompuServe.COM>

Stewart,

>Someone (Scott M.?) recently posted something regarding 9C1 replacement

training arms which will hold the HA sway bars better. I can't find it in my

files.  Can someone repost it for me?

The new design 9C1 lower rear control arms are #10289786. I got mine

yesterday, they come complete with both the front and rear bolts, as well as

all new swaybar attaching hardware! The new bolts actually look long enough

to install the Herb Adams rear bar without having to get different ones. The

swaybar attachment mechanism is MUCH stronger than the previous design, as

the new bolts go all the way through the control arm from top to bottom, and

do not rely on the weak bracket with welded in nuts like before. I've busted

several of these brackets with the Herb Adams bars on my '88 Caprice 9C1,

but I drive that car VERY hard! <g>

>I finally got the VIN for a European SS that I was looking for.  Thomas was

kind enough to give me the VIN for a non-emissions '95 SS he had for a bit.

The dealer has now ordered the T84 capsules for me. GM Canada had to give

in.  Horray for the list members.

Cool, I'm glad you were able to get the T84 lights, you will love them! Now

about that VIN, I'd sure like to have a copy of that VIN for my records! I

would love to run it through the GM DCS system to see what options came on

that car, plus that will tell me the PCM calibration they used. Thanks,

Scott.

------------------------------

Date: Wed, 6 Mar 1996 02:07:32 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: air foils and pulleys

Message-ID: <960306100731_73145.1566_FHD43-1@CompuServe.COM>

John,

>What the heck soured your milk so bad about air foils? My personal

experience and observations are quite contrary to yours. I consider the air

foil a  "must" for any throttle body equipped car.

Unlike the pulleys, which are definitely bogus, the effect of the airfoils

is harder to prove either way. I don't have access to a dyno to prove the

benefit of these devices either way. I'd sure like to see some documented

tests done on an otherwise stock engine with the standard F or B-car inlet

elbow and one of these airfoils. I just don't believe there is any

improvement to these things, and I base that on my own observations and

experience. Unfortunately until we get one on a dyno, with repeated runs

both with and without (just one run each way is not nearly precise enough to

be conclusive), we will both probably stick to our opinions!

>There is dyno proof that they work.

Well, that I would definitely like to see! Unfortunately I bet this thing

results in such a small gain or loss (I'd say plus or minus 3-5 HP max),

that this small difference is probably less than the precision in the dyno

measurement itself between back to back dyno pulls on an engine without even

making any changes. In other words, a given engine will not make EXACTLY the

same power even on back to back runs. I bet that a +-5 HP difference (or

more) can be experienced without making ANY changes to the engine. Heck,

even a 10 degree difference in ambient temperature or a slight difference in

humidity can result in a +-5 HP change! If back to back runs differ by 5HP

both with or without one of these foils installed, then that says it is

doing nothing IMHO. We'd have to make many runs and average them, and even

that may not be precise enough if the measured gain is less than the

precision or difference between runs without making any changes.

>Could the manufacturer really sell a bogus part for over ten years?

Come on, you KNOW the answer to that! There have been many bogus parts sold

over the years. The "power pulleys" for the LT1 are a perfect example. <g>

In fact I am continually surprised by what products are sold without any

solid engineering or scientific testing behind them. That is much of the

reason I try to stick with factory parts for most of my mods.

>I put one on my personal Impala and I swear it was noticeably better.

That reminds me of a test years ago in Stereo Review where they were

comparing different types of "monster" speaker wires, and the reviewer was

trying to explain how he could hear the differences in sound between the

different brands of wires! Ya, right! This guy was obviously lost in his

own world... I think he is now doing Splitfire sparkplug testimonials.

"you hit your passing gear, and you're gone!", (I LOVE that one! <g>) or

"I got 25% better gas mileage by using these plugs" (sounds like they

discovered cold fusion there <g>) These are just other examples where we

are expected to rely on somebodys opinion that they "felt" a change. I'm

sorry that I am so anal retentive, but I need to see hard data, accurately

measured, and backed up by significant "control" testing via the standard

scientific method!

Now if you are telling me that you can definitively notice a 1 or 2

percent difference in power during normal street driving, you must have a

pretty accurate accelerometer hard wired to your brain! Come to think of

it, exactly WHERE is that Vericom you once had, and what are those

stitches for on your neck?? <g>

>They ( TPIS) claim 10-12 Hp. I would agree this is optimistic and a best

case scenario. Even if it is only 5-8 Hp I still would give the air foil a

"buy" recommendation.

So it is OK that they exaggerate their claims by 100%? Unfortunately I think

the exaggeration is even more, as I estimate the foil to be worth 0 HP, but

then again it is my opinion against theirs (and yours) without any ACCURATE

data to prove it either way!

Don't you think that if bolting on this little piece of metal or plastic

shrouding would be worth any power at all, that GM or especially SLP

engineers would have employed it on the Camaro SS or WS6 Firebird? I mean

they spent significant money to optimize the induction system on those cars,

why not throw in this device if it works. In fact SLP sells these things I

believe, but yet they don't use it on the cars they produce? That says alot!

They probably sell these things simply because there has become a market for

them, and there is money to be made. The same goes for the performance

claims. One guy rigged up some test to show a 15 HP gain, so now they all

claim the same thing, I mean who would want a +5 HP airfoil when the other

guy sells one and claims +15? I have heard from SLP and GM engineers that

these foils are worth nothing, hence the reason they don't use them on the

factory cars!

>Regarding pulleys. Another buy recommendation. BUT, I warn all my customers

that they are a "compromise" part. You will notice a performance gain and

the associated voltage drops at idle.

I will admit that neither of us have the time, money, or equipment to

definitively prove much about the air foils, but the pulleys are a whole

other story. They are definitely BOGUS as applied to the LT1! I guess you

could call that a "compromise", as in "your performance will be

compromised". <g> This is really without much question, all you have to do

is understand the physics involved (you may want to review the numbers in my

first post on the subject). Only Q can change the laws of physics to suit

his desires, unfortunately us mortals have to contend with these laws as

they are! <g> Scott.

------------------------------

Date: Wed, 6 Mar 1996 02:07:57 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: cooling fans

Message-ID: <960306100757_73145.1566_FHD43-5@CompuServe.COM>

Mike,

>While checking under the hood today, I checked out an old request by Scott

about the cooling fan.  As it turned out in my 96 I had the 150/240 Watt set

up like the one under Troy's hood.  The parts number is 22137068 and the

code is DD.  So I guess only a small portion of the 95 got the 150/100 watt

DF set up.

Well, to update this situation, what I found out is that only the '95 9C1

cars came with the 100-watt secondary fan. All B/D-cars get the 150-watt

primary fan as standard (unless replaced by a mechanical clutch fan as a

part of the V08 cooling package standard with the V92 trailer towing

package). ALL Impalas from '94 to '96 have the 150-watt primary (RH) fan and

the larger 240-watt secondary (LH) fan as a standard part of the V03 extra

capacity cooling package.

For some strange reason the '95 9C1 cars only got 100-watt secondary fans,

which is the same as the base Caprice V02 cooling package, although these

cars did indeed have the V03 option on the SPID label, as well as the higher

capacity V03 radiator and A/C condenser. The '94 9C1 cars had the higher

wattage secondary fan like the Impala, and I don't know about the '96 9C1

cars as I have not been able to inspect one yet. The local cop shop is

expecting to get their '96 cars soon, so I should be able to check them out

when they come in.

In any case there is no need to further check the fans on any Impalas, as

every one I have seen came with the correct, higher wattage secondary fan.

Scott.

------------------------------

Date: Wed, 6 Mar 1996 02:07:39 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Engine dressup

Message-ID: <960306100738_73145.1566_FHD43-2@CompuServe.COM>

Dave,

>After removing homeplate, which is very easy, I noticed all the cables and

wires are exposed. What do you do about this? It really leaves mess to be

seen.

This is simple! You install the Corvette fuel injector covers, 1LE F-car

inlet elbow, and for a real nice look, add in the Corvette plastic or

magnesium valvecovers. All this requires is some minor tugging, tucking and

re-routing of the wire harness (no wires are cut) and brake vacuum hose to

fit under the injector covers. What you end up with is an absolutely killer

looking engine, in fact it will look like you just swapped it from a

Corvette.

>Also, per the instructions given here, the hole to be drilled in the airbox

says on the front left side.  On my '96, thats where the factory opening is.

On the right side, there is a bracket that mounts the box to the car. There

is some room there, but 1.25" might be pushing it. Any comments?

The airbox has not changed at all from '94-'96. The instructions you refer

to indicate that to remove the airbox restriction, you need to drill several

holes as large as possible along the front (one 1.5" hole), LH side (several

holes blended together and ground so as to actually appear as one large oval

opening along the entire LH side), and rear next to the wheel well (several

holes blended together into another oval opening). This will remove the

restriction caused by the airbox, and allow a K&N factory replacement type

filter to gulp much more cold air! A bonus is that all but the front hole

cannot be seen once the airbox base is re-installed in the car. Scott.

------------------------------

Date: Wed, 6 Mar 1996 05:53:26 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Subject: Information.

Message-ID: <9603061357.AA12774@karloff.fstrf.org>

Troy,

I'd like to know two things:

1) I sent a check for $20 a while back, and want to confirm that I'm now

a full-fledged member.

2) Can you email me a list of the current members?  I'm thinking a Western

New York chapter would be good - if there's the interest.

- mike

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

My '96 Impala SS: "It goes fast, FAST!"

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

Date: Wed, 6 Mar 1996 09:59:48 PST

From: "Michael W. Martin" <mwmartin@lanmail.rmc.com>

To: impala@east.xsis.xerox.com

Subject: 9C1 Models by state

Message-ID: <9603061500.AA17614@internet.rmc.com>

For you 9C1-heads:

I found a model, 1:43 scale & made of metal, of the 9C1 state police cars

for each state at a local drugstore (Vix).  The maker is "Road Champs,"

and the title of the series is "Caprice State Police Series."

The packaging is a flat, bright-red bordered card (6.5" x 8.5") with

the car attached in a clear plastic bubble.  Each car has plastic windows

(fronts are "rolled down") and plastic gumballs (some all red, some all

blue [no pun intended], and some mixed colors).

They have a generic "State Police" 9C1, and then one for each state in the

(apparently) proper color-scheme and logoing.  The front doors open (to

reveal

the *yuck* bench seating <g>), and the trunk opens as well.  I found mine

(NY state) with the model number of 6430, at $2.98.

I don't know what floodgates I might open, but I woulnd't mind making a

trip to

Vix and picking up some for any list members interested that can't find

these

in their area.  The only caveat: reinbursement for price (NYS sales tax is

7%)

and shipping (however you'd want them shipped) - of course I'd do it for

free!.

Let me know via email if you're interested.  Note: they don't always

have every state's 9C1 on-hand.

------  For those of you who want these, I have also seen them advertised

------  by an outfit called EXOTICAR MODEL COMPANY, located in Framingham,

------  MA. Their phone number is 1-800-348-9159.

Mike Martin

" Go fast, Turn left, Bump the fords to the Wall."

------------------------------

Date: Wed, 6 Mar 1996 07:15:04 PST

From: jserko@terrabyte.net (John Serko)

To: impala@east.xsis.xerox.com

Subject: New Groupie

Message-ID: <19960306151504515.AAA271@money.whole.net>

Hello all,

I've been following this list for the past 2 months and finally broke on Feb

10 and ordered my beast. Much to my dismay, my dealer *lost* my order for 2

weeks and didn't order my car until the 26th. When I called to give the

sales mangler h*ll, he said he has a car coming in in 4 weeks that is almost

identical to the one I ordered (execpt for the cd player that I don't want)

so I'll get my car actually weeks early!!! (YaaahhhHooooooo!)

I was interested to know if there are any Impala owners in the South Florida

area that would be willing to get together and possibly form a SFL chapter

of the NAISSO (if it's ok with you, Troy). If there are, (and I've seen you

all on the road) please E-mail me at jserko@terrabyte.net. I would be

willing to organize the effort, and do whatever needs to be done to make it

work!

Regards to all!

John J. Serko

Soon-to-be-owner of a 96 Black beast!

Regards,

John J. Serko

TerraByte Technical Support

------------------------------

Date: Wed, 6 Mar 1996 07:16:57 PST

From: Ed Walters <75763.466@compuserve.com>

To: impala list <impala@east.xsis.xerox.com>

Subject: power pulley --fact or friction?

Message-ID: <960306151656_75763.466_BHQ47-1@CompuServe.COM>

>>>Gosh, I hate it when this happens.... you asked for a Physics major and now

I have to step up.... the degree was a long time ago but...... Scott wins.

Go dust off those books, Steve.  Look up angular momentum.  Then think about

lightened flywheels, aluminum driveshafts, and other lightened rotating parts.

Power pulleys, under certain applications, have definite benefits.  Depending on

the vehicle's original design parameters, and your modifications, you can run

into situations where the accessories are always running faster than stock.

Friction increases proportionally to speed/rpm CUBED, thus, excessive speed

costs a lot of friction, not to mention added wear.  (Not talking power to do

the job, but losses due to friction.)   Also, Scott, while a GM and B-body

expert, is way off on the efficiency/power needed to run accessories.  Finally,

it takes power to accelerate those parts up to higher rpms, power which could be

used to accelerate the car instead.  This power is seen as stored energy (the

flywheel effect) that is then lost as the engine slows (tends to slow more

slowly due to this stored energy.)  Now, if you are tearing down the quarter,

and this engine speed slowing down occurs on the shift, and nothing

(clutch/trans/tires) slips, turning power into heat/frictional losses, then you

could argue you got the power back, but you still lose the stored energy at the

end of the run when the engine is at high rpm and you let up.  By the way, if

you doubt that angular momentum amounts to much, try two things:  1) with the

trans in neutral, engine at say 1500 rpm, punch it.  Did the rev limiter kick in

in ZERO seconds or did it take time (and power) to rev up that spinning

assembly?  2) rev the engine up to 5000 rpm and simultaneously remove foot from

gas and drop trans into gear.  Did the engine stop spinning, or did the tires

spin from the stored energy?

This power/stored energy will not really show up on a dyno at steady state

conditions (other than the small change in frictional loss).  It can and does

show up whenever the engine/car needs to accelerate (and decelerate), as in drag

racing and certain sports/road race applications.

Note well:  I do not support the use of power pulleys on street driven cars, and

have absolutely zero to do with any manufacturer thereof.  I do however posess

multiple degrees -- engineering/physics/math.  I also feel I must object to the

"physics" Steve presents, as it is inaccurate.

Ed Walters

94 LT1 Caprice, w/ mods.

------------------------------

End of IMPALA Digest 330

************************

IMPALA Digest 331

Topics covered in this issue include:

1) Re: IMPALA digest 330

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

2)

by Steve Das <sdas@sat.net>

3) I'll try this again...

by Charlie Argento <argento@ucsub.colorado.edu>

4) $2800 Wish list

by dhflores@iwl.net (Debbie/Henry Flores)

5) New Orders/Heat Reflective Windshield

by Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

6) Careful, Careful....

by Joe Mendrzycki <73541.1501@compuserve.com>

7) HOTELS

by Rock528@aol.com

8) Club Logo

by Rock528@aol.com

9) Suspension Upgrades

by John Latrobe <racerx@jagunet.com>

10) The Need for Speed

by randomtech@earthlink.net ( Random  Technology)

11) Torque converters

by bnyk@oanet.com (Bernie Nykyforuk)

12) Blue SS sighting

by daryl@austin.ibm.com (Daryl Green)

13) Shamu has Arrived!!

by "Scott Zeiter" <ScottZeiter@msn.com>

14) part #'s for Fuel Inj. Cvrs.

by Loren Peterson <equ_rino@cencom.net>

15) Bow tie decal for grill

by Loren Peterson <equ_rino@cencom.net>

----------------------------------------------------------------------

Date: Wed, 6 Mar 1996 07:32:42 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Cc: Impalaclub@aol.com

Subject: Re: IMPALA digest 330

Message-ID: <9603061536.AA03187@karloff.fstrf.org>

At 07:20 06/3/96 PST, I blabbed:

>------------------------------

>

>Date: Wed, 6 Mar 1996 05:53:26 PST

>From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

>To: impala@east.xsis.xerox.com

>Subject: Information.

>Message-ID: <9603061357.AA12774@karloff.fstrf.org>

>

>Troy,

>

>I'd like to know two things:

>

>1) I sent a check for $20 a while back, and want to confirm that I'm now

>   a full-fledged member.

>

>2) Can you email me a list of the current members?  I'm thinking a Western

>   New York chapter would be good - if there's the interest.

>

>- mike

>Turkish Proverb: "Coffee should be black as hell, strong as

>                  death, and sweet as love."

>

>My '96 Impala SS: "It goes fast, FAST!"

>---

>+-------------------------------------------------------+

>|         Michael C. Calanan  calanan@fstrf.org         |

>|                 Programmer / Analyst                  |

>|  Frontier Science and Technology Research Foundation  |

>|         4033  Maple Road  Amherst  NY  14226          |

>|           http://stewart.fstrf.org/~calanan           |

>+-------------------------------------------------------+

>

Sorry list!  This was meant for Troy at Impalaclub@aol.com -

I used the wrong email alias.

- mike

------------------------------

Date: Wed, 6 Mar 1996 08:31:54 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Message-ID: <199603061631.KAA18124@raven.sat.net>

Ed,

I don't disagree with your theory... I do disagree with your analysis.  The

implied question (if not in fact) was if there would be a MEASURABLE gain in

performance through the use of the so called "Power Pulleys".  I stand by my

original statement... and I quote...

"His logic is sound and, although I didn't check his math, his theory is

dead on.  About the only thing I think he left out of the equation is the

"turning losses" of pumping a belt.  However, although this is not a zero

number, the difference attributable to the "Power Pulleys" has to be

virtually unmeasureable."

I chose to call the loss through turning the belt as "pumping Losses".  Your

argument about angular momentum is sound.  Perhaps I should have chosen a

different word.  The point I was trying to make is that when you quantify

the savings in angular momentum or "pumping losses" or whatever you choose

to call it, gained through the use of Power Pulleys on the CAM DRIVEN WATER

PUMP LT1 MOTOR, I don't believe you will even begin to see the difference in

day to day performance.  In theory, you are absolutely correct.  In

practice, I won't believe it until you show me some hard numbers.  Please

see the post with regard to the repeatability error of the dyno.  Having

been exposed to dyno work more than once, I can tell you that the error from

run to run on an engine in the 250 horsepower catagory is easily within what

Scott suggested.  Therefore, I stand by my statement.  I don't believe you

will measure the difference, at the rear wheels, through the use of the

Power Pulleys.  Got any data to refute that???

------------------------------

Date: Wed, 6 Mar 1996 08:41:24 PST

From: Charlie Argento <argento@ucsub.colorado.edu>

To: Chevy Impala SS <Impala@east.xsis.xerox.com>

Subject: I'll try this again...

Message-ID: <Pine.SOL.3.91.960306092659.2794A-100000@ucsub.Colorado.EDU>

I just thought I'd say hello to the list, I just got on the other day.

Gray hairs my *SS!  I'm 22 years old and have been 'planning' my Impala

for a couple years now.  Unfortuantely it will have to wait until

after I graduate (Mechanical Engineering -  Boulder, Colorado), and more

accurately, get a job.  And if they are really going out of production

at the end of this year (*sob*), I'll have to find a nice used one.  If

anyone has any special tips on how to tell if an Impala has been seriously

abused I'd appreciate it.  Or if anyone knows of a little old lady who

owns one and might like to sell it in about a year.....well, maybe not.

Even though I don't own one, the info is still great, and it's really

cool to see that a list like this exists.  If anyone is going past the

Denver area on their way to the good times in Arlington, please let me

know, I'd love to try and go.

Still wet behind the ears,

Charlie

------------------------------

Date: Wed, 6 Mar 1996 08:42:44 PST

From: dhflores@iwl.net (Debbie/Henry Flores)

To: impala@east.xsis.xerox.com

Subject: $2800 Wish list

Message-ID: <96Mar6.084615pst."14846(3)"@alpha.xerox.com>

Ok fellas, here is my gear head question to each and every one of you.

I have about $2800 to spend on my next upgrade. If you had this money what

specifically would you buy for a performance oriented upgrade and from whom

and why?  This sounds like a collegiate essay test doesn't it?  I would

really appreciate all of your ideas. Thank you all up front.

The beast currently has: Trasgo shift kit, 3.42 gears, 52mm throttle body,

air foil, enlarged mass air housing, Hennessey induction system, Jacobs

ignition system, Hypertech programmer, Borla exhaust, Power Pulley at the

crank only. All these added up to a 14.94 ET at 95 mph.

Henry in Houston

------------------------------

Date: Wed, 6 Mar 1996 11:05:48 PST

From: Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

To: impala@east.xsis.xerox.com

Subject: New Orders/Heat Reflective Windshield

Message-ID: <313DE20C.56A3@lfwc.lockheed.com>

Since I didn't see this posted to the list, and I thought it might be of

interest to some, I'm trying it one more time.

> Date: Wed, 28 Feb 1996 03:27:37 PST

> From: "Scott Blum" <rsblum@directmedia.com>

> To: impala@east.xsis.xerox.com

> Subject: RE:  Radar/Heat Reflective Windshield

> Message-ID: <n1386625923.57240@smtp.directmedia.com>

>

> Kelly you asked,

>

> From my recent experience, it seems that some radar detectors do work in the Impala with a heat reflective windshield.  Every morning, on my

way to work, there is a predictable speed trap. My Solo is mounted

between the headliner and the top of the rear-view mirror.  The warning

begins about 1/4 to 1/2 a mile ahead of the trap.  I believe the warning

time is reduced but it is still there.  I plan to test this detector in

my wife's Blazer to compare warning times.

>

> >>Comments?

>

> Scott replied:

>

> >>I mistakenly ordered my '96 with the SunGuard windshield before the thread about the effects to radar detection began herein.  Although I

quickly tried to change my order, it had already been preferenced....

>

To those of you who are considering changing your order, your order can

be changed almost until the order hits the factory floor.  The change

must be initiated by the dealer, through the respective zone office, to

the factory.  The zone office can fax the change to the factory.  I found

out my order didn't have the reflective windshield, and I preferred

that option.  To make the story short, my car was delivered to me on

Monday, 02/26, with the reflective windshield in place.  Where there's a

will, there's a way.  This process of making changes is not widely

advertised.  My dealer didn't even know it could be done.  The name of

the person at the factory who is responsible for making the changes is

named Linda, but neither can I remember her department, nor find her fax

number.  I believe she works in traffic, but that's only a guess. Sorry.

If anyone is interested, I can try to search my files or find out from my

source in Arlington.  Later.

Mark

------------------------------

Date: Wed, 6 Mar 1996 09:56:25 PST

From: Joe Mendrzycki <73541.1501@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Careful, Careful....

Message-ID: <960306175624_73541.1501_EHT63-1@CompuServe.COM>

>>  Only Q can change the laws of physics to suit

his desires, unfortunately us mortals have to contend with these laws as

they are! <g> Scott.  <<

Careful, Scott!  We don't want anybody thinking we're turning this into a

trekkie forum, now do we???  :-)    :-)

Anyway, may the Impala SS live long and prosper........

Joe M.  ('95 Black Impala SS)

------------------------------

Date: Wed, 6 Mar 1996 11:18:36 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: HOTELS

Message-ID: <960306141834_161763122@mail04.mail.aol.com>

many have asked about the hotels in Arlington.  I made my reservations at the

Fairfield Inn on I-20 @ Cooper.   817-649-5800.  59 bucks a night.

hope to see you there, Troy

------------------------------

Date: Wed, 6 Mar 1996 11:18:28 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Club Logo

Message-ID: <960306141828_161763144@emout07.mail.aol.com>

Hi again,

Alright, let's put our heads together..  I have a club logo that I've used

for the past shows, etc.  It is the new style deer logo inside an oval with

"National Association" on the top outside of the oval and "94-6 Impala SS

Owners" on the bottom, outside the oval.  Inside the oval and behind the deer

logo we used to have the FL state outline.  I used this before we went

national.  I removed the state outline and replaced it with a nice early

cross flag set.  Here is my problem, it is too close for comfort to the

National Impala Associations logo.  Theirs is copy protected and there is

enough changes in the deer style between the old and new too not really get

into legal conflict.   However, I have a gentleman's agreement with Dennis

Naasz of NIA  that I won't make something that looks like a copy of their

logo.  Late Great Chevy uses a circle with a bowtie and twin flags.

I need something else (Chev. related) to put with the new deer logo to help

our cars stand out.  Maybe the new rounded bowtie or an old dead cobra emblem

or something.  Send me some suggestions.  I am trying to get this on a tee

shirt for April.  Troy

ps - anyone out there in the tshirt business?

"So many projects, so little time"

T. Willrick

NAISSO

Daytona Beach, home of "Bike Week", "Speedweeks", "Spring Break", "Black

College Reunion", "The World's Most Famous Beach", "The World Center of Stock

Car Racing" and other things that make driving in this town impossible.

------------------------------

Date: Wed, 6 Mar 1996 11:47:16 PST

From: John Latrobe <racerx@jagunet.com>

To: "'impala@east.xsis.xerox.com'" <impala@east.xsis.xerox.com>

Subject: Suspension Upgrades

Message-ID: <01BB0B6B.EB715560@dlup-a09.jaguNET.com>

Sorry if this subject is getting old, but I'm trying to decide on the

best way to improve my Impala's handling.

The following prices were recently quoted on the list.

Herb Adams sway bars $216 + $50 + $176 = $442

Bilstein shocks 4 * $65 = $260

Eibach springs $180

So far I've come up with the following rating scales (1 is best):

Price (easy)

1. springs    2. shocks    3. bars

Installation

1. shocks     2. bars      3. springs

(shocks win only because the bars require extra mounting hardware,

springs obviously require much more effort)

Appearance

1. springs    2. bars      3. shocks

(springs are way ahead in this one)

Performance

1. ?          2. ?         3. ?

(this is where I need everyone's help)

The only factor that has kept me from purchasing the springs is the

reduced ground clearance. Has this been a problem for anyone?

I love that lowered look but don't want to worry about where I can go.

I am mainly concerned about eliminating/reducing the wallowing feel in

the rear over bumps and dips.

If you couldn't afford all three, would you buy just one or get the

springs and shocks together (same price as the bars)?

Thanks for any feedback,

John Latrobe

'96 Impala SS, Black

------------------------------

Date: Wed, 6 Mar 1996 13:17:21 PST

From: randomtech@earthlink.net ( Random  Technology)

To: impala@east.xsis.xerox.com

Subject: The Need for Speed

Message-ID: <v01530500ad635d044583@[153.37.16.185]>

First a couple of answers, then a few comments. Several members of the

list have asked about the modifications necessary to install our RamMax ram

air system. Essentially, there are none. The only cutting that has to be

done is to the center section of the air dam, but we will include a precut

part with the system. All of the original intake system may be removed when

the RamMax components are installed. Our system includes an air filter

housing that accommodates the stock filter or a K&N replacement.

As for water, it's potentially a problem, as it is with any type of ram

air system. We have included some design features which should minimize any

problems, however, if there's standing water on the roads, or you're

driving through a monsoon for an hour or so, a sufficient amount of water

can enter the system to either ruin the filter or make its way into the

engine. To avoid these problems, we have incorporated a quick disconnect of

the ducting from the scoop to the air cleaner box. (We did the same thing

on our Camaro/Firebird system.) It takes less than a minute to disconnect

the duct, and with a little common sense, there should never be a problem

with water damage. Should the need arise, the system can be easily removed

and the stock parts reinstalled.

Now for a few comments. Regarding the opinin expressed regarding throttle

body air foils and underdrive pulleys--  Our experience with both the

throttle body air foil and underdrive pulleys has been positive. If you

ever have a chance to listen to a throttle body on a flow bench, with and

without an air foil, you won't believe the difference. The turbulence

caused by the stock configuration creates an outrageous amount of noise. As

soon as the air foil is slipped in, things quiet down. In terms of

horsepower improvement, actual numbers will differ depending on the amount

of engine modification, which determines engine air flow requirements. If

the engine doesn't have a high air demand, or if the intake system is too

restrictive, the maximum flow capacity of the stock throttle body will not

be reached, so increasing it won't have much effect. At higher air demands,

the air foil will add 5 to 10 horsepower.

Regarding underdrive pulleys, we've seen consistent improvements of .15

to .2 seconds in the quarter mile. With an LT1 engine, there should be no

negative effect because only the crankshaft pulley is changed. Alternator

speed is high enough at idle to keep the battery charging and since the

water pump is not belt driven, there's no effect on cooling.

------------------------------

Date: Wed, 6 Mar 1996 13:44:46 PST

From: bnyk@oanet.com (Bernie Nykyforuk)

To: impala@east.xsis.xerox.com

Subject: Torque converters

Message-ID: <199603062144.OAA17214@cronus.oanet.com>

Hi Guys my name is Nolan, I've read the entire archives and been on the mailing

list for about a month now. I don't own a Impala but I use alot of the info

and tips on my 93 Sonoma(195HP 4.3L CPI with 4L60E). In my magizine they

show the Chevy race shops 1955 210, equiped with a built up 4.3L V8 making

329HP and 273lb/ft troque. In the specs they list that they used the 4L60E

and  High stall 4.3L V6 locking torque converter p.n./24202310. I have this

torque converted in my truck and it stalls at around 2000-2200 RPM.I was

thinking that this converter might give as good or better results than the

85-86 corvette converter.My engine makes 260lb/ft torque at 3600 rpm and the

truck is

about 3000 pounds.With the 330pounds of torque and 4000+ weight of the

Impala it would give alot more stall!

It might be a better converter to use then the corvette converter. Anyway I

don't

know if I'm right or way off. Please  me the let me know what you think.

(eg Scott M)

Yours truely

Nolan Nykyforuk

bnyk@oanet.com

CPI Sonoma in the great white north eh.

------------------------------

Date: Wed, 6 Mar 1996 14:49:18 PST

From: daryl@austin.ibm.com (Daryl Green)

To: impala@east.xsis.xerox.com (impala)

Subject: Blue SS sighting

Message-ID: <9603062249.AA18480@augusta.austin.ibm.com>

I was driving along today and saw an SS in a parking lot that was a

sort of slate blue in color.  I turned around, pulled in, and wrote down

the VIN.  It was 1G1BL52P4SR167573.  It was either a '94 or '95 (no

floor shifter) and I'd swear it was not a 9C1 dressed up as an SS but it

was definitely not black or DCM or any shade of green.  It was blue.

Daryl Green

daryl@austin.ibm.com

------------------------------

Date: Wed, 6 Mar 1996 15:01:20 PST

From: "Scott Zeiter" <ScottZeiter@msn.com>

To: impala@east.xsis.xerox.com

Subject: Shamu has Arrived!!

Message-ID: <UPMAIL08.199603062256350576@msn.com>

The Black Beast has finally arrived.  If anyone is planning on ordering under

the GM Option I plan, you may have some waiting to do.  It's obvious that

employees are given less priority than others:

Ordered: 9/21/1995

Preferenced: 1/22/1996

Built: 2/20/1996

Shipped?: 2/20/1996

Received at dealer: 3/4/1996

I'm having the Felpro gasket installed before I take delivery tomorrow, no

problems at all from my dealer (Terry Thiel at  Champion Chevrolet, Howell,

Michigan), and a few minor cosmetic touch-ups (greasy finger prints all over

the interior).

Arlington?  Looks good!

------------------------------

Date: Wed, 6 Mar 1996 15:40:43 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: part #'s for Fuel Inj. Cvrs.

Message-ID: <313E227B.CC7@ns.cencom.net>

SSers

I am preparing to order all the parts necessary to install the fuel Inj.

covers per tech digest.  I am going to retain the stock valve covers.  I

wanted to know for sure what the part #'s were for the fuel injection

covers - non Corvette valve cover application.  Any other info in this

regard would be appreciated.

Fellow members have told me that the American Sunroof runs about 850-950

installed.  The best I can get (Cent. IL) is 1150 + tax.  Any ideas on

where I could go reasonable distance to get a reasonable price??  Any

other brands besides American?

State Police Caprice toy cars are available to some degree in Target

Stores.  I got one of the Utah cars and I think it was around 3-4 bucks.

Did GM ever make an Impala promo car like the Vette and Z-28 that is at

about every GM Parts counter I've seen?

Sure wish we knew which Hotel was going to be "our" base of operations

in Arlington.  Time draws near and reservations could be hard to make.

Started snowing again here in IL, wish I had the 2nd gear start!

Loren DCM96 SPE

Texas or Bust

------------------------------

Date: Wed, 6 Mar 1996 15:46:30 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Bow tie decal for grill

Message-ID: <313E23D6.60DB@ns.cencom.net>

I am having my decal guy make a decal - exact fit to go over the chrome

emblem on the grill.  If anyone wants one, let me know.  He can make

them in any color.  I will have RED of course.  This application would

be a little easier than the cut out method.  I am also working on a SS

decal (Vinyl) for the stock valve covers.  If anyone else has any ideas

for decals, let me know.  If my car was black, I would put BIG # 3's on

the doors!

Loren DCM96 SPE

Texas or Bust

------------------------------

End of IMPALA Digest 331

************************

IMPALA Digest 332

Topics covered in this issue include:

1) Fairfield Inn it is

by Loren Peterson <equ_rino@cencom.net>

2) Re: Torque converter

by Curt Platteborze <75511.1220@compuserve.com>

3) Arlington plant employees

by SManey0830@aol.com

4) CD players/changers

by SManey0830@aol.com

5) Re: Was I Foiled?

by Robert Cheek <rcheek@one.net>

6) Torque Converters

by Robert Cheek <rcheek@one.net>

7) Brakes

by Robert Cheek <rcheek@one.net>

8) T84 headlights???

by MeeKyung73@aol.com

9) 96 C 14 RECALL

by BGordon401@aol.com

10) pulleys and airfoils

by john spears <speartec@iquest.net>

11) Arlington Trip?

by bertran5@egr.msu.edu (Erik Bertrand)

12) Flash-to-Pass

by "C. K. Yee" <yu135008@yorku.ca>

13) #3 on the doors of Shamu??

by BGordon401@aol.com

14) CANCEL

by FierosRcul@aol.com

15) Aol SS Chat night.

by Bngwip@aol.com

16) Detailed Arlington Hotel Information

by Matt Platts <mplat@airmail.net>

----------------------------------------------------------------------

Date: Wed, 6 Mar 1996 16:18:13 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Fairfield Inn it is

Message-ID: <313E2B45.46F3@ns.cencom.net>

Troy,

Thank you oh great one.  I am getting on the phone as we ?speak?

to make reservations at the Fairfield 817-649-5800.

SSers,

Get on the phone, make your reservations.......theres safety in numbers!

Loren

DCM96 SPE...still

Texas or Bust

------------------------------

Date: Wed, 6 Mar 1996 16:54:54 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Torque converter

Message-ID: <960307005453_75511.1220_GHJ90-1@CompuServe.COM>

Steve/Razzor,

Friday night I installed the Trans Go shift kit and also the Vette Torque

converter. What a fantastic combo. It's harder than ever now not to punch it

every chance I get. My poor snows won't last the rest of this winter. 1st 2nd

shift at WOT will get rubber every time for extended periods. I can't wait to

get real tires on it to stick better to dry pavement. I really have to feather

it a bit to keep the car straight. But "What fun it is"!

Not sure you need higher stall than the Vette unit. Price is right too!

The GM part number is 24201203 and it was a remanufactured unit. I was surprised

at the cost as it was lower than I thought. $146.25 plus a $30.00 core charge

which I will eat and keep the original in a box with my other collection of

stock components.

Good luck!

Curt P.

------------------------------

Date: Wed, 6 Mar 1996 16:56:42 PST

From: SManey0830@aol.com

To: impala@east.xsis.xerox.com

Subject: Arlington plant employees

Message-ID: <960306195639_239399127@emout04.mail.aol.com>

Any Arlington GM employees watching this site? If so, please send me an

email. Thanks.

------------------------------

Date: Wed, 6 Mar 1996 17:08:22 PST

From: SManey0830@aol.com

To: impala@east.xsis.xerox.com

Subject: CD players/changers

Message-ID: <960306200822_239409443@emout08.mail.aol.com>

I saw an archive note or note awhile back about Imp SS that I thought would

be worthy of recycling: If you want to be able to add the $550 Delco 12 disc

cd changer, you must order the cassette as the optional cd player is not

built to directly connect the cd changer. I also phoned my dealer to verify

as it sounded strange to me! An important item for all those ordering Impalas

now.

------------------------------

Date: Wed, 6 Mar 1996 17:15:48 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Re: Was I Foiled?

Message-ID: <1.5.4b11.32.19960307011548.006878c4@mail.one.net>

>Alright so air foils are pretty much bunk?

Lessee, going by Scott's rough average of $20 per horsepower <G>, that

means that if you can squeeze about 3HP out of the thing (possible, I

think), then you're doing pretty good. I'd keep it...

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Wed, 6 Mar 1996 17:15:50 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Torque Converters

Message-ID: <1.5.4b11.32.19960307011550.0068e128@mail.one.net>

I posted this a while back, but no one responded: with a higher stall

converter, will it still lock up lumbering down the highway at, say,

1800 RPM? Also, what is the going price for the 87(?) converter and what is

its stall?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Wed, 6 Mar 1996 17:15:54 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Brakes

Message-ID: <1.5.4b11.32.19960307011554.0066a9b4@mail.one.net>

Several brake questions. Please reply ASAP as I have an appointment with the

dealer tomorrow. Thanks much.

1. (MOST IMPORTANT) I was looking at a '96 on the dealer lot today and

noticed that the front calipers were painted black and the rears were

painted silver. Cool, I thought. So, I went to look at mine to see if there

was paint underneath all of the road grime that might be on them. When

I looked at the front rotors, I noticed that there were some vertical lines

from the bottoms of them to the tops. I can scrape my fingernail over them

and it catches, so I think they are cracks. Could the person who stated that

the 9C1 deflectors cause problems on some cop cars please repost what they

said to him: I'm afraid this might be a side effect. Does anyone else who

drives their Impala in inclimate weather check and see if this has happened

to them, too? I'm afraid this might be a serious safety issue.

2. It's cold, wet, and nasty here, and I think I might be needing pads soon.

While they've got the wheels off for the bearing repack, I'm thinking about

just letting them change 'em. Is $110 fair? I assume, but do know know if,

this includes the pads.

3. Regarding the painted calipers, when did this start. The silver looks

pretty cool, but my '94 definitely does not have it.

Thanks for your help,

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Wed, 6 Mar 1996 17:37:21 PST

From: MeeKyung73@aol.com

To: impala@east.xsis.xerox.com

Subject: T84 headlights???

Message-ID: <960306203721_439958395@emout09.mail.aol.com>

Anybody know if the T84's are Xenon gas bulbs or know where I can get this

type of bulb, to fit on the Impala.

------------------------------

Date: Wed, 6 Mar 1996 17:42:32 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Subject: 96 C 14 RECALL

Message-ID: <960306204231_343324430@emout05.mail.aol.com>

O.K. folks, here's the latest on the recall.

I talked with the dealer today, and they informed me that the recall notice

was "sent in error" to me because the recall only applied to 9C1 police cars

and heavy duty taxis.  HMMM, isn't our car made by the same people with the

same parts as those who make these cars in Arlington??  I brought this point

up with the dealer, but it went through one ear and out the other.  They

couldn't do anything because my VIN was not valid for the recall.  Needless

to say, I was p*ssed about their attitude about my car.

BTW, the dealer I'm talking about here is the place from where I bought the

car, and not the one I get work done at, like my 3.42 gear swap.  This dealer

is the one that sent me the recall notice.  Since I've bought my SS, I've

found a MUCH MORE knowledgeable parts/service department with which I choose

to do business.

After talking with the service rep, with no real information with which to

work, except that they wouldn't help me, I called up the Chevy Customer

Service Center to find out what was going on.  These people, in my opinion,

are a wealth of information and gave me the information I needed.

It turns out that THERE IS A PROBLEM WITH THE TORQUE ON THE LOWER BALL

JOINTS.  Service vehicles have had the lower ball joint knuckle nuts come off

because of being too loose from the factory.  In other words, the nuts were

backed-off when tightened instead of going until the next groove in the nut

for the cotter pin to go through when proper torque was reached.

I informed the woman that this condition existed on our cars as well, and

told her my experience when I checked the ball joints and found them to be

under spec.  She was typing the whole time I was giving her the details.  At

the end, she wanted my name, address, and phone number and asked if I minded

if somebody contacted me concerning this issue.  Since our car came off the

same line, it made sense to her that this recall should apply to the Impalas

as well.

Hope this clears things up for some of you.  The moral of this story, get

those ball joint nuts checked before somthing potentially happens when you're

blowing past a Mustang!!

..Bruce

------------------------------

Date: Wed, 6 Mar 1996 17:51:00 PST

From: john spears <speartec@iquest.net>

To: impala@east.xsis.xerox.com

Subject: pulleys and airfoils

Message-ID: <m0tuUrg-004XGMC@iquest.net>

What the heck. I might as well throw my $.02 worth in here. I did some

dragstrip testing about a year and a half ago on my 87 Grand National with

an underdrive pulley. I put in on, took it off, put it on, and took it off.

This was done on different nights, not all in one night. I determined that

the pulley was worth a solid tenth of a second! However, this was called a

"race only" pulley with a 48% reduction rate!! With normal around town

driving I had to periodically put a battery charger on the car overnight. I

don't beleive the reduction rate on the LT1 pulley is close to this much, so

I don't beleive the ET reduction would be near this much either!! Plus the

fact that the LT1 drives the water pump internally, reducing the ET

reduction potential of the pulley even more. I don't expect the companies

pushing these parts to say that the parts are worthless in terms of ET

reduction, but many of them are just that. When I think back to all of the

parts I had on my GN to make it a true 11.80 car and then compare to what

was CLAIMED by the manufacturer of each part, my car should have been

running about mid 6.0 times!!! If the pulley claims were true a guy ought to

be able to parallel a couple of fully charged 12 volt batteries and leave

the belt off alltogether. There's the ultimate accesory load reduction;

should be worth a good second in the quarter mile, right? No way! There just

isn't that much load there. Look at the work that went into getting 30 HP

extra on the LT4. Why not throw in an airfoil for a 40 HP improvement? I

think Scott summed it up best in reference to SLP and the SS Camaro. They

are trying to extract every available legal horsepower/dollar on these cars.

Where's the airfoil and underdrive pulley......? Over on the shelf with the

splitfire sparkplugs!!!! You don't even have to be a physics major to figure

it out.

John Spears

91 alum. head SS wagon

------------------------------

Date: Wed, 6 Mar 1996 18:54:04 PST

From: bertran5@egr.msu.edu (Erik Bertrand)

To: impala@east.xsis.xerox.com

Subject: Arlington Trip?

Message-ID: <9603070254.AA12665@egr.msu.edu>

Hey Fellow SS'ers...

I have heard some rumors about a possible trip to the Arlington

area (plant) sometime in April?  Are these statements fact or fiction?

I live in Lansing, Michigan and I would be up for a trip of sorts!!

Erik Bertrand

------------------------------

Date: Wed, 6 Mar 1996 19:18:58 PST

From: "C. K. Yee" <yu135008@yorku.ca>

To: impala@east.xsis.xerox.com

Subject: Flash-to-Pass

Message-ID: <Pine.SOL.3.91.960306221649.19170C-100000@sunlight.ccs.yorku.ca>

Hello everyone, I've been following this list for the past couple

months and have enjoyed the quality of the posts here immensely!

I don't have an Impala, but do have the next best thing: a 1995

Fleetwood with 3.42 gears and the firm ride and handling suspension.

Scott M. is correct in pointing out that the Fleetwood comes with

the flash-to-pass feature.  Unfortunately, my car is equipped with

DRLs, so I have been unable to verify the use of this feature.  The

owner's and service manuals are at odds as to whether the high or

low beams are activated when the lever is pulled back.  However,

it does appear that the headlamps must be 'off' in order for the

flash-to-pass to work.  Since there is no change with the headlamps

when I pull back on the lever (without invoking the dimmer switch),

it probably functions on the low beams (as do the DRLS).

Upon examination of the headlamps wiring diagram, it appears that

only the headlamp relay and dimmer switch would differ on flash-to

pass equipped cars.  The headlamp relay has an extra terminal (E5) to

supply constant voltage (ultimately from maxifuse #11) through circuit

307 (yellow wire) to the dimmer switch (term. B).  I suppose the easiest

way to implement this would be to purchase the flash-to-pass dimmer

switch, and wire in B+ from headlamp circuit breaker #5.

Kevin Wong

(c/o yu135008@yorku.ca)

------------------------------

Date: Wed, 6 Mar 1996 19:42:30 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Subject: #3 on the doors of Shamu??

Message-ID: <960306224227_162229680@emout09.mail.aol.com>

>>If my car was black, I would put BIG # 3's on

the doors!<<

Jeez, I'm sorry to hear that.  If you had to have a BIG #3 on your car, you'd

be obligated to ram into all the other drivers that get in your way.  ;-)

Shamu wins with stealth and POWER...no need for tricks to "take out" the

competition.

..Bruce

------------------------------

Date: Wed, 6 Mar 1996 19:52:40 PST

From: FierosRcul@aol.com

To: impala@east.xsis.xerox.com

Subject: CANCEL

Message-ID: <960306225240_239561438@emout09.mail.aol.com>

fierosrcul@aol.com

------------------------------

Date: Wed, 6 Mar 1996 20:25:22 PST

From: Bngwip@aol.com

To: impala@east.xsis.xerox.com

Cc: Rock528@aol.com

Subject: Aol SS Chat night.

Message-ID: <960306232521_162270612@emout10.mail.aol.com>

Hi Folks,

Just a reminder for you aol guys. The first Impala SS room chat is Tomorrow

night at 10pm EST Hosted by yours truly, Bngwip .( Bng for short ) I hope to

see everybody there who can get there. The room will be called "Impala SS" .

( of course) I sure hope this works!!!

Brett.

------------------------------

Date: Wed, 6 Mar 1996 20:30:23 PST

From: Matt Platts <mplat@airmail.net>

To: "'Impala'" <impala@east.xsis.xerox.com>

Subject: Detailed Arlington Hotel Information

Message-ID: <01BB0BAC.981AE340@dal09-02.ppp.iadfw.net>

Living in the area, I decided to give you'all a hand finding lodging (if you haven't already).  I haven't seen any type of listing go out yet, if one has, please excuse the repetition.  I will start with South Arlington.  Partly because that's where I live,  and partly because it is better served with a major (and better) east-west highway, I-20.  Here is the list that I have put together, if anyone would like more info or a better first-hand description, just let me know.  Plus, if you know where you will be staying and would like instructions on how to get there just give me a call:

Matt Platts

Home: (817)472-6076

Pager: (817)651-2330 (put *99 after your number, I will know it's an Impala person)

* These are 'major' hotel chains/brands that are near I-30, HWY-360 (where the plant is), and I-20.

* None have covered parking

* Only the Hampton Inn in NORTH Arlington is fully booked, all others indicate plenty of rooms

*South Arlington (MAJOR retail/shopping/food area).

Best Western Cooper Inn & Suites

4024 Melear Dr (I-20, Cooper St. exit)

(817)784-9490

Area of HWY-360 and I-20, generally 7mi SW of the plant

2 story motel, 1 yr old

$68 King

Disc w/5 rooms or more $62

AAA Disc

Coffee, Refrig in each room

Hampton Inn - South Arlington

121 E I-20 (I-20, Matlock Rd. exit)

(817)467-3535  (800)426-7866

Area of HWY-360 and I-20, generally 6mi SW of the plant

2 story motel, 5+ yrs old

$65 King/2 Queen

No AAA disc

Holiday Inn Express

1721 Pleasant Place (I-20, Cooper St. exit)

(817)784-8750

Area of HWY-360 and I-20, generally 7mi SW of the plant

2 story motel, <1yr old

$68 King or 2 queen

(walk in discount, normally $76)

*Central/North Arlington (convention/Six Flags/tourist area):

Arlington Marriott Hotel

1500 Convention Center Dr E (I-30, Ballpark Way exit)

(817)261-8200  (800)228-9290

Area of HWY-360 and I-30, 2mi NW of the plant

Multi story (15+) hotel, 10+ yrs old

$90 King or 2 Double

Standard weekend discount rate

Budgetel Inns

2401 Diplomacy Dr (HWY-360, Six Flags Drive exit)

(817)633-2400

Area of HWY-360 and I-30, 3/4mi north of plant

2 story motel, <1yr old

2 Queen (1 person) $47.95 (2 person) $54.95

King (same)

AAA 10% disc

Comfort Inn Arlington

1601 E Division St (HWY-360, Division St exit)

(817)261-2300

Area of HWY-360 and I-30, 2mi E of plant

2-3? story motel, 10+ yrs old

King (1 person) $59

2 Double (1 person) $57

AAA 10% disc

Courtyard by Marriott

1500 Nolan Ryan Expwy (I-30, Ballpark Way exit)

(817)277-2774  (800)321-2211

Area of HWY-360 and I-30, 3mi NW of the plant

2 story motel, 10+ yrs old

King (1 person) $95 (2 person) $105

No AAA disc

Fairfield Inn

2500 E Lamar Blvd (HWY-360, Lamar/Avenue H exit)

(817)649-5800

Area of HWY-360 and I-30, 3mi N of the plant

2 story motel, 1yr old, kitchenettes & fireplaces (I've been told)

$79 King

$89 2 Double

No AAA

Flagship Inn

601 Avenue H East (HWY-360, Lamar/Avenue H exit)

(817)649-3313

Area of HWY-360 and I-30, 3mi N of the plant

$39 Reg room, 2 Double or 1 King

$49 Med room, 2 Queen

$59 Lg room, 1 King, couch, dining table

AAA 10% disc

Hampton Inn - North Arlington

2050 N Hwy-360 (?)

(214)988-8989  (800)426-7866

Area of HWY-360 and I-30, 4mi N of plant

2 story motel, 10+ yrs old

*completely booked for that weekend*

Hawthorn Suites Hotel

2401 Brookhollow Plaza Dr (HWY-360, Lamar/Ave H exit)

(817)640-1188  (800)527-1133

Area of HWY-360 and I-30, 3mi N of plant

$89 King suite, fireplace, kitchen

(weekend discount rate)

Hilton Arlington

2401 E Lamar Blvd (HWY-360, Lamar/Ave H exit)

(817)640-3322  (800)445-8667

Area of HWY-360 and I-30, 3mi N of plant

Multi story (15+) hotel, 15+yrs old

$145 Deluxe double/double

$135 King

No AAA

Holiday Inn Inns

HWY-360, Brown Blvd exit

(817)640-7712

Area of HWY-360 and I-30, 4mi N of plant

2 story motel, 10+yrs old

$87 regular, $78.80 w/AAA or Special weekend rate

King, King/sofa, 2 queen

Howard Johnson Arlington

HWY-360, Abrams St exit

(817)633-4000

DIRECTLY ACROSS HWY-360 FROM THE PLANT (closest location to plant, not the greatest area... :)

2 story motel, 1yr old

$45 King, 1 person

$50 2 Full, 2 person

AAA 10% disc

La Quinta Inn

825 N Watson Rd (HWY-360, Six Flags Drive exit)

(817)640-4142  (800)531-5900

Area of HWY-360 and I-30, 1mi N of plant

2 story motel, 10+ yrs old, recently renovated

$85 2 double

$89 King

AAA 20% disc

Lexington Hotel Suites

1607 N Watson Rd (HWY-360, Brown Blvd exit)

(817)640-4444

Area of HWY-360 and I-30, 4mi N of plant

3 story motel, residential setting, ?yrs old

$87 1 person

$97 2 person

Suite, Kitchen, King or 2 Double

Radisson Suite Hotel - Arlington

700 Avenue H East (HWY-360, Lamar/Ave H exit)

(817)640-0440  (800)333-3333

Area of HWY-360 and I-30, 3mi N of plant

Multi story hotel, 10+ yrs old

$99 King or 2 Double

No AAA

Residence Inn by Marriott

1050 Brookhollow Plaza Dr (HWY-360, Lamar/Ave H exit)

fax: (817)649-7600

Area of HWY-360 and I-30, 3mi N of plant

2 story motel, <3 yrs old

------------------------------

End of IMPALA Digest 332

************************

IMPALA Digest 333

Topics covered in this issue include:

1) Re: Rings for Impala

by Curt Platteborze <75511.1220@compuserve.com>

2) Re: Texas

by Curt Platteborze <75511.1220@compuserve.com>

3) Re: Vortech

by Curt Platteborze <75511.1220@compuserve.com>

4) Meeting place info

by Impalaclub@aol.com

5) Fwd: Vinci Hi Performance

by Impalaclub@aol.com

6) Re: FL chapters

by Rock528@aol.com

7) new steering wheel

by Impalaclub@aol.com

8) Impala models/posters

by Rock528@aol.com

9) Re: whattodo

by Rock528@aol.com

10) ARLINGTON UPDATE/NAMES

by Rock528@aol.com

11) RE: IMPALA digest 332

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

12) Power Pulleys

by mbailey@rinet.hunton.com (Bailey, Martin)

13) 3,000 Strike at Key GM Parts Factory

by Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

14) $2800 mod

by Erich Arndt <EARNDT@mednet.swmed.edu>

15) Re: bulb talk

by Rock528@aol.com

16) Re: calipers

by Rock528@aol.com

17) Re: GT Brake Rotors

by Rock528@aol.com

18) Re: motels

by Rock528@aol.com

----------------------------------------------------------------------

Date: Wed, 6 Mar 1996 20:53:18 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Rings for Impala

Message-ID: <960307045317_75511.1220_GHJ89-2@CompuServe.COM>

Scott,

Thanks for your comments regarding my lame Impala. I called the dealer today and

set up an appointment for Monday.

Talked to the service manager and told him about the leakage in cylinder #8. I

asked what he would do if they agree there is a problem. And if it is out of

spec for what GM considers a problem that needs to be fixed.

Told me did not know till I bring it in. I kept pushing and said I have had it

leak tested 3 times now and it is at 20% and I'm sure your equipment will read

the same. So let's assume it is at that level and GM decides it should be fixed.

"Would you put in a new block, engine or rebuild it?"  The reply was we would

pull the manifold, right head, oil pan and remove the piston from the block. We

would re-ring it and hone the cylinder and put it back together.

So I pretty much know what will happen at this point on Monday.

Stay tuned for the results. This really sucks dealing with internal parts to the

engine. I hope things go smoothly and they don't make it worse.

Curt P.

------------------------------

Date: Wed, 6 Mar 1996 21:45:52 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Texas

Message-ID: <960307054551_75511.1220_GHJ91-2@CompuServe.COM>

I also have reservations at the Fairfied Inn too.

By the way is anyone else driving 1800 miles to Texas? My wife keeps telling me

I'm becoming obsessed with the Impala. "Could she be right?"<G>

Sure is a long drive hope to drive with a few obsessed owners. Anyone interested

in this trip please Email me. I live near Boston and can re-route this trip to

include others along the way.

Curt P.    Mass.

------------------------------

Date: Wed, 6 Mar 1996 21:45:50 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Vortech

Message-ID: <960307054549_75511.1220_GHJ91-1@CompuServe.COM>

What's the verdict on the best blower for the LT1?

I think the next add for me should be the blower. I've heard a lot about Vortech

and have a price for one around $3000. Whoaaaa. Maybe a nice tax return. hmmm

Headers, blower, sway bars, heads on and on. Sure seems like this car is

addicting and always wants to go faster and faster. My head spins just thinking

about all the possibilities.

signed addicted,

Curt P.

------------------------------

Date: Wed, 6 Mar 1996 22:30:02 PST

From: Impalaclub@aol.com

To: impala@east.xsis.xerox.com

Subject: Meeting place info

Message-ID: <960307013001_239678636@mail04.mail.aol.com>

>Finally, do you know what our central meeting point (information center)

will be

at Arlington?  I know we'll be together at the GM plant and the Chevy show,

but

if there are a large number of us, arriving at different times, all staying

at

various hotels, it would be good to have one spot to get meeting times,

places,

directions, etc..  I know we don't want someone to come just for the show and

tour, but then be "lost" for the rest of the time.

Good idea.  Until further notice lets use the lobby of the Fairfield Inn.  It

is downtown Arlington and centrally located.  Maybe I can arrange a message

board to be put up there.

I am still hoping someone works for a large company that has an 800 number

that we can set up a voice mailbox.  Then we can just call in for the latest

updates as to where the group is headed.

Any other ideas??  Troy

------------------------------

Date: Wed, 6 Mar 1996 22:30:29 PST

From: Impalaclub@aol.com

To: Mbmarx@aol.com, stc@cypress.com, RAZZORFL@aol.com,

Subject: Fwd: Vinci Hi Performance

Message-ID: <960307013027_343618714@emout07.mail.aol.com>

In a message dated 96-03-05 17:20:30 EST, MarcusB360 writes:

Here is a forward from our local member that took his mothers 94 SS to Roger

Vinci's Performance in Orlando.  I saw the final graphs and they are very

impressive.  They charge about 170.00 to tune the car on the dyno.  The rear

wheel hp jumped up almost 40 hp.  The car has a JET stage 1, hennesey intake

and an airfoil only.    Troy

The 204 starting rear wheel hp was higher than I expected.

<< From:	MarcusB360@aol.com

To:	Impalaclub

CC:	MarcusB360

I took the Impala this morning to Vinci's. Started out at 204 hp and ended up

at 235 hp. Torque jumped up by about 40 lb/ft (i don't have the charts in

front of me and I don't remember the exact figures). There is a BIG

difference in the way the car drives now (remember, the Camaro SS was 225 hp

and 250 lb/ft).

If you'd like, I'll write up an article on Vinci's and give it to you for the

newsletter. The gave me a ton of info on how the dyno setup works (but not

their top secret tuning tips), and I think a lot of members will want to have

this done, especially when they see the charts.

If you're on tonight, e-mail me and I'll stop by the hangar.  >>

---------------------

Forwarded message:

Subj:    Vinci Hi Performance

Date:    96-03-05 17:20:30 EST

From:    MarcusB360

To:      Impalaclub

CC:      MarcusB360

I took the Impala this morning to Vinci's. Started out at 204 hp and ended up

at 235 hp. Torque jumped up by about 40 lb/ft (i don't have the charts in

front of me and I don't remember the exact figures). There is a BIG

difference in the way the car drives now (remember, the Camaro SS was 225 hp

and 250 lb/ft).

If you'd like, I'll write up an article on Vinci's and give it to you for the

newsletter. The gave me a ton of info on how the dyno setup works (but not

their top secret tuning tips), and I think a lot of members will want to have

this done, especially when they see the charts.

If you're on tonight, e-mail me and I'll stop by the hangar. Or call my

apartment at 274-1229 anytime tonight. I'll probably be up really late.

Marcus

------------------------------

Date: Wed, 6 Mar 1996 22:27:58 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: FL chapters

Message-ID: <960307012756_440200367@mail06.mail.aol.com>

In a message dated 96-03-06 10:22:53 EST, you write:

>I was interested to know if there are any Impala owners in the South Florida

>area that would be willing to get together and possibly form a SFL chapter

>of the NAISSO (if it's ok with you, Troy).

this is fine. I've sent you some contacts and I will provide some flyer

originals and applications.  Troy

------------------------------

Date: Wed, 6 Mar 1996 22:30:07 PST

From: Impalaclub@aol.com

To: impala@east.xsis.xerox.com

Subject: new steering wheel

Message-ID: <960307013007_239678671@emout05.mail.aol.com>

I got my steering wheel replaced today from the tilt and toot tsb.  One new

leather 1996 version steering wheel (twin bugle) and air bag assembly.

Total cost to dealer:  $906.00   ouch!   Troy

------------------------------

Date: Wed, 6 Mar 1996 22:27:58 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Impala models/posters

Message-ID: <960307012755_440200357@mail04.mail.aol.com>

I spoke with the manufacturer of the Buddy L Impala models today.  They

explained to me that they only received 72 orders for the impala ss models.

In a variety case of 12 Buddy L models there are only 4 impalas, 4 camaros,

etc..  They are made as they are ordered from china.  We could order a 1000

piece lot and pick them up on the dock in California but it takes a check up

front.  They are not stocked anywhere in the US.  You may find a few in

Target or We-Be-Toys depending on how many cases were ordered.  There is a

company placing an order that we could tag onto for a hundred quantity order

but I haven't got the funds in the club coffee can for the $1,000 bucks.

Anybody want to make a loan to the club?

I could also use some money up front to put the poster idea in motion.  It

will also take about a thousand dollars.  I need a loan for a few months till

we get off the ground.  Any one know any investors??   Troy

------------------------------

Date: Wed, 6 Mar 1996 22:27:58 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: whattodo

Message-ID: <960307012758_440200375@emout04.mail.aol.com>

In a message dated 96-03-06 18:50:12 EST, you write:

>I love that lowered look but don't want to worry about where I can go.

>I am mainly concerned about eliminating/reducing the wallowing feel in

>the rear over bumps and dips.

>If you couldn't afford all three, would you buy just one or get the

>springs and shocks together

John, do the springs for looks and the sway bars for feel.  I did the springs

and it firmed up the ride.  Don't worry about ground clearance if your not

using it off-road.

Just watch out for armadillos.  They're tough on the spoilers.  Troy

------------------------------

Date: Wed, 6 Mar 1996 22:27:54 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: ARLINGTON UPDATE/NAMES

Message-ID: <960307012754_440200345@mail02.mail.aol.com>

March 6, 1996

Greetings,

I spoke with the sponsor of the Texas Super Chevy show this am.  They will be

sending a letter to the local arranger that will let us in at the gate for

the preregistered price of $35.00.  There is really no need to preregister

since they will give us the ten dollars off at the gate.

They do need to know how many members we will have there for the show car

area.  Here is a list of persons that have contacted me with the intention of

driving/showing their impalas.  Another club member suggested a special drag

class for rental cars for the drags on Friday evening.  Try to rent Z28's, we

can have those heads switched in about 2 hours.

List of attendees driving Impalas:

Troy Willrick

Loren Peterson

Curt Platteborze

Mark Weber

Larry Lee

Jeff Green

Scott Mueller

Mike Sunderstrom

John Kelly

John Salmon

Mark Hawthorne

Daryl Green

Brett ??

Jeff Gatcha

Don Lambert

Steve Das

I sure hope we are going to have more than 16 at a national meet for all this

hype and trouble.

Troy

NAISSO

------------------------------

Date: Thu, 7 Mar 1996 03:31:04 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: impala@east.xsis.xerox.com

Subject: RE: IMPALA digest 332

Message-ID: <Chameleon.960307064422.hollenbe@mustang.east.xsis.xerox.com>

On Wed, 6 Mar 96 23:32:41 EST  impala@east.xsis.xerox.com wrote:

>Date: Wed, 6 Mar 1996 17:15:54 PST

>From: Robert Cheek <rcheek@one.net>

>To: impala@east.xsis.xerox.com

>Subject: Brakes

>Message-ID: <1.5.4b11.32.19960307011554.0066a9b4@mail.one.net>

>

>Several brake questions. Please reply ASAP as I have an appointment with the

>dealer tomorrow. Thanks much.

>

>1. (MOST IMPORTANT) I was looking at a '96 on the dealer lot today and

>noticed that the front calipers were painted black and the rears were

>painted silver. Cool, I thought. So, I went to look at mine to see if there

>was paint underneath all of the road grime that might be on them. When

>I looked at the front rotors, I noticed that there were some vertical lines

>from the bottoms of them to the tops. I can scrape my fingernail over them

>and it catches, so I think they are cracks. Could the person who stated that

>the 9C1 deflectors cause problems on some cop cars please repost what they

>said to him: I'm afraid this might be a side effect. Does anyone else who

>drives their Impala in inclimate weather check and see if this has happened

>to them, too? I'm afraid this might be a serious safety issue.

I spoke with a Virginia State Trooper who told me that their department

removes the deflectors as part of prepping a car to put it in service.

His claim was that these actually caused the brakes to overheat under

spirited driving conditions.

I have to think this guy was wrong, though, because these deflectors are

obviously an improvement over our stock Impala deflectors in terms of directing

air to the brakes, and removing them would seem to reduce the amount of air

directed to the brakes.  I'm not an expert in fluid dynamics, but when you

look at these things it seems like a no-brainer.

Scott Hollenbeck

------------------------------

Date: Thu, 7 Mar 1996 05:30:36 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (Impala1)

Subject: Power Pulleys

Message-ID: <1996Mar07.081900.1049.112961@rinet.hunton.com>

I should know better than to get into this, but.....   In the world of SHOs

(wash out my mouth), power pulleys are marketed as a device to avoid

damaging belt-driven accessories through high RPMs that can be achieved on

modified motors.  This can easily be >8500 rpm on a SHO motor.  Of course,

this would rarely, if ever, be the case on an LT1.  Wouldn't the pulleys

serve a useful function in such a case, even if they didn't add any HP?

Come on, you KNOW the answer to that! There have been many bogus parts sold

over the years. The "power pulleys" for the LT1 are a perfect example. <g>

In fact I am continually surprised by what products are sold without any

solid engineering or scientific testing behind them. That is much of the

reason I try to stick with factory parts for most of my mods.

------------------------------

Date: Thu, 7 Mar 1996 08:01:02 PST

From: Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

To: impala@east.xsis.xerox.com

Subject: 3,000 Strike at Key GM Parts Factory

Message-ID: <313F083E.64E6@lfwc.lockheed.com>

Not so good news for you guys who have new Impalas ordered.  The

Associated Press reports that workers at two factories that supply brakes

to nearly all GM assembly plants went on strike yesterday.  This turn of

events threatens to disrupt assembly of cars at the factory in Arlington.

GM evidently saw this coming, so there is a stockpile of parts.  The

strike will have no immediate effect.  They have about a 30-day supply of

parts on hand and in the supply pipeline.  The first plants to be

affected by the shortage of parts would most likely be Lansing and Orion

Township, Mich; Lordstown, Ohio; Wilmington, Del.; and Oshawa, Ontario.

Hopefully, the strike will be resolved soon.  In the event that some of

you do not put any faith in the AP, I asked a quality control person in

Arlington about it.  She confirmed the report.  Sorry for the news.

Mark

Stock '96

------------------------------

Date: Thu, 7 Mar 1996 06:58:35 PST

From: Erich Arndt <EARNDT@mednet.swmed.edu>

To: impala@east.xsis.xerox.com

Subject: $2800 mod

Message-ID: <s13ea570.016@MEDNET.SWMED.EDU>

Henry,

Scott's rocker arm mod sounds interesting and not too expensive.

Another option would be a crate LT4 motor...

A Vericom computer, maybe???

Thanks,

Erich Arndt

'94 Impala SS

Cedar Hill, TX

------------------------------

Date: Thu, 7 Mar 1996 08:37:49 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: bulb talk

Message-ID: <960307113748_239899533@mail02.mail.aol.com>

>Anybody know if the T84's are Xenon gas bulbs or know where I can get this

>type of bulb, to fit on the Impala.

We have this type of bulb in the nightsun of our helicopters.  It is the

20,000,000 candlepower version, however.  Generally, Xenon gas bulbs have a

shelf life.  Most of the highpower  80/100's are halogen, I believe.  The

T84's use the H4 style bulb instead of the 9004 bulb like in the stock impala

headlamp.  Troy

------------------------------

Date: Thu, 7 Mar 1996 08:37:41 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: calipers

Message-ID: <960307113739_239899508@emout07.mail.aol.com>

In a message dated 96-03-06 23:34:03 EST, you write:

>3. Regarding the painted calipers, when did this start. The silver looks

>pretty cool, but my '94 definitely does not have it.

I polished the rear aluminum calipers with a die grinder/scotchbrite pads and

metal polish.  They look like chrome.  Troy

------------------------------

Date: Thu, 7 Mar 1996 08:37:46 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: GT Brake Rotors

Message-ID: <960307113746_239899493@emout05.mail.aol.com>

I just set up a club discount for the GT drilled rotors.  These and the

polished factory wheels should look good.   These are the rotors with the

.030" dimples rather than the holes all the way through and have a light gold

protective finish.  Here is the club pricing.

GT Rotors

32 Wembly Drive

Mt. Laurel, NJ  08054

609-722-6707

Contact:  Conrad Till, owner

Regular price: 190.48 each x 4 = 761.92 - 10% discount = 685.73 (171.43 ea)

plus free freight

Troy

------------------------------

Date: Thu, 7 Mar 1996 08:37:53 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com, mplat@airmail.net

Subject: Re: motels

Message-ID: <960307113753_239899614@mail02.mail.aol.com>

In a message dated 96-03-06 23:34:03 EST, you write:

> Detailed Arlington Hotel Information

>Matt Platts

Matt,

great work on the hotels.  Many of us are staying at the Fairfield Inn.

Sounds like a nice place.  I got a room for $59.00 a night.  Can you go by

there and hammer them for a discount.  Tell them we will have 30 - 50 persons

checking in for 3-4 days and see what they can come up with.  Also, check on

the availablilty of a message board in the lobby area for members.  Thanks,

Troy

If you don't have any luck with them, tell them "Razzor the terminator" will

be calling!!

If you need any info you can reach me at:

home 904-253-6638

fax 904-258-3779

pager 904-248-1750, pin 1307 (digital or voice)

impalaclub@aol.com  or  rock528@aol.com

or within Volusia County dial 911

------------------------------

End of IMPALA Digest 333

************************

IMPALA Digest 334

Topics covered in this issue include:

1) Re: NAISSO Newsletter

by Rock528@aol.com

2) new member

by wooyang <wooyang@uclink2.berkeley.edu>

3) Re: Texas

by John.Salmon@bridge.bst.bls.com

4) Re: official statement

by Rock528@aol.com

5) Help 1996 Service Manuals Out-of-Stock.

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

6) Re:  $2800 Wish List

by Charlie Argento <argento@ucsub.colorado.edu>

7) injector covers

by Scott Mueller <73145.1566@compuserve.com>

8) bars, shocks, springs!

by Scott Mueller <73145.1566@compuserve.com>

9) Hotels

by Scott Mueller <73145.1566@compuserve.com>

10) Blue Impala = Green Grey

by Scott Mueller <73145.1566@compuserve.com>

11) Factory CD/tuners

by Charlie Argento <argento@ucsub.colorado.edu>

12) Decals

by Scott Mueller <73145.1566@compuserve.com>

13) Brake calipers

by Scott Mueller <73145.1566@compuserve.com>

14) Re: IMPALA digest 331

by michaela@lexis-nexis.com (Michael Affourtit)

15) Torque converters

by Scott Mueller <73145.1566@compuserve.com>

16) slate blue IMP SS

by john spears <speartec@iquest.net>

----------------------------------------------------------------------

Date: Thu, 7 Mar 1996 09:45:23 PST

From: Rock528@aol.com

To: mbailey@rinet.hunton.com, impala@east.xsis.xerox.com

Subject: Re: NAISSO Newsletter

Message-ID: <960307124522_343994226@emout09.mail.aol.com>

In a message dated 96-03-07 09:20:12 EST, you write:

>IMHO and FWIW, bulk mail is only worthwhile for mailing flyers in a few

>adjacent ZIP codes and for which there is no urgency at all.

Boy am I ever finding this out.  I sent the latest newsletter out last

Tuesday and separately mailed the Foreign ones by hand first class.  Of

course the one to Canada from Florida got there in a couple of days and

someone 20 minutes away still has not got the newsletter.  I think I will

have to switch back to first class.  Troy

Sorry guys, I guess this is the learning experience of progress.

------------------------------

Date: Thu, 7 Mar 1996 10:02:52 PST

From: wooyang <wooyang@uclink2.berkeley.edu>

To: impala <impala@east.xsis.xerox.com>

Subject: new member

Message-ID: <Pine.3.89.9603070924.A17809-0100000@uclink2.berkeley.edu>

I just thought I'd introduce myself to the list:

I dont own an impala but drive my dad's every chance I get (1995, Black,

9C1 deflectors, K & N).  I'm looking for a '93 & up 9C1 for myself.

Thanks to the list memebers and the Web site authors for all the info.

By the way, can anyone give me the tsb# for the tilt and toot?  I cant

find it in the archives.

Sincerely,

Woo Yang

U.C. Berkeley

------------------------------

Date: Thu, 7 Mar 1996 10:33:59 PST

From: John.Salmon@bridge.bst.bls.com

To: <impala@east.xsis.xerox.com>

Subject: Re: Texas

Message-ID: <H000018205b70c02>

I'm "Inn" at Fairfield! <G>

As to the long drive and "obsession" level, Curt, you should see the

look on people's faces when I explain that I paid to fly out to Dallas,

rent a car for 3 days, get a nice motel room, and pay the entry fee just

to see the IASCA finals last October.  They ask, "Just what is the IASCA

finals?"  I tell them IASCA is the Int'l Auto Sound Challenge Assoc.,

they say, "You mean you PAID all that to go hear cars with loud

stereos?"  Some people just don't understand, and never will....

For me, driving from Birmingham (AL) to Dallas won't be too bad, and $59

a night is better than the rate I got at the Holiday Inn Aristocrat in

downtown Dallas last year.  Baton Rouge/Florida convoy folks, I haven't

studied the route thoroughly yet, but if I-20 carries me close by, may I

meet up with you?

Thanks,John Salmon

'95 DCM, CD, Tint, KEF 160Qs & 690Qs, Soundstream Ref 405s and SPL 12s,

1LE intake, oil pressure gauge mod, Mobil 1 5W30 & rear lube, gasket.

"Off the screen, through the NIC, across the hub, over the router, past

the gateway, around the world...nothing but net!"

------------------------------

Date: Thu, 7 Mar 1996 10:41:19 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: official statement

Message-ID: <960307134118_162675951@mail04.mail.aol.com>

I wan't to apologize for the number of posts recently.  I don't mean to tie

up the digests with all this stuff.  I'll shut up for a while.

one more thing about being "obsessed" with impala:   Maybe we can look at

some farmland in West Texas where we could all live while we are there?  <g>

Troy

------------------------------

Date: Thu, 7 Mar 1996 12:45:48 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Subject: Help 1996 Service Manuals Out-of-Stock.

Message-ID: <9603072049.AA23065@karloff.fstrf.org>

Hi,

FWIW, I just got off the phone (7 Mar 96 15:30 EST) with Helm (service Rep.

was Erica) who told me that the service manuals for 1996 Impala SS's are

out-of-stock forat least 45 business days.  They took my name/address and will

send a postcard when they receive more copies.

They DO have the manuals for 1994 and 1995 Impala SS's.

- mike

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

My '96 Impala SS: "It goes fast, FAST!"

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

Date: Thu, 7 Mar 1996 13:01:51 PST

From: Charlie Argento <argento@ucsub.colorado.edu>

To: Chevy Impala SS <Impala@east.xsis.xerox.com>

Subject: Re:  $2800 Wish List

Message-ID: <Pine.SOL.3.91.960307134131.3950A-100000@ucsub.Colorado.EDU>

It sounds like you have quite a performer already.  If the money

doesn't have to go to upping the performance, $2800 could buy you a

really nice stereo system.  It's amazing what's been done in the past few

years with car audio.  You could even keep your factory Tape Player or CD

Player and just get new speakers and amplifiers, maybe an equalizer too.

Also, probably the neatest thing would be a DSP unit (Digital Signal

Processing).  Those things can make your car sound like anything from a

80,000 seat stadium to a small local bar.

Keep in mind that not all stereo upgrades are the type that rattle other

cars at red lights.  It all depends on what you want your music to sound

like.  Make a visit to a couple of local car audio shops and check out

what they can do.  I say a couple of shops because some will have

equipment that others don't, and the installers may not be as good.  Ask

to see if they have any pictures of past installations.  And look for

trophys too, there are many competitions for installations, volume

levels, and overall sound quality.  If a shop is really good at what

they do, their trophys should reflect that.

Check it out and have fun doing it!

Charlie

------------------------------

Date: Thu, 7 Mar 1996 13:08:56 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: injector covers

Message-ID: <960307210855_73145.1566_FHD34-16@CompuServe.COM>

Loren,

>I am preparing to order all the parts necessary to install the fuel Inj.

covers per tech digest.  I am going to retain the stock valve covers.  I

wanted to know for sure what the part #'s were for the fuel injection

covers - non Corvette valve cover application.  Any other info in this

regard would be appreciated.

The LH side injector cover should be ordered from the 94+ Corvette

application no matter what. That year was when the LT1 went to sequential

fuel injection and the pressure regulator and fuel inlet area changed. The

'93 or earlier injector cover will not clear the later pressure regulator.

On the RH side, it depends. If you are using the stock stamped steel

valvecovers, then you should use the RH injector cover from the '92-'93

Corvette application. This is because the stock valvecovers have the

crankcase vent tube on the top of the cover, as did the Magnesium LT1

valvecovers used on the '91-'92 Corvette. The RH injector cover for those

years has a clearance notch for the crankcase vent hose to pass through.

If however, you are also changing to the killer Corvette plastic valvecovers

(used on '94+ Corvette LT1/LT4 applications), then the RH cover has the vent

tube connection hidden on the side of the cover, and not right on top. In

that case, you should use the RH injector cover from the '94+ Corvette

application, since it does not have the clearance notch for the vent hose,

and would look much better.

An additional note is that if you order the covers from the '96 LT4

application, the embossed "Corvette" lettering is filled in with red paint.

This looks good if you use the covers as is, however in my case I have

factory "bowtie-SS" red emblems (from an S-10 "SS" pickup) on my injector

covers below the embossed "Corvette" lettering, which I think looks better

when using the LT1 injector covers which leave the Corvette lettering in

black.

BTW, although they seem expensive, the Corvette valve covers are also an

excellent addition to the engine. In addition to simply looking much better,

the Corvette plastic valvecovers offer several functional and performance

benefits over the stock stamped steel covers:

-They include special oil drippers for the rocker arms, which provides much

needed oiling to the stock stamped steel rockers. The additional oiling to

the rocker ball and tip prevents galling and reduces wear, in addition to

providing increased cooling of rocker and valvespring. The cheap stamped

steel covers lack these drippers.

-The very thick plastic construction (you will be surprised at how thick and

heavy they are) is designed to greatly reduce valvetrain noise. This

minimizes any valvetrain ticking noises you might otherwise hear with the

stamped steel covers.

-They never rust or corrode like the steel ones. All you ever have to do to

maintain them is apply a little protectant (armor-all, etc.) and they look

great! Scott.

------------------------------

Date: Thu, 7 Mar 1996 13:09:01 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: bars, shocks, springs!

Message-ID: <960307210901_73145.1566_FHD34-17@CompuServe.COM>

John,

>Performance

>1. ?          2. ?         3. ?

>(this is where I need everyone's help)

That one is easy:

1. Herb Adams Swaybars      2. Shocks  3. Springs

The bars will DEFINITELY give you the most improvement in the suspension,

which is why I listed them way out in front! <g> The shocks and springs are

closer together, and which one is first probably depends on exactly which

set of shocks you get, but usually I would place the shocks ahead of the

springs.

>The only factor that has kept me from purchasing the springs is the reduced

ground clearance. Has this been a problem for anyone? I love that lowered

look but don't want to worry about where I can go.

You may indeed have a problem with curbs, and probably end up scraping the

front spoiler quite a bit with the lowered springs. Also you may bottom out

on hard bumps. That might be improved by trimming the jounce bumpers some.

>I am mainly concerned about eliminating/reducing the wallowing feel in the

rear over bumps and dips.

The bars and shocks will do the most for that!

>If you couldn't afford all three, would you buy just one or get the springs

and shocks together (same price as the bars)?

I would definitely get the bars first, then the shocks. You might want to

re-consider the lower springs, especially if you drive the car a lot. If it

is a weekend cruiser or a track car, then the lower springs would be fine as

long as you are curb conscious. Scott.

------------------------------

Date: Thu, 7 Mar 1996 13:08:28 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Hotels

Message-ID: <960307210827_73145.1566_FHD34-11@CompuServe.COM>

Matt,

>Living in the area, I decided to give you'all a hand finding lodging (if

you haven't already).

THANKS for the hotel information!

I have only one question, where would YOU stay! I'd preferrably like a room

with a view (of my Impala in the lot right outside the window, that is)! <g>

Scott.

------------------------------

Date: Thu, 7 Mar 1996 13:08:40 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Blue Impala = Green Grey

Message-ID: <960307210840_73145.1566_FHD34-14@CompuServe.COM>

Daryl,

>I was driving along today and saw an SS in a parking lot that was a sort of

slate blue in color.

The color you saw is officially called Dark Green-Grey Metallic! I agree

that it looks like gun blue or slate blue to me also. Scott.

------------------------------

Date: Thu, 7 Mar 1996 13:43:45 PST

From: Charlie Argento <argento@ucsub.colorado.edu>

To: Chevy Impala SS <Impala@east.xsis.xerox.com>

Subject: Factory CD/tuners

Message-ID: <Pine.SOL.3.91.960307143239.3950B-100000@ucsub.Colorado.EDU>

I saw a picture of the Delco CD/tuner the other day, and I was

wondering if that was an 'AUX' button beneath the 'CD' button?  I couldn't

read the picture very well.  If so, I assume this is for an Auxilary

input to the stereo.  Does anyone know any details on hooking this up?

Can it be used for adding and aftermarket CD or MD changer?  If I was to

upgrade my stereo...maybe with a spare $2800....I would like to keep my

factory head unit to keep out any would-be theives.  A nice CD changer

would fit well into the glove box, and you wouldn't have to get into the

trunk to change the discs.

May The Force be with Impalas,

Charlie

------------------------------

Date: Thu, 7 Mar 1996 13:08:44 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Decals

Message-ID: <960307210844_73145.1566_FHD34-15@CompuServe.COM>

Loren,

>I am having my decal guy make a decal - exact fit to go over the chrome

emblem on the grill.  If anyone wants one, let me know.  He can make them in

any color.  I will have RED of course.

I'd be interested in any of the decals you have made. See if they can make

the front one in a deep red reflective tape. This really looks cool as it

lights up at night when headlights shine on it!

I think another decal idea would be an exact duplicate of the rear quarter

panel "Impala SS" emblems in reflective red or chrome (mylar). Scott.

------------------------------

Date: Thu, 7 Mar 1996 13:08:23 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Brake calipers

Message-ID: <960307210823_73145.1566_FHD34-10@CompuServe.COM>

Rob,

>Regarding the painted calipers, when did this start. The silver looks

pretty cool, but my '94 definitely does not have it.

That started in '94! In fact ALL Impalas get painted (black) front calipers,

and both the front and rear brake rotors are specially coated with a

corrosion resistant material. The rotors for the 9C1 cars (both front and

rear) actually have different numbers, as they come uncoated and of course

turn orange instantly. That would have looked quite ugly through the Impala

wheels, so Chevy arranged to have the Impala rotors coated, and the front

calipers painted.

BTW, the rear calipers are not painted silver, they are ALUMINUM! All

Impalas and 9C1 cars get the aluminum (unpainted) rear disc brake calipers.

Scott.

------------------------------

Date: Thu, 7 Mar 1996 14:14:41 PST

From: michaela@lexis-nexis.com (Michael Affourtit)

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 331

Message-ID: <9603072214.AA00511@mutant.lexis-nexis.com>

[Date: Wed, 6 Mar 1996 11:18:28 PST

[From: Rock528@aol.com

[To: impala@east.xsis.xerox.com

[Subject: Club Logo

[Message-ID: <960306141828_161763144@emout07.mail.aol.com>

[

[Hi again,

[

[Alright, let's put our heads together..  I have a club logo that I've used

[for the past shows, etc.  It is the new style deer logo inside an oval with

["National Association" on the top outside of the oval and "94-6 Impala SS

[Owners" on the bottom, outside the oval.  Inside the oval and behind the deer

[logo we used to have the FL state outline.  I used this before we went

[national.  I removed the state outline and replaced it with a nice early

[cross flag set.  Here is my problem, it is too close for comfort to the

[National Impala Associations logo.  Theirs is copy protected and there is

[enough changes in the deer style between the old and new too not really get

[into legal conflict.   However, I have a gentleman's agreement with Dennis

[Naasz of NIA  that I won't make something that looks like a copy of their

[logo.  Late Great Chevy uses a circle with a bowtie and twin flags.

[

[I need something else (Chev. related) to put with the new deer logo to help

[our cars stand out.  Maybe the new rounded bowtie or an old dead cobra emblem

[or something.  Send me some suggestions.  I am trying to get this on a tee

[shirt for April.  Troy

[

[ps - anyone out there in the tshirt business?

[

["So many projects, so little time"

Hmmmm... I am also "Mutant Grafix" and I was going to invest 1700 or so into a small shirt operation. I have done small runs with the equipment I have, but its not a multicolor setup.

I also do a lot of design work on the side.... Let me know if there is any interest.

Mike

| Mike Affourtit --------------------- | ---------------- |

|                                      |"Don't lay your   |

| Lexis-Nexis                          | heart out on a   |

| Unix System Administration/DCE       | plate and expect |

| michaela@lexis-nexis.com             | me to watch it   |

| http://www.lexis-nexis.com/~michaela | beat..."         |

| (513) 865-6800 x5618 --------------- | ---------------- |

------------------------------

Date: Thu, 7 Mar 1996 13:08:09 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Torque converters

Message-ID: <960307210809_73145.1566_FHD34-8@CompuServe.COM>

Steve,

>Do you know the stall speed of the stock Impala converter? I thought it was

around 1800-2200.

Unfortunately GM does not rate their converters by "stall speed", as this

will vary on a given converter with different torque outputs. It does seem

that the stock converter is pretty tight at 1800.

>The vette converter is 2500 (per your info).  I am thinking that to really

feel the difference of going to a higher stall converter, you would have to

jump at least 500 rpm over what we have now.

I would guess that the 'vette converter stalls at 2200-2500 or so. It does

feel significantly looser than the stock one!

>There are some converters out there that go to 2700/2800. Would these be

too high.

They are too high for me, but if you wanted maximum quarter mile

performance, they might be appropriate.

>BTW, I got a great price on the '86 Vette converter at $146.25 exchange.

Not bad! $175 for install and you got it for around $325.

You can't beat that! That is one reason I so highly recommend this mod, it

is all factory, it works, and it is cheap! Scott.

------------------------------

Date: Thu, 7 Mar 1996 14:39:00 PST

From: john spears <speartec@iquest.net>

To: impala@east.xsis.xerox.com

Subject: slate blue IMP SS

Message-ID: <m0tuoLF-004XM4C@iquest.net>

Daryl,

You mentioned seeing a slate blue SS in the parking lot the other day. I ran

the VIN and the car is a legitimate 95 SS. However it just shows it as a

normal green/gray version. Could it be a good re-paint? Very interesting.

John Spears

91 LT1 SS wagon

------------------------------

End of IMPALA Digest 334

************************

IMPALA Digest 335

Topics covered in this issue include:

1) gray haired SS's available soon

by Franklin Poole <franklin@intersurf.com>

2) Fairfield Inn

by Loren Peterson <equ_rino@cencom.net>

3) Re: Blue Impala = Green Grey

by daryl@austin.ibm.com (Daryl Green)

4) 52mm throttle body question

by Franklin Poole <franklin@intersurf.com>

5) Fairfield Inn Rate Change

by Steve Das <sdas@sat.net>

6) Where would I stay (in Arlington) ?

by Matt Platts <mplat@airmail.net>

7) Important [] If YOU are going to Arlington, RSVP

by Matt Platts <mplat@airmail.net>

8) Hotel Rooms Filling up!

by Steve Das <sdas@sat.net>

9) Warranty Extensions

by Robert Cheek <rcheek@one.net>

10) Re: enlarged mass air housing,

by Robert Cheek <rcheek@one.net>

11) Wish List part II

by jeffd@accessus.net (Jeff Dawson)

12) Smaller pulleys, audio equipment claims, questionable testing methods

by Don Hurter <don@sirius.com>

13) Re: clearance

by Rock528@aol.com

14) Arlington Issues.

by Impalaclub@aol.com

15) Re: IMPALA digest 320

by tom parker <parkertr@gte.net>

----------------------------------------------------------------------

Date: Thu, 7 Mar 1996 15:40:29 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: gray haired SS's available soon

Message-ID: <313F73ED.5F18@intersurf.com>

>Gray hairs my *SS!  I'm 22 years old and have been 'planning' my Impala

>for a couple years now.  Unfortuantely it will have to wait until

>after I graduate (Mechanical Engineering -  Boulder, Colorado), and more

>accurately, get a job.  And if they are really going out of production

>at the end of this year (*sob*), I'll have to find a nice used one.  If

>anyone has any special tips on how to tell if an Impala has been seriously

>abused I'd appreciate it.  Or if anyone knows of a little old lady who

>owns one and might like to sell it in about a year.....well, maybe not.

Unfortunately I have seen several Impala SS's being driven by the gray haired (I

mean 65+) set.  The black one I missed when I got my Dark Cherry was bought by a

75 year old widow who traded in her 82 caprice or whatever...  I figure that car

will be babied and probably not driven 10,000 miles before she dies.  Also the

local dealer has said that he has ordered several Impalas for older people.  I

think there'll be a few cherries out there in a year or two!

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Thu, 7 Mar 1996 16:25:19 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Fairfield Inn

Message-ID: <313F7E6F.5E8@ns.cencom.net>

When I made my reservations for the Wife and I last night at the

Fairfield Inn, I thought the rate was 59.  The clerk said the best he

could do was 69.95, but I could argue at check in/out.  Let me know if

anyone gets a better deal.

Loren DCM96 SPE

Lets get sideways

------------------------------

Date: Thu, 7 Mar 1996 16:38:01 PST

From: daryl@austin.ibm.com (Daryl Green)

To: impala@east.xsis.xerox.com (impala)

Subject: Re: Blue Impala = Green Grey

Message-ID: <9603080038.AA20958@augusta.austin.ibm.com>

>> I was driving along today and saw an SS in a parking lot that was a

>> sort of slate blue in color.

> The color you saw is officially called Dark Green-Grey Metallic! I agree

> that it looks like gun blue or slate blue to me also. Scott.

But there's a green '94 or '95 SS right here where I work that I see

a couple of times a week and it isn't the same color as the one I saw

yesterday.  Are there two colors of SS that are both called green?

Daryl Green

daryl@austin.ibm.com

------------------------------

Date: Thu, 7 Mar 1996 15:43:43 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: 52mm throttle body question

Message-ID: <313F74AF.196E@intersurf.com>

>The beast currently has: Trasgo shift kit, 3.42 gears, 52mm throttle body,

>air foil, enlarged mass air housing, Hennessey induction system, Jacobs

>ignition system, Hypertech programmer, Borla exhaust, Power Pulley at the

>crank only. All these added up to a 14.94 ET at 95 mph.

I'm asking, is a 52mm throttle body a good idea on an engine with stock intake?

Is an opened up exhaust going to provide enough flow to utilize the bigger

throttle body? Scott M?  I say a car at the local show on saturday who had the

ASM intake and throttle body, asm headers, etc. and he could only turn 14.99.

What are you'all's thought? (you'all is southern for you guys<g>)

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Thu, 7 Mar 1996 16:50:26 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: foxhound@txdirect.net, Rock528@aol.com

Subject: Fairfield Inn Rate Change

Message-ID: <199603080048.SAA04461@raven.sat.net>

Well I just called Fairfield to get reservations and they have decided the

rate for the Month of April is not $59.00 but $79.00!!  Those that have the

$59.00 deal are in at tha rate... at least according to the FORTH guy I

spoke with but the rest of us are $79.00 or go someplace else!  I'm going

someplace else!

Steve

------------------------------

Date: Thu, 7 Mar 1996 16:52:12 PST

From: Matt Platts <mplat@airmail.net>

To: "'Impala'" <impala@east.xsis.xerox.com>

Subject: Where would I stay (in Arlington) ?

Message-ID: <01BB0C57.62B77960@fw06-16.ppp.iadfw.net>

Scott M wrote:

=20

I have only one question, where would YOU stay! I'd preferrably like a =

room

with a view (of my Impala in the lot right outside the window, that is)! =

<g>

Scott.

Scott (and others), it seems to be the general concensus that the =

Fairfield Inn is the place to be.  I can't argue with that.  It was =

recently built (a year or so) and is AT the corner of I-30 and HWY-360, =

about 2-3 miles north of the plant.  You should have a good view of your =

cars, the parking is the standard outside-your-window layout for a =

motel.  The hotel is on well-groomed grounds in the 'convention' area, =

plenty of other motels/hotels and resturaunts very nearby.  That part of =

town is actually considered 'north' Arlington and the immediate area of =

the motel has seen a decent amount of growth in the last year.  I can =

see no reason not to stay at the Fairfield, if I had to choose, they =

would definately make my preferred list.

For a little local information about highways and stuff, I have some =

preferences that you might be interrested in.  If I were driving into =

Arlington, I would take I-20.  I-20 runs along the entire south-side of =

the Dallas/Fort Worth area.  The highway itself is always at least 4 =

lanes (each side) and well maintained.  It has wide shoulders and =

provides nice views of the less-hectic side of the cities.  I recommend =

I-20 over I-30 because I-30 runs smack through the middle of Dallas and =

Fort Worth.  The Dallas side is always a mess.  STAY AWAY FROM I-30 =

DURING RUSH HOURS (6-9am and 5-7pm), it is a nightmare... the maps make =

it look like you just go straight through but in reality you get to =

navigate narrow underpasses with SEVERAL on and off ramps that can get =

hairy.  I-20 on the other hand, almost never gets clogged.  Here's the =

bonus...  if you come to Arlington on I-20, you will take HWY-360 north =

to the Fairfield Inn.  This route takes you right past the plant, your =

first glimpse of heaven.  The trip from I-20 to the Fairfield on HWY-360 =

is about 8 miles, a small price to pay for keeping those shiny-dent-free =

Impala's away from downtown Dallas.

Again, if anyone is coming in late or can't find their way once they get =

here, please give me a call.  I will be more than happy to help out with =

anyone in need.  I am very familiar with the highways in the area and am =

intimate with Arlington in particular (I log sometimes 100 miles a day =

traveling to and from jobs in DFW, and from living in Arlington for 5 =

years and being an EMT, I know the roads fairly well).  I can also make =

copies of detailed area maps that could help the confused.

-Matt

mplat@airmail.net

Home: (817)472-6076 (unlisted)

Pager: (817)651-2330 (put *99 at the end, I will know you are from the =

list)

------------------------------

Date: Thu, 7 Mar 1996 16:51:18 PST

From: Matt Platts <mplat@airmail.net>

To: "'Impala'" <impala@east.xsis.xerox.com>

Subject: Important [] If YOU are going to Arlington, RSVP

Message-ID: <01BB0C57.60181660@fw06-16.ppp.iadfw.net>

I talked (actually called) to Troy today and we were trying to figure =

out exactly how many people were planning on making the trip.  This =

information will really help prepare for the tour and the show.  Please =

respond to me PERSONALLY (not to the list) if you plan on going, and =

please specify which of the following best fits your plans.  I will =

compile a list, and relay if needed.

Do you plan to:

1.  Stay in a local motel (which one)

2.  Attend factory tour (April 19th)

3.  Attend Super Chevy Show (April 20-21)

4.  ACTUALLY SHOW YOUR CAR at the Super Chevy Show

Again, please be specific, we are trying to get this as accurate as =

possible.  Remember to reply to me personally at mplat@airmail.net.

Thanks

-matt

------------------------------

Date: Thu, 7 Mar 1996 17:56:46 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: lavera.twine@hok.com, erunnio@infohwy.com, gbeaird@amoco.com,

Subject: Hotel Rooms Filling up!

Message-ID: <199603080155.TAA09083@raven.sat.net>

Called Ennis to make reservations for the National meet and found that

EVERYTHING is full.  Seems they are having a Super Chevy Sunday and all the

F%$# fans are coming to drool.... that's what they said.  Anyway, I found a

place in Waxahatchie (no kidding!) which is 10 miles out of Ennis toward

Arlington.  The place is the Comfort Inn and the rate is $52.00 + tax.  I

was able to get downstairs, non-smoking, car outside the door, or so they

said.... guess I have to have my cigar outside!  Anyway, if anybody is

interested, call Rita, at 214-937-4202 and tell her I told you to call.

She'll love it!

Steve..... that's my story and I'm sticking to it!

------------------------------

Date: Thu, 7 Mar 1996 17:48:45 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Warranty Extensions

Message-ID: <1.5.4b11.32.19960308014845.006b063c@mail.one.net>

OK, OK, I know this has been covered before. So, would anyone care

to send my private e-mail on their experiences with these warranty

companies? Automotive Warranty Service (AWS) called me the other day,

and, in a fit of panic over all of the sh*t that's wrong with my Impala

right now, I decided to let them send me the contract. I can cancel

within 30 days. The warranty is $859 and is good for 6 years / 100K miles

(from when the car was new, not from now). I know that they wouldn't

be in business if they paid more than that for repairs, but, on average,

I'd guess the Impala would have more stuff go wrong with it than, say,

your typical Accord. Whadday'all think?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Thu, 7 Mar 1996 17:48:47 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Re: enlarged mass air housing,

Message-ID: <1.5.4b11.32.19960308014847.0069b194@mail.one.net>

Debbie/Henry Flores wrote:

>I have about $2800...

..burning a hole in your pocket. I'd save about $1K more and:

A) Add a Vortech or an ATI blower. You can probably get by for about

$2800 if you install it yourself. Sell your Hennesey system to someone

else and get back some $$$ for it. Mark, how's the ATI coming, by

the way?

B) ZZ9 cam and ported aluminum heads. This will keep your stock

looking engine bay. You'd probably want to add headers, though,

so the bay wouldn't look _totally_ stock, but close.

These combinations appear to be just about equivalent as far as

HP figures (f-body people are turning high 12's on them at 115MPH

trap speeds). They should get an Impala well into the 13's, I would

think. The cam/head combination would probably be best for overall

engine durability, and, hey, if you ended up with another coupla

thousand bucks, you could still bolt on a blower ;-) Either way,

you'd have one of the baddest Impalas out there...

>enlarged mass air housing

How did you do this? Is this the f-body housing?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Thu, 7 Mar 1996 19:53:18 PST

From: jeffd@accessus.net (Jeff Dawson)

To: impala@east.xsis.xerox.com

Subject: Wish List part II

Message-ID: <199603080353.VAA25708@mtvernon1.accessus.net>

Ok guys here is the revised list with the help of a few Impala brothers. The

offer still stands for more input . Thanks for all the help so far.

=09

Impala Project List=09

Stage 1

1. Walker cat back exhaust with Utra Flow mufflers

2. 3.42 rear axle gear

3. K & N air filter in opened up stock airbox

4. Hypertech PCM (yea I am sticking with this one Scott)

5. 160=B0 thermostat

Stage 2

1. Edelbrock-BBK 52mm throttle body (58mm?)

2. Edelbrock Intake (ported stock?)

3. LPE  Headers=20

4. 1LE intake elbow

Stage 3

1. Edelbrock Heads (LPE?)

2. Crane 1.6 ratio roller rockers=20

3. cam (GM# 24502586, LPE# 74216, or TPIS ZZ9?)

4. 2500 rpm stall converter

Stage 4

1. Herb Adams F & R sway bars

2. Bilstien shocks

3. Hotchkis trailing arms

*notes:

At the end of stage 3 there should be roughly 325 hp at the rear wheels.

Any more power will have to be gained with the increase of cubic inches(383)

or Paxton supercharger($$$). 1/4 mi. times should be low 14's or high 13's.

Jeff Dawson 95 DCM SS mods in progress

------------------------------

Date: Thu, 7 Mar 1996 20:39:32 PST

From: Don Hurter <don@sirius.com>

To: impala@east.xsis.xerox.com

Subject: Smaller pulleys, audio equipment claims, questionable testing methods

Message-ID: <199603080442.UAA03318@moon.sirius.com>

Scott M. said,

> That reminds me of a test years ago in Stereo Review where they were

comparing different types of "monster" speaker wires, and the reviewer was

trying to explain how he could hear the differences in sound between the

different brands of wires! ... I'm sorry that I am so anal retentive,

but I need to see hard data, accurately measured, and backed up by

significant "control" testing via the standard scientific method!

I remember seeing 'Compact Disc Balancing Rings' for sale at a stereo

store some years ago, which were thin plastic rings with adhesive on one side

that you were supposed to stick to the label-side of your CD's. (You of course

needed to buy one ring for every CD in your collection.) The claim was that

they helped smooth out the mechanical vibrations induced by the CD player, and

result in 'a richer, more life-like music listening experience' or some such

B.S.

Rather than resort to methodical testing with control groups and

statistical analysis, I always thought in this case a more rigorous test

procedure was needed to prove or disprove the stated claim. I would sit the

promoter of these CD Rings down in a chair in front of a pair of studio

monitor speakers, and set up two laboratory-identical CD players wired through

an A/B switch to the amplifier, with two copies of any Compact Disc that the

test subject chooses (one sporting the CD Ring.) He would then have a wire

attached to each testicle, running back to a 110V source with a large knife

switch in the circuit. The subject then listens to ten passages from the CD's,

with a reward of $1000 for each correct time he identifies whether the CD has

a balancing ring on it or not, or else the knife switch for each incorrect

guess. This test would yield far more valuable results than any normal

scientific method.

In truth, the claims about the pulleys are _theoretically_ correct, but in

actual use their benefits would get lost in the noise level. You could

probably get better results by folding in the mirrors to reduce wind drag, and

draining the windshield washer reservoir before runs. But there again, this

all seems like a fatuous argument considering the group involved advocates

drag racing in automobiles weighing over 4000 lbs. <g>

-- Don

------------------------------

Date: Thu, 7 Mar 1996 21:53:26 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: clearance

Message-ID: <960308005326_163259600@mail06.mail.aol.com>

In a message dated 96-03-07 17:46:13 EST, you write:

>You may indeed have a problem with curbs, and probably end up scraping the

>front spoiler quite a bit with the lowered springs.

yes on this.

>Also you may bottom out on hard bumps. That might be improved by trimming

the jounce bumpers some.

Eibach recommends trimming 20 mm off the front snubbers.  I have never

bottomed it out yet.  Troy

------------------------------

Date: Thu, 7 Mar 1996 21:54:56 PST

From: Impalaclub@aol.com

To: impala@east.xsis.xerox.com

Subject: Arlington Issues.

Message-ID: <960308005456_240594467@mail04.mail.aol.com>

1.  There's been lots of talk about the motel and rate changes and now I

found out they don't have a message board, the Spice channel or a restaurant.

I will talk to the dist mgr tomorrow (friday) and try to repair the rate

problem.  We should have negotiated (sp?) a contract for a group block first,

then let everyone make reservations.  Having learned this late and actually

how many are coming to Dallas should we change or stay?  If we can't iron out

the 59-79.00 rate difference then we can always move.  We can worry about

that after tomorrow.  I will let you know what happens.

2.  email me with what wording you want on your name tag for the Tx show.

List your name and city and some other little clever buyline if you wish.

I'll spring for the clip on cover.

Troy, overdosing on Tagamet.

------------------------------

Date: Thu, 7 Mar 1996 22:02:20 PST

From: tom parker <parkertr@gte.net>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 320

Message-ID: <96Mar7.220345pst."14489(1)"@alpha.xerox.com>

Why would anyone want to start the mighty SS in second gear anyway?

TP

At 10:52 PM 3/2/96 PST, you wrote:

>			    IMPALA Digest 320

>

>Topics covered in this issue include:

>

>  1) Re: 2nd Gear Start

>	by Steve Das <sdas@sat.net>

>  2) Wish List

>	by jeffd@accessus.net (Jeff Dawson)

>  3) 2nd gear starts

>	by BGordon401@aol.com

>  4) PMS problems

>	by john spears <speartec@iquest.net>

>  5) Something New at Burger King

>	by Loren Peterson <equ_rino@cencom.net>

>  6) 2nd Gear Start

>	by Scott Mueller <73145.1566@compuserve.com>

>  7) Personal stuff

>	by Scott Mueller <73145.1566@compuserve.com>

>  8) Re: 2nd gear starts

>	by Steve Das <sdas@sat.net>

>  9) Testing my posting ability

>	by Franklin Poole <franklin@intersurf.com>

> 10) transmission broke !

>	by ML3001@aol.com

> 11) SS prices

>	by "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

> 12) Re: IMPALA digest 310

>	by RAZZORFL@aol.com

> 13) Re: IMPALA digest 319

>	by RAZZORFL@aol.com

> 14) Re: IMPALA digest 314

>	by RAZZORFL@aol.com

> 15) Re: IMPALA digest 317

>	by RAZZORFL@aol.com

> 16) 2nd gear

>	by "Jeff Gatcha" <foxhound@txdirect.net>

> 17) Wheel centers

>	by "Jeff Gatcha" <foxhound@txdirect.net>

>

>----------------------------------------------------------------------

>

>Date: Sat, 2 Mar 1996 14:59:32 PST

>From: Steve Das <sdas@sat.net>

>To: Ed Runnion <erunnio@infohwy.com>

>Cc: impala@east.xsis.xerox.com (Impala SS List)

>Subject: Re: 2nd Gear Start

>Message-ID: <199603022258.QAA09498@raven.sat.net>

>

>At 04:49 PM 3/2/96 -0600, Ed Runnion wrote:

>>>

>>> More on the ability to start off in second gear.... I went by my local

>>> Pontiac dealer today (I was looking at a new Suburban) and was looking at a

>>> new Firebird.  They have a button behind the shifter that is labeled "Second

>>> Gear Start".  Seems if it was already a feature of the transmission, they

>>> wouldn't have a button so labeled.  I tested my '95 again and it will take a

>>> lot to convince me that it isn't starting in first and shifting to second.

>>> I don't care what the manual says!  I can definitely feel it.

>>>

>>

>>I just tried mine in the parking lot : it is DEFINITELY starting

>>in first and shifting to second when in the "2" position.  Looks

>>like they may have removed this feature in some 95's and the

>>96's (mine is a 96).

>>

>Tell you what ED, I'll bet they are ALL the same and the other guys just

>can't tell it's shifting.

>

>Steve

>

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 15:13:53 PST

>From: jeffd@accessus.net (Jeff Dawson)

>To: impala@east.xsis.xerox.com

>Subject: Wish List

>Message-ID: <199603022313.RAA07348@mtvernon1.accessus.net>

>

>Sorry for the lenth of this message but I wanted to share my wish list with

>the folks who would most understand. This list is not carved in stone so

>comments and observations will be most appreciable. Stage 1 of this list in

>in progress as this is being written. Still waiting on the Walker exhaust,

>but all other parts of 1st stage are here or coming. I am a "blue collar

>worker" so this will take awhile to finish. Stage 3 will be done about the

>time the SS is paid off and warranty is gone. Hope no one shorts out their

>monitor while reading this so keep a napkin or hanky at the ready. Remember

>drool and electricity don't mix. I found a local garage to do the rear

>change. R & R Motorsports in Benton IL. The owner is knowlegable and did not

>ask why I wanted to change. With a little luck and no suprises (3 kids) I

>will have stage 1 done in time for Street Machine Nats here in DQ. =09

>

>P.S.=20

>Alot of these parts are going to come from LPE. John was nice enough to send

>me a catalog for free.  Three days after asking for one on his web page it

>was in my mailbox! This kind of service must be rewarded!=20

>

>			Impala Project List=09

>

>

>Stage 1

>

>1. Walker cat back exhaust with Utra Flow mufflers

>2. 3.42 rear axle gear

>3. K & N air filter in opened up stock airbox

>4. Hypertech PCM

>5. 160=B0 thermostat

>

>

>Stage 2

>

>1. Edelbrock-BBK 52mm throttle body {58mm?}

>2. Edelbrock Intake

>3. A. S. & M. Headers {LPE, Edelbrock?}

>4. 1LE intake elbow

>

>

>Stage 3

>

>1. Edelbrock Heads {possibly Trick Flow?}

>2. Crane 1.6 ratio roller rockers=20

>3. cam {undecided, LPE, Edelbrock or TPIS ZZ9?}

>4. 2500 rpm stall converter

>

>

>Stage 4

>

>1. Herb Adams F & R sway bars

>2. Bilstien shocks

>3. Hotchkis trailing arms

>4. Eibach Springs

>

>*notes:

> At the end of stage 3 there should be roughly 325 hp at the rear wheels.

>Any more power will have to be gained with the increase of cubic inches(383)

>or Paxton supercharger($$$). 1/4 mi. times should be low 14's or high 13's.

>

>Jeff Dawson 95 DCM still stock

>

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 17:42:01 PST

>From: BGordon401@aol.com

>To: sdas@sat.net

>Cc: impala@east.xsis.xerox.com

>Subject: 2nd gear starts

>Message-ID: <960302204201_339150601@mail02.mail.aol.com>

>

>Steve, you wrote:

>

>>>but I'm betting the thing is really starting

>in 1st and shifting to 2nd.  According to the shop manual, the convertor

>clutch is not suposed to lock in second so I don't think that is it but I'll

>guarantee it is shifting.  I tried it several times and it is

>unmistakeable... it's shifting.

>

>Anybody else got an opinion?<<

>

>I'll second your findings.  I have a 1995 SS with the stock programming and

>no mods to the tranny or linkage.

>

>If I put her in the "2" position, she DEFINATELY starts in 1st, and then

>shifts into 2nd and stays there until rev limit.  I was hoping to find that

>she started in 2nd and stayed there, but I was disappointed to find I was

>wrong.

>

>As far as why, I'm in the fog.  It appears that the shift lock works for

>some, but not for others in the 1995 model year.  Hopefully, somebody will

>come up with the answer, b/c running with 3.42 gears, my tires would stick

>much better if the 2nd gear start worked.

>

>..Bruce

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 17:40:00 PST

>From: john spears <speartec@iquest.net>

>To: impala@east.xsis.xerox.com

>Subject: PMS problems

>Message-ID: <m0tt2mx-004XLyC@iquest.net>

>

>Mike and Franklin,

>

>We may have to take this PMS discussion to a medical chat line!<BG>

>Seriously though, my guess on Franklins car is that you are having some type

>of transmission failure, but that is only a guess as I am not a transmission

>specialist. Did you try to hook up the Perf Mode Shift feature that I

>described in an earlier digest? As far as I know Mike Marx is the only one

>that actually hooked it up on the 95 SS calibration; and it was with less

>than favorable results. It did not act like it does on my 94 Trans Am

>calibration. It sounded like maybe just a partial calibration or something

>on the 95.

>

>Scott M...you mentioned in digest 319 about serious trouble using the 1.6

>roller rockers with stock valvesprings. Are your measurements based on the

>iron head valvespring or the aluminum head valvespring? Or do they use the

>same valvespring? I am using the Crane 1.6 roller rockers on my 91 "SS"

>wagon with the aluminum heads and stock springs. This setup with the 94 T/A

>PCM program has a WOT upshift of 5800 RPM. I have noticed no problems, but

>would like to know if I am flirting with disaster!         John.

>

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 17:44:54 PST

>From: Loren Peterson <equ_rino@cencom.net>

>To: impala@east.xsis.xerox.com

>Subject: Something New at Burger King

>Message-ID: <3138F996.3D59@ns.cencom.net>

>

>Hi All,

>Well not in Burger King exactly, but while waiting in line at the drive

>thru, my Wife made an interesting discovery.  On our '96 anyway, if you

>push the ash tray forward (inside drop down door) low and behold there

>is a cup/can holder.  Somehow we had missed this feature.  Glad She

>found it cause she wished we had 94/95 console for the cup holders.  Now

>were both happy.

>Loren DCM96 SPE

>Lets get sideways

>TEXAS or bust

>p.s. Chewed up a SHO on I74.....no contesto

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 18:18:23 PST

>From: Scott Mueller <73145.1566@compuserve.com>

>To: <impala@east.xsis.xerox.com>

>Subject: 2nd Gear Start

>Message-ID: <960303021823_73145.1566_FHD60-1@CompuServe.COM>

>

>Mike,

>

>>is there any thing in the tranny breakdown showing any additional solenoids

>that may be associated with this feature, causing problems, or maybe missing

>from the B-body version ?.

>

>Not really. There was a slight change in the '95+ 4L60-E transmissions with

>regards to the torque converter and the Torque Converter Clutch (TCC)

>circuit. In these models there is an extra PWM (Pulse Width Modulated)

>solenoid that controls the application of the TCC, softening the engagement.

>The '94 and earlier transmission do not have this solenoid. The TCC in the

>converter was also slightly re-designed for '95. None of these things should

>have any effect on the 2nd Gear Start capability, but it seems like many '95

>models lack this feature.

>

>It sounds to me like there may have been either a valve body design or PCM

>calibration change partway through the '95 model year, as some '95 have the

>2nd Gear Start feature while others seem to lack it. All '94s have this

>feature, but what about the '96 models?

>

>I wonder how many different PCM calibrations there are for '94, '95, and

>'96, and whether any of these are documented to remove the 2nd Gear Start

>function?

>

>Mike S. and I were testing his Impala the other night with a '94 F-car PCM

>calibration (including the bigger F-car MAF), and it ran great! The shifting

>was dramatically improved, as was the operation of the TCC. We were

>expecting problems because the '94 calibration did not have the code to

>operate the new PWM TCC solenoid in Mike's '95 transmission, but everything

>seemed to work fine. Unfortunately we did not think to check if the 2nd Gear

>Start function now worked with the F-car calibration, it definitly does not

>function in Mike's car with the stock calibration, which is an early '95

>build. We will probably swap in the F-car PCM again to check it out. Scott.

>

>

>

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 18:18:35 PST

>From: Scott Mueller <73145.1566@compuserve.com>

>To: <impala@east.xsis.xerox.com>

>Subject: Personal stuff

>Message-ID: <960303021834_73145.1566_FHD60-4@CompuServe.COM>

>

>Jeremy K. wrote me and very graciously apologized for the comment he made

>earlier on the list. I just wanted everybody to know as I would really like

>him to feel welcome here again. There are no hard feelings, and let's

>consider this matter closed! Scott.

>

>

>

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 18:55:10 PST

>From: Steve Das <sdas@sat.net>

>To: BGordon401@aol.com

>Cc: impala@east.xsis.xerox.com

>Subject: Re: 2nd gear starts

>Message-ID: <199603030253.UAA22076@raven.sat.net>

>

>

>>As far as why, I'm in the fog.  It appears that the shift lock works for

>>some, but not for others in the 1995 model year.  Hopefully, somebody will

>>come up with the answer, b/c running with 3.42 gears, my tires would stick

>>much better if the 2nd gear start worked.

>>

>Bruce,

>I'm betting that they all work the way yours and mine do.  Several of the

>other guys have written to me confirming the same thing you found.  It will

>be interesting to see if anybody can verify that their car truly starts in

>second.  I'm betting not.  Thanks for the response.

>

>Steve

>

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 19:39:10 PST

>From: Franklin Poole <franklin@intersurf.com>

>To: Impala List Server <impala@east.xsis.xerox.com>

>Subject: Testing my posting ability

>Message-ID: <3139145E.3611@intersurf.com>

>

>sorry to do this but my posts have all been disappearing for 8 days now!

>

>--

>Franklin Poole, Jr.

>Praecedo, Inc.

>================================================

>It's not how fast you go, it's how you go fast!!

>$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$$

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 19:54:31 PST

>From: ML3001@aol.com

>To: impala@east.xsis.xerox.com

>Subject: transmission broke !

>Message-ID: <960302225429_436630756@emout05.mail.aol.com>

>

>o.k. ineed some opinions please,i blew my transmission last week but i dont

>know exactly why. i just had installed the transgo shift kit and i was racing

>around town for the first decent chicago day in months. after a couple of

>hours of driving, my car will shift from  1-2 fine but when it should go into

>third the engine just revs freely as if the car were in nuetral,but if i

>stomp the gas at that point the car will downshift and really go, but no

>matter what it, will not go into third. i have had the trans apart 3 times

>now to check my installation or for any obvious problems, everything seems

>fine though.i have heard that the third gear clutch pack in the 4L60E is

>small in diameter and could be a weak point? before you doubt the integrity

>of a solid and proven transmission, let me tell you my car is an early 94

>with 30000 miles on it also a carrol/vortec supercharger kit, 3 extra fuel

>injectors, hi vol fuel pump, cold air induction w/ t.p.i. airfoil, as&m

>headers,& custom 2-1/2 cat-back exhaust w/dynomax ultra flow mufflers. it has

>other goodies too, but you can see i am abusing this transmission.

>PERFORMANCE TECHNICAL CENTER in elk grove village built this car with me so

>he is replacing it with a new 4L60E at no cost. warranties are not my

>problem. i just wonder if i wrecked it with the shift kit install, the

>horsepower, or it was just one of those things. i know 6 other people with

>this same engine and transmission combo supercharged, and higher mileage with

>no problems. the good part of this is, i can sneak in a new vigilante torque

>converter ! that ought to max out the visa card (and home equity loan and

>every other source of credit i could find to feed my car addiction)

>        P.S. has anyone tried the new baer racing, alcon brakes for our cars

>yet, I NEEEEED THOSE TOO !!!!

>

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 15:26:00 PST

>From: "Fischer, Donald W           EA" <FISCHERW@ihub.ea.unisys.com>

>To: impala <impala@east.xsis.xerox.com>

>Subject: SS prices

>Message-ID: <31392E07@po5.cp.unisys.com>

>

>

>Jeff,

>You write

>

>>They told me I would pay invoice + $X over invoice as dealer

>profit .

>

>So what was the bottom line price less license and tax?

>     '96 on the train some where so they tell me.

>          Don Fischer

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 21:31:07 PST

>From: RAZZORFL@aol.com

>To: impala@east.xsis.xerox.com

>Subject: Re: IMPALA digest 310

>Message-ID: <960303003107_339314123@emout10.mail.aol.com>

>

>In a message dated 96-02-27 02:24:38 EST, you write:

>

>>Comming soon:  Z-pcm, 3.42 gear, and a Tach

>>

>>

>

>Call me if you would like some suggestions on your choice of tach's and their

>mounting.

>

>RAZZORFL.

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 21:30:16 PST

>From: RAZZORFL@aol.com

>To: impala@east.xsis.xerox.com

>Subject: Re: IMPALA digest 319

>Message-ID: <960303003016_339314043@mail02.mail.aol.com>

>

>In a message dated 96-03-02 17:50:21 EST, you write:

>

>> I do

>>miss having a tach, but won't go for an add-on.

>>

>>

>>

>

>If your interested, I can tell you how I added a Auto Meter Tach to my Blk

>'95 so it looks like it sits on the dash just above the left part of the

>insrtument area with NO additional holes! In fact it has become somewhat of a

>trademark of mine.

>

>RAZZORFL.

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 21:31:27 PST

>From: RAZZORFL@aol.com

>To: impala@east.xsis.xerox.com

>Subject: Re: IMPALA digest 314

>Message-ID: <960303003125_339314108@emout09.mail.aol.com>

>

>In a message dated 96-02-29 00:26:56 EST, you write:

>

>>>What effect, if any, does a plug have on the performance of a tire/its

>>ability to withstand supersonic speed?

>>

>>

>

>Please trust me on this one!

>If you talk to anyone that is knowledgable in the tire industry they will

>tell you that pluging a tire, especially a redial tire is not done anymore!

>The proper way to repair a flat today is to dismount the tire and patch it

>from inside. A tire plug can damage the radial cords and/or allow water and

>other elements to damage the tire.

>

>RAZZORFL.

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 21:30:18 PST

>From: RAZZORFL@aol.com

>To: impala@east.xsis.xerox.com

>Subject: Re: IMPALA digest 317

>Message-ID: <960303003018_339314065@mail04.mail.aol.com>

>

>In a message dated 96-03-01 18:16:51 EST, you write:

>

>>Has anyone dyno'd the car before and after?  I know the benefits of

>>cold air for increase in power, but does it show with these changes?

>>

>>

>

>If your interested in 5 (five) additional REAR WHEEL HORSEPOWER gains PLUS,

>then please call me!?!

>

>RAZZORFL.

>

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 22:22:58 PST

>From: "Jeff Gatcha" <foxhound@txdirect.net>

>To: impala@east.xsis.xerox.com

>Subject: 2nd gear

>Message-ID: <199603030622.AAA07892@legend.txdirect.net>

>

>My car behaves just like  Steve Das's on this one.

>

> When my car is in #1,  starts in 1st gear and stays there

> When  it is in #2, it starts in 1st gear, then shifts to 2nd, and

> stays there.

>So mine has no 2nd gear start.   June 95 prod date if that makes any

>difference.

>

>God, grant me the courage to change the things I can,

>the serenity to accept the things I can't,

>And the wisdom to hide the bodies of the people who really

>pissed me off.

>

>Jeff Gatcha

>1995 GGM

>Foxhound@txdirect.net

>San Antonio Texas

>

>------------------------------

>

>Date: Sat, 2 Mar 1996 22:47:36 PST

>From: "Jeff Gatcha" <foxhound@txdirect.net>

>To: impala@east.xsis.xerox.com

>Subject: Wheel centers

>Message-ID: <199603030647.AAA08383@legend.txdirect.net>

>

>Does anyone know or can anyone find out the Part Number for the

>center caps on the Stock Impala wheels?    I had a rock or something

>hit my left rear wheel and it nicked and scratched the center cap a

>good bit.  I would like to get a new one and would like to have the p/n

> when I order it.

>  If anyone is confused, it is the plastic cap with the Impala symbol

> on it that covers the lug nuts on the stock wheels.

>

>Thanks

>God, grant me the courage to change the things I can,

>the serenity to accept the things I can't,

>And the wisdom to hide the bodies of the people who really

>pissed me off.

>

>Jeff Gatcha

>1995 GGM

>Foxhound@txdirect.net

>San Antonio Texas

>

>------------------------------

>

>End of IMPALA Digest 320

>************************

>

>

------------------------------

End of IMPALA Digest 335

************************

IMPALA Digest 336

Topics covered in this issue include:

1) My Impala will be returning home!!!

by Franklin Poole <franklin@intersurf.com>

2) Fairfield Inn

by Franklin Poole <franklin@intersurf.com>

3) '96 Impala invading Big D

by Franklin Poole <franklin@intersurf.com>

4) Impala Service Manuals

by Franklin Poole <franklin@intersurf.com>

5) RE: 52mm throttle body question

by "Scott Zeiter" <ScottZeiter@msn.com>

6) Fairfield Inn

by Franklin Poole <franklin@intersurf.com>

7) Decals

by "Jeff Gatcha" <foxhound@txdirect.net>

8) obsession

by Scott Mueller <73145.1566@compuserve.com>

9) that "reply" thing

by "Jeff Gatcha" <foxhound@txdirect.net>

10) rear control arms

by Scott Mueller <73145.1566@compuserve.com>

11) Control Arm Upgrade

by "Thomas L. Pierce" <75027.2421@compuserve.com>

12) Cheap tire prices

by mbailey@rinet.hunton.com (Bailey, Martin)

13) 52 mm Throttle Body

by Fuel Injection Specialties <fuelinjection@fuelinjection.com>

14) Re: IMPALA digest 335

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

15) AOL chat night

by "Brett A. Rudolph" <bngwip@mail.phoenix.net>

16) Crow problems!

by Franklin Poole <franklin@intersurf.com>

17) LEO's and their respect.

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

----------------------------------------------------------------------

Date: Thu, 7 Mar 1996 22:13:57 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Cc: Troy Wilrick <Rock528@aol.com>

Subject: My Impala will be returning home!!!

Message-ID: <313FD025.5645@intersurf.com>

>List of attendees driving Impalas:

>Troy Willrick

>Loren Peterson

>Curt Platteborze

>Mark Weber

>Larry Lee

>Jeff Green

>Scott Mueller (no really?).......

Maybe this is one of my posts that got lost... My Impala and I will be

returning home to Arlington!  I'm cooking you guys crawfish (in Baton

Rouge), then I'll expect someone else to drive me to Texas in my

car!<GGG> not really....

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Thu, 7 Mar 1996 22:32:52 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Cc: Troy Wilrick <Rock528@aol.com>

Subject: Fairfield Inn

Message-ID: <313FD494.4872@intersurf.com>

i just got off the phone with the night desk of the Fairfiedl Inn in

Arlington.  The $59 rate is pretty low but we might be able to beat them

up to th $49 weekend rate.  The day manager is calling me tommorow

afternoon to discuss details.  I have also tasks myself with the

bulletin board idea for group members.  I told the night desk to tell

the day manager to expect at least 30 members.  I hope more than 50 will

come.  If security for our cars is a concern we can hire local deputies

for a few dollars each if necessary....

Troy email me offline w/any questions.

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Thu, 7 Mar 1996 22:41:07 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: '96 Impala invading Big D

Message-ID: <313FD683.1D38@intersurf.com>

My '96 Dark Cherry and I will be invading Dallas on Sunday 10 March for

a Microsoft certification training class.  I'll be there Sunday and

Monday nights.  Will be downtown at Dallas Grand Hotel and will probably

hang out at the Ginger Man in midtown in the evening.  Any local

dallasites who'd like to get together and swap lies email me!

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Thu, 7 Mar 1996 22:43:07 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Impala Service Manuals

Message-ID: <313FD6FB.61F2@intersurf.com>

I can only hope that the reason the service manuals are out of stock

is because they are finally becoming the "final" edition. No more

preliminary.  My dealer is getting several copies for me and list

members at a $15 discount, but Helm won't ship him any.  Maybe they'll

ship to us dumb "end users" the old stuff but figure the dealers need

the "real thing".....

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Thu, 7 Mar 1996 23:03:41 PST

From: "Scott Zeiter" <ScottZeiter@msn.com>

To: impala@east.xsis.xerox.com

Subject: RE: 52mm throttle body question

Message-ID: <UPMAIL08.199603080658510033@msn.com>

>>The beast currently has: Trasgo shift kit, 3.42 gears, 52mm throttle body,

>>air foil, enlarged mass air housing, Hennessey induction system, Jacobs

>>ignition system, Hypertech programmer, Borla exhaust, Power Pulley at the

>>crank only. All these added up to a 14.94 ET at 95 mph.

>

>I'm asking, is a 52mm throttle body a good idea on an engine with stock

intake?

>Is an opened up exhaust going to provide enough flow to utilize the bigger

>throttle body? Scott M?  I say a car at the local show on saturday who had

the

>ASM intake and throttle body, asm headers, etc. and he could only turn 14.99.

>What are you'all's thought? (you'all is southern for you guys<g>)

Putting it in the simplest form, a car engine is nothing more than a big air

pump.

The more air you pump through it, the more horsepower you get.

Increasing the exhaust and intake is the first step in modifying your

Impala.  I agree that a 52mm throttle body is a little over doing it

with a stock intake, but you have to start somewhere.

96 - Black, just delivered, 13 miles, no mods yet.

------------------------------

Date: Thu, 7 Mar 1996 23:16:31 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Cc: equ_rino@cencon.net

Subject: Fairfield Inn

Message-ID: <313FDECF.5E53@intersurf.com>

>Fairfield Inn, I thought the rate was 59.  The clerk said the best he

>could do was 69.95, but I could argue at check in/out.  Let me know if

>anyone gets a better deal.

>Loren DCM96 SPE

--

I didn't have to even argue for a $59 rate, and I'm going to discuss a

lesser rate for the group!  When I get the facts I'll ask each of you to

call back to the hotel and confirm that you are members of NAISSO so

you'll get the cheaper rate.  I'm an old road warrior like Scott M and I

know how to beat those guys up for the best rate possible!!!

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Thu, 7 Mar 1996 23:43:17 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Decals

Message-ID: <199603080742.BAA29801@legend.txdirect.net>

>I am having my decal guy make a decal - exact fit to go over the

>chrome emblem on the grill.  If anyone wants one, let me know.  He can make

>them in any color.  I will have RED of course.

I want a big decal of  the leaping Impala (like the trunk emblem)  to put

on my rear window.

That way after seeing the huge RED Bow-Tie in their  rearview, the Mustangs

and SHO's will know just what kind of Chevy just blew them off the

road, and they will NEVER FORGET IT!!!!   heheheheheheh

Yeah, I do LOVE IT!!!!!

God, grant me the courage to change the things I can,

the serenity to accept the things I can't,

And the wisdom to hide the bodies of the people who really

pissed me off.

Jeff Gatcha

1995 "Modified" DGGM

Foxhound@txdirect.net

San Antonio Texas

------------------------------

Date: Fri, 8 Mar 1996 00:00:41 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: obsession

Message-ID: <960308080040_73145.1566_FHD40-2@CompuServe.COM>

Curt,

>Thanks for your comments regarding my lame Impala.

That is a word I thought I'd never see used to describe an Impala! I would

say "cylindrically impaired" instead. <g>

>By the way is anyone else driving 1800 miles to Texas? My wife keeps

telling me I'm becoming obsessed with the Impala. "Could she be right?"<G>

No, I tell you she is wrong! There is nothing wrong with driving 1800 miles

to meet with a few other people that have the same car as you do. No, I am

convinced that we are normal, it is everybody else who is weird. <G>

BTW, does this whole thing reminds anybody of those Saturn commercials, you

know where the bikers cruise up to the diner, and find the parking lot

overrun by Saturns, and the guy comes running out "Hurry, the factory guy is

about to talk!!" <g> Of course we're nothing like THOSE people, right? Nah,

we're not the same at all, we have a MUCH cooler car. <BG>

>What's the verdict on the best blower for the LT1? I think the next add for

me should be the blower...

OK, so now you want a blower! OK, that does take it up a notch, but I'd

still say you are normal. Now when you stop shaving and refuse to go back to

work until the blower is installed, only then could you be classified as

"obsessed" <G> My secret to prevent from going off the deep end is simple, I

keep saying to myself over and over "Must go to work so I can pay for Impala

parts. Work is good. First Work, then get paycheck, then buy more parts".

<g>

"I polished the rear aluminum calipers with a die grinder/scotchbrite pads and

metal polish.  They look like chrome.  Troy"

See, Troy also exhibits perfectly normal behavior, I mean doesn't

everybody polish their brake calipers? <g> The other night I had my 6 yr.

old son laying under the car helping me polish the stainless steel

exhaust. He's also become quite skillful at armoralling tires. I started

early on teaching him how to grow up to be normal just like his dad! <g>

Scott.

------------------------------

Date: Fri, 8 Mar 1996 00:06:36 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: that "reply" thing

Message-ID: <199603080806.CAA00353@legend.txdirect.net>

Guys, please check your E-mail when you reply to the list.  This

copying of entire digests into the new digest is getting old.

I get so excited when I get a new digest, only to be crushed when I

find out its filled with an entire Previous digest.   Makes me feel

cheated.

God, grant me the courage to change the things I can,

the serenity to accept the things I can't,

And the wisdom to hide the bodies of the people who really

pissed me off.

Jeff Gatcha

1995 "Modified" DGGM

Foxhound@txdirect.net

San Antonio Texas

------------------------------

Date: Fri, 8 Mar 1996 02:46:15 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: rear control arms

Message-ID: <960308104615_73145.1566_FHD33-3@CompuServe.COM>

Martin,

>I picked up my "fix kit" today.  Until I did, I was not aware that it

replaced the control arms altogether instead of just the inserted bracket

that the bars mount to.  This is one heck of a deal for $44!!

Yes, you normally can't even buy one control arm for that, not to mention

both PLUS all of the hardwar and swaybar brackets! Also these arms are a

killer deal for any older Caprice owners, especially those that are not 9C1

cars. That is because these arms are equipped with brand new 9C1/Impala

bushings, which are a higher durometer (harder) rubber than the normal

civillian Caprice bushings.

>I plan to install it this weekend. Because I don't yet have the manual,

could you please tell me what the torque values are for the bolts that mount

the control arms?

Sure, the bolts get tightened to 140 ft-lbs, or the nuts to 107 ft-lbs. You

tighten whichever one is easier to get to and hold the other with a wrench.

Make sure you do only one control arm at a time, to maintain the axle in

position. Let me know how the install goes, it should be pretty easy. Scott.

------------------------------

Date: Fri, 8 Mar 1996 04:00:05 PST

From: "Thomas L. Pierce" <75027.2421@compuserve.com>

To: impala <impala@east.xsis.xerox.com>

Subject: Control Arm Upgrade

Message-ID: <960308120004_75027.2421_GHJ81-1@CompuServe.COM>

Scott M. (or anybody who knows)

Do the upgraded lower control arms reuse the bracket(?) that the sway bar

bolts to from the old control arm???

I don't have the old lower control arms off yet for comparison, but from

looking at the new parts, it doesn't make much sense any other way...

Tom

------------------------------

Date: Fri, 8 Mar 1996 05:19:25 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (Impala1)

Subject: Cheap tire prices

Message-ID: <1996Mar08.064100.1049.113551@rinet.hunton.com>

Here is a tire source that is quite a bit cheaper than the Tire Rack:

The shipping cost per tire is $8.50.  To order, please call us at

1-800-TIREMAN M-F 8:30 am- 6:00 pm (EST) or Saturday from 8:30 am - 3

pm The cost for each 255/50-17 is $130.00.

------------------------------

Date: Fri, 8 Mar 1996 07:23:02 PST

From: Fuel Injection Specialties <fuelinjection@fuelinjection.com>

To: impala@east.xsis.xerox.com

Subject: 52 mm Throttle Body

Message-ID: <19960308152301562.AAA61@FIS>

Franklin,

Just wanted to make you aware that a stock throttle body cannot be bored to

52mm, max 50 mm.  If someone is selling a 52 that say its a stock rebored,

it cannot be possible.

BTW, I had a customer that we installed a SC on 94 Camaro that bought a

"52mm " throttle body and I removed it an measured 50mm.

Any questions let us know!

Donald

Fuel Injection Specialties

http: www.fuelinjection.com

1-210-654-0774

------------------------------

Date: Fri, 8 Mar 1996 07:57:08 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 335

Message-ID: <9603081601.AA17303@karloff.fstrf.org>

>Date: Thu, 7 Mar 1996 22:02:20 PST

>From: tom parker <parkertr@gte.net>

>To: impala@east.xsis.xerox.com

>Subject: Re: IMPALA digest 320

>Message-ID: <96Mar7.220345pst."14489(1)"@alpha.xerox.com>

>

>Why would anyone want to start the mighty SS in second gear anyway?

>

>TP

>

>At 10:52 PM 3/2/96 PST, you wrote:

>>			    IMPALA Digest 320

>>

>>Topics covered in this issue include:

>>

..edit...

Not again!  Can't someone develop some sort of computer-based torture

chamber, complete with rack and iron maiden, for these people?

(I think a good beta is MacOS <G>)

- mike

p.s. Welcome aboard, Tom, and welcome to the world of listservers!

------------------------------

Date: Fri, 8 Mar 1996 08:59:55 PST

From: "Brett A. Rudolph" <bngwip@mail.phoenix.net>

To: impala@east.xsis.xerox.com

Cc: bngwip@aol.com

Subject: AOL chat night

Message-ID: <1.5.4b11.16.19960308110414.2707d09c@phoenix.net>

Hi,

Just wanted to thank everyone who showed up on Thursday night in the

Impala SS chat room on AOL. I was hoping for a larger turn out, but it was

fun anyway. Troy suggested that we try this again next week. If anyone is

interested, It's Thursday night at 10pm Eastern, 9pm Central, etc...

I know that some of you had trouble finding us, If that was the case,

e-mail me and I will tell you how to make it easier the next time. Send

e-mail to bngwip@aol.com. If you are using Aol, It's just bngwip. (of

course) Hope to see you there next week.

Troy, if you speak with John Moss again before then, Ask him if he has aol

and would he like to join us.

If anyone would like to try this on the IRC, Let me know. I have both.

Brett

Brett A. Rudolph

96 SS Black

(still waiting)

If you build it,

I will come.

------------------------------

Date: Fri, 8 Mar 1996 10:17:18 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Crow problems!

Message-ID: <314079AE.332C@intersurf.com>

I'll admit I was credulous when another list member had problems with

crows...three days ago I watched two crows dive bomb the hood of my Dark

Cherry Impala and then dance around on the hood pecking the heck out of

it.  I threw down the phone and ran outside to throw pine cones at them

(all the ammo I had).  They flew about 20 feet and the just sat on the

ground an screamed at me.  No damage to shamu but gee whiz man,

whatzadeal with the crows!

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Fri, 8 Mar 1996 11:02:38 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Cc: calanan@karloff.fstrf.org

Subject: LEO's and their respect.

Message-ID: <9603081906.AA23332@karloff.fstrf.org>

FWIW,

I saw the most amazing sight this morning that I felt I had to relay

it here.

A local officer recently passed away (from Western New York), and his

funeral was this morning.  I was simply awed, by not only the

sheer number of vehicles in his procession, but by the amount of respect

and honor these people have for their fellow officers.  There had

to be a couple hundred cars, mostly 9C1's and some crown vics all with their

lights flashing, from every city and town in Western NY, along with most

of the state troopers in our local troops.  The procession was at least

a mile long, and when I crested a hill driving towards it and saw the

full length of it I was awed.

I'm sure some of you have probably seen something similar, and maybe

even more impressive, but the first time you actually witness this

it really is incredible and moving.  If any of you are LEO's from the WNY

area, my condolences to your community's loss.

And no, I saw no Impalas in the procession.

- mike

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

My '96 Impala SS: "It goes fast, FAST!"

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

End of IMPALA Digest 336

************************

IMPALA Digest 337

Topics covered in this issue include:

1) Tag

by Curt Platteborze <75511.1220@compuserve.com>

2) Crows.... or, See, Steve ISN'T Crazy!!

by Steve Das <sdas@sat.net>

3) Window Tint

by Mikado95@aol.com

4) Re: IMPALA digest 336

by Mikado95@aol.com

5) FYI

by Godley@aol.com

6) Recalls

by "Jeff Gatcha" <foxhound@txdirect.net>

7) Hotel Update

by Mbmarx@aol.com

8) Fairfield Inn in Arlington

by Franklin Poole <franklin@intersurf.com>

9) parts orders

by Franklin Poole <franklin@intersurf.com>

10) emmisions tests

by "Harris, Scott" <sharris@xyplex.com>

11) Re:96 C 14 RECALL

by lennie@earthlink.net (Lennie Dolphin)

12) Spark Plug Wires

by lennie@earthlink.net (Lennie Dolphin)

13) Re: IMPALA digest 336

by Ed Salazar <esalazar@hooked.net>

14) Security

by Scott Mueller <73145.1566@compuserve.com>

15) upgraded control arms

by Scott Mueller <73145.1566@compuserve.com>

16) Re: NAISSO Newletter??

by Kurt Kemmling <76165.1654@compuserve.com>

17) Effect of Simple Mods

by dstabb@epix.net (David Stabb)

18) Re: AOL forum

by Curt Platteborze <75511.1220@compuserve.com>

19) Re: Jegs Catalog

by Curt Platteborze <75511.1220@compuserve.com>

----------------------------------------------------------------------

Date: Fri, 8 Mar 1996 11:25:57 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: Troy <impala@east.xsis.xerox.com>

Subject: Tag

Message-ID: <960308192557_75511.1220_GHJ144-1@CompuServe.COM>

Troy,

Info for my name tag.

Curt Platteborze

Waltham, MASS

Can't think of a good line yet.

Curt P.

------------------------------

Date: Fri, 8 Mar 1996 11:48:37 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: Franklin Poole <franklin@intersurf.com>

Subject: Crows.... or, See, Steve ISN'T Crazy!!

Message-ID: <199603081948.NAA29830@raven.sat.net>

>I'll admit I was credulous when another list member had problems with

>crows...three days ago I watched two crows dive bomb the hood of my Dark

>Cherry Impala and then dance around on the hood pecking the heck out of

>it.  I threw down the phone and ran outside to throw pine cones at them

>(all the ammo I had).  They flew about 20 feet and the just sat on the

>ground an screamed at me.  No damage to shamu but gee whiz man,

>whatzadeal with the crows!

Franklin,

I LOVE IT!!!  I know there were guys out there that thought I was smoking

something but now I have a compadre!!!  I think they see their reflection

and decide it needs attacking.  If you need info on how to get rid of them,

I know!!  Have to be in the county though.  But then again, I DO live in

Texas and we tend to deal with things a little different than the rest of

the U.S.  In fact, I know some Texans that maintain that Texas is NOT part

of the U.S.  But, that's another story.... maybe over a brew!! Go get them

crows!!!

Steve

------------------------------

Date: Fri, 8 Mar 1996 11:59:39 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: Window Tint

Message-ID: <960308145936_345224431@emout06.mail.aol.com>

Does anyone know the laws in Illinois on window tinting?   I got pulled over

last night less than two blocks from my house.   The officer claimed that my

window tint is illegal, because I have the driver/passenger windows tinted.

The thing is, I had this done in December, and have driven past many

officers who never stopped me before.   And also, I had the "illegal" windows

done in the lightest possible tint--I only used tint for heat purposes--not

for looks.   I saw a new Buick two days ago with BLACK windows all

around--including a tinted windshield!    Mine is so light that you can see

who's driving, even at night.     I'm going to have a talk today with my

installer......Several people suggested that the officer didn't really have a

reason to pull me over--he just made that up after he stopped me, but I don't

know.....

HELP ME!

Randy

------------------------------

Date: Fri, 8 Mar 1996 11:59:42 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 336

Message-ID: <960308145935_345224413@emout04.mail.aol.com>

>I am having my decal guy make a decal - exact fit to go over the

>chrome emblem on the grill.  If anyone wants one, let me know.  He can make

>them in any color.  I will have RED of course.

How about a silver/white reflective bowtie for all of us who own Green-Grays

and Dark-Cherrys?   I'd buy one.

Randy

------------------------------

Date: Fri, 8 Mar 1996 12:25:47 PST

From: Godley@aol.com

To: impala@east.xsis.xerox.com

Subject: FYI

Message-ID: <960308152545_441448021@emout07.mail.aol.com>

Myron Cottrell is about to introduce his newest package for the 94-96 Impala.

A 409CID LT-1 engine swapout.  Cottrell main goal for the increased cubes is

TORQUE.  Also, Rockingham (NC) opens up this weekend with a test and tune.

Gates open at 5 and close at 10.  I won't be able to make it, but I will be

there in the future.  Anybody interested in going down there with me this

summer just e-mail me.

MARK

95DCM Impala

------------------------------

Date: Fri, 8 Mar 1996 12:18:42 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Recalls

Message-ID: <199603082018.OAA23221@legend.txdirect.net>

I went to my local Chevy dlr today to get the 9C1 Recall kit.  Well

seems the recall is for Taxi's too.  Because every taxi cab company

as well as Police dept had dibs on all the kits they had in stock,

and there is a waiting list 1/2 mile long.

Parts guy couldnt understand WHY Impalas were NOT included in the

recall but he ordered me a kit anyway.

I suggest we ALL call Chevy cust. service and ask them the same

question. "Why aren' Impalas included?"  I think a few hundred calls

from Worried/Angry Impala Owners about both the ball joint and control

arm recalls might be enough to make them Add us to the Recall Roster.

What do you guys/girls think?

Bruce, what was the number you called?  Was it the cust svc # in the

back of the manual?   I'm calling.

God, grant me the courage to change the things I can,

the serenity to accept the things I can't,

And the wisdom to hide the bodies of the people who really

pissed me off.

Jeff Gatcha

1995 "Modified" DGGM

Foxhound@txdirect.net

San Antonio Texas

------------------------------

Date: Fri, 8 Mar 1996 12:35:46 PST

From: Mbmarx@aol.com

To: Impala@east.xsis.xerox.com

Cc: Impalaclub@aol.com

Subject: Hotel Update

Message-ID: <960308153545_441455603@mail02.mail.aol.com>

This is a message from Troy:

We have blocked 30 rooms at Fairfield Inn for the Arlington dates at 59.95 +

tax per night.  When you reserve your room YOU MUST mention NAISSO and they

will give you the discounted room rate.  The number is (817) 649-5800.

------------------------------

Date: Fri, 8 Mar 1996 12:39:00 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Fairfield Inn in Arlington

Message-ID: <31409AE4.5E18@intersurf.com>

Yo guys!

I just got off the phone with Michelle Mabee a manager at the Fairfield

Inn in Arlington.  She said the rates in April are going UP and not

down.  Troy Wilrick was the smart one and made a group block for the

NAISSO.  If you got a rate higher than $59 per night you need to call

the hotel, cancel your reservation and re-make it.  Michelle said to

tell the reservation clerk that you are with Troy Wilrick and the NAISSO

and want in the block of rooms for $59 per night.  Ordinary people will

be paying $79 on Thursday and $89 on the weekend.  Michelle told the

"the boss" will not give discounts normally in April even if you offer

to reserve the entire hotel.  This is because they rent the entire thing

anyway without discounts.

So:  call 817-649-5800  Tell the desk clerk or reservation desk that

you are with Troy Wilrick and the NAISSO and you can get the

$59 rate until all 50 rooms are booked up.  Let's swamp that

hotel with Impalas!!

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Fri, 8 Mar 1996 12:56:09 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: parts orders

Message-ID: <31409EE9.62EF@intersurf.com>

I got 9 responses from people wanting the corvette injector covers.  I

tried to email them all back directly but the mail may have gone to the

bit bucket.

I'm offering to make a massive parts order for people who can't get a

local dealer to give them the 30% discount they deserve.  Remember that

the corvette injector covers require the plastic valve covers or you

need the right hand one from a '92 vette.  I have never seen that part

number for that.  I can get and ship the injector covers complete with

studs and power brake booster hose for $32US.  If you can't get a local

dealer to do that or better I'll do it for you.

Ordinarily I'll ask list members to spend at least $100 so you are

getting enough of a discount to make it worth it and make it worth my

time to help you.  I'll add a $5 "handling fee" to cover riding your

parts around in my car.

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Fri, 8 Mar 1996 02:57:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: 'Impala SS mailing list' <impala@east.xsis.xerox.com>

Subject: emmisions tests

Message-ID: <3140A001@corvette.xyplex.com>

Just to let everyone know, my car passed emissions with flying colors at

0.00% CO and 0ppm HC with the Z PCM.  I had heard from F-body people that

some cars were not passing after mucking with PROMs and other

manufacturers programs so I was a little concerned.  When I saw the

screen I broke out in a smile from ear to ear.

After they put on the sticker they asked if the exhaust was stock when

they heard the rumble at idle.  They were shocked when I told them about

removing the mufflers.

+Z - mufflers = ~30 HP and it still passes emissions......

Scott Harris

sharris@xyplex.com

------------------------------

Date: Fri, 8 Mar 1996 14:31:28 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: BGordon401@aol.com

Cc: Impala <impala@east.xsis.xerox.com>

Subject: Re:96 C 14 RECALL

Message-ID: <199603082231.OAA06252@holland.it.earthlink.net>

Bruce:

>It turns out that THERE IS A PROBLEM WITH THE TORQUE ON THE LOWER BALL

JOINTS.  Service vehicles have had the lower ball joint knuckle nuts come off

because of being too loose from the factory.

I had the same problem in my '95...under torqued and dry. :-)

... Lennie

------------------------------

Date: Fri, 8 Mar 1996 14:31:45 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: Scott Mueller <73145.1566@compuserve.com>

Cc: Impala <impala@east.xsis.xerox.com>

Subject: Spark Plug Wires

Message-ID: <199603082231.OAA06262@holland.it.earthlink.net>

Scott,

1.)     What spark plug wires do you recommend for an upgrade?

2.)     I need (5) more of those spark plug end caps, do you know the part #?

... Lennie

------------------------------

Date: Fri, 8 Mar 1996 14:45:17 PST

From: Ed Salazar <esalazar@hooked.net>

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 336

Message-ID: <199603082245.OAA08398@mom.hooked.net>

-----------------------------

>

>Date: Fri, 8 Mar 1996 10:17:18 PST

>From: Franklin Poole <franklin@intersurf.com>

>To: Impala List Server <impala@east.xsis.xerox.com>

>Subject: Crow problems!

>Message-ID: <314079AE.332C@intersurf.com>

>

>I'll admit I was credulous when another list member had problems with

>crows...three days ago I watched two crows dive bomb the hood of my Dark

>Cherry Impala and then dance around on the hood pecking the heck out of

>it.  I threw down the phone and ran outside to throw pine cones at them

>(all the ammo I had).  They flew about 20 feet and the just sat on the

>ground an screamed at me.  No damage to shamu but gee whiz man,

>whatzadeal with the crows!

>--

>Franklin Poole, Jr.

>Praecedo, Inc.

YOu see, Franklin, the crows are merely emulating their African brethren

by trying to perch and preen their 'host' animal.  Haven't you seen

pictures of birds on rhinos, emu and the like.

These birds just happen to prefer Impalas.  Very cool Impalas.  <BG>

Ed

DCM, pending mods...

------------------------------

Date: Fri, 8 Mar 1996 14:49:04 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Security

Message-ID: <960308224903_73145.1566_FHD57-4@CompuServe.COM>

Franklin,

>I hope more than 50 will come. If security for our cars is a concern we can

hire local deputies for a few dollars each if necessary....

I am all for that! I for one would love to see a guard on duty by the

Impalas all night. Capitol punishment for door dings! Heck if we all chipped

in an additional $5 to $10 that should cover the security, right? Scott.

------------------------------

Date: Fri, 8 Mar 1996 14:49:08 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: upgraded control arms

Message-ID: <960308224907_73145.1566_FHD57-5@CompuServe.COM>

Tom,

>Do the upgraded lower control arms reuse the bracket(?) that the sway ba

bolts to from the old control arm???

No, the kit comes with completely new brackets that are totally different in

design, and which replace the old ones. In fact this kit completely replaces

the old control arms, swaybar brackets and even the arm and bar attaching

hardware. Scott.

------------------------------

Date: Fri, 8 Mar 1996 15:14:51 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Cc: Troy Willrick <Rock528@aol.com>

Subject: Re: NAISSO Newletter??

Message-ID: <960308231451_76165.1654_GHM154-5@CompuServe.COM>

Today is Friday, March 8, 1996 and I still DO NOT have my newsletter.

Troy, you said you shipped these on Tuesday, February 27th (I think?)and you

wanted us to tell you when we got them. Well, its been over a week, and I didn't

get mine yet.

Has anyone else received their newsletter yet?

Kurt

'95 Dark Cherry Impala SS........no newsletter.........yet.........

------------------------------

Date: Fri, 8 Mar 1996 15:32:26 PST

From: dstabb@epix.net (David Stabb)

To: impala@east.xsis.xerox.com

Subject: Effect of Simple Mods

Message-ID: <v02130500ad666eb805cc@[199.224.67.38]>

I'm sure you have been over this before (but I haven't downloaded all the

old archives ... yet), so please indulge me as a new SS owner.

1) What is the effect of a 160 degree thermostat? On power, durablility,

and gas mileage?

2) And the 3.42 gears? (I haven't caught the cost of 3.42 gears, installed

by the dealer). What does that do to gas mileage, ET's, top end, and

1/4 mile speed? Or 0-60, which is more relevant.

3) Finally the K&N filter and opened up stock airbox. How is this mod done?

Is there a kit? Will it help without exhaust mods?

The last car I played with seriously was a F*ord Fiesta that I bought new

in 1978. Quite a different vehicle, and from the days when not so much was

electronic. Major fun, though, as I took the 1.6L pushrod engine and turned

it into a mini-screamer. Modified suspension, Ford Eurosport ignition,

free-flo air filter, Weber DATR 25-26 progressive 2 barrel, Ford Eurosport

manifold with long plenum, big valve gas flowed head, 1.5 grind cam, .040

Formula Ford overbore 9.5:1 pistons, and a quick-trip exhaust. Things were

cheaper in those days, and the little Fiesta did 0-60 in the low 7's. A

pretty twitchy little vehicle, not at all like going fast in the SS.

The week's snow gave me a chance to try the 2nd gear start. No dice on my

late in the year 95.

This is not a great snow car. But then that's not what I bought it for.

____|)avid             Good Morning, Good Afternoon, Good Evening.

How difficult it is to tell when

dstabb@epix.net               you're locked in a tiny room with

dstabb@bix.com                     no windows and artificial lights.

------------------------------

Date: Fri, 8 Mar 1996 15:37:44 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: AOL forum

Message-ID: <960308233743_75511.1220_GHJ99-1@CompuServe.COM>

I currently have Compuserve so I can't join the Thursday forum's.

Is it possible to meet on the Net and have an open line forum?

I'm not Net literate so don't know if this is possible.

Curt P

------------------------------

Date: Fri, 8 Mar 1996 15:37:47 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Jegs Catalog

Message-ID: <960308233746_75511.1220_GHJ99-2@CompuServe.COM>

Scott,

Sorry about the lame. I don't mean like lame Impala but like partially mauled,

or injured<G>

I just got a new Jegs catalog today and opened it up to page 152.

Impala Tee shirt "Room To Zoom" white with black SS, Looks like red bowtie on

front emblem. Guess they saw Scott's car and copied it. Scott do you have a copy

right on your idea?<G>  Also black quarter panel SS logo. Part #427-GC265  L,XL

or XXL $11.99 e.

Tel: 1-800-345-4545

Impala SS cap. Black with silver deer and rear quarter panel logo part #

427-RB118 $9.99 e.

"Room To Zoom"

Curt P.

------------------------------

End of IMPALA Digest 337

************************

IMPALA Digest 338

Topics covered in this issue include:

1) Which Mustangs May we Safely Blow Off?

by dstabb@epix.net (David Stabb)

2) SS Chat

by dstabb@epix.net (David Stabb)

3) Fleetwood VIN

by lennie@earthlink.net (Lennie Dolphin)

4) Crow Comedy

by dhflores@iwl.net (Debbie/Henry Flores)

5) mods in stages

by dhflores@iwl.net (Debbie/Henry Flores)

6) Tilt and Toot

by lennie@earthlink.net (Lennie Dolphin)

7)

by David Hardy <dhardy@crl.com>

8) Who the h*ll do you think you are?

by Rock528@aol.com

9) LATEST HOTEL UPDATE

by Rock528@aol.com

10) Re: rear calipers!

by Rock528@aol.com

11) Re: 1-900-IMPALA chat line

by Rock528@aol.com

12) Could someone forward the last two digests???

by Loren Peterson <equ_rino@cencom.net>

13)

by Rock528@aol.com

14) LIFETIME MEMBER NO. 1

by Impalaclub@aol.com

15) Jeg's Impala SS Merchandise

by BGordon401@aol.com

16) Chevy CSV Phone Number

by BGordon401@aol.com

----------------------------------------------------------------------

Date: Fri, 8 Mar 1996 15:45:21 PST

From: dstabb@epix.net (David Stabb)

To: impala@east.xsis.xerox.com

Subject: Which Mustangs May we Safely Blow Off?

Message-ID: <v02130501ad66734a18bf@[199.224.67.38]>

Gentlemen (and any ladies who read the list):

I am not one who cares to be embarrassed. There are some pretty hot

Camaros, and yes M*st*ngs out there. SHO's I'm not concerned about. Which,

if any must a stock SS'r avoid?

____|)avid             Good Morning, Good Afternoon, Good Evening.

How difficult it is to tell when

dstabb@epix.net               you're locked in a tiny room with

dstabb@bix.com                     no windows and artificial lights.

------------------------------

Date: Fri, 8 Mar 1996 15:45:29 PST

From: dstabb@epix.net (David Stabb)

To: impala@east.xsis.xerox.com

Subject: SS Chat

Message-ID: <v02130502ad6675fcbad5@[199.224.67.67]>

<<holding clove of Garlic, and standing in a pentagram, just in case>>

I'd not mind a chat session on IRC, but won't buy AOL again.

____|)avid             Good Morning, Good Afternoon, Good Evening.

How difficult it is to tell when

dstabb@epix.net               you're locked in a tiny room with

dstabb@bix.com                     no windows and artificial lights.

------------------------------

Date: Fri, 8 Mar 1996 16:09:39 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: Scott Mueller <73145.1566@compuserve.com>

Cc: Impala <impala@east.xsis.xerox.com>

Subject: Fleetwood VIN

Message-ID: <199603090009.QAA29825@iceland.it.earthlink.net>

Scott,

Have you looked into whether we can use the Fleetwood VIN with the 3.42

gears to flash our PCMs?

... Lennie

------------------------------

Date: Fri, 8 Mar 1996 16:39:37 PST

From: dhflores@iwl.net (Debbie/Henry Flores)

To: impala@east.xsis.xerox.com

Cc: franklin@intersurf.com

Subject: Crow Comedy

Message-ID: <96Mar8.164308pst."14905(7)"@alpha.xerox.com>

You're not going to believe this but I have been raped and pillaged by crows

also. I feel so violated.  Do you think there is a conspiracy forming?

Perhaps Ford in training crows to dive bomb our Impalas? Do you think

Chrysler is sending in mechanical crows, perhaps the local paint shops are

in on it?  Nonetheless, diving is one thind.......dropping their calling

cards is another.  If can get a good safe angle on them......Pellet rifle to

the rescue.

Henry in Houston

------------------------------

Date: Fri, 8 Mar 1996 16:49:08 PST

From: dhflores@iwl.net (Debbie/Henry Flores)

To: jeffd@accessus.net

Cc: impala@east.xsis.xerox.com

Subject: mods in stages

Message-ID: <96Mar8.165251pst."14640(15)"@alpha.xerox.com>

Jeff

I am interested in your stage approach to your Impala.  I also am taking

that approach.  To make a long story short here is the final outcome of my

efforts 14.94 ET at 94 MPH.  My car has been going through stages also.  It

ran this with:

The hennessey air induction

The 52mm throttle body

Enlarged Mass Air Housing unit

Crank Pulley (although the fellas said this means zip in power)

3.42 gears

Jacobs ignition

Hypertech chip

Good luck on your efforts. I would be interested in your progress. Stay in

touch.

Henry in Houston

------------------------------

Date: Fri, 8 Mar 1996 16:59:16 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: Woo Yang <wooyang@uclink2.berkeley.edu>

Cc: Impala <impala@east.xsis.xerox.com>

Subject: Tilt and Toot

Message-ID: <199603090059.QAA07603@finland.it.earthlink.net>

Woo:

>By the way, can anyone give me the tsb# for the tilt and toot?

Gonna do my '95 machine on Monday: TSB 53-32-05 (6/95), of course my service

tech wants me to bring it in first (whoever he is) so he can make an

assessment. I'll see how assessing he is.<g>.

- Wheel Assembly

# 16757633

- SIR Assembly

# 16757619

... Lennie

------------------------------

Date: Fri, 8 Mar 1996 18:42:20 PST

From: David Hardy <dhardy@crl.com>

To: impala@east.xsis.xerox.com

Message-ID: <Pine.SUN.3.91.960308182757.12390D-100000@crl3.crl.com>

I have the reservations for the Fairmont Inn, my days off were approved,

and I have a rigorous prep schedule for the Impala to make the 1800 mile

trek to Arlington from San Francisco!

Look out!

-Dave

------------------------------

Date: Fri, 8 Mar 1996 19:04:17 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Who the h*ll do you think you are?

Message-ID: <960308220416_441762719@emout07.mail.aol.com>

Hello Again,

1.  with all that's going on, can someone email me the info on the 9C1 arms.

I need the part # or the tsb # or a copy of it faxed to 904-258-3779.

2.  I only had 36 email messages to answer tonight, plus the digests..

Thanks to all for the help on the arlington meet.  It should work out to be

quite a fun time.  I am dreaming of April 17th when I can turn my brain off

and get behind the wheel for a 1000 mile drive.  I hope you all will come to

the Impalafest 97 in Daytona next year <g>.

3.  Someone asked me who I was and I had to think for a minute..  While

reading the digests, I also wonder who this guy Troy is?  Sometimes I have to

do a reality check because this all started with seeing that damn SS ad while

relaxing on the john one evening (sorry ladies).  I can assure you that while

it looks like I pioneered some parts of this venture, it takes work from

many.  You can call me anytime at 904-253-6638 in Daytona Beach.  I have

opened my home and served soup to many an Impala owner during the Daytona

shows.  I put my impala boxers on one leg at a time, just like you <g>.

this is me if you are wondering:

Troy C. Willrick

34 yoa

single - looking for a girl with a black ss for my collection (measurements

desired, 9C1-3.42-B19)

Flight paramedic/deputy sheriff/infection control officer/instructor

Volusia County Sheriff's Office

another 4 years and I'll have 20 in with the county.

VP of Advanced Medical & Pharmaceutical Suppliers,

Ormond Beach, FL  800-553-AMPS

Teach at the Community College, EMS related stuff mainly, ACLS, CPR.

got 2 golden retrievers also.  So you see, other than a few normal traits, I

guess I am pretty screwed up. <g>  oh, in my spare time I run a small 350+

member car club..

ps - I would like to see some better buy lines for your name tags, however.

Be creative or else I will  >-)

Till next time..Rock

------------------------------

Date: Fri, 8 Mar 1996 19:04:17 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com, dhardy@crl.com

Subject: LATEST HOTEL UPDATE

Message-ID: <960308220417_164054052@emout05.mail.aol.com>

group and Superdave for the web site.

ok all, you can rest about the hotel stuff and thanks to Erich for the

legwork.  After all the problems and calling their corp. office, a little @ss

kissing with the Gen. Mgr. ( I promised I would take her picture next to my

SS) I was able to do the room block for 30 rooms for the 59.00 rate.  On to

the next tagamet moment..   (Dave, don't include this paragraph)

Below is a forward from Erich on some other good info.  I recommend calling

ASAP if you want the Fairfield.  The rate will be 89.00 if they run out of

the 30 room block.  (unless we can extend it.  Our people are working on it)

Troy in Daytona.

---------------------

Forwarded message:

From:	earndt@airmail.net (Erich Arndt)

To:	rock528@aol.com, foxhound@txdirect.net, sdas@sat.net

Date: 96-03-08 13:32:29 EST

'Mornin Gents,

I just got back from my road trip and thought I'd share my results and a

couple of observations, FWIW.

Here's the distances to the major destinations:

Fairfield Inn to:

Arlington Plant --> 1.5 mi

Comfort Inn (Waxahachie) --> 39.9 mi

Texas Motorplex (Ennis) --> 50.9 mi

Comfort Inn (Waxahachie) to:

Arlington Plant --> 38.4 mi

Fairfield Inn (Arlington) --> 39.9 mi

Texas Motorplex (Ennis) --> 11.0 mi

Directions to the various sites will follow over the weekend; it's gonna

take a bit to write them up to where they're usable.

I stopped into both Fairfield and Comfort Inns to scope out the facilities

and the rooms.  Of the two, I'd probably prefer the Fairfield for a couple

of reasons:

(1) The Comfort Inn is full for that weekend; it seems there's more than

one show in town that weekend. The rooms look to have been "well used".

(2) It looks to be more secure. Both Fairfield and Comfort Inns are close

to the highway, but Fairfield is not as obvious.  The Comfort Inn is

probably fine, but I'd personally prefer to *not* have my car between the

hotel and a major highway where it can be seen by anyone driving by.

(3) The rate has been negotiated by Troy.  I believe the difference

between Fairfield and Comfort Inn has been cut to about $6 (Troy?).  The

problem turned out to be that they switch to summer rates starting that

weekend. (I believe it; Six Flags is a couple of blocks away...)

(4) I like an occasional "beverage of the foaming variety, generally

served in a frosted glass".  Forget finding any within a 20 mile radius of

Waxahachie; it's available in any restaurant in Arlington.

Doors to the rooms at the Fairfield are inside, at Comfort Inn they are

outside.

I got to the Fairfield Inn right after Troy called.  I do know that he was

able to negotiate a block of 30 rooms for our group at a significantly

reduced rate that would make me consider switching locations to be with the

rest of the group.  It's just a much nicer place and the restaurants and

bars are within walking/short drive distance.

Also, Fairfield Inn supplies a continental breakfast as part of the deal and

has a nice pool area.  Unfortunately, no meeting room of any kind.

Arlington is a "non-smoking" town, meaning that you can't smoke in any

restaurant or public place (like a mall or city building).  I don't know

about the rooms; not being a smoker I didn't think to ask.

The contact at Fairfield in is Suzanne Nikkel (General Manager) and the

phone is (817)649-5800 for anyone who is interested.  The number for Comfort

Inn is (214)937-4202, John and Hazel Carter Co-Managers.

Let me know if anyone has any questions and I'll try to answer them.

Thanks,

Erich

P.S. -- Troy, could you forward whatever of this you find useful to the

list?  I can't submit from this E-Mail address.   THNX, EA

------------------------------

Date: Fri, 8 Mar 1996 19:04:20 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: rear calipers!

Message-ID: <960308220419_441762641@mail04.mail.aol.com>

In a message dated 96-03-08 14:09:33 EST, you write:

>"I polished the rear aluminum calipers with a die grinder/scotchbrite pads

>and

>metal polish.  They look like chrome.  Troy"

>

>See, Troy also exhibits perfectly normal behavior, I mean doesn't

>everybody polish their brake calipers?

OK, fess up.  After you read it, how many of you went out and looked at them

and thought it would look kind of trick??   Troy

------------------------------

Date: Fri, 8 Mar 1996 19:04:21 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: 1-900-IMPALA chat line

Message-ID: <960308220420_441762655@emout04.mail.aol.com>

In a message dated 96-03-08 14:09:33 EST, you write:

> Troy, if you speak with John Moss again before then, Ask him if he has aol

>and would he like to join us.

I don't think he does but I will ask.  I know the brand manager for the

impala/lumina/montecarlo does.  That would be our best bet.  Troy

------------------------------

Date: Fri, 8 Mar 1996 19:51:39 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Cc: equ_rino@cencom.net

Subject: Could someone forward the last two digests???

Message-ID: <3141004B.62A6@ns.cencom.net>

Help,

Could someone forward the last two digests, I lost them before I could

read them.

Loren

DCM96

------------------------------

Date: Fri, 8 Mar 1996 20:04:11 PST

From: Rock528@aol.com

To: <impala@east.xsis.xerox.com>

Message-ID: <199603090404.XAA17615@emout05.mail.aol.com>

est...

50/50 Raffle to Support your NAISSO.

Guess how many 96's will be built and the closest gets half the dough.  One

dollar for each guess.  More poster raisin money..  for 96-97.

Troy.  Give me your buck at Arlington.

------------------------------

Date: Fri, 8 Mar 1996 20:06:39 PST

From: Impalaclub@aol.com

To: impala@east.xsis.xerox.com

Subject: LIFETIME MEMBER NO. 1

Message-ID: <960308230638_441805969@mail06.mail.aol.com>

Congratulations to our first lifetime member of NAISSO:

After my very humble request for donations to get our Impala SS Club projects

rolling, I received a very generous offer from one of our members.  All names

will be kept confidential and this person has received our first lifetime

membership.

I can now start work on some projects like the NAISSO club shirts, impala

poster reproductions, Buddy L impala model distributorship and those

beautiful velvet Elvis paintings for my home.  (you know I couldn't get all

the way through without a comment) <g>

Thank you VERY much and there is much work to be done.  I would encourage

anyone with similar interests of seeing our club grow to contact me at

904-253-6638, 24 hrs/day.

Respectfully,

Troy Willrick

President/NAISSO

------------------------------

Date: Fri, 8 Mar 1996 20:46:31 PST

From: BGordon401@aol.com

To: impala@east.xsis.xerox.com

Subject: Jeg's Impala SS Merchandise

Message-ID: <960308234630_164124111@mail02.mail.aol.com>

>>I just got a new Jegs catalog today and opened it up to page 152.

Impala Tee shirt "Room To Zoom" white with black SS, Looks like red bowtie on

front emblem. Guess they saw Scott's car and copied it. Scott do you have a

copy

right on your idea?<G>  Also black quarter panel SS logo. Part #427-GC265

L,XL

or XXL $11.99 e.

Tel: 1-800-345-4545

Impala SS cap. Black with silver deer and rear quarter panel logo part #

427-RB118 $9.99 e.

"Room To Zoom"<<

Just received both the cap and the T-shirt.  VERY COOL!!!.  The hat will be

worn quite often, but I think the T-shirt will be professionally pressed and

starched for it to be framed.

Whoever said that this "Impala Thing" is an obsession??  I think it is

perfectly normal to have such a COOL picture mounted and framed...even if

it's on a T-shirt.  When you all see it, I'm sure you'll feel the same way.

Give Jeg's a call.  BTW, I don't work for, hold stock in, or endorse this

company.  I do, however, think they have great stuff at decent prices.

..Bruce

------------------------------

Date: Fri, 8 Mar 1996 20:47:51 PST

From: BGordon401@aol.com

To: foxhound@legend.txdirect.net

Cc: impala@east.xsis.xerox.com

Subject: Chevy CSV Phone Number

Message-ID: <960308234751_164124091@emout09.mail.aol.com>

Jeff, you wrote:

>>I suggest we ALL call Chevy cust. service and ask them the same

question. "Why aren' Impalas included?"  I think a few hundred calls

from Worried/Angry Impala Owners about both the ball joint and control

arm recalls might be enough to make them Add us to the Recall Roster.

What do you guys/girls think?  <<

>>Bruce, what was the number you called?  Was it the cust svc # in the

back of the manual?   I'm calling. <<

Jeff, you're right on track with calling the CSV line.  I complained about

the ball joint recall that applies to the taxies and police Caprices, but not

our Impalas.  What's up with this??  Our cars come off the same production

line!!

The number is  1-800-222-1020.  I've called the Chevy CSV line many times,

and they are a wealth of information.  If you are concerned about TSBs and

recalls applying to taxies and police Caprices but not Impalas, give the CSV

line a call.  Personally, my name is on their system for the 96 C 14 ball

joint recall.  I urge anybody else who checked the ball joints, and found an

under-torqued condition, to call  and aire your concern for the "overlook" of

our Impalas.

..Bruce

------------------------------

End of IMPALA Digest 338

************************

IMPALA Digest 339

Topics covered in this issue include:

1) Super Chevy Count

by Rock528@aol.com

2) Re: IMPALA SS MERCHANDISE

by Rock528@aol.com

3) Bow ties

by "Jeff Gatcha" <foxhound@txdirect.net>

4) Room 2 Zoom/AOL

by "Jeff Gatcha" <foxhound@txdirect.net>

5) spark plug wires

by Scott Mueller <73145.1566@compuserve.com>

6) fleetwood cal

by Scott Mueller <73145.1566@compuserve.com>

7) new design control arms

by Scott Mueller <73145.1566@compuserve.com>

8)

by Steve Das <sdas@sat.net>

9) Painting front calipers

by jeffd@accessus.net (Jeff Dawson)

10) Converter/Transgo feedback

by dhflores@iwl.net (Debbie/Henry Flores)

11) Crows and crawfish

by "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

12) THIS is the greatest list going

by Loren Peterson <equ_rino@cencom.net>

13) Cruising Big D

by "Brett A. Rudolph" <bngwip@mail.phoenix.net>

14) angle of Z28 intake elbow?

by Loren Peterson <equ_rino@cencom.net>

15) Impala For Sale

by Mfetto@aol.com

16) Arlington RSVP's (first day totals)

by Matt Platts <mplat@airmail.net>

17) Fuel Pressure

by Mbmarx@aol.com

----------------------------------------------------------------------

Date: Fri, 8 Mar 1996 21:41:03 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Super Chevy Count

Message-ID: <960309004101_345773967@emout08.mail.aol.com>

Latest count:  21 impalas to show at Arlington for a national meet.  Surely

out of 350 list members we can do better.  I can pull together 30 at a local

show with an hour notice.

Hope to see more in TX.  Troy

------------------------------

Date: Fri, 8 Mar 1996 21:41:39 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA SS MERCHANDISE

Message-ID: <960309004138_345774055@emout09.mail.aol.com>

In a message dated 96-03-08 23:50:40 EST, you write:

>Impala Tee shirt "Room To Zoom" white with black SS, Looks like red bowtie

on

>front emblem. Guess they saw Scott's car and copied it. Scott do you have a

>copy

>right on your idea?<G>  Also black quarter panel SS logo. Part #427-GC265

>L,XL

>or XXL $11.99 e.

>

>Tel: 1-800-345-4545

>Impala SS cap. Black with silver deer and rear quarter panel logo part #

>427-RB118 $9.99 e.

Bruce:

Aw come on.  I have been offering these same exact items for 6 months.

Wouldn't you rather support your own club??   Troy.

ps - they beat me by a penny on the shirt and I beat them on the hat by a

dollar.  Subscribe to the club newsletter and get all the Impala items.

>Give Jeg's a call.  BTW, I don't work for, hold stock in, or endorse this

company.  I do, however, think they have great stuff at decent prices.

but you do hold an interest in NAISSO  ;-)  got you, didn't I, Troy

------------------------------

Date: Fri, 8 Mar 1996 23:28:48 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Bow ties

Message-ID: <199603090728.BAA21125@legend.txdirect.net>

>>I am having my decal guy make a decal - exact fit to go over the

>>chrome emblem on the grill.  If anyone wants one, let me know.  He can make

>>them in any color.  I will have RED of course.

>How about a silver/white reflective bowtie for all of us who own Green-Grays

>and Dark-Cherrys?   I'd buy one.

Guys, I have a Green Grey Impala and have used the Red trim brite on the

grill bow tie.  Red looks REALLY NICE (IMO) on the Green Grey also.

I dont know abou the DCM but it works on Green.  Must be Awesome on

black.  Just my 2 cents.

Jeff Gatcha

------------------------------

Date: Fri, 8 Mar 1996 23:48:19 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Room 2 Zoom/AOL

Message-ID: <199603090747.BAA21583@legend.txdirect.net>

I'll second the "room to zoom"  T-shirt. Its a great looker.  I

picked 2 up at a local speed shop for $12 each.  One to wear, one to

frame.

Also,  Im hooked up to an internet provider and dont have AOL but I

can get  IRC (internet relay chat) .  Can this work with AOL chat rooms?  Is AOL

compatible with IRC?  How do I get in on these on-line chats?

By the way, a note for you AOL users:

For those of you who still have an AOL account, or have an interest in

Privacy issues, I just discovered an interesting feature of America Online.

It is called the "Buddy List".  It basically allows you to track the online

movements of screennames that you designate.  And yes, it works in reverse.

If someone puts you on their "Buddy List", they can track your online movements.

This feature can be disabled, but its default is an "active" preference.

You must go in and disable it.  Problem is, many people don't even know it

exists.

For more information, go to keyword:  Buddy  Once there you can create your

own list and even check your "Preferences" where you can disable the feature.

Just FYI...

Jeff Gatcha

foxhound@txdirect.net

1995 DGGM

------------------------------

Date: Sat, 9 Mar 1996 00:13:34 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: spark plug wires

Message-ID: <960309081333_73145.1566_FHD36-3@CompuServe.COM>

Lennie,

>What spark plug wires do you recommend for an upgrade?

Why, the killer RED LT4 wires, of course! As a second choice, the improved

design '96 B-car wires.

>I need (5) more of those spark plug end caps, do you know the part #?

If you mean the heat shields, they are #12132232, and cost 98 cents each.

Scott.

------------------------------

Date: Sat, 9 Mar 1996 00:13:47 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: fleetwood cal

Message-ID: <960309081346_73145.1566_FHD36-6@CompuServe.COM>

Lennie,

>Have you looked into whether we can use the Fleetwood VIN with the 3.42

gears to flash our PCMs?

Sure, it would probably work OK, but there may be problems as the Fleetwood

has traction control linked to the ABS controller. You would be better off

using the F-car calibration (and MAF), which is absolutely perfect for the

3.42 gear in the Impala, plus you get a hotter spark, vastly better shifting

and transmission operation, not to mention the performance shift feature,

plus a bigger MAF for better breathing. THAT is the way to go if you want to

try a different cal IMHO. Scott.

------------------------------

Date: Sat, 9 Mar 1996 00:14:04 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: new design control arms

Message-ID: <960309081403_73145.1566_FHD36-9@CompuServe.COM>

Troy,

>with all that's going on, can someone email me the info on the 9C1 arms. I

need the part # or the tsb # or a copy of it faxed to 904-258-3779.

Here is what I originally posted, with some minor updates:

There is a new TSB that will affect us Impala owners. It was just released

and I don't have the TSB number right now, but it officially covers only the

9C1 cars, even though the Impala suspension is essentially the same.

It seems that the cops have been having problems with the swaybar brackets

and how the brackets attach to the rear lower control arm, and how the

swaybar then attaches to the brackets. I can tell you from personal

experience that this is a problem area, especially with the monster 1.5"

Herb Adams rear swaybar installed. I have literally ripped apart two sets of

brackets on my '88 9C1 with the HA bar! The first one I tore up I attributed

to the fact that the original brackets were weakened by rust, as the huge

bar actually pulled the attaching nut right through the steel bracket. The

second time I sheared off the side bolts which attach the bracket to the

control arm.

I have not had any similar problems on my Impala, however that is probably

because I used ARP grade 8+ stainless steel hardware for both the side

bracket bolts as well as the longer bolts to hold the bar to the bracket.

Also, the Impala was new and the bracket was of course not rusted like on my

'88 9C1.

In any case, this new TSB addresses this problem with a completely new rear

lower control arm design (including high durometer bushings) that has

different holes, a different swaybar attaching bracket, and long bolts with

nuts that go completely through the control arm from top to bottom. The new

arms, brackets and bolts result in a much stronger setup than before. This

would be especially recommended if you are using a larger rear swaybar like

the Herb Adams unit. BTW, it does look like we will need about a 1/2" longer

versions of the bar attach bolts supplied with the kit will be needed to

attach the super thick Herb Adams bars.

The TSB refers to a kit p/n 10289786 which includes everything to convert

from the old setup to the new one, including 2 rear lower control arms

complete with high durometer bushings, new control arm attachment bolts and

nuts, new (different design) swaybar attach brackets, swaybar attach long

bolts and nuts, washers, etc. Your dealer may tell you that this number is

no good, but that is only because it is brand new. Since the TSB only refers

to the '91-'96 9C1 cars, you will have to pay for this one yourself if you

want to make the change, but it is a bargain and only costs about $40 (less

than a single control arm purchased seperately!). Scott.

------------------------------

Date: Sat, 9 Mar 1996 01:21:30 PST

From: Steve Das <sdas@sat.net>

To: impala@east.xsis.xerox.com

Cc: dhflores@iwl.net (Debbie/Henry Flores)

Message-ID: <199603090919.DAA07046@raven.sat.net>

>You're not going to believe this but I have been raped and pillaged by crows

>also. I feel so violated.  Do you think there is a conspiracy forming?

>Perhaps Ford in training crows to dive bomb our Impalas? Do you think

>Chrysler is sending in mechanical crows, perhaps the local paint shops are

>in on it?  Nonetheless, diving is one thind.......dropping their calling

>cards is another.  If can get a good safe angle on them......Pellet rifle to

>the rescue.

Amazing!

Wonder if GM will provide 12 Ga. shells under the warranty program.  If so,

for how long???

SD

------------------------------

Date: Sat, 9 Mar 1996 04:33:41 PST

From: jeffd@accessus.net (Jeff Dawson)

To: impala@east.xsis.xerox.com

Subject: Painting front calipers

Message-ID: <199603091233.GAA28288@mtvernon1.accessus.net>

Polishing the rear calipers sounds good but what to do with the fronts. One

of the mags awhile back did a brake detail with the GT rotors and they

painted the f&r calipers black. My idea was to paint the front with a

aluminized type paint so it would match the back ones. Heres the question,

what kind of paint? It would have to be a Hi-temp type paint, I would think,

to stand up. Does VHT or anyone else make such a paint, in a color that

would be close to the unpolished rear?

Jeff Dawson 95 DCM

------------------------------

Date: Sat, 9 Mar 1996 06:11:42 PST

From: dhflores@iwl.net (Debbie/Henry Flores)

To: impala@east.xsis.xerox.com

Subject: Converter/Transgo feedback

Message-ID: <96Mar9.062450pst."14523(8)"@alpha.xerox.com>

Fellas,

I am taking my Impala in for the Transgo install March 15. The first shop

chose not to do it. He preferred a tranny shop to do it instead.

Nevertheless, here is my time sensitive feedback request:  (Your expertise

and assistance will be greatly appreciated)

1) I looked into the 1986 Torque Converter/Trango shift kit combination.

a) Is the price of $275 converter without exchange and a $175 R&R on the

install in the ballpark or is this expensive? This shop builds torque

converters.

b) Is there another approach to obtaining one of these converters a bit

more inexpensively?

c) I called the local dealer on the part number that Scott M. kindly

provided to me and the service/part rep had no idea of the torque converter

numbers. He needed the specific transmission type of that particular

Corvette.  Idiot or valid?

d) Will the use of a different converter effect the computer control of the

tranny in any way, shape or form? This question was posed by the converter

shop not the install shop.  What are your valued opions?

e) Lastly, and most importantly, any drawbacks to this coverter?

Henry in Houston

------------------------------

Date: Sat, 9 Mar 1996 00:24:58 PST

From: "jeffrey.s.steenwyk" <jeffrey.s.steenwyk@arthurandersen.com>

To: impala <impala@east.xsis.xerox.com>

Cc: franklin <franklin@intersurf.com>

Subject: Crows and crawfish

Message-ID: <9603091426.AA0690@CServe-Hub4.inhouse.compuserve.com>

Franklin:

>>  ...No damage to shamu but gee whiz man, whatzadeal with the crows! <<

You might want to change that Crawfish boil in April to the "First Annual Baton

Rouge Crawfish Boil & Crow Shoot".  BYO 12 Gauge!  BTW, wish I could make both

the Arlington Trip & your Crawfish boil (unfortunately, I gotta miss this one).

Jeff

'96 DCM

------------------------------

Date: Sat, 9 Mar 1996 06:43:02 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: THIS is the greatest list going

Message-ID: <314198F6.529E@ns.cencom.net>

Hey SSers!

Is this list great or what!  Thanks Lennie for the back issues, I

appreciate it.

CROWS, you guys are killing me.  My neighborhood is loaded with those

suckers!  Im going to be a nervous wreck.  Guess I'll hire my unemployed

Son to Set in the Locust tree with my Double barrel 10 gauge!

Thanks to those conerned for getting the Fairfield Inn deal fixed.

For those interested in decals, My guy is swamped but will get to it

soon I hope.  For all the special requests, please Email again, as I

lost all my files when I put in the new netscape 2.0.  Computer idiots

should not try it alone.

God I love those SS's, cant wait till Arlington SS FESTIVAL!

Loren DCM96

SPE

------------------------------

Date: Sat, 9 Mar 1996 06:58:35 PST

From: "Brett A. Rudolph" <bngwip@mail.phoenix.net>

To: impala@east.xsis.xerox.com

Subject: Cruising Big D

Message-ID: <1.5.4b11.16.19960309090233.097f3776@phoenix.net>

Howdy!

I was wondering if anyone had given any thought to an Impala Cruise for

Friday or Saturday night During the Meet in Dallas. I lived there for many

years and have a few suggestions about where to go to be seen.

First, there is somewhat of an icon for cruising in Dallas called Kellers

drive in. It is Located at Northwest Highway and Skillman in north east

Dallas. They have long neck beer for a buck and the burgers are great! Cheap

too. I think 15 or 20 Impala's would look way cool at the drive in.

Second, after going to Kellers for Burgers and beer, We could head down to

Lower Greenville and cruise the strip. It is not far at all and is on the

way to the final destination. BTW, Thats where you cruise in Dallas to be

seen and there are plenty of places to stop and get a cold one like The Blue

Goose or GBG's. ( Greenville Bar & Grill ) There are alot of good bars even

further down on the way to the next stop on our Dallas Extraviganza.

Third, After Greenville, We hang a right and cruise to Downtown and The

West end Market Place. For those of you who aren't familiar, It is a

revitalized Wharehouse district with gobs of Resturants and bars. It is a

lot of fun! Amoung the bars is a Hooters. W00H00!

These are only suggestions as I am not certain where to go in Arlington or

if there is any place to go in Arlington To cruise. I will probably stay in

Dallas rather than Arlington but I haven't decided. I get a good rate at a

couple of hotels nearby being a pilot. I right stay in Addison too. The

nicest Motel Six you will ever find is there and it's 30 bucks a night. And

it is right in the middle of some of the best resturants around. On The

Border is directly across the street. That way I can Stumble home late.

hehe. IT's also a well known spot for meeting the Ladies. FYI for you single

guys. Being close to the Factory for just Fridays tour is not that important

to me. Rather be close to the wimmens! ;)~

Sincerely,

Your Cruise Director,

Brett  :)

Brett A. Rudolph

96 SS Black

(still waiting)

If you build it,

I will come.

------------------------------

Date: Sat, 9 Mar 1996 06:58:24 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: angle of Z28 intake elbow?

Message-ID: <31419C90.2AA1@ns.cencom.net>

I just ordered the goodies to dress up the engine from my local Chevy

dealer.  I also ordered the Z28 intake elbow.  Is it the same angle as

the Impala's?  Just wondering if I'm going to have to modify my

fabulous, wonderful, cool Sewer Pipe arrangement.  This setup really

sucks!  Air, lots of air, its a beautiful sound.

Thanks for the info on T shirts and hats.  I am going to wear one hat

and stick the other in the rear deck.  I am going to buy 2 shirts too.

Wear one and ya guy your right, press the other and frame it.

See you SSers in TEX

Loren DCM96

------------------------------

Date: Sat, 9 Mar 1996 08:23:36 PST

From: Mfetto@aol.com

To: impala@east.xsis.xerox.com

Subject: Impala For Sale

Message-ID: <960309112336_442048772@mail04.mail.aol.com>

An associate of mine here in Western New York is selling his '95 Impala.

Details:

1995 Impala 'SS, Cherry Red.

900 miles, no winter.

Everything execpt CD player.

$21,000.

Any interested parties can send me an e-mail.

Mike

------------------------------

Date: Fri, 8 Mar 1996 18:03:00 PST

From: Matt Platts <mplat@airmail.net>

To: "'Impala'" <impala@east.xsis.xerox.com>

Subject: Arlington RSVP's (first day totals)

Message-ID: <01BB0DA0.A845DD60@dal07-03.ppp.iadfw.net>

Here are the first day totals for the Arlington trip (just thought some might be interrested).

Driving to area: 18

Actuall # of SS: 11

Flying to area: 5

Renting a car: 1

Staying @ Fairfield: 16

Actual # of rooms @ Fairfield: 8

Staying @ other motels: 2

Headcount for GM tour: 24

Headcount for SCS: 24

Imp SS's on display @ SCS: 11

I will keep everyone apprised of the data as it filters in...

-matt

------------------------------

Date: Sat, 9 Mar 1996 09:25:23 PST

From: Mbmarx@aol.com

To: Impala@east.xsis.xerox.com

Subject: Fuel Pressure

Message-ID: <960309122522_346048682@mail06.mail.aol.com>

Does anyone have the correct fuel pressure specs. for

the B-body LT1? My next purchase may be an adjustable fuel

regulator and I checked my stock unit and it only had 38 psi

at idle. All the test I've read show stock pressure to be

some where between 42 and 45 psi. At low vacuum or

full throttle "burp" the max pressure was 46 psi. Can anyone

elaborate? Scott M.?, Jason Haines?

Thanks in advance

Mike Marx

95 Black, Usual mods

------------------------------

End of IMPALA Digest 339

************************

IMPALA Digest 340

Topics covered in this issue include:

1) Re: Window tint

by Charlie311@aol.com

2) torque converter

by Scott Mueller <73145.1566@compuserve.com>

3) fuel pressure

by Scott Mueller <73145.1566@compuserve.com>

4) brake caliper detail

by Scott Mueller <73145.1566@compuserve.com>

5) tshirts

by Rock528@aol.com

6) CROW BOIL '96

by Rock528@aol.com

7)

by Rock528@aol.com

8) Re: Super Chevy Cost

by Rock528@aol.com

9) IMPALA SS FOR SALE

by TODD MCMURRY <73543.3717@compuserve.com>

10) Fleetwood cal

by "C. K. Yee" <yu135008@yorku.ca>

11) Re: IMPALA digest 339

by JGreen5275@aol.com

12) 86 Vette Converter

by dhflores@iwl.net (Debbie/Henry Flores)

13) New Monte attack

by Loren Peterson <equ_rino@cencom.net>

14) Re: IMPALA digest 339

by JGreen5275@aol.com

15) Chevrolet address of interest

by Loren Peterson <equ_rino@cencom.net>

16) New member

by Fulminata9@aol.com

17) RE: Arlington info (more general information)

by Matt Platts <mplat@airmail.net>

18) A little more information (please)

by Matt Platts <mplat@airmail.net>

----------------------------------------------------------------------

Date: Sat, 9 Mar 1996 10:53:00 PST

From: Charlie311@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Window tint

Message-ID: <960309135300_442134001@emout05.mail.aol.com>

I'm not sure if all states follow this, but the feds had a tinting rule

titled VESC-20 which limited tinting to rear side and rear windows, with a 35

% light transmissibility limit.  The windshield and front side windows can't

be tinted, period.  That's the law in my state. Other states might be more

lax, with a light transmissibility of 65 % for the front side windows.

Maybe you have seen others driving with tinted windows, but it's just a

matter of time before they get bagged.  I doubt that the officer made it up

once he stopped you.  You probably drove right by him and he saw the tinting.

It might seem unfair, but some police won't bother with enforcing tinting

laws, while others will make it seem like a felony.  As far as your tint

installer--it wouldn't surprise me if he slipped you a disclaimer in the

contract.  "For off-road use only"  or "Check local laws before use."  Good

luck.

Charlie

------------------------------

Date: Sat, 9 Mar 1996 10:58:42 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: torque converter

Message-ID: <960309185841_73145.1566_FHD58-1@CompuServe.COM>

Henry,

>I looked into the 1986 Torque Converter/Trango shift kit combination. Is

the price of $275 converter without exchange and a $175 R&R on the install

in the ballpark or is this expensive? This shop builds torque converters.

They are about $100 too high on the converter! The converter should cost you

about $145 plus a $30 core charge if you decide to keep your original.

>Is there another approach to obtaining one of these converters a bit more

inexpensively?

The $145 price plus $30 core (only if you keep your original) is available

at any local GM dealer!

>I called the local dealer on the part number that you kindly provided and

the service/part rep had no idea of the torque converter numbers. He needed

the specific transmission type of that particular Corvette.  Idiot or valid?

Eeeediot! The number is all they need. I can't fathom why there are so many

idiots in this industry!!

>Will the use of a different converter effect the computer control of the

tranny in any way, shape or form? This question was posed by the converter

shop not the install shop.  What are your valued opions?

Computer control will not be affected.

>Lastly, and most importantly, any drawbacks to this coverter?

Slighly greater heat output due to the higher stall. The Impala has TWO

transmission coolers with I dare say more cooling capacity than ANY other

car on market today, so don't worry about the extra heat. You might see a

quarter of a mpg less mileage around town, no difference whatsoever on the

highway. Scott.

------------------------------

Date: Sat, 9 Mar 1996 12:00:17 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: fuel pressure

Message-ID: <960309200017_73145.1566_FHD61-3@CompuServe.COM>

Mike,

>Does anyone have the correct fuel pressure specs. for the B-body LT1?

Yep, the spec is as follows:

Ignition ON, engine NOT running: 41-47 psi

Engine running at idle: 3-10 less than above

When you turn on the ignition, the fuel pump will run for 2 seconds and then

stop. You may have to cycle the ignition off and then back on to get it to

run for another 2 seconds to build maximum pressure (41-47 psi). With the

engine not running, there will be no vacuum on the pressure regulator, and

this indicate the maximum fuel pressure.

When you start the engine, vacuum is present at the regulator, causing it to

drop the pressure by 3 to 10 psi from what you read in the first test. The

actual amount of pressure drop depends on ambient pressure and system

tolerances.

>My next purchase may be an adjustable fuel regulator and I checked my stock

unit and it only had 38 psi at idle.

An adjustable regulator will not fix this, as this is not a problem! In fact

38 psi is exactly within the range of what should be expected at idle.

>All the test I've read show stock pressure to be some where between 42 and

45 psi. At low vacuum or full throttle "burp" the max pressure was 46 psi.

Maximum pressure is only achieved with no vacuum at the regulator. If you

are idling the engine, pull the vacuum hose to the regulator and you will

see the pressure immediately jump to the maximum. The test as described in

the service manual has you run the fuel pump with the ignition on but the

engine not running, hence there is no vacuum at the regulator and the

pressure read should be the maximum.

My system reads exactly 46 psi maximum, same as you! In other words, your

regulator is working perfectly. An adjustable regulator will simply cause

you to run either too rich or too lean, and when in closed loop the PCM will

alter the injector pulse width to compensate. In other words, you don't need

or want an adjustable regulator, unless you have made many mods to the

engine and the PCM is unable to compensate using the stock pressure.

BTW, I found out that the '97-'98 F/Y-car LS1 (Generation III smallblock)

runs an astounding 66 psi of fuel pressure! Scott.

------------------------------

Date: Sat, 9 Mar 1996 12:00:09 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: brake caliper detail

Message-ID: <960309200008_73145.1566_FHD61-2@CompuServe.COM>

Jeff,

>Polishing the rear calipers sounds good but what to do with the fronts. One

of the mags awhile back did a brake detail with the GT rotors and they

painted the f&r calipers black. My idea was to paint the front with a

aluminized type paint so it would match the back ones. Heres the question,

what kind of paint? It would have to be a Hi-temp type paint, I would think,

to stand up. Does VHT or anyone else make such a paint, in a color that

would be close to the unpolished rear?

That is my idea exactly! I like a matching look, so I am painting the front

calipers with high temperature stainless steel paint, which matches pretty

well with the dull (but clean!) aluminum rear calipers. I am using high

temperature (1300 degrees plus) stainless steel paint, which I also have on

my exhaust manifolds. The stuff seems to work great and can certainly take

the milder temperatures experienced on the calipers. You can get it through

Whitney, Eastwood, or several other suppliers. It brushes on or can be

sprayed, and is actually cured by heat. Scott.

------------------------------

Date: Sat, 9 Mar 1996 13:45:43 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: tshirts

Message-ID: <960309164542_442237575@emout06.mail.aol.com>

before you waste a good shirt on framing, I am checking on printing the

screen print on a piece of felt.  They did this when the shirts first came

out.  I may have a hundred printed for the shows.  They should be less than

the shirt.  Troy

------------------------------

Date: Sat, 9 Mar 1996 13:45:40 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: CROW BOIL '96

Message-ID: <960309164540_442237550@mail06.mail.aol.com>

In a message dated 96-03-09 12:34:36 EST, you write:

>CROWS, you guys are killing me.

Our hangar neighbors, NASCAR, hangs this thing in their aircraft hangar to

keep the birds out.  It's a big yellow beachball with two huge googly eyes.

The birds think it's an owl on steroids and they stay out of the hangar.

So, get yourself a 24" piece of cardboard and paint it yellow.  Make two

crazy eyes on it and add little pecker.  When you get out of the car place it

on the roof.  No more crows.

I am not responsible if you attract larger birds though..   Rock

------------------------------

Date: Sat, 9 Mar 1996 13:45:42 PST

From: Rock528@aol.com

To: <impala@east.xsis.xerox.com>

Message-ID: <199603092145.QAA26244@emout05.mail.aol.com>

d 96-03-09 12:34:36 EST, you write:

> Second, after going to Kellers for Burgers and beer, We could head down to

>Lower Greenville and cruise the strip.

Brett, congrats on your new cruz director's position.  Would you handle the

Saturday night cruise options.

Thanks, Troy

------------------------------

Date: Sat, 9 Mar 1996 13:47:11 PST

From: Rock528@aol.com

To: earndt@airmail.net, impala@east.xsis.xerox.com

Subject: Re: Super Chevy Cost

Message-ID: <960309164710_442237608@emout09.mail.aol.com>

In a message dated 96-03-09 13:34:54 EST, you write:

>I read in the digest that a $35 fee is payable at the gate.  What does that

>include?

show your car, visit the vendor area, visit the commercial display area,

watch the drags for 2 days to include local draggers and jet cars.  Troy

Visit the Super Chevy Web site at http://www.wmoen.com/schevy/index.html    I

think they updated the address but it should get you there.

------------------------------

Date: Sat, 9 Mar 1996 14:12:23 PST

From: TODD MCMURRY <73543.3717@compuserve.com>

To: IMPALA SS DIGEST <impala@east.xsis.xerox.com>

Subject: IMPALA SS FOR SALE

Message-ID: <960309221223_73543.3717_FHD70-1@CompuServe.COM>

The dealership in my town has a 95 Black Cherry Impala ss for sale, The car has

12,000 miles on it and is in

very good condition. Supposedly one of the men at the dealership's father had

owned it and he trades every

year.  they are asking  $21.500 if anyone is interested give me a call and I'll

tell you how to get in touch with

them

TODD MCMURRY

806-273-3781   WORK

806-273-6102   HOME

------------------------------

Date: Sat, 9 Mar 1996 15:04:15 PST

From: "C. K. Yee" <yu135008@yorku.ca>

To: impala@east.xsis.xerox.com

Subject: Fleetwood cal

Message-ID: <Pine.SOL.3.91.960309180307.11823A@sunspot.ccs.yorku.ca>

Flashing the Fleetwood calibration into a B-car would result

in a DTC 74.  There is one wire running from the traction control

module to the PCM that is normally B+.  It is pulled to 0 volts

during a traction control event.  Since the B-cars lack this wire,

the input will always be at 0 volts; if the transmission is in

park and both these conditions are met for longer than

2 seconds, a DTC 74 will result.  The PCM will then inhibit TCC

operation and 4th gear if in hot mode.  Otherwise, the car should

still function.

Kevin

------------------------------

Date: Sat, 9 Mar 1996 15:59:13 PST

From: JGreen5275@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 339

Message-ID: <960309185912_241912628@emout08.mail.aol.com>

How many  people are going to fly to Arlington from Orlando? I must due to

time costraints class schedules etc.  Anyway I' flying on Delta on the 19th

at 7:15am and arriving in Dallas at 8:53 am on flight 0691 I thought if

anyone else was going I have an upgrade certificate in first class if anyone

is interested let me know. I also just got off the phone with the Fairfield

Inn and they have plenty of rooms available for the special club rate fo

59.95, not bad since there regular rate for this time of year is 89.95.  I've

stayed at Fairfields before most are no more than a year old and have inside

pools and whirlpools and they also give free cont. breakfast every morning.

Its not the Sheraton but it is owned by Marriot, so there accomodations

definately are clean and comfortable as per Marriot standards, so I really

think its the best bang for the buck. Oh by the way I'm coming back on Delta

flight 0586 the 21st at 5:20 pm and arriving in Orlando at 10:53 there is an

earlier flight but I didn't want to miss any fo the show. Although that is

subject to change. Again if anyone is interested in the first class upgrade

let me know.

------------------------------

Date: Sat, 9 Mar 1996 16:12:28 PST

From: dhflores@iwl.net (Debbie/Henry Flores)

To: impala@east.xsis.xerox.com

Subject: 86 Vette Converter

Message-ID: <96Mar9.163010pst."14402(3)"@alpha.xerox.com>

Are their any Impala brothers who have installed the 1986 Corvette Torque

Converter?  I need your valuabe assistance and expertise as I am planning to

have this done this week.  What are your consulting fees for your feedback?

Henry in Houston

Desperate for Converter Assistance!!!!!!!!!!!!!

------------------------------

Date: Sat, 9 Mar 1996 16:44:57 PST

From: Loren Peterson <equ_rino@cencom.net>

To: ss <impala@east.xsis.xerox.com>

Subject: New Monte attack

Message-ID: <31422609.768B@ns.cencom.net>

SSers

Went over to Camanche IA today to check our a wrecking yard that I used

to frequent.  He always had real cool new stuff.  He bought up rental

fleet wrecks and Ins. Co. totals, but alas he has relocated to Arkansas!

Too bad.  Anyway, coming home on I74, I was just getting off when out of

nowhere there appears a brand new bright red Monte Carlo Z34 and he is

all over my bumper......so I just poke along till we get off the ramp

and he comes screaming around.....he never made it.  I guess they are

supposed to be quick, but the Impala did her leap routine and we were

buku yards ahead.  I then slowed to get off at the next exit when the

Z34 went by with two teenage boys grinning from ear to ear.  So was I.

Loren DCM96

------------------------------

Date: Sat, 9 Mar 1996 16:51:00 PST

From: JGreen5275@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 339

Message-ID: <960309195059_164644942@emout09.mail.aol.com>

I just read where one of our reliable, and  supportive club members actually

recommended that we all go to Jeg's and buy shirts, and hats, I thought Troy

had these items for sale. Hey Troy did you run out merchandise? Anyway it

sure would be nice if we would all support "OUR" club.  Speaking of shirts

and hats Troy I would like 2 hats and 3 of those black shirts XL please.

------------------------------

Date: Sat, 9 Mar 1996 17:55:39 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Chevrolet address of interest

Message-ID: <3142369B.3F6C@ns.cencom.net>

Ya'll might like checking out this address:

http://www.chevrolet.com/new/a402.htm

Loren DCM96 SPE

------------------------------

Date: Sat, 9 Mar 1996 18:51:24 PST

From: Fulminata9@aol.com

To: impala@east.xsis.xerox.com

Subject: New member

Message-ID: <960309215123_442414959@emout10.mail.aol.com>

I've been reading the list's for about a week or so and thought it was about

time to say hi and introduce myself. I've got a Black 96, built in September

and picked up on October 15th. I'm a Manufacturer's Rep/Tech Support for

several aftermarket companies (Edelbrock,MSD,NOS,Hypertech.etc.....) . I

cover the So-Cal area and I drive my SS

everywhere I go (18,500 miles since October). At this rate I'll have to

restore it in 2 more years :)

I hand-made the air inlet pipe and used a K&N cone filter behind the D/S

headlamp. I'll be adding the other parts such as intake,throttle

body,headers,cat-back as the parts become available from my manufacturers. I

was going to lower the car but as I see more and more lowered SS's the large

rounded top makes them look out of proportion to the rest of the body when

lowerd. (**Disclaimer Warning: The previous statement is just an opinion and

doesn't reflect the opinions of this station or its affiliates**) Anyway, the

car does look slightly lowered with several cases of catalogs in the trunk.

On a final note, I just wanted to thank all the people who put the Web sites

and this club together. The $20.00 check I mailed out today doesn't seem like

much compared to all the work that goes into maintaining all of this.

(p.s. just got my first newsletter. looks great !)

John Alger

------------------------------

Date: Fri, 8 Mar 1996 17:00:56 PST

From: Matt Platts <mplat@airmail.net>

To: "'Impala'" <impala@east.xsis.xerox.com>,

Subject: RE: Arlington info (more general information)

Message-ID: <01BB0DA0.9BFE8DE0@dal07-03.ppp.iadfw.net>

Ray, thanks for replying.  As for answers to your questions... here goes:

-Airport to Fairfield/Arlington: Approx  9 miles (straight shot down HWY-360)

-Cab fare: I've been told it's in the $10-$15 range (never done it)

Another alternative to a cab is a thing called the "Super Shuttle" (just remember SS <g>), they run just about everywhere and are generally cheaper than a cab.  Check them out if you like.

Depending on your arrival time, I'm sure we could arrange someone to pick you up and save the extra expense of a cab or other.  Let me know when you will be arriving, as I plan on taking the day off and might be able to come and get you.  If anyone else needs this service, let me know.  I don't have any problems with a couple of trips to the airport.

Now, it looks like the place of choice is still going to be the Fairfield.  I called there tonight and asked about rates (to confirm for everyone else), I still got the run-around but he finally acknowledged that there was a group of rooms blocked off for the club.  It is extremely important to mention the "car club" (as the clerk put it).  I would probably tell them I was with the "Impala Car Club" first, then if you don't have any luck tell them NAISSO.  It took me four times to make the guy understand that I was saying "national"...  he wasn't that good with english.

So far I have recieved 12 RSVP's, thanks for the good response.  I will look forward to hearing from the rest of you that are planning on the trip.  A few other question (for which I don't have answers) have come up, they are:

-How much is individual admission for the Super Chevy Show?

-How much extra (if any) if I plan on putting my car in the BFGoodrich tent?

-If I park my car in the BFG tent, will it be judged for a contest?

If anyone has answers to these questions, please post.  I will research any unanswered ones and get back to the list.

Anxiously awaiting...

-matt

------------------------------

Date: Fri, 8 Mar 1996 18:19:04 PST

From: Matt Platts <mplat@airmail.net>

To: "'jerryp@STL-06LSSC.ARMY.MIL'" <jerryp@stl-06lssc.army.mil>,

Cc: "'Impala'" <impala@east.xsis.xerox.com>

Subject: A little more information (please)

Message-ID: <01BB0DA0.ABC714E0@dal07-03.ppp.iadfw.net>

I would like a little more information to help track the weekends activities.  This isn't meant to invade your privacy, but I would like to get an idea on when people are coming to town and when they are leaving.  This will help plan things like a possible cookout (at a list-member's house) and evening activities while everyone is in town.

Also, those of you on the list that have not yet replied to my RSVP request (see Digest 332), please include this information as well.

I would also like to know:

Where are you coming from (what city/state do you live in)?

When do you plan to arrive in the DFW area?

When do you plan to leave the DFW area?

Will you be interrested in group activities (cookout, dinner, etc.)?

Thanks, and this information will be kept confidential (I will ONLY distribute the totals for statistical purposes).

-Matt Platts

mplat@airmail.net

Home: (817)472-6076

Pager: (817)651-2330 (put *99 after your number, I'll know you are an SS'er)

------------------------------

End of IMPALA Digest 340

************************

IMPALA Digest 341

Topics covered in this issue include:

1) Window Tint, Indiana

by Joe Palmer <laser@iquest.net>

2) Security

by Franklin Poole <franklin@intersurf.com>

3) Warped this guy it!

by Franklin Poole <franklin@intersurf.com>

4) My Monte

by Mikado95@aol.com

5) Window Tinting

by Franklin Poole <franklin@intersurf.com>

6) Stainless steel paint

by Franklin Poole <franklin@intersurf.com>

7) Stainless Steel Paint

by Franklin Poole <franklin@intersurf.com>

8) RE:  New Member John Alger

by ZTMP50A@prodigy.com ( DAVID NMN)

9) A little bit about breaks...

by glen.novitsky@nashville.com

10) The lights are on but......

by "Jeff Gatcha" <foxhound@txdirect.net>

11) 96 Impala Prices

by "David P. Fenzl" <dfnzl@mail.erols.com>

12) Re: Tee shirts and Texas

by Curt Platteborze <75511.1220@compuserve.com>

13) Re: Corvette torque converter

by Curt Platteborze <75511.1220@compuserve.com>

14) Re: Huh?

by "Thomas L. Pierce" <75027.2421@compuserve.com>

15) Re: rear calipers!

by Robert Cheek <rcheek@one.net>

16) Re: Caliper Question

by Robert Cheek <rcheek@one.net>

----------------------------------------------------------------------

Date: Sat, 9 Mar 1996 19:43:54 PST

From: Joe Palmer <laser@iquest.net>

To: Impala Page <impala@east.xsis.xerox.com>

Subject: Window Tint, Indiana

Message-ID: <31424FFA.67B0@iquest.net>

This a copy of the Indiana Code on window tinting. Tint is very common here even in the darker

shades. In fact a lot of the Police here have the dark tint.

------------------------------------------------------------------------

Indiana Code 9-19-19-4

Enacted 1991

Amended 1995

IC 9-19-19-4 Sec. 4. (a) This section does not apply to a manufacturer's tinting or glazing of

motor vehicle windows or windshields that is otherwise in compliance with or permitted by

FMVSS205 as promulgated in 49 CFR 571.205. Proof from the manufacturer, supplier, or installer

that the tinting or glazing is in compliance with or permitted by FMVSS205 must be carried in the

vehicle.

(b)	This section does not apply to the driver of a vehicle:

(1) that is owned by an individual required for medical reasons to be shielded from the direct

rays of the sun; or   (2) in which an individual required for medical reasons to be shielded from

the direct rays of the sun is a habitual passenger.  The medical reasons must be attested to by a

physician licensed to practice in Indiana, and the physician's certification of that condition

must be carried in the vehicle. The physician's certificate must be renewed annually.

(c)	A person may not drive a motor vehicle that has a:

(1) windshield;

(2) side wing;

(3) side window that is part of a front door; or   (4) rear back window; that is covered by or

treated with sunscreening material or is tinted to the extent or manufactured in a way that the

occupants of the vehicle cannot be easily identified or recognized through that window from

outside the vehicle.

However, it is a defense if the sunscreening material applied to those windows has a total solar

reflectance of visible light of not more than twenty-five percent (25%) as measured on the

nonfilm side and light transmittance of at least thirty percent (30%) in the visible light range.

As added by P.L.2-1991, SEC.7. Amended by P.L.128-1995, SEC.1.

---------------------------------------------------------------------------

------------------------------

Date: Sat, 9 Mar 1996 20:09:44 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Cc: Scott Mueller <73145.1566@compuserve.com>

Subject: Security

Message-ID: <31425608.3AC1@intersurf.com>

>>I hope more than 50 will come. If security for our cars is a concern we can

>>hire local deputies for a few dollars each if necessary....

>I am all for that! I for one would love to see a guard on duty by the

>Impalas all night. Capitol punishment for door dings! Heck if we all chipped

>in an additional $5 to $10 that should cover the security, right? Scott.

The local sheriff's deputies  (Baton Rouge)work for $12-15 per hour.  I

imagine it's the same or thereabouts in Dallas area.  I suggest one of our

brethren in BLUE call whatever county Arlington is and check on the facts.

Matt Plat here's your chance to do something really cool!

I'm normally not overly concerned about shamu, but parked with 50 just alike

is bound to draw all kinds of unwanted attention at 3:00am when I'm chillin'

with Scott M and Troy W over a Texas Tea!

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Sat, 9 Mar 1996 20:17:05 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Cc: Troy Wilrick <Rock528@aol.com>,

Subject: Warped this guy it!

Message-ID: <314257C1.72A0@intersurf.com>

>this is me if you are wondering:

>Troy C. Willrick

>34 yoa

>single - looking for a girl with a black ss for my collection

>(measurements desired, 9C1-3.42-B19)

Impala fever and that incessant chopper drone have finally,

completely, destroyed poor Troy's mind!  We can only hope that a trip

to the motherland of Arlington, and maybe some of my killer crawfish

will help to heal his mind.<BBBBBBBG>

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Sat, 9 Mar 1996 20:35:55 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: My Monte

Message-ID: <960309233554_346585592@mail06.mail.aol.com>

>new bright red Monte Carlo Z34 and he is

>all over my bumper......so I just poke along till we get off the ramp

>and he comes screaming around.....he never made it.  I guess they are

>supposed to be quick, but the Impala did her leap routine and we were

>buku yards ahead.  I then slowed to get off at the next exit when the

>Z34 went by with two teenage boys grinning from ear to ear.  So was I.

>Loren DCM96

I have a Torch Red 1995 Monte Carlo Z34, and they are pretty quick (0-60 in

7.6, top speed 140), but they are no match for ny father's Imp!   I have beat

a couple of Mustangs who thought smoking wheels were more important than the

actual race, and several Lexi (?).....    If anyone has any suggestions on

how to get my Z34 closer to Impala performance (besides dropping a LT1 in),

e-mail me!

Randy

------------------------------

Date: Sat, 9 Mar 1996 20:41:09 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Window Tinting

Message-ID: <31425D65.17FA@intersurf.com>

Regarding window tinting in Louisiana the rule is this: 50% light

transmission on the front windows, no limit on rear glass or rear side

windows.  Only a dumb sh@t would tint the back glass really dark but

many do....  The problem is this: you go to the local car store and the

carton says that the film they are selling has a light transmission of

say 65%.  The tinted windows on my car are already (I think) 75%.  No go

joe!  A local (actually several) shop will do a tint for front windows

that meets VSC-20 and 50% rule. Cost is between $149 and $180 depending

on discounts and dealer and quality of work.  Mine is soon but not yet.

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Sat, 9 Mar 1996 20:44:58 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Subject: Stainless steel paint

Message-ID: <31425E4A.361E@intersurf.com>

>That is my idea exactly! I like a matching look, so I am painting the front

>calipers with high temperature stainless steel paint, which matches pretty

>well with the dull (but clean!) aluminum rear calipers. I am using high

>temperature (1300 degrees plus) stainless steel paint, which I also have on

>my exhaust manifolds. The stuff seems to work great and can certainly take

>the milder temperatures experienced on the calipers. You can get it through

>Whitney, Eastwood, or several other suppliers. It brushes on or can be

>sprayed, and is actually cured by heat. Scott.

where'd you get stainless steel paint?

I like this idea but have never seen such.

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Sat, 9 Mar 1996 20:46:21 PST

From: Franklin Poole <franklin@intersurf.com>

To: Impala List Server <impala@east.xsis.xerox.com>

Cc: Scott Mueller <73145.1566@compuserve.com>

Subject: Stainless Steel Paint

Message-ID: <31425E9D.666A@intersurf.com>

Never mind. Hand is quicker than eye!<g>  Didn't read/comprehend all of

post on listserver..................................

--

Franklin Poole, Jr.

Praecedo, Inc.

====================================================

It's not how fast you go...it's how you go fast!! $$$$$$

------------------------------

Date: Fri, 8 Mar 1996 20:43:46 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: RE:  New Member John Alger

Message-ID: <097.03183301.ZTMP50A@prodigy.com>

Hello John!  Welcome aboard.  I'm glad to see another So Cal SS owner.

It seems that most of the members all reside on the left coast  <G>.

I'd be interested in hearing how you did the custom air inlet pipe,

as I want to change and delete the baffle.

Thanks again!

Dave in So Cal..'96 Black SS    (BTW, my Suburban has the plate "ME

BEAST", do you all think I should transfer it to the SS?  <BG>)

One last thing to add.  I checked the ball joints on my car, and they

WERE greased at the factory (or dealer.  Do you think GM finally got

on??  My cars build date was 12/4/95.)

------------------------------

Date: Sat, 9 Mar 1996 13:06:44 PST

From: glen.novitsky@nashville.com

To: impala@east.xsis.xerox.com

Subject: A little bit about breaks...

Message-ID: <96Mar9.215019pst."15086(11)"@alpha.xerox.com>

Had to replace my front brakes on my 95 LT1 Caprice equiped with HD towing

package with only 14,500 miles!!

Anyway, after purchacing a set of Performance Friction brakes all I got was

a bunch of Vibrating/Grinding and squeals. I was pissed, so I went to

another auto parts store. They had listed 10 different types!!

So a trip to Mr. Goodwrench was in store.  He had about 6 different sets

for about 12 different brake setups!!  After looking at the option code

sticker, it was determined that my car had the JM4 setup... yes the 9C6 HD

taxi brakes!!!  The set was $50.00 but it included bushings.  The other

sets included  Light Duty, Light Duty Taxi, HD Taxi, 2-3 sets of Police

brakes from med to HD, and Impala brakes.....and you DON'T want to know

what a set of those cost!!!........O.K., I'll tell you,  $117!!!!!!!! Makes

you kind-of want to not stop so much!!!

It was determined that Auto Zone sold the wrong set, thus the vibrating.

They cheerfully refunded my $27.

Just a little info before you wear them out!!!

-Glen

Back stopping smoothly, again!!!!!!!!!!!!!!

------------------------------

Date: Sun, 10 Mar 1996 00:38:26 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: The lights are on but......

Message-ID: <199603100837.CAA00177@legend.txdirect.net>

Hey guys,  This doesnt apply to Impalas but it is worth a good laugh.

Here on this list are some of the most "car-smart" people Ive met.

Then, on the other side of the world are these guys.....Got this from

the Saturn Group.....-Jeff

>Ok.  So I had the unfortunate luck of having someone drive my 96 SL2

>with the Parking Brake engaged one night.  About 15 mile trip.

>Smelled a little burned the next day, but appears to operate ok.

>

>Any other damage that might have occurred?  And does that affect only

>the rear brakes?  I did notice after glancing through the wheels that

>the front rotors are shiny, but the rear are not.  Applying the brakes

>do not cause any noise or shaking from the rear.

>

>Thanks for the advice.

-----------

In the year of darkness, 2029, the rulers of this planet

devised the ultimate plan. They would reshape the future by

changing the past. The plan required something that felt

No pity, No pain, No fear. Something Unstoppable.

They created The TERMINATOR...

Jeff Gatcha

foxhound@txdirect.net

------------------------------

Date: Sat, 9 Mar 1996 18:35:34 PST

From: "David P. Fenzl" <dfnzl@mail.erols.com>

To: "'Impala Digest'" <impala@east.xsis.xerox.com>

Subject: 96 Impala Prices

Message-ID: <01BB0E51.13C15DC0@ppp97.erols.com>

Greetings, Gang!  There have been a few recent posts concerning prices of

new Impalas (discounts, full sticker, buyers services, etc.)  Here is my

personal experience with my '96, so those of you who got a better deal can

chuckle, those of you who paid more can grind their teeth, and so those of

you who haven't bought one yet can have another data point to use when

negotiating with your purchasing agent.   I ordered my car on 16 Dec 95.  I

took delivery on 27 Feb 96.  The sticker prices below were taken straight

from the window sticker of my car.  The "invoice" prices are from a

computer printout which the dealer provided and must be taken with a grain

of salt.

item                             sticker                  invoice

1BL19                        $24,405.00           $22,330.58

1SE  equipment grp           490.00                 421.40

K05 engine block heater       20.00                  17.20

A23 heat reflect. windshld     52.00                  42.72

UN0 CD player                   155.00                133.30

destination charge            590.00                590.00

"adjustment" (BS??!!)             0.00                251.22

------------------------------------------------------------------------

----

total                            $25,712.00          $23,770.42

I paid $24,400, which is $1,312 less than sticker, or about $630 over

invoice (or about $880 over invoice without the bogus-sounding

"adjustment").

Dave F., stock green-gray;

dfnzl@erols.com

------------------------------

Date: Sun, 10 Mar 1996 05:02:17 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Tee shirts and Texas

Message-ID: <960310130216_75511.1220_GHJ42-1@CompuServe.COM>

Just to clear things up. Originally I posted the news about receiving the Jegs

catalog and seeing the Cap and Tee shirt.

I have been on the digest for some time now and only three weeks ago found out

there was a Impala newsletter. I sent in my dues 20.00 <G> happily and joined as

soon as I found out about it.

I was not aware there were gift items in the newsletter and have not received a

copy yet to peruse. If I had known about the Tee shirt and Cap offered by the

newsletter I would not have posted the Jegs information and would have ordered

them thru the club instead of Jegs. Please support our club and not order from

Jegs!

I am also disappointed in the low number of Impala's driving to Texas. I pushed

myself for the 1800 mile drive thinking there would be like 50 to 70 Impala's at

this event. The more cars there the more it would help me with my obsession<G>.

So please reconsider your decision about not making the trip!

The more cars there the more fun we will all have. We can also experience the

energy and force of this great club and incredible car.

So please if you're not going to Dallas reconsider.

Thanks,

Curt P. MA

------------------------------

Date: Sun, 10 Mar 1996 05:02:22 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Corvette torque converter

Message-ID: <960310130221_75511.1220_GHJ42-3@CompuServe.COM>

Henry,

I installed the Vette converter and Trans Go shift kit about two weeks ago. This

is a must for any Impala owner. The converter transfers a lot more power to the

rear end and the shift kit sharpens up the shifts and pushes you back in your

seat when you shift.

It feels like a totally different car after this mod. I hope you have changed

out your gears to 3.42 or higher. This combination of all three mods makes a

huge difference.

Below is a copy of a my post after I installed the mods.

>I received the torque converter today and it was part number 24201203 and was a

>remanufactured unit. I was surprised at the cost as it was lower than I

thought. >$146.25 plus a $30.00 core charge which I will eat and keep the

original in a box >with my other collection of Impala parts.

>Installed the Trans Go kit and vette converter last Friday night. One word

"POWER".

>This setup is amazing with stiff hard shifts and 1-2 barking till you want it

to stop. >"Even with my impaired #8 cylinder."

>The shift kit took a bit more time than I thought and two of the stock

components you >had to reinstall backwards. I kept looking at the instructions

with disbelief and >called Trans Go's tech line to confirm this possible diagram

error.  Yes you do >mount them upside down from stock. I used all three washers

for the hardest shifts.

>We removed the cats for easier access to the trany which made the whole job go

>fast. I'm not sure I would have wanted to do any of this without a lift. The

Trans Go >video sure makes the job for the shift kit look easy with just a

creeper under the car.

Best of luck,

Curt P.

------------------------------

Date: Sun, 10 Mar 1996 06:49:40 PST

From: "Thomas L. Pierce" <75027.2421@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Huh?

Message-ID: <960310144940_75027.2421_GHJ108-1@CompuServe.COM>

>>It took me four times to make the guy understand that I was saying

"national"...

So how do you pronounce NAISSO anyway ??? "nye-so" ? "nie-so" ? "nah-iss-so" ?

<G>

Tom

------------------------------

Date: Sun, 10 Mar 1996 05:25:02 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Re: rear calipers!

Message-ID: <1.5.4b11.32.19960310132502.00685570@mail.one.net>

>OK, fess up.  After you read it, how many of you went out and looked at them

>and thought it would look kind of trick??   Troy

<slowly and sheepishly raising hand> OK, OK, I did. I this a practical

mod to make if your Impala is driven in not-so-nice weather? Or would

I spend every weekend on polishing my brake calipers with metal polish?

(I'm fairly sure that this would mean my wife would have me committed...)

OTOH, I just got a Sears Auto Restoration mini catalog. Awesome stuff

in there! Request one if you don't have one. They've got some Eastwood

paints in there that might be sort of cool: AlumaBlast (stock aluminum

look), Silver Cad (silver Cadmium plating), and Reflective Aluminum.

They also sell Nyalic clear coat to put over polished metals. Anyway,

could I paint the fronts and rears to match with one of these, or does

aluminum expand and contract too much to do it? I'd like to have all

four of them match.

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Sun, 10 Mar 1996 05:24:49 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Re: Caliper Question

Message-ID: <1.5.4b11.32.19960310132449.00689adc@mail.one.net>

>That started in '94! In fact ALL Impalas get painted (black) front calipers,

>and both the front and rear brake rotors are specially coated with a

>corrosion resistant material.

OK, so maybe "definitely not" was a little strong in wording <G>. It turns

out, once I rubbed all of the slush and other crap off of them and got them

in the daylight, that the rears do look a bit silver and maybe the fronts

were black, too.

>BTW, the rear calipers are not painted silver, they are ALUMINUM! All

>Impalas and 9C1 cars get the aluminum (unpainted) rear disc brake calipers.

Why did they do this? Sure wasn't the weight savings. ;-) Why wouldn't you

want matching front and rears?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

End of IMPALA Digest 341

************************

IMPALA Digest 342

Topics covered in this issue include:

1) Supercharged SS

by CS_MWR@hal.lamar.edu

2) I second the E-motion

by Loren Peterson <equ_rino@cencom.net>

3) F-car calibration

by Scott Mueller <73145.1566@compuserve.com>

4) Security

by Scott Mueller <73145.1566@compuserve.com>

5) BRAKE CALIPERS

by jeffd@accessus.net (Jeff Dawson)

6) New member

by CFHjr@prodigy.com (MR CARL F HAEUSSLER, JR.)

7) Good S. Florida Dealers?

by LRoshak@aol.com

8) Digest 339

by Stewart McLellan <stewartm@magi.com>

9) Re: show questions

by Rock528@aol.com

10) Re: eastwood supplies

by Rock528@aol.com

11) Bless me father for I have sinned..

by Rock528@aol.com

12)

by Rock528@aol.com

13) Re: 1997 Impala SS??

by KEN52455@aol.com

14) control arms

by Scott Mueller <73145.1566@compuserve.com>

15) Re: IMPALA digest 340

by LFriend121@aol.com

16) Re: fuel pressure

by Robert Cheek <rcheek@one.net>

17) Rattling Doors

by Robert Cheek <rcheek@one.net>

----------------------------------------------------------------------

Date: Sun, 10 Mar 1996 08:54:35 PST

From: CS_MWR@hal.lamar.edu

To: impala@east.xsis.xerox.com

Subject: Supercharged SS

Message-ID: <960310105435.1cc3@HAL.LAMAR.EDU>

I took my ATI 9psi intercooled '94 Impala to the track yesterday, and

am very pleased with the results.  My 3 best runs:

60'   330   660  MPH  1000    1/4  MPH

---------------------------------------

1.985 5.550 8.468 84.15 11.030 13.194 103.83

2.078 5.669 8.598 85.32 11.123 13.270 104.48

2.008 5.588 8.519 85.85 11.039 13.196 103.60

These times are on the factory tires!  I also still have the 3.08 gear.

My next move to buy a pair of BFG Drag Radials and get into the 12's.

See you in Arlington!

Jim Fort

Houston, TX

------------------------------

Date: Sun, 10 Mar 1996 10:51:30 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: I second the E-motion

Message-ID: <314324B2.577A@ns.cencom.net>

I had the same experience as Curt P.  I have sent in my $20., but have

not received the Newsletter, therefor never knew of the shirts and hats

being available.  So, when I got word via the list yesterday, I ordered

two of ea. from Jeggs.  In the future, I will support the CLUB whenever

and wherever possible.  Hoping to get my 1st Newletter this

week......pleeze...Troy.

Come on folks, sign on and come to Arlington.  It will be an experience

you'll never forget.  I helped put on the 1st Taurus SHO Convention in

Peoria, and we had a blast.  The only thing I miss about the SHO is the

Club meetings and fun I had with the members.  Sure wish I could get

some interest around here in starting a SS Impala Club.  Guess I'm gonna

have to drive ALL the way up to Chicago once in a while if you guys up

there have a club...any info on this?

Loren DCM96 SPEquipped

Texas or bust

Lets get sideways

P.S.  Hey there Monte owner, get a K+N and work on the air flow for the

intake, and try some free breathing mufflers.  Have you raced a SHO, bet

you killed him!?

------------------------------

Date: Sun, 10 Mar 1996 11:05:59 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: F-car calibration

Message-ID: <960310190559_73145.1566_FHD46-6@CompuServe.COM>

Franklin,

>You originally mentioned some problems/concerns with the Fbody programming

on the Impala.  Are you now recommending this, assuming what?

I don't know if I'd recommend it, but I sure like it! <g>

>Do I need the Fbody MAF?, and what else so I don't dig myself a hole....

You definitely need the F-body MAF, but that's about it as far as other

parts go. Unless you have a 3.42 gear in your Impala (and you flash it with

the 3.23 automatic F-car calibration), your speedo will be off. The 3.23

F-car calibration is almost EXACTLY correct for an Impala with 3.42 gears

because of the tire differences. In other words, the speedo will be accurate

to within about one third of one mph at 100 actual speed!

Not only does the F-car calibration work perfectly with a 3.42 geared

Impala, but you also get the optional performance shift button, which you

can now hook up and it will be fully functional.

There are two minor problems, however. The F-car auto trans shift points are

a little too high for the Impala, and I think they should be reduced back to

where the stock Impala shift points are. Also I want the electric fans to

operate at a lower temperature, especially with my lower temp (160 degree)

thermostat installed. The F-car calibration with the shift points and fan

temps changed would be about perfect! Scott.

------------------------------

Date: Sun, 10 Mar 1996 11:05:52 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Security

Message-ID: <960310190552_73145.1566_FHD46-5@CompuServe.COM>

Franklin,

>I'm normally not overly concerned about shamu, but parked with 50 just

alike is bound to draw all kinds of unwanted attention at 3:00am when I'm

chillin' with Scott M and Troy W over a Texas Tea!

Yeah, then I would not have to get up every 5 minutes to look out the

window! <g> Scott.

------------------------------

Date: Sun, 10 Mar 1996 10:57:19 PST

From: jeffd@accessus.net (Jeff Dawson)

To: impala@east.xsis.xerox.com

Cc: rcheek@one.net

Subject: BRAKE CALIPERS

Message-ID: <199603101857.MAA13869@mtvernon1.accessus.net>

Ok this will start some controversy but I'll dive in anyway. As I understand

the brakes on the Impala's, the back caliper is a 'vette unit modified for

the mini-drum parking brake. The back, is off of the front of a late 80's

early 90's 'vette. This does not mean they will exchange as the mod for the

parking makes this impossible. The caprices had front disc brakes so they

"chevy" did not have to do anything weird on the fronts. I think this is why

we are stuck with iron front and alluminum back calipers. Unless of course

we want to pop $1300 for a matching set of Baer Claws. I have said my 2

cents worth now let the debate begin!

Jeff Dawson 95 DCM SS

-----Work is the curse of the drinking classes.-----Oscar Wilde

------------------------------

Date: Sat, 9 Mar 1996 09:07:10 PST

From: CFHjr@prodigy.com (MR CARL F HAEUSSLER, JR.)

To: impala@east.xsis.xerox.com

Subject: New member

Message-ID: <096.03203675.WJFG61A@prodigy.com>

Hi- My name is Carl Haeussler.  I am a new subscriber.  I recently

purchased a 1996 Green/Gray metallic Impala SS.  Love it!  I look

forward to sharing with you all.  I live in the south east region of

Pennsylvania (Bucks County).  I also own a 1975 Corvette Roadster

(show and occasional drive) and have been a member of the Corvette

Club of Delaware Valley since 1978.  Wonder if there will be an

Impala club in this area any time soon.  You may correspond with me

via E-mail on Prodigy at WJFG61A or on the net at CFHjr@prodigy.com

------------------------------

Date: Sun, 10 Mar 1996 11:52:47 PST

From: LRoshak@aol.com

To: impala@east.xsis.xerox.com

Subject: Good S. Florida Dealers?

Message-ID: <960310145244_347007417@mail04.mail.aol.com>

Hello All,

I have just cancelled my SS order!!

And placed a new order w/ Lee Chevy in CT and it saved me $800 (Thanks Kurt

John is very honest and wants to sell the car).  So I am in search of a good

dealer  in S. Florida to get the car delivered to: Any suggestions? I live in

Delray Beach just moved here from Arizona.

And what is the deal with these dealer preps!!!  My last new car  that I

bought in 91 had a $50 dealer fee now they are charging $250 to $300.  Is

this standard in FLA?

Thanks,

Order to be entered on Monday.

Larry (lroshak@aol.com)

------------------------------

Date: Sun, 10 Mar 1996 07:58:05 PST

From: Stewart McLellan <stewartm@magi.com>

To: impala@east.xsis.xerox.com

Subject: Digest 339

Message-ID: <3142FC0D.74AC@magi.com>

Hi Guys:

Was I the only one who didn't get Digest 339 or did it end up in the bit bucket?

Regards, Stewart

=====================================================================

"Nothing in life is so exhilarating as to be shot at without result."

- Winston Churchill

=====================================================================

------------------------------

Date: Sun, 10 Mar 1996 12:33:00 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: show questions

Message-ID: <960310153259_165124916@emout10.mail.aol.com>

In a message dated 96-03-09 22:04:41 EST, you write:

> -How much is individual admission for the Super Chevy Show?

normally 45.00 at the gate.  We get the prereg. discount of 35.00 at the

gate.

> -How much extra (if any) if I plan on putting my car in the BFGoodrich

tent?

Need to check on this.  The promoter called my on Friday and I need to call

back.

The rest of the Impalas will be in a show car area all together.

> -If I park my car in the BFG tent, will it be judged for a contest?

I believe all the entered cars will be judged.  I will check on this also.

I did ask if our impalas could be with the prototype ones chevy is sending.

Unfortunately, the commercial display areas are separated from the show

cars.

Troy

NAISSO

------------------------------

Date: Sun, 10 Mar 1996 12:32:04 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: eastwood supplies

Message-ID: <960310153203_165124982@emout06.mail.aol.com>

In a message dated 96-03-10 10:10:29 EST, you write:

>OTOH, I just got a Sears Auto Restoration mini catalog. Awesome stuff

>in there! Request one if you don't have one. They've got some Eastwood

>paints in there that might be sort of cool: AlumaBlast

Rob, be sure you go direct to Eastwoods for it.  $ear$ probably marks it up

tw

------------------------------

Date: Sun, 10 Mar 1996 12:31:57 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: Bless me father for I have sinned..

Message-ID: <960310153157_165124999@emout07.mail.aol.com>

I feel that this is the only place that I can ask for forgiveness.  I have

almost done the unthinkable.  Please put your mouse down for a moment and

pray for me.

The short version is:

Rock turns off computer and decides to go out one evening.

Rock puts sweet smelling juice on body.

Rock meets member of the other gender during casual social activity.

Rock likes looks of other.  Other seems interested?!

Rock makes joke, Other laughs.

Rock makes points, Other sticks around!

Fighting urge to show ace in hand, Rock accidentally slips comment about

Impala.

In attempt to win hand, Other mentions "Daddy" owns a phord dealership.

Rock gets queazy, runs in restroom & gets sick.

Rock goes home & turns on computer.

Bless me father for I have sinned.

------------------------------

Date: Sun, 10 Mar 1996 12:32:03 PST

From: Rock528@aol.com

To: <impala@east.xsis.xerox.com>

Message-ID: <199603102032.PAA01497@emout05.mail.aol.com>

>So how do you pronounce NAISSO anyway ??? "nye-so" ? "nie-so" ? "nah-iss-so"

>?

Boy, all these command decisions,  how about "nay-so"?  Franklin, how would

you say it in Laweezianna?   Troy  ( I hope to have it on my tag by showtime)

I am going to upload a couple of logo ideas to Superdave's ftp site.  Tell me

what you think.  Troy

------------------------------

Date: Sun, 10 Mar 1996 12:35:52 PST

From: KEN52455@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: 1997 Impala SS??

Message-ID: <960310153551_442824180@emout09.mail.aol.com>

Was talking to my dealer about leasing a new Suburban for the wife when, out

of the blue, he told me that Chevy is reconsidering extending the production

of the SS due to enormous pressure from police depts. and taxi companies.I

don`t think he was trying to sell me because i`m only 1 year into a 3 year

lease on my `95 SS.

Many of you going to Arlington might want to look into this. I`ve seen where

Mr. Moss has had some ongoing dialogue with Troy, although would Chevy give

any clue at this point in time as to a confirmation or denial?Probably not,

my guess.

Ken52455

------------------------------

Date: Sun, 10 Mar 1996 13:42:27 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: control arms

Message-ID: <960310214227_73145.1566_FHD39-1@CompuServe.COM>

Martin,

>At 10 degrees it's too cold to install my new control arms.  After

measuring, however, I'm convinced the bolts in fact aren't long enough to

work with the HA bars and have purchased 4 M10-1.5X120mm bolts.

Yup, I reached that same conclusion myself, and in fact have ordered some of

the same M10-1.5x120mm bolts, as well as some 7/16-14x4.75 bolts. The 7/16

bolts are bigger, and just barely fit in the existing holes. I think these

will be much stronger than the M10 bolts. I also ordered these as flanged

bolts and flanged nuts as well, grade 10.9 (metric) and grade 8 (english). I

will evaluate which of these work out better, and then send them out to be

Jet Hot coated before final installation.

>After looking at all the parts, it is obvious that this will strengthen the

swaybar/control arm connection.  However, I think it actually weakens the

control arm.  The bracket that was there, cheesy as it was, did provide some

additional stiffness for the control arm.  Clearly what GM has done is

design a fix based on existing parts.

I don't think you are right about any "weakening" of the arm! Part of the

problem was that the original arms were cracking, and the new setup

strengthens it further. The two bottom plates will effectively box the

control arm in two separate places, plus the sleeve on the inside adds

considerable additional strength to the whole assembly. Also all of the

swaybar loads are now transferred to the whole arm, and not just the bottom

portion. Overall the attaching hardware will be under much less stress. I

bet that the new arms with these new swaybar mounts are actually stiffer

than the older setup.

>If desiging from scratch don't you think they would have used a box section

instead of a C-section?

What do you mean "designing from scratch"! These are really just the same

exact arms as before, with two additional holes drilled in the top. These

arms even carry the same "LT" designation on the side as my '94 stock Impala

arms, plus they have all of the other holes present in those arms as well.

Obviously this was not anywere close to a scratch design! Even so, it looks

to me that the new sleeve and plate setup with the through bolt boxes the

arm and makes it much stronger than before, not only for the swaybar

attachment, but the whole arm in general.

As far as boxing the whole arm in, indeed that will strengthen it further. I

know that Dave Morgan has boxed in both his upper and lower control arms for

maximum effect. You could weld in a bottom section across the whole length

of the arm, or you can purchase aftermarket boxed lower arms from sources

like Hotchkiss and others. I'm not sure if the aftermarket offers boxed

upper arms.

Most of the aftermarket arms use stiffer polyurethane bushings, which I

didn't want in my Impala. They tend to squeak and make noise, although many

of the newer ones are greasable, and they definitely transmit much more road

harshness directly to the frame of the vehicle, most noticible over

expansion joints (thump, thump), rumble strips and other small bumps. I know

that Troy has the Hotchkiss arms in his Impala, they have greasable urethane

bushings, and seem to work very well. They also use the same through bolt

technique for attaching the swaybar, like the updated factory arms.

I installed factory 1LE control arms in my '89 Trans Am, but this was

because the stock non-1LE bushings were way too soft, and the rear axle

would hop under extreme acceleration. The higher durometer rubber 1LE

bushings totally stopped the wheel hop, without adding undue additional

harshness. Again, urethane may have made noise (squeaks) and will definitely

transmit much more harshness to the body. That is fine for the track, but

can be tiresome on a car, especially a luxury performance car like the

Impala. Of course I should talk, as I already have stiffer springs and

swaybars in mine! <g> Scott.

------------------------------

Date: Sun, 10 Mar 1996 15:30:28 PST

From: LFriend121@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 340

Message-ID: <960310183027_165247715@mail04.mail.aol.com>

In a message dated 96-03-09 22:03:47 EST, you write:

>To: impala@east.xsis.xerox.com

>Subject: Re: IMPALA digest 339

Troy, I would like one of the hats.

Steve

------------------------------

Date: Sun, 10 Mar 1996 15:40:27 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Re: fuel pressure

Message-ID: <1.5.4b11.32.19960310234027.0068d610@mail.one.net>

>My system reads exactly 46 psi maximum, same as you! In other words, your

>regulator is working perfectly. An adjustable regulator will simply cause

>you to run either too rich or too lean, and when in closed loop the PCM will

>alter the injector pulse width to compensate. In other words, you don't need

>or want an adjustable regulator, unless you have made many mods to the

>engine and the PCM is unable to compensate using the stock pressure.

But doesn't the system operate out of a fixed table at WOT and quit reading

the O2 sensors? Wouldn't any performance enhancement (especially the free-

breathing intake and exhaust stuff) cause this table to be a little (or a

lot) lean?

Another question: what would be the best way to tune one of these regulators?

I suppose a Tech 1 or Auto X-Ray in the car reading the O2 sensor would be

best, but that's a lot of $$$. What about an exhaust temperature gauge or

one of the Cyberdyne Air/Fuel Ratio gauges?

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Sun, 10 Mar 1996 15:40:30 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Rattling Doors

Message-ID: <1.5.4b11.32.19960310234030.0069c170@mail.one.net>

Just thought I'd let you know what my dealer found. In BOTH doors,

the latch lock mechanism was loose. They resecured it and reinsulated

the door panels, and it's nice and solid now, just like when it was

new.

BTW, they also tightened the rear control arm mounts and remounted the

front coil springs (said they were "binding". What a difference! I couldn't

believe that it felt so different during a normal corner that I was taking

on the way home! I'll definitely go over the rest with a torque wrench

when it warms up a little bit. Brrrrrr....

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

End of IMPALA Digest 342

************************

IMPALA Digest 343

Topics covered in this issue include:

1) Re: Window Tint

by Robert Cheek <rcheek@one.net>

2) Arlington RSVP update (3/10)

by Matt Platts <mplat@airmail.net>

3) Test Drive

by "Brett A. Rudolph" <bngwip@mail.phoenix.net>

4) List Member Window Stickers

by "Scott Zeiter" <ScottZeiter@msn.com>

5) RE: IMPALA digest 342

by "Scott Zeiter" <ScottZeiter@msn.com>

6) Chrome intake pipe

by MK2112@aol.com

7) Re: IMPALA digest 341

by Fulminata9@aol.com

8) left coast

by "Jeff Gatcha" <foxhound@txdirect.net>

9) $251 "Adjustment"

by "Jeff Gatcha" <foxhound@txdirect.net>

10) Vette converter etc

by "Jeff Gatcha" <foxhound@txdirect.net>

11) ATI Superchargers

by bnyk@oanet.com (Bernie Nykyforuk)

12) Please Help an new Impala Owner..

by TrRoClark@aol.com

13) Re:  Bless me father...

by Mike Sundstrom <102366.175@compuserve.com>

14) F-car calibration

by john spears <speartec@iquest.net>

15) Plant Closings

by Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

16) Swaybars

by mbailey@rinet.hunton.com (Bailey, Martin)

17) anti-theft

by mike sampson <sampson@mms.water.ci.detroit.mi.us>

18) Oil Additives

by T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

----------------------------------------------------------------------

Date: Sun, 10 Mar 1996 15:40:24 PST

From: Robert Cheek <rcheek@one.net>

To: impala@east.xsis.xerox.com

Subject: Re: Window Tint

Message-ID: <1.5.4b11.32.19960310234024.00686890@mail.one.net>

When I went in to check on getting my windows tinted, I talked to

the woman for a while about how dark was legal and what I could get

away with. It turns out the Ohio tint is rather light. In fact, I

think it is much lighter than the tint you see on a lot of factory

minivans and such in the rear windows. I really couldn't tell much

of a difference from stock! Anyway, she told me that if I was worried

about the law, to talk to my optician and tell him that I get headaches

when driving in bright sunlight. Apparently you can get a medical

exemption from the tint laws and they can't hassle you. She said I could

go as dark as a wanted then. Just FYI...

BTW, my wife won't let me tint the windows (or buy a Buick GN!) because

she says I'd look like a mafia hit-man then :-) :-) :-)

--

Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

------------------------------

Date: Sun, 10 Mar 1996 16:31:32 PST

From: Matt Platts <mplat@airmail.net>

To: "'Impala'" <impala@east.xsis.xerox.com>

Subject: Arlington RSVP update (3/10)

Message-ID: <01BB0EAF.CF314940@dal07-03.ppp.iadfw.net>

Here's the updated RSVP list...  remember, if you haven't already =

replied to me, please do so.  I am trying to make this as accurate as =

possible.  Thanks -matt

E-Mail recieved: 19

Driving to area: 26

Driving Impala SS: 18

Flying to area: 6

Renting car: 1

Staying @ Fairfield: 16

Actual rooms @ Fairfield: 10

Staying @ other motels: 2

Headcount for tour: 35

Headcount for SCS: 33

SS's on display @ SCS: 16

------------------------------

Date: Sun, 10 Mar 1996 17:33:55 PST

From: "Brett A. Rudolph" <bngwip@mail.phoenix.net>

To: impala@east.xsis.xerox.com

Subject: Test Drive

Message-ID: <1.5.4b11.16.19960310193808.3ecf5eb8@phoenix.net>

Hi All,

Well, I gotta tell you, This Sunday really sucked!!! A friend of mine's

sister wanted to buy my 94 Formula Firebird. She is all of 16 or 17. She

showed up this morning with her Mother in tow to look at my car and test

drive it. Well, about 20-30 minutes later, I got a call from her mother

saying they were 2 miles up the road and her daughter had wrecked my

pristine, red, 18,625 mile Formula. IT's not like I can go yell at her or

anything after all, She is just a kid. And when I pullrd up in my truck to

the wreck, she was falling down crying histericle female just like you see

on TV. Turns out she was turning around and fixing to return to my house

when she stomped the gas and started to fish tail, didn't take her little

foot of the gas, crossed two lanes of traffic, went into the on coming lane

and hit a furd ranger in a corner head on shot. Needless to say my car is

F*CKED UP! No one in either vehicle was injured, but my car is a mess. All I

wanted to do is sell my car so I could have my SS which is still on order.

<sheesh>

I am going to tell them tomorrow that I feel they are obligated to buy my

car for the agreed upon price and see what happens. Otherwise, I'm pretty

well screwed till the car is fixed. <sigh>

Brett

Brett A. Rudolph

96 SS Black

(still waiting)

If you build it,

I will come.

------------------------------

Date: Sun, 10 Mar 1996 18:49:05 PST

From: "Scott Zeiter" <ScottZeiter@msn.com>

To: impala@east.xsis.xerox.com

Subject: List Member Window Stickers

Message-ID: <UPMAIL08.199603110243480449@msn.com>

Has anyone thought about producing a small window sticker so that list/club

members could quickly recognize each other on the road?  Or does this already

exist and I'm missing something?

Scott Zeiter 96 Black Beauty - Stock (for now)

------------------------------

Date: Sun, 10 Mar 1996 18:54:35 PST

From: "Scott Zeiter" <ScottZeiter@msn.com>

To: impala@east.xsis.xerox.com

Subject: RE: IMPALA digest 342

Message-ID: <UPMAIL08.199603110249180865@msn.com>

Troy wrote:

>The rest of the Impalas will be in a show car area all together.

Does this mean that NAISSO will appear at the show as a club?  Most car shows

give club awards.  What could be more impressive than 50 SS's in a row?

------------------------------

Date: Sun, 10 Mar 1996 19:39:57 PST

From: MK2112@aol.com

To: impala@east.xsis.xerox.com

Subject: Chrome intake pipe

Message-ID: <960310223956_347441552@mail02.mail.aol.com>

I was wondering if anyone that has done the econo intake air modification

using the chrome 3.5" exhaust pipe considered this: The inside of the pipe

has a very weak if any type of coating, therefore the possibility of

corrosion is prevalent.  I've looked at several pipes at Pep Boys and some of

them contained visible but very fine patchy surface rust.  What would be the

best thing to do? Paint might flake off, wouldn't want that to happen;

though if left unchecked the possibility of rust being sucked in isn't good

either.

Mike K.

'95 Black

------------------------------

Date: Sun, 10 Mar 1996 21:52:39 PST

From: Fulminata9@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 341

Message-ID: <960311005238_347550198@emout04.mail.aol.com>

Hi Dave. No problem on the air intake. I used a 3" plastic plug to block of

the hole for 'Home-plate' while I'm waiting for the replacement elbow to come

in. I replaced the square sound baffle ( with the off-set inlet/outlet)  with

an "S" shaped piece of 3 1/2" exhaust tubing. I actually used 2  90-degree

elbows welded together and smoothed out to make the "S" shape. I also welded

a 3/4" piece of tubing at an angle on one of the curves  (parallel to the

radiator and facing towards the center of the engine) so I could use the

existing hose from the air pump. This setup allows the MAF sensor to stay in

its original location and by using a K&N filter # RE0920, you can make a

simple "L" braket and bolt the narrow tip of the filter to the same bolt that

mounts the horn to the left inner fender. I painted the elbow with

'semi-flat' black paint which duplicates the OEM color of the engine

compartment.

The last thing I did prior to re-assembly was to remove the screen from the

inlet side of the MAF. It has a simple metal 'O' ring retaining it in the

housing. If needed I can mail or fax you a diagram with dimensions. It

actually takes more time to describe it than it took to build.

John Alger

------------------------------

Date: Sun, 10 Mar 1996 21:57:55 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: left coast

Message-ID: <199603110557.XAA00893@legend.txdirect.net>

>Hello John!  Welcome aboard.  I'm glad to see another So Cal SS owner.

> It seems that most of the members all reside on the left coast

><G>.

Hey Dave,  check that map again, I think  YOU guys  are on the LEFT coast.

-----------

In the year of darkness, 2029, the rulers of this planet

devised the ultimate plan. They would reshape the future by

changing the past. The plan required something that felt

No pity, No pain, No fear. Something Unstoppable.

They created The TERMINATOR...

Jeff Gatcha

foxhound@txdirect.net

------------------------------

Date: Sun, 10 Mar 1996 22:06:03 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: $251 "Adjustment"

Message-ID: <199603110605.AAA01424@legend.txdirect.net>

>I paid $24,400, which is $1,312 less than sticker, or about $630 over

>invoice (or about $880 over invoice without the bogus-sounding

>"adjustment").

>"adjustment" (BS??!!)             0.00                251.22

On my 95 it was $241 and it was listed  on the  GM invoice as an

"advertising" fee. But it was only in the DEALERS column.

Whether that means GM advertising or the dlr putting stuff in local

papers I dont know.

I dont think you did too bad.  Especially since I have only seen ONE

96 Impala on a lot Anywhere.

-----------

In the year of darkness, 2029, the rulers of this planet

devised the ultimate plan. They would reshape the future by

changing the past. The plan required something that felt

No pity, No pain, No fear. Something Unstoppable.

They created The TERMINATOR...

Jeff Gatcha

foxhound@txdirect.net

------------------------------

Date: Sun, 10 Mar 1996 22:10:00 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Vette converter etc

Message-ID: <199603110609.AAA01623@legend.txdirect.net>

Hey Curt,

If you have the Vette torque converter AND the trans-go kit,  AND the

3.42, what PCM calibration are you running?. Most people Ive talked to

say you dont want to use a shift kit AND a PCM with  a performance

shift program like the one from Z-industries or Hypertech etc because

it will be too harsh.   With all 4 it must be brutal, or are they

just exagerating?

-----------

In the year of darkness, 2029, the rulers of this planet

devised the ultimate plan. They would reshape the future by

changing the past. The plan required something that felt

No pity, No pain, No fear. Something Unstoppable.

They created The TERMINATOR...

Jeff Gatcha

foxhound@txdirect.net

------------------------------

Date: Sun, 10 Mar 1996 22:34:32 PST

From: bnyk@oanet.com (Bernie Nykyforuk)

To: impala@east.xsis.xerox.com

Subject: ATI Superchargers

Message-ID: <199603110634.XAA05635@cronus.oanet.com>

Could some one please e-mail me the phone number for ATI Superchargers.

Thanks!

Nolan Nykyforuk

bnyk@oanet.com

------------------------------

Date: Sun, 10 Mar 1996 23:32:51 PST

From: TrRoClark@aol.com

To: impala@east.xsis.xerox.com

Subject: Please Help an new Impala Owner..

Message-ID: <960311023250_165568267@emout04.mail.aol.com>

It#s finally here!!! My black `96 built on Feb. 21 came in on March 7.  My

wife and I have already broken her in with 600miles over the weekend.  I just

love it!  I had introduced myself a few weeks back describing how long I had

been following along with the group through all of the archives.  It#s more

than an interest#.it#s a disease.

For you seasoned Impala owners I have a few questions:

1)  The beast seems to only warm up to about 140 deg. While cruzin# along on

the freeway.  I haven#t made any mods yet and this seems a bit cool.  Should

I be concerned or is this normal?  When in town it warms up to the middle of

the temp range.

2)  I would like to replace the mufflers and add an H crossover to increase

the power a bit and enhance that LT1 growl!  Through reading all the

discussion I have narrowed it down to two.. The Walker Ultra Flows or the

Flow Masters.  Can someone shed some light on any performance or sound

differences.

3)  I have decided to try some Zymol wax and happened to locate the famed

cream version.  I took it for a test application on  the hood (in the garage

at about 60 deg ambient temp) and following the directions, had a difficult

time buffing it off without leaving greasy streaks..  Those of you who clamed

to really prefer Zymol, could you help me out so that I could realize the

same beautiful finish on my Black Impala.

I am really excited about the club and thanks to all of you that make it

happen.  Troy, my check is in the mail.  (my name is also Troy, not to

confuse the issue)  I#m locate in the greater Sacramento, CA area if there

are others around.  I am a Product Planner at Intel (so I like to see all of

us communicating through our computers# All with Intel Inside I hope ;-)   As

far as "Gray Hairs" owning this car.. That may be the exception.. I may have

to take the title as the Youngest Impala Owner in the Country, at age 23.  I

look forward to burning all my peers in their weenie cars!

Thanks everyone, I am really enjoying the correspondence.  I hope to get some

help with my new car issues.

Troy Clark,

Black `96 4 days old and ~600 mi

------------------------------

Date: Sun, 10 Mar 1996 19:41:23 PST

From: Mike Sundstrom <102366.175@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re:  Bless me father...

Message-ID: <960311034123_102366.175_EHR248-3@CompuServe.COM>

Troy,

I'll say a couple Hail Mary's and Our Father's for you.

Mike

------------------------------

Date: Sun, 10 Mar 1996 20:20:00 PST

From: john spears <speartec@iquest.net>

To: impala@east.xsis.xerox.com

Subject: F-car calibration

Message-ID: <m0tvz64-004XUNC@iquest.net>

Scott, you mention 2 minor problems with the F-car cal being the higher

shift point and fan turn-on temp. I find the 5800 RPM shift point feels

pretty much ideal to me, but then again I'm using the F-car engine in my SS

wagon! Based on this I assume the difference must be that you are getting

past the range of that wimpy B-car cam! (just kidding guys!!) As far as fan

turn on temps I always recommend an aftermarket adjustable fan controller.

I've used and installed these on many cars including my LT1 wagon and like

the flexibility of turning a knob for the temp I want. If I want to change

thermostats for summer/winter use I can set the fans accordingly. IMO,

anyone not using a Hypertech PP, or other custom programmed PCM, should

consider a fan controller.       John.

------------------------------

Date: Mon, 11 Mar 1996 04:04:36 PST

From: Scott Hollenbeck <hollenbe@east.xsis.xerox.com>

To: Impala SS <impala@east.xsis.xerox.com>

Subject: Plant Closings

Message-ID: <Chameleon.960311070716.hollenbe@mustang.east.xsis.xerox.com>

This morning's local radio news included some info on GM having to stop

work at 11 plants because of parts shortages associated with the strike

at the brake parts plant.  They didn't specifically mention that the

Arlington plant has been closed, but I suppose it's only a matter of time

if it hasn't been already.  Can any of our Arlington sources see what's

going on?

==================================================

Scott A. Hollenbeck (hollenbe@east.xsis.xerox.com)

Manager, Software Systems Engineering

Xerox Special Information Systems

1901 North Moore Street, Lower Lobby, Suite 5

Arlington, Virginia, USA 22209

(703) 284-3865

==================================================

------------------------------

Date: Mon, 11 Mar 1996 04:15:30 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (impala)

Subject: Swaybars

Message-ID: <1996Mar11.070700.1049.115764@rinet.hunton.com>

Scott M. wrote:

>I don't think you are right about any "weakening" of the arm! Part of the

>problem was that the original arms were cracking, and the new setup

strengthens it further. The two bottom plates will effectively box the

control arm in two separate places, plus the sleeve on the inside adds

considerable additional strength to the whole assembly. Also all of the

swaybar loads are now transferred to the whole arm, and not just the bottom

portion. Overall the attaching hardware will be under much less stress. I

bet that the new arms with these new swaybar mounts are actually stiffer

>than the older setup.

As long as no load is placed on the control arm, I agree!  However, when the

car is cornering and one side of the swaybar is "pulling down" on a control

arm, all of that load will have to go to the top of the control arm, not the

bottom.  (Can't shoot pool with a rope).   Because the new bottom plate is

only connected to the control arm by the clamping of the swaybar bolt and

friction, as the swaybar connecting bolt strains, the plate will tend to

pull away from the bottom of the control arm (or, more precisely, be less

tightly clamped), thereby reducing any stiffness that it added to the

control arm.  In this situation, I think it is fair to say the new design is

less stiff laterally than the original design, where the bracket was, of

course, always rigidly -- if imperfectly -- attached.

If lateral stiffness of the rear control arms is what makes the rear axle

move under hard cornering, then I think a box section control arm would be a

much better way to (1) improve handling and (2) provide a better connection

for the HA bars.  Welding on a bottom section plate but leaving the stock

bushings would also avoid the undesirable effects of the polyurethane

bushings.

Just my $.02

Martin Bailey

------------------------------

Date: Mon, 11 Mar 1996 05:41:28 PST

From: mike sampson <sampson@mms.water.ci.detroit.mi.us>

To: impala@east.xsis.xerox.com

Subject: anti-theft

Message-ID: <Pine.OSF.3.91.960311083702.25040B-100000@mms.water.ci.detroit.mi.us>

a question to all ss owners what if any theft devices do you use or recommend

also have any ever been defeated that are being used. thanks in advance

mike-96 black

------------------------------

Date: Sat, 9 Mar 1996 13:13:30 PST

From: T.MAKICHUK@fa-eps.nhq.drhc-hrdc.x400.gc.ca

To: IMPALA@east.xsis.xerox.com (Non Receipt Notification Requested)

Subject: Oil Additives

Message-ID: <0092vfxpblxh*/I=T/S=MAKICHUK/OU=FA.EPS/O=NHQ/PRMD=GC+DRHC.HRDC/ADMD=GOVMT.CANADA/C=CA/@MHS>

Loren:

>Is there an additive out there that is worth considering?  I know the O.M.

says NO NO NO.

>Is plain ole STP oil treatment worth a s%#*?  I've used it off an on thru

the years.  Anyone heard any studies which gave an aswer if the stuff is

good/bad or whatever?

In the GM of Canada publication <Chemicals and Maintenance Products Product

Information> I quoted in Digest 313, there is this entry for a product

called Engine Oil Supplement:

-------

Competitive additives focus on preventing the oil from thinning out,

sometimes causing the oil to thicken or increase in viscosity to an

undesirable level.  The diagram on Page 2 illustrates test results from the

Compulsory Industry Sequence 111C test for "SE" oil qualifications, which

highlights the undesirable viscosity increases of competitive products.

<There is a graph with '% CHANGE IN VISCOSITY' on the X and hours on the Y.

It compares: '10% Competitive Brand - Oil Screen plugging 100%.  397%

increase in viscosity.  Test ended at 14 hours.'  And '10% E.O.S. - Oil

Screen plugging 5%.  122% increase in viscosity.  Test completed full 64

hours.'  It continues...>

TEST HOURS

GENERAL MOTORS SPECIFICATIONS ON THE ABOVE TEST AND FOR "SE" ENGINE OIL

QUALIFICATION, ALLOWS ONLY UP TO 400% CHANGE IN VISCOSITY IN 40 HOURS.

FURTHERMORE, THE OIL MUST COMPLETE THE FULL 64 HOUR TEST.

ENGINE INSPECTION DATA	"SE" SPEC.	10% EOS	10% OTHER

RING LAND DEPOSIT		X		6.0		7.4			7.7

PISTON VARNISH		X		9.3		9.6			7.7

SLUDGE					X		9.0		9.7

9.5

OIL SCREEN PLUGGING			N/A		5.0%		100.0%

BEARING WEIGHT LOSS (MG)		N/A		23.5			401.0

X CRC RATINGS 10 CLEAN

-------

There is no mention what the competitive product was, but since the

references to SE grade oil would date this test to at least 15 years ago,

and as STP is probably one of the oldest and best selling additives out

there, it may have been the one tested.

Tip Makichuk

"I can't sell 'em to ya, cause they're illegal"

------------------------------

End of IMPALA Digest 343

************************

IMPALA Digest 344

Topics covered in this issue include:

1) RE:  Test Drive

by "Scott Blum" <rsblum@directmedia.com>

2) Re: Shift points

by Mike Sundstrom <102366.175@compuserve.com>

3) NAISSO window stickers

by Loren Peterson <equ_rino@cencom.net>

4) He said 'member' <G>

by "christopher.m.schroeder" <christopher.m.schroeder@arthurandersen.com>

5) Locating K&N

by Mikado95@aol.com

6) Re: Coated Rotors

by Kurt Kemmling <76165.1654@compuserve.com>

7) LRoshak@aol.com s.Fl.Dealership

by John George <johng@alpha.acast.nova.edu>

8) Non-lubed '96 suspension.

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

9) Newsletter???? (IMPALA digest 337) -Reply

by John Pinkerton <jpinkert@sigg.com>

10) Re: CROW BOIL '96

by daryl@austin.ibm.com (Daryl Green)

11) Another SS Owner

by Thomas.Drake-1@pp.ksc.nasa.gov

12) Grey Hair Owners?

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

13) Prices

by Joe Palmer <laser@iquest.net>

14) fuel press. regulator

by Scott Mueller <73145.1566@compuserve.com>

15) brakes

by Scott Mueller <73145.1566@compuserve.com>

16) Supercharged Smog

by william@discoball.com

17) Super Chevy Show Tips

by Doug.DePew@gsbsc.gensig.com

----------------------------------------------------------------------

Date: Mon, 11 Mar 1996 04:33:40 PST

From: "Scott Blum" <rsblum@directmedia.com>

To: "Brett A. Rudolph" <bngwip@mail.phoenix.net>

Cc: impala@east.xsis.xerox.com

Subject: RE:  Test Drive

Message-ID: <n1385585148.99916@smtp.directmedia.com>

Brett,

>I am going to tell them tomorrow that I feel they are obligated to buy my car

for the agreed upon price and see what happens. Otherwise, I'm pretty well

screwed till the car is fixed. <sigh>

Friends or not--If they don't have the common decency to buy the car at your

agreed price, I'd sue--no question.  But the question arises, what in the heck

was going through your head letting a 17 year old with or without her mother

drive your car???  Not intending to offend you nor pour salt in an open wound,

but man oh man--you just don't let that happen, no matter if they're God in

person, you go in the car--period.  Only after you are comfortable that they

are a responsible individual--while you are driving, let them take over--on a

side street!

Not that this will completely protect your vehicle as my friend will admit.

He let someone drive his '85 Vette and they smashed it with him in the

passenger seat.  He sued and won, but had to swallow $3300 in attorney's fees.

The attorney said afterwards, in the future, get people to sign an agreement

to buy at your asking price if they cause a even a scratch.  Further to pay

your attorney's fees if they later object.  If they say no--tell them to take

a hike!  Just to many idiots out there...

He said, although this won't prevent litigation and can be argued--it will

speed the process, help the Judge lean in your favor and most of all, get the

people to think before they act irresponsibly.  Also, by being so protective

of your car, the car becomes that much more attractive in the prospective

buyers mind.

Good Luck Brett,

-Scott B.

------------------------------

Date: Mon, 11 Mar 1996 07:22:51 PST

From: Mike Sundstrom <102366.175@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Shift points

Message-ID: <960311152250_102366.175_EHR108-1@CompuServe.COM>

John,

>I find the 5800 RPM shift point feels

pretty much ideal to me, but then again I'm using the F-car engine in my SS

wagon! Based on this I assume the difference must be that you are getting

past the range of that wimpy B-car cam! (just kidding guys!!)

No, it's not the wimpy cam we are running, it's the valve float we run into with

the Crane 1.6 roller rockers (which gives us about the same lift as your nice

aluminum headed LT-1).  Mike

------------------------------

Date: Mon, 11 Mar 1996 08:09:09 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: NAISSO window stickers

Message-ID: <31445025.1B06@ns.cencom.net>

For the Show, how about a nice NAISSO in bright red with bow ties

on either side? OR?  Time is running out, but my decal guy probably

could have something done by SHOW TIME.  I hope to get the red (other

colors avail.) bow ties for the grill this week.  Will notify list when

it happens.  Also there was some interest in the rear quarter Impala SS

emblem in decal form.  My guy said he would go purchase one of these to

reproduce if there was enough interest.  Also, yes he can do in chrome

or gold, but it sure scratches easy.

ALSO

What does it help to remove the screen from the Mass Air?

What does it hurt?  I need to know, cause if theres power to be had, I

WANT IT!

Oh ya, my decal guy is going to make some funky decal for the Sewer pipe

induction system.  Any ideas?

Loren DCM96 SPEquipped

Texas or Bust

------------------------------

Date: Mon, 11 Mar 1996 02:13:25 PST

From: "christopher.m.schroeder" <christopher.m.schroeder@arthurandersen.com>

To: impala <impala@east.xsis.xerox.com>

Subject: He said 'member' <G>

Message-ID: <9603111625.AA4198@CServe-Hub4.inhouse.compuserve.com>

Just thought I'd take a minute and introduce myself. I joined the list about

two weeks ago and am, being politically correct, Impala impaired. I do however

share the same feelings toward the blue oval brigade as well as the pentastar

crowd. I think they should just stay chained to the porch if you know what I

mean. I live in the greater Chicago area (Naperville to be exact). The trip to

Arlington sounds very interesting and my '91 Z-28 convertible would love a long

road trip (not to mention warmer weather), but due to work constraints I am

unable to go.

My only complaints are that my F-car, because it's a convertible, only came

with a 5.0. I've performed a few mods, but still nothing compared to dropping

in a new LT1, or better yet an LT4, and then start with the mods. I still have

to try an sell the (new) wife on the idea, so my hopes aren't too high yet.

I've been raised around Bow-tie power and performance machinery. My father used

to drag race and build engines. His greatest claim to fame is racing a (don't

quote me on this) '67 vette with a 427 with his 55 wagon powered by a 350 H.P.

327 and kicking its butt three times in a row. So I have performance racing in

my blood somewhere. The only problem is that I don't have enough U.S. $ in my

bank account. Anyway, (said holding mug high in the sky) Here's to later days

and better ways.

Shredder

------------------------------

Date: Mon, 11 Mar 1996 09:59:38 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: Locating K&N

Message-ID: <960311125936_243115984@mail04.mail.aol.com>

I'm in Naperville, outside Chicago.   Where is the best place for me to look

for a K&N filter for my 95 Monte Carlo Z34 ?   I'd like to try that mod, but

I have no idea where to look!  I've tried Trak Auto, but no luck.

Suggestions?   Please e-mail me directly as to not clog the digest!

Thanks.

Randy

Torch Red 95 Monte Carlo Z34

Green-Gray 95 Impala SS

------------------------------

Date: Mon, 11 Mar 1996 10:25:07 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Coated Rotors

Message-ID: <960311182507_76165.1654_GHM117-1@CompuServe.COM>

In digest 334, Scott Mueller wrote:

>>>That started in '94! In fact ALL Impalas get painted (black) front calipers,

and both the front and rear brake rotors are specially coated with a

corrosion resistant material.<<<

Does this material wear off as the rotor wears, or is it impregnated completely

thru the rotor?

Do you know what this coating is, ie; the chemical or trade name?

Will this coating prevent rust on the rotor suface? Like when the car sits for a

day or two in damp rainy conditions, and there is usually some rust starting to

form.

I ask this because I had the front rotors replaced under warranty due to

warping, and they now seem to rust if I don't drive the car for a couple days.

Thanks

Kurt

'95 Dark Cherry Impala SS.....no mods.......rusty rotors.........

------------------------------

Date: Mon, 11 Mar 1996 10:21:36 PST

From: John George <johng@alpha.acast.nova.edu>

To: impala@east.xsis.xerox.com

Subject: LRoshak@aol.com s.Fl.Dealership

Message-ID: <Pine.3.89.9603111351.B2283-0100000@alpha.acast.nova.edu>

I spoke to my chevrolet dealership manager friend and she told me that

she could help LRoshak@aol.com with his s. Fl dealership needs and also

the she has 2 Impala SS's coming in in the next 2 weeks that are

available for sale.  both are black.  If she can help, call her at Ed

Morse Chevrolet at Lake Park which is in Palm Beach County.  Her number

is 407-844-5262.  Tell her that John from Davie refered you. She is a

great source for chevy addicts like me!!!

John George

johng@alpha

------------------------------

Date: Mon, 11 Mar 1996 11:27:38 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Subject: Non-lubed '96 suspension.

Message-ID: <9603111931.AA27194@karloff.fstrf.org>

Regarding my '96 Impala, build in January of 1996:

Well, over the weekend I decided it would be in my best interest, and the

beast's best interest <g>, to check the lubrication of the suspension.

With grease gun in hand I slid under the beast to find to my displeasure

and amazement that none of the grease fittings were lubed!  I surmised

this by two things: 1) by moving some suspension parts (tie-rod ends) and

hearing and feeling a metal-on-metal grinding, and 2) noticing that all

fittings with dust caps/boots puffed up like fattening crows in the springtime

with the brown, lithium-based Pennzoil grease I eagerly fed them.

It looks like this problem is still happening to the '96 models

built after 1995.

A related question: is the '96 HD Caprice TSB that may or may not

apply to the Impala <g> based on the fact that ball joints are being destroyed

by lack of proper torque and lube (which we all can prevent ourselves) or

is there something more that only GM can fix?  Anyone hear more about this TSB?

I'll be calling Chevy's CAC to ask/complain about the Impala not being included.

Thanks,

- mike

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

My '96 Impala SS: "It goes fast, FAST!"

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

Date: Mon, 11 Mar 1996 12:13:16 PST

From: John Pinkerton <jpinkert@sigg.com>

To: impala@east.xsis.xerox.com

Subject: Newsletter???? (IMPALA digest 337) -Reply

Message-ID: <96Mar11.135845cst.26499@firewall.sigg.com>

>Troy, you said you shipped these on Tuesday, February 27th (I

>think?)and you wanted us to tell you when we got them. Well, its been

>over a week, and I didn't get mine yet.  Has anyone else received their

>newsletter yet?

Today is Monday, March 11, 1996 and I still NO newsletter.

------------------------------

Date: Mon, 11 Mar 1996 12:12:14 PST

From: daryl@austin.ibm.com (Daryl Green)

To: impala@east.xsis.xerox.com (impala)

Subject: Re: CROW BOIL '96

Message-ID: <9603112012.AA17190@augusta.austin.ibm.com>

Rock528 writes :

> Our hangar neighbors, NASCAR, hangs this thing in their aircraft hangar to

> keep the birds out.  It's a big yellow beachball with two huge googly eyes.

> The birds think it's an owl on steroids and they stay out of the hangar.

>

> So, get yourself a 24" piece of cardboard and paint it yellow.  Make two

> crazy eyes on it and add little pecker.

^^^^^^

I knew that fake owls were used to scare off crows but until now I

wasn't sure exactly what it was the crows were afraid the owls were

going to do to them! <G>

Daryl Green

daryl@austin.ibm.com

------------------------------

Date: Mon, 11 Mar 1996 15:30:00 PST

From: Thomas.Drake-1@pp.ksc.nasa.gov

To: impala@east.xsis.xerox.com

Subject: Another SS Owner

Message-ID: <9603112033.AA03234@attila.ksc.nasa.gov>

Hello fellow SS'ers.  With a new baby last July my wife and I needed some

more room and I won the minivan discussion and ordered an Impala.  I just

received my Black '96 SS today at lunchtime.  I ordered it on December 15th

and it came in this past weekend (March 8).  I saw it Sunday at the

dealership pinned in by two Caprice 9C1's (I really like the vomit proof

vinyl seats and floor mat), but they wouldn't let me have it until it got

it's final inspection (come on John Boy Goodwrench let's go thrash this new

Impala).  I told them to be sure to check the ball joints, knuckles, etc.

for grease (had it) and differential fluid (OK) so maybe GM is taking

feedback.  The only problem I have is that January 1 my wife made a wager

with me.  Whoever lost the most New Years pounds by March drives the car for

the first month.  Welllll..... I lost, but she is letting me drive it on

Mondays (I am living up to the bet so she didn't totally humiliate me and

gave in a little....what a girl...).  I will get to drive it on weekends (I

hope) and when the first month is over watch out (I'll let her do the break

in mileage).

Ted Drake

'96 Impala SS (no mods yet - grinning from ear to ear)

------------------------------

Date: Mon, 11 Mar 1996 13:10:48 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: Grey Hair Owners?

Message-ID: <Pine.SOL.3.91.960311130524.11876A-100000@netinfo.ubc.ca>

Well Troy:

>All with Intel Inside I hope ;-)   AsHR248-3@CompuServe.COM>

>far as "Gray Hairs" owning this car.. That may be the exception.. I may have

>to take the title as the Youngest Impala Owner in the Country, at age

>23.  I

>look forward to burning all my peers in their weenie cars!

Well Troy, I am sorry but I have to say that title of youngest

Impala Owner belongs to me unless some other people claims otherwise.  I

am only 22 years old right now!

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red (9C1 Air Deflactors, Gas Bib)

------------------------------

Date: Mon, 11 Mar 1996 13:19:17 PST

From: Joe Palmer <laser@iquest.net>

To: Impala Page <impala@east.xsis.xerox.com>

Subject: Prices

Message-ID: <314498D5.63CA@iquest.net>

Below are the GM Employee Prices

Maybe this is why we have to wait longer for our orders?

I still think short delivery times are cars allready in

the system.

Not really sure what the adjustment figure some of the

members mention is. Its not on the invoice GM mailed me.

MSRP			INV AMT

1BL19  IMPALA SS 4DOOR-SEDAN		24405.00		20988.20

A23	HEAT REFLECTIVE WS.		   52.00		   42.64

UNO	CD PLAYER			  155.00		  127.10

1SE	PREF. EQUIP. GROUP		  490.00		  401.80

TOTAL MODEL & OPTIONS			25102.00		21559.84

DESTINATION CHARGE ** note	  	  590.00		  590.00

BOTTOM LINE				25692.00		22149.84

$3542.16 below invoice

** note  The TEAMSTERS need to make a buck !!

------------------------------

Date: Mon, 11 Mar 1996 12:44:48 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: fuel press. regulator

Message-ID: <960311204448_73145.1566_FHD31-2@CompuServe.COM>

Rob,

>But doesn't the system operate out of a fixed table at WOT and quit reading

the O2 sensors?

I'm not sure it still works that way on the '94 and up systems. In any case,

WOT was always calibrated to be very rich. One thing that should be noted is

that the LT4 uses larger injectors than the LT1. I would think that an ECM

recalibration for the larger injectors would be an interesting experiment.

Even so, more fuel than is necessary will only cause problems.

>Another question: what would be the best way to tune one of these

regulators? I suppose a Tech 1 or Auto X-Ray in the car reading the O2

sensor would be best, but that's a lot of $$$. What about an exhaust

temperature gauge or one of the Cyberdyne Air/Fuel Ratio gauges?

I don't think any of those would react quick enough to tell what is

happening at WOT. The best way to tune this would be to make adjustments and

perform back to back testing on a chassis dyno. That is how a company like

Superchips or Hypertech makes their calibration adjustments. Scott.

------------------------------

Date: Mon, 11 Mar 1996 12:44:44 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: brakes

Message-ID: <960311204443_73145.1566_FHD31-1@CompuServe.COM>

Jeff,

>As I understand the brakes on the Impala's, the back caliper is a 'vette

unit modified for the mini-drum parking brake. The back, is off of the front

of a late 80's early 90's 'vette.

The front brakes on a 'vette are dual piston aluminum PBR units, not

anything like the single piston unit on the Impala rear. The rear aluminum

PBR (Australian made) single piston brakes are similar to what is on the

rear of '88 and up F-cars, but not quite the same as those either. The

'88-'92 F-cars with 1LE suspension came with the dual piston pad guided

aluminum PBR front brakes, which actually used Caprice 9C1 rotors drilled

for the different 4.75" bolt circle on the F-cars (B-cars have a 5" bolt

circle). Those killer 'vette brakes also came standard on my '89 Turbo TA.

Scott.

------------------------------

Date: Mon, 11 Mar 1996 14:05:39 PST

From: william@discoball.com

To: impala@east.xsis.xerox.com

Subject: Supercharged Smog

Message-ID: <v02140b00ad6a51d25f02@[206.215.67.3]>

Here are the results of my SSupercharged Impala SS passing the California

Smog Test today.

It is a '94 Impala SS with SVI/Vortech supercharger, Z Industries PCM, 160

degree thermostat, Level Ride Control from a Buick Roadmaster, Sony 4-disc

MiniDisc changer, dual power antennas with diversity tuning (that

automatically lower when listening to a MiniDisc), 16 speakers including 4

12" woofers, car phone with hidden antenna inside the car, and a Sony GPS

Navigational Computer with 5" color LCD screen.  It's amazing that I find

time to have a job!

2500 RPM TEST:

MAX ALLOWABLE     MEASURED

HC PPM          0220           0010

CO %            1.20           0.00

CO2                            14.1

O2                              1.8

RPM                            2692

IDLE RPM TEST:

MAX ALLOWABLE     MEASURED

HC PPM          0100           0010

CO %            1.00           0.00

CO2                            14.5

O2                              1.1

RPM                            0862

------------------------------

Date: Mon, 11 Mar 1996 13:23:22 PST

From: Doug.DePew@gsbsc.gensig.com

To: impala@east.xsis.xerox.com

Subject: Super Chevy Show Tips

Message-ID: <144a5d80@gsbsc.gensig.com>

For all of you headed over to Arlington, maybe I can offer you some

insight into the Super Chevy world.  I've been actively showing and

winning at Super Chevy events now for 10+ years, and have seen some

major changes in the way they handle things.  In fact, just in 1994/5

they made some staff and idea changes.

Item #1:  Club participation.  Super Chevy in 1995 instituted a Club

Participation award.  I have managed with our club to sweep the

Florida events for this award last year.  EVERYONE that enters the

show WITH a vehicle must go to the registration tent and sign their

name and show class to the page for your club.  It would be awesome to

see this club garner that award at its forst annual convention!

Item #2:  Classification.  There are 3 classes of car shwo entry.

Stock, Street, Modified.

Stock is for just that, stock, as built by the factory.  Slight

changes such as tires, battery, radio are permitted, but the car must

be stock or stock appearing.  Changes to original must have been

available as an RPO from the factory.  In other words, you could take

a 69 Base Camaro and build it into a Z/28, as long as everything was

done according to factory design and options.  You could not have a

Big Block in a 69 Z/28 and be stock.

Street is for slightly modified cars, with basically stock or

single color paint.  Slight pinstriping is also allowed.  No

candy/metalflake/pearl paints are allowed.  Chrome is allowed, as are

braided steel hose COVERS.  Actual braided hoses move you to Modified.

Wheels are permitted to be changed as well.

Modified is almost an anything goes class.  Frame mods and tubbing

move to Pro street or Pro Engineered.  Other than that, wild paint,

heavy chrome, engine swaps, blowers, exotic induction are all

allowable.

In cases of many cars in a class, Super Chevy may split the class by

year breaks, at their discretion.  In other words, Impalas start out

in Big Car (All years - again subject to 1996 changes - I'll check

later this week).  With your crowd, there may actually be Big Car, Pre

1994, Big Car 1994, Big Car 1995, Big Car 1996 (a minimum of 8 is

needed to cause the split).

Item #3:  Judging.  Judging is done in 4 categories.  Paint, Body,

Engine, Interior.  Each category is worth 25 points.  Judging is done

from "average".  In other words, an average car gets 12.5 in each

category.  Points are awarded or deducted based upon quality from

there.  Judging is continuous from noon on Saturday to late on Sunday

(even up to actual awards presentations).  On Saturday, if your car is

reviewed, it will receive a sticker dot on the windshield.  These also

dictate class.  Judges will reclassify if need be.  Blue=Stock,

orange=Modified, Green=Street. (unless changed this year).  on Sunday,

when your car receives FINAL judging, the judge will initial the dot.

DO NOT remove the dot during the course of the weekend.  Alignment of

body panels is checked.  Doors should be left to rest against their

striker for the judge to open and look in and check alignment as well.

Windows up is preferred so that all glass can be inspected.  If you

do not have it open, the judge WILL NOT open it for you.  Open your

trunk, hood, and doors.  If your windows are dark tinted, and your

doors and trunk are closed, you get 0 for interior, not even average

score.  A filthy engine is still worth 4 points or so, which is better

than 0.  I have been beaten by 1 point before, and have won by 1.

Every point is critical.  Cleanliness counts heavily.  IMPORTANT

THING:  leave no wax residue in cracks, seams, crevises.  This costs

points.Undercarriages will not be judged, only in event of a tie will

they be glanced at.  In most cases, ties are broken without looking

undeneath anyway.

Item #4:  Times.  You can come and go at will.  Your vehicle is only

required to be there Sunday from 11:00 AM till awards to be judged.

If time for awards is drawing near, it pays to make sure you have been

judged.  If not, grab a judge and KINDLY ask about it.  There are

awards on Saturday at 3:00 for Editor's Choice Top 10 and a few

specialty awards.  On Sunday, awards are supposed to start at 3:00 but

usually start later.  Sunday awards are for classes.  Each class gets

a winner and 3 runners up.  A few additional specialties are given out

on Sunday, although most are not applicable to Impalas (sorry gang).

Item #5:  Parking.  Parking is by broad class.  In other words, all

Impalas of all years are parked in the "Big Car" class.  No cars can

be parked in the BFG area, no matter what you are willing to pay, and

be judged.  Same thing with the Chevy trailers.  When you arrive and

find your class parking, try to section off an area with chairs, etc.

to make sure you can all be together and make that impressive sight.

It might garner some magazine photos.

Item #6:  Magazine photos.  The editor rides around on a scooter or

golf cart all weekend, looking for items of interest to photograph.

Not to be insulting, but your Impalas, while looking ominous and

impressive all in a row, are not going to catch his eye for magazine

coverage simply due to their appearance.  If you have some items done

to your car that might be worthy, find the editor and strike up a

conversation.  He can't see much from his cart, so the injector cover

swaps, etc. are invisible unless he gets interested or talked into

looking.  Be nice, and be understanding.  Everyone there wants their

car/truck in the magazine.  Photo shoots are done during the weekend,

at a location away from the show area.  You will have to move your car

and be at their mercy.  In Gainesville, I missed half of awards

(luckily not mine) due to being 1.5 miles away for a photo shoot.

Also, being photographed does not guarantee publication, trust me on

this <G>.  I am still waiting for mine to be published, since October.

That about covers it.  If any of you have anything you'd like to know,

or can think of tips I did not cover that you want to know about,

simply drop me a message privately or to the digest and I'll do what I

can to help.

Above all, enjoy the event, best of luck to EVERYONE and have a safe,

fun trip.  The Jet Cars and Pro Mods run exhibitions throughout the

weekend for the speed freaks.

Doug DePew

Vice-President/Editor, Sunshine State Camaro Club (Florida)

1948 Chevy Pickup (In progress, IFS, IRS, SBC, etc.)

1976 Camaro Type LT (Heavily modified, Super Chevy winner, multi ISCA

winner)

1993 Chevy Indy Pace Truck (Fully restored, National Champion,

Multi-Super Chevy Winner)

1995 Chevy Tahoe 4x4 (To haul these and parts around, Super Chevy

winner)

Former F-Bodies Owned:

1989 IROC-Z (Slightly modified, National Champion, Multi-Super Chevy

Winner)

1991 RS (Daily Driver - no wins)

doug.depew@gsbsc.gensig.com

------------------------------

End of IMPALA Digest 344

************************

IMPALA Digest 345

Topics covered in this issue include:

1) My SS

by CS_MWR@hal.lamar.edu

2) Re: Lower Control Arms

by "Thomas L. Pierce" <75027.2421@compuserve.com>

3) Newsletter received

by Ray Berilla <writes4u@ganet.net>

4) 1/4 mile attempt-6/8/96

by ZTMP50A@prodigy.com ( DAVID NMN)

5) Interior lights not coming on with keyless entry

by SManey0830@aol.com

6) K & N Question

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

7) Re: Zymol wax

by Charlie311@aol.com

8) Re: "Brutal"????

by Curt Platteborze <75511.1220@compuserve.com>

9) 6-Speed?

by Dave Carlson <dacarl@courant.infi.net>

10) Re: IMPALA digest 343

by LFriend121@aol.com

11) Re: IMPALA digest 343

by LFriend121@aol.com

12) Re: Leakdown test blues!

by Curt Platteborze <75511.1220@compuserve.com>

13) Re: IMPALA digest 342

by Mbmarx@aol.com

14) Re: IMPALA digest 342

by Kurt Kemmling <76165.1654@compuserve.com>

15) Re: Texas trip

by Curt Platteborze <75511.1220@compuserve.com>

16) RE: IMPALA digest 342

by "J.R. LEVITZ" <4everGreen@msn.com>

----------------------------------------------------------------------

Date: Mon, 11 Mar 1996 14:15:27 PST

From: CS_MWR@hal.lamar.edu

To: impala@east.xsis.xerox.com

Subject: My SS

Message-ID: <960311161527.3831@HAL.LAMAR.EDU>

I received a few replies asking for more information on my Supercharged

SS, sorry to be so brief in my previous post.

I got the ATI Procharger 9psi intercooled kit, with the optional 3-core

intercooler and 10-rib belt.  The kit, with the two upgrades, came to about

$3500.  ATI doesn't make a kit for the Impala, so I had to buy the Z-28

package.

Conley's Performance Plus, the Turbo Buick specialists deserve all the

credit for my great times.  Give them a call at (713) 540-FAST for more

info.  I'm lucky enough to live about five minutes from their shop, plus

i've known Lawrence Conley for a while, so I got the pleasure of being

their "guinea pig."  CPP is putting together some literature on the package

(at least I hope they are) and i'll be bringing that to the show...

Someone asked about other mods.  The only other things i've done are a

power pulley, airfoil, crossover pipe, and a pair of three-chamber

flowmasters.  This car has so much potential.  Before my best time/mph

was 14.873 @ 92.81, now it

has jumped to 13.194 and 104.48 - And I hesitate to mention this for fear

of not being believed:  The car isn't 100% yet!  I still have the stock

injectors in the car, but they are getting deadheaded at the shift points.

That will be fixed this week...  Bigger injectors, 3.73 gear (I still have

3.08s), 3" free flow exhaust, and a set of slicks and who knows what the

car will run.  Any other questions?  If it's a technical question you would

be better off calling CPP at the above number, or feel free to call me at

(713) 852-3731.

Jim "What's that whistling noise?" in the soon to be 12 second '94 Impala SS!

------------------------------

Date: Mon, 11 Mar 1996 14:24:11 PST

From: "Thomas L. Pierce" <75027.2421@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Lower Control Arms

Message-ID: <960311222411_75027.2421_GHJ154-1@CompuServe.COM>

Scott M. wrote: >I don't think you are right about any "weakening" of the arm!

Martin B. wrote: >(Can't shoot pool with a rope).

Martin:

(great quote by the way <G> )

I won't have my daily driver apart to compare parts until this weekend, but does

the old swaybar bracket fit into the new lower control arm?

I gather from Scotts comments that it will fit...

If it can be modified to help take up the load, then some of the load could be

transferred to the sides of the control arm, as the old bracket did.

Comments?

Tom

------------------------------

Date: Mon, 11 Mar 1996 14:31:01 PST

From: Ray Berilla <writes4u@ganet.net>

To: Impala list <impala@east.xsis.xerox.com>

Subject: Newsletter received

Message-ID: <199603112239.RAA27967@s1.GANet.NET>

>Troy, you said you shipped these on Tuesday, February 27th (I

>think?)and you wanted us to tell you when we got them. Well, its been

>over a week, and I didn't get mine yet.  Has anyone else received their

>newsletter yet?

>Today is Monday, March 11, 1996 and I still NO newsletter.

Got mine today. (Monday, March 11)

Ray B.

------------------------------

Date: Sun, 10 Mar 1996 14:29:08 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: 1/4 mile attempt-6/8/96

Message-ID: <097.03304710.ZTMP50A@prodigy.com>

Hey Guys..was wondering.  In June, I'm going to take my car to the

track and finally test it (legally, <G>).  What is the best launch

procedure?  I was thinking to drop it in 2 at launch, feathering the

pedal until the tires hook, run it within 500rpms of red...and shift

to 3rd.  Whats the opinion??  What have you done with yours?  I know

to have about 1/4 tank of gas, and the possible removal of the spare,

but would like to turn as quick at time as possible.  Car is bone

stock now, but should have airbox opened by then..(time permitting).

Also, car only has 1600 miles on it currently.  I project that in

June, the car will still have less than 3000 miles on it.  Any

downside to run the car with this "low" mileage?.

Thanks!

Dave in So Cal..'96 Black SS...

------------------------------

Date: Mon, 11 Mar 1996 15:09:03 PST

From: SManey0830@aol.com

To: impala@east.xsis.xerox.com

Subject: Interior lights not coming on with keyless entry

Message-ID: <960311180903_443761157@emout07.mail.aol.com>

I recall that the lights don't come on even though the brochure mentions

this. I've checked the maillist archive and the FAQ... I could have sworn

there was mention of how to make it work. Anyone know how to get this to

work? (Don't have my car yet, trying to fix my father in law's new '96

Caprice. The dealer told my mother in law that it just don't work, should've

bought a Pontiac! No kidding!)

------------------------------

Date: Mon, 11 Mar 1996 16:23:38 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: K & N Question

Message-ID: <Pine.SOL.3.91.960311162134.15802A-100000@netinfo.ubc.ca>

To Scott and who ever else that had the K&N Corn filter:

I am just wondering for those of you who had convered to the K&N

corn filter, what do you do with the air hose that's attach to the air

box now that you have the air box assembly removed altogether to

faciliate the filter?

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red (9C1 Air Deflactors, Gas Bib)

------------------------------

Date: Mon, 11 Mar 1996 16:48:26 PST

From: Charlie311@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: Zymol wax

Message-ID: <960311194824_166116018@emout06.mail.aol.com>

In a message dated 96-03-11 08:48:49 EST, you write:

>3)  I have decided to try some Zymol wax and happened to locate the famed

>cream version.

The cream version is to be avoided!  Buy the paste direct from Zymol.  It's

twice the price, but contains much more carnauba.  The paste will last much

longer, and give the results you desire.

Charlie

------------------------------

Date: Mon, 11 Mar 1996 17:32:17 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: "Brutal"????

Message-ID: <960312013216_75511.1220_GHJ219-1@CompuServe.COM>

Jeff,

Explain brutal. <G>  I am using the Hypertech programing.

I first installed the Hypertech, than the 3.42 gears, then the Transgo and

Vette converter.

I had some questions about the Hyperetech fighting with the Trans Go kit so I

called both companies and both said no problem. I also posted a note to a

certain SM.<G> and got the same response. I had the same concerns as you.

I also needed to keep the Hypertech to calibrate my speedo for the gear change.

Scott Harris told me that the Z program would maybe be a problem, And if you

have the Z unit not to install the Trans Go kit. I already had the Hypertech

unit so went with the Trans Go to tighten up the shifts.

The shifting under grandma's foot is just a bit more noticeable than stock.

However at open throttle conditions it's awesome, bit I don't think it's

damaging to the trany or anything else. Sure is "eventful"

I would recommend this combination to all.

By the way I was a not at all into cars till I got the SS in August of 95. I

also did not know the difference from a keeper to a throttle body. This digest

and all the people at Compuserve taught me a whole bunch. Thanks!.  I was quite

squimish about working on my car and sure did not know much. Whats the Home

plate??etc..

So I guess what I'm saying is I asked a lot of questions and experimented  quite

a bit and so far with great results.

Thanks to all for your input and sharing your knowledge with me.

Curt P.

------------------------------

Date: Mon, 11 Mar 1996 18:00:54 PST

From: Dave Carlson <dacarl@courant.infi.net>

To: impala@east.xsis.xerox.com

Subject: 6-Speed?

Message-ID: <199603120200.VAA06948@mh001.infi.net>

I'm fairly new to the mailing list, and was considering purchasing a new

Impala.  I wondered if anyone had any experience with switching to a 6-speed

transmission.  Any thoughts?

Thanks.

Dave Carlson

------------------------------

Date: Mon, 11 Mar 1996 18:16:52 PST

From: LFriend121@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 343

Message-ID: <960311211650_166196569@emout06.mail.aol.com>

In a message dated 96-03-11 08:48:49 EST, you write:

>ate: Sun, 10 Mar 1996 15:40:24 PST

>From: Robert Cheek <rcheek@one.net>

>To: impala@east.xsis.xerox.com

>Subject: Re: Window Tint

>Message-ID: <1.5.4b11.32.19960310234024.00686890@mail.one.net>

>

>When I went in to check on getting my windows tinted, I talked to

>the woman for a while about how dark was legal and what I could get

>away with. It turns out the Ohio tint is rather light. In fact, I

>think it is much lighter than the tint you see on a lot of factory

>minivans and such in the rear windows. I really couldn't tell much

>of a difference from stock! Anyway, she told me that if I was worried

>about the law, to talk to my optician and tell him that I get headaches

>when driving in bright sunlight. Apparently you can get a medical

>exemption from the tint laws and they can't hassle you. She said I could

>go as dark as a wanted then. Just FYI...

>

>BTW, my wife won't let me tint the windows (or buy a Buick GN!) because

>she says I'd look like a mafia hit-man then :-) :-) :-)

>--

>Rob Cheek -- Cincinnati, Ohio -- rcheek@one.net

>1994 Impala SS, AS&M Air Intake, Walker CVX Mufflers, H Pipe,

>"Z" PCM, TPIS Airfoil, Analog Tach, 160 Degree Thermostat

>

>

Rob, California has the same rule on the side windows for the driver and

passenger.  The back windows can be mega dark.  Might check on that combo.

It is hard to get a Doc. to sign off on a tint. I had zero luck.   What I

tried, and it has worked so far, is a real dark tint, limo black in the back

and rear seat windows, and a 50% tint on the drivers and passenger window.

The contrast looks like no tint on the front window but you get the

protection from UV.  Doesn't look as good as all the way round but no cops.

As long as they can see the driver, all is cool.

Did you add the tach,  if so how?

I am interested in anyone that has just done the hypertech "chip" change.  I

know that the chip is not really changed and the computer is reprogrammed.  I

want to know if the power change is noticable and if mileage is affected.

I am interested in the 'vette cats.  What is the number?

Steve,

late '95, black

------------------------------

Date: Mon, 11 Mar 1996 18:19:36 PST

From: LFriend121@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 343

Message-ID: <960311211936_166199223@emout04.mail.aol.com>

In a message dated 96-03-11 08:48:49 EST, you write:

>

>Date: Sun, 10 Mar 1996 19:39:57 PST

>From: MK2112@aol.com

>To: impala@east.xsis.xerox.com

>Subject: Chrome intake pipe

>Message-ID: <960310223956_347441552@mail02.mail.aol.com>

>

>I was wondering if anyone that has done the econo intake air modification

>using the chrome 3.5" exhaust pipe considered this: The inside of the pipe

>has a very weak if any type of coating, therefore the possibility of

>corrosion is prevalent.  I've looked at several pipes at Pep Boys and some

of

>them contained visible but very fine patchy surface rust.  What would be the

>best thing to do? Paint might flake off, wouldn't want that to happen;

> though if left unchecked the possibility of rust being sucked in isn't good

>either.

>

>Mike K.

>'95 Black

>

>

Leave it the way is it.

------------------------------

Date: Mon, 11 Mar 1996 18:18:34 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Leakdown test blues!

Message-ID: <960312021834_75511.1220_GHJ147-1@CompuServe.COM>

Brought my car to the dealer today to confirm the 20% leakage in cylinder #8. I

told the service manager that the car was tested 3 different times and I'm sure

it was a ring problem in #8 etc.... I called about 11:30 and they had not

started it yet. Called about 3 Pm and was told it's all set.

What do you mean all set? Well we checked it out and it's running fine. Did you

do the leakdown test? No we did a compression test. Why didn't you do the

leakdown test? The mechanic says your car is running fine and has good

compression. Do you have a leakage tester? Yes! Didn't I explain about me having

it tested 3 times. Yes but the mechanic says it's fine. I want you to do the

leakdown test,  a compression test won't show you the problem because there is

oil on the rings and a compression will look good. Remember I told you about the

air coming out of the valve cover? oh yeah well we'll test it. Called back

again. Ok it;s ready. What!etc....

To close up this long story. I went to the dealer told him there is definattly a

problem and I'm not satisfied. Sorry but we feel the engine meets spec!

I called GM customer assistance and they will call the dealer tomorrow but told

me "We can't make the dealer do any tests they don't feel are necessary" Any

suggestions. "Sure call other dealers and see if one of the others will do it!"

What the he..?

Geeeze my heart is sinking in my chest and I'm wondering what to do.

So GM will call the dealer tomorrow and call me back with the conversation

details.

I'll keep you posted to the results.

Curt P.

------------------------------

Date: Mon, 11 Mar 1996 18:47:54 PST

From: Mbmarx@aol.com

To: impala@east.xsis.xerox.com

Subject: Re: IMPALA digest 342

Message-ID: <960311214754_243635342@emout10.mail.aol.com>

In a message dated 96-03-10 18:41:06 EST, you write:

Rob,

>But doesn't the system operate out of a fixed table at WOT and quit reading

>the O2 sensors? Wouldn't any performance enhancement (especially the free-

>breathing intake and exhaust stuff) cause this table to be a little (or a

>lot) lean?

My thoughts exactly, the feedback I hear and read indicates that the late

model LT1 responds well to increased fuel pressure. We have a local

company that reprograms your PCM on a dyno with before and after

hp and torque numbers. The people that have had it done (both f and b

bodies) have been totally satisfied with the results. Some had Hypertech

already loaded and still came away with hp increases and better "seat of the

pants feel". This company sez it is just about manditory that the regulator

be replaced with an adjustable type before tuning.

A recent post to this list indicated an Impala with the all the  "usual mods"

plus headers, 52mm throttle body, TransGo, 3:42 gears etc.and ran 14.9s.

We have cars in Fl that run this without headers, throttle body etc. but with

"just" the normal mods: cold air, Hypertech, 160 stat, airfoil. Scott Harris

what were some of your times? My point is that maybe when you do open

the intake and exhaust that at some point fuel (or lack of) becomes an issue.

Anyone have any thoughts on this...any F-body people out there?

Mike Marx

95 Black.....

------------------------------

Date: Mon, 11 Mar 1996 19:21:28 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: IMPALA digest 342

Message-ID: <960312032128_76165.1654_GHM120-2@CompuServe.COM>

In digest 342 Larry wrote:

>>>And placed a new order w/ Lee Chevy in CT and it saved me $800 (Thanks Kurt

John is very honest and wants to sell the car).<<<

You are very welcome. I am glad I was able to help.

As I said in my original post, John Gallichio at Lee Chevrolet

in New Britain, CT will order a 1996 Impala SS any way you want

it at only $100 over invoice. Tell him I sent you, you won't be

disappointed.

Larry, LMK when you take delivery. I hope you are able to find

a good dealer down where you are that will be just as honest

as the guys at Lee. BTW, the phone # for Lee Chevy is 860-229-0345.

Kurt

'95 Dark Cherry Impala SS.....bought from Lee Chevrolet at $100 over invoice in

September 1994...

------------------------------

Date: Mon, 11 Mar 1996 20:20:38 PST

From: Curt Platteborze <75511.1220@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Texas trip

Message-ID: <960312042038_75511.1220_GHJ150-1@CompuServe.COM>

I got map info from AAA today and I'll share the route with you. Keep in mind if

you would like to join up for this trip please Email me.

Boston

90w to Sturbridge MA

84w to Danbury CT

684s to White plains NY

287w to Morristown NJ

78w to Lebanon PA

81s to White Pine TN

40w to Littlerock AK

440s to

30w to Arlington

or:

90w to

84w to

95s to

78w 81,40,440 to 30

1800 miles! Anyone care to join me?

Give me a call if you want.

Work 508-988-3182  or  800-370-5050 ext 182

Home 617-899-6280

Curt P.

------------------------------

Date: Mon, 11 Mar 1996 17:27:31 PST

From: "J.R. LEVITZ" <4everGreen@msn.com>

To: impala@east.xsis.xerox.com

Subject: RE: IMPALA digest 342

Message-ID: <UPMAIL01.199603120149240195@msn.com>

Jim Fort writes In digest 342:

I took my ATI 9psi intercooled '94 Impala to the track yesterday, and

am very pleased with the results.  My 3 best runs:

60'   330   660  MPH  1000    1/4  MPH

---------------------------------------

1.985 5.550 8.468 84.15 11.030 13.194 103.83

2.078 5.669 8.598 85.32 11.123 13.270 104.48

2.008 5.588 8.519 85.85 11.039 13.196 103.60

These times are on the factory tires!  I also still have the 3.08 gear.

My next move to buy a pair of BFG Drag Radials and get into the 12's.

Jim ,Congratulations, that is fantastic!  A really good running Impala.

I have a program called "Quarter jr". It is a Horsepower/E.T. computer. After

entering all the variables, and based on the ET and MPH numbers, It says you

are making just shy of 450 horsepower!!! WOW. Let us know when you get in the

twelve's!

Regards John

------------------------------

End of IMPALA Digest 345

************************

IMPALA Digest 346

Topics covered in this issue include:

1) shift kits and pcms

by "Jeff Gatcha" <foxhound@txdirect.net>

2) Pcm's

by "Jeff Gatcha" <foxhound@txdirect.net>

3) RE: 1/4 mile attempt-6/8/96

by "Harris, Scott" <sharris@xyplex.com>

4) RE: usual mods 1/4 mile times

by "Harris, Scott" <sharris@xyplex.com>

5) Financing  my beast

by Joe Hudetz <hudetz@xnet.com>

6) GM svc

by "Jeff Gatcha" <foxhound@txdirect.net>

7) control arms

by Scott Mueller <73145.1566@compuserve.com>

8) F-body cal

by Scott Mueller <73145.1566@compuserve.com>

9) brake rotors

by Scott Mueller <73145.1566@compuserve.com>

10) "SHO" down at a stop light!

by JHNS81A@prodigy.com (MR GREG L SULLIVAN SR)

11) CORN filter?

by Mikado95@aol.com

----------------------------------------------------------------------

Date: Mon, 11 Mar 1996 22:02:57 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: shift kits and pcms

Message-ID: <199603120602.AAA26805@legend.txdirect.net>

Jeff,

Explain brutal. <G>  I am using the Hypertech programing.

Brutal as in ruining the transmission.

Scott Harris told me that the Z program would maybe be a problem, And if you

have the Z unit not to install the Trans Go kit. I already had the Hypertech

unit so went with the Trans Go to tighten up the shifts.

That is correct.  John Levitz at Evergreen said the same thing.

The hypertech doesnt mess with   shift "firmness" only raises the rpm

limits.   The Z-pcm programing Does increase the "firmness"

of the shift  and therefore the transgo is to be avoided with the

Z-pcm UNLESS you get it with the stock shift program.

I didnt know until today that the Hypertech unit DOESNT do anything

to the firmness of the shift.   Thats what had me confused.

I wondered why the hypertech worked ok with the transgo

and the Z-didnt.  Now I know...  Thanks for the post.

-----------

In the year of darkness, 2029, the rulers of this planet

devised the ultimate plan. They would reshape the future by

changing the past. The plan required something that felt

No pity, No pain, No fear. Something Unstoppable.

They created The TERMINATOR...

Jeff Gatcha

foxhound@txdirect.net

------------------------------

Date: Mon, 11 Mar 1996 22:12:27 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: Pcm's

Message-ID: <199603120611.AAA27178@legend.txdirect.net>

Just a note on Pcms,

I asked Scott H if he knew anything about the new hypertech program

and he said that the last he had heard,  the HyperTech "revised program"

is putting the shift points back to stock.  All you get is a low temp fan

turn on and rear gear correction for $375!

plus the 8-12 hp that goes with it that Henry in Houston reported.

Hardly seems worth it when the Z-pcm give you this for $300 (club price):

*Spark and fuel tables optimized to gain 26hp@3000 and 16hp@5000rpm. Yes,

premium fuel required.

* Coolant fan temps lowered

*A/C compressor WOT cut out.

*RPM and speed limiters raised

*Transmission shift recalibrating. Progressive firmness, IE light throttle is

stock and WOT chirps the tires. Quicker upshifts AND downshifts.

*Custom programming available for gears, blowers, heads, cams

So other than being able to put your programing back to stock, why

get the hypertech?

Im really interested in reasons as I havent bought either yet and have

all but decided to go with the Z.

I welcome your opinions...

-----------

In the year of darkness, 2029, the rulers of this planet

devised the ultimate plan. They would reshape the future by

changing the past. The plan required something that felt

No pity, No pain, No fear. Something Unstoppable.

They created The TERMINATOR...

Jeff Gatcha

foxhound@txdirect.net

------------------------------

Date: Mon, 11 Mar 1996 12:11:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Cc: "'ZTMP50A@prodigy.com'" <ZTMP50A@prodigy.com>

Subject: RE: 1/4 mile attempt-6/8/96

Message-ID: <314516BB@corvette.xyplex.com>

>>What is the best launch procedure?  I was thinking to drop it in 2 at

launch, feathering the pedal until the tires hook, run it within 500rpms

of red...and shift

to 3rd.  Whats the opinion??  What have you done with yours?  I know

to have about 1/4 tank of gas, and the possible removal of the spare,

but would like to turn as quick at time as possible.  Car is bone

stock now, but should have airbox opened by then..(time permitting).

Also, car only has 1600 miles on it currently.  I project that in

June, the car will still have less than 3000 miles on it.  Any

downside to run the car with this "low" mileage?.

The best launch I found in near stock form is to hold ~1,300 at the line

with your foot firmly on the brake.  Let off the brake on the second

yellow leaving the throttle at the 1,300 setting and it will rev on it's

own.  When the light goes green you will already be rolling, now squeeze

the throttle.  You should see a 2.3 or better short time.

If you mash the throttle instead of squeezing, you are going to spin the

tires instead of getting traction.  This means worse times but pretty

much the same MPH numbers.  In searching for engine power improvements

look at the MPH only.

If you have done some mods like PCM, intake AND exhaust mods you may want

to experiment with lower rpm launches, all the way down to idle even!

Whole on this subject, don't even think about messing with tire pressures

on the stock BFGs.  Leaving them in the 30's somewhere versus trying

something lower netted me zilch.  In fact traction seemed to get worse in

the mid-20's which says the tire was bowing upward in the center and only

the edges were on the ground.

Leave it in drive and concentrate on the launch.  The 2-3 shift is so far

down the track leaving it in 2nd versus letting it shift will not make

enough difference to worry about and may in fact create worse times.  The

torque of the Impala will pull 3rd gear through the lights better than

winding out 2nd.

As for the low miles, it may still be a little tight.  You could very

well see better numbers as it continues to break in.  At 3,000 miles I

doubt you'll hurt anything letting it see full throttle for 15 seconds at

a time.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Mon, 11 Mar 1996 12:11:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: usual mods 1/4 mile times

Message-ID: <314516BD@corvette.xyplex.com>

>>A recent post to this list indicated an Impala with the all the  "usual

mods"

plus headers, 52mm throttle body, TransGo, 3:42 gears etc.and ran 14.9s.

We have cars in Fl that run this without headers, throttle body etc. but

with

"just" the normal mods: cold air, Hypertech, 160 stat, airfoil. Scott

Harris

what were some of your times? My point is that maybe when you do open

the intake and exhaust that at some point fuel (or lack of) becomes an

issue.

============

My runs last year were low 14.7's in the 95's.  Stock exhaust manifold

with cats, rear end and throttle body.  The mods were HyperTech (which I

don't think helped), opened air box with K&N and my now famous

(infamous?) muffler delete option with resonators in place.  The removal

of the mufflers seems to be worth in the ballpark of 25 HP.

I would think that my car with headers and the 3.42s should get into the

14.4's.

I would be VERY careful with an adjustable fuel pressure regulator.  All

the tests I have seen say that while they work well on L98/TPI engines

but there is little to be gained and MUCH to be lost on LT1 engines.

Scott Harris

sharris@xyplex.com

------------------------------

Date: Mon, 11 Mar 1996 22:32:52 PST

From: Joe Hudetz <hudetz@xnet.com>

To: impala@east.xsis.xerox.com

Subject: Financing  my beast

Message-ID: <31451A94.7E88@xnet.com>

Hi all!  This is my first time posting (I have been lurking for some

time now), so let me introduce myself.  My name is Joe Hudetz (26 years

old, 6'5", 240 lbs, bust size...), I live in Western Springs, Illinois,

and my wife and I am soon to be the proud owners of a new 1996 Black

Impala SS. (YEAH!!!)  I placed my order on Jan 16, the car was

preferenced around Feb 22, and is arriving around Mar 20.  I am truely

grateful for all the information provided here, and even though I am not

as technically inclined as I would like, this group has given me the

confidence to try at least some of the modifications suggested to make

my beast extra-special.

This will be our first new car ever, and therefore the loan issue is new

to us.  In doing my homework, I have found that I have several options.

I can go through my bank, which offered me a 4 year $20,000 loan at 8%.

My insurance agent said she can beat that at 7.75% (through a local

bank).  The Chevy sales manager (who happens to be the finance manager

as well) says not to worry (uh-oh), that I should take care of all the

loan stuff when I come in to pick up the car, and that he will meet or

beat any rate I can get elsewhere!

So my question (not really an Impala specific question, but I needed an

excuse to introduce myself) is, what should I do?  My insurance agent

warns me against going through the dealer because they often include

strange (read not in the customer's favor) stuff in the loan agreement

and that they are only motivated by the commission they will make from

the bank that sold them the 'bulk' loan (she said they buy the

loan at around 6.5% and will give it to customers at as high a rate as

possible).  She is a great agent and I respect her judgement, but I'm

not sure she is an authority on auto loans (besides, she has to be

motivated by a commission too).  I don't want to offend the GM

sales/finance manager either, because I will be using the dealership for

service now and probably in the future.  This guy and the service

manager seem very helpful and willing to accomodate my requests.  For

example, when I mentioned the problem with the differential gasket, they

were very open to doing the work before I pick up the car using parts

(Fel-Pro) provided by me, no questions asked (they approximated about a

one hour labor charge).  What to do?

Joe '96 Black SS very soon...

------------------------------

Date: Mon, 11 Mar 1996 22:45:08 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: GM svc

Message-ID: <199603120644.AAA28195@legend.txdirect.net>

Dont feel Bad Curt,

I called GM cust service and complained about the ball joint recall

and the control arm recall for the 9C1 and taxi but NOT THE IMPALA.

Girl just kept parroting the party line,  "We know what cars they are

by the vin number" yadda yadda yadda .   Wouldnt listen to my reasoning

at all.  didnt care that like 8 of 10 impala owners have under torqed ball joints

and NO grease in them just like the recalled cars.

All she would do is tell me that there are no recalls on MY car.   NO SHIT!

Bruce,  must have had the only operator at GM who gives a rats A$$.

-----------

In the year of darkness, 2029, the rulers of this planet

devised the ultimate plan. They would reshape the future by

changing the past. The plan required something that felt

No pity, No pain, No fear. Something Unstoppable.

They created The TERMINATOR...

Jeff Gatcha

foxhound@txdirect.net

------------------------------

Date: Mon, 11 Mar 1996 23:40:58 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: control arms

Message-ID: <960312074057_73145.1566_FHD44-1@CompuServe.COM>

Martin,

>As long as no load is placed on the control arm, I agree!  However, when

the car is cornering and one side of the swaybar is "pulling down" on a

control arm, all of that load will have to go to the top of the control arm,

not the bottom.

I don't think the entire load is carried by the head of the bolt at all!

Note that the bar does not really just pull straight down, it twists, which

means it actually rotates around an axis central to the bar itself. Thus

when you corner, the front part of the bar tries to come down while the rear

part tries to go up. The bar is clamped against the metal plates, which are

pressed against both the arm and the metal sleeve within. Trust me with this

arrangement there is no way the top of the bar carries all of the load! It

is only thin sheet metal, and is actually much thinner than the bracket used

on the original design. If you were correct, then the bolt would simply pull

right through the top of the arm, much easier than it pulled the nut through

the bracket (which actually happened to me in my '88 9C1).

>Because the new bottom plate is only connected to the control arm by the

clamping of the swaybar bolt and friction, as the swaybar connecting bolt

strains, the plate will tend to pull away from the bottom of the control arm

(or, more precisely, be less tightly clamped), thereby reducing any

stiffness that it added to the control arm. In this situation, I think it is

fair to say the new design is less stiff laterally than the original design,

where the bracket was, of course, always rigidly -- if imperfectly --

attached.

Again I would not agree. The old bracket was only attached with two bolts on

each side, through the thin sheet metal of the arm. All of the load was

placed on these bolts as a shearing load transferred from the swaybar. The

new design plates are very tightly clamped along the bottom of the arms, and

are even contoured to mate better with them. The sleeve allows the load to

be transferred from the top to the bottom of the bolt, and I doubt that it

stretches at all under load. The sides of the arm are effectively locked in

by the curled up edges of the plates which are tightly clamped to them. Now

I definitely agree with you in that a fully boxed arm would be the strongest

possible setup by a long shot, but I also believe that this new setup is

stronger than the old one in every way.

>If lateral stiffness of the rear control arms is what makes the rear axle

move under hard cornering, then I think a box section control arm would be a

much better way to (1) improve handling and (2) provide a better connection

for the HA bars.  Welding on a bottom section plate but leaving the stock

bushings would also avoid the undesirable effects of the polyurethane

bushings.

This I agree with. What would be trick is to use the new bars, and weld in a

full length plate matching the smaller plates that come with the kit. The

sleeves should be tack welded inside the arms between the plate and the

holes on top to transfer the bolt load through the arm, and this would be a

very strong setup! Of course this is essentially what the Hotchkiss arms

already offer, so you could just use them instead and live with the urethane

bushings. Scott.

------------------------------

Date: Mon, 11 Mar 1996 23:41:05 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: F-body cal

Message-ID: <960312074104_73145.1566_FHD44-2@CompuServe.COM>

John,

>I find the 5800 RPM shift point feels pretty much ideal to me, but then

again I'm using the F-car engine in my SS wagon! Based on this I assume the

difference must be that you are getting past the range of that wimpy B-car

cam! (just kidding guys!!)

OK, that does it! Now I'm just going to HAVE to order those bolt-on LT4

heads that are listed in the GM performance parts catalog! <g> Well,

actually that might be a project for next winter. I've got too much stuff

going on with the car right now, and I would like to stop working on it and

actually drive it a little this summer. <g>

>As far as fan turn on temps I always recommend an aftermarket adjustable

fan controller. I've used and installed these on many cars including my LT1

wagon and like the flexibility of turning a knob for the temp I want. If I

want to change thermostats for summer/winter use I can set the fans

accordingly.

Is the controller you are talking about one that is dial adjustable like the

Hayden units I have seen? What does the thermostatic sensor actually look

like with these units and where does it mount? What I did was to use a

thermostatic switch wired to the fan relays. I have a small switch in the

relay box that allows me to have the switch turn on either the primary or

secondary fan at the intended temperature (203 degrees in my case), or

disable the switch completely allowing normal fan operation. At the time I

installed the switch (in the LH cylinder head plugged hole), the 203 degree

switch was all I could find, now I can get even lower temperature ones. This

is not adjustable, but I always run the same thermostat anyway and don't see

any need for adjusting once I settle on an activation temperature. Scott.

------------------------------

Date: Mon, 11 Mar 1996 23:41:24 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: brake rotors

Message-ID: <960312074124_73145.1566_FHD44-6@CompuServe.COM>

Kurt,

>Does this material wear off as the rotor wears, or is it impregnated

completely thru the rotor?

I think you have a misconception here. You must be thinking that the rotors

are coated even on the braking surface! The coating, which is actually some

type of heat resistant paint on the front rotors and a metallic ceramic type

coating on the rear, is applied everywhere except of course the surface upon

which the pad rides, which would inhibint proper braking. Most brake rotors,

such as those on the 9C1 cars, do not have any coating or paint anywhere,

and the hub area and edges rust within days after manufacture, and of course

look horrible through nice wheels like on the Impala.

I noticed that GM has begun to apply a coating to the rotors and even the

calipers on the F-cars. The '93 and '94 models did not have any coating and

again the rotors and calipers generally looked pretty bad after a short

time. I coated the hub and edge area of my '89 Turbo TA, as the rotors were

bright orange when delivered new!

>Will this coating prevent rust on the rotor suface? Like when the car sits

for a day or two in damp rainy conditions, and there is usually some rust

starting to form.

There is no coating applied to any rotors where the brake pad rides, and the

rotor's pad surface will indeed rust within a few days, or even a few hours

in a damp climate. The first application of brakes will of course wipe that

rust right off! Scott.

------------------------------

Date: Mon, 11 Mar 1996 01:03:26 PST

From: JHNS81A@prodigy.com (MR GREG L SULLIVAN SR)

To: impala@east.xsis.xerox.com

Subject: "SHO" down at a stop light!

Message-ID: <013.03355756.JHNS81A@prodigy.com>

I just thought maybe other Impala owners would be amused by the

"confrontation" I ran into this morning.

This (true) story actually began about 2 years ago.  On many occasions

I noticed a new 1994 Taurus SHO (green) on the freeway in the morning

getting off at the same off-ramp as me.  Then I noticed that the same

Taurus would park next door to the building where I work.  I suppose I

noticed the SHO because I was a little envious.  My car at that time

was a 1988 T-Bird Turbo-Coupe 5 speed.  I know the SHO had 220 HP while

my T-Bird had 190 HP.  Well, one morning I pulled up behind him in my T-

Bird at a traffic light.  Traffic was very light and I had a feeling he

was going to take off hard when the light turned green.  I was ready

and also took off hard to see if I could keep up with him.  Gradually,

he pulled further and further ahead.  I saw him looking in his mirror

with obvious satisfaction as he gradually pulled away.  Now, for the

last couple of years I have had a reminder of that defeat every time I

look over at that SHO parked next door to my work.

About 2 weeks ago I pulled up to a light (on my way home) in my '96 SS

and noticed that same SHO on my left.  Normally this would have been an

excellent opportunity for a re-match.  Unfortunately, the pavement was

wet from an earlier rain.  Like a dummy, I tried to keep up with him

when he took off hard from the light.  His front wheel drive Taurus

easily pulled away from my rear wheel drive Impala as I encountered

serious wheel spin at about 1/3 throttle.  Even though it was not a

meaningful race, I felt he had done it to me again.  I was 0 for 2 with

this guy.

This morning I finally got a clear shot at this guy in the SHO.  As I

got off the freeway I looked in my mirror and saw the Taurus pull up

behind me at the light.  I made my left turn with him right behind me.

As we approached the next red light, I took the number 1 lane (closest

to the center) while he took the number 3 lane.  I was fairly sure he

would launch hard from the green light.  I hesitated when the light

turned green to make sure he was going to "go for it".  He went

blasting off the line with everything he had.  As soon as I was sure of

his intentions, I stood on it.  At the point where I hit the gas, he

was about 1 car length ahead.  We were on clean dry pavement with no

cars or other hazards ahead of us.  To my surprise, my SS laid about 15

feet of rubber off the line. The very loud tire squeel was not

something I wanted or expected.   I caught the SHO just before my SS

hit 2nd gear.  Then after hitting 2nd, the Impala pulled decisively

ahead.  We shut down at about 80 MPH.  I was about 2 car lengths ahead

when we shut down.

Now, when I look over into the neighboring parking lot and see that

green SHO, I will have a big smile on my face :)  :)  .  I suppose the

SHO guy will think twice before tangling with 2 ton Chevys.

BTW - I just got my SS newsletter.  Very well done.  I will be sending

my $20 membership fee ASAP.  Thx a bunch!

Also, based on what I learned from our web site, I asked my dealer to

grease the caliper pins on my left front brakes to stop the clicking

and popping I was hearing during slow speed stops.  He did what I asked

and no more clicking and popping!

Happy motoring!

Greg Sullivan.  '96 Black SS, tinted windows, chromed wheels, 12 CD

player. (no other mods).

------------------------------

Date: Tue, 12 Mar 1996 01:09:24 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: CORN filter?

Message-ID: <960312040923_444241766@mail02.mail.aol.com>

>To Scott and who ever else that had the K&N Corn filter:

>

>	I am just wondering for those of you who had convered to the K&N

>corn filter, what do you do with the air hose that's attach to the air

>box now that you have the air box assembly removed altogether to

>faciliate the filter?

Personally I prefer the soybean filter, but I hear the wheat filter works

well also.......

=)

Randy

------------------------------

End of IMPALA Digest 346

************************

IMPALA Digest 347

Topics covered in this issue include:

1) Much more on control arms

by mbailey@rinet.hunton.com (Bailey, Martin)

2) additional infor on dealer

by John George <johng@alpha.acast.nova.edu>

3) Test Drive Too

by "Brett A. Rudolph" <bngwip@mail.phoenix.net>

4) GM service???

by David <71223.325@compuserve.com>

5) Arlington Shutdown

by Erich Arndt <EARNDT@mednet.swmed.edu>

6) GM Prices from Fuel Injection Specialties

by Fuel Injection Specialties <fuelinjection@fuelinjection.com>

7) HA Swaybars & Chicago Area Stuff

by "John P. Kelly" <76400.2245@compuserve.com>

8) E-mail Address Change

by Erich Arndt <EARNDT@mednet.swmed.edu>

9) AIRLINE UPGRADES

by JGreen5275@aol.com

10) RE: is my car stuck ...??(followup)

by John Latrobe <racerx@jagunet.com>

11) Reintroduction & Misc. Stuff

by Scott Widmer <srwidmer@cle.ab.com>

12) Good race!

by Loren Peterson <equ_rino@cencom.net>

13) More Internet Address' FYI

by Loren Peterson <equ_rino@cencom.net>

14) PCM programs

by Scott Mueller <73145.1566@compuserve.com>

----------------------------------------------------------------------

Date: Tue, 12 Mar 1996 04:14:56 PST

From: mbailey@rinet.hunton.com (Bailey, Martin)

To: impala@east.xsis.xerox.com (Impala1)

Subject: Much more on control arms

Message-ID: <1996Mar12.070600.1049.116909@rinet.hunton.com>

Scott:

I have been researching an alternative to the urethane bushings, which

are available from Hotchkis, Energy Suspension, and Performance Suspension.

I found that Edelbrock makes a *very* trick rear control arm for the

F-bodies and for the Mustangs.  It is a box section arm with greaseable

bushings.  Edelbrock's Performance Suspension Division's home page (sorry, I

didn't bring the URL to work) does not show that they make a similar product

for the B-car.  If they did, this would have to be the ideal solution!!

Stiff but no squeaks (why does that sound so bad?).  I have e-mailed

Edelbrock to find out if they do make rear arms for the B-car and, if not,

if they plan to.  I will keep the list posted on what I find out.

In the meantime, has anyone had experience with some of these other

polyurethane bushings?  Just how noisy are they?

Martin Bailey

'95 GG with full soybean filters.

>This I agree with. What would be trick is to use the new bars, and weld in

a

>full length plate matching the smaller plates that come with the kit. The

>sleeves should be tack welded inside the arms between the plate and the

>holes on top to transfer the bolt load through the arm, and this would be a

>very strong setup! Of course this is essentially what the Hotchkiss arms

>already offer, so you could just use them instead and live with the

urethane

>bushings. Scott.

------------------------------

Date: Tue, 12 Mar 1996 05:18:51 PST

From: John George <johng@alpha.acast.nova.edu>

To: impala@east.xsis.xerox.com

Subject: additional infor on dealer

Message-ID: <Pine.3.89.9603120832.A24903-0100000@alpha.acast.nova.edu>

In the message I wrote yesterday, I failed to include the representative

at Ed Morse Chevrolet in Lake Park to contact!  Her name is Becky

Chabler at 407-844-5262.

John George

johng@alpha

------------------------------

Date: Tue, 12 Mar 1996 05:21:07 PST

From: "Brett A. Rudolph" <bngwip@mail.phoenix.net>

To: impala@east.xsis.xerox.com

Subject: Test Drive Too

Message-ID: <1.5.4b11.16.19960312072530.3e279b08@phoenix.net>

First, I will report on the status of my SS. It has been preferenced but I

don't know the build date yet. My dealer said he should know today.

Second, My adjuster came buy yesterday to look at my wrecked Firebird. The

damage is going to exceed $6000.00. State Farm, the test drivers insurance

company is on the hook for my car. I called them yesterday and they said

someone would call me yesterday and if not, I should call them today. State

Farm Sucks! Just like a nieghbor....or something. The girl was 20 years old

and is currently still alive....

Brett

Brett A. Rudolph

96 SS Black

(still waiting)

If you build it,

I will come.

------------------------------

Date: Tue, 12 Mar 1996 06:00:28 PST

From: David <71223.325@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: GM service???

Message-ID: <960312140027_71223.325_GHL86-1@CompuServe.COM>

Arrrrggghhhhhhh!!!!!!!!!!!!!!

I used to have a good GM service department..Used to.

The service writer who I have worked with for three years (Corvette and now Impala,

was fired last Friday.    Word from my very reliable insider at the dealer

is that my service writer spent too much time with customers, and not enough

time selling "Service."    You could always count on this person to

help you, listen to you, and go above and beyond the call to find the problem.

Apparently, satisfying the customer is not what this dealership wants.

I stopped in yesterday (Monday) to talk to the new service guy about

some of the usual problems.  I need an alignment, and I am now having the

noise from the rear end.    When I started talking to the new guy, it became

obvious that the dealer hired a "Glad hander" who's sole function is to sell, not

listen to the customer.

I asked if they did "thrust angle" alignments, and if I could see the specs

when they did it.  He just looked at me funny, and gave me the

standard line about the mechanics will look at it a adjust as necessary.

When I asked if he knew Nyting about the recent recalls on the lower control arms, etc.. on the service vehicles, he told me that it's o

obviously not a problem on the Impalas, or GM woud have

included them too.....

Finally, I asked a bout installing the 3.42 gears. He said "why would

you want to do that???" and kind of chuckled at me.

That's it!!!!! I can't take it any more!!!!!!

Thank you for indulging my venting.

------------------------------

Date: Tue, 12 Mar 1996 06:35:50 PST

From: Erich Arndt <EARNDT@mednet.swmed.edu>

To: impala@east.xsis.xerox.com

Subject: Arlington Shutdown

Message-ID: <s145381e.021@MEDNET.SWMED.EDU>

Bad news, boys and girls;

The Dallas media is reporting this morning that the Arlington plant is

scheduled to be "shutdown" sometime today.  Depending upon which

newsie you believe, this can range from partial to full and could occur as

early as noon.  Stay tooned....

Dave, a six speed conversion is going to set you back about  $7500 and

takes some substantial mods to the transmission hump and tunnel area.

Also, someone was asking about security systems.  That's a source of

great debate but remember two things.  (1) No security system is going

to stop a real professional thief, but find a good quality one that disables

the ignition/starter and fuel pump circuit.  (2) Don't waste your money on

gadgets like the now famous (infamous?) "Club".  Any vermin can get

around one of these in under 20 seconds... (I won't elaborate on a public

forum for obvious reasons, but I'll be glad to take personal emails on the

subject).

Thanks,

Erich Arndt

'94 Impala SS

------------------------------

Date: Tue, 12 Mar 1996 06:52:08 PST

From: Fuel Injection Specialties <fuelinjection@fuelinjection.com>

To: impala@east.xsis.xerox.com

Subject: GM Prices from Fuel Injection Specialties

Message-ID: <19960312145203515.AAB183@FIS>

To All Impala Owners,

Know you "guys" buy alot of GM parts, and unless you have connections

with a dealer, you might check with us before buying parts.  We get such

a discount from GM that we should be able to save you money.  Jeff Gotcha

will tell you as he has checked on some prices with us; foxhound@txdirect.net

Regards,

Donald

Fuel Injection Specialties

http: www.fuelinjection.com

email: fuelinjection@fuelinjection.com

1-210-654-0774

------------------------------

Date: Tue, 12 Mar 1996 06:55:22 PST

From: "John P. Kelly" <76400.2245@compuserve.com>

To: Impala List <impala@east.xsis.xerox.com>

Subject: HA Swaybars & Chicago Area Stuff

Message-ID: <960312145521_76400.2245_HHL86-1@CompuServe.COM>

Well all, I officially took the beast out of the garage this weekend!!!  The LT1

beast sure beats driving a company minivan for the winter.  I just received my

new Eibach Springs & Bilstein Shocks at an unbelievable price through the

NAISSO.  Make sure to check the new newsletter for the info.  I also got my

Crane roller rockers & springs and hope to get this installed before the Dallas

meet.  Which leads me to a couple of points.....

1.  I am going to takeover this Herb Adams Sway Bar Discount thing for Troy W.

Troy sent me the following list.  If you are on the list, please drop me a note

to confirm you are still interested.  If you are not on the list, but are

interested drop me a note too!!  The more people, the better bargaining

potential.  ASAP!!!!  (I am in a hurry to get my bars installed, so the sooner

the better)

1.  Mark Boggs			6.  Jay Griffiths

2.  Bruce Gordon		7.  Tim Laurent

3.  Troy Willrick		8.  Todd McMurrey

4.  John P. Kelly		9.   Mike Longeran

5.  Andrew Cohen		10. Mike Sundstrom

2.  All Chicago area Impalers,  I am waiting to see when the first car

show/cruise night at STRAT'S is.  This is a great place to meet and swap stories

etc.  Keep an eye out for the details.  (Strat's is on Manheim Rd in Franklin

Park)

3.  Trip to Arlington.   Mike Sundstrum, Scott Mueller and myself are all

driving down to the Super Chevy Show together, if you are interested in joining

us, drop me a note.

That's all folks

John P. Kelly

76400.2245@compuserve.com

------------------------------

Date: Tue, 12 Mar 1996 07:03:24 PST

From: Erich Arndt <EARNDT@mednet.swmed.edu>

To: impala@east.xsis.xerox.com

Subject: E-mail Address Change

Message-ID: <s1453e97.066@MEDNET.SWMED.EDU>

FYI,

Recently I've been corresponding with a BUNCH of people about the

Arlington meet.  There have been many questions and lots of interest;

PLEEEZZZZ!!!  Let Troy or Matt know if you're coming!  If you think you're

not coming for some reason, think some more before you give in.  We all

want to see you here in Arlington!!!

Be aware that I now only have one E-Mail address

(EARNDT@MEDNET.SWMED.EDU).  The EARNDT@AIRMAIL.NET address

is no longer valid. (I had to "terminate their services" for one too many

busy signals).

Thanks,

Erich

------------------------------

Date: Tue, 12 Mar 1996 07:32:25 PST

From: JGreen5275@aol.com

To: impala@east.xsis.xerox.com

Subject: AIRLINE UPGRADES

Message-ID: <960312103223_166626975@emout09.mail.aol.com>

IF ANYONE IS FLYING FROM ORLANDO TO ARLINGTON ON DELTA., I HAVE 4 FIRST CLASS

UPGRADES, I'M TAKING A DIRECT FLIGHT ON THE 19TH AT 7:15 AM AND ARRIVING IN

DALLAS AT 8:45 AM. ALSO IF ANYONE IS INTERESTED IN SHARING A RENTAL CAR WITH

ME, I'LL PAY HALF. PLEASE LET ME KNOW. I CAN ALSO MAKE ALL THE RESERVATIONS

FROM HERE, IF YOU'D LIKE, BESIDES I GET 500 FF MILES IF I DO. HOME PHONE 407

328 8912 FAX 407 323 3615

------------------------------

Date: Tue, 12 Mar 1996 07:39:21 PST

From: John Latrobe <racerx@jagunet.com>

To: "'impala@east.xsis.xerox.com'" <impala@east.xsis.xerox.com>

Subject: RE: is my car stuck ...??(followup)

Message-ID: <01BB1000.4595A4E0@dlup-a27.jaguNET.com>

>>Shamu my 2TONLT1 seems to be stuck in what would be called

>>performance mode shifting.  She won't go to fourth gear until

>>52mph no matter what and won't "lock up" until 61mph.  I have

>>stock gears and stock PCM programming.  The only serious

>>performance mods are the fresh air airbox cutout, no homeplate,

>>no muffler thing.

>

>The same thing happened to me and I haven't done any mods, yet :-)

>Two dealers basically told me I was full of it, the car was supposed to

>act that way. After I showed them the shift chart from the manual they

>agreed to replace the PCM. It shifted exactly the same afterwards.

>After 1 month, the 4th gear shift occurred correctly 50% of the time.

>A month after that, it shifted right about 90%, and now (3 months) it

>shifts correctly all the time.

>According to the manual, only two inputs affect the shift points:

>the throttle position sensor and the vehicle speed sensor.

Ok, me and my big mouth.  The day after I originally responded to

Franklin Poole's post, my tranny went back to its old tricks.  For

the past week I'm back to a 50/50 routine.  It's most aggravating when

I'm doing 60 and it's impossible to accelerate without the tranny

dropping back to third.  Lately, it's downshifting to third and then

hesitates like it wants to go to second, but then grabs third again.

The wife complains that I'm driving like a maniac. "Honest hon, it's

the transmission."

Once the weather warms up I'll attach a remote voltmeter to the TPS

to see if I can narrow down the culprit.

John Latrobe

'96 Impala SS, Black

------------------------------

Date: Tue, 12 Mar 1996 07:49:35 PST

From: Scott Widmer <srwidmer@cle.ab.com>

To: impala@east.xsis.xerox.com

Subject: Reintroduction & Misc. Stuff

Message-ID: <31459D0F.255D@cle.ab.com>

Howdy,

Haven't posted anything in quite awhile and thought I'd best

re-introduce myself to the group before the Arlington field trip.  Scott

Widmer is my name (yes, another Scott!).  I reside, work, and read these

wonderful digests in a suburb of not so sunny (and pretty darn cold this

time of year) Cleveland, Ohio.  I've been reading these here pages

regularly since August of '95- posting messages sparingly...until now.

So beware!

My Impala was delivered on February 21 during a week long thaw (thank

the Big Guy!).  I was really afraid it would be delivered in the midst

of a blizzard and that the service monkeys would do their PDI test drive

in all of that slop and slush which blankets our roadways during

majority of the winter (I too am extremely anal about my cars).  Anyway,

I had barely enough nice weather to get it home and cleaned up (service

guys were specifically instructed not to wash the car).  With barely 100

miles on the clock, it's been parked in the garage ever since (I hate

winter!).

Anyway, I am looking forward to making the pilgrimage back to Shamu's

homeland of Arlington.  So count me in and, most likely, my Impala too

(need to break it in a bit more -weather permitting- before the long

freeway trek).  Incidentally, is anyone else from the Ohio area making

the trip??  With their Impalas??

On a separate note, I am interested in loading the "export" PCM

algorhythm in my beast.  And yes, I'm fully aware of the implications

(tampering, emissions legalities, etc.).  Besides, I might actually take

it overseas- yeah, that's it.

Anyway, how might I go about obtaining the export program and then

getting it loaded??  Any ideas (Scott M.)??  I am guessing that I would

need a VIN from an exported SS for starters.  I can't imagine that the

Chevy service departments would be any too helpful in this whole

process.  Would the aftermarket guys (Z-industries, Hypertech,

Evergreen) be able (and willing) to create such a program??  Any

thoughts, advice, or input on this matter would certainly be

appreciated.

Well, I think I've tied up enough bytes for one day.  Thanks for

listening.  Looking forward to your responses.

-Scott Widmer

srwidmer@cle.ab.com

------------------------------

Date: Tue, 12 Mar 1996 07:50:12 PST

From: Loren Peterson <equ_rino@cencom.net>

To: jhns81a@prodigy.com

Cc: impala@east.xsis.xerox.com

Subject: Good race!

Message-ID: <31459D34.7FE1@ns.cencom.net>

Greg,

Sure did enjoy your race story.  Hey, I had a '88 Black Turbo Coupe 5

spd.  I loved that car, in fact, after the SS I think I liked it best.

I also owned several SHO's.  They were okay, but just didnt have

character.  I will never forget the time my nephew had just bought a new

Transam.  I think it was an 89, but it had a 305.  I told him that It

wouldnt out run my Ford.  He said, hell no your Mustang 5.0 has Nitros.

I said nitros or not it would not touch my Mustang, but what I meant

was his car wouldnt keep up with my 4 cyliner Tbird.  To make a long

story short, we went out to the local 1/4 mi blacktop and I dusted him

off with the 4 lung bird.  God I love it.

BTW there is a guy where I work (Maytag Corp) that has a Green SHO, if

ever we meet, he is history!

Good Luck Greg!

Loren DCM96

------------------------------

Date: Tue, 12 Mar 1996 07:53:26 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: More Internet Address' FYI

Message-ID: <31459DF6.A43@ns.cencom.net>

Here are a couple of address' I just found.

http://www.valvoline.com

http://www.motorville.com/kn_home/kn.html

Mr. Monte owner, maybe you could

get a location near you from the above K + N address?

If not, I have a 1-800 # around here someplace for K+N.

Loren DCM96

------------------------------

Date: Tue, 12 Mar 1996 08:06:07 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: PCM programs

Message-ID: <960312160606_73145.1566_FHD50-1@CompuServe.COM>

Jeff,

>I asked Scott H if he knew anything about the new hypertech program and he

said that the last he had heard,  the HyperTech "revised program" is putting

the shift points back to stock.  All you get is a low temp fan turn on and

rear gear correction for $375! plus the 8-12 hp that goes with it that Henry

in Houston reported. Hardly seems worth it when the Z-pcm give you this for

$300 (club price):

The Hypertech does much more than that! In fact it does offer at least a 10

HP improvement (1 mph in the quarter mile) over the stock settings. I have

verified this in my own back to back testing. This is primarily from the

full and part throttle ignition advance and fuel changes. The ability to go

back and forth from stock to modified is also a BIG plus in my estimation.

>*Spark and fuel tables optimized to gain 26hp@3000 and 16hp@5000rpm. Yes,

premium fuel required.

Hypertech also alters these tables, but I cannot vouch for the Z-industries

HP claims. Their PCM would have to be worth 2 to 3 mph in the quarter to

back up that statement, which I tend to doubt (but would be happy to have

proven to be true!). I don't believe most manufacturers HP gain claims, if I

did, then why get any of these PCM reprogrammers when an "airfoil" and some

"power pulleys" will get you even greater (claimed) results! <G>

>* Coolant fan temps lowered

Hypertech does this.

>*A/C compressor WOT cut out.

The STOCK PCM calibration does this!!

>*RPM and speed limiters raised

Hypertech does this also, and in the case of the rpm limiter this is

something I absolutely DO NOT agree with or recommend! The engine is totally

out of breath and even into valve float at the stock rpm limiter, so raising

it will only allow you to destroy your engine more easily with serious

float. If I were to have anybody make a custom program for me, the rpm

limiter would be STOCK, unless I go with the aluminum LT4 heads.

Also, rasing the speed limiter would be great for Caprices, which have a 108

mph limit, but the Impala and most 9C1 cars already have a 154 mph limit,

which is pretty high already. Raising the limit to 200 mph or more sounds

cool and makes great conversation, but how many of you have actually driven

over 154 mph in your Impalas? <g>

>*Transmission shift recalibrating. Progressive firmness, IE light throttle

is stock and WOT chirps the tires. Quicker upshifts AND downshifts.

This is something that Hypertech currently does not do. However, they are

coming out with this, and by simply changing a small flash memory chip in

the Power Programmer device (which they can send you), you can then update

your car to the new program without removing your PCM. The ability to update

the car yourself is a major benefit. I would like to see the others like

Z-industries and Superchips offer upgrade chips for the Hypertech Power

Programmer! With the other guys, I feel the need to purchase a spare PCM for

all of the back and forth shipping, which significantly adds to the cost of

the whole deal. Also, they charge $300 initially, but what about later when

you change your rearend and have to update it again? What about when you

install LT4 heads and now want the shift points raised? What if you decide

to change the fan temperatures? What if you want to incorporate the fixes in

an updated factory calibration?

For example, I have had my Hypertech for over a year. Of course I was able

to use it to reprogram the PCM initially right in the car, and did not have

to remove the PCM. I was also able to go to the strip and perform back to

back testing with the stock program and the Hypertech enhanced program, in

fact I was reprogramming the car back and forth in the staging lanes!

Since I initially got the programmer I have changed the rear axle ratio,

which I was able to reprogram the PCM for without having to send my PCM back

in for an update. After that I installed the 1.6 roller rockers, and

immediately noticed valve float at the higher rpm shift points the Hypertech

programmer set. I was able to return the PCM to stock to verify that the

float was greatly reduced at the stock shift points, but the float still did

not go completely away. I changed valvesprings, which has totally eliminated

the float at the stock shift points, and was even able to verify that the

higher rpm shift points only occasionally now have only the slightest float

occur right at the shift.

Now I am switching to the F-car PCM program and larger MAF, which actually

seems to perform better than the B-car PCM program with the Hypertech mods.

Fortunately the newest revision of the Hypertech programmer works with both

F and B-cars!, which means that I can now use it to update the F-car PCM

program as well. If I want custom programming, again all I have to do is to

call Hypertech and they can send me a chip and I can reprogram the car with

the new settings, again without having to send them my PCM.

My point is that to me the car is an evolving thing, and if I had the

Z-industries programming, I would have REQUIRED a spare PCM to be shipping

back and forth to them. When I bought the Hypertech programmer, nobody else

could handle the LT1 PCM at all anyway, so I did not have much choice at the

time. Now that others such as Z-industries and Superchips can reprogram the

LT1 PCM, there are more choices, but having to send them your PCM (which

renders the vehicle inoperable) is a royal pain! Not to mention that I have

been able to do several tests on the shift points, and valvetrain mods that

I would otherwise not have been able to do otherwise, without the ability to

return to stock.

Another point, do you realize that there are 4 DIFFERENT factory PCM

calibrations for the '94 Impala, and 3 DIFFERENT calibrations for the '95?

The '96 currently has only 1 calibration. If you have a '94 or '95, which

one is the Z-industries program you will get based on? What happens when the

factory releases a newer calibration with another fix?

With the Hypertech unit, you can return your vehicle to stock, take the car

to the dealer to have the latest factory PCM calibration installed, and then

use the Hypertech module to modify THAT calibration! In fact, if you had a

'95, and wanted the 2nd gear start function, you could install the first

revision factory PCM calibration, and then even after Hyperteching it you

would have that feature!

The difference between the Hypertech and the others is that it dynamically

modifies whatever PCM calibration is in the car. In other words, it uses the

existing stock calibration as a base (whatever that may be) and modifies it

from there. This plus the ability to return the vehicle to stock allows you

to get the PCM calibration updates from your dealer and then install the

Hypertech mods on top of them.

*Custom programming available for gears, blowers, heads, cams

Hypertech does that also. Did you know that John Lingenfelter, arguably the

best LT1 family engine builder on the planet right now, uses Hypertech for

all of his programming mods! In every test I have seen comparing a

Lingenfelter car to other tuner cars, his always come out on top, not only

meeting, but exceeding his HP claims.

>So other than being able to put your programing back to stock, why get the

hypertech?

I think I have answered that by now! <g>

>Im really interested in reasons as I havent bought either yet and have all

but decided to go with the Z.

Don't get me wrong, I am not saying anything bad about either Z-industries,

Superchips, or Hypertech. I can say that Hypertech was first with LT1 PCM

mods, and the Programmer they have allows you to modify the PCM yourself,

which the others definitely do not have. I have heard great things about

Z-industries for some time now, but then I have heard of at least one

problem situation as well. Hypertech is not perfect either, as they

currently don't offer shift improvements, however this is coming. Superchips

has the advantage of being able to run your car on a chassis dyno for custom

tuning, provided you can get to one of their dyno facilities. They have

straightened out some highly custom applications that the others could not

do.

I agree that this is a tough choice! Personally I would like to try them

all. <G> Scott.

------------------------------

End of IMPALA Digest 347

************************

IMPALA Digest 348

Topics covered in this issue include:

1) Scott Widmer from Cleveland

by aas@riker.neoucom.edu (Anuj A. Shah)

2) Decal for Arlington

by Loren Peterson <equ_rino@cencom.net>

3) New member introduction

by mweber@westpub.com

4) The Arlington plant is officially closed

by Matt Platts <mplat@airmail.net>

5) Corn Filter!  Oops

by Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

6) Superchips Programming

by Stewart McLellan <stewartm@magi.com>

7) 94 Formula

by ZTMP50A@prodigy.com ( DAVID NMN)

8) Wives and the SS

by Loren Peterson <equ_rino@cencom.net>

9) Plant Closings

by Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

10) HAPPY BIRTHDAY

by Rock528@aol.com

11) SUPER CHEVY UPDATE

by Rock528@aol.com

12) Intake Mods - Gates Hose.

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

13) Speaking of the 1LE elbow...

by "Michael C. Calanan" <calanan@karloff.fstrf.org>

14) PCM WARS

by "Jeff Gatcha" <foxhound@txdirect.net>

----------------------------------------------------------------------

Date: Tue, 12 Mar 1996 08:29:24 PST

From: aas@riker.neoucom.edu (Anuj A. Shah)

To: impala@east.xsis.xerox.com

Subject: Scott Widmer from Cleveland

Message-ID: <9603121629.AA15307@riker.neoucom.edu>

This is directed to Scott:

Congrats on your new Impala........I also live in a suburb of

Cleveland (North Royalton), where this winter has been hellish.  If you don't

mind me asking, where are you?----I'd LOVE to take a look at your beast.  I'm

currently in graduate school, and as soon as I'm out and making money, my

mind is set on acquiring my own SS.  See ya later......

Anuj A. Shah			aas@riker.neoucom.edu

**********

*        *

* o   o  *

*   |    *

* \___/  *

**********

------------------------------

Date: Tue, 12 Mar 1996 09:00:24 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Decal for Arlington

Message-ID: <3145ADA8.2D0F@ns.cencom.net>

Troy and SSers,

I am going to have my decal guy make me a red peel and stick decal to

put on my back window for the Arlington trip. It will be block letters

saying NAISSO with a bow tie on the side.  Troy, if this is a bad idea,

let me know.  I would like to be able to id fellow SSers on the trip

down and around Dallas.  I imagine the cost would be less than 6 or 7

bucks.  I could mail them to anyone interested or members could contact

decal guy direct.

Troy........what ya think?

Loren DCM96 SPE

Texas or Bust

Lets get Sideways!

------------------------------

Date: Tue, 12 Mar 1996 08:58:43 PST

From: mweber@westpub.com

To: impala@east.xsis.xerox.com

Subject: New member introduction

Message-ID: <s1455937.083@westlan.com>

Hello.

My name is Mark Weber and I am pleased to join the list.  I live in

Eagan MN (a suburb or Minneapolis / St Paul).

My Black 96 is sitting at the dealer now and I will pick it up this

weekend.  It has all options except the heat reflective windshield

and I got it for $200 over invoice.

Other cars I own:

71 400 ci Chevelle SuperSport. (454 laying on floor to go in)

79 Chev Caprice winter beater.

87 Trans Am GTA .

I will be going to Arlington for the tour, arriving in Arlington Wed

night or Thursday morning and staying until Sat morning when

we have to leave to get back home.  We will be staying at the

Fairfield Inn in one of the NAISSO block rooms.

Look forward to meeting you guys and seeing your cars!

Mark Weber

Black 1996.

------------------------------

Date: Tue, 12 Mar 1996 10:18:26 PST

From: Matt Platts <mplat@airmail.net>

To: "'Impala'" <impala@east.xsis.xerox.com>

Subject: The Arlington plant is officially closed

Message-ID: <01BB100E.05A66C80@dal07-03.ppp.iadfw.net>

This morning before noon, the GM plant closed and sent around 2,000 =

workers home.  As most of you probably know, this is because of the =

shortage of parts (mostly brakes, I think) because of the UAW strike in =

the Detroit area that makes several of the necessary parts to make most =

GM vehicles.  The report said that all GM assembly plants are shutting =

down as a result of this parts shortage, with exception of the Corvette =

plant in Bowling Green, KY, which does not use any of the parts in short =

supply.  They said the shutdown could last as few as a couple of days, =

but go as long as a few weeks.  To this point, there have been no =

reports of 'talks' between GM and the striking UAW parts group.

More info to come if warranted.

-matt

------------------------------

Date: Tue, 12 Mar 1996 10:41:22 PST

From: Mike Kai-Kan Yang <mkyang@unixg.ubc.ca>

To: Impala SS Mailing List <impala@east.xsis.xerox.com>

Subject: Corn Filter!  Oops

Message-ID: <Pine.SOL.3.91.960312103822.16381A-100000@netinfo.ubc.ca>

Hi everybody:

I guess I posted my article yesterday without noticeing that my

auto-correct spell checker (not much of correction in retrospect) has

replaced all the cone with corn.  But glad to see that everyone has a

such good sense of humour.

Have a nice day!

Mike Yang

University of British Columbia

1996 Impala SS Dark Cherry Red (9C1 Air Deflactors, Gas Bib)

In search of a buck wheat filter =>

------------------------------

Date: Tue, 12 Mar 1996 11:32:14 PST

From: Stewart McLellan <stewartm@magi.com>

To: impala@east.xsis.xerox.com

Subject: Superchips Programming

Message-ID: <3145D13E.6C5F@magi.com>

Hi Guys:

I just got off the phone with a speed shop here in Ottawa talking about the Superchips

PCM reprogramming.  He didn't have a lot details other than the price, $325 Cdn.  This is a

great price considering that a Hypertech Power programmer will set me back about $500 if I order

it from the U.S. of A. or about $600 is I buy it locally.

Apparently there is a shop in Montreal that will be able to do the Superchips

programming on site.  As soon as I get more details I'll pass them on for the eastern Canadian

list member (Tip, Les and ?).

Does any one out there have any details on the programming parameters/capabilities of

Superchips,i.e., shift firmness, fans, rear gears, cams, etc?  Similar to Z-industries or

Hypertech?

Regards, Stewart

=====================================================================

"Nothing in life is so exhilarating as to be shot at without result."

-  Winston Churchill

=====================================================================

------------------------------

Date: Mon, 11 Mar 1996 12:20:33 PST

From: ZTMP50A@prodigy.com ( DAVID NMN)

To: impala@east.xsis.xerox.com

Subject: 94 Formula

Message-ID: <097.03383415.ZTMP50A@prodigy.com>

Sorry to hear about the mishap with your 'bird.  I can relate

somewhat.  I got a call from the fleet manager back in December

telling me my SS was in.  On the way to the dealership  (They were

too busy to deliver it to me), I was getting on the onramp to the

freeway in my '89 Grand Am SE.  The ramp was monitored at the time,

as I pulled away from the light, some idiot in a pickup shot by me on

the inside (no lane), ran out of space, and cut me off by moving over

towards my right fender.  Not wanting to get hit and no where to go,

I hit the brakes (we were are on curve in the onramp) and ended up

skidding in the guardrail, wiping out the drivers side of my car, etc.

The guy, of course stomped on the gas and left the seen, leaving me

to sit there in a wrecked car that was for sale also.  Needless to

say, I was pissed, but with no one to blame it on, I ended up eating

the car. ( I didn't want to report it to my insurance company with me

adding a new car).  Upside I guess (and yours too) is that at least

it wasn't the Impala that got wrecked, right? <BG>.  BTW, still

picked up my SS 2 hours later.  Want to talk about a paranoid drive

home??

Take care

Dave in So Cal..'96 Black SS

------------------------------

Date: Tue, 12 Mar 1996 12:48:59 PST

From: Loren Peterson <equ_rino@cencom.net>

To: impala@east.xsis.xerox.com

Subject: Wives and the SS

Message-ID: <3145E33B.287E@ns.cencom.net>

How many of the Wives/Girlfriends are going to Arlington?  Troy, any

word on this?   My Wife wants to know if she is going to be the only

significant other in attendance.

Loren DCM96 SPE

P>S> and if your a Female owner, are you bringing your significant

other?

------------------------------

Date: Tue, 12 Mar 1996 14:49:06 PST

From: Mark Hawthorne <mahawthorne@cliffy.lfwc.lockheed.com>

To: impala@east.xsis.xerox.com

Subject: Plant Closings

Message-ID: <3145FF62.3928@lfwc.lockheed.com>

It's official.  Today is the last day Arlington assembly personnel will

work until the strike at the brake parts factory is settled.  A guy I

know at the factory said they will be laid off without any pay.  No one

knows how long this will last.  This will probably affect the total

number of Impalas made and probably drive the price up, too.  For those

of you waiting on your cars, I hope this does not send you into total

despair.  Sorry to be the bearer of bad news.  Later.

Mark

------------------------------

Date: Tue, 12 Mar 1996 13:15:56 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: HAPPY BIRTHDAY

Message-ID: <960312161556_349165856@emout05.mail.aol.com>

btw, Scott Mueller you old fart.   Happy Birthday!    Rock

------------------------------

Date: Tue, 12 Mar 1996 13:15:54 PST

From: Rock528@aol.com

To: 75511.1220@compuserve.com, impala@east.xsis.xerox.com

Subject: SUPER CHEVY UPDATE

Message-ID: <960312161554_349165832@mail06.mail.aol.com>

In a message dated 96-03-11 23:26:44 EST, you write:

>Write the whole picture of events and post it on the digest as to what the

>choices are so everyone can decide and you will also know who's going where

>and so on.

Here is an update of the Super Chevy Events in Arlington:

1.  Pick up a Super Chevy Magazine.  They have a 2 page spread on the events

that will take place for the two day show.

2.  The BFG tent is saved for speakers, editor choice awards, seminars, etc.

3.  We will be in an area of the show cars.  We will be all together and you

will have the option of being judged or not.  They generally do not judge the

undercarriage which is a saving grace for those that will be driving across

country.

4.  The 35 dollar admission is good for all areas including the pits.  The

only thing you will not be able to do is race your car.  This includes, show,

vendors, spectator areas..

5.  For those interested, Super Chevy will have a judging seminar on Saturday

for all those interested in what the judges look for.

6.  This event will be covered by ESPN (races) and ESPN-2 (human interest

stuff, i.e., car show area, Scott's clearcoated undercarriage, etc.)

7.  Major Vendors: BFG, Soft Seal, Maguires, Simpson Safety and Chevrolet

will have their concept cars there (at our request I might add! <BG>)

8.  I may even speak at a "Race Against Drugs" seminar for the kids.  Be sure

and get one of my baseball cards before they're gone <g>.

9.  For you folks in the NE that can't come:  I spoke with Super Chevy about

our group attending one of the other SC shows in Epping, NH on May 25,6 or

Maplegrove, PA in late July.  Scott Harris, would you like to spearhead

this??

10.  SUPER CHEVY HAS BEEN VERY ACCOMMODATING TO US.  THEY HAVE ONE REQUEST.

WE MUST ARRIVE IN A GROUP ON SAT AND SUN.  OTHERWISE THE PARKING GETS HAIRY.

WE CAN PLAN THIS MORE ON 4-19-96.

Thanks,

Troy Willrick

NAISSO

------------------------------

Date: Tue, 12 Mar 1996 13:17:17 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Subject: Intake Mods - Gates Hose.

Message-ID: <9603122121.AA09251@karloff.fstrf.org>

I'm ready to do another intake mod - I've removed home plate - and

am looking at performing the "Replacing the Baffle with a Chrome Pipe"

mod listed at:

http://w3.one.net/~rcheek/impala/technical/technical.html

under "Eliminating the Rectangular Baffle" section.

..edit...

>Method 2: Replacing the Baffle with a Chrome Pipe

>

>This method replaces the baffle with a piece of chrome exhaust pipe. It looks

>much like some of the aftermarket kits that are out there for a lot less money.

>This method is recommended for those who don't mind a non-stock looking

>engine bay and are on a budget.

>

>Parts Required:

>

>      A 3.5" chromed exhaust pipe cut to 13". About $20 from Pep Boys.

>      A 2"-3" section of 3.5" ID rubber hose. Check with a local heavy truck

>      supply store for Gates hoses. "GATES Green Strip 3 1/2" I.D. (89 mm)",

>      P/N 24256, seems to work great. About $10 per foot.

>      Two pipe clamps to secure the tubing to the MAF and the pipe. $2.

..edit...

I'll eventually be replacing the stock elbow with the 1LE, but for now

my wallet can only afford this el cheapo mod.  My main question is this:

I can get the Gates hose locally, but only in 36" pieces.  I'm also concerned

about rusting/flaking of chromed pipe even though I can't find anyone locally

that carries any in 3.5" diameter anyway!  I'm thinking of just buying the 36"

piece of Gates hose, and cutting it to fit without the chrome pipe.  I don't

mind that it'll look ugly, since I'll be buying the elbow later.

Although this hose looks very heavy duty, will it tolerate the heat from engine?

Is there any other reason than looks that no one has just used a length of it

versus the chrome pipe?

Also, my first quote for the 36" piece is $62.  I can probably bargain or

get it cheaper by looking around, and wouldn't mind splitting the cost with

someone, preferably in my area (Western NY), since it looks like 18" will be

more than enough to replace the baffle.  It would cost me $20 for the pipe,

plus the cost of the Gates hose anyway - $31 and a slightly less "pretty" intake

would suit me fine.  Anyone interested?

- mike

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

My '96 Impala SS: "It goes fast, FAST!"

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

Date: Tue, 12 Mar 1996 13:24:44 PST

From: "Michael C. Calanan" <calanan@karloff.fstrf.org>

To: impala@east.xsis.xerox.com

Subject: Speaking of the 1LE elbow...

Message-ID: <9603122129.AA10314@karloff.fstrf.org>

I know that both 1LE elbows:

"94 elbow (cheap one) 25147210 (no vent nipple, make your own and hide it)

95 elbow (get raped) 25147187 (has vent nipple, $60 more expensive at retail)

--

Franklin Poole, Jr."

(thanks Franklin)

will fit my '96, but I'm curious to know exactly which models of GM

f-body's did these elbows come with in 94 and 95.  Anyone have an image

of either one of these?  I want to make sure I'm getting the right one

when I finally order it, and may even try to find one at a junk yard.

Thanks,

- mike

Turkish Proverb: "Coffee should be black as hell, strong as

death, and sweet as love."

My '96 Impala SS: "It goes fast, FAST!"

---

+-------------------------------------------------------+

|         Michael C. Calanan  calanan@fstrf.org         |

|                 Programmer / Analyst                  |

|  Frontier Science and Technology Research Foundation  |

|         4033  Maple Road  Amherst  NY  14226          |

|           http://stewart.fstrf.org/~calanan           |

+-------------------------------------------------------+

------------------------------

Date: Tue, 12 Mar 1996 14:08:17 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: PCM WARS

Message-ID: <199603122207.QAA29487@legend.txdirect.net>

Well guys, this is the 2nd time Im writing this.  My puter crashed in

the middle of the last post.   Hope it didnt somehow find its way to

the list or you will be reading this 2x.

Well, I asked for opinions on PCMs didnt I?  Thanks Scott M for your

input.   It raises more questions.....

Regarding Transmission shift firmness:

>This is something that Hypertech currently does not do. However, they are

>coming out with this, and by simply changing a small flash memory chip in

>the Power Programmer device (which they can send you), you can then update

>your car to the new program without removing your PCM

I know that hypertech charges $150 for any changes made to their

program even at the time of order.   Will these "update chips" be

free!?   Or will it cost $100-$150 for every update chip?  If this is

the case then there is no difference between Z and Hypertech except

the inconvenience of the downtime, and Z charges $100 for their

programing changes (I think).   I am doing the 3.42 now and dont

think I will get into the 1.6:1 rockers or the any other Major engine

mods but you never know. ...

>Another point, do you realize that there are 4 DIFFERENT factory PCM

>calibrations for the '94 Impala, and 3 DIFFERENT calibrations for the '95?

>The '96 currently has only 1 calibration. If you have a '94 or '95, which

>one is the Z-industries program you will get based on? What happens when the

>factory releases a newer calibration with another fix?

Perhaps John Leviz at Evergreen can answer this....John?

>With the Hypertech unit, you can return your vehicle to stock, take the car

>to the dealer to have the latest factory PCM calibration installed, and then

>use the Hypertech module to modify THAT calibration! In fact, if you had a

'>95, and wanted the 2nd gear start function, you could install the first

>revision factory PCM calibration, and then even after Hyperteching it you

>would have that feature!

>The difference between the Hypertech and the others is that it dynamically

>modifies whatever PCM calibration is in the car. In other words, it uses the

>existing stock calibration as a base (whatever that may be) and modifies it

>from there. This plus the ability to return the vehicle to stock allows you

>to get the PCM calibration updates from your dealer and then install the

>Hypertech mods on top of them.

This I didnt know.  I was under the impression that the hypertech

flashed your PCM with whatever program was in the programmer. I didnt

know that it "modified" whatever program you have.   I thought THEY

installed an already modified program and it didnt matter what

program you had.   This is a significant point.   Anyone else care to

agree or disagree?   John, Scott H.  Donald@FIS ???

>Did you know that John Lingenfelter, arguably the best LT1 family

>engine builder on the planet right now, uses Hypertech for all of his

>programming mods! In every test I have seen comparing a Lingenfelter

>car to other tuner cars, his always come out on top, not only

>meeting, but exceeding his HP claims.

Yes but It is my understanding that Lingenfelter makes his OWN PCM

program and programs THAT to a Hypertech chip, then uses the

programer to flash the car.

>Hypertech is not perfect either, as they currently don't offer shift

>improvements, however this is coming

True?   Can anyone confirm or deny?  WHEN!!!

Differing opinions and good arguements make for better decisions.  I

thank both "Scotts" for their input and welcome any others.

I am in a holding pattern for now....

-----------

In the year of darkness, 2029, the rulers of this planet

devised the ultimate plan. They would reshape the future by

changing the past. The plan required something that felt

No pity, No pain, No fear. Something Unstoppable.

They created The TERMINATOR...

Jeff Gatcha

foxhound@txdirect.net

------------------------------

End of IMPALA Digest 348

************************

IMPALA Digest 349

Topics covered in this issue include:

1) GM Parts

by "Jeff Gatcha" <foxhound@txdirect.net>

2) HOTCHKIS REAR BARS

by Rock528@aol.com

3) Re: Newsletter: We Need A Name!

by Impalaclub@aol.com

4) RE: Plant Closings

by Gary Rotter <gr5@elsegundoca.attgis.com>

5) RE: IMPALA digest 348

by "Harris, Scott" <sharris@xyplex.com>

6) Another Horn Story

by Ken Orlando <korlando@blkbox.com>

7) WOT A/C cutout?

by Scott Mueller <73145.1566@compuserve.com>

8) eliminating baffle w/chrome pipe

by "Jeff Gatcha" <foxhound@txdirect.net>

9) Re: coated rotors

by Kurt Kemmling <76165.1654@compuserve.com>

10) Aol Automotive Folders

by "Brett A. Rudolph" <bngwip@mail.phoenix.net>

11) Price and ordering news

by SManey0830@aol.com

12) TORQUE COVERTER

by JGreen5275@aol.com

13) Computer Change

by JGreen5275@aol.com

14) Suspension

by HPERF@aol.com

15) new web site

by ken <ken@jbiz.com>

16) Arlington?  I'm there!

by Scott Hollums <ash@antec.com>

----------------------------------------------------------------------

Date: Tue, 12 Mar 1996 14:08:17 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Cc: fuelinjection@fuelinjection.com

Subject: GM Parts

Message-ID: <199603122207.QAA29483@legend.txdirect.net>

Donald @FIS wrote:

>To All Impala Owners,

>

>Know you "guys" buy alot of GM parts, and unless you have connections

>with a dealer, you might check with us before buying parts.  We get such

>a discount from GM that we should be able to save you money.  Jeff Gotcha

>will tell you as he has checked on some prices with us;

Yes, I agree Donald has some good prices on GM parts.  You are

looking at about a 20% discount or so, give or take a few bucks.

-----------

In the year of darkness, 2029, the rulers of this planet

devised the ultimate plan. They would reshape the future by

changing the past. The plan required something that felt

No pity, No pain, No fear. Something Unstoppable.

They created The TERMINATOR...

Jeff Gatcha

foxhound@txdirect.net

------------------------------

Date: Tue, 12 Mar 1996 14:28:05 PST

From: Rock528@aol.com

To: impala@east.xsis.xerox.com

Subject: HOTCHKIS REAR BARS

Message-ID: <960312172804_166937070@emout05.mail.aol.com>

In a message dated 96-03-11 10:39:50 EST, you write:

>In  a recent posting, Scott M. reported that you had installed the Hotchkis

>rear control arms.  If so, where did you get them, how much were they, and

>what difference do they make.?

Yes, I do have the Hotchkis Rear Bars and I am waiting on the top ones to be

manufactured.  Compared to many of the SS aftermarket parts, the lower bars

are a work of art.  I actually looked at them for several days before putting

them on. <g>

I would definitely reccommend these over boxing your own bars.  John Hotchkis

definitely did his homework.  They have grease fittings on both ends and fit

and work great.

> Are they pre-drilled top and bottom so that HA bars can be attached.  Do

they have polyurethane bushings (that squeak)?

They do use a poly.. bushing on both ends and they do not squeak.  The HP

arms for the Impala have the softest durometer polyurethane, so you do not

get a harsh transfer of noise or vibration.  I checked on this prior to

purchasing them.  The sway bar bolts completely through each torsion bar.

If you are interested in the bars, let me know and we will try to do a group

purchase using the club.

Troy

------------------------------

Date: Tue, 12 Mar 1996 14:29:48 PST

From: Impalaclub@aol.com

To: 73541.1501@compuserve.com, impala@east.xsis.xerox.com

Subject: Re: Newsletter: We Need A Name!

Message-ID: <960312172947_444765320@emout05.mail.aol.com>

Here is a great suggestion from Joe on the NAISSO newsletter:

>One suggestion, however:  our newsletter needs a NAME!!!!   Something like

>"Super Sport News" (I know, I know, that is REALLY lame, but you get the

>idea).

The person with the winning name will get an Impala Tshirt.

Right now the newsletter has the club name on it but I agree we do need a

catchy name for it.  I have been working on several logos.  Maybe those

Saturn folks will let us hang with them......  Troy

Reply directly to impalaclub@aol.com with your suggestions.  Winner will be

announced at Arlington.

------------------------------

Date: Tue, 12 Mar 1996 14:53:36 PST

From: Gary Rotter <gr5@elsegundoca.attgis.com>

To: impala@east.xsis.xerox.com

Subject: RE: Plant Closings

Message-ID: <9603122253.AA23740@mailhost.ElSegundoCA.ATTGIS.COM>

Mark Hawthorne writes...

>For those of you waiting on your cars, I hope this does not

>send you into total despair.

It did.  I have an Impala on order with a target production

month of May.

------------------------------

Date: Tue, 12 Mar 1996 04:59:00 PST

From: "Harris, Scott" <sharris@xyplex.com>

To: impala <impala@east.xsis.xerox.com>

Subject: RE: IMPALA digest 348

Message-ID: <314601F8@corvette.xyplex.com>

>>9.  For you folks in the NE that can't come:  I spoke with Super Chevy

about

>>our group attending one of the other SC shows in Epping, NH on May 25,6

or

>>Maplegrove, PA in late July.  Scott Harris, would you like to spearhead

>>this??

Already underway.  Since New England Dragway is "the track" of the

ISSCONE (Impala SS Club of New England, a renegade little splinter group

of the NAISSO) that's the event we're going to attend.  I have heard from

a couple people that are interested already.  Anyone else that wants to

join in, let me know.  I plan on doing both the show AND race events.

One guy will be bringing his wife's 96 with: 377 cu in, AFR heads, Comp

Cam, 52mm throttle, extrude-honed intake, AS&M headers, Borla exhaust,

custom PCM, 3.73 rear, 335/35 tires on 11" Forgeline mags.  Did you catch

that first part?  This is his wife's car!

Maplegrove is a little too far for me so I'll defer the responsibilities

to someone closer.

Scott Harris

Impala SS Club of New England

sharris@xyplex.com

------------------------------

Date: Tue, 12 Mar 1996 15:49:59 PST

From: Ken Orlando <korlando@blkbox.com>

To: impala@east.xsis.xerox.com

Subject: Another Horn Story

Message-ID: <9603121658.aa08727@blkbox.COM>

Hello again!  Been up in the frozen north (NJ) for about 4 weeks and glad to

be back in TX. Just a short follow up to my "20 pound sledge hammer horn"

problem.

After two trips to the dealer, (they didn't think you needed to order the

new steering wheel even if it was in the TSB) I have a new horn.  MAN WHAT A

DIFFERENCE! Anybody with the old (bad?) horn needs to get the new one.  I

have to ride with my stereo volume turned way down because I'm afraid the

bass will cause the horn to blow <g>.

Anyway I know it seems minor but it was relly bugging me.  It was kind of

embarassing after smokin' somebody to be seen pounding on the horn as you

waved bye-bye!

FWIW - I have done the red reflective bow tie on the Green Machine and it

looks pretty good.  It was a snap and as an added bonus I get stares from a

couple of the other Impala owners trying to figure out what the red bowtie

is all about.

Gonna miss the Arlington trip - The kid is in the State Competition for

one-act in Austin and I can't miss that. Oh well - I know its going to be a

great time - wish I could be there, but look forward to the war stories and

pictures (?) after yall get back.

And Troy - No newsletter yet (12 Mar).

Ken in the 95 mean green machine....{with the red bow tie!

Ken Orlando

korlando@blkbox.com -or-

korlando@explorer.csc.com -or-

korlando@jcals.csc.com

------------------------------

Date: Tue, 12 Mar 1996 16:28:19 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: WOT A/C cutout?

Message-ID: <960313002819_73145.1566_FHD59-1@CompuServe.COM>

William,

>before switching to the "Z Industries" PCM, I checked for the A/C

compressor cutout at WOT on my stock '94 Impala and it did NOT turn off the

compressor.  I ran wires from the compressor to inside the car, and had my

passenger watch the voltmeter while I turned the compressor on & off

manually and then when I left the compressor on and floored it for awhile.

It definately did not turn off the A/C on MY car.  I called up Hypertech

(before decided on the Z PCM) and talked to one of their "custom tuning"

guys.  I think his name was "TJ", or something like that.  He told me that

the stock PCM does NOT turn off the A/C at WOT and neither does their

program.  I asked him if that was something that he could add to a "custom"

program, and he told me that he doesn't know how to do it.

>Have you actually verified this on your car?  I'd be curious if your

findings were different than mine.  Maybe I have an older PCM than you, I

dunno.

Well, thanks for correcting me on this! I actually did not verify this one

myself through testing. Instead I read in the 9C1 brochure that they have

the WOT A/C cutout, and simply assumed (and you know what that makes me <g>)

that this applied to the Impala as well. Now based on your observations and

testing, it seems like indeed the Impala may not have this feature. That

also casts doubt on whether the 9C1 cars also have it, but of course maybe

the brochure is right after all and this is an exclusive 9C1 feature. By

comparing the 9C1 and Impala code, the chip tuners like Z-industries,

Superchips would be able to easily figure out how to add this feature.

BTW, the WOT A/C cutout has long been a 9C1 feature, my '88 Caprice 9C1 has

it. Even my '89 Turbo TA has a WOT A/C cutout, you'd think that GM would

have put this in all of the cars by now! If Z-industries has this feature

and nobody else does, than that is a definite plus for them! BTW, I suppose

that this may vary among the different Impala PCM calibrations, but I doubt

it. Maybe somebody will have a definitive answer as to which LT1 B-cars have

this and which ones do not! Scott.

------------------------------

Date: Tue, 12 Mar 1996 16:32:59 PST

From: "Jeff Gatcha" <foxhound@txdirect.net>

To: impala@east.xsis.xerox.com

Subject: eliminating baffle w/chrome pipe

Message-ID: <199603130032.SAA07339@legend.txdirect.net>

Mike Says:

>Method 2: Replacing the Baffle with a Chrome Pipe<snip>

>Although this hose looks very heavy duty, will it tolerate the heat from engine?

Probably but the physics majors in the audience might want to give

another lesson on a black body soaking up heat.  etc....

>Is there any other reason than looks that no one has just used a length of it

>versus the chrome pipe?

yeah, you still need something rigid to connect the hose to the

intake elbow, so I just used the 13" length to make it look like

the hennessy intake.  Plus you dont want the MAF sensor floping all

over the place yanking on the wires.  The pipe gives it that

rigidity, and keeps the MAF in its place for the most part.

If you just use the hose, you might end up having  a rock on a rope.

Although the gates hose is fairly rigid, it still might move around

too much. .  If you can find a small  piece of 3.5" metal piping, or pvc

you could probably use that as a connector and use the hose from that

to the MAF.

$62 for the 36" length isnt too bad.  The only place in South Texas I

could find the hose wanted $98 for the 3' lenght!!!!!

Others have gotten it for about $10 a foot at heavy duty truck shops.

I never really considered the flaking/rusting of the chrome pipe.  I

will have to check mine every now and then.

-----------

In the year of darkness, 2029, the rulers of this planet

devised the ultimate plan. They would reshape the future by

changing the past. The plan required something that felt

No pity, No pain, No fear. Something Unstoppable.

They created The TERMINATOR...

Jeff Gatcha

foxhound@txdirect.net

------------------------------

Date: Tue, 12 Mar 1996 16:44:51 PST

From: Kurt Kemmling <76165.1654@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: coated rotors

Message-ID: <960313004451_76165.1654_GHM195-1@CompuServe.COM>

In digest 346 Scott Mueller writes:

>>>I think you have a misconception here. You must be thinking that the rotors

are coated even on the braking surface! The coating, which is actually some

type of heat resistant paint on the front rotors and a metallic ceramic type

coating on the rear, is applied everywhere except of course the surface upon

which the pad rides,<<<

Scott,

Yes, you are correct. I did misunderstand you. I thanks you for your

clarification, as I was really surprised to hear that a coating was developed

that would *not* inhibit braking performance! Oh well, maybe in the

future........................<g>

Kurt

'95 Dark Cherry Impala SS.....coated rotors......

------------------------------

Date: Tue, 12 Mar 1996 16:50:02 PST

From: "Brett A. Rudolph" <bngwip@mail.phoenix.net>

To: impala@east.xsis.xerox.com

Subject: Aol Automotive Folders

Message-ID: <1.5.4b11.16.19960312185404.386f2c62@phoenix.net>

How many of you aolers have visited the automotive message boards on aol?

There is a folder that is for requests of new folders. I want all aolers who

visit this place to request an Impala SS folder. I already have but had no

results. If you need help finding this area on aol, let me know. The more

requests they get, the more likely we will get our own folder. I'm on a

mission from god. Please help!

Brett

Brett A. Rudolph

96 SS Black

(still waiting)

If you build it,

I will come.

------------------------------

Date: Tue, 12 Mar 1996 16:56:56 PST

From: SManey0830@aol.com

To: impala@east.xsis.xerox.com

Subject: Price and ordering news

Message-ID: <960312195656_349407203@mail06.mail.aol.com>

Just heard. $500 price increase for Impalas. If your order has been accepted,

you're protected, otherwise... Also, I spoke with Lee Chevy and they can't do

the delivery anywhere besides CT. Please respond if above info is inaccurate.

FYI

------------------------------

Date: Tue, 12 Mar 1996 17:52:30 PST

From: JGreen5275@aol.com

To: impala@east.xsis.xerox.com

Subject: TORQUE COVERTER

Message-ID: <960312205228_244558504@mail02.mail.aol.com>

This message is to inform all you people who intend to purchase the Corvette

12" torque converter to install in your 4000 lb ss and for some reason or

another or because you heard it would work expect a performance change.

(wrong) its a waste of 125.00 bucks For starters that converter was designed

for a lighter car ( Corvette) which by the way is the same size as the one in

our impala's.  The idea of a torque converter stater blade inside the

converter is to increase stall. But when you use the Corvette type your only

going to gain a 250 RPM's but at the same time you lose your torque multiple

because of the size with that configuration.  The ideal is to keep high

torque multiple and increase the stall at the same time.  The only way to do

that is to go down in diameter of the torque converter.  You want a coverter

for street,strip application.  The perfect size for the impala application is

the 9 1/2" to accomplish all of the above, that will launch the mass of 4000

lb's.  But by using a converter designed for the Corvette will result in

disaster.  You must have torque multiple to launch mass.  You must have HIGH

TORQUE and HIGH STALL.  A 9 1/2 in converter will give a 2500 stall plus the

necessary torque to launch the ss, it would be like adding another cylinder

to your engine, the change would be worth at least 1/2 sec in the quarter,

plus with only 2500 stall your trans won't overheat,  I hope you people pay

attention to this before you find out the hard way.

------------------------------

Date: Tue, 12 Mar 1996 17:58:49 PST

From: JGreen5275@aol.com

To: impala@east.xsis.xerox.com

Subject: Computer Change

Message-ID: <960312205849_244564636@emout06.mail.aol.com>

If there's anyone out there that wants to take your ss and optimize the spark

and fuel curve, change the fan temp, turn off the egr at part throttle,

increase the mph to 200 and the rpm to 6000, and in short make your impala

KICK BUTT, and are willing to spend about 200.00 bucks, call JP at 407 328

8912 for imp club member only, all other pay 300.00 By the way I'm in the

Daytona area, so if its to far than you'll have to send it to me and wait 2

days.

------------------------------

Date: Tue, 12 Mar 1996 18:45:19 PST

From: HPERF@aol.com

To: impala@east.xsis.xerox.com

Subject: Suspension

Message-ID: <960312214518_445008790@emout07.mail.aol.com>

Dear Scott:

Maybe I can answer some suspension questions for you.

Hotchkis Performance manufactures trailing arms for the Impala SS.  These are

simular in design to our other very popular Camaro, Monte Carlo, Grand

National and Chevelle products.

Hotchkis Performance designed and manufactured all the suspension products

that enabled Edelbrock to enter the suspension market.  They were sold under

the Edelbrock/Hotchkis label.  Since then we have gone our separate ways with

Hotchkis Performance concentrating soley on suspension products.  This

company offers the best suspension products on the market.

The Impala SS trailing arms were designed with the Impala SS owner in mind.

The polyurethane bushing are our softest durometer (best insulation from

road noise), yet still provide great stability and traction.  The bushing

have grease fittings and do not squeak.  We have sold thousands of this type

of trailing arm to very demanding GM performance car owners.  If there was a

problem with Hotchkis Performance trailing arms everyone would know it, but

they love them.

Stock and stock type mounting aftermarket sway bars will bolt right up.

Hotchkis Performance even supplies the hardware.  Wider tires can be used

because the trailing arms keep the axle centered under the car. No inner

fender rub when navigating your favorite off ramp.

Hotchkis Performance trailing arms are laser cut, TIG welded and powdercoated

gloss black.  There is no expense spared in bringing Impala SS owners

products that will perform well and last for many years.

These trailing make the Impala more fun to drive by eliminating wheel hop,

providing more traction and offering excellent ride charactoristics.

I invite you and all club members to e-mail or call Hotchkis Performance for

more information.

Drive fast.

John Hotchkis

Hotchkis Performance

310.907.7757

HPERF@aol.com

------------------------------

Date: Tue, 12 Mar 1996 19:13:13 PST

From: ken <ken@jbiz.com>

To: impala@east.xsis.xerox.com

Subject: new web site

Message-ID: <1.5.4b11.32.19960313031313.006b5f1c@10.10.10.1>

Hi fellow SS lovers, Check out my new web site at http://jbiz.com/hrh. The

goal of "HOT ROD HEAVEN" is to provide my fellow "car nuts" with a single

place to find anything they might need. This site will be continually

changing so I would welcome any suggestions. Please feel free to email me

(direct so we don't take up space on our list) and let me know what you

think. Thanks! Ken Sundquist, Sunshine Performance, Dark Cherry  96 that

thinks it's a 95 z-28

------------------------------

Date: Tue, 12 Mar 1996 19:41:18 PST

From: Scott Hollums <ash@antec.com>

To: impala@east.xsis.xerox.com

Subject: Arlington?  I'm there!

Message-ID: <199603130341.WAA29704@ta.antec.com>

Hi Gang,

Reading about the Arlington trip on the list has finally worn down my

resistance.  I made my reservations at the Fairfield Inn at the club discount

rate.  The guy at the hotel that took my call knew exactly what I was talking

about when I mentioned the club (didn't get his name though).

I will be driving my '95 black SS from Atlanta.  For my trip to Arlington I

figure that I have three driving options:

1) Hit I-20 in Atlanta, drive straight to Arlington, staying overnight in

Jackson, MS.  Are there any others driving across I-20 from the east?

2) Take I-85 down to Baton-Rouge to meet up with you guys/gals coming from

Florida.  Do you know how many are going to be in your convoy?  This option

would add about 4 hours to my total drive time.  The good times down in

Baton-Rouge may make up for it.

3) Leave Atlanta Thursday after lunch, drive WOT @ 160 mph, arriving in

Arlington Thursday dinner time <BG>.  (Sudden image of the chase scene in

"The Blues Brothers".  ATTENTION:  The use of excessive force in the

apprehension of the Beast Brothers has been approved!  We're on a mission

from the General.)  Has anyone got a reliable radar/laser jammer?

It looks like it will be a great time.  I look forward to meeting everyone.

BTW, I found out that the NRA Annual meeting will be in Dallas the same

weekend as the Chevy Show.  Any gun-totin' Impala drivers can mail me direct

and I will give you what info I have.

Scott Hollums

____________________________________________________________________________

|                             \                                              |

| Scott Hollums                \    When you are in your virtual reality and |

| Arris Interactive             \   standing at the virtual urinal, remember |

| Phone: (770) 734-0400 x8647    \  to only take a VIRTUAL whiz!             |

| Email: scott.hollums@antec.com  \                                          |

|__________________________________\_________________________________________|

------------------------------

End of IMPALA Digest 349

************************

IMPALA Digest 350

Topics covered in this issue include:

1) Arlington RSVP update (3/12)

by Matt Platts <mplat@airmail.net>

2) service manuals

by john spears <speartec@iquest.net>

3) Strike info

by David Hardy <dhardy@crl.com>

4) Re: Interior lights not coming on with keyless entry

by Mike Kerr <zap@imis.com>

5) Super Chevy Show

by Scott Mueller <73145.1566@compuserve.com>

6) Re:PCM programs

by lennie@earthlink.net (Lennie Dolphin)

7) Re:Non-lubed '96 suspension.

by lennie@earthlink.net (Lennie Dolphin)

8) A/C Clutch & WOT

by "C. K. Yee" <yu135008@yorku.ca>

9) Re: Significant other

by Mike Sundstrom <102366.175@compuserve.com>

10) SE contingent to Arlington

by Rock528@aol.com

11) Torque converters

by Scott Mueller <73145.1566@compuserve.com>

12) Hotchkiss control arms

by Scott Mueller <73145.1566@compuserve.com>

13) Impala Sites and Safety

by Mikado95@aol.com

14) Plant closings

by jserko@terrabyte.net (John Serko)

15) lease information

by Michele Buckingham <michele@acesoft.com>

----------------------------------------------------------------------

Date: Tue, 12 Mar 1996 17:50:52 PST

From: Matt Platts <mplat@airmail.net>

To: "'Impala'" <impala@east.xsis.xerox.com>

Subject: Arlington RSVP update (3/12)

Message-ID: <01BB1062.F6549B80@dal17-29.ppp.iadfw.net>

Here's the updated stats...  mail keeps coming in, keep it coming!

Pieces of e-mail recieved: 26

# of cars/parties Driving to area: 25

# of parties driving SS: 21

# of parties flying to area: 3

# of parties renting car: 1 (and driving it down)

# of people staying @ Fairfield: 25

Actual # of rooms @ Fairfield: 18

# of parties staying @ other motels: 2

Headcount for tour: 41

Headcount for SCS: 34

Impala SS's on display @ SCS: 17

Again, if you haven't already replied to me, please do so.  Please give =

me enough information to fill in the above fields plus: your city/state, =

arrival date, departure date, any other interesting details <g>.

Thanks -matt

------------------------------

Date: Tue, 12 Mar 1996 20:27:00 PST

From: john spears <speartec@iquest.net>

To: impala@east.xsis.xerox.com

Subject: service manuals

Message-ID: <m0twi9n-004XWPC@iquest.net>

Just to let you guys know, I received my 1996 final edition Y-car service

manuals Monday. So at least the final editions are starting to come out.

Hopefully the B-car manual will not be far behind.   John.

------------------------------

Date: Tue, 12 Mar 1996 21:04:48 PST

From: David Hardy <dhardy@crl.com>

To: Impala SS LIST <impala@east.xsis.xerox.com>

Subject: Strike info

Message-ID: <Pine.SUN.3.91.960312205731.12903C-100000@crl10.crl.com>

Pardon me if we've travelled this path already, I have 102 messages in

this box!

I saw on NBC, that due to the brake part plant strike, a plant in Texas

closed today, (unknown if it's Arlington), and that the close of many

plants is eminent.  This could be a real bummer to those projected

Impala production numbers.

Even so, I must voice my support for the UAW and their families during

this time, and hope for a speedy and satisfactory resolution to their

grievances!  Let's get those cars a movin' General!

-Dave

dhardy@crl.com  %%%%%%%%%%%%%%%%%%%%%%%%%%%%%

"The thief is sorry he is to be hanged,

not that he is a thief" (Thomas Fuller, 1732)

------------------------------

Date: Tue, 12 Mar 1996 21:15:44 PST

From: Mike Kerr <zap@imis.com>

To: impala@east.xsis.xerox.com (Impala SS LIST)

Subject: Re: Interior lights not coming on with keyless entry

Message-ID: <199603130515.AAA27177@clark.net>

> I recall that the lights don't come on even though the brochure mentions

> this. I've checked the maillist archive and the FAQ... I could have sworn

> there was mention of how to make it work. Anyone know how to get this to

> work? (Don't have my car yet, trying to fix my father in law's new '96

> Caprice. The dealer told my mother in law that it just don't work, should've

> bought a Pontiac! No kidding!)

The Remove control receiver is the same as other GM vehicles.  It has all

the electronics to turn on the interior lights, but the Chevys don't have an

ouput wire going to the interior light relay.  I did a write up in July 95 on

how I modified mine to do this.  I'll e-mail it to you.  Anyeone else?

Also available on the remote control receiver is the ability to have the

doors lock when you shift out of park.  This is also documented in the

article.

- Mike

'95 Impala SS

--

Mike Kerr

Project Leader, Citicorp Credit Services, Inc.        mike.kerr@citicorp.com

Vice President, Interactive MIS, Inc.                         mkerr@imis.com

------------------------------

Date: Tue, 12 Mar 1996 21:50:38 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Super Chevy Show

Message-ID: <960313055038_73145.1566_FHD30-1@CompuServe.COM>

Doug (or Troy),

Since I have to setup a seminar classroom back in Lisle IL on Sunday night,

I will be leaving Dallas/Arlington sometime Saturday night to drive back

home. I know several others will have to leave after Saturday as well, and

will miss the Sunday show events and wrapup. I'm sure you can empathize with

how difficult it will be to tear myself away knowing that there is one more

day of Chevy festivities to go! Aaarrgh!

Question: Will the cars that are entered on Saturday only be eligible for

judging, and depending on the outcome, does one have to be present to get an

award? Judged or not, I will put the same amount of work into cleaning up

the car, which I expect will be a little difficult after the 1000 mile

drive! But don't worry 'bout me as others (Curt man!) have a bit more of a

drive ahead of them!! <G> In any case, I do have a number of special mods

planned for my Impala by show time, which I won't say much about until I am

certain I will have time to complete them! No matter what the car will be

there, but if I get everything done as is currently planned, it should be

somewhat unique.

I must say that NUMBER ONE I am REALLY looking forward to meeting

everybody!! I have made so many friends through this common passion for

automobiles we share, and meeting and talking with fellow enthusiasts has to

be the biggest kick of all. I am also excited to see and learn about what

everybody has done with their cars, and to be able to compare notes (as in

exhaust <g>) with so many at once. I have never been to a gathering of more

than say 4 Impalas at a time, so this will be sensory overload for me!

>btw, Scott Mueller you old fart.   Happy Birthday!    Rock

Hey, thanks! I'm now officially on the downhill side of 40 (GASP!) as I

turned 36 today. More interesting than that, my Impala turns 2 on the 16th

(in only 4 days!). I can't believe I have had the car this long, and every

day I am having more fun with it now then EVER. Will it ever stop?? <g> If

we could distill the power that this car has over people, we could rule the

world!!... Ahem - excuse me, I don't know what got into me there for a

second. (do you think they put something in the paint, or is it the magical

combination of the deer, the bowtie symbol and the SS that does it?) <g>

Scott.

------------------------------

Date: Tue, 12 Mar 1996 22:07:20 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: Scott Mueller <73145.1566@compuserve.com>

Cc: Impala <impala@east.xsis.xerox.com>

Subject: Re:PCM programs

Message-ID: <199603130607.WAA10033@norway.it.earthlink.net>

Scott:

>In fact, if you had a'95, and wanted the 2nd gear start function, you could

install the first revision factory PCM calibration, and then even after

Hyperteching it you would have that feature!

Are you saying that Mr. GM has come out with a revised PCM calibration for a

'95 2nd gear start function?

... Lennie

------------------------------

Date: Tue, 12 Mar 1996 22:07:16 PST

From: lennie@earthlink.net (Lennie Dolphin)

To: "Michael C. Calanan" <calanan@karloff.fstrf.org>

Cc: Impala <impala@east.xsis.xerox.com>

Subject: Re:Non-lubed '96 suspension.

Message-ID: <199603130607.WAA10027@norway.it.earthlink.net>

Michael:

>Well, over the weekend I decided it would be in my best interest, and the

beast's best interest <g>, to check the lubrication of the suspension.

With grease gun in hand I slid under the beast to find to my displeasure

and amazement that none of the grease fittings were lubed!

It looks like this problem is still happening to the '96 models built after

1995.

When I put my torque wrench to all those suspension parts, I was in for a

real surprise. Here's what I found while lubing and torquing.

Under torqued:

o       LH/RH lower ball joints and dry causing popping sound

o       LH/RH front caliber bolts dry causing clicking sound when braking

o       LH/RH front sway bar nuts

o       Steering gear box bolts

o       LH steering linkage outer tie rod nut

No wonder the front-end wouldn't stay in alignment. The thing handles like

it was re-born.

BTW, rear suspenion is NEXT!!

... Lennie

------------------------------

Date: Tue, 12 Mar 1996 22:09:53 PST

From: "C. K. Yee" <yu135008@yorku.ca>

To: impala@east.xsis.xerox.com

Subject: A/C Clutch & WOT

Message-ID: <Pine.SOL.3.91.960313004201.4076A-100000@sunspot.ccs.yorku.ca>

The recent discussions about A/C cutout when at full throttle has

been interesting.  My '80 Seville with DEFI disengages the compressor

clutch at WOT, and I've always thought that GM cars with electronic

engine controls did this.  Anyways, I found a section on A/C clutch

circuit diagnosis (6E3-C10-1) in the service manual.  This is from

the Fleetwood manual, but I don't see any reason why it should differ

from the B-cars.

**********

General Description:    In order to improve engine operation, and

protect the A/C compressor and A/C clutch during Wide Open Throttle

(WOT) performance, the A/C compressor operation is controlled by the

PCM.

Operation:     This system consists of a heater and A/C control, A/C

pressure cycling switch, an A/C refrigerant pressure sensor, an A/C

clutch relay, the compressor clutch, and the PCM.

The A/C refrigerant pressure cycling switch is closed when there

is sufficient system pressure (depending on refrigerant charge (134a)

and ambient temperature).

When heater and A/C control is placed in the A/C mode, a request

signal is sent to the PCM.  The PCM will then energize the A/C clutch

relay, unless abnormally high or low A/C pressure is detected by the

A/C refrigerant pressure sensor.  The PCM will also turn the cooling

fans "ON" when A/C is requested.  Refer to "Electric Cooling Fan(s),"

Section "6E3-C12" for more information on cooling fan operation.

The A/C clutch relay is controlled by the PCM so that the PCM can

increase idle speed before turning "ON" the clutch or disable the

clutch during WOT, and high coolant temperature.

Diagnosis:    The PCM will energize the A/C clutch whenever the engine

is running less than 3800 RPM and A/C has been requested, unless any of

the following conditions are met:

- Wide Open Throttle (WOT)

- A/C head pressure greater than 429 psi or less than 0 psi

(as determined by the A/C refrigerant pressure sensor)

- Low ignition voltage

- Engine speed greater than 5250 RPM for 30 seconds.

- ECT greater than 124C (255F)

**********

Since is has been too cold for A/C operation here (Toronto, Canada)

I can't personally verify this.  Based on this passage from the

service manual, I would conclude that the Fleetwood disables the

compressor clutch at WOT.  Does the Impala manual have the same

section?

Kevin

------------------------------

Date: Tue, 12 Mar 1996 21:14:51 PST

From: Mike Sundstrom <102366.175@compuserve.com>

To: "INTERNET:impala@east.xsis.xerox.com" <impala@east.xsis.xerox.com>

Subject: Re: Significant other

Message-ID: <960313051450_102366.175_EHR87-1@CompuServe.COM>

Loren,

>How many of the Wives/Girlfriends are going to Arlington?  Troy, any

word on this?   My Wife wants to know if she is going to be the only

significant other in attendance.

Loren DCM96 SPE

P>S> and if your a Female owner, are you bringing your significant

other?

I'll be bringing my significant other.  Actually, my significant other will be

bringing me (in style and comfort of course). <G>  Mike

------------------------------

Date: Tue, 12 Mar 1996 22:35:48 PST

From: Rock528@aol.com

To: ash@antec.com, impala@east.xsis.xerox.com

Subject: SE contingent to Arlington

Message-ID: <960313013548_167370639@emout10.mail.aol.com>

Scott,

> Take I-85 down to Baton-Rouge to meet up with you guys/gals coming from

Florida.  Do you know how many are going to be in your convoy?  This

option

would add about 4 hours to my total drive time.  The good times down in

Baton-Rouge may make up for it.

I see your driving from Atlanta.  The FL group plans on leaving Wed am with a

stop over in Baton Rouge with Franklin Poole on Wed evening.  He promises to

have several bottles of BR tea iced down, in the ready.  Thursday we will be

on to Arlington.

>From FL

I-95 or I-75 to I-10 West

I-10 to Baton Rouge, LA

I-10 to I-20 to Arlington on Thursday.

Confirmed drivers from here: 2 now with more interest as the newsletters get

out.  Many are flying.

Troy (rock528@aol.com)

"Could this be the first good use of the Internet?"

------------------------------

Date: Tue, 12 Mar 1996 23:02:12 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Torque converters

Message-ID: <960313070211_73145.1566_FHD39-2@CompuServe.COM>

JGreen,

>This message is to inform all you people who intend to purchase the

Corvette 12" torque converter to install in your 4000 lb ss and for some

reason or another or because you heard it would work expect a performance

change. (wrong) its a waste of 125.00 bucks...

I can tell from what you wrote that you don't have any experience with the

converter mentioned. The information about the '86 'vette converter (which

is apparently the highest stall factory convertor available for the 700-R4,

4L60 and 4L60-E trans) came straight from John Lingenfelter, who also uses

this converter in his auto equipped applications. That information has now

been verified several times over by those who have installed it! In fact I

know quite a few people who have these converters installed, and their

results are quite different from what you are describing.

A 9.5" converter might be fine for you, that is if you want to turn your

Impala into nothing but a quarter mile car. You should also have at least a

4.10 rear gear as well, remove the swaybars for a better launch, ice the

intake manifold, run slicks, put the battery in the trunk, remove the

interior, etc. There is much more to the Impala than that, but if that is

indeed your goal, you'd really do much better starting off with a Camaro or

something a little lighter! <g> Scott.

------------------------------

Date: Tue, 12 Mar 1996 23:02:16 PST

From: Scott Mueller <73145.1566@compuserve.com>

To: <impala@east.xsis.xerox.com>

Subject: Hotchkiss control arms

Message-ID: <960313070215_73145.1566_FHD39-3@CompuServe.COM>

John,

>Maybe I can answer some suspension questions for you.

Gee I don't know, is there somebody higher up we can talk to? <BG> Now you

know I'm kidding... <g> Being able to talk directly with manufacturers like

yourself has to be one of the greatest things about the 'net in general and

this list in particular. Welcome to the list!

>The Impala SS trailing arms were designed with the Impala SS owner in mind.

The polyurethane bushing are our softest durometer (best insulation from

road noise), yet still provide great stability and traction.  The bushing

have grease fittings and do not squeak.

I appreciate the detailed information about your products! You have

addressed the problems I have had in the past with squeaking urethane

bushings. First of all being greasable is a major plus, and the fact that

you don't use as hard a urethane as perhaps what I have had experience with

is a big plus as well. I found that the unlubed hard urethane bushings used

by some of your competitors would not only squeak, but since there was no

lube they would also wear oversized and then begin to rattle. Some companies

started providing grease that you would use during assembly and

installation, but even this would quickly wash out and then you'd be back to

no lube at all. Having the grease fittings and I assume channels to hold the

grease in the bushing would solve this problem. I look forward to seeing the

control arms on Troy's car and perhaps several others at the show in April!

Will you or your company be at the Super Chevy show? Scott.

------------------------------

Date: Wed, 13 Mar 1996 00:05:45 PST

From: Mikado95@aol.com

To: impala@east.xsis.xerox.com

Subject: Impala Sites and Safety

Message-ID: <960313030544_445231325@mail06.mail.aol.com>

Brett wrote:

>How many of you aolers have visited the automotive message boards on aol?

>There is a folder that is for requests of new folders. I want all aolers who

>visit this place to request an Impala SS folder. I already have but had no

>results.

I have seen this, and I believe they can only have a certain amount of

folders at any one time, and that's why we aren't (I tried too) able to get

an "SS Folder".    I did search through some of the "muscle car" folders, and

read about two disturbing incidents involving Impalas.   Both took place in

Chicago this last December.   One man while at a stoplight was attacked and

locked in the trunk of his Impala while the thieves stole the car--the man

was found dead later.   Another situation occurred at a Chevy dealer in

Chicago.   Two men came during the day, test drove the Impala, then came back

later after the dealership had closed, killed the two managers, and stole the

Impala and another car.    Please be careful out there, as it seems the dark

side (not our friend Darth Vader) has taken too strong a liking to our cars.

Randy

------------------------------

Date: Wed, 13 Mar 1996 05:58:01 PST

From: jserko@terrabyte.net (John Serko)

To: impala@east.xsis.xerox.com

Subject: Plant closings

Message-ID: <19960313135801140.AAA14124@money.whole.net>

Woe are us who are waiting for GM to get it (and our cars) together.

Does anybody know an Internet address where we could monitor the strike

proceedings? CNN is good, but I was hoping for something more direct from

the source. Thanks in advance!

BTW, maybe we should all go on strike and refuse to send in those monthly

payments...

John Serko

waiting for 96 black - on strike

"Hey Beavis... this uuuhhhh, sucks or something..."

------------------------------

Date: Wed, 13 Mar 1996 06:42:19 PST

From: Michele Buckingham <michele@acesoft.com>

To: impala list <impala@east.xsis.xerox.com>

Subject: lease information

Message-ID: <Pine.A32.3.91.960313090013.5014A-100000@acesoft.com>

Hello, I would like to take a few minutes and induce myself.  My name is

Michele Buckingham I live in Grove City, Ohio(burb of columbus) and I have

never in my life been so excited about a car.

My husband had a 1966 impala when we were dating, so when the

impala's came back out he was on cloud nine.  Because he had just signed

a 4 year lease for a rodeo he could not get a new impala.  I thought at

the time he was just going off the deep end just for a car.  Well, the

company I have worked for over 11 years has been for the last 4 years

providing me with a van as a company car.  The van's lease will be up in

a few months so we went to the car show 2 weeks ago and looked at new

cars.  The only car/van I saw that I liked and had enough room was the

impala SS(which my mother also liked, she has a bonneville now and grey

hair, but has always had good taste in cars).  So this past saturday the

president of the company(my father) said ok I talked with the lease

manager and he has 3 go test drive one.

I just can't believe it my husband was right!!!(For all you men who say a

wife never admits that she was wrong, well I am admitting it now, I WAS

WRONG!!!) I fell in love with a car!!!!!!!

So, we talked to the lease manager (who I have all my driving

life leased cars from) about the Impala SS.  He said because the market

does not know what will happen with the Impala SS the residual was not

that high and the payments were about $100 a month higher.  I know by

looking at the resale of the impala's the residual should be higher but my

father just won't go for a $500.00 a month payment.

Does anyone know of a dealer/leasing company where we can get a closed

end lease at 48 months, No money down, 15,000 miles a year with a

residual around 48%??????

Desperately Seeking a Impala SS,

Michele Buckingham

michele@acesoft.com

------------------------------

End of IMPALA Digest 350

************************